Electron-Ion Collider, Requirements, Specifications, and Interfaces

Interface Control Document: [###]

Ancillary Detectors (6.03.10) to Collider Integration (6.05.01)

Prepared by:

{{Sig_es_:signer1:signature}}	Date: {{Dte_es_:signer1:date}}

Name, Role

Electron-Ion Collider at Institution

Reviewed by:

{{Sig_es_:signer2:signature}}	Date: {{Dte_es_:signer2:date}}

Name, Role

Electron-Ion Collider at Institution

{{Sig_es_:signer3:signature}}	Date: {{Dte_es_:signer3:date}}

Name, Role

Electron-Ion Collider at Institution

{{Sig_es_:signer4:signature}}	Date: {{Dte_es_:signer4:date}}

Name, Role

Electron-Ion Collider at Institution

Approved by:

{{Sig_es_:signer5:signature}}	Date: {{Dte_es_:signer5:date}}

Name, Role

Electron-Ion Collider at Institution

REVISION HISTORY

| Revision # | Effective Date | Additional Reviewers | Summary of Change |
| --- | --- | --- | --- |
| 00 |  |  | Initial release. |

TABLE OF CONTENTS

1.	Purpose and Scope	5

2.	Introduction	5

2.1	Ancillary Detectors	5

2.2	Overview of Interface Relationships	5

2.3	Safety and Environmental Considerations	5

2.4	Design Considerations	5

2.5	Organization	5

3.	Interface summary	6

4.	Ancillary Detectors (DET-ANC) to Collider Integration (COL-INT)	15

5.	Applicable Documents	24

6.	Detailed change Log	25

7.	References	25

LIST OF ACRONYMS

| B0 | B0 is the name of the forward magnet just past the Hadron Endcap |  |
| --- | --- | --- |
| CDR | Conceptual Design Report |  |
| COTS | Commercial Off the Shelf |  |
| DAQ | Data Acquisition |  |
| DET | EIC Detector |  |
| DIRC | Detection of Internally Reflected Cherenkov Light |  |
| dRICH | Dual radiator ring imaging Cherenkov |  |
| DVCS | Deeply Virtual Compton Scattering |  |
| EIC | Electron-Ion Collider |  |
| EMCAL | Electro-magnetic Calorimeter |  |
| ESR | Electron Storage Ring |  |
| FCC | Federal Communications Commission |  |
| FDR | Final Design Review |  |
| GEM | Gas Electron Multiplier |  |
| HCAL | Hadronic Calorimeter |  |
| HSR | Hadron Storage Ring |  |
| HVAC | Heating/Ventilation/Air Conditioning |  |
| IP | Interaction Point |  |
| IR | Interaction Region |  |
| MPGD | Micro Pattern Gaseous Detectors |  |
| NAS | National Academy of Sciences |  |
| NEC | National Electric Code |  |
| NEMA | National Electrical Manufacturers Association |  |
| NFPA | National Fire Protection Association |  |
| NRTL | Nationally Recognized Testing Laboratory |  |
| PID | Particle Identification |  |
| PRR | Production Readiness Review |  |
| PWO | PbWO4 |  |
| RCS | Rapid Cycling Synchrotron |  |
| TBD | To Be Determined |  |
| UL | Underwriter Laboratories |  |
| WBS | Work Break Down Structure |  |

# Interface Control Document:

Ancillary Detectors (6.03.10) to Collider Integration (6.05.01)

# Purpose and Scope

This document records the agreed upon interfaces between the Ancillary Detectors and other sub-systems within the Collider Integration. These interfaces are based on the system requirements that are necessary to integrate these sub-systems as part of the Electron Ion Collider Project.

# Introduction

These interfaces describe the relationships between the Ancillary Detectors and the Collider Integration including (examples by inclusion) located in (areas by inclusion).

## Ancillary Detectors

Provide an introductory description of your sub-system and an overview of the things that it receives, provides, supports, constrains, or otherwise requires as part of its function --- specifically if these things are manifested as interfaces.

## Overview of Interface Relationships

The following are the sub-systems within the Collider Integration that have interfaces connected to the Ancillary Detectors.

Collider Integration (COL-INT): Identify the general types of interface relationships between the ICD reference system and this sub-system.

## Safety and Environmental Considerations

General safety and environmental considerations are identified here. Any considerations that are specific to the individual sub-system, will be addressed in external safety documents and are referenced here.

### Considerations for all systems

Add general safety and environmental considerations here.

### Special considerations and references

Add references here as necessary.

## Design Considerations

Notes here about general design considerations.

## Organization

The interfaces in this document are grouped by sub-system as identified in the sub-system identifier. All interfaces between the Ancillary Detectors and a related sub-system will be included in a single section. Entries within that section will be sorted by the interface identifier or WBS number, as appropriate.

# Interface summary

This tables in this section provide a list of all interfaces that are defined within this document. The interfaces are grouped by sub-system/WBS. Each table has the following fields:

Section: the section number in this document where the interface is addressed.

WBS (1): the WBS number within the ICD reference system, unless the reference system is the owner.

WBS (2): the WBS number for the connected system.

ID: the interface identifier as defined in the Interface Requirement Document (IRD).

Name: the name of the interface as defined in the IRD.

Description: the description of the interface as defined in the IRD.

Owner: identifies the WBS number of the entity that is responsible for ensuring the interface is provided and that it meets validation/verification requirements.

Ancillary Detectors to Collider Integration

| Section | WBS1 | WBS2 | ID | Name | Description | Owner |
| --- | --- | --- | --- | --- | --- | --- |
| 4 |  | 6.05.01 |  |  | Collider Integration |  |
| 4.1 | 6.03.10 | 6.05.01 | I-DET-ANC-LOWQ2.002 | Accelerator Interference | The performance of the LowQ2 detector will depend on the inner apertures and locations of IR magnets (from IP to B2eR) | 6.03.10.04 |
| 4.2 | 6.03.10 | 6.05.01 | I-DET-ANC-LOWQ2.003 | Electronics Placement and Shielding | The Low-Q2 detector electronics enclosures must be positioned near the detector and must be shielded from radiation. | 6.03.10.04 |
| 4.3 | 6.03.10 | 6.05.01 | I-DET-ANC-LOWQ2.004 | LowQ2 Beamline Proximity | The LowQ2 detector must be positioned as close as possible electron beampipe in order to detector particles at a shallow angle from the electron beam. | 6.03.10.04 |
| 4.4 | 6.03.10 | 6.05.01 | I-DET-ANC-LOWQ2.005 | Process Cooling | Either a liquid or gas cooling system will be required to remove heat from the calorimeter, tracking and readout electronics, to maintain them at room temperature. A chiller should be installed near the sub-system and a space in the tunnel should be allocated for this chiller. | 6.03.10.04 |
| 4.5 | 6.03.10 | 6.05.01 | I-DET-ANC-LUMI.002 | Accelerator Interference | Detector must be positioned between the electron and hadron beamlines in the far backward area, and its size and position are governed by the adjacent accelerator components. | 6.03.10.05 |
| 4.6 | 6.03.10 | 6.05.01 | I-DET-ANC-LUMI.003 | Beamline Alignment | The center of the luminosity detector must be positioned along the centerline of the incoming electron beam as it passes through the interaction point. | 6.03.10.05 |
| 4.7 | 6.03.10 | 6.05.01 | I-DET-ANC-LUMI.004 | Detector Placement | The detector must be positioned to avoid magnetic interference from adjacent beamline magnets. | 6.03.10.05 |
| 4.8 | 6.03.10 | 6.05.01 | I-DET-ANC-LUMI.005 | Dipole Cooling | The two dipoles in front of the detector will need LCW cooling. | 6.03.10.05 |
| 4.9 | 6.03.10 | 6.05.01 | I-DET-ANC-LUMI.006 | Electronics Placement and Shielding | The detector electronics enclosures must be positioned near the detector and must be shielded from radiation. | 6.03.10.05 |
| 4.10 | 6.03.10 | 6.05.01 | I-DET-ANC-LUMI.007 | Placement | The luminosity detector must be positioned between 30 and 100 meters from the interaction point in the backward direction, and the detector's center point must be positioned at beam level. | 6.03.10.05 |
| 4.11 | 6.03.10 | 6.05.01 | I-DET-ANC-LUMI.008 | Process Cooling | A cooling system will be required to remove heat from the tracking and readout electronics to prevent heat buildup. A chiller should be installed near the sub-system and a space in the tunnel should be allocated for this chiller. | 6.03.10.05 |
| 4.12 | 6.03.10 | 6.05.01 | I-DET-ANC-OFFMO.001 | Photon Cone Interference | There should minimal interference within the 1 mrad cone of direct photons from IP (with respect to the incoming e-line) in front of the lumi-system. | 6.03.10.01 |
| 4.13 | 6.03.10 | 6.05.01 | I-DET-ANC-OFFMO.003 | Accelerator Interference | This detector will contribute to the hadron accelerator impedance. Placement and location of detector layers have to be optimized. | 6.03.10.01 |
| 4.14 | 6.03.10 | 6.05.01 | I-DET-ANC-OFFMO.004 | Backward Space Constraint | The detector must be positioned after the B1APF dipole in the hadron going direction. | 6.03.10.01 |
| 4.15 | 6.03.10 | 6.05.01 | I-DET-ANC-OFFMO.005 | Forward Space Constraint | The detector must be positioned before the Roman Pots in the hadron going direction. | 6.03.10.01 |
| 4.16 | 6.03.10 | 6.05.01 | I-DET-ANC-OFFMO.006 | Process Cooling | A cooling system will be required to remove heat from the tracking and readout electronics to prevent heat buildup. A chiller should be installed near the sub-system and a space in the tunnel should be allocated for this chiller. | 6.03.10.01 |
| 4.17 | 6.03.10 | 6.05.01 | I-DET-ANC-OFFMO.007 | RF Shielding | This detector will generate RF radiation that the accelerator beamline must be shielded from. | 6.03.10.01 |
| 4.18 | 6.03.10 | 6.05.01 | I-DET-ANC-OFFMO.008 | Zero-Degree Calorimeter Interference | The size and placement of the Off-momentum detector and their vacuum scattering chamber should be designed to have minimal interference with the 4 milliradian neutron cone required by the zero-degree calorimeter. | 6.03.10.01 |
| 4.19 | 6.03.10 | 6.05.01 | I-DET-ANC-ROMAN.002 | Accelerator Interference | The Roman Pot detectors will come within 10 sigma of the center of the beam, and will impact the beam characteristics in a way that must be compensated for. | 6.03.10.01 |
| 4.20 | 6.03.10 | 6.05.01 | I-DET-ANC-ROMAN.003 | Accelerator Interference | This detector will contribute to the hadron accelerator impedance. Placement and location of detector layers have to be optimized. | 6.03.10.01 |
| 4.21 | 6.03.10 | 6.05.01 | I-DET-ANC-ROMAN.004 | Backward Space Constraint | The detector must be positioned after the off-momentum detectors in the hadron going direction. | 6.03.10.01 |
| 4.22 | 6.03.10 | 6.05.01 | I-DET-ANC-ROMAN.005 | Beamline Integration | A cooling system will be required to remove heat from the tracking and readout electronics to prevent heat buildup. Feedthroughs in the vacuum scattering chamber have to be allocated. Feedthroughs in the vacuum scattering chamber have to be allocated for cables and for mechanical connections to the moving stages. | 6.03.10.01 |
| 4.23 | 6.03.10 | 6.05.01 | I-DET-ANC-ROMAN.006 | Forward Space Constraint | The detector must be positioned before the B2PF magnet in the hadron going direction. | 6.03.10.01 |
| 4.24 | 6.03.10 | 6.05.01 | I-DET-ANC-ROMAN.007 | Process Cooling | A cooling system will be required to remove heat from the tracking and readout electronics to prevent heat buildup. A chiller should be installed near the sub-system and a space in the tunnel should be allocated for this chiller. | 6.03.10.01 |
| 4.25 | 6.03.10 | 6.05.01 | I-DET-ANC-ROMAN.008 | RF Shielding | This detector will generate RF radiation that the accelerator beamline must be shielded from. | 6.03.10.01 |
| 4.26 | 6.03.10 | 6.05.01 | I-DET-ANC-ZDC.001 | Neutron Cone Dependency | The placement of the Zero Degree Calorimeter is dependent on the neutron cone and should be positioned to optimize detector performance and resolution. | 6.03.10.03 |
| 4.27 | 6.03.10 | 6.05.01 | I-DET-ANC-ZDC.002 | Neutron Cone Interference | There should minimal interference within the 4 milliradian neutron cone in front of the Zero Degree Calorimeter detector. | 6.03.10.03 |
| 4.28 | 6.03.10 | 6.05.01 | I-DET-ANC-ZDC.003 | Process Cooling | A cooling system will be required to remove heat from the calorimeter and readout electronics to prevent heat buildup. A chiller should be installed near the sub-system and a space in the tunnel should be allocated for this chiller. | 6.03.10.03 |
| 4.29 | 6.03.10 | 6.05.01 | I-DET-ANC-ZDC.004 | ZDC Placement | The Zero Degree Calorimeter will be positioned between the outgoing hadron and incoming electron beamlines. | 6.03.10.03 |
| 4.30 | 6.03.10 | 6.05.01 | I-DET-INF-END.002 | Detector Support | The off-momentum detectors will be supported by an integrated support stand that is fully integrated with the vacuum system and allows the position of the detector to be adjusted. | 6.03.09.02 |
| 4.31 | 6.03.10 | 6.05.01 | I-DET-INF-END.003 | Detector Support | The roman-pot detectors will be supported by an integrated support stand that is fully integrated with the vacuum system and allows the position of the detector to be adjusted. | 6.03.09.02 |
| 4.32 | 6.03.10 | 6.05.01 | I-DET-INF-INT.004 | Process Cooling | A cooling system will be required to remove heat from the B0-calorimeter and electronics readouts, to maintain an acceptable temperature. A chiller should be installed near the sub-system and a space should be allocated for this chiller. | 6.03.09.03 |
| 4.33 | 6.03.10 | 6.05.01 | I-DET-INF-INT.005 | Space Constraint | Access to the B0 detector is constrained by the vacuum valve in front of the B0 magnet. A minimum of 15 centimeters of free space will be required on the IP-side of the B0-dipole for the installation of the B0 detectors. | 6.03.09.03 |
| 4.34 | 6.03.10 | 6.05.01 | I-DET-INF-INT.006 | Space Constraint | The electron and hadron beampipes must be contained within the B0 detector. | 6.03.09.03 |
| 4.35 | 6.03.10 | 6.05.01 | I-DET-INF-INT.010 | Position Constraint | The Low-Q2 detector must be upstream in the electron direction from the Q3ER magnet. | 6.03.09.03 |
| 4.36 | 6.03.10 | 6.05.01 | I-DET-INF-INT.011 | Position Constraint | The Low-Q2 detector must be downstream in the electron direction from the B2ER magnet. | 6.03.09.03 |
| 4.37 | 6.03.10 | 6.05.01 | I-DET-INF-INT.012 | Position Constraint | The Low-Q2 detector must be at the same level as the electron beamline. | 6.03.09.03 |

# Ancillary Detectors (DET-ANC) to Collider Integration (COL-INT)

I-DET-ANC-LOWQ2.002	Accelerator Interference

System 1:	Low Q2 Detectors (6.03.10.04)

System 2:	Collider Integration (6.05.01)

Type:	ENV

Description:	The performance of the LowQ2 detector will depend on the inner apertures and locations of IR magnets (from IP to B2eR)

Provider:	Low Q2 Detectors (6.03.10.04)

Details:	See details in the Ancillary Detectors Interface Control Document.

Requirements:

References:

I-DET-ANC-LOWQ2.003	Electronics Placement and Shielding

System 1:	Low Q2 Detectors (6.03.10.04)

System 2:	Collider Integration (6.05.01)

Type:	ENV

Description:	The Low-Q2 detector electronics enclosures must be positioned near the detector and must be shielded from radiation.

Provider:	Low Q2 Detectors (6.03.10.04)

Details:	See details in the Ancillary Detectors Interface Control Document.

Requirements:

References:

I-DET-ANC-LOWQ2.004	LowQ2 Beamline Proximity

System 1:	Low Q2 Detectors (6.03.10.04)

System 2:	Collider Integration (6.05.01)

Type:	SPACE

Description:	The LowQ2 detector must be positioned as close as possible electron beampipe in order to detector particles at a shallow angle from the electron beam.

Provider:	Low Q2 Detectors (6.03.10.04)

Details:	See details in the Ancillary Detectors Interface Control Document.

Requirements:	P-DET-ANC-LOWQ2.16

References:

I-DET-ANC-LOWQ2.005	Process Cooling

System 1:	Low Q2 Detectors (6.03.10.04)

System 2:	Collider Integration (6.05.01)

Type:	SPACE

Description:	Either a liquid or gas cooling system will be required to remove heat from the calorimeter, tracking and readout electronics, to maintain them at room temperature. A chiller should be installed near the sub-system and a space in the tunnel should be allocated for this chiller.

Provider:	Low Q2 Detectors (6.03.10.04)

Details:	See details in the Ancillary Detectors Interface Control Document.

Requirements:	P-DET-ANC.3

References:

I-DET-ANC-LUMI.002	Accelerator Interference

System 1:	Luminosity Detectors (6.03.10.05)

System 2:	Collider Integration (6.05.01)

Type:	SPACE

Description:	Detector must be positioned between the electron and hadron beamlines in the far backward area, and its size and position are governed by the adjacent accelerator components.

Provider:	Luminosity Detectors (6.03.10.05)

Details:	See details in the Ancillary Detectors Interface Control Document.

Requirements:	P-DET-ANC.1

References:

I-DET-ANC-LUMI.003	Beamline Alignment

System 1:	Luminosity Detectors (6.03.10.05)

System 2:	Collider Integration (6.05.01)

Type:	SPACE

Description:	The center of the luminosity detector must be positioned along the centerline of the incoming electron beam as it passes through the interaction point.

Provider:	Luminosity Detectors (6.03.10.05)

Details:	See details in the Ancillary Detectors Interface Control Document.

Requirements:	P-DET-ANC.1

References:

I-DET-ANC-LUMI.004	Detector Placement

System 1:	Luminosity Detectors (6.03.10.05)

System 2:	Collider Integration (6.05.01)

Type:	ENV

Description:	The detector must be positioned to avoid magnetic interference from adjacent beamline magnets.

Provider:	Luminosity Detectors (6.03.10.05)

Details:	See details in the Ancillary Detectors Interface Control Document.

Requirements:	P-DET-ANC-LUMI.9

References:

I-DET-ANC-LUMI.005	Dipole Cooling

System 1:	Luminosity Detectors (6.03.10.05)

System 2:	Collider Integration (6.05.01)

Type:	COOL

Description:	The two dipoles in front of the detector will need LCW cooling.

Provider:	Luminosity Detectors (6.03.10.05)

Details:	See details in the Ancillary Detectors Interface Control Document.

Requirements:	P-DET-ANC.3

References:

I-DET-ANC-LUMI.006	Electronics Placement and Shielding

System 1:	Luminosity Detectors (6.03.10.05)

System 2:	Collider Integration (6.05.01)

Type:	ENV

Description:	The detector electronics enclosures must be positioned near the detector and must be shielded from radiation.

Provider:	Luminosity Detectors (6.03.10.05)

Details:	See details in the Ancillary Detectors Interface Control Document.

Requirements:	P-DET-ANC-LUMI.9

References:

I-DET-ANC-LUMI.007	Placement

System 1:	Luminosity Detectors (6.03.10.05)

System 2:	Collider Integration (6.05.01)

Type:	SPACE

Description:	The luminosity detector must be positioned between 30 and 100 meters from the interaction point in the backward direction, and the detector's center point must be positioned at beam level.

Provider:	Luminosity Detectors (6.03.10.05)

Details:	See details in the Ancillary Detectors Interface Control Document.

Requirements:	P-DET-ANC.1

References:

I-DET-ANC-LUMI.008	Process Cooling

System 1:	Luminosity Detectors (6.03.10.05)

System 2:	Collider Integration (6.05.01)

Type:	SPACE

Description:	A cooling system will be required to remove heat from the tracking and readout electronics to prevent heat buildup. A chiller should be installed near the sub-system and a space in the tunnel should be allocated for this chiller.

Provider:	Luminosity Detectors (6.03.10.05)

Details:	See details in the Ancillary Detectors Interface Control Document.

Requirements:	P-DET-ANC.3

References:

I-DET-ANC-OFFMO.001	Photon Cone Interference

System 1:	Luminosity Detectors (6.03.10.05)

System 2:	Collider Integration (6.05.01)

Type:	ENV

Description:	There should minimal interference within the 1 mrad cone of direct photons from IP (with respect to the incoming e-line) in front of the lumi-system.

Provider:	Off-Momentum Detectors (6.03.10.01)

Details:	See details in the Ancillary Detectors Interface Control Document.

Requirements:	P-DET-ANC-LUMI.3

References:

I-DET-ANC-OFFMO.003	Accelerator Interference

System 1:	Off-Momentum Detectors (6.03.10.01)

System 2:	Collider Integration (6.05.01)

Type:	ENV

Description:	This detector will contribute to the hadron accelerator impedance. Placement and location of detector layers have to be optimized.

Provider:	Off-Momentum Detectors (6.03.10.01)

Details:	See details in the Ancillary Detectors Interface Control Document.

Requirements:	P-DET-ANC-OFFMO

References:

I-DET-ANC-OFFMO.004	Backward Space Constraint

System 1:	Off-Momentum Detectors (6.03.10.01)

System 2:	Collider Integration (6.05.01)

Type:	SPACE

Description:	The detector must be positioned after the B1APF dipole in the hadron going direction.

Provider:	Off-Momentum Detectors (6.03.10.01)

Details:	See details in the Ancillary Detectors Interface Control Document.

Requirements:	P-DET-ANC.1

References:

I-DET-ANC-OFFMO.005	Forward Space Constraint

System 1:	Off-Momentum Detectors (6.03.10.01)

System 2:	Collider Integration (6.05.01)

Type:	SPACE

Description:	The detector must be positioned before the Roman Pots in the hadron going direction.

Provider:	Off-Momentum Detectors (6.03.10.01)

Details:	See details in the Ancillary Detectors Interface Control Document.

Requirements:	P-DET-ANC.1

References:

I-DET-ANC-OFFMO.006	Process Cooling

System 1:	Off-Momentum Detectors (6.03.10.01)

System 2:	Collider Integration (6.05.01)

Type:	SPACE

Description:	A cooling system will be required to remove heat from the tracking and readout electronics to prevent heat buildup. A chiller should be installed near the sub-system and a space in the tunnel should be allocated for this chiller.

Provider:	Off-Momentum Detectors (6.03.10.01)

Details:	See details in the Ancillary Detectors Interface Control Document.

Requirements:	P-DET-ANC.3

References:

I-DET-ANC-OFFMO.007	RF Shielding

System 1:	Off-Momentum Detectors (6.03.10.01)

System 2:	Collider Integration (6.05.01)

Type:	ENV

Description:	This detector will generate RF radiation that the accelerator beamline must be shielded from.

Provider:	Off-Momentum Detectors (6.03.10.01)

Details:	See details in the Ancillary Detectors Interface Control Document.

Requirements:	P-DET-ANC-OFFMO.9

References:

I-DET-ANC-OFFMO.008	Zero-Degree Calorimeter Interference

System 1:	Off-Momentum Detectors (6.03.10.01)

System 2:	Collider Integration (6.05.01)

Type:	SPACE

Description:	The size and placement of the Off-momentum detector and their vacuum scattering chamber should be designed to have minimal interference with the 4 milliradian neutron cone required by the zero-degree calorimeter.

Provider:	Off-Momentum Detectors (6.03.10.01)

Details:	See details in the Ancillary Detectors Interface Control Document.

Requirements:	P-DET-ANC.1

References:

I-DET-ANC-ROMAN.002	Accelerator Interference

System 1:	Roman Pots (6.03.10.01)

System 2:	Collider Integration (6.05.01)

Type:	PARAM

Description:	The Roman Pot detectors will come within 10 sigma of the center of the beam, and will impact the beam characteristics in a way that must be compensated for.

Provider:	Roman Pots (6.03.10.01)

Details:	See details in the Ancillary Detectors Interface Control Document.

Requirements:	P-DET-ANC-ROMAN.10

References:

I-DET-ANC-ROMAN.003	Accelerator Interference

System 1:	Roman Pots (6.03.10.01)

System 2:	Collider Integration (6.05.01)

Type:	ENV

Description:	This detector will contribute to the hadron accelerator impedance. Placement and location of detector layers have to be optimized.

Provider:	Roman Pots (6.03.10.01)

Details:	See details in the Ancillary Detectors Interface Control Document.

Requirements:	P-DET-ANC-ROMAN.9

References:

I-DET-ANC-ROMAN.004	Backward Space Constraint

System 1:	Roman Pots (6.03.10.01)

System 2:	Collider Integration (6.05.01)

Type:	SPACE

Description:	The detector must be positioned after the off-momentum detectors in the hadron going direction.

Provider:	Roman Pots (6.03.10.01)

Details:	See details in the Ancillary Detectors Interface Control Document.

Requirements:	P-DET-ANC.1

References:

I-DET-ANC-ROMAN.005	Beamline Integration

System 1:	Roman Pots (6.03.10.01)

System 2:	Collider Integration (6.05.01)

Type:	VAC

Description:	A cooling system will be required to remove heat from the tracking and readout electronics to prevent heat buildup. Feedthroughs in the vacuum scattering chamber have to be allocated. Feedthroughs in the vacuum scattering chamber have to be allocated for cables and for mechanical connections to the moving stages.

Provider:	Roman Pots (6.03.10.01)

Details:	See details in the Ancillary Detectors Interface Control Document.

Requirements:	P-DET-ANC.3

References:

I-DET-ANC-ROMAN.006	Forward Space Constraint

System 1:	Roman Pots (6.03.10.01)

System 2:	Collider Integration (6.05.01)

Type:	SPACE

Description:	The detector must be positioned before the B2PF magnet in the hadron going direction.

Provider:	Roman Pots (6.03.10.01)

Details:	See details in the Ancillary Detectors Interface Control Document.

Requirements:	P-DET-ANC.1

References:

I-DET-ANC-ROMAN.007	Process Cooling

System 1:	Roman Pots (6.03.10.01)

System 2:	Collider Integration (6.05.01)

Type:	SPACE

Description:	A cooling system will be required to remove heat from the tracking and readout electronics to prevent heat buildup. A chiller should be installed near the sub-system and a space in the tunnel should be allocated for this chiller.

Provider:	Roman Pots (6.03.10.01)

Details:	See details in the Ancillary Detectors Interface Control Document.

Requirements:	P-DET-ANC.3

References:

I-DET-ANC-ROMAN.008	RF Shielding

System 1:	Roman Pots (6.03.10.01)

System 2:	Collider Integration (6.05.01)

Type:	ENV

Description:	This detector will generate RF radiation that the accelerator beamline must be shielded from.

Provider:	Roman Pots (6.03.10.01)

Details:	See details in the Ancillary Detectors Interface Control Document.

Requirements:	P-DET-ANC-ROMAN.9

References:

I-DET-ANC-ZDC.001	Neutron Cone Dependency

System 1:	Zero Degree Calorimeter (6.03.10.03)

System 2:	Collider Integration (6.05.01)

Type:	SPACE

Description:	The placement of the Zero Degree Calorimeter is dependent on the neutron cone and should be positioned to optimize detector performance and resolution.

Provider:	Zero Degree Calorimeter (6.03.10.03)

Details:	See details in the Ancillary Detectors Interface Control Document.

Requirements:	P-DET-ANC.1

References:

I-DET-ANC-ZDC.002	Neutron Cone Interference

System 1:	Zero Degree Calorimeter (6.03.10.03)

System 2:	Collider Integration (6.05.01)

Type:	ENV

Description:	There should minimal interference within the 4 milliradian neutron cone in front of the Zero Degree Calorimeter detector.

Provider:	Zero Degree Calorimeter (6.03.10.03)

Details:	See details in the Ancillary Detectors Interface Control Document.

Requirements:	P-DET-ANC.1

References:

I-DET-ANC-ZDC.003	Process Cooling

System 1:	Zero Degree Calorimeter (6.03.10.03)

System 2:	Collider Integration (6.05.01)

Type:	SPACE

Description:	A cooling system will be required to remove heat from the calorimeter and readout electronics to prevent heat buildup. A chiller should be installed near the sub-system and a space in the tunnel should be allocated for this chiller.

Provider:	Zero Degree Calorimeter (6.03.10.03)

Details:	See details in the Ancillary Detectors Interface Control Document.

Requirements:	P-DET-ANC.3

References:

I-DET-ANC-ZDC.004	ZDC Placement

System 1:	Zero Degree Calorimeter (6.03.10.03)

System 2:	Collider Integration (6.05.01)

Type:	SPACE

Description:	The Zero Degree Calorimeter will be positioned between the outgoing hadron and incoming electron beamlines.

Provider:	Zero Degree Calorimeter (6.03.10.03)

Details:	See details in the Ancillary Detectors Interface Control Document.

Requirements:	P-DET-ANC.1

References:

I-DET-INF-END.002	Detector Support

System 1:	Off-Momentum Detectors (6.03.10.01)

System 2:	Collider Integration (6.05.01)

Type:	STRUCT

Description:	The off-momentum detectors will be supported by an integrated support stand that is fully integrated with the vacuum system and allows the position of the detector to be adjusted.

Provider:	Hadron and Electron Endcap Structures (6.03.09.02)

Details:	See details in the Detector Infrastructure Interface Control Document.

Requirements:	P-DET-ANC.1, P-DET-ANC.2

References:

I-DET-INF-END.003	Detector Support

System 1:	Roman Pots (6.03.10.01)

System 2:	Collider Integration (6.05.01)

Type:	STRUCT

Description:	The roman-pot detectors will be supported by an integrated support stand that is fully integrated with the vacuum system and allows the position of the detector to be adjusted.

Provider:	Hadron and Electron Endcap Structures (6.03.09.02)

Details:	See details in the Detector Infrastructure Interface Control Document.

Requirements:	P-DET-ANC.1, P-DET-ANC.2

References:

I-DET-INF-INT.004	Process Cooling

System 1:	B-Zero Detectors (6.03.10.02)

System 2:	Collider Integration (6.05.01)

Type:	SPACE

Description:	A cooling system will be required to remove heat from the B0-calorimeter and electronics readouts, to maintain an acceptable temperature. A chiller should be installed near the sub-system and a space should be allocated for this chiller.

Provider:	Detector Infrastructure and Utilities Integration (6.03.09.03)

Details:	See details in the Detector Infrastructure Interface Control Document.

Requirements:	P-DET-ANC.3

References:

I-DET-INF-INT.005	Space Constraint

System 1:	B-Zero Detectors (6.03.10.02)

System 2:	Collider Integration (6.05.01)

Type:	SPACE

Description:	Access to the B0 detector is constrained by the vacuum valve in front of the B0 magnet. A minimum of 15 centimeters of free space will be required on the IP-side of the B0-dipole for the installation of the B0 detectors.

Provider:	Detector Infrastructure and Utilities Integration (6.03.09.03)

Details:	See details in the Detector Infrastructure Interface Control Document.

Requirements:	P-DET-ANC.1

References:

I-DET-INF-INT.006	Space Constraint

System 1:	B-Zero Detectors (6.03.10.02)

System 2:	Collider Integration (6.05.01)

Type:	SPACE

Description:	The electron and hadron beampipes must be contained within the B0 detector.

Provider:	Detector Infrastructure and Utilities Integration (6.03.09.03)

Details:	See details in the Detector Infrastructure Interface Control Document.

Requirements:	P-DET-ANC.1

References:

I-DET-INF-INT.010	Position Constraint

System 1:	Low Q2 Detectors (6.03.10.04)

System 2:	Collider Integration (6.05.01)

Type:	SPACE

Description:	The Low-Q2 detector must be upstream in the electron direction from the Q3ER magnet.

Provider:	Detector Infrastructure and Utilities Integration (6.03.09.03)

Details:	See details in the Detector Infrastructure Interface Control Document.

Requirements:	P-DET-ANC.1

References:

I-DET-INF-INT.011	Position Constraint

System 1:	Low Q2 Detectors (6.03.10.04)

System 2:	Collider Integration (6.05.01)

Type:	SPACE

Description:	The Low-Q2 detector must be downstream in the electron direction from the B2ER magnet.

Provider:	Detector Infrastructure and Utilities Integration (6.03.09.03)

Details:	See details in the Detector Infrastructure Interface Control Document.

Requirements:	P-DET-ANC.1

References:

I-DET-INF-INT.012	Position Constraint

System 1:	Low Q2 Detectors (6.03.10.04)

System 2:	Collider Integration (6.05.01)

Type:	SPACE

Description:	The Low-Q2 detector must be at the same level as the electron beamline.

Provider:	Detector Infrastructure and Utilities Integration (6.03.09.03)

Details:	See details in the Detector Infrastructure Interface Control Document.

Requirements:	P-DET-ANC.1

References:

# Applicable Documents

List applicable documents considered to be part of this document. Applicable documents shall be obtained from the EIC SharePoint Document Center.

# Detailed change Log

| Revision | Date | Change |
| --- | --- | --- |

# References

- doi:10.17226/25171., National Academies of Sciences, Engineering, and Medicine, "An Assessment of U.S.-Based Electron-Ion Collider Science", The National Academies Press, Washington, DC, 2018.

- EIC-ORG-PLN-010, Electron-Ion Collider L1 Requirements (Global Requirements Document)

- BNL-221006-2021-FORE, EIC Conceptual Design Report

- arXiv:2103.05419, Science Requirements and Detector Concepts for the Electron-Ion Collider: EIC Yellow Report

- EIC-SEG-PLN-016, Requirements Management Plan

- EIC-SEG-PLN-022, Systems Engineering Management Plan

- EIC-SEG-PLN-020, EIC Interface Management Plan