Electron-Ion Collider, Requirements, Specifications, and Interfaces

Interface Control Document: [###]

Data Acquisition and Computing Systems (6.03.08) to Collider Global Systems (6.05.02)

Prepared by:

{{Sig_es_:signer1:signature}}	Date: {{Dte_es_:signer1:date}}

Name, Role

Electron-Ion Collider at Institution

Reviewed by:

{{Sig_es_:signer2:signature}}	Date: {{Dte_es_:signer2:date}}

Name, Role

Electron-Ion Collider at Institution

{{Sig_es_:signer3:signature}}	Date: {{Dte_es_:signer3:date}}

Name, Role

Electron-Ion Collider at Institution

{{Sig_es_:signer4:signature}}	Date: {{Dte_es_:signer4:date}}

Name, Role

Electron-Ion Collider at Institution

Approved by:

{{Sig_es_:signer5:signature}}	Date: {{Dte_es_:signer5:date}}

Name, Role

Electron-Ion Collider at Institution

REVISION HISTORY

| Revision # | Effective Date | Additional Reviewers | Summary of Change |
| --- | --- | --- | --- |
| 00 |  |  | Initial release. |

TABLE OF CONTENTS

1.	Purpose and Scope	5

2.	Introduction	5

2.1	Data Acquisition and Computing Systems	5

2.2	Overview of Interface Relationships	5

2.3	Safety and Environmental Considerations	5

2.4	Design Considerations	5

2.5	Organization	5

3.	Interface summary	6

4.	Data Acquisition and Computing Systems (DET-COMP) to Collider Global Systems (COL-GBL)	7

5.	Applicable Documents	8

6.	Detailed change Log	8

7.	References	8

LIST OF ACRONYMS

| B0 | B0 is the name of the forward magnet just past the Hadron Endcap |  |
| --- | --- | --- |
| CDR | Conceptual Design Report |  |
| COTS | Commercial Off the Shelf |  |
| DAQ | Data Acquisition |  |
| DET | EIC Detector |  |
| DIRC | Detection of Internally Reflected Cherenkov Light |  |
| dRICH | Dual radiator ring imaging Cherenkov |  |
| DVCS | Deeply Virtual Compton Scattering |  |
| EIC | Electron-Ion Collider |  |
| EMCAL | Electro-magnetic Calorimeter |  |
| ESR | Electron Storage Ring |  |
| FCC | Federal Communications Commission |  |
| FDR | Final Design Review |  |
| GEM | Gas Electron Multiplier |  |
| HCAL | Hadronic Calorimeter |  |
| HSR | Hadron Storage Ring |  |
| HVAC | Heating/Ventilation/Air Conditioning |  |
| IP | Interaction Point |  |
| IR | Interaction Region |  |
| MPGD | Micro Pattern Gaseous Detectors |  |
| NAS | National Academy of Sciences |  |
| NEC | National Electric Code |  |
| NEMA | National Electrical Manufacturers Association |  |
| NFPA | National Fire Protection Association |  |
| NRTL | Nationally Recognized Testing Laboratory |  |
| PID | Particle Identification |  |
| PRR | Production Readiness Review |  |
| PWO | PbWO4 |  |
| RCS | Rapid Cycling Synchrotron |  |
| TBD | To Be Determined |  |
| UL | Underwriter Laboratories |  |
| WBS | Work Break Down Structure |  |

# Interface Control Document:

Data Acquisition and Computing Systems (6.03.08) to Collider Global Systems (6.05.02)

# Purpose and Scope

This document records the agreed upon interfaces between the Data Acquisition and Computing Systems and other sub-systems within the Collider Global Systems. These interfaces are based on the system requirements that are necessary to integrate these sub-systems as part of the Electron Ion Collider Project.

# Introduction

These interfaces describe the relationships between the Data Acquisition and Computing Systems and the Collider Global Systems including (examples by inclusion) located in (areas by inclusion).

## Data Acquisition and Computing Systems

Provide an introductory description of your sub-system and an overview of the things that it receives, provides, supports, constrains, or otherwise requires as part of its function --- specifically if these things are manifested as interfaces.

## Overview of Interface Relationships

The following are the sub-systems within the Collider Global Systems that have interfaces connected to the Data Acquisition and Computing Systems.

Collider Global Systems (COL-GBL): Identify the general types of interface relationships between the ICD reference system and this sub-system.

## Safety and Environmental Considerations

General safety and environmental considerations are identified here. Any considerations that are specific to the individual sub-system, will be addressed in external safety documents and are referenced here.

### Considerations for all systems

Add general safety and environmental considerations here.

### Special considerations and references

Add references here as necessary.

## Design Considerations

Notes here about general design considerations.

## Organization

The interfaces in this document are grouped by sub-system as identified in the sub-system identifier. All interfaces between the Data Acquisition and Computing Systems and a related sub-system will be included in a single section. Entries within that section will be sorted by the interface identifier or WBS number, as appropriate.

# Interface summary

This tables in this section provide a list of all interfaces that are defined within this document. The interfaces are grouped by sub-system/WBS. Each table has the following fields:

Section: the section number in this document where the interface is addressed.

WBS (1): the WBS number within the ICD reference system, unless the reference system is the owner.

WBS (2): the WBS number for the connected system.

ID: the interface identifier as defined in the Interface Requirement Document (IRD).

Name: the name of the interface as defined in the IRD.

Description: the description of the interface as defined in the IRD.

Owner: identifies the WBS number of the entity that is responsible for ensuring the interface is provided and that it meets validation/verification requirements.

Data Acquisition and Computing Systems to Collider Global Systems

| Section | WBS1 | WBS2 | ID | Name | Description | Owner |
| --- | --- | --- | --- | --- | --- | --- |
| 4 |  | 6.05.02 |  |  | Collider Global Systems |  |
| 4.1 | 6.03.08 | 6.05.02 | I-DET-COMP-CONTROLS.001 | Detector Feedback Interface | An interface should be provided that allows the accelerator control system to read the condition and state of the detector systems. | 6.03.08.02 |
| 4.2 | 6.03.08 | 6.05.02 | I-DET-COMP-DAQ.004 | Accelerator Data Interface | An interface to the accelerator beam databases must be provided that allows the DAQ to read data, and on a limited basis, write data. | 6.03.08.01 |
| 4.3 | 6.03.08 | 6.05.02 | I-DET-COMP-DAQ.005 | Accelerator Timing Interface | A beam synchronization signal must be provided to the DAQ system that provides beam bunch information. | 6.03.08.01 |

# Data Acquisition and Computing Systems (DET-COMP) to Collider Global Systems (COL-GBL)

I-DET-COMP-CONTROLS.001	Detector Feedback Interface

System 1:	Slow Controls Integration (6.03.08.02)

System 2:	Collider Global Systems (6.05.02)

Type:	DATA

Description:	An interface should be provided that allows the accelerator control system to read the condition and state of the detector systems.

Provider:	Slow Controls Integration (6.03.08.02)

Details:	See details in the Data Acquisition and Computing Systems Interface Control Document.

Requirements:	F-DET-COMP.6

References:

I-DET-COMP-DAQ.004	Accelerator Data Interface

System 1:	Online Data Systems (6.03.08.01)

System 2:	Collider Global Systems (6.05.02)

Type:	DATA

Description:	An interface to the accelerator beam databases must be provided that allows the DAQ to read data, and on a limited basis, write data.

Provider:	Online Data Systems (6.03.08.01)

Details:	See details in the Data Acquisition and Computing Systems Interface Control Document.

Requirements:	F-DET-COMP.6

References:

I-DET-COMP-DAQ.005	Accelerator Timing Interface

System 1:	Online Data Systems (6.03.08.01)

System 2:	Collider Global Systems (6.05.02)

Type:	DATA

Description:	A beam synchronization signal must be provided to the DAQ system that provides beam bunch information.

Provider:	Online Data Systems (6.03.08.01)

Details:	See details in the Data Acquisition and Computing Systems Interface Control Document.

Requirements:	F-DET-COMP.6

References:

# Applicable Documents

List applicable documents considered to be part of this document. Applicable documents shall be obtained from the EIC SharePoint Document Center.

# Detailed change Log

| Revision | Date | Change |
| --- | --- | --- |

# References

- doi:10.17226/25171., National Academies of Sciences, Engineering, and Medicine, "An Assessment of U.S.-Based Electron-Ion Collider Science", The National Academies Press, Washington, DC, 2018.

- EIC-ORG-PLN-010, Electron-Ion Collider L1 Requirements (Global Requirements Document)

- BNL-221006-2021-FORE, EIC Conceptual Design Report

- arXiv:2103.05419, Science Requirements and Detector Concepts for the Electron-Ion Collider: EIC Yellow Report

- EIC-SEG-PLN-016, Requirements Management Plan

- EIC-SEG-PLN-022, Systems Engineering Management Plan

- EIC-SEG-PLN-020, EIC Interface Management Plan