Electron-Ion Collider, Requirements, Specifications, and Interfaces

Interface Control Document: [###]

Ancillary Detectors (6.03.01.10) to Detector System (6.03.01)

Prepared by:

{{Sig_es_:signer1:signature}}	Date: {{Dte_es_:signer1:date}}

Name, Role

Electron-Ion Collider at Institution

Reviewed by:

{{Sig_es_:signer2:signature}}	Date: {{Dte_es_:signer2:date}}

Name, Role

Electron-Ion Collider at Institution

{{Sig_es_:signer3:signature}}	Date: {{Dte_es_:signer3:date}}

Name, Role

Electron-Ion Collider at Institution

{{Sig_es_:signer4:signature}}	Date: {{Dte_es_:signer4:date}}

Name, Role

Electron-Ion Collider at Institution

Approved by:

{{Sig_es_:signer5:signature}}	Date: {{Dte_es_:signer5:date}}

Name, Role

Electron-Ion Collider at Institution

REVISION HISTORY

| Revision # | Effective Date | Additional Reviewers | Summary of Change |
| --- | --- | --- | --- |
| 00 |  |  | Initial release. |

TABLE OF CONTENTS

1.	Purpose and Scope	5

2.	Introduction	5

2.1	Ancillary Detectors	5

2.2	Overview of Interface Relationships	5

2.3	Interface Responsibilities	5

2.4	Safety and Environmental Considerations	5

2.5	Design Considerations	6

2.6	Organization	6

3.	Interface summary	6

4.	Ancillary Detectors (DET-DS-ANC) to Ancillary Detectors (DET-DS-ANC)	18

5.	Ancillary Detectors (DET-DS-ANC) to Data Acquisition and Computing Systems (DET-DS-COMP)	19

6.	Ancillary Detectors (DET-DS-ANC) to Electronic Systems (DET-DS-ELEC)	24

7.	Ancillary Detectors (DET-DS-ANC) to Hadronic Calorimetry Systems (DET-DS-HCAL)	28

8.	Ancillary Detectors (DET-DS-ANC) to Detector Infrastructure (DET-DS-INF)	28

9.	Applicable Documents	34

10.	Detailed change Log	34

11.	References	34

LIST OF ACRONYMS

| B0 | B0 is the name of the forward magnet just past the Hadron Endcap |  |
| --- | --- | --- |
| CDR | Conceptual Design Report |  |
| COTS | Commercial Off the Shelf |  |
| DAQ | Data Acquisition |  |
| DET | EIC Detector |  |
| DIRC | Detection of Internally Reflected Cherenkov Light |  |
| dRICH | Dual radiator ring imaging Cherenkov |  |
| DVCS | Deeply Virtual Compton Scattering |  |
| EIC | Electron-Ion Collider |  |
| EMCAL | Electro-magnetic Calorimeter |  |
| ESR | Electron Storage Ring |  |
| FCC | Federal Communications Commission |  |
| FDR | Final Design Review |  |
| GEM | Gas Electron Multiplier |  |
| HCAL | Hadronic Calorimeter |  |
| HSR | Hadron Storage Ring |  |
| HVAC | Heating/Ventilation/Air Conditioning |  |
| IP | Interaction Point |  |
| IR | Interaction Region |  |
| MPGD | Micro Pattern Gaseous Detectors |  |
| NAS | National Academy of Sciences |  |
| NEC | National Electric Code |  |
| NEMA | National Electrical Manufacturers Association |  |
| NFPA | National Fire Protection Association |  |
| NRTL | Nationally Recognized Testing Laboratory |  |
| PID | Particle Identification |  |
| PRR | Production Readiness Review |  |
| PWO | PbWO4 |  |
| RCS | Rapid Cycling Synchrotron |  |
| TBD | To Be Determined |  |
| UL | Underwriter Laboratories |  |
| WBS | Work Break Down Structure |  |

# Interface Control Document:

Ancillary Detectors (6.03.01.10) to Detector System (6.03.01)

# Purpose and Scope

This document records the agreed upon interfaces between the Ancillary Detectors and other sub-systems within the Detector System. These interfaces are based on the system requirements that are necessary to integrate these sub-systems as part of the Electron Ion Collider Project.

# Introduction

These interfaces describe the relationships between the Ancillary Detectors and the Detector System including (examples by inclusion) located in (areas by inclusion).

## Ancillary Detectors

Provide an introductory description of your sub-system and an overview of the things that it receives, provides, supports, constrains, or otherwise requires as part of its function --- specifically if these things are manifested as interfaces.

Ancillary Detectors (DET-DS-ANC): Description of the TYPES of interfaces that exist between the ICD reference system and this sub-system, and a BRIEF overview of the roles they play.

Data Acquisition and Computing Systems (DET-DS-COMP): Description of the TYPES of interfaces that exist between the ICD reference system and this sub-system, and a BRIEF overview of the roles they play.

Electronic Systems (DET-DS-ELEC): Description of the TYPES of interfaces that exist between the ICD reference system and this sub-system, and a BRIEF overview of the roles they play.

Hadronic Calorimetry Systems (DET-DS-HCAL): Description of the TYPES of interfaces that exist between the ICD reference system and this sub-system, and a BRIEF overview of the roles they play.

Detector Infrastructure (DET-DS-INF): Description of the TYPES of interfaces that exist between the ICD reference system and this sub-system, and a BRIEF overview of the roles they play.

## Overview of Interface Relationships

The following are the sub-systems within the Detector System that have interfaces connected to the Ancillary Detectors.

## Interface Responsibilities

The various ownerships of responsibilities may be listed here. However, this section might also refer to the Provider section of the interface detail if all pertinent responsibilities are listed there. If this is the case, simply enter: “See the Provider section within the interface details for information about the responsibilities associated with individual interfaces.”

## Safety and Environmental Considerations

General safety and environmental considerations are identified here. Any considerations that are specific to the individual sub-system, will be addressed in external safety documents and are referenced here.

### Considerations for all systems

Add general safety and environmental considerations here.

### Special considerations and references

Add references here as necessary.

## Design Considerations

Notes here about general design considerations.

## Organization

The interfaces in this document are grouped by sub-system as identified in the sub-system identifier. All interfaces between the Ancillary Detectors and a related sub-system will be included in a single section. Entries within that section will be sorted by the interface identifier or WBS number, as appropriate.

# Interface summary

This tables in this section provide a list of all interfaces that are defined within this document. The interfaces are grouped by sub-system/WBS. Each table has the following fields:

Section: the section number in this document where the interface is addressed.

WBS (1): the WBS number within the ICD reference system, unless the reference system is the owner.

WBS (2): the WBS number for the connected system.

ID: the interface identifier as defined in the Interface Requirement Document (IRD).

Name: the name of the interface as defined in the IRD.

Description: the description of the interface as defined in the IRD.

Owner: identifies the WBS number of the entity that is responsible for ensuring the interface is provided and that it meets validation/verification requirements.

Ancillary Detectors to Ancillary Detectors

| Section | WBS1 | WBS2 | ID | Name | Description | Owner |
| --- | --- | --- | --- | --- | --- | --- |
| 4 |  | 6.03.01.10 |  |  | Ancillary Detectors |  |
| 4.1 | 6.03.01.10 | 6.03.01.10 | I-DET-DS-ANC-LOWQ2.004 | Lumi-Dipole Interference | The LowQ2 detector should be position such that it receives minimal magnetic interference from the Lumi-Dipole. | 6.03.01.10 |

Ancillary Detectors to Data Acquisition and Computing Systems

| Section | WBS1 | WBS2 | ID | Name | Description | Owner |
| --- | --- | --- | --- | --- | --- | --- |
| 5 |  | 6.03.01.08 |  |  | Data Acquisition and Computing Systems |  |
| 5.1 | 6.03.01.10 | 6.03.01.08 | I-DET-DS-COMP-ONLINE.002 | Data Transfer and Control Interface | A fiber connection will be provided from the DAQ system to the B0 detector's readout board to perform configuration, control, and data acquisition. | 6.03.01.08.02 |
| 5.2 | 6.03.01.10 | 6.03.01.08 | I-DET-DS-COMP-ONLINE.003 | Slow Controls | A network connection will be provided from the DAQ system to the B0 detector's slow controls interface. | 6.03.01.08.02 |
| 5.3 | 6.03.01.10 | 6.03.01.08 | I-DET-DS-COMP-ONLINE.004 | Timing Interface | A fiber connection will be provided from the DAQ system to the B0 detector's readout board for timing synchronization. | 6.03.01.08.02 |
| 5.4 | 6.03.01.10 | 6.03.01.08 | I-DET-DS-COMP-ONLINE.005 | Data Transfer and Control Interface | A fiber connection will be provided from the DAQ system to the Low Q2 detector's readout board to perform configuration, control, and data acquisition. | 6.03.01.08.02 |
| 5.5 | 6.03.01.10 | 6.03.01.08 | I-DET-DS-COMP-ONLINE.006 | Slow Controls | A network connection will be provided from the DAQ system to the Low Q2 detector's slow controls interface. | 6.03.01.08.02 |
| 5.6 | 6.03.01.10 | 6.03.01.08 | I-DET-DS-COMP-ONLINE.007 | Timing Interface | A fiber connection will be provided from the DAQ system to the Low Q2 detector's readout board for timing synchronization. | 6.03.01.08.02 |
| 5.7 | 6.03.01.10 | 6.03.01.08 | I-DET-DS-COMP-ONLINE.008 | Data Transfer and Control Interface | Fiber connection from the DAQ system to the detector's RDO to perform configuration, control, and data acquisition. | 6.03.01.08.02 |
| 5.8 | 6.03.01.10 | 6.03.01.08 | I-DET-DS-COMP-ONLINE.009 | Slow Controls | Network connection from the DAQ system to the detector's slow controls interface. | 6.03.01.08.02 |
| 5.9 | 6.03.01.10 | 6.03.01.08 | I-DET-DS-COMP-ONLINE.010 | Timing Interface | Fiber connection from the DAQ system to the detector's RDO used for timing synchronization. | 6.03.01.08.02 |
| 5.10 | 6.03.01.10 | 6.03.01.08 | I-DET-DS-COMP-ONLINE.011 | Data Transfer and Control Interface | A fiber connection will be provided from the DAQ system to the Off-Momentum detector's readout board to perform configuration, control, and data acquisition. | 6.03.01.08.02 |
| 5.11 | 6.03.01.10 | 6.03.01.08 | I-DET-DS-COMP-ONLINE.012 | Slow Controls | A network connection will be provided from the DAQ system to the Off-Momentum slow controls interface. | 6.03.01.08.02 |
| 5.12 | 6.03.01.10 | 6.03.01.08 | I-DET-DS-COMP-ONLINE.013 | Timing Interface | A fiber connection will be provided from the DAQ system to the Off-Momentum detector's readout board for timing synchronization. | 6.03.01.08.02 |
| 5.13 | 6.03.01.10 | 6.03.01.08 | I-DET-DS-COMP-ONLINE.014 | Data Transfer and Control Interface | A fiber connection will be provided from the DAQ system to the Roman Pot's readout board to perform configuration, control, and data acquisition. | 6.03.01.08.02 |
| 5.14 | 6.03.01.10 | 6.03.01.08 | I-DET-DS-COMP-ONLINE.015 | Slow Controls | Network connection from the DAQ system to the detector's slow controls interface. | 6.03.01.08.02 |
| 5.15 | 6.03.01.10 | 6.03.01.08 | I-DET-DS-COMP-ONLINE.016 | Timing Interface | A fiber connection will be provided from the DAQ system to the Roman Pot's readout board for timing synchronization. | 6.03.01.08.02 |
| 5.16 | 6.03.01.10 | 6.03.01.08 | I-DET-DS-COMP-ONLINE.017 | Data Transfer and Control Interface | A fiber connection will be provided from the DAQ system to the Zero-Degree Calorimeter's readout board to perform configuration, control, and data acquisition. | 6.03.01.08.02 |
| 5.17 | 6.03.01.10 | 6.03.01.08 | I-DET-DS-COMP-ONLINE.018 | Slow Controls | A network connection will be provided from the DAQ system to the Zero-Degree Calorimeter's slow controls interface. | 6.03.01.08.02 |
| 5.18 | 6.03.01.10 | 6.03.01.08 | I-DET-DS-COMP-ONLINE.019 | Timing Interface | A fiber connection will be provided from the DAQ system to the Zero-Degree Calorimeter's readout board for timing synchronization. | 6.03.01.08.02 |

Ancillary Detectors to Electronic Systems

| Section | WBS1 | WBS2 | ID | Name | Description | Owner |
| --- | --- | --- | --- | --- | --- | --- |
| 6 |  | 6.03.01.07 |  |  | Electronic Systems |  |
| 6.1 | 6.03.01.10 | 6.03.01.07 | I-DET-DS-ELEC.001 | Bias Voltage | The detector will receive bias voltage DC power provided from the electronics racks to support electronics. | 6.03.01.07 |
| 6.2 | 6.03.01.10 | 6.03.01.07 | I-DET-DS-ELEC.002 | High Voltage | The detector will receive high voltage DC power provided by the Detector Electronics group to support silicon sensors and calorimeter. | 6.03.01.07 |
| 6.3 | 6.03.01.10 | 6.03.01.07 | I-DET-DS-ELEC.003 | Low Voltage | The detector will receive low voltage DC power provided by the Detector Electronics group. | 6.03.01.07 |
| 6.4 | 6.03.01.10 | 6.03.01.07 | I-DET-DS-ELEC.004 | Bias Voltage | The detector will receive bias voltage DC power provided by the Detector Electronics group to support electronics. | 6.03.01.07 |
| 6.5 | 6.03.01.10 | 6.03.01.07 | I-DET-DS-ELEC.005 | High Voltage | The detector will receive high voltage DC power provided by the Detector Electronics group to support silicon sensors and calorimeter. | 6.03.01.07 |
| 6.6 | 6.03.01.10 | 6.03.01.07 | I-DET-DS-ELEC.006 | Low Voltage | The detector will receive low voltage DC power provided by the Detector Electronics group. | 6.03.01.07 |
| 6.7 | 6.03.01.10 | 6.03.01.07 | I-DET-DS-ELEC.007 | Bias Voltage | The detector will receive bias voltage DC power provided by the Detector Electronics group to support electronics. | 6.03.01.07 |
| 6.8 | 6.03.01.10 | 6.03.01.07 | I-DET-DS-ELEC.008 | High Voltage | The detector will receive high voltage DC power provided by the Detector Electronics group to support silicon sensors and calorimeter. | 6.03.01.07 |
| 6.9 | 6.03.01.10 | 6.03.01.07 | I-DET-DS-ELEC.009 | Low Voltage | The detector will receive low voltage DC power provided by the Detector Electronics group. | 6.03.01.07 |
| 6.10 | 6.03.01.10 | 6.03.01.07 | I-DET-DS-ELEC.010 | Bias Voltage | The detector will receive bias voltage DC power provided by the Detector Electronics group to support electronics. | 6.03.01.07 |
| 6.11 | 6.03.01.10 | 6.03.01.07 | I-DET-DS-ELEC.011 | High Voltage | The detector will receive high voltage DC power provided by the Detector Electronics group to support silicon sensors and calorimeter. | 6.03.01.07 |
| 6.12 | 6.03.01.10 | 6.03.01.07 | I-DET-DS-ELEC.012 | Low Voltage | The detector will receive low voltage DC power provided by the Detector Electronics group. | 6.03.01.07 |
| 6.13 | 6.03.01.10 | 6.03.01.07 | I-DET-DS-ELEC.013 | Bias Voltage | The detector will receive bias voltage DC power provided by the Detector Electronics group to support electronics. | 6.03.01.07 |
| 6.14 | 6.03.01.10 | 6.03.01.07 | I-DET-DS-ELEC.014 | High Voltage | The detector will receive high voltage DC power provided by the Detector Electronics group to support silicon sensors and calorimeter. | 6.03.01.07 |
| 6.15 | 6.03.01.10 | 6.03.01.07 | I-DET-DS-ELEC.015 | Low Voltage | The detector will receive low voltage DC power provided by the Detector Electronics group. | 6.03.01.07 |
| 6.16 | 6.03.01.10 | 6.03.01.07 | I-DET-DS-ELEC.016 | Bias Voltage | The detector will receive bias voltage DC power provided by the Detector Electronics group to support electronics. | 6.03.01.07 |
| 6.17 | 6.03.01.10 | 6.03.01.07 | I-DET-DS-ELEC.017 | High Voltage | The detector will receive high voltage DC power provided by the Detector Electronics group to support silicon sensors and calorimeter. | 6.03.01.07 |
| 6.18 | 6.03.01.10 | 6.03.01.07 | I-DET-DS-ELEC.018 | Low Voltage | The detector will receive low voltage DC power provided by the Detector Electronics group. | 6.03.01.07 |

Ancillary Detectors to Hadronic Calorimetry Systems

| Section | WBS1 | WBS2 | ID | Name | Description | Owner |
| --- | --- | --- | --- | --- | --- | --- |
| 7 |  | 6.03.01.05 |  |  | Hadronic Calorimetry Systems |  |
| 7.1 | 6.03.01.10 | 6.03.01.05 | I-DET-DS-INF-INT.005 | Space Constraint | The position of the forward HCAL is bounded by the B0 magnet in the forward direction. Changes to the size or position of either must be coordinated with the other. | 6.03.01.09.04 |

Ancillary Detectors to Detector Infrastructure

| Section | WBS1 | WBS2 | ID | Name | Description | Owner |
| --- | --- | --- | --- | --- | --- | --- |
| 8 |  | 6.03.01.09 |  |  | Detector Infrastructure |  |
| 8.1 | 6.03.01.10 | 6.03.01.09 | I-DET-DS-INF-BCK.004 | LowQ2 Support | The LowQ2 detector is supported by a freestanding platform that is adjacent to the outgoing electron beam. | 6.03.01.09.02 |
| 8.2 | 6.03.01.10 | 6.03.01.09 | I-DET-DS-INF-BCK.005 | Lumi Support System | The luminosity detector will be supported by an independent support system. | 6.03.01.09.02 |
| 8.3 | 6.03.01.10 | 6.03.01.09 | I-DET-DS-INF-FWD.001 | Detector Support | The detector will be supported by an integrated support stand that is fully integrated with the vacuum system and allows the position of the detector to be adjusted. | 6.03.01.09.03 |
| 8.4 | 6.03.01.10 | 6.03.01.09 | I-DET-DS-INF-FWD.002 | Detector Support | The detector will be supported by an integrated support stand that is fully integrated with the vacuum system and allows the position of the detector to be adjusted. | 6.03.01.09.03 |
| 8.5 | 6.03.01.10 | 6.03.01.09 | I-DET-DS-INF-FWD.006 | B0 Support System | The B0 detector will be supported in the interior of the B0 magnet and will be dependent on the B0 magnet design, and must be adequate to support the weight of the crystals and the tracking detectors. | 6.03.01.09.03 |
| 8.6 | 6.03.01.10 | 6.03.01.09 | I-DET-DS-INF-FWD.007 | ZDC Support System | The detector will be supported on a free standing structure. | 6.03.01.09.03 |
| 8.7 | 6.03.01.10 | 6.03.01.09 | I-DET-DS-INF-INT.004 | Space Constraint | B0 detector must fit entirely within the bore of the B0 magnet. | 6.03.01.09.04 |
| 8.8 | 6.03.01.10 | 6.03.01.09 | I-DET-DS-INF-INT.005 | Space Constraint | The position of the forward HCAL is bounded by the B0 magnet in the forward direction. Changes to the size or position of either must be coordinated with the other. | 6.03.01.09.04 |
| 8.9 | 6.03.01.10 | 6.03.01.09 | I-DET-DS-INF-INT.006 | Space Constraint | The position of and accessibility of the B0 detector in the backward direction is limited by the cryostat containing the final focusing magnets. | 6.03.01.09.04 |
| 8.10 | 6.03.01.10 | 6.03.01.09 | I-DET-DS-INF-INT.007 | Space Constraint | Access to the B0 detector is constrained by the vacuum valve in front of the B0 magnet. A minimum of 15 centimeters of free space will be required on the IP-side of the B0-dipole for the installation of the B0 detectors. | 6.03.01.09.04 |
| 8.11 | 6.03.01.10 | 6.03.01.09 | I-DET-DS-INF-INT.008 | Space Constraint | The electron and hadron beam pipes must be contained within the B0 detector. | 6.03.01.09.04 |
| 8.12 | 6.03.01.10 | 6.03.01.09 | I-DET-DS-INF-INT.009 | Position Constraint | The LowQ2 detector must be upstream in the electron direction from the Q3ER magnet. | 6.03.01.09.04 |
| 8.13 | 6.03.01.10 | 6.03.01.09 | I-DET-DS-INF-INT.010 | Position Constraint | The LowQ2 detector must be downstream in the electron direction from the B2ER magnet. | 6.03.01.09.04 |
| 8.14 | 6.03.01.10 | 6.03.01.09 | I-DET-DS-INF-INT.011 | Position Constraint | The LOWQ2 detector must be at the same level as the electron beamline. | 6.03.01.09.04 |
| 8.15 | 6.03.01.10 | 6.03.01.09 | I-DET-DS-INF-INT.040 | Process Cooling | A cooling system will be required to remove heat from the calorimeter and electronics readouts, to maintain an acceptable temperature. A chiller should be installed near the sub-system and a space should be allocated for this chiller. | 6.03.01.09.04 |
| 8.16 | 6.03.01.10 | 6.03.01.09 | I-DET-DS-INF-INT.041 | Process Cooling | Either a liquid or gas cooling system will be required to remove heat from the calorimeter, tracking and readout electronics, to maintain them at room temperature. | 6.03.01.09.04 |
| 8.17 | 6.03.01.10 | 6.03.01.09 | I-DET-DS-INF-INT.042 | Dipole Power | The two dipoles in front of the detector will require an infrastructure power source. | 6.03.01.09.04 |
| 8.18 | 6.03.01.10 | 6.03.01.09 | I-DET-DS-INF-INT.043 | Dipole Vacuum | The beamline between the two luminosity detector dipoles must be under vacuum. | 6.03.01.09.04 |
| 8.19 | 6.03.01.10 | 6.03.01.09 | I-DET-DS-INF-INT.044 | Process Cooling | A cooling system will be required to remove heat from the tracking and readout electronics to prevent heat buildup. | 6.03.01.09.04 |
| 8.20 | 6.03.01.10 | 6.03.01.09 | I-DET-DS-INF-INT.045 | Process Cooling | A cooling system will be required to remove heat from the tracking and readout electronics to prevent heat buildup. | 6.03.01.09.04 |
| 8.21 | 6.03.01.10 | 6.03.01.09 | I-DET-DS-INF-INT.046 | Process Cooling | A cooling system will be required to remove heat from the calorimeter and readout electronics to prevent heat buildup. | 6.03.01.09.04 |

# Ancillary Detectors (DET-DS-ANC) to Ancillary Detectors (DET-DS-ANC)

I-DET-DS-ANC-LOWQ2.004	Lumi-Dipole Interference

System 1:	Low Q2 Detectors (6.03.01.10)

System 2:	Luminosity Detectors (6.03.01.10)

Type:	ENV

Description:	The LowQ2 detector should be position such that it receives minimal magnetic interference from the Lumi-Dipole.

Provider:	Low Q2 Detectors (6.03.01.10)

Details:	See details in the Ancillary Detectors Interface Control Document.

Requirements:	P-DET-DS-ANC-LOWQ2.15

References:

# Ancillary Detectors (DET-DS-ANC) to Data Acquisition and Computing Systems (DET-DS-COMP)

I-DET-DS-COMP-ONLINE.002	Data Transfer and Control Interface

System 1:	Online DAQ and Computing Systems (6.03.01.08.02)

System 2:	B-Zero Detectors (6.03.01.10)

Type:	DATA

Description:	A fiber connection will be provided from the DAQ system to the B0 detector's readout board to perform configuration, control, and data acquisition.

Provider:	Online DAQ and Computing Systems (6.03.01.08.02)

Details:	See details in the Data Acquisition and Computing Systems Interface Control Document.

Requirements:	F-DET-DS-COMP.6

References:

I-DET-DS-COMP-ONLINE.003	Slow Controls

System 1:	Online DAQ and Computing Systems (6.03.01.08.02)

System 2:	B-Zero Detectors (6.03.01.10)

Type:	CONTROL

Description:	A network connection will be provided from the DAQ system to the B0 detector's slow controls interface.

Provider:	Online DAQ and Computing Systems (6.03.01.08.02)

Details:	See details in the Data Acquisition and Computing Systems Interface Control Document.

Requirements:	F-DET-DS-COMP-SC.1

References:

I-DET-DS-COMP-ONLINE.004	Timing Interface

System 1:	Online DAQ and Computing Systems (6.03.01.08.02)

System 2:	B-Zero Detectors (6.03.01.10)

Type:	DATA

Description:	A fiber connection will be provided from the DAQ system to the B0 detector's readout board for timing synchronization.

Provider:	Online DAQ and Computing Systems (6.03.01.08.02)

Details:	See details in the Data Acquisition and Computing Systems Interface Control Document.

Requirements:	F-DET-DS-COMP.11

References:

I-DET-DS-COMP-ONLINE.005	Data Transfer and Control Interface

System 1:	Online DAQ and Computing Systems (6.03.01.08.02)

System 2:	Low Q2 Detectors (6.03.01.10)

Type:	DATA

Description:	A fiber connection will be provided from the DAQ system to the Low Q2 detector's readout board to perform configuration, control, and data acquisition.

Provider:	Online DAQ and Computing Systems (6.03.01.08.02)

Details:	See details in the Data Acquisition and Computing Systems Interface Control Document.

Requirements:	F-DET-DS-COMP.6

References:

I-DET-DS-COMP-ONLINE.006	Slow Controls

System 1:	Online DAQ and Computing Systems (6.03.01.08.02)

System 2:	Low Q2 Detectors (6.03.01.10)

Type:	CONTROL

Description:	A network connection will be provided from the DAQ system to the Low Q2 detector's slow controls interface.

Provider:	Online DAQ and Computing Systems (6.03.01.08.02)

Details:	See details in the Data Acquisition and Computing Systems Interface Control Document.

Requirements:	F-DET-DS-COMP-SC.1

References:

I-DET-DS-COMP-ONLINE.007	Timing Interface

System 1:	Online DAQ and Computing Systems (6.03.01.08.02)

System 2:	Low Q2 Detectors (6.03.01.10)

Type:	DATA

Description:	A fiber connection will be provided from the DAQ system to the Low Q2 detector's readout board for timing synchronization.

Provider:	Online DAQ and Computing Systems (6.03.01.08.02)

Details:	See details in the Data Acquisition and Computing Systems Interface Control Document.

Requirements:	F-DET-DS-COMP.11

References:

I-DET-DS-COMP-ONLINE.008	Data Transfer and Control Interface

System 1:	Online DAQ and Computing Systems (6.03.01.08.02)

System 2:	Luminosity Detectors (6.03.01.10)

Type:	DATA

Description:	Fiber connection from the DAQ system to the detector's RDO to perform configuration, control, and data acquisition.

Provider:	Online DAQ and Computing Systems (6.03.01.08.02)

Details:	See details in the Data Acquisition and Computing Systems Interface Control Document.

Requirements:	F-DET-DS-COMP.6

References:

I-DET-DS-COMP-ONLINE.009	Slow Controls

System 1:	Online DAQ and Computing Systems (6.03.01.08.02)

System 2:	Luminosity Detectors (6.03.01.10)

Type:	CONTROL

Description:	Network connection from the DAQ system to the detector's slow controls interface.

Provider:	Online DAQ and Computing Systems (6.03.01.08.02)

Details:	See details in the Data Acquisition and Computing Systems Interface Control Document.

Requirements:	F-DET-DS-COMP-SC.1

References:

I-DET-DS-COMP-ONLINE.010	Timing Interface

System 1:	Online DAQ and Computing Systems (6.03.01.08.02)

System 2:	Luminosity Detectors (6.03.01.10)

Type:	DATA

Description:	Fiber connection from the DAQ system to the detector's RDO used for timing synchronization.

Provider:	Online DAQ and Computing Systems (6.03.01.08.02)

Details:	See details in the Data Acquisition and Computing Systems Interface Control Document.

Requirements:	F-DET-DS-COMP.11

References:

I-DET-DS-COMP-ONLINE.011	Data Transfer and Control Interface

System 1:	Online DAQ and Computing Systems (6.03.01.08.02)

System 2:	Off-Momentum Detectors (6.03.01.10)

Type:	DATA

Description:	A fiber connection will be provided from the DAQ system to the Off-Momentum detector's readout board to perform configuration, control, and data acquisition.

Provider:	Online DAQ and Computing Systems (6.03.01.08.02)

Details:	See details in the Data Acquisition and Computing Systems Interface Control Document.

Requirements:	F-DET-DS-COMP.6

References:

I-DET-DS-COMP-ONLINE.012	Slow Controls

System 1:	Online DAQ and Computing Systems (6.03.01.08.02)

System 2:	Off-Momentum Detectors (6.03.01.10)

Type:	CONTROL

Description:	A network connection will be provided from the DAQ system to the Off-Momentum slow controls interface.

Provider:	Online DAQ and Computing Systems (6.03.01.08.02)

Details:	See details in the Data Acquisition and Computing Systems Interface Control Document.

Requirements:	F-DET-DS-COMP-SC.1

References:

I-DET-DS-COMP-ONLINE.013	Timing Interface

System 1:	Online DAQ and Computing Systems (6.03.01.08.02)

System 2:	Off-Momentum Detectors (6.03.01.10)

Type:	DATA

Description:	A fiber connection will be provided from the DAQ system to the Off-Momentum detector's readout board for timing synchronization.

Provider:	Online DAQ and Computing Systems (6.03.01.08.02)

Details:	See details in the Data Acquisition and Computing Systems Interface Control Document.

Requirements:	F-DET-DS-COMP.11

References:

I-DET-DS-COMP-ONLINE.014	Data Transfer and Control Interface

System 1:	Online DAQ and Computing Systems (6.03.01.08.02)

System 2:	Roman Pots (6.03.01.10)

Type:	DATA

Description:	A fiber connection will be provided from the DAQ system to the Roman Pot's readout board to perform configuration, control, and data acquisition.

Provider:	Online DAQ and Computing Systems (6.03.01.08.02)

Details:	See details in the Data Acquisition and Computing Systems Interface Control Document.

Requirements:	F-DET-DS-COMP.6

References:

I-DET-DS-COMP-ONLINE.015	Slow Controls

System 1:	Online DAQ and Computing Systems (6.03.01.08.02)

System 2:	Roman Pots (6.03.01.10)

Type:	CONTROL

Description:	Network connection from the DAQ system to the detector's slow controls interface.

Provider:	Online DAQ and Computing Systems (6.03.01.08.02)

Details:	See details in the Data Acquisition and Computing Systems Interface Control Document.

Requirements:	F-DET-DS-COMP-SC.1

References:

I-DET-DS-COMP-ONLINE.016	Timing Interface

System 1:	Online DAQ and Computing Systems (6.03.01.08.02)

System 2:	Roman Pots (6.03.01.10)

Type:	DATA

Description:	A fiber connection will be provided from the DAQ system to the Roman Pot's readout board for timing synchronization.

Provider:	Online DAQ and Computing Systems (6.03.01.08.02)

Details:	See details in the Data Acquisition and Computing Systems Interface Control Document.

Requirements:	F-DET-DS-COMP.11

References:

I-DET-DS-COMP-ONLINE.017	Data Transfer and Control Interface

System 1:	Online DAQ and Computing Systems (6.03.01.08.02)

System 2:	Zero Degree Calorimeter (6.03.01.10)

Type:	DATA

Description:	A fiber connection will be provided from the DAQ system to the Zero-Degree Calorimeter's readout board to perform configuration, control, and data acquisition.

Provider:	Online DAQ and Computing Systems (6.03.01.08.02)

Details:	See details in the Data Acquisition and Computing Systems Interface Control Document.

Requirements:	F-DET-DS-COMP.6

References:

I-DET-DS-COMP-ONLINE.018	Slow Controls

System 1:	Online DAQ and Computing Systems (6.03.01.08.02)

System 2:	Zero Degree Calorimeter (6.03.01.10)

Type:	CONTROL

Description:	A network connection will be provided from the DAQ system to the Zero-Degree Calorimeter's slow controls interface.

Provider:	Online DAQ and Computing Systems (6.03.01.08.02)

Details:	See details in the Data Acquisition and Computing Systems Interface Control Document.

Requirements:	F-DET-DS-COMP-SC.1

References:

I-DET-DS-COMP-ONLINE.019	Timing Interface

System 1:	Online DAQ and Computing Systems (6.03.01.08.02)

System 2:	Zero Degree Calorimeter (6.03.01.10)

Type:	DATA

Description:	A fiber connection will be provided from the DAQ system to the Zero-Degree Calorimeter's readout board for timing synchronization.

Provider:	Online DAQ and Computing Systems (6.03.01.08.02)

Details:	See details in the Data Acquisition and Computing Systems Interface Control Document.

Requirements:	F-DET-DS-COMP.11

References:

# Ancillary Detectors (DET-DS-ANC) to Electronic Systems (DET-DS-ELEC)

I-DET-DS-ELEC.001	Bias Voltage

System 1:	Electronic Systems (6.03.01.07)

System 2:	B-Zero Detectors (6.03.01.10)

Type:	ELEC

Description:	The detector will receive bias voltage DC power provided from the electronics racks to support electronics.

Provider:	Electronic Systems (6.03.01.07)

Details:	See details in the Electronic Systems Interface Control Document.

Requirements:	P-DET-DS-ANC.4

References:

I-DET-DS-ELEC.002	High Voltage

System 1:	Electronic Systems (6.03.01.07)

System 2:	B-Zero Detectors (6.03.01.10)

Type:	ELEC

Description:	The detector will receive high voltage DC power provided by the Detector Electronics group to support silicon sensors and calorimeter.

Provider:	Electronic Systems (6.03.01.07)

Details:	See details in the Electronic Systems Interface Control Document.

Requirements:	P-DET-DS-ANC.4

References:

I-DET-DS-ELEC.003	Low Voltage

System 1:	Electronic Systems (6.03.01.07)

System 2:	B-Zero Detectors (6.03.01.10)

Type:	ELEC

Description:	The detector will receive low voltage DC power provided by the Detector Electronics group.

Provider:	Electronic Systems (6.03.01.07)

Details:	See details in the Electronic Systems Interface Control Document.

Requirements:	P-DET-DS-ANC.4

References:

I-DET-DS-ELEC.004	Bias Voltage

System 1:	Electronic Systems (6.03.01.07)

System 2:	Low Q2 Detectors (6.03.01.10)

Type:	ELEC

Description:	The detector will receive bias voltage DC power provided by the Detector Electronics group to support electronics.

Provider:	Electronic Systems (6.03.01.07)

Details:	See details in the Electronic Systems Interface Control Document.

Requirements:	P-DET-DS-ANC.4

References:

I-DET-DS-ELEC.005	High Voltage

System 1:	Electronic Systems (6.03.01.07)

System 2:	Low Q2 Detectors (6.03.01.10)

Type:	ELEC

Description:	The detector will receive high voltage DC power provided by the Detector Electronics group to support silicon sensors and calorimeter.

Provider:	Electronic Systems (6.03.01.07)

Details:	See details in the Electronic Systems Interface Control Document.

Requirements:	P-DET-DS-ANC.4

References:

I-DET-DS-ELEC.006	Low Voltage

System 1:	Electronic Systems (6.03.01.07)

System 2:	Low Q2 Detectors (6.03.01.10)

Type:	ELEC

Description:	The detector will receive low voltage DC power provided by the Detector Electronics group.

Provider:	Electronic Systems (6.03.01.07)

Details:	See details in the Electronic Systems Interface Control Document.

Requirements:	P-DET-DS-ANC.4

References:

I-DET-DS-ELEC.007	Bias Voltage

System 1:	Electronic Systems (6.03.01.07)

System 2:	Luminosity Detectors (6.03.01.10)

Type:	ELEC

Description:	The detector will receive bias voltage DC power provided by the Detector Electronics group to support electronics.

Provider:	Electronic Systems (6.03.01.07)

Details:	See details in the Electronic Systems Interface Control Document.

Requirements:	P-DET-DS-ANC.4

References:

I-DET-DS-ELEC.008	High Voltage

System 1:	Electronic Systems (6.03.01.07)

System 2:	Luminosity Detectors (6.03.01.10)

Type:	ELEC

Description:	The detector will receive high voltage DC power provided by the Detector Electronics group to support silicon sensors and calorimeter.

Provider:	Electronic Systems (6.03.01.07)

Details:	See details in the Electronic Systems Interface Control Document.

Requirements:	P-DET-DS-ANC.4

References:

I-DET-DS-ELEC.009	Low Voltage

System 1:	Electronic Systems (6.03.01.07)

System 2:	Luminosity Detectors (6.03.01.10)

Type:	ELEC

Description:	The detector will receive low voltage DC power provided by the Detector Electronics group.

Provider:	Electronic Systems (6.03.01.07)

Details:	See details in the Electronic Systems Interface Control Document.

Requirements:	P-DET-DS-ANC.4

References:

I-DET-DS-ELEC.010	Bias Voltage

System 1:	Electronic Systems (6.03.01.07)

System 2:	Off-Momentum Detectors (6.03.01.10)

Type:	ELEC

Description:	The detector will receive bias voltage DC power provided by the Detector Electronics group to support electronics.

Provider:	Electronic Systems (6.03.01.07)

Details:	See details in the Electronic Systems Interface Control Document.

Requirements:	P-DET-DS-ANC.4

References:

I-DET-DS-ELEC.011	High Voltage

System 1:	Electronic Systems (6.03.01.07)

System 2:	Off-Momentum Detectors (6.03.01.10)

Type:	ELEC

Description:	The detector will receive high voltage DC power provided by the Detector Electronics group to support silicon sensors and calorimeter.

Provider:	Electronic Systems (6.03.01.07)

Details:	See details in the Electronic Systems Interface Control Document.

Requirements:	P-DET-DS-ANC.4

References:

I-DET-DS-ELEC.012	Low Voltage

System 1:	Electronic Systems (6.03.01.07)

System 2:	Off-Momentum Detectors (6.03.01.10)

Type:	ELEC

Description:	The detector will receive low voltage DC power provided by the Detector Electronics group.

Provider:	Electronic Systems (6.03.01.07)

Details:	See details in the Electronic Systems Interface Control Document.

Requirements:	P-DET-DS-ANC.4

References:

I-DET-DS-ELEC.013	Bias Voltage

System 1:	Electronic Systems (6.03.01.07)

System 2:	Roman Pots (6.03.01.10)

Type:	ELEC

Description:	The detector will receive bias voltage DC power provided by the Detector Electronics group to support electronics.

Provider:	Electronic Systems (6.03.01.07)

Details:	See details in the Electronic Systems Interface Control Document.

Requirements:	P-DET-DS-ANC.4

References:

I-DET-DS-ELEC.014	High Voltage

System 1:	Electronic Systems (6.03.01.07)

System 2:	Roman Pots (6.03.01.10)

Type:	ELEC

Description:	The detector will receive high voltage DC power provided by the Detector Electronics group to support silicon sensors and calorimeter.

Provider:	Electronic Systems (6.03.01.07)

Details:	See details in the Electronic Systems Interface Control Document.

Requirements:	P-DET-DS-ANC.4

References:

I-DET-DS-ELEC.015	Low Voltage

System 1:	Electronic Systems (6.03.01.07)

System 2:	Roman Pots (6.03.01.10)

Type:	ELEC

Description:	The detector will receive low voltage DC power provided by the Detector Electronics group.

Provider:	Electronic Systems (6.03.01.07)

Details:	See details in the Electronic Systems Interface Control Document.

Requirements:	P-DET-DS-ANC.4

References:

I-DET-DS-ELEC.016	Bias Voltage

System 1:	Electronic Systems (6.03.01.07)

System 2:	Zero Degree Calorimeter (6.03.01.10)

Type:	ELEC

Description:	The detector will receive bias voltage DC power provided by the Detector Electronics group to support electronics.

Provider:	Electronic Systems (6.03.01.07)

Details:	See details in the Electronic Systems Interface Control Document.

Requirements:	P-DET-DS-ANC.4

References:

I-DET-DS-ELEC.017	High Voltage

System 1:	Electronic Systems (6.03.01.07)

System 2:	Zero Degree Calorimeter (6.03.01.10)

Type:	ELEC

Description:	The detector will receive high voltage DC power provided by the Detector Electronics group to support silicon sensors and calorimeter.

Provider:	Electronic Systems (6.03.01.07)

Details:	See details in the Electronic Systems Interface Control Document.

Requirements:	P-DET-DS-ANC.4

References:

I-DET-DS-ELEC.018	Low Voltage

System 1:	Electronic Systems (6.03.01.07)

System 2:	Zero Degree Calorimeter (6.03.01.10)

Type:	ELEC

Description:	The detector will receive low voltage DC power provided by the Detector Electronics group.

Provider:	Electronic Systems (6.03.01.07)

Details:	See details in the Electronic Systems Interface Control Document.

Requirements:	P-DET-DS-ANC.4

References:

# Ancillary Detectors (DET-DS-ANC) to Hadronic Calorimetry Systems (DET-DS-HCAL)

I-DET-DS-INF-INT.005	Space Constraint

System 1:	B-Zero Detectors (6.03.01.10)

System 2:	Forward HCal Systems (6.03.01.05)

Type:	SPACE

Description:	The position of the forward HCAL is bounded by the B0 magnet in the forward direction. Changes to the size or position of either must be coordinated with the other.

Provider:	Detector Infrastructure and Utilities Integration (6.03.01.09.04)

Details:	See details in the Detector Infrastructure Interface Control Document.

Requirements:	P-DET-DS-ANC.1, P-DET-DS-HCAL.1

References:

# Ancillary Detectors (DET-DS-ANC) to Detector Infrastructure (DET-DS-INF)

I-DET-DS-INF-BCK.004	LowQ2 Support

System 1:	Electron Endcap Structures (6.03.01.09.02)

System 2:	Low Q2 Detectors (6.03.01.10)

Type:	STRUCT

Description:	The LowQ2 detector is supported by a freestanding platform that is adjacent to the outgoing electron beam.

Provider:	Electron Endcap Structures (6.03.01.09.02)

Details:	See details in the Detector Infrastructure Interface Control Document.

Requirements:	P-DET-DS-ANC.1, P-DET-DS-ANC.2

References:

I-DET-DS-INF-BCK.005	Lumi Support System

System 1:	Electron Endcap Structures (6.03.01.09.02)

System 2:	Luminosity Detectors (6.03.01.10)

Type:	STRUCT

Description:	The luminosity detector will be supported by an independent support system.

Provider:	Electron Endcap Structures (6.03.01.09.02)

Details:	See details in the Detector Infrastructure Interface Control Document.

Requirements:	P-DET-DS-ANC.1, P-DET-DS-ANC.2

References:

I-DET-DS-INF-FWD.001	Detector Support

System 1:	Off-Momentum Detectors (6.03.01.10)

System 2:	IR Accelerator Systems (6.04.03)

Type:	STRUCT

Description:	The detector will be supported by an integrated support stand that is fully integrated with the vacuum system and allows the position of the detector to be adjusted.

Provider:	Hadron Endcap Structures (6.03.01.09.03)

Details:	See details in the Detector Infrastructure Interface Control Document.

Requirements:	P-DET-DS-ANC.1, P-DET-DS-ANC.2

References:

I-DET-DS-INF-FWD.002	Detector Support

System 1:	Roman Pots (6.03.01.10)

System 2:	IR Accelerator Systems (6.04.03)

Type:	STRUCT

Description:	The detector will be supported by an integrated support stand that is fully integrated with the vacuum system and allows the position of the detector to be adjusted.

Provider:	Hadron Endcap Structures (6.03.01.09.03)

Details:	See details in the Detector Infrastructure Interface Control Document.

Requirements:	P-DET-DS-ANC.1, P-DET-DS-ANC.2

References:

I-DET-DS-INF-FWD.006	B0 Support System

System 1:	B-Zero Detectors (6.03.01.10)

System 2:	IR Superconducting Magnets (6.04.02)

Type:	STRUCT

Description:	The B0 detector will be supported in the interior of the B0 magnet and will be dependent on the B0 magnet design, and must be adequate to support the weight of the crystals and the tracking detectors.

Provider:	Hadron Endcap Structures (6.03.01.09.03)

Details:	See details in the Detector Infrastructure Interface Control Document.

Requirements:	P-DET-DS-ANC.1, P-DET-DS-ANC.2

References:

I-DET-DS-INF-FWD.007	ZDC Support System

System 1:	Hadron Endcap Structures (6.03.01.09.03)

System 2:	Zero Degree Calorimeter (6.03.01.10)

Type:	STRUCT

Description:	The detector will be supported on a free standing structure.

Provider:	Hadron Endcap Structures (6.03.01.09.03)

Details:	See details in the Detector Infrastructure Interface Control Document.

Requirements:	P-DET-DS-ANC.1, P-DET-DS-ANC.2

References:

I-DET-DS-INF-INT.004	Space Constraint

System 1:	B-Zero Detectors (6.03.01.10)

System 2:	B-Zero Detectors (6.03.01.10)

Type:	SPACE

Description:	B0 detector must fit entirely within the bore of the B0 magnet.

Provider:	Detector Infrastructure and Utilities Integration (6.03.01.09.04)

Details:	See details in the Detector Infrastructure Interface Control Document.

Requirements:	P-DET-DS-ANC.1

References:

I-DET-DS-INF-INT.005	Space Constraint

System 1:	B-Zero Detectors (6.03.01.10)

System 2:	Forward HCal Systems (6.03.01.05)

Type:	SPACE

Description:	The position of the forward HCAL is bounded by the B0 magnet in the forward direction. Changes to the size or position of either must be coordinated with the other.

Provider:	Detector Infrastructure and Utilities Integration (6.03.01.09.04)

Details:	See details in the Detector Infrastructure Interface Control Document.

Requirements:	P-DET-DS-ANC.1, P-DET-DS-HCAL.1

References:

I-DET-DS-INF-INT.006	Space Constraint

System 1:	B-Zero Detectors (6.03.01.10)

System 2:	IR Superconducting Magnets (6.04.02)

Type:	SPACE

Description:	The position of and accessibility of the B0 detector in the backward direction is limited by the cryostat containing the final focusing magnets.

Provider:	Detector Infrastructure and Utilities Integration (6.03.01.09.04)

Details:	See details in the Detector Infrastructure Interface Control Document.

Requirements:	P-DET-DS-ANC.1

References:

I-DET-DS-INF-INT.007	Space Constraint

System 1:	B-Zero Detectors (6.03.01.10)

System 2:	IR Accelerator Systems (6.04.03)

Type:	SPACE

Description:	Access to the B0 detector is constrained by the vacuum valve in front of the B0 magnet. A minimum of 15 centimeters of free space will be required on the IP-side of the B0-dipole for the installation of the B0 detectors.

Provider:	Detector Infrastructure and Utilities Integration (6.03.01.09.04)

Details:	See details in the Detector Infrastructure Interface Control Document.

Requirements:	P-DET-DS-ANC.1

References:

I-DET-DS-INF-INT.008	Space Constraint

System 1:	B-Zero Detectors (6.03.01.10)

System 2:	IR Accelerator Systems (6.04.03)

Type:	SPACE

Description:	The electron and hadron beam pipes must be contained within the B0 detector.

Provider:	Detector Infrastructure and Utilities Integration (6.03.01.09.04)

Details:	See details in the Detector Infrastructure Interface Control Document.

Requirements:	P-DET-DS-ANC.1

References:

I-DET-DS-INF-INT.009	Position Constraint

System 1:	Low Q2 Detectors (6.03.01.10)

System 2:	IR Accelerator Systems (6.04.03)

Type:	SPACE

Description:	The LowQ2 detector must be upstream in the electron direction from the Q3ER magnet.

Provider:	Detector Infrastructure and Utilities Integration (6.03.01.09.04)

Details:	See details in the Detector Infrastructure Interface Control Document.

Requirements:	P-DET-DS-ANC.1

References:

I-DET-DS-INF-INT.010	Position Constraint

System 1:	Low Q2 Detectors (6.03.01.10)

System 2:	IR Accelerator Systems (6.04.03)

Type:	SPACE

Description:	The LowQ2 detector must be downstream in the electron direction from the B2ER magnet.

Provider:	Detector Infrastructure and Utilities Integration (6.03.01.09.04)

Details:	See details in the Detector Infrastructure Interface Control Document.

Requirements:	P-DET-DS-ANC.1

References:

I-DET-DS-INF-INT.011	Position Constraint

System 1:	Low Q2 Detectors (6.03.01.10)

System 2:	IR Systems Integration (6.04.04)

Type:	SPACE

Description:	The LOWQ2 detector must be at the same level as the electron beamline.

Provider:	Detector Infrastructure and Utilities Integration (6.03.01.09.04)

Details:	See details in the Detector Infrastructure Interface Control Document.

Requirements:	P-DET-DS-ANC.1

References:

I-DET-DS-INF-INT.040	Process Cooling

System 1:	B-Zero Detectors (6.03.01.10)

System 2:	IR Systems Integration (6.04.04)

Type:	SPACE

Description:	A cooling system will be required to remove heat from the calorimeter and electronics readouts, to maintain an acceptable temperature. A chiller should be installed near  the sub-system and  a space should be allocated  for this chiller.

Provider:	Detector Infrastructure and Utilities Integration (6.03.01.09.04)

Details:	See details in the Detector Infrastructure Interface Control Document.

Requirements:	P-DET-DS-ANC.3

References:

I-DET-DS-INF-INT.041	Process Cooling

System 1:	Detector Infrastructure and Utilities Integration (6.03.01.09.04)

System 2:	Low Q2 Detectors (6.03.01.10)

Type:	COOL

Description:	Either a liquid or gas cooling system will be required to remove heat from the calorimeter, tracking and readout electronics, to maintain them at room temperature.

Provider:	Detector Infrastructure and Utilities Integration (6.03.01.09.04)

Details:	See details in the Detector Infrastructure Interface Control Document.

Requirements:	P-DET-DS-ANC.3

References:

I-DET-DS-INF-INT.042	Dipole Power

System 1:	Detector Infrastructure and Utilities Integration (6.03.01.09.04)

System 2:	Luminosity Detectors (6.03.01.10)

Type:	ELEC

Description:	The two dipoles in front of the detector will require an infrastructure power source.

Provider:	Detector Infrastructure and Utilities Integration (6.03.01.09.04)

Details:	See details in the Detector Infrastructure Interface Control Document.

Requirements:	P-DET-DS-ANC.4

References:

I-DET-DS-INF-INT.043	Dipole Vacuum

System 1:	Detector Infrastructure and Utilities Integration (6.03.01.09.04)

System 2:	Luminosity Detectors (6.03.01.10)

Type:	VAC

Description:	The beamline between the two luminosity detector dipoles must be under vacuum.

Provider:	Detector Infrastructure and Utilities Integration (6.03.01.09.04)

Details:	See details in the Detector Infrastructure Interface Control Document.

Requirements:	P-DET-DS-ANC-LUMI.10

References:

I-DET-DS-INF-INT.044	Process Cooling

System 1:	Detector Infrastructure and Utilities Integration (6.03.01.09.04)

System 2:	Off-Momentum Detectors (6.03.01.10)

Type:	COOL

Description:	A cooling system will be required to remove heat from the tracking and readout electronics to prevent heat buildup.

Provider:	Detector Infrastructure and Utilities Integration (6.03.01.09.04)

Details:	See details in the Detector Infrastructure Interface Control Document.

Requirements:	P-DET-DS-ANC.3

References:

I-DET-DS-INF-INT.045	Process Cooling

System 1:	Detector Infrastructure and Utilities Integration (6.03.01.09.04)

System 2:	Roman Pots (6.03.01.10)

Type:	COOL

Description:	A cooling system will be required to remove heat from the tracking and readout electronics to prevent heat buildup.

Provider:	Detector Infrastructure and Utilities Integration (6.03.01.09.04)

Details:	See details in the Detector Infrastructure Interface Control Document.

Requirements:	P-DET-DS-ANC.3

References:

I-DET-DS-INF-INT.046	Process Cooling

System 1:	Detector Infrastructure and Utilities Integration (6.03.01.09.04)

System 2:	Zero Degree Calorimeter (6.03.01.10)

Type:	COOL

Description:	A cooling system will be required to remove heat from the calorimeter and readout electronics to prevent heat buildup.

Provider:	Detector Infrastructure and Utilities Integration (6.03.01.09.04)

Details:	See details in the Detector Infrastructure Interface Control Document.

Requirements:	P-DET-DS-ANC.3

References:

# Applicable Documents

List applicable documents considered to be part of this document. Applicable documents shall be obtained from the EIC SharePoint Document Center.

# Detailed change Log

| Revision | Date | Change |
| --- | --- | --- |

# References

- doi:10.17226/25171., National Academies of Sciences, Engineering, and Medicine, "An Assessment of U.S.-Based Electron-Ion Collider Science", The National Academies Press, Washington, DC, 2018.

- EIC-ORG-PLN-010, Electron-Ion Collider L1 Requirements (Global Requirements Document)

- BNL-221006-2021-FORE, EIC Conceptual Design Report

- arXiv:2103.05419, Science Requirements and Detector Concepts for the Electron-Ion Collider: EIC Yellow Report

- EIC-SEG-PLN-016, Requirements Management Plan

- EIC-SEG-PLN-022, Systems Engineering Management Plan

- EIC-SEG-PLN-020, EIC Interface Management Plan