Electron-Ion Collider, Requirements, Specifications, and Interfaces

Interface Control Document: [###]

Ancillary Detectors (6.03.01.10) to IR Accelerator Systems (6.04.03)

Prepared by:

{{Sig_es_:signer1:signature}}	Date: {{Dte_es_:signer1:date}}

Name, Role

Electron-Ion Collider at Institution

Reviewed by:

{{Sig_es_:signer2:signature}}	Date: {{Dte_es_:signer2:date}}

Name, Role

Electron-Ion Collider at Institution

{{Sig_es_:signer3:signature}}	Date: {{Dte_es_:signer3:date}}

Name, Role

Electron-Ion Collider at Institution

{{Sig_es_:signer4:signature}}	Date: {{Dte_es_:signer4:date}}

Name, Role

Electron-Ion Collider at Institution

Approved by:

{{Sig_es_:signer5:signature}}	Date: {{Dte_es_:signer5:date}}

Name, Role

Electron-Ion Collider at Institution

REVISION HISTORY

| Revision # | Effective Date | Additional Reviewers | Summary of Change |
| --- | --- | --- | --- |
| 00 |  |  | Initial release. |

TABLE OF CONTENTS

1.	Purpose and Scope	5

2.	Introduction	5

2.1	Ancillary Detectors	5

2.2	Overview of Interface Relationships	5

2.3	Interface Responsibilities	5

2.4	Safety and Environmental Considerations	5

2.5	Design Considerations	5

2.6	Organization	6

3.	Interface summary	6

4.	Ancillary Detectors (DET-DS-ANC) to IR Accelerator Systems (IR-AS)	11

5.	Applicable Documents	17

6.	Detailed change Log	17

7.	References	17

LIST OF ACRONYMS

| B0 | B0 is the name of the forward magnet just past the Hadron Endcap |  |
| --- | --- | --- |
| CDR | Conceptual Design Report |  |
| COTS | Commercial Off the Shelf |  |
| DAQ | Data Acquisition |  |
| DET | EIC Detector |  |
| DIRC | Detection of Internally Reflected Cherenkov Light |  |
| dRICH | Dual radiator ring imaging Cherenkov |  |
| DVCS | Deeply Virtual Compton Scattering |  |
| EIC | Electron-Ion Collider |  |
| EMCAL | Electro-magnetic Calorimeter |  |
| ESR | Electron Storage Ring |  |
| FCC | Federal Communications Commission |  |
| FDR | Final Design Review |  |
| GEM | Gas Electron Multiplier |  |
| HCAL | Hadronic Calorimeter |  |
| HSR | Hadron Storage Ring |  |
| HVAC | Heating/Ventilation/Air Conditioning |  |
| IP | Interaction Point |  |
| IR | Interaction Region |  |
| MPGD | Micro Pattern Gaseous Detectors |  |
| NAS | National Academy of Sciences |  |
| NEC | National Electric Code |  |
| NEMA | National Electrical Manufacturers Association |  |
| NFPA | National Fire Protection Association |  |
| NRTL | Nationally Recognized Testing Laboratory |  |
| PID | Particle Identification |  |
| PRR | Production Readiness Review |  |
| PWO | PbWO4 |  |
| RCS | Rapid Cycling Synchrotron |  |
| TBD | To Be Determined |  |
| UL | Underwriter Laboratories |  |
| WBS | Work Break Down Structure |  |

# Interface Control Document:

Ancillary Detectors (6.03.01.10) to IR Accelerator Systems (6.04.03)

# Purpose and Scope

This document records the agreed upon interfaces between the Ancillary Detectors and other sub-systems within the IR Accelerator Systems. These interfaces are based on the system requirements that are necessary to integrate these sub-systems as part of the Electron Ion Collider Project.

# Introduction

These interfaces describe the relationships between the Ancillary Detectors and the IR Accelerator Systems including (examples by inclusion) located in (areas by inclusion).

## Ancillary Detectors

Provide an introductory description of your sub-system and an overview of the things that it receives, provides, supports, constrains, or otherwise requires as part of its function --- specifically if these things are manifested as interfaces.

IR Accelerator Systems (IR-AS): Description of the TYPES of interfaces that exist between the ICD reference system and this sub-system, and a BRIEF overview of the roles they play.

## Overview of Interface Relationships

The following are the sub-systems within the IR Accelerator Systems that have interfaces connected to the Ancillary Detectors.

## Interface Responsibilities

The various ownerships of responsibilities may be listed here. However, this section might also refer to the Provider section of the interface detail if all pertinent responsibilities are listed there. If this is the case, simply enter: “See the Provider section within the interface details for information about the responsibilities associated with individual interfaces.”

## Safety and Environmental Considerations

General safety and environmental considerations are identified here. Any considerations that are specific to the individual sub-system, will be addressed in external safety documents and are referenced here.

### Considerations for all systems

Add general safety and environmental considerations here.

### Special considerations and references

Add references here as necessary.

## Design Considerations

Notes here about general design considerations.

## Organization

The interfaces in this document are grouped by sub-system as identified in the sub-system identifier. All interfaces between the Ancillary Detectors and a related sub-system will be included in a single section. Entries within that section will be sorted by the interface identifier or WBS number, as appropriate.

# Interface summary

This tables in this section provide a list of all interfaces that are defined within this document. The interfaces are grouped by sub-system/WBS. Each table has the following fields:

Section: the section number in this document where the interface is addressed.

WBS (1): the WBS number within the ICD reference system, unless the reference system is the owner.

WBS (2): the WBS number for the connected system.

ID: the interface identifier as defined in the Interface Requirement Document (IRD).

Name: the name of the interface as defined in the IRD.

Description: the description of the interface as defined in the IRD.

Owner: identifies the WBS number of the entity that is responsible for ensuring the interface is provided and that it meets validation/verification requirements.

Ancillary Detectors to IR Accelerator Systems

| Section | WBS1 | WBS2 | ID | Name | Description | Owner |
| --- | --- | --- | --- | --- | --- | --- |
| 4 |  | 6.04.03 |  |  | IR Accelerator Systems |  |
| 4.1 | 6.03.01.10 | 6.04.03 | I-DET-DS-ANC-LOWQ2.003 | LowQ2 Beamline Proximity | The LowQ2 detector must be positioned as close as possible electron beam pipe in order to detector particles at a shallow angle from the electron beam. | 6.03.01.10 |
| 4.2 | 6.03.01.10 | 6.04.03 | I-DET-DS-ANC-LOWQ2.005 | Accelerator Interference | The performance of the LowQ2 detector will depend on the inner apertures and locations of IR magnets ( from IP to B2eR) | 6.03.01.10 |
| 4.3 | 6.03.01.10 | 6.04.03 | I-DET-DS-ANC-LUMI.003 | Accelerator Interference | Detector must be positioned between the electron and hadron beamlines in the far backward area, and it's size and position are governed by the adjacent accelerator components. | 6.03.01.10 |
| 4.4 | 6.03.01.10 | 6.04.03 | I-DET-DS-ANC-LUMI.004 | Beamline Alignment | The center of the luminosity detector must be positioned along the centerline of the incoming electron beam as it passes through the interaction point. | 6.03.01.10 |
| 4.5 | 6.03.01.10 | 6.04.03 | I-DET-DS-ANC-LUMI.005 | Detector Placement | The detector must be positioned to avoid magnetic interference from adjacent beamline magnets. | 6.03.01.10 |
| 4.6 | 6.03.01.10 | 6.04.03 | I-DET-DS-ANC-OFFMO.002 | Backward Space Constraint | The detector must be positioned after the B1APF dipole in the hadron going direction. | 6.03.01.10 |
| 4.7 | 6.03.01.10 | 6.04.03 | I-DET-DS-ANC-OFFMO.003 | Beamline Integration | These detectors will be integrated with the outgoing hadron beamline and will be enclosed within it's vacuum system. | 6.03.01.10 |
| 4.8 | 6.03.01.10 | 6.04.03 | I-DET-DS-ANC-OFFMO.004 | Forward Space Constraint | The detector must be positioned before the Roman Pots in the hadron going direction. | 6.03.01.10 |
| 4.9 | 6.03.01.10 | 6.04.03 | I-DET-DS-ANC-OFFMO.006 | Accelerator Interference | This detector will contribute to the hadron accelerator impedance. Placement and location of detector layers have to be optimized. | 6.03.01.10 |
| 4.10 | 6.03.01.10 | 6.04.03 | I-DET-DS-ANC-OFFMO.007 | Zero-Degree Calorimeter Interference | The size and placement of the Off-momentum detector and their vacuum scattering chamber should be designed to have minimal interference with the 4 milliradian neutron cone required by the zero-degree calorimeter. | 6.03.01.10 |
| 4.11 | 6.03.01.10 | 6.04.03 | I-DET-DS-ANC-ROMAN.002 | Beamline Integration | A cooling system will be required to remove heat from the tracking and readout electronics to prevent heat buildup. Feedthroughs in the vacuum scattering chamber have to be allocated.. Feedthroughs in the vacuum scattering chamber have to be allocated for cables and for mechanical connections to the moving stages. | 6.03.01.10 |
| 4.12 | 6.03.01.10 | 6.04.03 | I-DET-DS-ANC-ROMAN.003 | Accelerator Interference | The Roman Pot detectors will come within 10 sigma of the center of the beam, and will impact the beam characteristics in a way that must be compensated for. | 6.03.01.10 |
| 4.13 | 6.03.01.10 | 6.04.03 | I-DET-DS-ANC-ROMAN.004 | Backward Space Constraint | The detector must be positioned after the off-momentum detectors in the hadron going direction. | 6.03.01.10 |
| 4.14 | 6.03.01.10 | 6.04.03 | I-DET-DS-ANC-ROMAN.005 | Beamline Integration | These detectors will be integrated with the outgoing hadron beamline and will be enclosed within it's vacuum system. | 6.03.01.10 |
| 4.15 | 6.03.01.10 | 6.04.03 | I-DET-DS-ANC-ROMAN.006 | Forward Space Constraint | The detector must be positioned before the B2PF magnet in the hadron going direction. | 6.03.01.10 |
| 4.16 | 6.03.01.10 | 6.04.03 | I-DET-DS-ANC-ROMAN.007 | RF Shielding | This detector will generate RF radiation that the accelerator beamline must be shielded from. | 6.03.01.10 |
| 4.17 | 6.03.01.10 | 6.04.03 | I-DET-DS-ANC-ROMAN.008 | Accelerator Interference | This detector will contribute to the hadron accelerator impedance. Placement and location of detector layers have to be optimized. | 6.03.01.10 |
| 4.18 | 6.03.01.10 | 6.04.03 | I-DET-DS-INF-FWD.001 | Detector Support | The detector will be supported by an integrated support stand that is fully integrated with the vacuum system and allows the position of the detector to be adjusted. | 6.03.01.09.03 |
| 4.19 | 6.03.01.10 | 6.04.03 | I-DET-DS-INF-FWD.002 | Detector Support | The detector will be supported by an integrated support stand that is fully integrated with the vacuum system and allows the position of the detector to be adjusted. | 6.03.01.09.03 |
| 4.20 | 6.03.01.10 | 6.04.03 | I-DET-DS-INF-INT.007 | Space Constraint | Access to the B0 detector is constrained by the vacuum valve in front of the B0 magnet. A minimum of 15 centimeters of free space will be required on the IP-side of the B0-dipole for the installation of the B0 detectors. | 6.03.01.09.04 |
| 4.21 | 6.03.01.10 | 6.04.03 | I-DET-DS-INF-INT.008 | Space Constraint | The electron and hadron beam pipes must be contained within the B0 detector. | 6.03.01.09.04 |
| 4.22 | 6.03.01.10 | 6.04.03 | I-DET-DS-INF-INT.009 | Position Constraint | The LowQ2 detector must be upstream in the electron direction from the Q3ER magnet. | 6.03.01.09.04 |
| 4.23 | 6.03.01.10 | 6.04.03 | I-DET-DS-INF-INT.010 | Position Constraint | The LowQ2 detector must be downstream in the electron direction from the B2ER magnet. | 6.03.01.09.04 |

# Ancillary Detectors (DET-DS-ANC) to IR Accelerator Systems (IR-AS)

I-DET-DS-ANC-LOWQ2.003	LowQ2 Beamline Proximity

System 1:	Low Q2 Detectors (6.03.01.10)

System 2:	IR Accelerator Systems (6.04.03)

Type:	SPACE

Description:	The LowQ2 detector must be positioned as close as possible electron beam pipe in order to detector particles at a shallow angle from the electron beam.

Provider:	Low Q2 Detectors (6.03.01.10)

Details:	See details in the Detector System Interface Control Document.

Requirements:	P-DET-DS-ANC-LOWQ2.16

References:

I-DET-DS-ANC-LOWQ2.005	Accelerator Interference

System 1:	Low Q2 Detectors (6.03.01.10)

System 2:	IR Accelerator Systems (6.04.03)

Type:	ENV

Description:	The performance of the LowQ2 detector will depend on the inner apertures and locations of IR magnets ( from IP to B2eR)

Provider:	Low Q2 Detectors (6.03.01.10)

Details:	See details in the Detector System Interface Control Document.

Requirements:

References:

I-DET-DS-ANC-LUMI.003	Accelerator Interference

System 1:	Luminosity Detectors (6.03.01.10)

System 2:	IR Accelerator Systems (6.04.03)

Type:	SPACE

Description:	Detector must be positioned between the electron and hadron beamlines in the far backward area, and it's size and position are governed by the adjacent accelerator components.

Provider:	Luminosity Detectors (6.03.01.10)

Details:	See details in the Detector System Interface Control Document.

Requirements:	P-DET-DS-ANC.1

References:

I-DET-DS-ANC-LUMI.004	Beamline Alignment

System 1:	Luminosity Detectors (6.03.01.10)

System 2:	IR Accelerator Systems (6.04.03)

Type:	SPACE

Description:	The center of the luminosity detector must be positioned along the centerline of the incoming electron beam as it passes through the interaction point.

Provider:	Luminosity Detectors (6.03.01.10)

Details:	See details in the Detector System Interface Control Document.

Requirements:	P-DET-DS-ANC.1

References:

I-DET-DS-ANC-LUMI.005	Detector Placement

System 1:	Luminosity Detectors (6.03.01.10)

System 2:	IR Accelerator Systems (6.04.03)

Type:	ENV

Description:	The detector must be positioned to avoid magnetic interference from adjacent beamline magnets.

Provider:	Luminosity Detectors (6.03.01.10)

Details:	See details in the Detector System Interface Control Document.

Requirements:	P-DET-DS-ANC-LUMI.9

References:

I-DET-DS-ANC-OFFMO.002	Backward Space Constraint

System 1:	Off-Momentum Detectors (6.03.01.10)

System 2:	IR Accelerator Systems (6.04.03)

Type:	SPACE

Description:	The detector must be positioned after the B1APF dipole in the hadron going direction.

Provider:	Off-Momentum Detectors (6.03.01.10)

Details:	See details in the Detector System Interface Control Document.

Requirements:	P-DET-DS-ANC.1

References:

I-DET-DS-ANC-OFFMO.003	Beamline Integration

System 1:	Off-Momentum Detectors (6.03.01.10)

System 2:	IR Accelerator Systems (6.04.03)

Type:	MECH

Description:	These detectors will be integrated with the outgoing hadron beamline and will be enclosed within it's vacuum system.

Provider:	Off-Momentum Detectors (6.03.01.10)

Details:	See details in the Detector System Interface Control Document.

Requirements:	P-DET-DS-ANC.1

References:

I-DET-DS-ANC-OFFMO.004	Forward Space Constraint

System 1:	Off-Momentum Detectors (6.03.01.10)

System 2:	IR Accelerator Systems (6.04.03)

Type:	SPACE

Description:	The detector must be positioned before the Roman Pots in the hadron going direction.

Provider:	Off-Momentum Detectors (6.03.01.10)

Details:	See details in the Detector System Interface Control Document.

Requirements:	P-DET-DS-ANC.1

References:

I-DET-DS-ANC-OFFMO.006	Accelerator Interference

System 1:	Off-Momentum Detectors (6.03.01.10)

System 2:	IR Accelerator Systems (6.04.03)

Type:	ENV

Description:	This detector will  contribute to the hadron accelerator  impedance. Placement and location of detector layers have to be optimized.

Provider:	Off-Momentum Detectors (6.03.01.10)

Details:	See details in the Detector System Interface Control Document.

Requirements:	P-DET-DS-ANC-OFFMO

References:

I-DET-DS-ANC-OFFMO.007	Zero-Degree Calorimeter Interference

System 1:	Off-Momentum Detectors (6.03.01.10)

System 2:	IR Accelerator Systems (6.04.03)

Type:	SPACE

Description:	The size and placement of the Off-momentum detector  and their  vacuum scattering chamber should be designed to have minimal interference with the 4 milliradian neutron cone required by the zero-degree calorimeter.

Provider:	Off-Momentum Detectors (6.03.01.10)

Details:	See details in the Detector System Interface Control Document.

Requirements:	P-DET-DS-ANC.1

References:

I-DET-DS-ANC-ROMAN.002	Beamline Integration

System 1:	Roman Pots (6.03.01.10)

System 2:	IR Accelerator Systems (6.04.03)

Type:	VAC

Description:	A cooling system will be required to remove heat from the tracking and readout electronics to prevent heat buildup.  Feedthroughs in the vacuum scattering chamber have to be allocated..  Feedthroughs in the vacuum scattering chamber have to be allocated for cables and  for mechanical  connections to the moving stages.

Provider:	Roman Pots (6.03.01.10)

Details:	See details in the Detector System Interface Control Document.

Requirements:	P-DET-DS-ANC.3

References:

I-DET-DS-ANC-ROMAN.003	Accelerator Interference

System 1:	Roman Pots (6.03.01.10)

System 2:	IR Accelerator Systems (6.04.03)

Type:	PARAM

Description:	The Roman Pot detectors will come within 10 sigma of the center of the beam, and will impact the beam characteristics in a way that must be compensated for.

Provider:	Roman Pots (6.03.01.10)

Details:	See details in the Detector System Interface Control Document.

Requirements:	P-DET-DS-ANC-ROMAN.10

References:

I-DET-DS-ANC-ROMAN.004	Backward Space Constraint

System 1:	Roman Pots (6.03.01.10)

System 2:	IR Accelerator Systems (6.04.03)

Type:	SPACE

Description:	The detector must be positioned after the off-momentum detectors in the hadron going direction.

Provider:	Roman Pots (6.03.01.10)

Details:	See details in the Detector System Interface Control Document.

Requirements:	P-DET-DS-ANC.1

References:

I-DET-DS-ANC-ROMAN.005	Beamline Integration

System 1:	Roman Pots (6.03.01.10)

System 2:	IR Accelerator Systems (6.04.03)

Type:	MECH

Description:	These detectors will be integrated with the outgoing hadron beamline and will be enclosed within it's vacuum system.

Provider:	Roman Pots (6.03.01.10)

Details:	See details in the Detector System Interface Control Document.

Requirements:	P-DET-DS-ANC.1

References:

I-DET-DS-ANC-ROMAN.006	Forward Space Constraint

System 1:	Roman Pots (6.03.01.10)

System 2:	IR Accelerator Systems (6.04.03)

Type:	SPACE

Description:	The detector must be positioned before the B2PF magnet in the hadron going direction.

Provider:	Roman Pots (6.03.01.10)

Details:	See details in the Detector System Interface Control Document.

Requirements:	P-DET-DS-ANC.1

References:

I-DET-DS-ANC-ROMAN.007	RF Shielding

System 1:	Roman Pots (6.03.01.10)

System 2:	IR Accelerator Systems (6.04.03)

Type:	ENV

Description:	This detector will generate RF radiation that the accelerator beamline must be shielded from.

Provider:	Roman Pots (6.03.01.10)

Details:	See details in the Detector System Interface Control Document.

Requirements:	P-DET-DS-ANC-ROMAN.9

References:

I-DET-DS-ANC-ROMAN.008	Accelerator Interference

System 1:	Roman Pots (6.03.01.10)

System 2:	IR Accelerator Systems (6.04.03)

Type:	ENV

Description:	This detector will  contribute to the hadron accelerator  impedance. Placement and location of detector layers have to be optimized.

Provider:	Roman Pots (6.03.01.10)

Details:	See details in the Detector System Interface Control Document.

Requirements:	P-DET-DS-ANC-ROMAN.9

References:

I-DET-DS-INF-FWD.001	Detector Support

System 1:	Off-Momentum Detectors (6.03.01.10)

System 2:	IR Accelerator Systems (6.04.03)

Type:	STRUCT

Description:	The detector will be supported by an integrated support stand that is fully integrated with the vacuum system and allows the position of the detector to be adjusted.

Provider:	Hadron Endcap Structures (6.03.01.09.03)

Details:	See details in the Detector System Interface Control Document.

Requirements:	P-DET-DS-ANC.1, P-DET-DS-ANC.2

References:

I-DET-DS-INF-FWD.002	Detector Support

System 1:	Roman Pots (6.03.01.10)

System 2:	IR Accelerator Systems (6.04.03)

Type:	STRUCT

Description:	The detector will be supported by an integrated support stand that is fully integrated with the vacuum system and allows the position of the detector to be adjusted.

Provider:	Hadron Endcap Structures (6.03.01.09.03)

Details:	See details in the Detector System Interface Control Document.

Requirements:	P-DET-DS-ANC.1, P-DET-DS-ANC.2

References:

I-DET-DS-INF-INT.007	Space Constraint

System 1:	B-Zero Detectors (6.03.01.10)

System 2:	IR Accelerator Systems (6.04.03)

Type:	SPACE

Description:	Access to the B0 detector is constrained by the vacuum valve in front of the B0 magnet. A minimum of 15 centimeters of free space will be required on the IP-side of the B0-dipole for the installation of the B0 detectors.

Provider:	Detector Infrastructure and Utilities Integration (6.03.01.09.04)

Details:	See details in the Detector System Interface Control Document.

Requirements:	P-DET-DS-ANC.1

References:

I-DET-DS-INF-INT.008	Space Constraint

System 1:	B-Zero Detectors (6.03.01.10)

System 2:	IR Accelerator Systems (6.04.03)

Type:	SPACE

Description:	The electron and hadron beam pipes must be contained within the B0 detector.

Provider:	Detector Infrastructure and Utilities Integration (6.03.01.09.04)

Details:	See details in the Detector System Interface Control Document.

Requirements:	P-DET-DS-ANC.1

References:

I-DET-DS-INF-INT.009	Position Constraint

System 1:	Low Q2 Detectors (6.03.01.10)

System 2:	IR Accelerator Systems (6.04.03)

Type:	SPACE

Description:	The LowQ2 detector must be upstream in the electron direction from the Q3ER magnet.

Provider:	Detector Infrastructure and Utilities Integration (6.03.01.09.04)

Details:	See details in the Detector System Interface Control Document.

Requirements:	P-DET-DS-ANC.1

References:

I-DET-DS-INF-INT.010	Position Constraint

System 1:	Low Q2 Detectors (6.03.01.10)

System 2:	IR Accelerator Systems (6.04.03)

Type:	SPACE

Description:	The LowQ2 detector must be downstream in the electron direction from the B2ER magnet.

Provider:	Detector Infrastructure and Utilities Integration (6.03.01.09.04)

Details:	See details in the Detector System Interface Control Document.

Requirements:	P-DET-DS-ANC.1

References:

# Applicable Documents

List applicable documents considered to be part of this document. Applicable documents shall be obtained from the EIC SharePoint Document Center.

# Detailed change Log

| Revision | Date | Change |
| --- | --- | --- |

# References

- doi:10.17226/25171., National Academies of Sciences, Engineering, and Medicine, "An Assessment of U.S.-Based Electron-Ion Collider Science", The National Academies Press, Washington, DC, 2018.

- EIC-ORG-PLN-010, Electron-Ion Collider L1 Requirements (Global Requirements Document)

- BNL-221006-2021-FORE, EIC Conceptual Design Report

- arXiv:2103.05419, Science Requirements and Detector Concepts for the Electron-Ion Collider: EIC Yellow Report

- EIC-SEG-PLN-016, Requirements Management Plan

- EIC-SEG-PLN-022, Systems Engineering Management Plan

- EIC-SEG-PLN-020, EIC Interface Management Plan