Electron-Ion Collider, Requirements, Specifications, and Interfaces

Interface Control Document: [###]

Ancillary Detectors (6.03.01.10) to IR Systems Integration (6.04.04)

Prepared by:

{{Sig_es_:signer1:signature}}	Date: {{Dte_es_:signer1:date}}

Name, Role

Electron-Ion Collider at Institution

Reviewed by:

{{Sig_es_:signer2:signature}}	Date: {{Dte_es_:signer2:date}}

Name, Role

Electron-Ion Collider at Institution

{{Sig_es_:signer3:signature}}	Date: {{Dte_es_:signer3:date}}

Name, Role

Electron-Ion Collider at Institution

{{Sig_es_:signer4:signature}}	Date: {{Dte_es_:signer4:date}}

Name, Role

Electron-Ion Collider at Institution

Approved by:

{{Sig_es_:signer5:signature}}	Date: {{Dte_es_:signer5:date}}

Name, Role

Electron-Ion Collider at Institution

REVISION HISTORY

| Revision # | Effective Date | Additional Reviewers | Summary of Change |
| --- | --- | --- | --- |
| 00 |  |  | Initial release. |

TABLE OF CONTENTS

1.	Purpose and Scope	5

2.	Introduction	5

2.1	Ancillary Detectors	5

2.2	Overview of Interface Relationships	5

2.3	Interface Responsibilities	5

2.4	Safety and Environmental Considerations	5

2.5	Design Considerations	5

2.6	Organization	6

3.	Interface summary	6

4.	Ancillary Detectors (DET-DS-ANC) to IR Systems Integration (IR-SI)	10

5.	Applicable Documents	15

6.	Detailed change Log	15

7.	References	15

LIST OF ACRONYMS

| B0 | B0 is the name of the forward magnet just past the Hadron Endcap |  |
| --- | --- | --- |
| CDR | Conceptual Design Report |  |
| COTS | Commercial Off the Shelf |  |
| DAQ | Data Acquisition |  |
| DET | EIC Detector |  |
| DIRC | Detection of Internally Reflected Cherenkov Light |  |
| dRICH | Dual radiator ring imaging Cherenkov |  |
| DVCS | Deeply Virtual Compton Scattering |  |
| EIC | Electron-Ion Collider |  |
| EMCAL | Electro-magnetic Calorimeter |  |
| ESR | Electron Storage Ring |  |
| FCC | Federal Communications Commission |  |
| FDR | Final Design Review |  |
| GEM | Gas Electron Multiplier |  |
| HCAL | Hadronic Calorimeter |  |
| HSR | Hadron Storage Ring |  |
| HVAC | Heating/Ventilation/Air Conditioning |  |
| IP | Interaction Point |  |
| IR | Interaction Region |  |
| MPGD | Micro Pattern Gaseous Detectors |  |
| NAS | National Academy of Sciences |  |
| NEC | National Electric Code |  |
| NEMA | National Electrical Manufacturers Association |  |
| NFPA | National Fire Protection Association |  |
| NRTL | Nationally Recognized Testing Laboratory |  |
| PID | Particle Identification |  |
| PRR | Production Readiness Review |  |
| PWO | PbWO4 |  |
| RCS | Rapid Cycling Synchrotron |  |
| TBD | To Be Determined |  |
| UL | Underwriter Laboratories |  |
| WBS | Work Break Down Structure |  |

# Interface Control Document:

Ancillary Detectors (6.03.01.10) to IR Systems Integration (6.04.04)

# Purpose and Scope

This document records the agreed upon interfaces between the Ancillary Detectors and other sub-systems within the IR Systems Integration. These interfaces are based on the system requirements that are necessary to integrate these sub-systems as part of the Electron Ion Collider Project.

# Introduction

These interfaces describe the relationships between the Ancillary Detectors and the IR Systems Integration including (examples by inclusion) located in (areas by inclusion).

## Ancillary Detectors

Provide an introductory description of your sub-system and an overview of the things that it receives, provides, supports, constrains, or otherwise requires as part of its function --- specifically if these things are manifested as interfaces.

IR Systems Integration (IR-SI): Description of the TYPES of interfaces that exist between the ICD reference system and this sub-system, and a BRIEF overview of the roles they play.

## Overview of Interface Relationships

The following are the sub-systems within the IR Systems Integration that have interfaces connected to the Ancillary Detectors.

## Interface Responsibilities

The various ownerships of responsibilities may be listed here. However, this section might also refer to the Provider section of the interface detail if all pertinent responsibilities are listed there. If this is the case, simply enter: “See the Provider section within the interface details for information about the responsibilities associated with individual interfaces.”

## Safety and Environmental Considerations

General safety and environmental considerations are identified here. Any considerations that are specific to the individual sub-system, will be addressed in external safety documents and are referenced here.

### Considerations for all systems

Add general safety and environmental considerations here.

### Special considerations and references

Add references here as necessary.

## Design Considerations

Notes here about general design considerations.

## Organization

The interfaces in this document are grouped by sub-system as identified in the sub-system identifier. All interfaces between the Ancillary Detectors and a related sub-system will be included in a single section. Entries within that section will be sorted by the interface identifier or WBS number, as appropriate.

# Interface summary

This tables in this section provide a list of all interfaces that are defined within this document. The interfaces are grouped by sub-system/WBS. Each table has the following fields:

Section: the section number in this document where the interface is addressed.

WBS (1): the WBS number within the ICD reference system, unless the reference system is the owner.

WBS (2): the WBS number for the connected system.

ID: the interface identifier as defined in the Interface Requirement Document (IRD).

Name: the name of the interface as defined in the IRD.

Description: the description of the interface as defined in the IRD.

Owner: identifies the WBS number of the entity that is responsible for ensuring the interface is provided and that it meets validation/verification requirements.

Ancillary Detectors to IR Systems Integration

| Section | WBS1 | WBS2 | ID | Name | Description | Owner |
| --- | --- | --- | --- | --- | --- | --- |
| 4 |  | 6.04.04 |  |  | IR Systems Integration |  |
| 4.1 | 6.03.01.10 | 6.04.04 | I-DET-DS-ANC-LOWQ2.001 | Process Cooling | Either a liquid or gas cooling system will be required to remove heat from the calorimeter, tracking and readout electronics, to maintain them at room temperature. A chiller should be installed near the sub-system and a space in the tunnel should be allocated for this chiller. | 6.03.01.10 |
| 4.2 | 6.03.01.10 | 6.04.04 | I-DET-DS-ANC-LOWQ2.002 | Beam pipe Exit Window | The performance of the LowQ2 detector will depend on the thickness and shape of the electron beam pipe exit window. | 6.03.01.10 |
| 4.3 | 6.03.01.10 | 6.04.04 | I-DET-DS-ANC-LOWQ2.006 | Electronics Placement and Shielding | The Low-Q2 detector electronics enclosures must be positioned near the detector and must be shielded from radiation. | 6.03.01.10 |
| 4.4 | 6.03.01.10 | 6.04.04 | I-DET-DS-ANC-LUMI.001 | Process Cooling | A cooling system will be required to remove heat from the tracking and readout electronics to prevent heat buildup. A chiller should be installed near the sub-system and a space in the tunnel should be allocated for this chiller. | 6.03.01.10 |
| 4.5 | 6.03.01.10 | 6.04.04 | I-DET-DS-ANC-LUMI.006 | Dipole Cooling | The two dipoles in front of the detector will need LCW cooling. | 6.03.01.10 |
| 4.6 | 6.03.01.10 | 6.04.04 | I-DET-DS-ANC-LUMI.007 | Electronics Placement and Shielding | The detector electronics enclosures must be positioned near the detector and must be shielded from radiation. | 6.03.01.10 |
| 4.7 | 6.03.01.10 | 6.04.04 | I-DET-DS-ANC-LUMI.008 | Placement | The luminosity detector must be positioned between 30 and 100 meters from the interaction point in the backward direction, and the dectors centerpoint must be positioned at beam level. | 6.03.01.10 |
| 4.8 | 6.03.01.10 | 6.04.04 | I-DET-DS-ANC-OFFMO.001 | Process Cooling | A cooling system will be required to remove heat from the tracking and readout electronics to prevent heat buildup. A chiller should be installed near the sub-system and a space in the tunnel should be allocated for this chiller. | 6.03.01.10 |
| 4.9 | 6.03.01.10 | 6.04.04 | I-DET-DS-ANC-OFFMO.005 | RF Shielding | This detector will generate RF radiation that the accelerator beamline must be shielded from. | 6.03.01.10 |
| 4.10 | 6.03.01.10 | 6.04.04 | I-DET-DS-ANC-OFFMO.008 | Photon Cone Interference | There should minimal interference within the 1 mrad cone of direct photons from IP ( with respect to the incoming e-line) in front of the lumi-syste, | 6.03.01.10 |
| 4.11 | 6.03.01.10 | 6.04.04 | I-DET-DS-ANC-ROMAN.001 | Process Cooling | A cooling system will be required to remove heat from the tracking and readout electronics to prevent heat buildup. A chiller should be installed near the sub-system and a space in the tunnel should be allocated for this chiller. | 6.03.01.10 |
| 4.12 | 6.03.01.10 | 6.04.04 | I-DET-DS-ANC-ZDC.001 | Process Cooling | A cooling system will be required to remove heat from the calorimeter and readout electronics to prevent heat buildup. A chiller should be installed near the sub-system and a space in the tunnel should be allocated for this chiller. | 6.03.01.10 |
| 4.13 | 6.03.01.10 | 6.04.04 | I-DET-DS-ANC-ZDC.002 | Neutron Cone Dependency | The placement of the Zero Degree Calorimeter is dependent on the neutron cone and should be positioned to optimize detector performance and resolution. | 6.03.01.10 |
| 4.14 | 6.03.01.10 | 6.04.04 | I-DET-DS-ANC-ZDC.003 | Neutron Cone Interference | There should minimal interference within the 4 milliradian neutron cone in front of the Zero Degree Calorimeter detector. | 6.03.01.10 |
| 4.15 | 6.03.01.10 | 6.04.04 | I-DET-DS-ANC-ZDC.004 | ZDC Placement | The Zero Degree Calorimeter will be positioned between the outgoing hadron and incoming electron beamlines. | 6.03.01.10 |
| 4.16 | 6.03.01.10 | 6.04.04 | I-DET-DS-INF-INT.011 | Position Constraint | The LOWQ2 detector must be at the same level as the electron beamline. | 6.03.01.09.04 |
| 4.17 | 6.03.01.10 | 6.04.04 | I-DET-DS-INF-INT.040 | Process Cooling | A cooling system will be required to remove heat from the calorimeter and electronics readouts, to maintain an acceptable temperature. A chiller should be installed near the sub-system and a space should be allocated for this chiller. | 6.03.01.09.04 |

# Ancillary Detectors (DET-DS-ANC) to IR Systems Integration (IR-SI)

I-DET-DS-ANC-LOWQ2.001	Process Cooling

System 1:	Low Q2 Detectors (6.03.01.10)

System 2:	IR Systems Integration (6.04.04)

Type:	SPACE

Description:	Either a liquid or gas cooling system will be required to remove heat from the calorimeter, tracking and readout electronics, to maintain them at room temperature. A chiller should be installed near  the sub-system and  a space in the tunnel should be allocated  for this chiller.

Provider:	Low Q2 Detectors (6.03.01.10)

Details:	See details in the Detector System Interface Control Document.

Requirements:	P-DET-DS-ANC.3

References:

I-DET-DS-ANC-LOWQ2.002	Beam pipe Exit Window

System 1:	Low Q2 Detectors (6.03.01.10)

System 2:	IR Systems Integration (6.04.04)

Type:	ENV

Description:	The performance of the LowQ2 detector will depend on the thickness and shape of the electron beam pipe exit window.

Provider:	Low Q2 Detectors (6.03.01.10)

Details:	See details in the Detector System Interface Control Document.

Requirements:	P-DET-DS-ANC-LOWQ2.16

References:

I-DET-DS-ANC-LOWQ2.006	Electronics Placement and Shielding

System 1:	Low Q2 Detectors (6.03.01.10)

System 2:	IR Systems Integration (6.04.04)

Type:	ENV

Description:	The Low-Q2  detector electronics enclosures must be positioned near the detector and must be shielded from radiation.

Provider:	Low Q2 Detectors (6.03.01.10)

Details:	See details in the Detector System Interface Control Document.

Requirements:

References:

I-DET-DS-ANC-LUMI.001	Process Cooling

System 1:	Luminosity Detectors (6.03.01.10)

System 2:	IR Systems Integration (6.04.04)

Type:	SPACE

Description:	A cooling system will be required to remove heat from the tracking and readout electronics to prevent heat buildup. A chiller should be installed near  the sub-system and  a space in the tunnel should be allocated  for this chiller.

Provider:	Luminosity Detectors (6.03.01.10)

Details:	See details in the Detector System Interface Control Document.

Requirements:	P-DET-DS-ANC.3

References:

I-DET-DS-ANC-LUMI.006	Dipole Cooling

System 1:	Luminosity Detectors (6.03.01.10)

System 2:	IR Systems Integration (6.04.04)

Type:	COOL

Description:	The two dipoles in front of the detector will need LCW cooling.

Provider:	Luminosity Detectors (6.03.01.10)

Details:	See details in the Detector System Interface Control Document.

Requirements:	P-DET-DS-ANC.3

References:

I-DET-DS-ANC-LUMI.007	Electronics Placement and Shielding

System 1:	Luminosity Detectors (6.03.01.10)

System 2:	IR Systems Integration (6.04.04)

Type:	ENV

Description:	The detector electronics enclosures must be positioned near the detector and must be shielded from radiation.

Provider:	Luminosity Detectors (6.03.01.10)

Details:	See details in the Detector System Interface Control Document.

Requirements:	P-DET-DS-ANC-LUMI.9

References:

I-DET-DS-ANC-LUMI.008	Placement

System 1:	Luminosity Detectors (6.03.01.10)

System 2:	IR Systems Integration (6.04.04)

Type:	SPACE

Description:	The luminosity detector must be positioned between 30 and 100 meters from the interaction point in the backward direction, and the dectors centerpoint must be positioned at beam level.

Provider:	Luminosity Detectors (6.03.01.10)

Details:	See details in the Detector System Interface Control Document.

Requirements:	P-DET-DS-ANC.1

References:

I-DET-DS-ANC-OFFMO.001	Process Cooling

System 1:	Off-Momentum Detectors (6.03.01.10)

System 2:	IR Systems Integration (6.04.04)

Type:	SPACE

Description:	A cooling system will be required to remove heat from the tracking and readout electronics to prevent heat buildup. A chiller should be installed near  the sub-system and  a space in the tunnel should be allocated  for this chiller.

Provider:	Off-Momentum Detectors (6.03.01.10)

Details:	See details in the Detector System Interface Control Document.

Requirements:	P-DET-DS-ANC.3

References:

I-DET-DS-ANC-OFFMO.005	RF Shielding

System 1:	Off-Momentum Detectors (6.03.01.10)

System 2:	IR Systems Integration (6.04.04)

Type:	ENV

Description:	This detector will generate RF radiation that the accelerator beamline must be shielded from.

Provider:	Off-Momentum Detectors (6.03.01.10)

Details:	See details in the Detector System Interface Control Document.

Requirements:	P-DET-DS-ANC-OFFMO.9

References:

I-DET-DS-ANC-OFFMO.008	Photon Cone Interference

System 1:	Luminosity Detectors (6.03.01.10)

System 2:	IR Systems Integration (6.04.04)

Type:	ENV

Description:	There should minimal interference within the 1 mrad  cone  of direct photons from IP ( with respect to the incoming e-line)  in front of the lumi-syste,

Provider:	Off-Momentum Detectors (6.03.01.10)

Details:	See details in the Detector System Interface Control Document.

Requirements:	P-DET-DS-ANC-LUMI.3

References:

I-DET-DS-ANC-ROMAN.001	Process Cooling

System 1:	Roman Pots (6.03.01.10)

System 2:	IR Systems Integration (6.04.04)

Type:	SPACE

Description:	A cooling system will be required to remove heat from the tracking and readout electronics to prevent heat buildup. A chiller should be installed near  the sub-system and  a space in the tunnel should be allocated  for this chiller.

Provider:	Roman Pots (6.03.01.10)

Details:	See details in the Detector System Interface Control Document.

Requirements:	P-DET-DS-ANC.3

References:

I-DET-DS-ANC-ZDC.001	Process Cooling

System 1:	Zero Degree Calorimeter (6.03.01.10)

System 2:	IR Systems Integration (6.04.04)

Type:	SPACE

Description:	A cooling system will be required to remove heat from the calorimeter and readout electronics to prevent heat buildup. A chiller should be installed near  the sub-system and  a space in the tunnel should be allocated  for this chiller.

Provider:	Zero Degree Calorimeter (6.03.01.10)

Details:	See details in the Detector System Interface Control Document.

Requirements:	P-DET-DS-ANC.3

References:

I-DET-DS-ANC-ZDC.002	Neutron Cone Dependency

System 1:	Zero Degree Calorimeter (6.03.01.10)

System 2:	IR Systems Integration (6.04.04)

Type:	SPACE

Description:	The placement of the Zero Degree Calorimeter is dependent on the neutron cone and should be positioned to optimize detector performance and resolution.

Provider:	Zero Degree Calorimeter (6.03.01.10)

Details:	See details in the Detector System Interface Control Document.

Requirements:	P-DET-DS-ANC.1

References:

I-DET-DS-ANC-ZDC.003	Neutron Cone Interference

System 1:	Zero Degree Calorimeter (6.03.01.10)

System 2:	IR Systems Integration (6.04.04)

Type:	ENV

Description:	There should minimal interference within the 4 milliradian neutron cone in front of  the Zero Degree Calorimeter detector.

Provider:	Zero Degree Calorimeter (6.03.01.10)

Details:	See details in the Detector System Interface Control Document.

Requirements:	P-DET-DS-ANC.1

References:

I-DET-DS-ANC-ZDC.004	ZDC Placement

System 1:	Zero Degree Calorimeter (6.03.01.10)

System 2:	IR Systems Integration (6.04.04)

Type:	SPACE

Description:	The Zero Degree Calorimeter will be positioned between the outgoing hadron and incoming electron beamlines.

Provider:	Zero Degree Calorimeter (6.03.01.10)

Details:	See details in the Detector System Interface Control Document.

Requirements:	P-DET-DS-ANC.1

References:

I-DET-DS-INF-INT.011	Position Constraint

System 1:	Low Q2 Detectors (6.03.01.10)

System 2:	IR Systems Integration (6.04.04)

Type:	SPACE

Description:	The LOWQ2 detector must be at the same level as the electron beamline.

Provider:	Detector Infrastructure and Utilities Integration (6.03.01.09.04)

Details:	See details in the Detector System Interface Control Document.

Requirements:	P-DET-DS-ANC.1

References:

I-DET-DS-INF-INT.040	Process Cooling

System 1:	B-Zero Detectors (6.03.01.10)

System 2:	IR Systems Integration (6.04.04)

Type:	SPACE

Description:	A cooling system will be required to remove heat from the calorimeter and electronics readouts, to maintain an acceptable temperature. A chiller should be installed near  the sub-system and  a space should be allocated  for this chiller.

Provider:	Detector Infrastructure and Utilities Integration (6.03.01.09.04)

Details:	See details in the Detector System Interface Control Document.

Requirements:	P-DET-DS-ANC.3

References:

# Applicable Documents

List applicable documents considered to be part of this document. Applicable documents shall be obtained from the EIC SharePoint Document Center.

# Detailed change Log

| Revision | Date | Change |
| --- | --- | --- |

# References

- doi:10.17226/25171., National Academies of Sciences, Engineering, and Medicine, "An Assessment of U.S.-Based Electron-Ion Collider Science", The National Academies Press, Washington, DC, 2018.

- EIC-ORG-PLN-010, Electron-Ion Collider L1 Requirements (Global Requirements Document)

- BNL-221006-2021-FORE, EIC Conceptual Design Report

- arXiv:2103.05419, Science Requirements and Detector Concepts for the Electron-Ion Collider: EIC Yellow Report

- EIC-SEG-PLN-016, Requirements Management Plan

- EIC-SEG-PLN-022, Systems Engineering Management Plan

- EIC-SEG-PLN-020, EIC Interface Management Plan