Electron-Ion Collider, Requirements, Specifications, and Interfaces

Interface Control Document: [###]

Data Acquisition and Computing Systems (6.03.01.08) to Detector System (6.03.01)

Prepared by:

{{Sig_es_:signer1:signature}}	Date: {{Dte_es_:signer1:date}}

Name, Role

Electron-Ion Collider at Institution

Reviewed by:

{{Sig_es_:signer2:signature}}	Date: {{Dte_es_:signer2:date}}

Name, Role

Electron-Ion Collider at Institution

{{Sig_es_:signer3:signature}}	Date: {{Dte_es_:signer3:date}}

Name, Role

Electron-Ion Collider at Institution

{{Sig_es_:signer4:signature}}	Date: {{Dte_es_:signer4:date}}

Name, Role

Electron-Ion Collider at Institution

Approved by:

{{Sig_es_:signer5:signature}}	Date: {{Dte_es_:signer5:date}}

Name, Role

Electron-Ion Collider at Institution

REVISION HISTORY

| Revision # | Effective Date | Additional Reviewers | Summary of Change |
| --- | --- | --- | --- |
| 00 |  |  | Initial release. |

TABLE OF CONTENTS

1.	Purpose and Scope	5

2.	Introduction	5

2.1	Data Acquisition and Computing Systems	5

2.2	Overview of Interface Relationships	5

2.3	Interface Responsibilities	6

2.4	Safety and Environmental Considerations	6

2.5	Design Considerations	6

2.6	Organization	6

3.	Interface summary	6

4.	Data Acquisition and Computing Systems (DET-DS-COMP) to Ancillary Detectors (DET-DS-ANC)	27

5.	Data Acquisition and Computing Systems (DET-DS-COMP) to Electromagnetic Calorimetry Systems (DET-DS-ECAL)	32

6.	Data Acquisition and Computing Systems (DET-DS-COMP) to Electronic Systems (DET-DS-ELEC)	35

7.	Data Acquisition and Computing Systems (DET-DS-COMP) to Hadronic Calorimetry Systems (DET-DS-HCAL)	35

8.	Data Acquisition and Computing Systems (DET-DS-COMP) to Detector Infrastructure (DET-DS-INF)	38

9.	Data Acquisition and Computing Systems (DET-DS-COMP) to Solenoid Magnet (DET-DS-MAG)	42

10.	Data Acquisition and Computing Systems (DET-DS-COMP) to Particle Identification Systems (DET-DS-PID)	44

11.	Data Acquisition and Computing Systems (DET-DS-COMP) to Polarimetry and Luminosity (DET-DS-POL)	47

12.	Data Acquisition and Computing Systems (DET-DS-COMP) to Tracking Systems (DET-DS-TRAK)	54

13.	Applicable Documents	55

14.	Detailed change Log	56

15.	References	56

LIST OF ACRONYMS

| B0 | B0 is the name of the forward magnet just past the Hadron Endcap |  |
| --- | --- | --- |
| CDR | Conceptual Design Report |  |
| COTS | Commercial Off the Shelf |  |
| DAQ | Data Acquisition |  |
| DET | EIC Detector |  |
| DIRC | Detection of Internally Reflected Cherenkov Light |  |
| dRICH | Dual radiator ring imaging Cherenkov |  |
| DVCS | Deeply Virtual Compton Scattering |  |
| EIC | Electron-Ion Collider |  |
| EMCAL | Electro-magnetic Calorimeter |  |
| ESR | Electron Storage Ring |  |
| FCC | Federal Communications Commission |  |
| FDR | Final Design Review |  |
| GEM | Gas Electron Multiplier |  |
| HCAL | Hadronic Calorimeter |  |
| HSR | Hadron Storage Ring |  |
| HVAC | Heating/Ventilation/Air Conditioning |  |
| IP | Interaction Point |  |
| IR | Interaction Region |  |
| MPGD | Micro Pattern Gaseous Detectors |  |
| NAS | National Academy of Sciences |  |
| NEC | National Electric Code |  |
| NEMA | National Electrical Manufacturers Association |  |
| NFPA | National Fire Protection Association |  |
| NRTL | Nationally Recognized Testing Laboratory |  |
| PID | Particle Identification |  |
| PRR | Production Readiness Review |  |
| PWO | PbWO4 |  |
| RCS | Rapid Cycling Synchrotron |  |
| TBD | To Be Determined |  |
| UL | Underwriter Laboratories |  |
| WBS | Work Break Down Structure |  |

# Interface Control Document:

Data Acquisition and Computing Systems (6.03.01.08) to Detector System (6.03.01)

# Purpose and Scope

This document records the agreed upon interfaces between the Data Acquisition and Computing Systems and other sub-systems within the Detector System. These interfaces are based on the system requirements that are necessary to integrate these sub-systems as part of the Electron Ion Collider Project.

# Introduction

These interfaces describe the relationships between the Data Acquisition and Computing Systems and the Detector System including (examples by inclusion) located in (areas by inclusion).

## Data Acquisition and Computing Systems

Provide an introductory description of your sub-system and an overview of the things that it receives, provides, supports, constrains, or otherwise requires as part of its function --- specifically if these things are manifested as interfaces.

Ancillary Detectors (DET-DS-ANC): Description of the TYPES of interfaces that exist between the ICD reference system and this sub-system, and a BRIEF overview of the roles they play.

Electromagnetic Calorimetry Systems (DET-DS-ECAL): Description of the TYPES of interfaces that exist between the ICD reference system and this sub-system, and a BRIEF overview of the roles they play.

Electronic Systems (DET-DS-ELEC): Description of the TYPES of interfaces that exist between the ICD reference system and this sub-system, and a BRIEF overview of the roles they play.

Hadronic Calorimetry Systems (DET-DS-HCAL): Description of the TYPES of interfaces that exist between the ICD reference system and this sub-system, and a BRIEF overview of the roles they play.

Detector Infrastructure (DET-DS-INF): Description of the TYPES of interfaces that exist between the ICD reference system and this sub-system, and a BRIEF overview of the roles they play.

Solenoid Magnet (DET-DS-MAG): Description of the TYPES of interfaces that exist between the ICD reference system and this sub-system, and a BRIEF overview of the roles they play.

Particle Identification Systems (DET-DS-PID): Description of the TYPES of interfaces that exist between the ICD reference system and this sub-system, and a BRIEF overview of the roles they play.

Polarimetry and Luminosity (DET-DS-POL): Description of the TYPES of interfaces that exist between the ICD reference system and this sub-system, and a BRIEF overview of the roles they play.

Tracking Systems (DET-DS-TRAK): Description of the TYPES of interfaces that exist between the ICD reference system and this sub-system, and a BRIEF overview of the roles they play.

## Overview of Interface Relationships

The following are the sub-systems within the Detector System that have interfaces connected to the Data Acquisition and Computing Systems.

## Interface Responsibilities

The various ownerships of responsibilities may be listed here. However, this section might also refer to the Provider section of the interface detail if all pertinent responsibilities are listed there. If this is the case, simply enter: “See the Provider section within the interface details for information about the responsibilities associated with individual interfaces.”

## Safety and Environmental Considerations

General safety and environmental considerations are identified here. Any considerations that are specific to the individual sub-system, will be addressed in external safety documents and are referenced here.

### Considerations for all systems

Add general safety and environmental considerations here.

### Special considerations and references

Add references here as necessary.

## Design Considerations

Notes here about general design considerations.

## Organization

The interfaces in this document are grouped by sub-system as identified in the sub-system identifier. All interfaces between the Data Acquisition and Computing Systems and a related sub-system will be included in a single section. Entries within that section will be sorted by the interface identifier or WBS number, as appropriate.

# Interface summary

This tables in this section provide a list of all interfaces that are defined within this document. The interfaces are grouped by sub-system/WBS. Each table has the following fields:

Section: the section number in this document where the interface is addressed.

WBS (1): the WBS number within the ICD reference system, unless the reference system is the owner.

WBS (2): the WBS number for the connected system.

ID: the interface identifier as defined in the Interface Requirement Document (IRD).

Name: the name of the interface as defined in the IRD.

Description: the description of the interface as defined in the IRD.

Owner: identifies the WBS number of the entity that is responsible for ensuring the interface is provided and that it meets validation/verification requirements.

Data Acquisition and Computing Systems to Ancillary Detectors

| Section | WBS1 | WBS2 | ID | Name | Description | Owner |
| --- | --- | --- | --- | --- | --- | --- |
| 4 |  | 6.03.01.10 |  |  | Ancillary Detectors |  |
| 4.1 | 6.03.01.08 | 6.03.01.10 | I-DET-DS-COMP-ONLINE.002 | Data Transfer and Control Interface | A fiber connection will be provided from the DAQ system to the B0 detector's readout board to perform configuration, control, and data acquisition. | 6.03.01.08.02 |
| 4.2 | 6.03.01.08 | 6.03.01.10 | I-DET-DS-COMP-ONLINE.003 | Slow Controls | A network connection will be provided from the DAQ system to the B0 detector's slow controls interface. | 6.03.01.08.02 |
| 4.3 | 6.03.01.08 | 6.03.01.10 | I-DET-DS-COMP-ONLINE.004 | Timing Interface | A fiber connection will be provided from the DAQ system to the B0 detector's readout board for timing synchronization. | 6.03.01.08.02 |
| 4.4 | 6.03.01.08 | 6.03.01.10 | I-DET-DS-COMP-ONLINE.005 | Data Transfer and Control Interface | A fiber connection will be provided from the DAQ system to the Low Q2 detector's readout board to perform configuration, control, and data acquisition. | 6.03.01.08.02 |
| 4.5 | 6.03.01.08 | 6.03.01.10 | I-DET-DS-COMP-ONLINE.006 | Slow Controls | A network connection will be provided from the DAQ system to the Low Q2 detector's slow controls interface. | 6.03.01.08.02 |
| 4.6 | 6.03.01.08 | 6.03.01.10 | I-DET-DS-COMP-ONLINE.007 | Timing Interface | A fiber connection will be provided from the DAQ system to the Low Q2 detector's readout board for timing synchronization. | 6.03.01.08.02 |
| 4.7 | 6.03.01.08 | 6.03.01.10 | I-DET-DS-COMP-ONLINE.008 | Data Transfer and Control Interface | Fiber connection from the DAQ system to the detector's RDO to perform configuration, control, and data acquisition. | 6.03.01.08.02 |
| 4.8 | 6.03.01.08 | 6.03.01.10 | I-DET-DS-COMP-ONLINE.009 | Slow Controls | Network connection from the DAQ system to the detector's slow controls interface. | 6.03.01.08.02 |
| 4.9 | 6.03.01.08 | 6.03.01.10 | I-DET-DS-COMP-ONLINE.010 | Timing Interface | Fiber connection from the DAQ system to the detector's RDO used for timing synchronization. | 6.03.01.08.02 |
| 4.10 | 6.03.01.08 | 6.03.01.10 | I-DET-DS-COMP-ONLINE.011 | Data Transfer and Control Interface | A fiber connection will be provided from the DAQ system to the Off-Momentum detector's readout board to perform configuration, control, and data acquisition. | 6.03.01.08.02 |
| 4.11 | 6.03.01.08 | 6.03.01.10 | I-DET-DS-COMP-ONLINE.012 | Slow Controls | A network connection will be provided from the DAQ system to the Off-Momentum slow controls interface. | 6.03.01.08.02 |
| 4.12 | 6.03.01.08 | 6.03.01.10 | I-DET-DS-COMP-ONLINE.013 | Timing Interface | A fiber connection will be provided from the DAQ system to the Off-Momentum detector's readout board for timing synchronization. | 6.03.01.08.02 |
| 4.13 | 6.03.01.08 | 6.03.01.10 | I-DET-DS-COMP-ONLINE.014 | Data Transfer and Control Interface | A fiber connection will be provided from the DAQ system to the Roman Pot's readout board to perform configuration, control, and data acquisition. | 6.03.01.08.02 |
| 4.14 | 6.03.01.08 | 6.03.01.10 | I-DET-DS-COMP-ONLINE.015 | Slow Controls | Network connection from the DAQ system to the detector's slow controls interface. | 6.03.01.08.02 |
| 4.15 | 6.03.01.08 | 6.03.01.10 | I-DET-DS-COMP-ONLINE.016 | Timing Interface | A fiber connection will be provided from the DAQ system to the Roman Pot's readout board for timing synchronization. | 6.03.01.08.02 |
| 4.16 | 6.03.01.08 | 6.03.01.10 | I-DET-DS-COMP-ONLINE.017 | Data Transfer and Control Interface | A fiber connection will be provided from the DAQ system to the Zero-Degree Calorimeter's readout board to perform configuration, control, and data acquisition. | 6.03.01.08.02 |
| 4.17 | 6.03.01.08 | 6.03.01.10 | I-DET-DS-COMP-ONLINE.018 | Slow Controls | A network connection will be provided from the DAQ system to the Zero-Degree Calorimeter's slow controls interface. | 6.03.01.08.02 |
| 4.18 | 6.03.01.08 | 6.03.01.10 | I-DET-DS-COMP-ONLINE.019 | Timing Interface | A fiber connection will be provided from the DAQ system to the Zero-Degree Calorimeter's readout board for timing synchronization. | 6.03.01.08.02 |

Data Acquisition and Computing Systems to Electromagnetic Calorimetry Systems

| Section | WBS1 | WBS2 | ID | Name | Description | Owner |
| --- | --- | --- | --- | --- | --- | --- |
| 5 |  | 6.03.01.04 |  |  | Electromagnetic Calorimetry Systems |  |
| 5.1 | 6.03.01.08 | 6.03.01.04 | I-DET-DS-COMP-ONLINE.020 | Data Transfer and Control Interface | A fiber connection will be provided from the DAQ system to the barrel ECAL's readout board to perform configuration, control, and data acquisition. | 6.03.01.08.02 |
| 5.2 | 6.03.01.08 | 6.03.01.04 | I-DET-DS-COMP-ONLINE.021 | Slow Controls | A network connection will be provided from the DAQ system to the barrel ECAL's slow controls interface. | 6.03.01.08.02 |
| 5.3 | 6.03.01.08 | 6.03.01.04 | I-DET-DS-COMP-ONLINE.022 | Timing Interface | A fiber connection will be provided from the DAQ system to the barrel ECAL's readout board for timing synchronization. | 6.03.01.08.02 |
| 5.4 | 6.03.01.08 | 6.03.01.04 | I-DET-DS-COMP-ONLINE.023 | Data Transfer and Control Interface | A fiber connection will be provided from the DAQ system to the backward ECAL's readout board to perform configuration, control, and data acquisition. | 6.03.01.08.02 |
| 5.5 | 6.03.01.08 | 6.03.01.04 | I-DET-DS-COMP-ONLINE.024 | Slow Controls | A network connection will be provided from the DAQ system to the backward ECAL's slow controls interface. | 6.03.01.08.02 |
| 5.6 | 6.03.01.08 | 6.03.01.04 | I-DET-DS-COMP-ONLINE.025 | Timing Interface | A fiber connection will be provided from the DAQ system to the backward ECAL's readout board for timing synchronization. | 6.03.01.08.02 |
| 5.7 | 6.03.01.08 | 6.03.01.04 | I-DET-DS-COMP-ONLINE.026 | Data Transfer and Control Interface | A fiber connection will be provided from the DAQ system to the forward ECAL's readout board to perform configuration, control, and data acquisition. | 6.03.01.08.02 |
| 5.8 | 6.03.01.08 | 6.03.01.04 | I-DET-DS-COMP-ONLINE.027 | Slow Controls | A network connection will be provided from the DAQ system to the forward ECAL's slow controls interface. | 6.03.01.08.02 |
| 5.9 | 6.03.01.08 | 6.03.01.04 | I-DET-DS-COMP-ONLINE.028 | Timing Interface | A fiber connection will be provided from the DAQ system to the forward ECAL's readout board for timing synchronization. | 6.03.01.08.02 |

Data Acquisition and Computing Systems to Electronic Systems

| Section | WBS1 | WBS2 | ID | Name | Description | Owner |
| --- | --- | --- | --- | --- | --- | --- |
| 6 |  | 6.03.01.07 |  |  | Electronic Systems |  |
| 6.1 | 6.03.01.08 | 6.03.01.07 | I-DET-DS-COMP-ONLINE.029 | Monitoring Systems | Monitoring system cables will run from the south platform to the DAQ control room to allow monitoring of the power supplies and the electronics. | 6.03.01.08.02 |
| 6.2 | 6.03.01.08 | 6.03.01.07 | I-DET-DS-COMP-ONLINE.030 | Slow Controls | Controls cables allowing fast shutdown and machine interlocks will run from the south platform to the DAQ control room. | 6.03.01.08.02 |

Data Acquisition and Computing Systems to Hadronic Calorimetry Systems

| Section | WBS1 | WBS2 | ID | Name | Description | Owner |
| --- | --- | --- | --- | --- | --- | --- |
| 7 |  | 6.03.01.05 |  |  | Hadronic Calorimetry Systems |  |
| 7.1 | 6.03.01.08 | 6.03.01.05 | I-DET-DS-COMP-ONLINE.031 | Data Transfer and Control Interface | A fiber connection will be provided from the DAQ system to the barrel HCAL's readout board to perform configuration, control, and data acquisition. | 6.03.01.08.02 |
| 7.2 | 6.03.01.08 | 6.03.01.05 | I-DET-DS-COMP-ONLINE.032 | Slow Controls | A connection from the DAQ system to the barrel HCAL's slow controls interface. CLARIFY with Data acquisition whether a slow control interface will be independent of the regular control interface. | 6.03.01.08.02 |
| 7.3 | 6.03.01.08 | 6.03.01.05 | I-DET-DS-COMP-ONLINE.033 | Timing Interface | A fiber connection will be provided from the DAQ system to the barrel HCAL's readout board for timing synchronization. | 6.03.01.08.02 |
| 7.4 | 6.03.01.08 | 6.03.01.05 | I-DET-DS-COMP-ONLINE.034 | Data Transfer and Control Interface | A fiber connection will be provided from the DAQ system to the backward HCAL's readout board to perform configuration, control, and data acquisition. | 6.03.01.08.02 |
| 7.5 | 6.03.01.08 | 6.03.01.05 | I-DET-DS-COMP-ONLINE.035 | Slow Controls | A connection from the DAQ system to the barrel HCAL's slow controls interface. CLARIFY with Data acquisition whether a slow control interface will be independent of the regular control interface. | 6.03.01.08.02 |
| 7.6 | 6.03.01.08 | 6.03.01.05 | I-DET-DS-COMP-ONLINE.036 | Timing Interface | A fiber connection will be provided from the DAQ system to the backward HCAL's readout board for timing synchronization. | 6.03.01.08.02 |
| 7.7 | 6.03.01.08 | 6.03.01.05 | I-DET-DS-COMP-ONLINE.037 | Data Transfer and Control Interface | A fiber connection will be provided from the DAQ system to the forward HCAL's readout board to perform configuration, control, and data acquisition. | 6.03.01.08.02 |
| 7.8 | 6.03.01.08 | 6.03.01.05 | I-DET-DS-COMP-ONLINE.038 | Slow Controls | A connection from the DAQ system to the forward HCAL's slow controls interface. CLARIFY with Data acquisition whether a slow control interface will be independent of the regular control interface. | 6.03.01.08.02 |
| 7.9 | 6.03.01.08 | 6.03.01.05 | I-DET-DS-COMP-ONLINE.039 | Timing Interface | A fiber connection will be provided from the DAQ system to the forward HCAL's readout board for timing synchronization. | 6.03.01.08.02 |

Data Acquisition and Computing Systems to Detector Infrastructure

| Section | WBS1 | WBS2 | ID | Name | Description | Owner |
| --- | --- | --- | --- | --- | --- | --- |
| 8 |  | 6.03.01.09 |  |  | Detector Infrastructure |  |
| 8.1 | 6.03.01.08 | 6.03.01.09 | I-DET-DS-INF-BAR.013 | Fiber Support System | A fiber support system must be provided that allows the communications cabling to remain connected while the detector is moved from the experimental hall to the assembly area. | 6.03.01.09.01 |
| 8.2 | 6.03.01.08 | 6.03.01.09 | I-DET-DS-INF-INT.047 | Computer Room Cooling | HVAC systems must be provided to maintain stable temperature and humidity in the computer room. | 6.03.01.09.04 |
| 8.3 | 6.03.01.08 | 6.03.01.09 | I-DET-DS-INF-INT.048 | Experimental and Assembly Area Cooling | General HVAC systems must be provided to maintain a normal operating temperature in locations where DAQ systems are in use. | 6.03.01.09.04 |
| 8.4 | 6.03.01.08 | 6.03.01.09 | I-DET-DS-INF-INT.049 | Backup Power | UPS systems must be provided to support critical systems in order to tolerate transient conditions and to allow for controlled shutdown. | 6.03.01.09.04 |
| 8.5 | 6.03.01.08 | 6.03.01.09 | I-DET-DS-INF-INT.050 | Computer Room Power | Adequate power must be provided to support the DAQ systems and computing infrastructure. | 6.03.01.09.04 |
| 8.6 | 6.03.01.08 | 6.03.01.09 | I-DET-DS-INF-INT.051 | Control Room Power | Standard electrical service to support consoles and systems in the control room. | 6.03.01.09.04 |
| 8.7 | 6.03.01.08 | 6.03.01.09 | I-DET-DS-INF-INT.052 | Hall Power | Clean power (either transformer or source isolated) should be provided for all DAQ, computing and network equipment. | 6.03.01.09.04 |
| 8.8 | 6.03.01.08 | 6.03.01.09 | I-DET-DS-INF-INT.053 | Carriage Rack Space | Adequate rack space for the DAQ system will be required on the detector carriages. | 6.03.01.09.04 |
| 8.9 | 6.03.01.08 | 6.03.01.09 | I-DET-DS-INF-INT.054 | Computer Room | Adequate raised-floor space with racks will be required to support DAQ computers, communications systems, and computing infrastructure. | 6.03.01.09.04 |
| 8.10 | 6.03.01.08 | 6.03.01.09 | I-DET-DS-INF-INT.055 | Control Systems Furniture | Space for computing consoles and furniture adequate to manage the system during operation will be required. Consoles and furniture will be provided by DET-INT. | 6.03.01.09.04 |
| 8.11 | 6.03.01.08 | 6.03.01.09 | I-DET-DS-INF-INT.056 | Detector Plenums | Adequate space to route fiberoptic cables from the racks to the detector readout boards will be required. | 6.03.01.09.04 |
| 8.12 | 6.03.01.08 | 6.03.01.09 | I-DET-DS-INF-INT.057 | Fixed Platform Conduits | Adequate conduits and cableways must be provided to deliver cabling to the racks on the fixed platforms. | 6.03.01.09.04 |
| 8.13 | 6.03.01.08 | 6.03.01.09 | I-DET-DS-INF-INT.058 | Platform/Tunnel Rack Space | Adequate rack space for the DAQ system must be provided on the fixed platforms, tunnels, and in the experimental hall. | 6.03.01.09.04 |
| 8.14 | 6.03.01.08 | 6.03.01.09 | I-DET-DS-INF-INT.059 | Sensitive Electronics | DAQ systems that are sensitive to magnetism or radiation must be placed at a sufficient distance from sources OR shielding must be provided to protect the equipment. | 6.03.01.09.04 |
| 8.15 | 6.03.01.08 | 6.03.01.09 | I-DET-DS-INF-INT.060 | Tunnel Conduits | Conduits must be provided in the west and east tunnels to connect the detectors to the DAQ system. | 6.03.01.09.04 |
| 8.16 | 6.03.01.08 | 6.03.01.09 | I-DET-DS-INF-INT.061 | Fiber Support System | A fiber support system must be provided that allows the communications cabling to remain connected while the detector is moved from the experimental hall to the assembly area. | 6.03.01.09.04 |

Data Acquisition and Computing Systems to Solenoid Magnet

| Section | WBS1 | WBS2 | ID | Name | Description | Owner |
| --- | --- | --- | --- | --- | --- | --- |
| 9 |  | 6.03.01.06 |  |  | Solenoid Magnet |  |
| 9.1 | 6.03.01.08 | 6.03.01.06 | I-DET-DS-COMP-ONLINE.040 | Controls and Instrumentation | Control cabling for the magnet instrumentation will be run from the solenoid to the instrumentation rack on the carriage. | 6.03.01.08.02 |
| 9.2 | 6.03.01.08 | 6.03.01.06 | I-DET-DS-COMP-ONLINE.041 | Monitoring Data | Live monitoring data from the solenoid's operation must be recorded and maintained for diagnostic purposes. | 6.03.01.08.02 |
| 9.3 | 6.03.01.08 | 6.03.01.06 | I-DET-DS-COMP-ONLINE.042 | Slow Controls | A network connection will be provided from the DAQ system to the solenoid cryogenic's slow controls interface. | 6.03.01.08.02 |
| 9.4 | 6.03.01.08 | 6.03.01.06 | I-DET-DS-COMP-ONLINE.043 | Slow Controls | A network connection will be provided from the DAQ system to the solenoid's slow controls interface. | 6.03.01.08.02 |
| 9.5 | 6.03.01.08 | 6.03.01.06 | I-DET-DS-COMP-ONLINE.044 | Slow Controls | A network connection will be provided from the DAQ system to the slow controls interface of the solenoid's power supply. | 6.03.01.08.02 |

Data Acquisition and Computing Systems to Particle Identification Systems

| Section | WBS1 | WBS2 | ID | Name | Description | Owner |
| --- | --- | --- | --- | --- | --- | --- |
| 10 |  | 6.03.01.03 |  |  | Particle Identification Systems |  |
| 10.1 | 6.03.01.08 | 6.03.01.03 | I-DET-DS-COMP-ONLINE.045 | Data Transfer and Control Interface | A fiber connection will be provided from the DAQ system to the barrel PID detector's readout board to perform configuration, control, and data acquisition. | 6.03.01.08.02 |
| 10.2 | 6.03.01.08 | 6.03.01.03 | I-DET-DS-COMP-ONLINE.046 | Slow Controls | A network connection will be provided from the DAQ system to the barrel PID detector's slow controls interface. | 6.03.01.08.02 |
| 10.3 | 6.03.01.08 | 6.03.01.03 | I-DET-DS-COMP-ONLINE.047 | Timing Interface | A fiber connection will be provided from the DAQ system to the barrel PID detector's readout board for timing synchronization. | 6.03.01.08.02 |
| 10.4 | 6.03.01.08 | 6.03.01.03 | I-DET-DS-COMP-ONLINE.048 | Data Acquisition | Signal cables will run from the DIRC electronics to the DAQ system. | 6.03.01.08.02 |
| 10.5 | 6.03.01.08 | 6.03.01.03 | I-DET-DS-COMP-ONLINE.049 | Slow Control Interface | Signal cables from environmental sensors will run from the DIRC electronics to the DAQ system to provide detector shutdown/protection system. | 6.03.01.08.02 |
| 10.6 | 6.03.01.08 | 6.03.01.03 | I-DET-DS-COMP-ONLINE.050 | Data Acquisition | Signal cables will run from the DIRC electronics to the DAQ system. | 6.03.01.08.02 |
| 10.7 | 6.03.01.08 | 6.03.01.03 | I-DET-DS-COMP-ONLINE.051 | Slow Control Interface | Signal cables from environmental sensors will run from the DIRC electronics to the DAQ system to provide detector shutdown/protection system. | 6.03.01.08.02 |
| 10.8 | 6.03.01.08 | 6.03.01.03 | I-DET-DS-COMP-ONLINE.052 | Data Acquisition | Signal cables will run from the pfRICH/mRICH electronics to the DAQ system. | 6.03.01.08.02 |
| 10.9 | 6.03.01.08 | 6.03.01.03 | I-DET-DS-COMP-ONLINE.053 | Slow Control Interface | Signal cables from environmental sensors will run from the pfRICH/mRICH electronics to the DAQ system to provide a detector shutdown/protection system. | 6.03.01.08.02 |
| 10.10 | 6.03.01.08 | 6.03.01.03 | I-DET-DS-COMP-ONLINE.054 | Data Acquisition | Signal cables will run from the dRICH electronics to the DAQ system. | 6.03.01.08.02 |
| 10.11 | 6.03.01.08 | 6.03.01.03 | I-DET-DS-COMP-ONLINE.055 | Slow Control Interface | Signal cables from environmental sensors will run from the cold photosensors to the DAQ system to provide a detector shutdown/protection system. | 6.03.01.08.02 |
| 10.12 | 6.03.01.08 | 6.03.01.03 | I-DET-DS-PID-BAR-DIRC.001 | Light Pulser System | The light pulser system will require synchronization using a clock that will be provided by data acquisition. | 6.03.01.03 |
| 10.13 | 6.03.01.08 | 6.03.01.03 | I-DET-DS-PID-BCK-RICH.001 | Light Pulser System | The light pulser system will require synchronization using a clock that will be provided by data acquisition. | 6.03.01.03 |
| 10.14 | 6.03.01.08 | 6.03.01.03 | I-DET-DS-PID-FWD-RICH.001 | Light Pulser System | The light pulser system will require synchronization using a clock that will be provided by data acquisition. | 6.03.01.03 |

Data Acquisition and Computing Systems to Polarimetry and Luminosity

| Section | WBS1 | WBS2 | ID | Name | Description | Owner |
| --- | --- | --- | --- | --- | --- | --- |
| 11 |  | 6.03.01.10 |  |  | Polarimetry and Luminosity |  |
| 11.1 | 6.03.01.08 | 6.03.01.10 | I-DET-DS-COMP-ONLINE.056 | Data Transfer and Control Interface | A fiber connection will be provided from the DAQ system to the ESR Polarimeter's readout board to perform configuration, control, and data acquisition. | 6.03.01.08.02 |
| 11.2 | 6.03.01.08 | 6.03.01.10 | I-DET-DS-COMP-ONLINE.057 | Electron Detector Data | Data transfer lines will be run from the electron detector to a DAQ system. | 6.03.01.08.02 |
| 11.3 | 6.03.01.08 | 6.03.01.10 | I-DET-DS-COMP-ONLINE.058 | Laser Data | Data transfer lines will be run from the laser enclosure on the beamline to a DAQ system. | 6.03.01.08.02 |
| 11.4 | 6.03.01.08 | 6.03.01.10 | I-DET-DS-COMP-ONLINE.059 | Photon Detector Data | Data transfer lines will be run from the photon detector to a DAQ system. | 6.03.01.08.02 |
| 11.5 | 6.03.01.08 | 6.03.01.10 | I-DET-DS-COMP-ONLINE.060 | Slow Controls | A network connection will be provided from the DAQ system to the ESR Polarimeter's slow controls interface. | 6.03.01.08.02 |
| 11.6 | 6.03.01.08 | 6.03.01.10 | I-DET-DS-COMP-ONLINE.061 | Timing Interface | A fiber connection will be provided from the DAQ system to the ESR Polarimeter's readout board for timing synchronization. | 6.03.01.08.02 |
| 11.7 | 6.03.01.08 | 6.03.01.10 | I-DET-DS-COMP-ONLINE.062 | Data Transfer and Control Interface | A fiber connection will be provided from the DAQ system to the RCS polarimeter's readout board to perform configuration, control, and data acquisition. | 6.03.01.08.02 |
| 11.8 | 6.03.01.08 | 6.03.01.10 | I-DET-DS-COMP-ONLINE.063 | Laser Data | Data transfer lines will be run from the laser enclosure on the beamline to a DAQ system. | 6.03.01.08.02 |
| 11.9 | 6.03.01.08 | 6.03.01.10 | I-DET-DS-COMP-ONLINE.064 | Photon Detector Data | Data transfer lines will be run from the photon detector to a DAQ system. | 6.03.01.08.02 |
| 11.10 | 6.03.01.08 | 6.03.01.10 | I-DET-DS-COMP-ONLINE.065 | Slow Controls | A network connection will be provided from the DAQ system to the RCS polarimeter's slow controls interface. | 6.03.01.08.02 |
| 11.11 | 6.03.01.08 | 6.03.01.10 | I-DET-DS-COMP-ONLINE.066 | Timing Interface | A fiber connection will be provided from the DAQ system to the RCS polarimeter's readout board for timing synchronization. | 6.03.01.08.02 |
| 11.12 | 6.03.01.08 | 6.03.01.10 | I-DET-DS-COMP-ONLINE.067 | Readout cables IR-4 | Readout cables need to run from the polarimeter detectors to the data acquisition system at IR-4. | 6.03.01.08.02 |
| 11.13 | 6.03.01.08 | 6.03.01.10 | I-DET-DS-COMP-ONLINE.068 | Slow Control Database IR-4 | Slow control information should be written to the HSR database (5 second intervals) | 6.03.01.08.02 |
| 11.14 | 6.03.01.08 | 6.03.01.10 | I-DET-DS-COMP-ONLINE.069 | Data Transfer and Control Interface | A fiber connection will be provided from the DAQ system to the HJET polarimeter's readout board to perform configuration, control, and data acquisition. | 6.03.01.08.02 |
| 11.15 | 6.03.01.08 | 6.03.01.10 | I-DET-DS-COMP-ONLINE.070 | Slow Control Cables IR-4 | Slow control cables for the HJET target system need to run from the hall at IR-4 to the polarimeter in the tunnel. | 6.03.01.08.02 |
| 11.16 | 6.03.01.08 | 6.03.01.10 | I-DET-DS-COMP-ONLINE.071 | Slow Controls | A network connection will be provided from the DAQ system to the HJET polarimeter's slow controls interface. | 6.03.01.08.02 |
| 11.17 | 6.03.01.08 | 6.03.01.10 | I-DET-DS-COMP-ONLINE.072 | Timing Interface | A fiber connection will be provided from the DAQ system to the HJET polarimeter's readout board for timing synchronization. | 6.03.01.08.02 |
| 11.18 | 6.03.01.08 | 6.03.01.10 | I-DET-DS-COMP-ONLINE.073 | Data Transfer and Control Interface | A fiber connection will be provided from the DAQ system to the proton-carbon polarimeter's readout board to perform configuration, control, and data acquisition. | 6.03.01.08.02 |
| 11.19 | 6.03.01.08 | 6.03.01.10 | I-DET-DS-COMP-ONLINE.074 | Slow Control Database IR-6 | Results from the target scan polarimeter runs should be written to the HSR database | 6.03.01.08.02 |
| 11.20 | 6.03.01.08 | 6.03.01.10 | I-DET-DS-COMP-ONLINE.075 | Slow Controls | A network connection will be provided from the DAQ system to the proton-carbon polarimeter's slow controls interface. | 6.03.01.08.02 |
| 11.21 | 6.03.01.08 | 6.03.01.10 | I-DET-DS-COMP-ONLINE.076 | Timing Interface | A fiber connection will be provided from the DAQ system to the proton-carbon polarimeter's readout board for timing synchronization. | 6.03.01.08.02 |
| 11.22 | 6.03.01.08 | 6.03.01.10 | I-DET-DS-COMP-ONLINE.077 | Slow Control IR-4 | Slow control cables for the target motion need to run from the hall at IR-4 to the polarimeter. | 6.03.01.08.02 |
| 11.23 | 6.03.01.08 | 6.03.01.10 | I-DET-DS-COMP-ONLINE.078 | Target Operation IR-4 | Fiber target operation should be controlled from the HSR main control room | 6.03.01.08.02 |
| 11.24 | 6.03.01.08 | 6.03.01.10 | I-DET-DS-COMP-ONLINE.079 | Slow Control IR-6 | Slow control cables for the target motion need to run from the DAQ room at IR-6 to the polarimeter. | 6.03.01.08.02 |
| 11.25 | 6.03.01.08 | 6.03.01.10 | I-DET-DS-COMP-ONLINE.080 | Target Operation IR-6 | Fiber target operation should be controlled from the EPIC control room | 6.03.01.08.02 |
| 11.26 | 6.03.01.08 | 6.03.01.10 | I-DET-DS-POL-HPOL-PC-IR6.002 | Data Acquisition IR-6 | The data acquisition for the local polarimeter shall be integrated into the experimental readout. | 6.03.01.11.02.02 |

Data Acquisition and Computing Systems to Tracking Systems

| Section | WBS1 | WBS2 | ID | Name | Description | Owner |
| --- | --- | --- | --- | --- | --- | --- |
| 12 |  | 6.03.01.02 |  |  | Tracking Systems |  |
| 12.1 | 6.03.01.08 | 6.03.01.02 | I-DET-DS-COMP-ONLINE.081 | Data Acquisition | Cables required to transfer data from the detector to the online data acquisition system. | 6.03.01.08.02 |
| 12.2 | 6.03.01.08 | 6.03.01.02 | I-DET-DS-COMP-ONLINE.082 | Data Transfer and Control Interface | A fiber connection will be provided from the DAQ system to the barrel tracking system's readout board to perform configuration, control, and data acquisition. | 6.03.01.08.02 |
| 12.3 | 6.03.01.08 | 6.03.01.02 | I-DET-DS-COMP-ONLINE.083 | Slow Controls | A network connection will be provided from the DAQ system to the barrel tracking system's slow controls interface. | 6.03.01.08.02 |
| 12.4 | 6.03.01.08 | 6.03.01.02 | I-DET-DS-COMP-ONLINE.084 | Timing Interface | A fiber connection will be provided from the DAQ system to the barrel tracking system's readout board for timing synchronization. | 6.03.01.08.02 |

# Data Acquisition and Computing Systems (DET-DS-COMP) to Ancillary Detectors (DET-DS-ANC)

I-DET-DS-COMP-ONLINE.002	Data Transfer and Control Interface

System 1:	Online DAQ and Computing Systems (6.03.01.08.02)

System 2:	B-Zero Detectors (6.03.01.10)

Type:	DATA

Description:	A fiber connection will be provided from the DAQ system to the B0 detector's readout board to perform configuration, control, and data acquisition.

Provider:	Online DAQ and Computing Systems (6.03.01.08.02)

Details:	See details in the Data Acquisition and Computing Systems Interface Control Document.

Requirements:	F-DET-DS-COMP.6

References:

I-DET-DS-COMP-ONLINE.003	Slow Controls

System 1:	Online DAQ and Computing Systems (6.03.01.08.02)

System 2:	B-Zero Detectors (6.03.01.10)

Type:	CONTROL

Description:	A network connection will be provided from the DAQ system to the B0 detector's slow controls interface.

Provider:	Online DAQ and Computing Systems (6.03.01.08.02)

Details:	See details in the Data Acquisition and Computing Systems Interface Control Document.

Requirements:	F-DET-DS-COMP-SC.1

References:

I-DET-DS-COMP-ONLINE.004	Timing Interface

System 1:	Online DAQ and Computing Systems (6.03.01.08.02)

System 2:	B-Zero Detectors (6.03.01.10)

Type:	DATA

Description:	A fiber connection will be provided from the DAQ system to the B0 detector's readout board for timing synchronization.

Provider:	Online DAQ and Computing Systems (6.03.01.08.02)

Details:	See details in the Data Acquisition and Computing Systems Interface Control Document.

Requirements:	F-DET-DS-COMP.11

References:

I-DET-DS-COMP-ONLINE.005	Data Transfer and Control Interface

System 1:	Online DAQ and Computing Systems (6.03.01.08.02)

System 2:	Low Q2 Detectors (6.03.01.10)

Type:	DATA

Description:	A fiber connection will be provided from the DAQ system to the Low Q2 detector's readout board to perform configuration, control, and data acquisition.

Provider:	Online DAQ and Computing Systems (6.03.01.08.02)

Details:	See details in the Data Acquisition and Computing Systems Interface Control Document.

Requirements:	F-DET-DS-COMP.6

References:

I-DET-DS-COMP-ONLINE.006	Slow Controls

System 1:	Online DAQ and Computing Systems (6.03.01.08.02)

System 2:	Low Q2 Detectors (6.03.01.10)

Type:	CONTROL

Description:	A network connection will be provided from the DAQ system to the Low Q2 detector's slow controls interface.

Provider:	Online DAQ and Computing Systems (6.03.01.08.02)

Details:	See details in the Data Acquisition and Computing Systems Interface Control Document.

Requirements:	F-DET-DS-COMP-SC.1

References:

I-DET-DS-COMP-ONLINE.007	Timing Interface

System 1:	Online DAQ and Computing Systems (6.03.01.08.02)

System 2:	Low Q2 Detectors (6.03.01.10)

Type:	DATA

Description:	A fiber connection will be provided from the DAQ system to the Low Q2 detector's readout board for timing synchronization.

Provider:	Online DAQ and Computing Systems (6.03.01.08.02)

Details:	See details in the Data Acquisition and Computing Systems Interface Control Document.

Requirements:	F-DET-DS-COMP.11

References:

I-DET-DS-COMP-ONLINE.008	Data Transfer and Control Interface

System 1:	Online DAQ and Computing Systems (6.03.01.08.02)

System 2:	Luminosity Detectors (6.03.01.10)

Type:	DATA

Description:	Fiber connection from the DAQ system to the detector's RDO to perform configuration, control, and data acquisition.

Provider:	Online DAQ and Computing Systems (6.03.01.08.02)

Details:	See details in the Data Acquisition and Computing Systems Interface Control Document.

Requirements:	F-DET-DS-COMP.6

References:

I-DET-DS-COMP-ONLINE.009	Slow Controls

System 1:	Online DAQ and Computing Systems (6.03.01.08.02)

System 2:	Luminosity Detectors (6.03.01.10)

Type:	CONTROL

Description:	Network connection from the DAQ system to the detector's slow controls interface.

Provider:	Online DAQ and Computing Systems (6.03.01.08.02)

Details:	See details in the Data Acquisition and Computing Systems Interface Control Document.

Requirements:	F-DET-DS-COMP-SC.1

References:

I-DET-DS-COMP-ONLINE.010	Timing Interface

System 1:	Online DAQ and Computing Systems (6.03.01.08.02)

System 2:	Luminosity Detectors (6.03.01.10)

Type:	DATA

Description:	Fiber connection from the DAQ system to the detector's RDO used for timing synchronization.

Provider:	Online DAQ and Computing Systems (6.03.01.08.02)

Details:	See details in the Data Acquisition and Computing Systems Interface Control Document.

Requirements:	F-DET-DS-COMP.11

References:

I-DET-DS-COMP-ONLINE.011	Data Transfer and Control Interface

System 1:	Online DAQ and Computing Systems (6.03.01.08.02)

System 2:	Off-Momentum Detectors (6.03.01.10)

Type:	DATA

Description:	A fiber connection will be provided from the DAQ system to the Off-Momentum detector's readout board to perform configuration, control, and data acquisition.

Provider:	Online DAQ and Computing Systems (6.03.01.08.02)

Details:	See details in the Data Acquisition and Computing Systems Interface Control Document.

Requirements:	F-DET-DS-COMP.6

References:

I-DET-DS-COMP-ONLINE.012	Slow Controls

System 1:	Online DAQ and Computing Systems (6.03.01.08.02)

System 2:	Off-Momentum Detectors (6.03.01.10)

Type:	CONTROL

Description:	A network connection will be provided from the DAQ system to the Off-Momentum slow controls interface.

Provider:	Online DAQ and Computing Systems (6.03.01.08.02)

Details:	See details in the Data Acquisition and Computing Systems Interface Control Document.

Requirements:	F-DET-DS-COMP-SC.1

References:

I-DET-DS-COMP-ONLINE.013	Timing Interface

System 1:	Online DAQ and Computing Systems (6.03.01.08.02)

System 2:	Off-Momentum Detectors (6.03.01.10)

Type:	DATA

Description:	A fiber connection will be provided from the DAQ system to the Off-Momentum detector's readout board for timing synchronization.

Provider:	Online DAQ and Computing Systems (6.03.01.08.02)

Details:	See details in the Data Acquisition and Computing Systems Interface Control Document.

Requirements:	F-DET-DS-COMP.11

References:

I-DET-DS-COMP-ONLINE.014	Data Transfer and Control Interface

System 1:	Online DAQ and Computing Systems (6.03.01.08.02)

System 2:	Roman Pots (6.03.01.10)

Type:	DATA

Description:	A fiber connection will be provided from the DAQ system to the Roman Pot's readout board to perform configuration, control, and data acquisition.

Provider:	Online DAQ and Computing Systems (6.03.01.08.02)

Details:	See details in the Data Acquisition and Computing Systems Interface Control Document.

Requirements:	F-DET-DS-COMP.6

References:

I-DET-DS-COMP-ONLINE.015	Slow Controls

System 1:	Online DAQ and Computing Systems (6.03.01.08.02)

System 2:	Roman Pots (6.03.01.10)

Type:	CONTROL

Description:	Network connection from the DAQ system to the detector's slow controls interface.

Provider:	Online DAQ and Computing Systems (6.03.01.08.02)

Details:	See details in the Data Acquisition and Computing Systems Interface Control Document.

Requirements:	F-DET-DS-COMP-SC.1

References:

I-DET-DS-COMP-ONLINE.016	Timing Interface

System 1:	Online DAQ and Computing Systems (6.03.01.08.02)

System 2:	Roman Pots (6.03.01.10)

Type:	DATA

Description:	A fiber connection will be provided from the DAQ system to the Roman Pot's readout board for timing synchronization.

Provider:	Online DAQ and Computing Systems (6.03.01.08.02)

Details:	See details in the Data Acquisition and Computing Systems Interface Control Document.

Requirements:	F-DET-DS-COMP.11

References:

I-DET-DS-COMP-ONLINE.017	Data Transfer and Control Interface

System 1:	Online DAQ and Computing Systems (6.03.01.08.02)

System 2:	Zero Degree Calorimeter (6.03.01.10)

Type:	DATA

Description:	A fiber connection will be provided from the DAQ system to the Zero-Degree Calorimeter's readout board to perform configuration, control, and data acquisition.

Provider:	Online DAQ and Computing Systems (6.03.01.08.02)

Details:	See details in the Data Acquisition and Computing Systems Interface Control Document.

Requirements:	F-DET-DS-COMP.6

References:

I-DET-DS-COMP-ONLINE.018	Slow Controls

System 1:	Online DAQ and Computing Systems (6.03.01.08.02)

System 2:	Zero Degree Calorimeter (6.03.01.10)

Type:	CONTROL

Description:	A network connection will be provided from the DAQ system to the Zero-Degree Calorimeter's slow controls interface.

Provider:	Online DAQ and Computing Systems (6.03.01.08.02)

Details:	See details in the Data Acquisition and Computing Systems Interface Control Document.

Requirements:	F-DET-DS-COMP-SC.1

References:

I-DET-DS-COMP-ONLINE.019	Timing Interface

System 1:	Online DAQ and Computing Systems (6.03.01.08.02)

System 2:	Zero Degree Calorimeter (6.03.01.10)

Type:	DATA

Description:	A fiber connection will be provided from the DAQ system to the Zero-Degree Calorimeter's readout board for timing synchronization.

Provider:	Online DAQ and Computing Systems (6.03.01.08.02)

Details:	See details in the Data Acquisition and Computing Systems Interface Control Document.

Requirements:	F-DET-DS-COMP.11

References:

# Data Acquisition and Computing Systems (DET-DS-COMP) to Electromagnetic Calorimetry Systems (DET-DS-ECAL)

I-DET-DS-COMP-ONLINE.020	Data Transfer and Control Interface

System 1:	Online DAQ and Computing Systems (6.03.01.08.02)

System 2:	Barrel EMCal Systems (6.03.01.04)

Type:	DATA

Description:	A fiber connection will be provided from the DAQ system to the barrel ECAL's readout board to perform configuration, control, and data acquisition.

Provider:	Online DAQ and Computing Systems (6.03.01.08.02)

Details:	See details in the Data Acquisition and Computing Systems Interface Control Document.

Requirements:	F-DET-DS-COMP.6

References:

I-DET-DS-COMP-ONLINE.021	Slow Controls

System 1:	Online DAQ and Computing Systems (6.03.01.08.02)

System 2:	Barrel EMCal Systems (6.03.01.04)

Type:	CONTROL

Description:	A network connection will be provided from the DAQ system to the barrel ECAL's slow controls interface.

Provider:	Online DAQ and Computing Systems (6.03.01.08.02)

Details:	See details in the Data Acquisition and Computing Systems Interface Control Document.

Requirements:	F-DET-DS-COMP-SC.1

References:

I-DET-DS-COMP-ONLINE.022	Timing Interface

System 1:	Online DAQ and Computing Systems (6.03.01.08.02)

System 2:	Barrel EMCal Systems (6.03.01.04)

Type:	DATA

Description:	A fiber connection will be provided from the DAQ system to the barrel ECAL's readout board for timing synchronization.

Provider:	Online DAQ and Computing Systems (6.03.01.08.02)

Details:	See details in the Data Acquisition and Computing Systems Interface Control Document.

Requirements:	F-DET-DS-COMP.11

References:

I-DET-DS-COMP-ONLINE.023	Data Transfer and Control Interface

System 1:	Online DAQ and Computing Systems (6.03.01.08.02)

System 2:	Backward EMCal Systems (6.03.01.04)

Type:	DATA

Description:	A fiber connection will be provided from the DAQ system to the backward ECAL's readout board to perform configuration, control, and data acquisition.

Provider:	Online DAQ and Computing Systems (6.03.01.08.02)

Details:	See details in the Data Acquisition and Computing Systems Interface Control Document.

Requirements:	F-DET-DS-COMP.6

References:

I-DET-DS-COMP-ONLINE.024	Slow Controls

System 1:	Online DAQ and Computing Systems (6.03.01.08.02)

System 2:	Backward EMCal Systems (6.03.01.04)

Type:	CONTROL

Description:	A network connection will be provided from the DAQ system to the backward ECAL's slow controls interface.

Provider:	Online DAQ and Computing Systems (6.03.01.08.02)

Details:	See details in the Data Acquisition and Computing Systems Interface Control Document.

Requirements:	F-DET-DS-COMP-SC.1

References:

I-DET-DS-COMP-ONLINE.025	Timing Interface

System 1:	Online DAQ and Computing Systems (6.03.01.08.02)

System 2:	Backward EMCal Systems (6.03.01.04)

Type:	DATA

Description:	A fiber connection will be provided from the DAQ system to the backward ECAL's readout board for timing synchronization.

Provider:	Online DAQ and Computing Systems (6.03.01.08.02)

Details:	See details in the Data Acquisition and Computing Systems Interface Control Document.

Requirements:	F-DET-DS-COMP.11

References:

I-DET-DS-COMP-ONLINE.026	Data Transfer and Control Interface

System 1:	Online DAQ and Computing Systems (6.03.01.08.02)

System 2:	Forward EMCal Systems (6.03.01.04)

Type:	DATA

Description:	A fiber connection will be provided from the DAQ system to the forward ECAL's readout board to perform configuration, control, and data acquisition.

Provider:	Online DAQ and Computing Systems (6.03.01.08.02)

Details:	See details in the Data Acquisition and Computing Systems Interface Control Document.

Requirements:	F-DET-DS-COMP.6

References:

I-DET-DS-COMP-ONLINE.027	Slow Controls

System 1:	Online DAQ and Computing Systems (6.03.01.08.02)

System 2:	Forward EMCal Systems (6.03.01.04)

Type:	CONTROL

Description:	A network connection will be provided from the DAQ system to the forward ECAL's slow controls interface.

Provider:	Online DAQ and Computing Systems (6.03.01.08.02)

Details:	See details in the Data Acquisition and Computing Systems Interface Control Document.

Requirements:	F-DET-DS-COMP-SC.1

References:

I-DET-DS-COMP-ONLINE.028	Timing Interface

System 1:	Online DAQ and Computing Systems (6.03.01.08.02)

System 2:	Forward EMCal Systems (6.03.01.04)

Type:	DATA

Description:	A fiber connection will be provided from the DAQ system to the forward ECAL's readout board for timing synchronization.

Provider:	Online DAQ and Computing Systems (6.03.01.08.02)

Details:	See details in the Data Acquisition and Computing Systems Interface Control Document.

Requirements:	F-DET-DS-COMP.11

References:

# Data Acquisition and Computing Systems (DET-DS-COMP) to Electronic Systems (DET-DS-ELEC)

I-DET-DS-COMP-ONLINE.029	Monitoring Systems

System 1:	Online DAQ and Computing Systems (6.03.01.08.02)

System 2:	Electronic Systems (6.03.01.07)

Type:	CONTROL

Description:	Monitoring system cables will run from the south platform to the DAQ control room to allow monitoring of the power supplies and the electronics.

Provider:	Online DAQ and Computing Systems (6.03.01.08.02)

Details:	See details in the Data Acquisition and Computing Systems Interface Control Document.

Requirements:	F-DET-DS-COMP.6

References:

I-DET-DS-COMP-ONLINE.030	Slow Controls

System 1:	Online DAQ and Computing Systems (6.03.01.08.02)

System 2:	Electronic Systems (6.03.01.07)

Type:	CONTROL

Description:	Controls cables allowing fast shutdown and machine interlocks will run from the south platform to the DAQ control room.

Provider:	Online DAQ and Computing Systems (6.03.01.08.02)

Details:	See details in the Data Acquisition and Computing Systems Interface Control Document.

Requirements:	F-DET-DS-COMP-SC.1

References:

# Data Acquisition and Computing Systems (DET-DS-COMP) to Hadronic Calorimetry Systems (DET-DS-HCAL)

I-DET-DS-COMP-ONLINE.031	Data Transfer and Control Interface

System 1:	Online DAQ and Computing Systems (6.03.01.08.02)

System 2:	Barrel HCal Systems (6.03.01.05)

Type:	DATA

Description:	A fiber connection will be provided from the DAQ system to the barrel  HCAL's readout board to perform configuration, control, and data acquisition.

Provider:	Online DAQ and Computing Systems (6.03.01.08.02)

Details:	See details in the Data Acquisition and Computing Systems Interface Control Document.

Requirements:	F-DET-DS-COMP.6

References:

I-DET-DS-COMP-ONLINE.032	Slow Controls

System 1:	Online DAQ and Computing Systems (6.03.01.08.02)

System 2:	Barrel HCal Systems (6.03.01.05)

Type:	CONTROL

Description:	A connection from the DAQ system to the barrel HCAL's slow controls interface. CLARIFY with Data acquisition whether a slow control interface will be independent of the regular control interface.

Provider:	Online DAQ and Computing Systems (6.03.01.08.02)

Details:	See details in the Data Acquisition and Computing Systems Interface Control Document.

Requirements:	F-DET-DS-COMP.6

References:

I-DET-DS-COMP-ONLINE.033	Timing Interface

System 1:	Online DAQ and Computing Systems (6.03.01.08.02)

System 2:	Barrel HCal Systems (6.03.01.05)

Type:	DATA

Description:	A fiber connection will be provided from the DAQ system to the barrel  HCAL's readout board for timing synchronization.

Provider:	Online DAQ and Computing Systems (6.03.01.08.02)

Details:	See details in the Data Acquisition and Computing Systems Interface Control Document.

Requirements:	F-DET-DS-COMP.11

References:

I-DET-DS-COMP-ONLINE.034	Data Transfer and Control Interface

System 1:	Online DAQ and Computing Systems (6.03.01.08.02)

System 2:	Backward HCal Systems (6.03.01.05)

Type:	DATA

Description:	A fiber connection will be provided from the DAQ system to the backward HCAL's readout board to perform configuration, control, and data acquisition.

Provider:	Online DAQ and Computing Systems (6.03.01.08.02)

Details:	See details in the Data Acquisition and Computing Systems Interface Control Document.

Requirements:	F-DET-DS-COMP.6

References:

I-DET-DS-COMP-ONLINE.035	Slow Controls

System 1:	Online DAQ and Computing Systems (6.03.01.08.02)

System 2:	Backward HCal Systems (6.03.01.05)

Type:	CONTROL

Description:	A connection from the DAQ system to the barrel HCAL's slow controls interface. CLARIFY with Data acquisition whether a slow control interface will be independent of the regular control interface.

Provider:	Online DAQ and Computing Systems (6.03.01.08.02)

Details:	See details in the Data Acquisition and Computing Systems Interface Control Document.

Requirements:	F-DET-DS-COMP-SC.1

References:

I-DET-DS-COMP-ONLINE.036	Timing Interface

System 1:	Online DAQ and Computing Systems (6.03.01.08.02)

System 2:	Backward HCal Systems (6.03.01.05)

Type:	DATA

Description:	A fiber connection will be provided from the DAQ system to the backward HCAL's readout board for timing synchronization.

Provider:	Online DAQ and Computing Systems (6.03.01.08.02)

Details:	See details in the Data Acquisition and Computing Systems Interface Control Document.

Requirements:	F-DET-DS-COMP.11

References:

I-DET-DS-COMP-ONLINE.037	Data Transfer and Control Interface

System 1:	Online DAQ and Computing Systems (6.03.01.08.02)

System 2:	Forward HCal Systems (6.03.01.05)

Type:	DATA

Description:	A fiber connection will be provided from the DAQ system to the forward HCAL's readout board to perform configuration, control, and data acquisition.

Provider:	Online DAQ and Computing Systems (6.03.01.08.02)

Details:	See details in the Data Acquisition and Computing Systems Interface Control Document.

Requirements:	F-DET-DS-COMP.6

References:

I-DET-DS-COMP-ONLINE.038	Slow Controls

System 1:	Online DAQ and Computing Systems (6.03.01.08.02)

System 2:	Forward HCal Systems (6.03.01.05)

Type:	CONTROL

Description:	A connection from the DAQ system to the forward HCAL's slow controls interface. CLARIFY with Data acquisition whether a slow control interface will be independent of the regular control interface.

Provider:	Online DAQ and Computing Systems (6.03.01.08.02)

Details:	See details in the Data Acquisition and Computing Systems Interface Control Document.

Requirements:	F-DET-DS-COMP-SC.1

References:

I-DET-DS-COMP-ONLINE.039	Timing Interface

System 1:	Online DAQ and Computing Systems (6.03.01.08.02)

System 2:	Forward HCal Systems (6.03.01.05)

Type:	DATA

Description:	A fiber connection will be provided from the DAQ system to the forward HCAL's readout board for timing synchronization.

Provider:	Online DAQ and Computing Systems (6.03.01.08.02)

Details:	See details in the Data Acquisition and Computing Systems Interface Control Document.

Requirements:	F-DET-DS-COMP.11

References:

# Data Acquisition and Computing Systems (DET-DS-COMP) to Detector Infrastructure (DET-DS-INF)

I-DET-DS-INF-BAR.013	Fiber Support System

System 1:	Barrel Detector Structures (6.03.01.09.01)

System 2:	Online DAQ and Computing Systems (6.03.01.08.02)

Type:	STRUCT

Description:	A fiber support system must be provided that allows the communications cabling to remain connected while the detector is moved from the experimental hall to the assembly area.

Provider:	Barrel Detector Structures (6.03.01.09.01)

Details:	See details in the Detector Infrastructure Interface Control Document.

Requirements:	P-DET-DS-COMP.2

References:

I-DET-DS-INF-INT.047	Computer Room Cooling

System 1:	Detector Infrastructure and Utilities Integration (6.03.01.09.04)

System 2:	Online DAQ and Computing Systems (6.03.01.08.02)

Type:	COOL

Description:	HVAC systems must be provided to maintain stable temperature and humidity in the computer room.

Provider:	Detector Infrastructure and Utilities Integration (6.03.01.09.04)

Details:	See details in the Detector Infrastructure Interface Control Document.

Requirements:	P-DET-DS-COMP.3

References:

I-DET-DS-INF-INT.048	Experimental and Assembly Area Cooling

System 1:	Detector Infrastructure and Utilities Integration (6.03.01.09.04)

System 2:	Online DAQ and Computing Systems (6.03.01.08.02)

Type:	COOL

Description:	General HVAC systems must be provided to maintain a normal operating temperature in locations where DAQ systems are in use.

Provider:	Detector Infrastructure and Utilities Integration (6.03.01.09.04)

Details:	See details in the Detector Infrastructure Interface Control Document.

Requirements:	P-DET-DS-COMP.3

References:

I-DET-DS-INF-INT.049	Backup Power

System 1:	Detector Infrastructure and Utilities Integration (6.03.01.09.04)

System 2:	Online DAQ and Computing Systems (6.03.01.08.02)

Type:	ELEC

Description:	UPS systems must be provided to support critical systems in order to tolerate transient conditions and to allow for controlled shutdown.

Provider:	Detector Infrastructure and Utilities Integration (6.03.01.09.04)

Details:	See details in the Detector Infrastructure Interface Control Document.

Requirements:	P-DET-DS-COMP.4

References:

I-DET-DS-INF-INT.050	Computer Room Power

System 1:	Detector Infrastructure and Utilities Integration (6.03.01.09.04)

System 2:	Online DAQ and Computing Systems (6.03.01.08.02)

Type:	ELEC

Description:	Adequate power must be provided to support the DAQ systems and computing infrastructure.

Provider:	Detector Infrastructure and Utilities Integration (6.03.01.09.04)

Details:	See details in the Detector Infrastructure Interface Control Document.

Requirements:	P-DET-DS-COMP.4

References:

I-DET-DS-INF-INT.051	Control Room Power

System 1:	Detector Infrastructure and Utilities Integration (6.03.01.09.04)

System 2:	Online DAQ and Computing Systems (6.03.01.08.02)

Type:	ELEC

Description:	Standard electrical service to support consoles and systems in the control room.

Provider:	Detector Infrastructure and Utilities Integration (6.03.01.09.04)

Details:	See details in the Detector Infrastructure Interface Control Document.

Requirements:	P-DET-DS-COMP.4

References:

I-DET-DS-INF-INT.052	Hall Power

System 1:	Detector Infrastructure and Utilities Integration (6.03.01.09.04)

System 2:	Online DAQ and Computing Systems (6.03.01.08.02)

Type:	ELEC

Description:	Clean power (either transformer or source isolated) should be provided for all DAQ, computing and network equipment.

Provider:	Detector Infrastructure and Utilities Integration (6.03.01.09.04)

Details:	See details in the Detector Infrastructure Interface Control Document.

Requirements:	P-DET-DS-COMP.6

References:

I-DET-DS-INF-INT.053	Carriage Rack Space

System 1:	Detector Infrastructure and Utilities Integration (6.03.01.09.04)

System 2:	Online DAQ and Computing Systems (6.03.01.08.02)

Type:	SPACE

Description:	Adequate rack space for the DAQ system will be required on the detector carriages.

Provider:	Detector Infrastructure and Utilities Integration (6.03.01.09.04)

Details:	See details in the Detector Infrastructure Interface Control Document.

Requirements:	P-DET-DS-COMP.2

References:

I-DET-DS-INF-INT.054	Computer Room

System 1:	Detector Infrastructure and Utilities Integration (6.03.01.09.04)

System 2:	Online DAQ and Computing Systems (6.03.01.08.02)

Type:	SPACE

Description:	Adequate raised-floor space with racks will be required  to support DAQ computers, communications systems, and computing infrastructure.

Provider:	Detector Infrastructure and Utilities Integration (6.03.01.09.04)

Details:	See details in the Detector Infrastructure Interface Control Document.

Requirements:	P-DET-DS-COMP.9

References:

I-DET-DS-INF-INT.055	Control Systems Furniture

System 1:	Detector Infrastructure and Utilities Integration (6.03.01.09.04)

System 2:	Online DAQ and Computing Systems (6.03.01.08.02)

Type:	SPACE

Description:	Space for computing consoles and furniture adequate to manage the system during operation will be required. Consoles and furniture will be provided by DET-INT.

Provider:	Detector Infrastructure and Utilities Integration (6.03.01.09.04)

Details:	See details in the Detector Infrastructure Interface Control Document.

Requirements:	P-DET-DS-COMP.2

References:

I-DET-DS-INF-INT.056	Detector Plenums

System 1:	Detector Infrastructure and Utilities Integration (6.03.01.09.04)

System 2:	Online DAQ and Computing Systems (6.03.01.08.02)

Type:	SPACE

Description:	Adequate space to route fiberoptic cables from the racks to the detector readout boards will be required.

Provider:	Detector Infrastructure and Utilities Integration (6.03.01.09.04)

Details:	See details in the Detector Infrastructure Interface Control Document.

Requirements:	P-DET-DS-COMP.2

References:

I-DET-DS-INF-INT.057	Fixed Platform Conduits

System 1:	Detector Infrastructure and Utilities Integration (6.03.01.09.04)

System 2:	Online DAQ and Computing Systems (6.03.01.08.02)

Type:	SPACE

Description:	Adequate conduits and cableways must be provided to deliver cabling to the racks on the fixed platforms.

Provider:	Detector Infrastructure and Utilities Integration (6.03.01.09.04)

Details:	See details in the Detector Infrastructure Interface Control Document.

Requirements:	P-DET-DS-COMP.2

References:

I-DET-DS-INF-INT.058	Platform/Tunnel Rack Space

System 1:	Detector Infrastructure and Utilities Integration (6.03.01.09.04)

System 2:	Online DAQ and Computing Systems (6.03.01.08.02)

Type:	SPACE

Description:	Adequate rack space for the DAQ system must be provided on the fixed platforms, tunnels, and in the experimental hall.

Provider:	Detector Infrastructure and Utilities Integration (6.03.01.09.04)

Details:	See details in the Detector Infrastructure Interface Control Document.

Requirements:	P-DET-DS-COMP.2

References:

I-DET-DS-INF-INT.059	Sensitive Electronics

System 1:	Detector Infrastructure and Utilities Integration (6.03.01.09.04)

System 2:	Online DAQ and Computing Systems (6.03.01.08.02)

Type:	SPACE

Description:	DAQ systems that are sensitive to magnetism or radiation must be placed at a sufficient distance from sources OR shielding must be provided to protect the equipment.

Provider:	Detector Infrastructure and Utilities Integration (6.03.01.09.04)

Details:	See details in the Detector Infrastructure Interface Control Document.

Requirements:	P-DET-DS-COMP.14

References:

I-DET-DS-INF-INT.060	Tunnel Conduits

System 1:	Detector Infrastructure and Utilities Integration (6.03.01.09.04)

System 2:	Online DAQ and Computing Systems (6.03.01.08.02)

Type:	SPACE

Description:	Conduits must be provided in the west and east tunnels to connect the detectors to the DAQ system.

Provider:	Detector Infrastructure and Utilities Integration (6.03.01.09.04)

Details:	See details in the Detector Infrastructure Interface Control Document.

Requirements:	P-DET-DS-COMP.8

References:

I-DET-DS-INF-INT.061	Fiber Support System

System 1:	Detector Infrastructure and Utilities Integration (6.03.01.09.04)

System 2:	Online DAQ and Computing Systems (6.03.01.08.02)

Type:	STRUCT

Description:	A fiber support system must be provided that allows the communications cabling to remain connected while the detector is moved from the experimental hall to the assembly area.

Provider:	Detector Infrastructure and Utilities Integration (6.03.01.09.04)

Details:	See details in the Detector Infrastructure Interface Control Document.

Requirements:	P-DET-DS-COMP.8

References:

# Data Acquisition and Computing Systems (DET-DS-COMP) to Solenoid Magnet (DET-DS-MAG)

I-DET-DS-COMP-ONLINE.040	Controls and Instrumentation

System 1:	Online DAQ and Computing Systems (6.03.01.08.02)

System 2:	Solenoid Magnet (6.03.01.06)

Type:	CONTROL

Description:	Control cabling for the magnet instrumentation will be run from the solenoid to the instrumentation rack on the carriage.

Provider:	Online DAQ and Computing Systems (6.03.01.08.02)

Details:	See details in the Data Acquisition and Computing Systems Interface Control Document.

Requirements:	F-DET-DS-COMP.6

References:

I-DET-DS-COMP-ONLINE.041	Monitoring Data

System 1:	Online DAQ and Computing Systems (6.03.01.08.02)

System 2:	Solenoid Magnet (6.03.01.06)

Type:	DATA

Description:	Live monitoring data from the solenoid's operation must be recorded and maintained for diagnostic purposes.

Provider:	Online DAQ and Computing Systems (6.03.01.08.02)

Details:	See details in the Data Acquisition and Computing Systems Interface Control Document.

Requirements:	F-DET-DS-COMP.6

References:

I-DET-DS-COMP-ONLINE.042	Slow Controls

System 1:	Online DAQ and Computing Systems (6.03.01.08.02)

System 2:	Magnet Cryogenics (6.03.01.06)

Type:	CONTROL

Description:	A network connection will be provided from the DAQ system to the solenoid cryogenic's slow controls interface.

Provider:	Online DAQ and Computing Systems (6.03.01.08.02)

Details:	See details in the Data Acquisition and Computing Systems Interface Control Document.

Requirements:	F-DET-DS-COMP-SC.1

References:

I-DET-DS-COMP-ONLINE.043	Slow Controls

System 1:	Online DAQ and Computing Systems (6.03.01.08.02)

System 2:	Magnet Instrumentation and Control (6.03.01.06)

Type:	CONTROL

Description:	A network connection will be provided from the DAQ system to the solenoid's slow controls interface.

Provider:	Online DAQ and Computing Systems (6.03.01.08.02)

Details:	See details in the Data Acquisition and Computing Systems Interface Control Document.

Requirements:	F-DET-DS-COMP.6

References:

I-DET-DS-COMP-ONLINE.044	Slow Controls

System 1:	Online DAQ and Computing Systems (6.03.01.08.02)

System 2:	Magnet Power Supply (6.03.01.06)

Type:	CONTROL

Description:	A network connection will be provided from the DAQ system to the slow controls interface of the solenoid's  power supply.

Provider:	Online DAQ and Computing Systems (6.03.01.08.02)

Details:	See details in the Data Acquisition and Computing Systems Interface Control Document.

Requirements:	F-DET-DS-COMP.6

References:

# Data Acquisition and Computing Systems (DET-DS-COMP) to Particle Identification Systems (DET-DS-PID)

I-DET-DS-COMP-ONLINE.045	Data Transfer and Control Interface

System 1:	Online DAQ and Computing Systems (6.03.01.08.02)

System 2:	Barrel Particle ID Systems (6.03.01.03)

Type:	DATA

Description:	A fiber connection will be provided from the DAQ system to the barrel PID detector's readout board to perform configuration, control, and data acquisition.

Provider:	Online DAQ and Computing Systems (6.03.01.08.02)

Details:	See details in the Data Acquisition and Computing Systems Interface Control Document.

Requirements:	F-DET-DS-COMP.6

References:

I-DET-DS-COMP-ONLINE.046	Slow Controls

System 1:	Online DAQ and Computing Systems (6.03.01.08.02)

System 2:	Barrel Particle ID Systems (6.03.01.03)

Type:	CONTROL

Description:	A network connection will be provided from the DAQ system to the barrel PID detector's slow controls interface.

Provider:	Online DAQ and Computing Systems (6.03.01.08.02)

Details:	See details in the Data Acquisition and Computing Systems Interface Control Document.

Requirements:	F-DET-DS-COMP-SC.1

References:

I-DET-DS-COMP-ONLINE.047	Timing Interface

System 1:	Online DAQ and Computing Systems (6.03.01.08.02)

System 2:	Barrel Particle ID Systems (6.03.01.03)

Type:	DATA

Description:	A fiber connection will be provided from the DAQ system to the barrel PID detector's readout board for timing synchronization.

Provider:	Online DAQ and Computing Systems (6.03.01.08.02)

Details:	See details in the Data Acquisition and Computing Systems Interface Control Document.

Requirements:	F-DET-DS-COMP.11

References:

I-DET-DS-COMP-ONLINE.048	Data Acquisition

System 1:	Online DAQ and Computing Systems (6.03.01.08.02)

System 2:	Barrel DIRC Particle ID Systems (6.03.01.03)

Type:	DATA

Description:	Signal cables will run from the DIRC electronics to the DAQ system.

Provider:	Online DAQ and Computing Systems (6.03.01.08.02)

Details:	See details in the Data Acquisition and Computing Systems Interface Control Document.

Requirements:	F-DET-DS-COMP.6

References:

I-DET-DS-COMP-ONLINE.049	Slow Control Interface

System 1:	Online DAQ and Computing Systems (6.03.01.08.02)

System 2:	Barrel DIRC Particle ID Systems (6.03.01.03)

Type:	CONTROL

Description:	Signal cables from environmental sensors will run from the DIRC electronics to the DAQ system to provide detector  shutdown/protection system.

Provider:	Online DAQ and Computing Systems (6.03.01.08.02)

Details:	See details in the Data Acquisition and Computing Systems Interface Control Document.

Requirements:	F-DET-DS-COMP-SC.1

References:

I-DET-DS-COMP-ONLINE.050	Data Acquisition

System 1:	Online DAQ and Computing Systems (6.03.01.08.02)

System 2:	Barrel TOF Particle ID Systems (6.03.01.03)

Type:	DATA

Description:	Signal cables will run from the DIRC electronics to the DAQ system.

Provider:	Online DAQ and Computing Systems (6.03.01.08.02)

Details:	See details in the Data Acquisition and Computing Systems Interface Control Document.

Requirements:	F-DET-DS-COMP.6

References:

I-DET-DS-COMP-ONLINE.051	Slow Control Interface

System 1:	Online DAQ and Computing Systems (6.03.01.08.02)

System 2:	Barrel TOF Particle ID Systems (6.03.01.03)

Type:	CONTROL

Description:	Signal cables from environmental sensors will run from the DIRC electronics to the DAQ system to provide detector  shutdown/protection system.

Provider:	Online DAQ and Computing Systems (6.03.01.08.02)

Details:	See details in the Data Acquisition and Computing Systems Interface Control Document.

Requirements:	F-DET-DS-COMP.6

References:

I-DET-DS-COMP-ONLINE.052	Data Acquisition

System 1:	Online DAQ and Computing Systems (6.03.01.08.02)

System 2:	Backward Particle ID Systems (6.03.01.03)

Type:	DATA

Description:	Signal cables will run from the pfRICH/mRICH electronics to the DAQ system.

Provider:	Online DAQ and Computing Systems (6.03.01.08.02)

Details:	See details in the Data Acquisition and Computing Systems Interface Control Document.

Requirements:	F-DET-DS-COMP.6

References:

I-DET-DS-COMP-ONLINE.053	Slow Control Interface

System 1:	Online DAQ and Computing Systems (6.03.01.08.02)

System 2:	Backward Particle ID Systems (6.03.01.03)

Type:	CONTROL

Description:	Signal cables from environmental sensors will run from the pfRICH/mRICH electronics to the DAQ system to provide a detector  shutdown/protection system.

Provider:	Online DAQ and Computing Systems (6.03.01.08.02)

Details:	See details in the Data Acquisition and Computing Systems Interface Control Document.

Requirements:	F-DET-DS-COMP.6

References:

I-DET-DS-COMP-ONLINE.054	Data Acquisition

System 1:	Online DAQ and Computing Systems (6.03.01.08.02)

System 2:	Forward Particle ID Systems (6.03.01.03)

Type:	DATA

Description:	Signal cables will run from the dRICH electronics to the DAQ system.

Provider:	Online DAQ and Computing Systems (6.03.01.08.02)

Details:	See details in the Data Acquisition and Computing Systems Interface Control Document.

Requirements:	F-DET-DS-COMP.6

References:

I-DET-DS-COMP-ONLINE.055	Slow Control Interface

System 1:	Online DAQ and Computing Systems (6.03.01.08.02)

System 2:	Forward Particle ID Systems (6.03.01.03)

Type:	CONTROL

Description:	Signal cables from environmental sensors will run from the cold photosensors to the DAQ system to provide a detector  shutdown/protection system.

Provider:	Online DAQ and Computing Systems (6.03.01.08.02)

Details:	See details in the Data Acquisition and Computing Systems Interface Control Document.

Requirements:	F-DET-DS-COMP-SC.1

References:

I-DET-DS-PID-BAR-DIRC.001	Light Pulser System

System 1:	Barrel DIRC Particle ID Systems (6.03.01.03)

System 2:	Online DAQ and Computing Systems (6.03.01.08.02)

Type:	CONTROL

Description:	The light pulser system will require synchronization using a clock that will be provided by data acquisition.

Provider:	Barrel DIRC Particle ID Systems (6.03.01.03)

Details:	See details in the Particle Identification Systems Interface Control Document.

Requirements:	P-DET-DS-PID-BAR-DIRC.4

References:

I-DET-DS-PID-BCK-RICH.001	Light Pulser System

System 1:	Backward RICH Detector Systems (6.03.01.03)

System 2:	Online DAQ and Computing Systems (6.03.01.08.02)

Type:	CONTROL

Description:	The light pulser system will require synchronization using a clock that will be provided by data acquisition.

Provider:	Backward RICH Detector Systems (6.03.01.03)

Details:	See details in the Particle Identification Systems Interface Control Document.

Requirements:	P-DET-DS-PID-BCK-RICH.4

References:

I-DET-DS-PID-FWD-RICH.001	Light Pulser System

System 1:	Forward RICH Detector Systems (6.03.01.03)

System 2:	Online DAQ and Computing Systems (6.03.01.08.02)

Type:	CONTROL

Description:	The light pulser system will require synchronization using a clock that will be provided by data acquisition.

Provider:	Forward RICH Detector Systems (6.03.01.03)

Details:	See details in the Particle Identification Systems Interface Control Document.

Requirements:	P-DET-DS-PID-FWD-RICH.4

References:

# Data Acquisition and Computing Systems (DET-DS-COMP) to Polarimetry and Luminosity (DET-DS-POL)

I-DET-DS-COMP-ONLINE.056	Data Transfer and Control Interface

System 1:	Online DAQ and Computing Systems (6.03.01.08.02)

System 2:	Electron Storage Ring Polarimetry (6.03.01.11.01.01)

Type:	DATA

Description:	A fiber connection will be provided from the DAQ system to the ESR Polarimeter's readout board to perform configuration, control, and data acquisition.

Provider:	Online DAQ and Computing Systems (6.03.01.08.02)

Details:	See details in the Data Acquisition and Computing Systems Interface Control Document.

Requirements:	F-DET-DS-COMP.6

References:

I-DET-DS-COMP-ONLINE.057	Electron Detector Data

System 1:	Online DAQ and Computing Systems (6.03.01.08.02)

System 2:	Electron Storage Ring Polarimetry (6.03.01.11.01.01)

Type:	DATA

Description:	Data transfer lines will be run from the electron detector  to a DAQ system.

Provider:	Online DAQ and Computing Systems (6.03.01.08.02)

Details:	See details in the Data Acquisition and Computing Systems Interface Control Document.

Requirements:	F-DET-DS-COMP.6

References:

I-DET-DS-COMP-ONLINE.058	Laser Data

System 1:	Online DAQ and Computing Systems (6.03.01.08.02)

System 2:	Electron Storage Ring Polarimetry (6.03.01.11.01.01)

Type:	DATA

Description:	Data transfer lines will be run from the laser enclosure on the beamline to a DAQ system.

Provider:	Online DAQ and Computing Systems (6.03.01.08.02)

Details:	See details in the Data Acquisition and Computing Systems Interface Control Document.

Requirements:	F-DET-DS-COMP.6

References:

I-DET-DS-COMP-ONLINE.059	Photon Detector Data

System 1:	Online DAQ and Computing Systems (6.03.01.08.02)

System 2:	Electron Storage Ring Polarimetry (6.03.01.11.01.01)

Type:	DATA

Description:	Data transfer lines will be run from the photon detector  to a DAQ system.

Provider:	Online DAQ and Computing Systems (6.03.01.08.02)

Details:	See details in the Data Acquisition and Computing Systems Interface Control Document.

Requirements:	F-DET-DS-COMP.6

References:

I-DET-DS-COMP-ONLINE.060	Slow Controls

System 1:	Online DAQ and Computing Systems (6.03.01.08.02)

System 2:	Electron Storage Ring Polarimetry (6.03.01.11.01.01)

Type:	CONTROL

Description:	A network connection will be provided from the DAQ system to the ESR Polarimeter's slow controls interface.

Provider:	Online DAQ and Computing Systems (6.03.01.08.02)

Details:	See details in the Data Acquisition and Computing Systems Interface Control Document.

Requirements:	F-DET-DS-COMP-SC.1

References:

I-DET-DS-COMP-ONLINE.061	Timing Interface

System 1:	Online DAQ and Computing Systems (6.03.01.08.02)

System 2:	Electron Storage Ring Polarimetry (6.03.01.11.01.01)

Type:	DATA

Description:	A fiber connection will be provided from the DAQ system to the ESR Polarimeter's readout board for timing synchronization.

Provider:	Online DAQ and Computing Systems (6.03.01.08.02)

Details:	See details in the Data Acquisition and Computing Systems Interface Control Document.

Requirements:	F-DET-DS-COMP.11

References:

I-DET-DS-COMP-ONLINE.062	Data Transfer and Control Interface

System 1:	Online DAQ and Computing Systems (6.03.01.08.02)

System 2:	Rapid Cycling Synchrotron Polarimetry (6.03.01.11.01.02)

Type:	DATA

Description:	A fiber connection will be provided from the DAQ system to the RCS polarimeter's readout board to perform configuration, control, and data acquisition.

Provider:	Online DAQ and Computing Systems (6.03.01.08.02)

Details:	See details in the Data Acquisition and Computing Systems Interface Control Document.

Requirements:	F-DET-DS-COMP.6

References:

I-DET-DS-COMP-ONLINE.063	Laser Data

System 1:	Online DAQ and Computing Systems (6.03.01.08.02)

System 2:	Rapid Cycling Synchrotron Polarimetry (6.03.01.11.01.02)

Type:	DATA

Description:	Data transfer lines will be run from the laser enclosure on the beamline to a DAQ system.

Provider:	Online DAQ and Computing Systems (6.03.01.08.02)

Details:	See details in the Data Acquisition and Computing Systems Interface Control Document.

Requirements:	F-DET-DS-COMP.6

References:

I-DET-DS-COMP-ONLINE.064	Photon Detector Data

System 1:	Online DAQ and Computing Systems (6.03.01.08.02)

System 2:	Rapid Cycling Synchrotron Polarimetry (6.03.01.11.01.02)

Type:	DATA

Description:	Data transfer lines will be run from the photon detector  to a DAQ system.

Provider:	Online DAQ and Computing Systems (6.03.01.08.02)

Details:	See details in the Data Acquisition and Computing Systems Interface Control Document.

Requirements:	F-DET-DS-COMP.6

References:

I-DET-DS-COMP-ONLINE.065	Slow Controls

System 1:	Online DAQ and Computing Systems (6.03.01.08.02)

System 2:	Rapid Cycling Synchrotron Polarimetry (6.03.01.11.01.02)

Type:	CONTROL

Description:	A network connection will be provided from the DAQ system to the RCS polarimeter's slow controls interface.

Provider:	Online DAQ and Computing Systems (6.03.01.08.02)

Details:	See details in the Data Acquisition and Computing Systems Interface Control Document.

Requirements:	F-DET-DS-COMP-SC.1

References:

I-DET-DS-COMP-ONLINE.066	Timing Interface

System 1:	Online DAQ and Computing Systems (6.03.01.08.02)

System 2:	Rapid Cycling Synchrotron Polarimetry (6.03.01.11.01.02)

Type:	DATA

Description:	A fiber connection will be provided from the DAQ system to the RCS polarimeter's readout board for timing synchronization.

Provider:	Online DAQ and Computing Systems (6.03.01.08.02)

Details:	See details in the Data Acquisition and Computing Systems Interface Control Document.

Requirements:	F-DET-DS-COMP.11

References:

I-DET-DS-COMP-ONLINE.067	Readout cables IR-4

System 1:	Online DAQ and Computing Systems (6.03.01.08.02)

System 2:	Hadron Polarimetry (6.03.01.11.02)

Type:	DATA

Description:	Readout cables need to run from the polarimeter detectors to the data acquisition system at IR-4.

Provider:	Online DAQ and Computing Systems (6.03.01.08.02)

Details:	See details in the Data Acquisition and Computing Systems Interface Control Document.

Requirements:	F-DET-DS-COMP.6

References:

I-DET-DS-COMP-ONLINE.068	Slow Control Database IR-4

System 1:	Online DAQ and Computing Systems (6.03.01.08.02)

System 2:	Hadron Polarimetry (6.03.01.11.02)

Type:	DATA

Description:	Slow control information should be written to the HSR database (5 second intervals)

Provider:	Online DAQ and Computing Systems (6.03.01.08.02)

Details:	See details in the Data Acquisition and Computing Systems Interface Control Document.

Requirements:	F-DET-DS-COMP-SC.1

References:

I-DET-DS-COMP-ONLINE.069	Data Transfer and Control Interface

System 1:	Online DAQ and Computing Systems (6.03.01.08.02)

System 2:	HJET Polarimetery (6.03.01.11.02.01)

Type:	DATA

Description:	A fiber connection will be provided from the DAQ system to the HJET polarimeter's readout board to perform configuration, control, and data acquisition.

Provider:	Online DAQ and Computing Systems (6.03.01.08.02)

Details:	See details in the Data Acquisition and Computing Systems Interface Control Document.

Requirements:	F-DET-DS-COMP.6

References:

I-DET-DS-COMP-ONLINE.070	Slow Control Cables IR-4

System 1:	Online DAQ and Computing Systems (6.03.01.08.02)

System 2:	HJET Polarimetery (6.03.01.11.02.01)

Type:	DATA

Description:	Slow control cables for the HJET target system need to run from the hall at IR-4 to the polarimeter in the tunnel.

Provider:	Online DAQ and Computing Systems (6.03.01.08.02)

Details:	See details in the Data Acquisition and Computing Systems Interface Control Document.

Requirements:	F-DET-DS-COMP-SC.1

References:

I-DET-DS-COMP-ONLINE.071	Slow Controls

System 1:	Online DAQ and Computing Systems (6.03.01.08.02)

System 2:	HJET Polarimetery (6.03.01.11.02.01)

Type:	CONTROL

Description:	A network connection will be provided from the DAQ system to the HJET polarimeter's slow controls interface.

Provider:	Online DAQ and Computing Systems (6.03.01.08.02)

Details:	See details in the Data Acquisition and Computing Systems Interface Control Document.

Requirements:	F-DET-DS-COMP-SC.1

References:

I-DET-DS-COMP-ONLINE.072	Timing Interface

System 1:	Online DAQ and Computing Systems (6.03.01.08.02)

System 2:	HJET Polarimetery (6.03.01.11.02.01)

Type:	DATA

Description:	A fiber connection will be provided from the DAQ system to the HJET polarimeter's readout board for timing synchronization.

Provider:	Online DAQ and Computing Systems (6.03.01.08.02)

Details:	See details in the Data Acquisition and Computing Systems Interface Control Document.

Requirements:	F-DET-DS-COMP.11

References:

I-DET-DS-COMP-ONLINE.073	Data Transfer and Control Interface

System 1:	Online DAQ and Computing Systems (6.03.01.08.02)

System 2:	Proton-Carbon Polarimeter (6.03.01.11.02.02)

Type:	DATA

Description:	A fiber connection will be provided from the DAQ system to the proton-carbon polarimeter's readout board to perform configuration, control, and data acquisition.

Provider:	Online DAQ and Computing Systems (6.03.01.08.02)

Details:	See details in the Data Acquisition and Computing Systems Interface Control Document.

Requirements:	F-DET-DS-COMP.6

References:

I-DET-DS-COMP-ONLINE.074	Slow Control Database IR-6

System 1:	Online DAQ and Computing Systems (6.03.01.08.02)

System 2:	Proton-Carbon Polarimeter (6.03.01.11.02.02)

Type:	DATA

Description:	Results from the target scan polarimeter runs should be written to the HSR database

Provider:	Online DAQ and Computing Systems (6.03.01.08.02)

Details:	See details in the Data Acquisition and Computing Systems Interface Control Document.

Requirements:	F-DET-DS-COMP-SC.1

References:

I-DET-DS-COMP-ONLINE.075	Slow Controls

System 1:	Online DAQ and Computing Systems (6.03.01.08.02)

System 2:	Proton-Carbon Polarimeter (6.03.01.11.02.02)

Type:	CONTROL

Description:	A network connection will be provided from the DAQ system to the proton-carbon polarimeter's slow controls interface.

Provider:	Online DAQ and Computing Systems (6.03.01.08.02)

Details:	See details in the Data Acquisition and Computing Systems Interface Control Document.

Requirements:	F-DET-DS-COMP-SC.1

References:

I-DET-DS-COMP-ONLINE.076	Timing Interface

System 1:	Online DAQ and Computing Systems (6.03.01.08.02)

System 2:	Proton-Carbon Polarimeter (6.03.01.11.02.02)

Type:	DATA

Description:	A fiber connection will be provided from the DAQ system to the proton-carbon polarimeter's readout board for timing synchronization.

Provider:	Online DAQ and Computing Systems (6.03.01.08.02)

Details:	See details in the Data Acquisition and Computing Systems Interface Control Document.

Requirements:	F-DET-DS-COMP.11

References:

I-DET-DS-COMP-ONLINE.077	Slow Control IR-4

System 1:	Online DAQ and Computing Systems (6.03.01.08.02)

System 2:	Proton-Carbon Polarimeter in IR 4 (6.03.01.11.02.02)

Type:	CONTROL

Description:	Slow control cables for the target motion need to run from the hall at IR-4 to the polarimeter.

Provider:	Online DAQ and Computing Systems (6.03.01.08.02)

Details:	See details in the Data Acquisition and Computing Systems Interface Control Document.

Requirements:	F-DET-DS-COMP-SC.1

References:

I-DET-DS-COMP-ONLINE.078	Target Operation IR-4

System 1:	Online DAQ and Computing Systems (6.03.01.08.02)

System 2:	Proton-Carbon Polarimeter in IR 4 (6.03.01.11.02.02)

Type:	CONTROL

Description:	Fiber target operation should be controlled from the HSR main control room

Provider:	Online DAQ and Computing Systems (6.03.01.08.02)

Details:	See details in the Data Acquisition and Computing Systems Interface Control Document.

Requirements:	F-DET-DS-COMP.6

References:

I-DET-DS-COMP-ONLINE.079	Slow Control IR-6

System 1:	Online DAQ and Computing Systems (6.03.01.08.02)

System 2:	Proton-Carbon Polarimeter in IR 6 (6.03.01.11.02.02)

Type:	CONTROL

Description:	Slow control cables for the target motion need to run from the DAQ room at IR-6 to the polarimeter.

Provider:	Online DAQ and Computing Systems (6.03.01.08.02)

Details:	See details in the Data Acquisition and Computing Systems Interface Control Document.

Requirements:	F-DET-DS-COMP-SC.1

References:

I-DET-DS-COMP-ONLINE.080	Target Operation IR-6

System 1:	Online DAQ and Computing Systems (6.03.01.08.02)

System 2:	Proton-Carbon Polarimeter in IR 6 (6.03.01.11.02.02)

Type:	CONTROL

Description:	Fiber target operation should be controlled from the EPIC control room

Provider:	Online DAQ and Computing Systems (6.03.01.08.02)

Details:	See details in the Data Acquisition and Computing Systems Interface Control Document.

Requirements:	F-DET-DS-COMP.6

References:

I-DET-DS-POL-HPOL-PC-IR6.002	Data Acquisition IR-6

System 1:	Proton-Carbon Polarimeter in IR 6 (6.03.01.11.02.02)

System 2:	Online DAQ and Computing Systems (6.03.01.08.02)

Type:	SPACE

Description:	The data acquisition for the local polarimeter shall be integrated into the experimental readout.

Provider:	Proton-Carbon Polarimeter in IR 6 (6.03.01.11.02.02)

Details:	See details in the Polarimetry and Luminosity Interface Control Document.

Requirements:	P-DET-DS-POL.1

References:

# Data Acquisition and Computing Systems (DET-DS-COMP) to Tracking Systems (DET-DS-TRAK)

I-DET-DS-COMP-ONLINE.081	Data Acquisition

System 1:	Online DAQ and Computing Systems (6.03.01.08.02)

System 2:	Barrel Tracking Systems (6.03.01.02)

Type:	DATA

Description:	Cables required to transfer data from the detector to the online data acquisition system.

Provider:	Online DAQ and Computing Systems (6.03.01.08.02)

Details:	See details in the Data Acquisition and Computing Systems Interface Control Document.

Requirements:	F-DET-DS-COMP.6

References:

I-DET-DS-COMP-ONLINE.082	Data Transfer and Control Interface

System 1:	Online DAQ and Computing Systems (6.03.01.08.02)

System 2:	Barrel Tracking Systems (6.03.01.02)

Type:	DATA

Description:	A fiber connection will be provided from the DAQ system to the barrel tracking system's readout board to perform configuration, control, and data acquisition.

Provider:	Online DAQ and Computing Systems (6.03.01.08.02)

Details:	See details in the Data Acquisition and Computing Systems Interface Control Document.

Requirements:	F-DET-DS-COMP.6

References:

I-DET-DS-COMP-ONLINE.083	Slow Controls

System 1:	Online DAQ and Computing Systems (6.03.01.08.02)

System 2:	Barrel Tracking Systems (6.03.01.02)

Type:	CONTROL

Description:	A network connection will be provided from the DAQ system to the barrel tracking system's slow controls interface.

Provider:	Online DAQ and Computing Systems (6.03.01.08.02)

Details:	See details in the Data Acquisition and Computing Systems Interface Control Document.

Requirements:	F-DET-DS-COMP-SC.1

References:

I-DET-DS-COMP-ONLINE.084	Timing Interface

System 1:	Online DAQ and Computing Systems (6.03.01.08.02)

System 2:	Barrel Tracking Systems (6.03.01.02)

Type:	DATA

Description:	A fiber connection will be provided from the DAQ system to the barrel tracking system's readout board for timing synchronization.

Provider:	Online DAQ and Computing Systems (6.03.01.08.02)

Details:	See details in the Data Acquisition and Computing Systems Interface Control Document.

Requirements:	F-DET-DS-COMP.11

References:

# Applicable Documents

List applicable documents considered to be part of this document. Applicable documents shall be obtained from the EIC SharePoint Document Center.

# Detailed change Log

| Revision | Date | Change |
| --- | --- | --- |

# References

- doi:10.17226/25171., National Academies of Sciences, Engineering, and Medicine, "An Assessment of U.S.-Based Electron-Ion Collider Science", The National Academies Press, Washington, DC, 2018.

- EIC-ORG-PLN-010, Electron-Ion Collider L1 Requirements (Global Requirements Document)

- BNL-221006-2021-FORE, EIC Conceptual Design Report

- arXiv:2103.05419, Science Requirements and Detector Concepts for the Electron-Ion Collider: EIC Yellow Report

- EIC-SEG-PLN-016, Requirements Management Plan

- EIC-SEG-PLN-022, Systems Engineering Management Plan

- EIC-SEG-PLN-020, EIC Interface Management Plan