Electron-Ion Collider, Requirements, Specifications, and Interfaces

Interface Control Document: [###]

Electromagnetic Calorimetry Systems (6.03.01.04) to Detector System (6.03.01)

Prepared by:

{{Sig_es_:signer1:signature}}	Date: {{Dte_es_:signer1:date}}

Name, Role

Electron-Ion Collider at Institution

Reviewed by:

{{Sig_es_:signer2:signature}}	Date: {{Dte_es_:signer2:date}}

Name, Role

Electron-Ion Collider at Institution

{{Sig_es_:signer3:signature}}	Date: {{Dte_es_:signer3:date}}

Name, Role

Electron-Ion Collider at Institution

{{Sig_es_:signer4:signature}}	Date: {{Dte_es_:signer4:date}}

Name, Role

Electron-Ion Collider at Institution

Approved by:

{{Sig_es_:signer5:signature}}	Date: {{Dte_es_:signer5:date}}

Name, Role

Electron-Ion Collider at Institution

REVISION HISTORY

| Revision # | Effective Date | Additional Reviewers | Summary of Change |
| --- | --- | --- | --- |
| 00 |  |  | Initial release. |

TABLE OF CONTENTS

1.	Purpose and Scope	5

2.	Introduction	5

2.1	Electromagnetic Calorimetry Systems	5

2.2	Overview of Interface Relationships	5

2.3	Interface Responsibilities	5

2.4	Safety and Environmental Considerations	6

2.5	Design Considerations	6

2.6	Organization	6

3.	Interface summary	6

4.	Electromagnetic Calorimetry Systems (DET-DS-ECAL) to Data Acquisition and Computing Systems (DET-DS-COMP)	23

5.	Electromagnetic Calorimetry Systems (DET-DS-ECAL) to Electronic Systems (DET-DS-ELEC)	25

6.	Electromagnetic Calorimetry Systems (DET-DS-ECAL) to Hadronic Calorimetry Systems (DET-DS-HCAL)	27

7.	Electromagnetic Calorimetry Systems (DET-DS-ECAL) to Detector Infrastructure (DET-DS-INF)	30

8.	Electromagnetic Calorimetry Systems (DET-DS-ECAL) to Solenoid Magnet (DET-DS-MAG)	37

9.	Electromagnetic Calorimetry Systems (DET-DS-ECAL) to Particle Identification Systems (DET-DS-PID)	38

10.	Applicable Documents	41

11.	Detailed change Log	41

12.	References	41

LIST OF ACRONYMS

| B0 | B0 is the name of the forward magnet just past the Hadron Endcap |  |
| --- | --- | --- |
| CDR | Conceptual Design Report |  |
| COTS | Commercial Off the Shelf |  |
| DAQ | Data Acquisition |  |
| DET | EIC Detector |  |
| DIRC | Detection of Internally Reflected Cherenkov Light |  |
| dRICH | Dual radiator ring imaging Cherenkov |  |
| DVCS | Deeply Virtual Compton Scattering |  |
| EIC | Electron-Ion Collider |  |
| EMCAL | Electro-magnetic Calorimeter |  |
| ESR | Electron Storage Ring |  |
| FCC | Federal Communications Commission |  |
| FDR | Final Design Review |  |
| GEM | Gas Electron Multiplier |  |
| HCAL | Hadronic Calorimeter |  |
| HSR | Hadron Storage Ring |  |
| HVAC | Heating/Ventilation/Air Conditioning |  |
| IP | Interaction Point |  |
| IR | Interaction Region |  |
| MPGD | Micro Pattern Gaseous Detectors |  |
| NAS | National Academy of Sciences |  |
| NEC | National Electric Code |  |
| NEMA | National Electrical Manufacturers Association |  |
| NFPA | National Fire Protection Association |  |
| NRTL | Nationally Recognized Testing Laboratory |  |
| PID | Particle Identification |  |
| PRR | Production Readiness Review |  |
| PWO | PbWO4 |  |
| RCS | Rapid Cycling Synchrotron |  |
| TBD | To Be Determined |  |
| UL | Underwriter Laboratories |  |
| WBS | Work Break Down Structure |  |

# Interface Control Document:

Electromagnetic Calorimetry Systems (6.03.01.04) to Detector System (6.03.01)

# Purpose and Scope

This document records the agreed upon interfaces between the Electromagnetic Calorimetry Systems and other sub-systems within the Detector System. These interfaces are based on the system requirements that are necessary to integrate these sub-systems as part of the Electron Ion Collider Project.

# Introduction

These interfaces describe the relationships between the Electromagnetic Calorimetry Systems and the Detector System including (examples by inclusion) located in (areas by inclusion).

## Electromagnetic Calorimetry Systems

Provide an introductory description of your sub-system and an overview of the things that it receives, provides, supports, constrains, or otherwise requires as part of its function --- specifically if these things are manifested as interfaces.

Data Acquisition and Computing Systems (DET-DS-COMP): Description of the TYPES of interfaces that exist between the ICD reference system and this sub-system, and a BRIEF overview of the roles they play.

Electronic Systems (DET-DS-ELEC): Description of the TYPES of interfaces that exist between the ICD reference system and this sub-system, and a BRIEF overview of the roles they play.

Hadronic Calorimetry Systems (DET-DS-HCAL): Description of the TYPES of interfaces that exist between the ICD reference system and this sub-system, and a BRIEF overview of the roles they play.

Detector Infrastructure (DET-DS-INF): Description of the TYPES of interfaces that exist between the ICD reference system and this sub-system, and a BRIEF overview of the roles they play.

Solenoid Magnet (DET-DS-MAG): Description of the TYPES of interfaces that exist between the ICD reference system and this sub-system, and a BRIEF overview of the roles they play.

Particle Identification Systems (DET-DS-PID): Description of the TYPES of interfaces that exist between the ICD reference system and this sub-system, and a BRIEF overview of the roles they play.

## Overview of Interface Relationships

The following are the sub-systems within the Detector System that have interfaces connected to the Electromagnetic Calorimetry Systems.

## Interface Responsibilities

The various ownerships of responsibilities may be listed here. However, this section might also refer to the Provider section of the interface detail if all pertinent responsibilities are listed there. If this is the case, simply enter: “See the Provider section within the interface details for information about the responsibilities associated with individual interfaces.”

## Safety and Environmental Considerations

General safety and environmental considerations are identified here. Any considerations that are specific to the individual sub-system, will be addressed in external safety documents and are referenced here.

### Considerations for all systems

Add general safety and environmental considerations here.

### Special considerations and references

Add references here as necessary.

## Design Considerations

Notes here about general design considerations.

## Organization

The interfaces in this document are grouped by sub-system as identified in the sub-system identifier. All interfaces between the Electromagnetic Calorimetry Systems and a related sub-system will be included in a single section. Entries within that section will be sorted by the interface identifier or WBS number, as appropriate.

# Interface summary

This tables in this section provide a list of all interfaces that are defined within this document. The interfaces are grouped by sub-system/WBS. Each table has the following fields:

Section: the section number in this document where the interface is addressed.

WBS (1): the WBS number within the ICD reference system, unless the reference system is the owner.

WBS (2): the WBS number for the connected system.

ID: the interface identifier as defined in the Interface Requirement Document (IRD).

Name: the name of the interface as defined in the IRD.

Description: the description of the interface as defined in the IRD.

Owner: identifies the WBS number of the entity that is responsible for ensuring the interface is provided and that it meets validation/verification requirements.

Electromagnetic Calorimetry Systems to Data Acquisition and Computing Systems

| Section | WBS1 | WBS2 | ID | Name | Description | Owner |
| --- | --- | --- | --- | --- | --- | --- |
| 4 |  | 6.03.01.08 |  |  | Data Acquisition and Computing Systems |  |
| 4.1 | 6.03.01.04 | 6.03.01.08 | I-DET-DS-COMP-ONLINE.020 | Data Transfer and Control Interface | A fiber connection will be provided from the DAQ system to the barrel ECAL's readout board to perform configuration, control, and data acquisition. | 6.03.01.08.02 |
| 4.2 | 6.03.01.04 | 6.03.01.08 | I-DET-DS-COMP-ONLINE.021 | Slow Controls | A network connection will be provided from the DAQ system to the barrel ECAL's slow controls interface. | 6.03.01.08.02 |
| 4.3 | 6.03.01.04 | 6.03.01.08 | I-DET-DS-COMP-ONLINE.022 | Timing Interface | A fiber connection will be provided from the DAQ system to the barrel ECAL's readout board for timing synchronization. | 6.03.01.08.02 |
| 4.4 | 6.03.01.04 | 6.03.01.08 | I-DET-DS-COMP-ONLINE.023 | Data Transfer and Control Interface | A fiber connection will be provided from the DAQ system to the backward ECAL's readout board to perform configuration, control, and data acquisition. | 6.03.01.08.02 |
| 4.5 | 6.03.01.04 | 6.03.01.08 | I-DET-DS-COMP-ONLINE.024 | Slow Controls | A network connection will be provided from the DAQ system to the backward ECAL's slow controls interface. | 6.03.01.08.02 |
| 4.6 | 6.03.01.04 | 6.03.01.08 | I-DET-DS-COMP-ONLINE.025 | Timing Interface | A fiber connection will be provided from the DAQ system to the backward ECAL's readout board for timing synchronization. | 6.03.01.08.02 |
| 4.7 | 6.03.01.04 | 6.03.01.08 | I-DET-DS-COMP-ONLINE.026 | Data Transfer and Control Interface | A fiber connection will be provided from the DAQ system to the forward ECAL's readout board to perform configuration, control, and data acquisition. | 6.03.01.08.02 |
| 4.8 | 6.03.01.04 | 6.03.01.08 | I-DET-DS-COMP-ONLINE.027 | Slow Controls | A network connection will be provided from the DAQ system to the forward ECAL's slow controls interface. | 6.03.01.08.02 |
| 4.9 | 6.03.01.04 | 6.03.01.08 | I-DET-DS-COMP-ONLINE.028 | Timing Interface | A fiber connection will be provided from the DAQ system to the forward ECAL's readout board for timing synchronization. | 6.03.01.08.02 |

Electromagnetic Calorimetry Systems to Electronic Systems

| Section | WBS1 | WBS2 | ID | Name | Description | Owner |
| --- | --- | --- | --- | --- | --- | --- |
| 5 |  | 6.03.01.07 |  |  | Electronic Systems |  |
| 5.1 | 6.03.01.04 | 6.03.01.07 | I-DET-DS-ELEC.019 | Bias Voltage | Bias voltage DC power provided from the electronics racks to support electronics in the silicon photomultipliers. | 6.03.01.07 |
| 5.2 | 6.03.01.04 | 6.03.01.07 | I-DET-DS-ELEC.020 | High Voltage | High voltage DC power will be provided from the electronics racks to support silicon sensors and gas detectors. | 6.03.01.07 |
| 5.3 | 6.03.01.04 | 6.03.01.07 | I-DET-DS-ELEC.021 | Low Voltage | Low voltage DC power provided from the electronics racks to support electronics in the detector. | 6.03.01.07 |
| 5.4 | 6.03.01.04 | 6.03.01.07 | I-DET-DS-ELEC.022 | Bias Voltage | Bias voltage DC power will be provided from the electronics racks to support electronics the silicon photomultipliers. | 6.03.01.07 |
| 5.5 | 6.03.01.04 | 6.03.01.07 | I-DET-DS-ELEC.023 | High Voltage | High voltage DC power will be provided from the electronics racks to support silicon sensors and gas detectors. | 6.03.01.07 |
| 5.6 | 6.03.01.04 | 6.03.01.07 | I-DET-DS-ELEC.024 | Low Voltage | Low voltage DC power will be provided from the electronics racks to support electronics in the detector. | 6.03.01.07 |
| 5.7 | 6.03.01.04 | 6.03.01.07 | I-DET-DS-ELEC.025 | Bias Voltage | Bias voltage DC power provided from the electronics racks to support electronics in the silicon photomultipliers. | 6.03.01.07 |
| 5.8 | 6.03.01.04 | 6.03.01.07 | I-DET-DS-ELEC.026 | High Voltage | High voltage DC power will be provided from the electronics racks to support silicon sensors and gas detectors. | 6.03.01.07 |
| 5.9 | 6.03.01.04 | 6.03.01.07 | I-DET-DS-ELEC.027 | Low Voltage | Low voltage DC power provided from the electronics racks to support electronics in the detector. | 6.03.01.07 |

Electromagnetic Calorimetry Systems to Hadronic Calorimetry Systems

| Section | WBS1 | WBS2 | ID | Name | Description | Owner |
| --- | --- | --- | --- | --- | --- | --- |
| 6 |  | 6.03.01.05 |  |  | Hadronic Calorimetry Systems |  |
| 6.1 | 6.03.01.04 | 6.03.01.05 | I-DET-DS-INF-BAR.001 | Barrel ECAL Support | The barrel ECAL will be supported by a structural support system that extends through the bore of the solenoid magnet and is supported by the barrel Hadron Calorimeter. | 6.03.01.09.01 |
| 6.2 | 6.03.01.04 | 6.03.01.05 | I-DET-DS-INF-BAR.002 | Weight Distributed | The weight of the barrel ECAL will be transferred to the barrel Hadron Calorimeter and must be accommodated by all intermediate and subsequent support systems. | 6.03.01.09.01 |
| 6.3 | 6.03.01.04 | 6.03.01.05 | I-DET-DS-INF-BAR.004 | Backward ECAL Support | The backward ECAL will be supported by an integrated support structure that is attached to the DIRC support frame. | 6.03.01.09.01 |
| 6.4 | 6.03.01.04 | 6.03.01.05 | I-DET-DS-INF-BAR.007 | Forward ECAL Support | The forward ECAL will be supported by the forward Hadron Calorimeter endcap. Because the forward ECAL must split into two parts to provide access to the barrel, each half must be independently affixed to the Hadron Calorimeter halves. | 6.03.01.09.01 |
| 6.5 | 6.03.01.04 | 6.03.01.05 | I-DET-DS-INF-BAR.008 | Weight Transfer | The weight of the forward ECAL will be transferred to the forward Hadron Calorimeter endcap and must be accommodated by all subsequent support systems. | 6.03.01.09.01 |
| 6.6 | 6.03.01.04 | 6.03.01.05 | I-DET-DS-INF-FWD.004 | Sub-Detector Support | The forward HCAL will support the weight of the forward electromagnetic calorimeter that is embedded within it. | 6.03.01.09.03 |
| 6.7 | 6.03.01.04 | 6.03.01.05 | I-DET-DS-INF-INT.016 | Space Constraint | The backward position of the backward ECAL is limited by the backward Hadron Calorimeter and the adjacent cabling pathway that provides services to the interior detectors. Modifications to either must be coordinated. | 6.03.01.09.04 |
| 6.8 | 6.03.01.04 | 6.03.01.05 | I-DET-DS-INF-INT.023 | Space Constraint | The exterior radius of the forward ECAL is limited by the interior bore of the barrel Hadron Calorimeter. Modifications to either must be coordinated. | 6.03.01.09.04 |
| 6.9 | 6.03.01.04 | 6.03.01.05 | I-DET-DS-INF-INT.024 | Space Constraint | The forward position of the forward ECAL is limited by the forward Hadron Calorimeter. Modifications to either must be coordinated. | 6.03.01.09.04 |

Electromagnetic Calorimetry Systems to Detector Infrastructure

| Section | WBS1 | WBS2 | ID | Name | Description | Owner |
| --- | --- | --- | --- | --- | --- | --- |
| 7 |  | 6.03.01.09 |  |  | Detector Infrastructure |  |
| 7.1 | 6.03.01.04 | 6.03.01.09 | I-DET-DS-INF-BAR.001 | Barrel ECAL Support | The barrel ECAL will be supported by a structural support system that extends through the bore of the solenoid magnet and is supported by the barrel Hadron Calorimeter. | 6.03.01.09.01 |
| 7.2 | 6.03.01.04 | 6.03.01.09 | I-DET-DS-INF-BAR.002 | Weight Distributed | The weight of the barrel ECAL will be transferred to the barrel Hadron Calorimeter and must be accommodated by all intermediate and subsequent support systems. | 6.03.01.09.01 |
| 7.3 | 6.03.01.04 | 6.03.01.09 | I-DET-DS-INF-BAR.003 | Support Structure | The DIRC bar boxes will be supported by a frame inside the barrel Electromagnetic Calorimeter, that allows the boxes to be extracted using a system of rollers. | 6.03.01.09.01 |
| 7.4 | 6.03.01.04 | 6.03.01.09 | I-DET-DS-INF-BAR.004 | Backward ECAL Support | The backward ECAL will be supported by an integrated support structure that is attached to the DIRC support frame. | 6.03.01.09.01 |
| 7.5 | 6.03.01.04 | 6.03.01.09 | I-DET-DS-INF-BAR.005 | Weight Transfer | The weight of the backward ECAL will be transferred to the DIRC detector support and must be accommodated by all subsequent support systems. | 6.03.01.09.01 |
| 7.6 | 6.03.01.04 | 6.03.01.09 | I-DET-DS-INF-BAR.006 | EMCal Support System | The DIRC support system will provide support for the backward electromagnetic calorimeter. | 6.03.01.09.01 |
| 7.7 | 6.03.01.04 | 6.03.01.09 | I-DET-DS-INF-BAR.007 | Forward ECAL Support | The forward ECAL will be supported by the forward Hadron Calorimeter endcap. Because the forward ECAL must split into two parts to provide access to the barrel, each half must be independently affixed to the Hadron Calorimeter halves. | 6.03.01.09.01 |
| 7.8 | 6.03.01.04 | 6.03.01.09 | I-DET-DS-INF-BAR.008 | Weight Transfer | The weight of the forward ECAL will be transferred to the forward Hadron Calorimeter endcap and must be accommodated by all subsequent support systems. | 6.03.01.09.01 |
| 7.9 | 6.03.01.04 | 6.03.01.09 | I-DET-DS-INF-BAR.014 | Forward ECAL Mobility Support | Each half of the forward ECAL must be continuously supported and stabilized while it is moved between the opened and closed positions. | 6.03.01.09.01 |
| 7.10 | 6.03.01.04 | 6.03.01.09 | I-DET-DS-INF-FWD.004 | Sub-Detector Support | The forward HCAL will support the weight of the forward electromagnetic calorimeter that is embedded within it. | 6.03.01.09.03 |
| 7.11 | 6.03.01.04 | 6.03.01.09 | I-DET-DS-INF-FWD.008 | Forward ECAL Mobility Support | Each half of the forward ECAL must be continuously supported and stabilized while it is moved between the opened and closed positions. | 6.03.01.09.03 |
| 7.12 | 6.03.01.04 | 6.03.01.09 | I-DET-DS-INF-INT.012 | Space Constraint | The exterior radius of the barrel ECAL (and its support system) is limited by the interior bore of the solenoid magnet. Modifications to either must be coordinated. | 6.03.01.09.04 |
| 7.13 | 6.03.01.04 | 6.03.01.09 | I-DET-DS-INF-INT.013 | Space Constraint | The backward position and shape of the barrel EMCAL is limited by the size and shape of the DIRC readout supports. Changes to the size or position of either must be coordinated with the other. | 6.03.01.09.04 |
| 7.14 | 6.03.01.04 | 6.03.01.09 | I-DET-DS-INF-INT.014 | Space Constraint | The maximum size of the DIRC is limited to the interior bore of the barrel ECAL (and its support structures). Modifications to either must be coordinated. | 6.03.01.09.04 |
| 7.15 | 6.03.01.04 | 6.03.01.09 | I-DET-DS-INF-INT.015 | Space Constraint | The forward position and shape of the barrel ECAL is limited by the backward face of the Dual RICH detector and the adjacent cabling pathway that provides services to the interior detectors. Modifications to either must be coordinated. | 6.03.01.09.04 |
| 7.16 | 6.03.01.04 | 6.03.01.09 | I-DET-DS-INF-INT.016 | Space Constraint | The backward position of the backward ECAL is limited by the backward Hadron Calorimeter and the adjacent cabling pathway that provides services to the interior detectors. Modifications to either must be coordinated. | 6.03.01.09.04 |
| 7.17 | 6.03.01.04 | 6.03.01.09 | I-DET-DS-INF-INT.017 | Space Constraint | The maximum backward location for the DIRC is limited by the position of the backward electromagnetic calorimeter. Modifications to either must be coordinated. | 6.03.01.09.04 |
| 7.18 | 6.03.01.04 | 6.03.01.09 | I-DET-DS-INF-INT.018 | Space Constraint | The exterior radius of the backward ECAL is limited by the interior bore of the DIRC support system. Modifications to either must be coordinated. | 6.03.01.09.04 |
| 7.19 | 6.03.01.04 | 6.03.01.09 | I-DET-DS-INF-INT.019 | Heat Mitigation | The backward ECAL must mitigated the heat generated by the backward RICH detector. | 6.03.01.09.04 |
| 7.20 | 6.03.01.04 | 6.03.01.09 | I-DET-DS-INF-INT.020 | Space Constraint | The position of the backward ECAL in the forward direction is limited by the backward face of the pfRICH/mRICH detector. Modifications to either must be coordinated. | 6.03.01.09.04 |
| 7.21 | 6.03.01.04 | 6.03.01.09 | I-DET-DS-INF-INT.021 | Backward ECAL Bore Clearance | The bore of the backward ECAL must be designed to allow it to be inserted/removed over the existing beamline flanges. | 6.03.01.09.04 |
| 7.22 | 6.03.01.04 | 6.03.01.09 | I-DET-DS-INF-INT.022 | Space Constraint | The interior radius of the backward ECAL is governed by the size of the beamline. | 6.03.01.09.04 |
| 7.23 | 6.03.01.04 | 6.03.01.09 | I-DET-DS-INF-INT.023 | Space Constraint | The exterior radius of the forward ECAL is limited by the interior bore of the barrel Hadron Calorimeter. Modifications to either must be coordinated. | 6.03.01.09.04 |
| 7.24 | 6.03.01.04 | 6.03.01.09 | I-DET-DS-INF-INT.024 | Space Constraint | The forward position of the forward ECAL is limited by the forward Hadron Calorimeter. Modifications to either must be coordinated. | 6.03.01.09.04 |
| 7.25 | 6.03.01.04 | 6.03.01.09 | I-DET-DS-INF-INT.025 | Space Constraint | The maximum forward location for the dRICH is limited by the forward ECAL and the adjacent cabling pathway that provides services to the interior detectors. Modifications to either must be coordinated. | 6.03.01.09.04 |
| 7.26 | 6.03.01.04 | 6.03.01.09 | I-DET-DS-INF-INT.026 | Space Constraint | The interior radius of the forward ECAL is governed by the size of the beamline. Modifications to either must be coordinated. | 6.03.01.09.04 |
| 7.27 | 6.03.01.04 | 6.03.01.09 | I-DET-DS-INF-INT.062 | Imaging Layers Heat Rejection | The imaging layers must be cooled with an independent system (TBD). | 6.03.01.09.04 |
| 7.28 | 6.03.01.04 | 6.03.01.09 | I-DET-DS-INF-INT.063 | Silicon PM Heat Rejection | The barrel ECAL will require chilled water or LCW cooling to maintain a safe operating temperature. | 6.03.01.09.04 |
| 7.29 | 6.03.01.04 | 6.03.01.09 | I-DET-DS-INF-INT.064 | Heat Rejection | The backward ECAL will require chilled water or LCW cooling to maintain the temperature of the silicon photomultipliers, the electronics, and the crystals. | 6.03.01.09.04 |
| 7.30 | 6.03.01.04 | 6.03.01.09 | I-DET-DS-INF-INT.065 | Heat Rejection | The forward ECAL will require chilled water or LCW cooling to maintain a safe operating temperature. | 6.03.01.09.04 |

Electromagnetic Calorimetry Systems to Solenoid Magnet

| Section | WBS1 | WBS2 | ID | Name | Description | Owner |
| --- | --- | --- | --- | --- | --- | --- |
| 8 |  | 6.03.01.06 |  |  | Solenoid Magnet |  |
| 8.1 | 6.03.01.04 | 6.03.01.06 | I-DET-DS-INF-INT.012 | Space Constraint | The exterior radius of the barrel ECAL (and its support system) is limited by the interior bore of the solenoid magnet. Modifications to either must be coordinated. | 6.03.01.09.04 |

Electromagnetic Calorimetry Systems to Particle Identification Systems

| Section | WBS1 | WBS2 | ID | Name | Description | Owner |
| --- | --- | --- | --- | --- | --- | --- |
| 9 |  | 6.03.01.03 |  |  | Particle Identification Systems |  |
| 9.1 | 6.03.01.04 | 6.03.01.03 | I-DET-DS-INF-BAR.003 | Support Structure | The DIRC bar boxes will be supported by a frame inside the barrel Electromagnetic Calorimeter, that allows the boxes to be extracted using a system of rollers. | 6.03.01.09.01 |
| 9.2 | 6.03.01.04 | 6.03.01.03 | I-DET-DS-INF-BAR.005 | Weight Transfer | The weight of the backward ECAL will be transferred to the DIRC detector support and must be accommodated by all subsequent support systems. | 6.03.01.09.01 |
| 9.3 | 6.03.01.04 | 6.03.01.03 | I-DET-DS-INF-BAR.006 | EMCal Support System | The DIRC support system will provide support for the backward electromagnetic calorimeter. | 6.03.01.09.01 |
| 9.4 | 6.03.01.04 | 6.03.01.03 | I-DET-DS-INF-INT.013 | Space Constraint | The backward position and shape of the barrel EMCAL is limited by the size and shape of the DIRC readout supports. Changes to the size or position of either must be coordinated with the other. | 6.03.01.09.04 |
| 9.5 | 6.03.01.04 | 6.03.01.03 | I-DET-DS-INF-INT.014 | Space Constraint | The maximum size of the DIRC is limited to the interior bore of the barrel ECAL (and its support structures). Modifications to either must be coordinated. | 6.03.01.09.04 |
| 9.6 | 6.03.01.04 | 6.03.01.03 | I-DET-DS-INF-INT.015 | Space Constraint | The forward position and shape of the barrel ECAL is limited by the backward face of the Dual RICH detector and the adjacent cabling pathway that provides services to the interior detectors. Modifications to either must be coordinated. | 6.03.01.09.04 |
| 9.7 | 6.03.01.04 | 6.03.01.03 | I-DET-DS-INF-INT.017 | Space Constraint | The maximum backward location for the DIRC is limited by the position of the backward electromagnetic calorimeter. Modifications to either must be coordinated. | 6.03.01.09.04 |
| 9.8 | 6.03.01.04 | 6.03.01.03 | I-DET-DS-INF-INT.018 | Space Constraint | The exterior radius of the backward ECAL is limited by the interior bore of the DIRC support system. Modifications to either must be coordinated. | 6.03.01.09.04 |
| 9.9 | 6.03.01.04 | 6.03.01.03 | I-DET-DS-INF-INT.019 | Heat Mitigation | The backward ECAL must mitigated the heat generated by the backward RICH detector. | 6.03.01.09.04 |
| 9.10 | 6.03.01.04 | 6.03.01.03 | I-DET-DS-INF-INT.020 | Space Constraint | The position of the backward ECAL in the forward direction is limited by the backward face of the pfRICH/mRICH detector. Modifications to either must be coordinated. | 6.03.01.09.04 |
| 9.11 | 6.03.01.04 | 6.03.01.03 | I-DET-DS-INF-INT.025 | Space Constraint | The maximum forward location for the dRICH is limited by the forward ECAL and the adjacent cabling pathway that provides services to the interior detectors. Modifications to either must be coordinated. | 6.03.01.09.04 |

# Electromagnetic Calorimetry Systems (DET-DS-ECAL) to Data Acquisition and Computing Systems (DET-DS-COMP)

I-DET-DS-COMP-ONLINE.020	Data Transfer and Control Interface

System 1:	Online DAQ and Computing Systems (6.03.01.08.02)

System 2:	Barrel EMCal Systems (6.03.01.04)

Type:	DATA

Description:	A fiber connection will be provided from the DAQ system to the barrel ECAL's readout board to perform configuration, control, and data acquisition.

Provider:	Online DAQ and Computing Systems (6.03.01.08.02)

Details:	See details in the Data Acquisition and Computing Systems Interface Control Document.

Requirements:	F-DET-DS-COMP.6

References:

I-DET-DS-COMP-ONLINE.021	Slow Controls

System 1:	Online DAQ and Computing Systems (6.03.01.08.02)

System 2:	Barrel EMCal Systems (6.03.01.04)

Type:	CONTROL

Description:	A network connection will be provided from the DAQ system to the barrel ECAL's slow controls interface.

Provider:	Online DAQ and Computing Systems (6.03.01.08.02)

Details:	See details in the Data Acquisition and Computing Systems Interface Control Document.

Requirements:	F-DET-DS-COMP-SC.1

References:

I-DET-DS-COMP-ONLINE.022	Timing Interface

System 1:	Online DAQ and Computing Systems (6.03.01.08.02)

System 2:	Barrel EMCal Systems (6.03.01.04)

Type:	DATA

Description:	A fiber connection will be provided from the DAQ system to the barrel ECAL's readout board for timing synchronization.

Provider:	Online DAQ and Computing Systems (6.03.01.08.02)

Details:	See details in the Data Acquisition and Computing Systems Interface Control Document.

Requirements:	F-DET-DS-COMP.11

References:

I-DET-DS-COMP-ONLINE.023	Data Transfer and Control Interface

System 1:	Online DAQ and Computing Systems (6.03.01.08.02)

System 2:	Backward EMCal Systems (6.03.01.04)

Type:	DATA

Description:	A fiber connection will be provided from the DAQ system to the backward ECAL's readout board to perform configuration, control, and data acquisition.

Provider:	Online DAQ and Computing Systems (6.03.01.08.02)

Details:	See details in the Data Acquisition and Computing Systems Interface Control Document.

Requirements:	F-DET-DS-COMP.6

References:

I-DET-DS-COMP-ONLINE.024	Slow Controls

System 1:	Online DAQ and Computing Systems (6.03.01.08.02)

System 2:	Backward EMCal Systems (6.03.01.04)

Type:	CONTROL

Description:	A network connection will be provided from the DAQ system to the backward ECAL's slow controls interface.

Provider:	Online DAQ and Computing Systems (6.03.01.08.02)

Details:	See details in the Data Acquisition and Computing Systems Interface Control Document.

Requirements:	F-DET-DS-COMP-SC.1

References:

I-DET-DS-COMP-ONLINE.025	Timing Interface

System 1:	Online DAQ and Computing Systems (6.03.01.08.02)

System 2:	Backward EMCal Systems (6.03.01.04)

Type:	DATA

Description:	A fiber connection will be provided from the DAQ system to the backward ECAL's readout board for timing synchronization.

Provider:	Online DAQ and Computing Systems (6.03.01.08.02)

Details:	See details in the Data Acquisition and Computing Systems Interface Control Document.

Requirements:	F-DET-DS-COMP.11

References:

I-DET-DS-COMP-ONLINE.026	Data Transfer and Control Interface

System 1:	Online DAQ and Computing Systems (6.03.01.08.02)

System 2:	Forward EMCal Systems (6.03.01.04)

Type:	DATA

Description:	A fiber connection will be provided from the DAQ system to the forward ECAL's readout board to perform configuration, control, and data acquisition.

Provider:	Online DAQ and Computing Systems (6.03.01.08.02)

Details:	See details in the Data Acquisition and Computing Systems Interface Control Document.

Requirements:	F-DET-DS-COMP.6

References:

I-DET-DS-COMP-ONLINE.027	Slow Controls

System 1:	Online DAQ and Computing Systems (6.03.01.08.02)

System 2:	Forward EMCal Systems (6.03.01.04)

Type:	CONTROL

Description:	A network connection will be provided from the DAQ system to the forward ECAL's slow controls interface.

Provider:	Online DAQ and Computing Systems (6.03.01.08.02)

Details:	See details in the Data Acquisition and Computing Systems Interface Control Document.

Requirements:	F-DET-DS-COMP-SC.1

References:

I-DET-DS-COMP-ONLINE.028	Timing Interface

System 1:	Online DAQ and Computing Systems (6.03.01.08.02)

System 2:	Forward EMCal Systems (6.03.01.04)

Type:	DATA

Description:	A fiber connection will be provided from the DAQ system to the forward ECAL's readout board for timing synchronization.

Provider:	Online DAQ and Computing Systems (6.03.01.08.02)

Details:	See details in the Data Acquisition and Computing Systems Interface Control Document.

Requirements:	F-DET-DS-COMP.11

References:

# Electromagnetic Calorimetry Systems (DET-DS-ECAL) to Electronic Systems (DET-DS-ELEC)

I-DET-DS-ELEC.019	Bias Voltage

System 1:	Electronic Systems (6.03.01.07)

System 2:	Barrel EMCal Systems (6.03.01.04)

Type:	ELEC

Description:	Bias voltage DC power provided from the electronics racks to support electronics in the silicon photomultipliers.

Provider:	Electronic Systems (6.03.01.07)

Details:	See details in the Electronic Systems Interface Control Document.

Requirements:	P-DET-DS-ECAL.4

References:

I-DET-DS-ELEC.020	High Voltage

System 1:	Electronic Systems (6.03.01.07)

System 2:	Barrel EMCal Systems (6.03.01.04)

Type:	ELEC

Description:	High voltage DC power will be provided from the electronics racks to support silicon sensors and gas detectors.

Provider:	Electronic Systems (6.03.01.07)

Details:	See details in the Electronic Systems Interface Control Document.

Requirements:	P-DET-DS-ECAL.4

References:

I-DET-DS-ELEC.021	Low Voltage

System 1:	Electronic Systems (6.03.01.07)

System 2:	Barrel EMCal Systems (6.03.01.04)

Type:	ELEC

Description:	Low voltage DC power provided from the electronics racks to support electronics in the detector.

Provider:	Electronic Systems (6.03.01.07)

Details:	See details in the Electronic Systems Interface Control Document.

Requirements:	P-DET-DS-ECAL.4

References:

I-DET-DS-ELEC.022	Bias Voltage

System 1:	Electronic Systems (6.03.01.07)

System 2:	Backward EMCal Systems (6.03.01.04)

Type:	ELEC

Description:	Bias voltage DC power will be provided from the electronics racks to support electronics the silicon photomultipliers.

Provider:	Electronic Systems (6.03.01.07)

Details:	See details in the Electronic Systems Interface Control Document.

Requirements:	P-DET-DS-ECAL.4

References:

I-DET-DS-ELEC.023	High Voltage

System 1:	Electronic Systems (6.03.01.07)

System 2:	Backward EMCal Systems (6.03.01.04)

Type:	ELEC

Description:	High voltage DC power will be provided from the electronics racks to support silicon sensors and gas detectors.

Provider:	Electronic Systems (6.03.01.07)

Details:	See details in the Electronic Systems Interface Control Document.

Requirements:	P-DET-DS-ECAL.4

References:

I-DET-DS-ELEC.024	Low Voltage

System 1:	Electronic Systems (6.03.01.07)

System 2:	Backward EMCal Systems (6.03.01.04)

Type:	ELEC

Description:	Low voltage DC power will be provided from the electronics racks to support electronics in the detector.

Provider:	Electronic Systems (6.03.01.07)

Details:	See details in the Electronic Systems Interface Control Document.

Requirements:	P-DET-DS-ECAL.4

References:

I-DET-DS-ELEC.025	Bias Voltage

System 1:	Electronic Systems (6.03.01.07)

System 2:	Forward EMCal Systems (6.03.01.04)

Type:	ELEC

Description:	Bias voltage DC power provided from the electronics racks to support electronics in the silicon photomultipliers.

Provider:	Electronic Systems (6.03.01.07)

Details:	See details in the Electronic Systems Interface Control Document.

Requirements:	P-DET-DS-ECAL.4

References:

I-DET-DS-ELEC.026	High Voltage

System 1:	Electronic Systems (6.03.01.07)

System 2:	Forward EMCal Systems (6.03.01.04)

Type:	ELEC

Description:	High voltage DC power will be provided from the electronics racks to support silicon sensors and gas detectors.

Provider:	Electronic Systems (6.03.01.07)

Details:	See details in the Electronic Systems Interface Control Document.

Requirements:	P-DET-DS-ECAL.4

References:

I-DET-DS-ELEC.027	Low Voltage

System 1:	Electronic Systems (6.03.01.07)

System 2:	Forward EMCal Systems (6.03.01.04)

Type:	ELEC

Description:	Low voltage DC power provided from the electronics racks to support electronics in the detector.

Provider:	Electronic Systems (6.03.01.07)

Details:	See details in the Electronic Systems Interface Control Document.

Requirements:	P-DET-DS-ECAL.4

References:

# Electromagnetic Calorimetry Systems (DET-DS-ECAL) to Hadronic Calorimetry Systems (DET-DS-HCAL)

I-DET-DS-INF-BAR.001	Barrel ECAL Support

System 1:	Barrel EMCal Systems (6.03.01.04)

System 2:	Barrel HCal Systems (6.03.01.05)

Type:	STRUCT

Description:	The barrel ECAL will be supported by a structural support system that extends through the bore of the solenoid magnet and is supported by the barrel Hadron Calorimeter.

Provider:	Barrel Detector Structures (6.03.01.09.01)

Details:	See details in the Detector Infrastructure Interface Control Document.

Requirements:	P-DET-DS-ECAL.2, P-DET-DS-HCAL.2

References:

I-DET-DS-INF-BAR.002	Weight Distributed

System 1:	Barrel EMCal Systems (6.03.01.04)

System 2:	Forward HCal Systems (6.03.01.05)

Type:	STRUCT

Description:	The weight of the barrel ECAL will be transferred to the barrel Hadron Calorimeter and must be accommodated by all intermediate and subsequent support systems.

Provider:	Barrel Detector Structures (6.03.01.09.01)

Details:	See details in the Detector Infrastructure Interface Control Document.

Requirements:	P-DET-DS-ECAL.2, P-DET-DS-HCAL.2

References:

I-DET-DS-INF-BAR.004	Backward ECAL Support

System 1:	Backward EMCal Systems (6.03.01.04)

System 2:	Forward HCal Systems (6.03.01.05)

Type:	STRUCT

Description:	The backward ECAL will be supported by an integrated support structure that is attached to the DIRC support frame.

Provider:	Barrel Detector Structures (6.03.01.09.01)

Details:	See details in the Detector Infrastructure Interface Control Document.

Requirements:	P-DET-DS-ECAL.2, P-DET-DS-HCAL.2

References:

I-DET-DS-INF-BAR.007	Forward ECAL Support

System 1:	Forward EMCal Systems (6.03.01.04)

System 2:	Forward HCal Systems (6.03.01.05)

Type:	STRUCT

Description:	The forward ECAL will be supported by the forward Hadron Calorimeter endcap. Because the forward ECAL must split into two parts to provide access to the barrel, each half must be independently affixed to the Hadron Calorimeter halves.

Provider:	Barrel Detector Structures (6.03.01.09.01)

Details:	See details in the Detector Infrastructure Interface Control Document.

Requirements:	P-DET-DS-ECAL.1, P-DET-DS-ECAL.2, P-DET-DS-HCAL.2

References:

I-DET-DS-INF-BAR.008	Weight Transfer

System 1:	Forward EMCal Systems (6.03.01.04)

System 2:	Forward HCal Systems (6.03.01.05)

Type:	STRUCT

Description:	The weight of the forward ECAL will be transferred to the forward Hadron Calorimeter endcap and must be accommodated by all subsequent support systems.

Provider:	Barrel Detector Structures (6.03.01.09.01)

Details:	See details in the Detector Infrastructure Interface Control Document.

Requirements:	P-DET-DS-ECAL.1, P-DET-DS-ECAL.2, P-DET-DS-HCAL.2

References:

I-DET-DS-INF-FWD.004	Sub-Detector Support

System 1:	Forward HCal Systems (6.03.01.05)

System 2:	Forward EMCal Systems (6.03.01.04)

Type:	STRUCT

Description:	The forward HCAL will support the weight of the forward electromagnetic calorimeter that is embedded within it.

Provider:	Hadron Endcap Structures (6.03.01.09.03)

Details:	See details in the Detector Infrastructure Interface Control Document.

Requirements:	P-DET-DS-HCAL.2

References:

I-DET-DS-INF-INT.016	Space Constraint

System 1:	Backward EMCal Systems (6.03.01.04)

System 2:	Backward HCal Systems (6.03.01.05)

Type:	SPACE

Description:	The backward position of the backward ECAL is limited by the backward Hadron Calorimeter and the adjacent cabling pathway that provides services to the interior detectors. Modifications to either must be coordinated.

Provider:	Detector Infrastructure and Utilities Integration (6.03.01.09.04)

Details:	See details in the Detector Infrastructure Interface Control Document.

Requirements:	P-DET-DS-ECAL.1, P-DET-DS-HCAL.1

References:

I-DET-DS-INF-INT.023	Space Constraint

System 1:	Forward EMCal Systems (6.03.01.04)

System 2:	Barrel HCal Systems (6.03.01.05)

Type:	SPACE

Description:	The exterior radius of the forward ECAL is limited by the interior bore of the barrel Hadron Calorimeter. Modifications to either must be coordinated.

Provider:	Detector Infrastructure and Utilities Integration (6.03.01.09.04)

Details:	See details in the Detector Infrastructure Interface Control Document.

Requirements:	P-DET-DS-ECAL.1, P-DET-DS-HCAL.1

References:

I-DET-DS-INF-INT.024	Space Constraint

System 1:	Forward EMCal Systems (6.03.01.04)

System 2:	Forward HCal Systems (6.03.01.05)

Type:	SPACE

Description:	The forward position of the forward ECAL is limited by the forward Hadron Calorimeter. Modifications to either must be coordinated.

Provider:	Detector Infrastructure and Utilities Integration (6.03.01.09.04)

Details:	See details in the Detector Infrastructure Interface Control Document.

Requirements:	P-DET-DS-ECAL.1, P-DET-DS-HCAL.1

References:

# Electromagnetic Calorimetry Systems (DET-DS-ECAL) to Detector Infrastructure (DET-DS-INF)

I-DET-DS-INF-BAR.001	Barrel ECAL Support

System 1:	Barrel EMCal Systems (6.03.01.04)

System 2:	Barrel HCal Systems (6.03.01.05)

Type:	STRUCT

Description:	The barrel ECAL will be supported by a structural support system that extends through the bore of the solenoid magnet and is supported by the barrel Hadron Calorimeter.

Provider:	Barrel Detector Structures (6.03.01.09.01)

Details:	See details in the Detector Infrastructure Interface Control Document.

Requirements:	P-DET-DS-ECAL.2, P-DET-DS-HCAL.2

References:

I-DET-DS-INF-BAR.002	Weight Distributed

System 1:	Barrel EMCal Systems (6.03.01.04)

System 2:	Forward HCal Systems (6.03.01.05)

Type:	STRUCT

Description:	The weight of the barrel ECAL will be transferred to the barrel Hadron Calorimeter and must be accommodated by all intermediate and subsequent support systems.

Provider:	Barrel Detector Structures (6.03.01.09.01)

Details:	See details in the Detector Infrastructure Interface Control Document.

Requirements:	P-DET-DS-ECAL.2, P-DET-DS-HCAL.2

References:

I-DET-DS-INF-BAR.003	Support Structure

System 1:	Barrel EMCal Systems (6.03.01.04)

System 2:	Barrel DIRC Particle ID Systems (6.03.01.03)

Type:	STRUCT

Description:	The DIRC bar boxes will be supported by a frame inside the barrel Electromagnetic Calorimeter, that allows the boxes to be extracted using a system of rollers.

Provider:	Barrel Detector Structures (6.03.01.09.01)

Details:	See details in the Detector Infrastructure Interface Control Document.

Requirements:	P-DET-DS-ECAL.1, P-DET-DS-ECAL.2, P-DET-DS-PID.4, P-DET-DS-PID.5

References:

I-DET-DS-INF-BAR.004	Backward ECAL Support

System 1:	Backward EMCal Systems (6.03.01.04)

System 2:	Forward HCal Systems (6.03.01.05)

Type:	STRUCT

Description:	The backward ECAL will be supported by an integrated support structure that is attached to the DIRC support frame.

Provider:	Barrel Detector Structures (6.03.01.09.01)

Details:	See details in the Detector Infrastructure Interface Control Document.

Requirements:	P-DET-DS-ECAL.2, P-DET-DS-HCAL.2

References:

I-DET-DS-INF-BAR.005	Weight Transfer

System 1:	Backward EMCal Systems (6.03.01.04)

System 2:	Barrel DIRC Particle ID Systems (6.03.01.03)

Type:	STRUCT

Description:	The weight of the backward ECAL will be transferred to the DIRC detector support and must be accommodated by all subsequent support systems.

Provider:	Barrel Detector Structures (6.03.01.09.01)

Details:	See details in the Detector Infrastructure Interface Control Document.

Requirements:	P-DET-DS-ECAL.2, P-DET-DS-HCAL.2

References:

I-DET-DS-INF-BAR.006	EMCal Support System

System 1:	Backward EMCal Systems (6.03.01.04)

System 2:	Barrel DIRC Particle ID Systems (6.03.01.03)

Type:	STRUCT

Description:	The DIRC support system will provide support for the backward electromagnetic calorimeter.

Provider:	Barrel Detector Structures (6.03.01.09.01)

Details:	See details in the Detector Infrastructure Interface Control Document.

Requirements:	P-DET-DS-ECAL.2, P-DET-DS-PID.4

References:

I-DET-DS-INF-BAR.007	Forward ECAL Support

System 1:	Forward EMCal Systems (6.03.01.04)

System 2:	Forward HCal Systems (6.03.01.05)

Type:	STRUCT

Description:	The forward ECAL will be supported by the forward Hadron Calorimeter endcap. Because the forward ECAL must split into two parts to provide access to the barrel, each half must be independently affixed to the Hadron Calorimeter halves.

Provider:	Barrel Detector Structures (6.03.01.09.01)

Details:	See details in the Detector Infrastructure Interface Control Document.

Requirements:	P-DET-DS-ECAL.1, P-DET-DS-ECAL.2, P-DET-DS-HCAL.2

References:

I-DET-DS-INF-BAR.008	Weight Transfer

System 1:	Forward EMCal Systems (6.03.01.04)

System 2:	Forward HCal Systems (6.03.01.05)

Type:	STRUCT

Description:	The weight of the forward ECAL will be transferred to the forward Hadron Calorimeter endcap and must be accommodated by all subsequent support systems.

Provider:	Barrel Detector Structures (6.03.01.09.01)

Details:	See details in the Detector Infrastructure Interface Control Document.

Requirements:	P-DET-DS-ECAL.1, P-DET-DS-ECAL.2, P-DET-DS-HCAL.2

References:

I-DET-DS-INF-BAR.014	Forward ECAL Mobility Support

System 1:	Barrel Detector Structures (6.03.01.09.01)

System 2:	Forward EMCal Systems (6.03.01.04)

Type:	MECH

Description:	Each half of the forward ECAL must be continuously supported and stabilized while it is moved between the opened and closed positions.

Provider:	Barrel Detector Structures (6.03.01.09.01)

Details:	See details in the Detector Infrastructure Interface Control Document.

Requirements:	P-DET-DS-ECAL.1, P-DET-DS-ECAL.2

References:

I-DET-DS-INF-FWD.004	Sub-Detector Support

System 1:	Forward HCal Systems (6.03.01.05)

System 2:	Forward EMCal Systems (6.03.01.04)

Type:	STRUCT

Description:	The forward HCAL will support the weight of the forward electromagnetic calorimeter that is embedded within it.

Provider:	Hadron Endcap Structures (6.03.01.09.03)

Details:	See details in the Detector Infrastructure Interface Control Document.

Requirements:	P-DET-DS-HCAL.2

References:

I-DET-DS-INF-FWD.008	Forward ECAL Mobility Support

System 1:	Hadron Endcap Structures (6.03.01.09.03)

System 2:	Forward EMCal Systems (6.03.01.04)

Type:	MECH

Description:	Each half of the forward ECAL must be continuously supported and stabilized while it is moved between the opened and closed positions.

Provider:	Hadron Endcap Structures (6.03.01.09.03)

Details:	See details in the Detector Infrastructure Interface Control Document.

Requirements:	P-DET-DS-ECAL.1, P-DET-DS-ECAL.2

References:

I-DET-DS-INF-INT.012	Space Constraint

System 1:	Barrel EMCal Systems (6.03.01.04)

System 2:	Solenoid Magnet (6.03.01.06)

Type:	SPACE

Description:	The exterior radius of the barrel ECAL (and its support system) is limited by the interior bore of the solenoid magnet. Modifications to either must be coordinated.

Provider:	Detector Infrastructure and Utilities Integration (6.03.01.09.04)

Details:	See details in the Detector Infrastructure Interface Control Document.

Requirements:	P-DET-DS-ECAL.1, P-DET-DS-MAG.1

References:

I-DET-DS-INF-INT.013	Space Constraint

System 1:	Barrel EMCal Systems (6.03.01.04)

System 2:	Barrel DIRC Particle ID Systems (6.03.01.03)

Type:	SPACE

Description:	The backward position and shape of the barrel EMCAL is limited by the size and shape of the DIRC readout supports.  Changes to the size or position of either must be coordinated with the other.

Provider:	Detector Infrastructure and Utilities Integration (6.03.01.09.04)

Details:	See details in the Detector Infrastructure Interface Control Document.

Requirements:	P-DET-DS-ECAL.1, P-DET-DS-PID.5

References:

I-DET-DS-INF-INT.014	Space Constraint

System 1:	Barrel EMCal Systems (6.03.01.04)

System 2:	Barrel DIRC Particle ID Systems (6.03.01.03)

Type:	SPACE

Description:	The maximum size of the DIRC is limited to the interior bore of the barrel ECAL (and its support structures).  Modifications to either must be coordinated.

Provider:	Detector Infrastructure and Utilities Integration (6.03.01.09.04)

Details:	See details in the Detector Infrastructure Interface Control Document.

Requirements:	P-DET-DS-ECAL.1, P-DET-DS-PID.5

References:

I-DET-DS-INF-INT.015	Space Constraint

System 1:	Barrel EMCal Systems (6.03.01.04)

System 2:	Forward RICH Detector Systems (6.03.01.03)

Type:	SPACE

Description:	The forward position and shape of the barrel ECAL is limited  by the backward face of the Dual RICH detector and the adjacent cabling pathway that provides services to the interior detectors. Modifications to either must be coordinated.

Provider:	Detector Infrastructure and Utilities Integration (6.03.01.09.04)

Details:	See details in the Detector Infrastructure Interface Control Document.

Requirements:	P-DET-DS-ECAL.1, P-DET-DS-PID.5

References:

I-DET-DS-INF-INT.016	Space Constraint

System 1:	Backward EMCal Systems (6.03.01.04)

System 2:	Backward HCal Systems (6.03.01.05)

Type:	SPACE

Description:	The backward position of the backward ECAL is limited by the backward Hadron Calorimeter and the adjacent cabling pathway that provides services to the interior detectors. Modifications to either must be coordinated.

Provider:	Detector Infrastructure and Utilities Integration (6.03.01.09.04)

Details:	See details in the Detector Infrastructure Interface Control Document.

Requirements:	P-DET-DS-ECAL.1, P-DET-DS-HCAL.1

References:

I-DET-DS-INF-INT.017	Space Constraint

System 1:	Backward EMCal Systems (6.03.01.04)

System 2:	Barrel DIRC Particle ID Systems (6.03.01.03)

Type:	SPACE

Description:	The maximum backward location for the DIRC  is limited by the position of the backward electromagnetic calorimeter. Modifications to either must be coordinated.

Provider:	Detector Infrastructure and Utilities Integration (6.03.01.09.04)

Details:	See details in the Detector Infrastructure Interface Control Document.

Requirements:	P-DET-DS-ECAL.1, P-DET-DS-PID.5

References:

I-DET-DS-INF-INT.018	Space Constraint

System 1:	Backward EMCal Systems (6.03.01.04)

System 2:	Barrel DIRC Particle ID Systems (6.03.01.03)

Type:	SPACE

Description:	The exterior radius of the backward ECAL is limited by the interior bore of the DIRC support system. Modifications to either must be coordinated.

Provider:	Detector Infrastructure and Utilities Integration (6.03.01.09.04)

Details:	See details in the Detector Infrastructure Interface Control Document.

Requirements:	P-DET-DS-ECAL.1, P-DET-DS-PID.5

References:

I-DET-DS-INF-INT.019	Heat Mitigation

System 1:	Backward EMCal Systems (6.03.01.04)

System 2:	Backward Particle ID Systems (6.03.01.03)

Type:	ENV

Description:	The backward ECAL must mitigated the heat generated by the backward RICH detector.

Provider:	Detector Infrastructure and Utilities Integration (6.03.01.09.04)

Details:	See details in the Detector Infrastructure Interface Control Document.

Requirements:	P-DET-DS-ECAL.3

References:

I-DET-DS-INF-INT.020	Space Constraint

System 1:	Backward EMCal Systems (6.03.01.04)

System 2:	Backward RICH Detector Systems (6.03.01.03)

Type:	SPACE

Description:	The position of the backward ECAL in the forward direction is limited by the backward face of the pfRICH/mRICH detector.  Modifications to either must be coordinated.

Provider:	Detector Infrastructure and Utilities Integration (6.03.01.09.04)

Details:	See details in the Detector Infrastructure Interface Control Document.

Requirements:	P-DET-DS-ECAL.1, P-DET-DS-PID.5

References:

I-DET-DS-INF-INT.021	Backward ECAL Bore Clearance

System 1:	Backward EMCal Systems (6.03.01.04)

System 2:	IR Accelerator Systems (6.04.03)

Type:	SPACE

Description:	The bore of the backward ECAL must be designed to allow it to be inserted/removed over the existing beamline flanges.

Provider:	Detector Infrastructure and Utilities Integration (6.03.01.09.04)

Details:	See details in the Detector Infrastructure Interface Control Document.

Requirements:	P-DET-DS-ECAL.1

References:

I-DET-DS-INF-INT.022	Space Constraint

System 1:	Backward EMCal Systems (6.03.01.04)

System 2:	IR Accelerator Systems (6.04.03)

Type:	SPACE

Description:	The interior radius of the backward ECAL is governed by the size of the beamline.

Provider:	Detector Infrastructure and Utilities Integration (6.03.01.09.04)

Details:	See details in the Detector Infrastructure Interface Control Document.

Requirements:	P-DET-DS-ECAL.1

References:

I-DET-DS-INF-INT.023	Space Constraint

System 1:	Forward EMCal Systems (6.03.01.04)

System 2:	Barrel HCal Systems (6.03.01.05)

Type:	SPACE

Description:	The exterior radius of the forward ECAL is limited by the interior bore of the barrel Hadron Calorimeter. Modifications to either must be coordinated.

Provider:	Detector Infrastructure and Utilities Integration (6.03.01.09.04)

Details:	See details in the Detector Infrastructure Interface Control Document.

Requirements:	P-DET-DS-ECAL.1, P-DET-DS-HCAL.1

References:

I-DET-DS-INF-INT.024	Space Constraint

System 1:	Forward EMCal Systems (6.03.01.04)

System 2:	Forward HCal Systems (6.03.01.05)

Type:	SPACE

Description:	The forward position of the forward ECAL is limited by the forward Hadron Calorimeter. Modifications to either must be coordinated.

Provider:	Detector Infrastructure and Utilities Integration (6.03.01.09.04)

Details:	See details in the Detector Infrastructure Interface Control Document.

Requirements:	P-DET-DS-ECAL.1, P-DET-DS-HCAL.1

References:

I-DET-DS-INF-INT.025	Space Constraint

System 1:	Forward EMCal Systems (6.03.01.04)

System 2:	Forward RICH Detector Systems (6.03.01.03)

Type:	SPACE

Description:	The maximum forward location for the dRICH is limited by the forward ECAL  and the adjacent cabling pathway that provides services to the interior detectors. Modifications to either must be coordinated.

Provider:	Detector Infrastructure and Utilities Integration (6.03.01.09.04)

Details:	See details in the Detector Infrastructure Interface Control Document.

Requirements:	P-DET-DS-ECAL.1, P-DET-DS-PID.5

References:

I-DET-DS-INF-INT.026	Space Constraint

System 1:	Forward EMCal Systems (6.03.01.04)

System 2:	To Be Determined (TBD)

Type:	SPACE

Description:	The interior radius of the forward ECAL is governed by the size of the beamline. Modifications to either must be coordinated.

Provider:	Detector Infrastructure and Utilities Integration (6.03.01.09.04)

Details:	See details in the Detector Infrastructure Interface Control Document.

Requirements:	P-DET-DS-ECAL.1

References:

I-DET-DS-INF-INT.062	Imaging Layers Heat Rejection

System 1:	Detector Infrastructure and Utilities Integration (6.03.01.09.04)

System 2:	Barrel EMCal Systems (6.03.01.04)

Type:	COOL

Description:	The imaging layers must be cooled with an independent system (TBD).

Provider:	Detector Infrastructure and Utilities Integration (6.03.01.09.04)

Details:	See details in the Detector Infrastructure Interface Control Document.

Requirements:	P-DET-DS-ECAL.3

References:

I-DET-DS-INF-INT.063	Silicon PM Heat Rejection

System 1:	Detector Infrastructure and Utilities Integration (6.03.01.09.04)

System 2:	Barrel EMCal Systems (6.03.01.04)

Type:	COOL

Description:	The barrel ECAL will require chilled water or LCW cooling to maintain a safe operating temperature.

Provider:	Detector Infrastructure and Utilities Integration (6.03.01.09.04)

Details:	See details in the Detector Infrastructure Interface Control Document.

Requirements:	P-DET-DS-ECAL.3

References:

I-DET-DS-INF-INT.064	Heat Rejection

System 1:	Detector Infrastructure and Utilities Integration (6.03.01.09.04)

System 2:	Backward EMCal Systems (6.03.01.04)

Type:	COOL

Description:	The backward ECAL will require chilled water or LCW cooling to maintain the temperature of the silicon photomultipliers, the electronics, and the crystals.

Provider:	Detector Infrastructure and Utilities Integration (6.03.01.09.04)

Details:	See details in the Detector Infrastructure Interface Control Document.

Requirements:	P-DET-DS-ECAL.3

References:

I-DET-DS-INF-INT.065	Heat Rejection

System 1:	Detector Infrastructure and Utilities Integration (6.03.01.09.04)

System 2:	Forward EMCal Systems (6.03.01.04)

Type:	COOL

Description:	The forward ECAL will require chilled water or LCW cooling to maintain a safe operating temperature.

Provider:	Detector Infrastructure and Utilities Integration (6.03.01.09.04)

Details:	See details in the Detector Infrastructure Interface Control Document.

Requirements:	P-DET-DS-ECAL.3

References:

# Electromagnetic Calorimetry Systems (DET-DS-ECAL) to Solenoid Magnet (DET-DS-MAG)

I-DET-DS-INF-INT.012	Space Constraint

System 1:	Barrel EMCal Systems (6.03.01.04)

System 2:	Solenoid Magnet (6.03.01.06)

Type:	SPACE

Description:	The exterior radius of the barrel ECAL (and its support system) is limited by the interior bore of the solenoid magnet. Modifications to either must be coordinated.

Provider:	Detector Infrastructure and Utilities Integration (6.03.01.09.04)

Details:	See details in the Detector Infrastructure Interface Control Document.

Requirements:	P-DET-DS-ECAL.1, P-DET-DS-MAG.1

References:

# Electromagnetic Calorimetry Systems (DET-DS-ECAL) to Particle Identification Systems (DET-DS-PID)

I-DET-DS-INF-BAR.003	Support Structure

System 1:	Barrel EMCal Systems (6.03.01.04)

System 2:	Barrel DIRC Particle ID Systems (6.03.01.03)

Type:	STRUCT

Description:	The DIRC bar boxes will be supported by a frame inside the barrel Electromagnetic Calorimeter, that allows the boxes to be extracted using a system of rollers.

Provider:	Barrel Detector Structures (6.03.01.09.01)

Details:	See details in the Detector Infrastructure Interface Control Document.

Requirements:	P-DET-DS-ECAL.1, P-DET-DS-ECAL.2, P-DET-DS-PID.4, P-DET-DS-PID.5

References:

I-DET-DS-INF-BAR.005	Weight Transfer

System 1:	Backward EMCal Systems (6.03.01.04)

System 2:	Barrel DIRC Particle ID Systems (6.03.01.03)

Type:	STRUCT

Description:	The weight of the backward ECAL will be transferred to the DIRC detector support and must be accommodated by all subsequent support systems.

Provider:	Barrel Detector Structures (6.03.01.09.01)

Details:	See details in the Detector Infrastructure Interface Control Document.

Requirements:	P-DET-DS-ECAL.2, P-DET-DS-HCAL.2

References:

I-DET-DS-INF-BAR.006	EMCal Support System

System 1:	Backward EMCal Systems (6.03.01.04)

System 2:	Barrel DIRC Particle ID Systems (6.03.01.03)

Type:	STRUCT

Description:	The DIRC support system will provide support for the backward electromagnetic calorimeter.

Provider:	Barrel Detector Structures (6.03.01.09.01)

Details:	See details in the Detector Infrastructure Interface Control Document.

Requirements:	P-DET-DS-ECAL.2, P-DET-DS-PID.4

References:

I-DET-DS-INF-INT.013	Space Constraint

System 1:	Barrel EMCal Systems (6.03.01.04)

System 2:	Barrel DIRC Particle ID Systems (6.03.01.03)

Type:	SPACE

Description:	The backward position and shape of the barrel EMCAL is limited by the size and shape of the DIRC readout supports.  Changes to the size or position of either must be coordinated with the other.

Provider:	Detector Infrastructure and Utilities Integration (6.03.01.09.04)

Details:	See details in the Detector Infrastructure Interface Control Document.

Requirements:	P-DET-DS-ECAL.1, P-DET-DS-PID.5

References:

I-DET-DS-INF-INT.014	Space Constraint

System 1:	Barrel EMCal Systems (6.03.01.04)

System 2:	Barrel DIRC Particle ID Systems (6.03.01.03)

Type:	SPACE

Description:	The maximum size of the DIRC is limited to the interior bore of the barrel ECAL (and its support structures).  Modifications to either must be coordinated.

Provider:	Detector Infrastructure and Utilities Integration (6.03.01.09.04)

Details:	See details in the Detector Infrastructure Interface Control Document.

Requirements:	P-DET-DS-ECAL.1, P-DET-DS-PID.5

References:

I-DET-DS-INF-INT.015	Space Constraint

System 1:	Barrel EMCal Systems (6.03.01.04)

System 2:	Forward RICH Detector Systems (6.03.01.03)

Type:	SPACE

Description:	The forward position and shape of the barrel ECAL is limited  by the backward face of the Dual RICH detector and the adjacent cabling pathway that provides services to the interior detectors. Modifications to either must be coordinated.

Provider:	Detector Infrastructure and Utilities Integration (6.03.01.09.04)

Details:	See details in the Detector Infrastructure Interface Control Document.

Requirements:	P-DET-DS-ECAL.1, P-DET-DS-PID.5

References:

I-DET-DS-INF-INT.017	Space Constraint

System 1:	Backward EMCal Systems (6.03.01.04)

System 2:	Barrel DIRC Particle ID Systems (6.03.01.03)

Type:	SPACE

Description:	The maximum backward location for the DIRC  is limited by the position of the backward electromagnetic calorimeter. Modifications to either must be coordinated.

Provider:	Detector Infrastructure and Utilities Integration (6.03.01.09.04)

Details:	See details in the Detector Infrastructure Interface Control Document.

Requirements:	P-DET-DS-ECAL.1, P-DET-DS-PID.5

References:

I-DET-DS-INF-INT.018	Space Constraint

System 1:	Backward EMCal Systems (6.03.01.04)

System 2:	Barrel DIRC Particle ID Systems (6.03.01.03)

Type:	SPACE

Description:	The exterior radius of the backward ECAL is limited by the interior bore of the DIRC support system. Modifications to either must be coordinated.

Provider:	Detector Infrastructure and Utilities Integration (6.03.01.09.04)

Details:	See details in the Detector Infrastructure Interface Control Document.

Requirements:	P-DET-DS-ECAL.1, P-DET-DS-PID.5

References:

I-DET-DS-INF-INT.019	Heat Mitigation

System 1:	Backward EMCal Systems (6.03.01.04)

System 2:	Backward Particle ID Systems (6.03.01.03)

Type:	ENV

Description:	The backward ECAL must mitigated the heat generated by the backward RICH detector.

Provider:	Detector Infrastructure and Utilities Integration (6.03.01.09.04)

Details:	See details in the Detector Infrastructure Interface Control Document.

Requirements:	P-DET-DS-ECAL.3

References:

I-DET-DS-INF-INT.020	Space Constraint

System 1:	Backward EMCal Systems (6.03.01.04)

System 2:	Backward RICH Detector Systems (6.03.01.03)

Type:	SPACE

Description:	The position of the backward ECAL in the forward direction is limited by the backward face of the pfRICH/mRICH detector.  Modifications to either must be coordinated.

Provider:	Detector Infrastructure and Utilities Integration (6.03.01.09.04)

Details:	See details in the Detector Infrastructure Interface Control Document.

Requirements:	P-DET-DS-ECAL.1, P-DET-DS-PID.5

References:

I-DET-DS-INF-INT.025	Space Constraint

System 1:	Forward EMCal Systems (6.03.01.04)

System 2:	Forward RICH Detector Systems (6.03.01.03)

Type:	SPACE

Description:	The maximum forward location for the dRICH is limited by the forward ECAL  and the adjacent cabling pathway that provides services to the interior detectors. Modifications to either must be coordinated.

Provider:	Detector Infrastructure and Utilities Integration (6.03.01.09.04)

Details:	See details in the Detector Infrastructure Interface Control Document.

Requirements:	P-DET-DS-ECAL.1, P-DET-DS-PID.5

References:

# Applicable Documents

List applicable documents considered to be part of this document. Applicable documents shall be obtained from the EIC SharePoint Document Center.

# Detailed change Log

| Revision | Date | Change |
| --- | --- | --- |

# References

- doi:10.17226/25171., National Academies of Sciences, Engineering, and Medicine, "An Assessment of U.S.-Based Electron-Ion Collider Science", The National Academies Press, Washington, DC, 2018.

- EIC-ORG-PLN-010, Electron-Ion Collider L1 Requirements (Global Requirements Document)

- BNL-221006-2021-FORE, EIC Conceptual Design Report

- arXiv:2103.05419, Science Requirements and Detector Concepts for the Electron-Ion Collider: EIC Yellow Report

- EIC-SEG-PLN-016, Requirements Management Plan

- EIC-SEG-PLN-022, Systems Engineering Management Plan

- EIC-SEG-PLN-020, EIC Interface Management Plan