Electron-Ion Collider, Requirements, Specifications, and Interfaces

Interface Control Document: [###]

Electronic Systems (6.03.01.07) to Detector System (6.03.01)

Prepared by:

{{Sig_es_:signer1:signature}}	Date: {{Dte_es_:signer1:date}}

Name, Role

Electron-Ion Collider at Institution

Reviewed by:

{{Sig_es_:signer2:signature}}	Date: {{Dte_es_:signer2:date}}

Name, Role

Electron-Ion Collider at Institution

{{Sig_es_:signer3:signature}}	Date: {{Dte_es_:signer3:date}}

Name, Role

Electron-Ion Collider at Institution

{{Sig_es_:signer4:signature}}	Date: {{Dte_es_:signer4:date}}

Name, Role

Electron-Ion Collider at Institution

Approved by:

{{Sig_es_:signer5:signature}}	Date: {{Dte_es_:signer5:date}}

Name, Role

Electron-Ion Collider at Institution

REVISION HISTORY

| Revision # | Effective Date | Additional Reviewers | Summary of Change |
| --- | --- | --- | --- |
| 00 |  |  | Initial release. |

TABLE OF CONTENTS

1.	Purpose and Scope	5

2.	Introduction	5

2.1	Electronic Systems	5

2.2	Overview of Interface Relationships	5

2.3	Interface Responsibilities	6

2.4	Safety and Environmental Considerations	6

2.5	Design Considerations	6

2.6	Organization	6

3.	Interface summary	6

4.	Electronic Systems (DET-DS-ELEC) to Ancillary Detectors (DET-DS-ANC)	21

5.	Electronic Systems (DET-DS-ELEC) to Data Acquisition and Computing Systems (DET-DS-COMP)	26

6.	Electronic Systems (DET-DS-ELEC) to Electromagnetic Calorimetry Systems (DET-DS-ECAL)	26

7.	Electronic Systems (DET-DS-ELEC) to Hadronic Calorimetry Systems (DET-DS-HCAL)	29

8.	Electronic Systems (DET-DS-ELEC) to Detector Infrastructure (DET-DS-INF)	31

9.	Electronic Systems (DET-DS-ELEC) to Solenoid Magnet (DET-DS-MAG)	33

10.	Electronic Systems (DET-DS-ELEC) to Particle Identification Systems (DET-DS-PID)	35

11.	Electronic Systems (DET-DS-ELEC) to Polarimetry and Luminosity (DET-DS-POL)	38

12.	Electronic Systems (DET-DS-ELEC) to Tracking Systems (DET-DS-TRAK)	41

13.	Applicable Documents	42

14.	Detailed change Log	42

15.	References	42

LIST OF ACRONYMS

| B0 | B0 is the name of the forward magnet just past the Hadron Endcap |  |
| --- | --- | --- |
| CDR | Conceptual Design Report |  |
| COTS | Commercial Off the Shelf |  |
| DAQ | Data Acquisition |  |
| DET | EIC Detector |  |
| DIRC | Detection of Internally Reflected Cherenkov Light |  |
| dRICH | Dual radiator ring imaging Cherenkov |  |
| DVCS | Deeply Virtual Compton Scattering |  |
| EIC | Electron-Ion Collider |  |
| EMCAL | Electro-magnetic Calorimeter |  |
| ESR | Electron Storage Ring |  |
| FCC | Federal Communications Commission |  |
| FDR | Final Design Review |  |
| GEM | Gas Electron Multiplier |  |
| HCAL | Hadronic Calorimeter |  |
| HSR | Hadron Storage Ring |  |
| HVAC | Heating/Ventilation/Air Conditioning |  |
| IP | Interaction Point |  |
| IR | Interaction Region |  |
| MPGD | Micro Pattern Gaseous Detectors |  |
| NAS | National Academy of Sciences |  |
| NEC | National Electric Code |  |
| NEMA | National Electrical Manufacturers Association |  |
| NFPA | National Fire Protection Association |  |
| NRTL | Nationally Recognized Testing Laboratory |  |
| PID | Particle Identification |  |
| PRR | Production Readiness Review |  |
| PWO | PbWO4 |  |
| RCS | Rapid Cycling Synchrotron |  |
| TBD | To Be Determined |  |
| UL | Underwriter Laboratories |  |
| WBS | Work Break Down Structure |  |

# Interface Control Document:

Electronic Systems (6.03.01.07) to Detector System (6.03.01)

# Purpose and Scope

This document records the agreed upon interfaces between the Electronic Systems and other sub-systems within the Detector System. These interfaces are based on the system requirements that are necessary to integrate these sub-systems as part of the Electron Ion Collider Project.

# Introduction

These interfaces describe the relationships between the Electronic Systems and the Detector System including (examples by inclusion) located in (areas by inclusion).

## Electronic Systems

Provide an introductory description of your sub-system and an overview of the things that it receives, provides, supports, constrains, or otherwise requires as part of its function --- specifically if these things are manifested as interfaces.

Ancillary Detectors (DET-DS-ANC): Description of the TYPES of interfaces that exist between the ICD reference system and this sub-system, and a BRIEF overview of the roles they play.

Data Acquisition and Computing Systems (DET-DS-COMP): Description of the TYPES of interfaces that exist between the ICD reference system and this sub-system, and a BRIEF overview of the roles they play.

Electromagnetic Calorimetry Systems (DET-DS-ECAL): Description of the TYPES of interfaces that exist between the ICD reference system and this sub-system, and a BRIEF overview of the roles they play.

Hadronic Calorimetry Systems (DET-DS-HCAL): Description of the TYPES of interfaces that exist between the ICD reference system and this sub-system, and a BRIEF overview of the roles they play.

Detector Infrastructure (DET-DS-INF): Description of the TYPES of interfaces that exist between the ICD reference system and this sub-system, and a BRIEF overview of the roles they play.

Solenoid Magnet (DET-DS-MAG): Description of the TYPES of interfaces that exist between the ICD reference system and this sub-system, and a BRIEF overview of the roles they play.

Particle Identification Systems (DET-DS-PID): Description of the TYPES of interfaces that exist between the ICD reference system and this sub-system, and a BRIEF overview of the roles they play.

Polarimetry and Luminosity (DET-DS-POL): Description of the TYPES of interfaces that exist between the ICD reference system and this sub-system, and a BRIEF overview of the roles they play.

Tracking Systems (DET-DS-TRAK): Description of the TYPES of interfaces that exist between the ICD reference system and this sub-system, and a BRIEF overview of the roles they play.

## Overview of Interface Relationships

The following are the sub-systems within the Detector System that have interfaces connected to the Electronic Systems.

## Interface Responsibilities

The various ownerships of responsibilities may be listed here. However, this section might also refer to the Provider section of the interface detail if all pertinent responsibilities are listed there. If this is the case, simply enter: “See the Provider section within the interface details for information about the responsibilities associated with individual interfaces.”

## Safety and Environmental Considerations

General safety and environmental considerations are identified here. Any considerations that are specific to the individual sub-system, will be addressed in external safety documents and are referenced here.

### Considerations for all systems

Add general safety and environmental considerations here.

### Special considerations and references

Add references here as necessary.

## Design Considerations

Notes here about general design considerations.

## Organization

The interfaces in this document are grouped by sub-system as identified in the sub-system identifier. All interfaces between the Electronic Systems and a related sub-system will be included in a single section. Entries within that section will be sorted by the interface identifier or WBS number, as appropriate.

# Interface summary

This tables in this section provide a list of all interfaces that are defined within this document. The interfaces are grouped by sub-system/WBS. Each table has the following fields:

Section: the section number in this document where the interface is addressed.

WBS (1): the WBS number within the ICD reference system, unless the reference system is the owner.

WBS (2): the WBS number for the connected system.

ID: the interface identifier as defined in the Interface Requirement Document (IRD).

Name: the name of the interface as defined in the IRD.

Description: the description of the interface as defined in the IRD.

Owner: identifies the WBS number of the entity that is responsible for ensuring the interface is provided and that it meets validation/verification requirements.

Electronic Systems to Ancillary Detectors

| Section | WBS1 | WBS2 | ID | Name | Description | Owner |
| --- | --- | --- | --- | --- | --- | --- |
| 4 |  | 6.03.01.10 |  |  | Ancillary Detectors |  |
| 4.1 | 6.03.01.07 | 6.03.01.10 | I-DET-DS-ELEC.001 | Bias Voltage | The detector will receive bias voltage DC power provided from the electronics racks to support electronics. | 6.03.01.07 |
| 4.2 | 6.03.01.07 | 6.03.01.10 | I-DET-DS-ELEC.002 | High Voltage | The detector will receive high voltage DC power provided by the Detector Electronics group to support silicon sensors and calorimeter. | 6.03.01.07 |
| 4.3 | 6.03.01.07 | 6.03.01.10 | I-DET-DS-ELEC.003 | Low Voltage | The detector will receive low voltage DC power provided by the Detector Electronics group. | 6.03.01.07 |
| 4.4 | 6.03.01.07 | 6.03.01.10 | I-DET-DS-ELEC.004 | Bias Voltage | The detector will receive bias voltage DC power provided by the Detector Electronics group to support electronics. | 6.03.01.07 |
| 4.5 | 6.03.01.07 | 6.03.01.10 | I-DET-DS-ELEC.005 | High Voltage | The detector will receive high voltage DC power provided by the Detector Electronics group to support silicon sensors and calorimeter. | 6.03.01.07 |
| 4.6 | 6.03.01.07 | 6.03.01.10 | I-DET-DS-ELEC.006 | Low Voltage | The detector will receive low voltage DC power provided by the Detector Electronics group. | 6.03.01.07 |
| 4.7 | 6.03.01.07 | 6.03.01.10 | I-DET-DS-ELEC.007 | Bias Voltage | The detector will receive bias voltage DC power provided by the Detector Electronics group to support electronics. | 6.03.01.07 |
| 4.8 | 6.03.01.07 | 6.03.01.10 | I-DET-DS-ELEC.008 | High Voltage | The detector will receive high voltage DC power provided by the Detector Electronics group to support silicon sensors and calorimeter. | 6.03.01.07 |
| 4.9 | 6.03.01.07 | 6.03.01.10 | I-DET-DS-ELEC.009 | Low Voltage | The detector will receive low voltage DC power provided by the Detector Electronics group. | 6.03.01.07 |
| 4.10 | 6.03.01.07 | 6.03.01.10 | I-DET-DS-ELEC.010 | Bias Voltage | The detector will receive bias voltage DC power provided by the Detector Electronics group to support electronics. | 6.03.01.07 |
| 4.11 | 6.03.01.07 | 6.03.01.10 | I-DET-DS-ELEC.011 | High Voltage | The detector will receive high voltage DC power provided by the Detector Electronics group to support silicon sensors and calorimeter. | 6.03.01.07 |
| 4.12 | 6.03.01.07 | 6.03.01.10 | I-DET-DS-ELEC.012 | Low Voltage | The detector will receive low voltage DC power provided by the Detector Electronics group. | 6.03.01.07 |
| 4.13 | 6.03.01.07 | 6.03.01.10 | I-DET-DS-ELEC.013 | Bias Voltage | The detector will receive bias voltage DC power provided by the Detector Electronics group to support electronics. | 6.03.01.07 |
| 4.14 | 6.03.01.07 | 6.03.01.10 | I-DET-DS-ELEC.014 | High Voltage | The detector will receive high voltage DC power provided by the Detector Electronics group to support silicon sensors and calorimeter. | 6.03.01.07 |
| 4.15 | 6.03.01.07 | 6.03.01.10 | I-DET-DS-ELEC.015 | Low Voltage | The detector will receive low voltage DC power provided by the Detector Electronics group. | 6.03.01.07 |
| 4.16 | 6.03.01.07 | 6.03.01.10 | I-DET-DS-ELEC.016 | Bias Voltage | The detector will receive bias voltage DC power provided by the Detector Electronics group to support electronics. | 6.03.01.07 |
| 4.17 | 6.03.01.07 | 6.03.01.10 | I-DET-DS-ELEC.017 | High Voltage | The detector will receive high voltage DC power provided by the Detector Electronics group to support silicon sensors and calorimeter. | 6.03.01.07 |
| 4.18 | 6.03.01.07 | 6.03.01.10 | I-DET-DS-ELEC.018 | Low Voltage | The detector will receive low voltage DC power provided by the Detector Electronics group. | 6.03.01.07 |

Electronic Systems to Data Acquisition and Computing Systems

| Section | WBS1 | WBS2 | ID | Name | Description | Owner |
| --- | --- | --- | --- | --- | --- | --- |
| 5 |  | 6.03.01.08 |  |  | Data Acquisition and Computing Systems |  |
| 5.1 | 6.03.01.07 | 6.03.01.08 | I-DET-DS-COMP-ONLINE.029 | Monitoring Systems | Monitoring system cables will run from the south platform to the DAQ control room to allow monitoring of the power supplies and the electronics. | 6.03.01.08.02 |
| 5.2 | 6.03.01.07 | 6.03.01.08 | I-DET-DS-COMP-ONLINE.030 | Slow Controls | Controls cables allowing fast shutdown and machine interlocks will run from the south platform to the DAQ control room. | 6.03.01.08.02 |

Electronic Systems to Electromagnetic Calorimetry Systems

| Section | WBS1 | WBS2 | ID | Name | Description | Owner |
| --- | --- | --- | --- | --- | --- | --- |
| 6 |  | 6.03.01.04 |  |  | Electromagnetic Calorimetry Systems |  |
| 6.1 | 6.03.01.07 | 6.03.01.04 | I-DET-DS-ELEC.019 | Bias Voltage | Bias voltage DC power provided from the electronics racks to support electronics in the silicon photomultipliers. | 6.03.01.07 |
| 6.2 | 6.03.01.07 | 6.03.01.04 | I-DET-DS-ELEC.020 | High Voltage | High voltage DC power will be provided from the electronics racks to support silicon sensors and gas detectors. | 6.03.01.07 |
| 6.3 | 6.03.01.07 | 6.03.01.04 | I-DET-DS-ELEC.021 | Low Voltage | Low voltage DC power provided from the electronics racks to support electronics in the detector. | 6.03.01.07 |
| 6.4 | 6.03.01.07 | 6.03.01.04 | I-DET-DS-ELEC.022 | Bias Voltage | Bias voltage DC power will be provided from the electronics racks to support electronics the silicon photomultipliers. | 6.03.01.07 |
| 6.5 | 6.03.01.07 | 6.03.01.04 | I-DET-DS-ELEC.023 | High Voltage | High voltage DC power will be provided from the electronics racks to support silicon sensors and gas detectors. | 6.03.01.07 |
| 6.6 | 6.03.01.07 | 6.03.01.04 | I-DET-DS-ELEC.024 | Low Voltage | Low voltage DC power will be provided from the electronics racks to support electronics in the detector. | 6.03.01.07 |
| 6.7 | 6.03.01.07 | 6.03.01.04 | I-DET-DS-ELEC.025 | Bias Voltage | Bias voltage DC power provided from the electronics racks to support electronics in the silicon photomultipliers. | 6.03.01.07 |
| 6.8 | 6.03.01.07 | 6.03.01.04 | I-DET-DS-ELEC.026 | High Voltage | High voltage DC power will be provided from the electronics racks to support silicon sensors and gas detectors. | 6.03.01.07 |
| 6.9 | 6.03.01.07 | 6.03.01.04 | I-DET-DS-ELEC.027 | Low Voltage | Low voltage DC power provided from the electronics racks to support electronics in the detector. | 6.03.01.07 |

Electronic Systems to Hadronic Calorimetry Systems

| Section | WBS1 | WBS2 | ID | Name | Description | Owner |
| --- | --- | --- | --- | --- | --- | --- |
| 7 |  | 6.03.01.05 |  |  | Hadronic Calorimetry Systems |  |
| 7.1 | 6.03.01.07 | 6.03.01.05 | I-DET-DS-ELEC.028 | Bias Voltage | Bias voltage DC power will be provided from the electronics racks to support the silicon photomultipliers. | 6.03.01.07 |
| 7.2 | 6.03.01.07 | 6.03.01.05 | I-DET-DS-ELEC.029 | High Voltage | High voltage DC power will be provided from the electronics racks to support silicon sensors and gas detectors. | 6.03.01.07 |
| 7.3 | 6.03.01.07 | 6.03.01.05 | I-DET-DS-ELEC.030 | Low Voltage | Low voltage DC power will be provided from the electronics racks to support electronics in the detector. | 6.03.01.07 |
| 7.4 | 6.03.01.07 | 6.03.01.05 | I-DET-DS-ELEC.031 | Bias Voltage | Bias voltage DC power will be provided from the electronics racks to support the silicon photomultipliers. | 6.03.01.07 |
| 7.5 | 6.03.01.07 | 6.03.01.05 | I-DET-DS-ELEC.032 | High Voltage | High voltage DC power will be provided from the electronics racks to support silicon sensors and gas detectors. | 6.03.01.07 |
| 7.6 | 6.03.01.07 | 6.03.01.05 | I-DET-DS-ELEC.033 | Low Voltage | Low voltage DC power will be provided from the electronics racks to support electronics in the detector. | 6.03.01.07 |
| 7.7 | 6.03.01.07 | 6.03.01.05 | I-DET-DS-ELEC.034 | Bias Voltage | Bias voltage DC power will be provided from the electronics racks to support the silicon photomultipliers. | 6.03.01.07 |
| 7.8 | 6.03.01.07 | 6.03.01.05 | I-DET-DS-ELEC.035 | High Voltage | High voltage DC power will be provided from the electronics racks to support silicon sensors and gas detectors. | 6.03.01.07 |
| 7.9 | 6.03.01.07 | 6.03.01.05 | I-DET-DS-ELEC.036 | Low Voltage | Low voltage DC power will be provided from the electronics racks to support electronics in the detector. | 6.03.01.07 |

Electronic Systems to Detector Infrastructure

| Section | WBS1 | WBS2 | ID | Name | Description | Owner |
| --- | --- | --- | --- | --- | --- | --- |
| 8 |  | 6.03.01.09 |  |  | Detector Infrastructure |  |
| 8.1 | 6.03.01.07 | 6.03.01.09 | I-DET-DS-INF-INT.066 | Clean A/C Power | Electronics components on the carriage will require clean or transformer isolated power. | 6.03.01.09.04 |
| 8.2 | 6.03.01.07 | 6.03.01.09 | I-DET-DS-INF-INT.067 | UPS Power | Critical systems, such as controls, will require sufficient UPS power to allow them to be shutdown graceful, or to tolerate transients. | 6.03.01.09.04 |
| 8.3 | 6.03.01.07 | 6.03.01.09 | I-DET-DS-INF-INT.068 | Computing Environment | The electronics and computing systems will require a stable environment in terms of temperature and humidity. | 6.03.01.09.04 |
| 8.4 | 6.03.01.07 | 6.03.01.09 | I-DET-DS-INF-INT.069 | EMI Shielding | The electronics and computing systems on the carriage may require some EMI shielding from detectors and other components. | 6.03.01.09.04 |
| 8.5 | 6.03.01.07 | 6.03.01.09 | I-DET-DS-INF-INT.070 | Cabling Pathways | Adequate space for cables and cable trays will be required to deliver low voltage and high voltage, fiber and signal cables from the carriage/platform to the sub-detectors. | 6.03.01.09.04 |
| 8.6 | 6.03.01.07 | 6.03.01.09 | I-DET-DS-INF-INT.071 | Platform Rack Space | Adequate space must be provided for computing enclosures (racks) on the south platform to support the electronics systems. | 6.03.01.09.04 |

Electronic Systems to Solenoid Magnet

| Section | WBS1 | WBS2 | ID | Name | Description | Owner |
| --- | --- | --- | --- | --- | --- | --- |
| 9 |  | 6.03.01.06 |  |  | Solenoid Magnet |  |
| 9.1 | 6.03.01.07 | 6.03.01.06 | I-DET-DS-ELEC.037 | Bias Voltage | Bias voltage DC power will be provided from the electronics racks to support the magnet cryogenic systems. | 6.03.01.07 |
| 9.2 | 6.03.01.07 | 6.03.01.06 | I-DET-DS-ELEC.038 | High Voltage | High voltage DC power will be provided from the electronics racks to support the magnet instrumentation and control systems. | 6.03.01.07 |
| 9.3 | 6.03.01.07 | 6.03.01.06 | I-DET-DS-ELEC.039 | Low Voltage | Lower voltage DC power will be provided from the electronics racks to support the magnet cryogenic systems. | 6.03.01.07 |
| 9.4 | 6.03.01.07 | 6.03.01.06 | I-DET-DS-ELEC.040 | Bias Voltage | Bias voltage DC power will be provided from the electronics racks to support the magnet instrumentation and control systems. | 6.03.01.07 |
| 9.5 | 6.03.01.07 | 6.03.01.06 | I-DET-DS-ELEC.041 | High Voltage | High voltage DC power will be provided from the electronics racks to support the magnet power supply. | 6.03.01.07 |
| 9.6 | 6.03.01.07 | 6.03.01.06 | I-DET-DS-ELEC.042 | Low Voltage | Low voltage DC power will be provided from the electronics racks to support the magnet instrumentation and control systems. | 6.03.01.07 |
| 9.7 | 6.03.01.07 | 6.03.01.06 | I-DET-DS-ELEC.043 | Bias Voltage | Bias voltage DC power will be provided from the electronics racks to support the magnet power supply. | 6.03.01.07 |
| 9.8 | 6.03.01.07 | 6.03.01.06 | I-DET-DS-ELEC.044 | High Voltage | High voltage DC power will be provided from the electronics racks to support electronics in the detector. | 6.03.01.07 |
| 9.9 | 6.03.01.07 | 6.03.01.06 | I-DET-DS-ELEC.045 | Low Voltage | Low voltage DC power will be provided from the electronics racks to support the magnet power supply. | 6.03.01.07 |

Electronic Systems to Particle Identification Systems

| Section | WBS1 | WBS2 | ID | Name | Description | Owner |
| --- | --- | --- | --- | --- | --- | --- |
| 10 |  | 6.03.01.03 |  |  | Particle Identification Systems |  |
| 10.1 | 6.03.01.07 | 6.03.01.03 | I-DET-DS-ELEC.046 | Bias Voltage | Bias voltage DC power will be provided from the electronics racks to support electronics the silicon photomultipliers. | 6.03.01.07 |
| 10.2 | 6.03.01.07 | 6.03.01.03 | I-DET-DS-ELEC.047 | High Voltage | High voltage DC power will be provided from the electronics racks to support silicon sensors and gas detectors. | 6.03.01.07 |
| 10.3 | 6.03.01.07 | 6.03.01.03 | I-DET-DS-ELEC.048 | Low Voltage | Low voltage DC power will be provided from the electronics racks to support electronics in the detector. | 6.03.01.07 |
| 10.4 | 6.03.01.07 | 6.03.01.03 | I-DET-DS-ELEC.049 | High Voltage | High voltage power will be delivered from the power supplies on the carriage to the photosensors in the DIRC. | 6.03.01.07 |
| 10.5 | 6.03.01.07 | 6.03.01.03 | I-DET-DS-ELEC.050 | Low Voltage | Low voltage power will be delivered from the power supplies on the carriage to the DIRC electronics. | 6.03.01.07 |
| 10.6 | 6.03.01.07 | 6.03.01.03 | I-DET-DS-ELEC.051 | High Voltage | High voltage power will be delivered from the power supplies on the carriage to the photosensors in the DIRC. | 6.03.01.07 |
| 10.7 | 6.03.01.07 | 6.03.01.03 | I-DET-DS-ELEC.052 | Low Voltage | Low voltage power will be delivered from the power supplies on the carriage to the DIRC electronics. | 6.03.01.07 |
| 10.8 | 6.03.01.07 | 6.03.01.03 | I-DET-DS-ELEC.053 | High Voltage | High voltage power will be delivered from the power supplies on the carriage to the photosensors in the pfRICH/mRICH. | 6.03.01.07 |
| 10.9 | 6.03.01.07 | 6.03.01.03 | I-DET-DS-ELEC.054 | Low Voltage | Low voltage power will be delivered from the power supplies on the carriage to the pfRICH/mRICH electronics. | 6.03.01.07 |
| 10.10 | 6.03.01.07 | 6.03.01.03 | I-DET-DS-ELEC.055 | Bias Voltage | Bias voltage DC power will be delivered from the power supplies on the carriage to the photosensors. | 6.03.01.07 |
| 10.11 | 6.03.01.07 | 6.03.01.03 | I-DET-DS-ELEC.056 | Low Voltage | Low voltage power will be delivered from the power supplies on the carriage to the dRICH electronics. | 6.03.01.07 |

Electronic Systems to Polarimetry and Luminosity

| Section | WBS1 | WBS2 | ID | Name | Description | Owner |
| --- | --- | --- | --- | --- | --- | --- |
| 11 |  | 6.03.01.10 |  |  | Polarimetry and Luminosity |  |
| 11.1 | 6.03.01.07 | 6.03.01.10 | I-DET-DS-ELEC.057 | Bias Voltage | Bias voltage DC power will be provided from the electronics racks to support electronics the silicon photomultipliers. | 6.03.01.07 |
| 11.2 | 6.03.01.07 | 6.03.01.10 | I-DET-DS-ELEC.058 | High Voltage | High voltage DC power will be provided from the electronics racks to support silicon sensors and gas detectors. | 6.03.01.07 |
| 11.3 | 6.03.01.07 | 6.03.01.10 | I-DET-DS-ELEC.059 | Low Voltage | Low voltage DC power will be provided from the electronics racks to support electronics in the detector. | 6.03.01.07 |
| 11.4 | 6.03.01.07 | 6.03.01.10 | I-DET-DS-ELEC.060 | Bias Voltage | Bias voltage DC power will be provided from the electronics racks to support electronics the silicon photomultipliers. | 6.03.01.07 |
| 11.5 | 6.03.01.07 | 6.03.01.10 | I-DET-DS-ELEC.061 | High Voltage | High voltage DC power will be provided from the electronics racks to support silicon sensors and gas detectors. | 6.03.01.07 |
| 11.6 | 6.03.01.07 | 6.03.01.10 | I-DET-DS-ELEC.062 | Low Voltage | Low voltage DC power will be provided from the electronics racks to support electronics in the detector. | 6.03.01.07 |
| 11.7 | 6.03.01.07 | 6.03.01.10 | I-DET-DS-ELEC.063 | Bias Voltage | Bias voltage DC power will be provided from the electronics racks to support electronics the silicon photomultipliers. | 6.03.01.07 |
| 11.8 | 6.03.01.07 | 6.03.01.10 | I-DET-DS-ELEC.064 | High Voltage | High voltage DC power will be provided from the electronics racks to support silicon sensors and gas detectors. | 6.03.01.07 |
| 11.9 | 6.03.01.07 | 6.03.01.10 | I-DET-DS-ELEC.065 | Low Voltage | Low voltage DC power will be provided from the electronics racks to support electronics in the detector. | 6.03.01.07 |
| 11.10 | 6.03.01.07 | 6.03.01.10 | I-DET-DS-ELEC.066 | Bias Voltage | Bias voltage DC power will be provided from the electronics racks to support electronics the silicon photomultipliers. | 6.03.01.07 |
| 11.11 | 6.03.01.07 | 6.03.01.10 | I-DET-DS-ELEC.067 | High Voltage | High voltage DC power will be provided from the electronics racks to support silicon sensors and gas detectors. | 6.03.01.07 |
| 11.12 | 6.03.01.07 | 6.03.01.10 | I-DET-DS-ELEC.068 | Low Voltage | Low voltage DC power will be provided from the electronics racks to support electronics in the detector. | 6.03.01.07 |

Electronic Systems to Tracking Systems

| Section | WBS1 | WBS2 | ID | Name | Description | Owner |
| --- | --- | --- | --- | --- | --- | --- |
| 12 |  | 6.03.01.02 |  |  | Tracking Systems |  |
| 12.1 | 6.03.01.07 | 6.03.01.02 | I-DET-DS-ELEC.069 | Bias Voltage | Bias voltage DC power will be provided from the electronics racks to support electronics in the silicon photomultipliers. | 6.03.01.07 |
| 12.2 | 6.03.01.07 | 6.03.01.02 | I-DET-DS-ELEC.070 | High Voltage | High voltage DC power will be provided from the electronics racks to support silicon sensors and gas detectors. | 6.03.01.07 |
| 12.3 | 6.03.01.07 | 6.03.01.02 | I-DET-DS-ELEC.071 | Low Voltage | Low voltage DC power will be provided from the electronics racks to support electronics in the detector. | 6.03.01.07 |

# Electronic Systems (DET-DS-ELEC) to Ancillary Detectors (DET-DS-ANC)

I-DET-DS-ELEC.001	Bias Voltage

System 1:	Electronic Systems (6.03.01.07)

System 2:	B-Zero Detectors (6.03.01.10)

Type:	ELEC

Description:	The detector will receive bias voltage DC power provided from the electronics racks to support electronics.

Provider:	Electronic Systems (6.03.01.07)

Details:

Requirements:	P-DET-DS-ANC.4

References:

I-DET-DS-ELEC.002	High Voltage

System 1:	Electronic Systems (6.03.01.07)

System 2:	B-Zero Detectors (6.03.01.10)

Type:	ELEC

Description:	The detector will receive high voltage DC power provided by the Detector Electronics group to support silicon sensors and calorimeter.

Provider:	Electronic Systems (6.03.01.07)

Details:

Requirements:	P-DET-DS-ANC.4

References:

I-DET-DS-ELEC.003	Low Voltage

System 1:	Electronic Systems (6.03.01.07)

System 2:	B-Zero Detectors (6.03.01.10)

Type:	ELEC

Description:	The detector will receive low voltage DC power provided by the Detector Electronics group.

Provider:	Electronic Systems (6.03.01.07)

Details:

Requirements:	P-DET-DS-ANC.4

References:

I-DET-DS-ELEC.004	Bias Voltage

System 1:	Electronic Systems (6.03.01.07)

System 2:	Low Q2 Detectors (6.03.01.10)

Type:	ELEC

Description:	The detector will receive bias voltage DC power provided by the Detector Electronics group to support electronics.

Provider:	Electronic Systems (6.03.01.07)

Details:

Requirements:	P-DET-DS-ANC.4

References:

I-DET-DS-ELEC.005	High Voltage

System 1:	Electronic Systems (6.03.01.07)

System 2:	Low Q2 Detectors (6.03.01.10)

Type:	ELEC

Description:	The detector will receive high voltage DC power provided by the Detector Electronics group to support silicon sensors and calorimeter.

Provider:	Electronic Systems (6.03.01.07)

Details:

Requirements:	P-DET-DS-ANC.4

References:

I-DET-DS-ELEC.006	Low Voltage

System 1:	Electronic Systems (6.03.01.07)

System 2:	Low Q2 Detectors (6.03.01.10)

Type:	ELEC

Description:	The detector will receive low voltage DC power provided by the Detector Electronics group.

Provider:	Electronic Systems (6.03.01.07)

Details:

Requirements:	P-DET-DS-ANC.4

References:

I-DET-DS-ELEC.007	Bias Voltage

System 1:	Electronic Systems (6.03.01.07)

System 2:	Luminosity Detectors (6.03.01.10)

Type:	ELEC

Description:	The detector will receive bias voltage DC power provided by the Detector Electronics group to support electronics.

Provider:	Electronic Systems (6.03.01.07)

Details:

Requirements:	P-DET-DS-ANC.4

References:

I-DET-DS-ELEC.008	High Voltage

System 1:	Electronic Systems (6.03.01.07)

System 2:	Luminosity Detectors (6.03.01.10)

Type:	ELEC

Description:	The detector will receive high voltage DC power provided by the Detector Electronics group to support silicon sensors and calorimeter.

Provider:	Electronic Systems (6.03.01.07)

Details:

Requirements:	P-DET-DS-ANC.4

References:

I-DET-DS-ELEC.009	Low Voltage

System 1:	Electronic Systems (6.03.01.07)

System 2:	Luminosity Detectors (6.03.01.10)

Type:	ELEC

Description:	The detector will receive low voltage DC power provided by the Detector Electronics group.

Provider:	Electronic Systems (6.03.01.07)

Details:

Requirements:	P-DET-DS-ANC.4

References:

I-DET-DS-ELEC.010	Bias Voltage

System 1:	Electronic Systems (6.03.01.07)

System 2:	Off-Momentum Detectors (6.03.01.10)

Type:	ELEC

Description:	The detector will receive bias voltage DC power provided by the Detector Electronics group to support electronics.

Provider:	Electronic Systems (6.03.01.07)

Details:

Requirements:	P-DET-DS-ANC.4

References:

I-DET-DS-ELEC.011	High Voltage

System 1:	Electronic Systems (6.03.01.07)

System 2:	Off-Momentum Detectors (6.03.01.10)

Type:	ELEC

Description:	The detector will receive high voltage DC power provided by the Detector Electronics group to support silicon sensors and calorimeter.

Provider:	Electronic Systems (6.03.01.07)

Details:

Requirements:	P-DET-DS-ANC.4

References:

I-DET-DS-ELEC.012	Low Voltage

System 1:	Electronic Systems (6.03.01.07)

System 2:	Off-Momentum Detectors (6.03.01.10)

Type:	ELEC

Description:	The detector will receive low voltage DC power provided by the Detector Electronics group.

Provider:	Electronic Systems (6.03.01.07)

Details:

Requirements:	P-DET-DS-ANC.4

References:

I-DET-DS-ELEC.013	Bias Voltage

System 1:	Electronic Systems (6.03.01.07)

System 2:	Roman Pots (6.03.01.10)

Type:	ELEC

Description:	The detector will receive bias voltage DC power provided by the Detector Electronics group to support electronics.

Provider:	Electronic Systems (6.03.01.07)

Details:

Requirements:	P-DET-DS-ANC.4

References:

I-DET-DS-ELEC.014	High Voltage

System 1:	Electronic Systems (6.03.01.07)

System 2:	Roman Pots (6.03.01.10)

Type:	ELEC

Description:	The detector will receive high voltage DC power provided by the Detector Electronics group to support silicon sensors and calorimeter.

Provider:	Electronic Systems (6.03.01.07)

Details:

Requirements:	P-DET-DS-ANC.4

References:

I-DET-DS-ELEC.015	Low Voltage

System 1:	Electronic Systems (6.03.01.07)

System 2:	Roman Pots (6.03.01.10)

Type:	ELEC

Description:	The detector will receive low voltage DC power provided by the Detector Electronics group.

Provider:	Electronic Systems (6.03.01.07)

Details:

Requirements:	P-DET-DS-ANC.4

References:

I-DET-DS-ELEC.016	Bias Voltage

System 1:	Electronic Systems (6.03.01.07)

System 2:	Zero Degree Calorimeter (6.03.01.10)

Type:	ELEC

Description:	The detector will receive bias voltage DC power provided by the Detector Electronics group to support electronics.

Provider:	Electronic Systems (6.03.01.07)

Details:

Requirements:	P-DET-DS-ANC.4

References:

I-DET-DS-ELEC.017	High Voltage

System 1:	Electronic Systems (6.03.01.07)

System 2:	Zero Degree Calorimeter (6.03.01.10)

Type:	ELEC

Description:	The detector will receive high voltage DC power provided by the Detector Electronics group to support silicon sensors and calorimeter.

Provider:	Electronic Systems (6.03.01.07)

Details:

Requirements:	P-DET-DS-ANC.4

References:

I-DET-DS-ELEC.018	Low Voltage

System 1:	Electronic Systems (6.03.01.07)

System 2:	Zero Degree Calorimeter (6.03.01.10)

Type:	ELEC

Description:	The detector will receive low voltage DC power provided by the Detector Electronics group.

Provider:	Electronic Systems (6.03.01.07)

Details:

Requirements:	P-DET-DS-ANC.4

References:

# Electronic Systems (DET-DS-ELEC) to Data Acquisition and Computing Systems (DET-DS-COMP)

I-DET-DS-COMP-ONLINE.029	Monitoring Systems

System 1:	Online DAQ and Computing Systems (6.03.01.08.02)

System 2:	Electronic Systems (6.03.01.07)

Type:	CONTROL

Description:	Monitoring system cables will run from the south platform to the DAQ control room to allow monitoring of the power supplies and the electronics.

Provider:	Online DAQ and Computing Systems (6.03.01.08.02)

Details:	See details in the Data Acquisition and Computing Systems Interface Control Document.

Requirements:	F-DET-DS-COMP.6

References:

I-DET-DS-COMP-ONLINE.030	Slow Controls

System 1:	Online DAQ and Computing Systems (6.03.01.08.02)

System 2:	Electronic Systems (6.03.01.07)

Type:	CONTROL

Description:	Controls cables allowing fast shutdown and machine interlocks will run from the south platform to the DAQ control room.

Provider:	Online DAQ and Computing Systems (6.03.01.08.02)

Details:	See details in the Data Acquisition and Computing Systems Interface Control Document.

Requirements:	F-DET-DS-COMP-SC.1

References:

# Electronic Systems (DET-DS-ELEC) to Electromagnetic Calorimetry Systems (DET-DS-ECAL)

I-DET-DS-ELEC.019	Bias Voltage

System 1:	Electronic Systems (6.03.01.07)

System 2:	Barrel EMCal Systems (6.03.01.04)

Type:	ELEC

Description:	Bias voltage DC power provided from the electronics racks to support electronics in the silicon photomultipliers.

Provider:	Electronic Systems (6.03.01.07)

Details:

Requirements:	P-DET-DS-ECAL.4

References:

I-DET-DS-ELEC.020	High Voltage

System 1:	Electronic Systems (6.03.01.07)

System 2:	Barrel EMCal Systems (6.03.01.04)

Type:	ELEC

Description:	High voltage DC power will be provided from the electronics racks to support silicon sensors and gas detectors.

Provider:	Electronic Systems (6.03.01.07)

Details:

Requirements:	P-DET-DS-ECAL.4

References:

I-DET-DS-ELEC.021	Low Voltage

System 1:	Electronic Systems (6.03.01.07)

System 2:	Barrel EMCal Systems (6.03.01.04)

Type:	ELEC

Description:	Low voltage DC power provided from the electronics racks to support electronics in the detector.

Provider:	Electronic Systems (6.03.01.07)

Details:

Requirements:	P-DET-DS-ECAL.4

References:

I-DET-DS-ELEC.022	Bias Voltage

System 1:	Electronic Systems (6.03.01.07)

System 2:	Backward EMCal Systems (6.03.01.04)

Type:	ELEC

Description:	Bias voltage DC power will be provided from the electronics racks to support electronics the silicon photomultipliers.

Provider:	Electronic Systems (6.03.01.07)

Details:

Requirements:	P-DET-DS-ECAL.4

References:

I-DET-DS-ELEC.023	High Voltage

System 1:	Electronic Systems (6.03.01.07)

System 2:	Backward EMCal Systems (6.03.01.04)

Type:	ELEC

Description:	High voltage DC power will be provided from the electronics racks to support silicon sensors and gas detectors.

Provider:	Electronic Systems (6.03.01.07)

Details:

Requirements:	P-DET-DS-ECAL.4

References:

I-DET-DS-ELEC.024	Low Voltage

System 1:	Electronic Systems (6.03.01.07)

System 2:	Backward EMCal Systems (6.03.01.04)

Type:	ELEC

Description:	Low voltage DC power will be provided from the electronics racks to support electronics in the detector.

Provider:	Electronic Systems (6.03.01.07)

Details:

Requirements:	P-DET-DS-ECAL.4

References:

I-DET-DS-ELEC.025	Bias Voltage

System 1:	Electronic Systems (6.03.01.07)

System 2:	Forward EMCal Systems (6.03.01.04)

Type:	ELEC

Description:	Bias voltage DC power provided from the electronics racks to support electronics in the silicon photomultipliers.

Provider:	Electronic Systems (6.03.01.07)

Details:

Requirements:	P-DET-DS-ECAL.4

References:

I-DET-DS-ELEC.026	High Voltage

System 1:	Electronic Systems (6.03.01.07)

System 2:	Forward EMCal Systems (6.03.01.04)

Type:	ELEC

Description:	High voltage DC power will be provided from the electronics racks to support silicon sensors and gas detectors.

Provider:	Electronic Systems (6.03.01.07)

Details:

Requirements:	P-DET-DS-ECAL.4

References:

I-DET-DS-ELEC.027	Low Voltage

System 1:	Electronic Systems (6.03.01.07)

System 2:	Forward EMCal Systems (6.03.01.04)

Type:	ELEC

Description:	Low voltage DC power provided from the electronics racks to support electronics in the detector.

Provider:	Electronic Systems (6.03.01.07)

Details:

Requirements:	P-DET-DS-ECAL.4

References:

# Electronic Systems (DET-DS-ELEC) to Hadronic Calorimetry Systems (DET-DS-HCAL)

I-DET-DS-ELEC.028	Bias Voltage

System 1:	Electronic Systems (6.03.01.07)

System 2:	Barrel HCal Systems (6.03.01.05)

Type:	ELEC

Description:	Bias voltage DC power will be provided from the electronics racks to support the silicon photomultipliers.

Provider:	Electronic Systems (6.03.01.07)

Details:

Requirements:	P-DET-DS-HCAL.4

References:

I-DET-DS-ELEC.029	High Voltage

System 1:	Electronic Systems (6.03.01.07)

System 2:	Barrel HCal Systems (6.03.01.05)

Type:	ELEC

Description:	High voltage DC power will be provided from the electronics racks to support silicon sensors and gas detectors.

Provider:	Electronic Systems (6.03.01.07)

Details:

Requirements:	P-DET-DS-HCAL.4

References:

I-DET-DS-ELEC.030	Low Voltage

System 1:	Electronic Systems (6.03.01.07)

System 2:	Barrel HCal Systems (6.03.01.05)

Type:	ELEC

Description:	Low voltage DC power will be provided from the electronics racks to support electronics in the detector.

Provider:	Electronic Systems (6.03.01.07)

Details:

Requirements:	P-DET-DS-HCAL.4

References:

I-DET-DS-ELEC.031	Bias Voltage

System 1:	Electronic Systems (6.03.01.07)

System 2:	Backward HCal Systems (6.03.01.05)

Type:	ELEC

Description:	Bias voltage DC power will be provided from the electronics racks to support the silicon photomultipliers.

Provider:	Electronic Systems (6.03.01.07)

Details:

Requirements:	P-DET-DS-HCAL.4

References:

I-DET-DS-ELEC.032	High Voltage

System 1:	Electronic Systems (6.03.01.07)

System 2:	Backward HCal Systems (6.03.01.05)

Type:	ELEC

Description:	High voltage DC power will be provided from the electronics racks to support silicon sensors and gas detectors.

Provider:	Electronic Systems (6.03.01.07)

Details:

Requirements:	P-DET-DS-HCAL.4

References:

I-DET-DS-ELEC.033	Low Voltage

System 1:	Electronic Systems (6.03.01.07)

System 2:	Backward HCal Systems (6.03.01.05)

Type:	ELEC

Description:	Low voltage DC power will be provided from the electronics racks to support electronics in the detector.

Provider:	Electronic Systems (6.03.01.07)

Details:

Requirements:	P-DET-DS-HCAL.4

References:

I-DET-DS-ELEC.034	Bias Voltage

System 1:	Electronic Systems (6.03.01.07)

System 2:	Forward HCal Systems (6.03.01.05)

Type:	ELEC

Description:	Bias voltage DC power will be provided from the electronics racks to support the silicon photomultipliers.

Provider:	Electronic Systems (6.03.01.07)

Details:

Requirements:	P-DET-DS-HCAL.4

References:

I-DET-DS-ELEC.035	High Voltage

System 1:	Electronic Systems (6.03.01.07)

System 2:	Forward HCal Systems (6.03.01.05)

Type:	ELEC

Description:	High voltage DC power will be provided from the electronics racks to support silicon sensors and gas detectors.

Provider:	Electronic Systems (6.03.01.07)

Details:

Requirements:	P-DET-DS-HCAL.4

References:

I-DET-DS-ELEC.036	Low Voltage

System 1:	Electronic Systems (6.03.01.07)

System 2:	Forward HCal Systems (6.03.01.05)

Type:	ELEC

Description:	Low voltage DC power will be provided from the electronics racks to support electronics in the detector.

Provider:	Electronic Systems (6.03.01.07)

Details:

Requirements:	P-DET-DS-HCAL.4

References:

# Electronic Systems (DET-DS-ELEC) to Detector Infrastructure (DET-DS-INF)

I-DET-DS-INF-INT.066	Clean A/C Power

System 1:	Detector Infrastructure and Utilities Integration (6.03.01.09.04)

System 2:	Electronic Systems (6.03.01.07)

Type:	ELEC

Description:	Electronics components on the carriage will require clean or transformer isolated power.

Provider:	Detector Infrastructure and Utilities Integration (6.03.01.09.04)

Details:	See details in the Detector Infrastructure Interface Control Document.

Requirements:	P-DET-DS-ELEC.4

References:

I-DET-DS-INF-INT.067	UPS Power

System 1:	Detector Infrastructure and Utilities Integration (6.03.01.09.04)

System 2:	Electronic Systems (6.03.01.07)

Type:	ELEC

Description:	Critical systems, such as controls, will require sufficient UPS power to allow them to be shutdown graceful, or to tolerate transients.

Provider:	Detector Infrastructure and Utilities Integration (6.03.01.09.04)

Details:	See details in the Detector Infrastructure Interface Control Document.

Requirements:	P-DET-DS-ELEC.4

References:

I-DET-DS-INF-INT.068	Computing Environment

System 1:	Detector Infrastructure and Utilities Integration (6.03.01.09.04)

System 2:	Electronic Systems (6.03.01.07)

Type:	ENV

Description:	The electronics and computing systems will require a stable environment in terms of temperature and humidity.

Provider:	Detector Infrastructure and Utilities Integration (6.03.01.09.04)

Details:	See details in the Detector Infrastructure Interface Control Document.

Requirements:	P-DET-DS-ELEC.3

References:

I-DET-DS-INF-INT.069	EMI Shielding

System 1:	Detector Infrastructure and Utilities Integration (6.03.01.09.04)

System 2:	Electronic Systems (6.03.01.07)

Type:	ENV

Description:	The electronics and computing systems on the carriage may require some EMI shielding from detectors and other components.

Provider:	Detector Infrastructure and Utilities Integration (6.03.01.09.04)

Details:	See details in the Detector Infrastructure Interface Control Document.

Requirements:	P-DET-DS-ELEC.6

References:

I-DET-DS-INF-INT.070	Cabling Pathways

System 1:	Detector Infrastructure and Utilities Integration (6.03.01.09.04)

System 2:	Electronic Systems (6.03.01.07)

Type:	SPACE

Description:	Adequate space for cables and cable trays will be required to deliver low voltage and high voltage, fiber and signal cables from the carriage/platform to the sub-detectors.

Provider:	Detector Infrastructure and Utilities Integration (6.03.01.09.04)

Details:	See details in the Detector Infrastructure Interface Control Document.

Requirements:	P-DET-DS-ELEC.1

References:

I-DET-DS-INF-INT.071	Platform Rack Space

System 1:	Detector Infrastructure and Utilities Integration (6.03.01.09.04)

System 2:	Electronic Systems (6.03.01.07)

Type:	SPACE

Description:	Adequate space must be provided for computing enclosures (racks) on the south platform to support the electronics systems.

Provider:	Detector Infrastructure and Utilities Integration (6.03.01.09.04)

Details:	See details in the Detector Infrastructure Interface Control Document.

Requirements:	P-DET-DS-ELEC.1

References:

# Electronic Systems (DET-DS-ELEC) to Solenoid Magnet (DET-DS-MAG)

I-DET-DS-ELEC.037	Bias Voltage

System 1:	Electronic Systems (6.03.01.07)

System 2:	Magnet Cryogenics (6.03.01.06)

Type:	ELEC

Description:	Bias voltage DC power will be provided from the electronics racks to support the magnet cryogenic systems.

Provider:	Electronic Systems (6.03.01.07)

Details:

Requirements:	P-DET-DS-MAG.6

References:

I-DET-DS-ELEC.038	High Voltage

System 1:	Electronic Systems (6.03.01.07)

System 2:	Magnet Cryogenics (6.03.01.06)

Type:	ELEC

Description:	High voltage DC power will be provided from the electronics racks to support the magnet instrumentation and control systems.

Provider:	Electronic Systems (6.03.01.07)

Details:

Requirements:	P-DET-DS-MAG.6

References:

I-DET-DS-ELEC.039	Low Voltage

System 1:	Electronic Systems (6.03.01.07)

System 2:	Magnet Cryogenics (6.03.01.06)

Type:	ELEC

Description:	Lower voltage DC power will be provided from the electronics racks to support the magnet cryogenic systems.

Provider:	Electronic Systems (6.03.01.07)

Details:

Requirements:	P-DET-DS-MAG.6

References:

I-DET-DS-ELEC.040	Bias Voltage

System 1:	Electronic Systems (6.03.01.07)

System 2:	Magnet Instrumentation and Control (6.03.01.06)

Type:	ELEC

Description:	Bias voltage DC power will be provided from the electronics racks to support the magnet instrumentation and control systems.

Provider:	Electronic Systems (6.03.01.07)

Details:

Requirements:	P-DET-DS-MAG.6

References:

I-DET-DS-ELEC.041	High Voltage

System 1:	Electronic Systems (6.03.01.07)

System 2:	Magnet Instrumentation and Control (6.03.01.06)

Type:	ELEC

Description:	High voltage DC power will be provided from the electronics racks to support the magnet power supply.

Provider:	Electronic Systems (6.03.01.07)

Details:

Requirements:	P-DET-DS-MAG.6

References:

I-DET-DS-ELEC.042	Low Voltage

System 1:	Electronic Systems (6.03.01.07)

System 2:	Magnet Instrumentation and Control (6.03.01.06)

Type:	ELEC

Description:	Low voltage DC power will be provided from the electronics racks to support the magnet instrumentation and control systems.

Provider:	Electronic Systems (6.03.01.07)

Details:

Requirements:	P-DET-DS-MAG.6

References:

I-DET-DS-ELEC.043	Bias Voltage

System 1:	Electronic Systems (6.03.01.07)

System 2:	Magnet Power Supply (6.03.01.06)

Type:	ELEC

Description:	Bias voltage DC power will be provided from the electronics racks to support the magnet power supply.

Provider:	Electronic Systems (6.03.01.07)

Details:

Requirements:	P-DET-DS-MAG.6

References:

I-DET-DS-ELEC.044	High Voltage

System 1:	Electronic Systems (6.03.01.07)

System 2:	Magnet Power Supply (6.03.01.06)

Type:	ELEC

Description:	High voltage DC power will be provided from the electronics racks to support electronics in the detector.

Provider:	Electronic Systems (6.03.01.07)

Details:

Requirements:	P-DET-DS-MAG.6

References:

I-DET-DS-ELEC.045	Low Voltage

System 1:	Electronic Systems (6.03.01.07)

System 2:	Magnet Power Supply (6.03.01.06)

Type:	ELEC

Description:	Low voltage DC power will be provided from the electronics racks to support the magnet power supply.

Provider:	Electronic Systems (6.03.01.07)

Details:

Requirements:	P-DET-DS-MAG.6

References:

# Electronic Systems (DET-DS-ELEC) to Particle Identification Systems (DET-DS-PID)

I-DET-DS-ELEC.046	Bias Voltage

System 1:	Electronic Systems (6.03.01.07)

System 2:	Barrel Particle ID Systems (6.03.01.03)

Type:	ELEC

Description:	Bias voltage DC power will be provided from the electronics racks to support electronics the silicon photomultipliers.

Provider:	Electronic Systems (6.03.01.07)

Details:

Requirements:	P-DET-DS-PID.2

References:

I-DET-DS-ELEC.047	High Voltage

System 1:	Electronic Systems (6.03.01.07)

System 2:	Barrel Particle ID Systems (6.03.01.03)

Type:	ELEC

Description:	High voltage DC power will be provided from the electronics racks to support silicon sensors and gas detectors.

Provider:	Electronic Systems (6.03.01.07)

Details:

Requirements:	P-DET-DS-PID.2

References:

I-DET-DS-ELEC.048	Low Voltage

System 1:	Electronic Systems (6.03.01.07)

System 2:	Barrel Particle ID Systems (6.03.01.03)

Type:	ELEC

Description:	Low voltage DC power will be provided from the electronics racks to support electronics in the detector.

Provider:	Electronic Systems (6.03.01.07)

Details:

Requirements:	P-DET-DS-PID.2

References:

I-DET-DS-ELEC.049	High Voltage

System 1:	Electronic Systems (6.03.01.07)

System 2:	Barrel DIRC Particle ID Systems (6.03.01.03)

Type:	ELEC

Description:	High voltage power will be delivered from the power supplies on the carriage to the photosensors in the DIRC.

Provider:	Electronic Systems (6.03.01.07)

Details:

Requirements:	P-DET-DS-PID.2

References:

I-DET-DS-ELEC.050	Low Voltage

System 1:	Electronic Systems (6.03.01.07)

System 2:	Barrel DIRC Particle ID Systems (6.03.01.03)

Type:	ELEC

Description:	Low voltage power will be delivered from the power supplies on the carriage to the DIRC electronics.

Provider:	Electronic Systems (6.03.01.07)

Details:

Requirements:	P-DET-DS-PID.2

References:

I-DET-DS-ELEC.051	High Voltage

System 1:	Electronic Systems (6.03.01.07)

System 2:	Barrel TOF Particle ID Systems (6.03.01.03)

Type:	ELEC

Description:	High voltage power will be delivered from the power supplies on the carriage to the photosensors in the DIRC.

Provider:	Electronic Systems (6.03.01.07)

Details:

Requirements:	P-DET-DS-PID.2

References:

I-DET-DS-ELEC.052	Low Voltage

System 1:	Electronic Systems (6.03.01.07)

System 2:	Barrel TOF Particle ID Systems (6.03.01.03)

Type:	ELEC

Description:	Low voltage power will be delivered from the power supplies on the carriage to the DIRC electronics.

Provider:	Electronic Systems (6.03.01.07)

Details:

Requirements:	P-DET-DS-PID.2

References:

I-DET-DS-ELEC.053	High Voltage

System 1:	Electronic Systems (6.03.01.07)

System 2:	Backward Particle ID Systems (6.03.01.03)

Type:	ELEC

Description:	High voltage power will be delivered from the power supplies on the carriage to the photosensors in the pfRICH/mRICH.

Provider:	Electronic Systems (6.03.01.07)

Details:

Requirements:	P-DET-DS-PID.2

References:

I-DET-DS-ELEC.054	Low Voltage

System 1:	Electronic Systems (6.03.01.07)

System 2:	Backward Particle ID Systems (6.03.01.03)

Type:	ELEC

Description:	Low voltage power will be delivered from the power supplies on the carriage to the pfRICH/mRICH electronics.

Provider:	Electronic Systems (6.03.01.07)

Details:

Requirements:	P-DET-DS-PID.2

References:

I-DET-DS-ELEC.055	Bias Voltage

System 1:	Electronic Systems (6.03.01.07)

System 2:	Forward Particle ID Systems (6.03.01.03)

Type:	ELEC

Description:	Bias voltage DC power will be delivered from the power supplies on the carriage to the photosensors.

Provider:	Electronic Systems (6.03.01.07)

Details:

Requirements:	P-DET-DS-PID.2

References:

I-DET-DS-ELEC.056	Low Voltage

System 1:	Electronic Systems (6.03.01.07)

System 2:	Forward Particle ID Systems (6.03.01.03)

Type:	ELEC

Description:	Low voltage power will be delivered from the power supplies on the carriage to the dRICH electronics.

Provider:	Electronic Systems (6.03.01.07)

Details:

Requirements:	P-DET-DS-PID.2

References:

# Electronic Systems (DET-DS-ELEC) to Polarimetry and Luminosity (DET-DS-POL)

I-DET-DS-ELEC.057	Bias Voltage

System 1:	Electronic Systems (6.03.01.07)

System 2:	Electron Storage Ring Polarimetry (6.03.01.11.01.01)

Type:	ELEC

Description:	Bias voltage DC power will be provided from the electronics racks to support electronics the silicon photomultipliers.

Provider:	Electronic Systems (6.03.01.07)

Details:

Requirements:	P-DET-DS-POL.4

References:

I-DET-DS-ELEC.058	High Voltage

System 1:	Electronic Systems (6.03.01.07)

System 2:	Electron Storage Ring Polarimetry (6.03.01.11.01.01)

Type:	ELEC

Description:	High voltage DC power will be provided from the electronics racks to support silicon sensors and gas detectors.

Provider:	Electronic Systems (6.03.01.07)

Details:

Requirements:	P-DET-DS-POL.4

References:

I-DET-DS-ELEC.059	Low Voltage

System 1:	Electronic Systems (6.03.01.07)

System 2:	Electron Storage Ring Polarimetry (6.03.01.11.01.01)

Type:	ELEC

Description:	Low voltage DC power will be provided from the electronics racks to support electronics in the detector.

Provider:	Electronic Systems (6.03.01.07)

Details:

Requirements:	P-DET-DS-POL.4

References:

I-DET-DS-ELEC.060	Bias Voltage

System 1:	Electronic Systems (6.03.01.07)

System 2:	Rapid Cycling Synchrotron Polarimetry (6.03.01.11.01.02)

Type:	ELEC

Description:	Bias voltage DC power will be provided from the electronics racks to support electronics the silicon photomultipliers.

Provider:	Electronic Systems (6.03.01.07)

Details:

Requirements:	P-DET-DS-POL.4

References:

I-DET-DS-ELEC.061	High Voltage

System 1:	Electronic Systems (6.03.01.07)

System 2:	Rapid Cycling Synchrotron Polarimetry (6.03.01.11.01.02)

Type:	ELEC

Description:	High voltage DC power will be provided from the electronics racks to support silicon sensors and gas detectors.

Provider:	Electronic Systems (6.03.01.07)

Details:

Requirements:	P-DET-DS-POL.4

References:

I-DET-DS-ELEC.062	Low Voltage

System 1:	Electronic Systems (6.03.01.07)

System 2:	Rapid Cycling Synchrotron Polarimetry (6.03.01.11.01.02)

Type:	ELEC

Description:	Low voltage DC power will be provided from the electronics racks to support electronics in the detector.

Provider:	Electronic Systems (6.03.01.07)

Details:

Requirements:	P-DET-DS-POL.4

References:

I-DET-DS-ELEC.063	Bias Voltage

System 1:	Electronic Systems (6.03.01.07)

System 2:	HJET Polarimetery (6.03.01.11.02.01)

Type:	ELEC

Description:	Bias voltage DC power will be provided from the electronics racks to support electronics the silicon photomultipliers.

Provider:	Electronic Systems (6.03.01.07)

Details:

Requirements:	P-DET-DS-POL.4

References:

I-DET-DS-ELEC.064	High Voltage

System 1:	Electronic Systems (6.03.01.07)

System 2:	HJET Polarimetery (6.03.01.11.02.01)

Type:	ELEC

Description:	High voltage DC power will be provided from the electronics racks to support silicon sensors and gas detectors.

Provider:	Electronic Systems (6.03.01.07)

Details:

Requirements:	P-DET-DS-POL.4

References:

I-DET-DS-ELEC.065	Low Voltage

System 1:	Electronic Systems (6.03.01.07)

System 2:	HJET Polarimetery (6.03.01.11.02.01)

Type:	ELEC

Description:	Low voltage DC power will be provided from the electronics racks to support electronics in the detector.

Provider:	Electronic Systems (6.03.01.07)

Details:

Requirements:	P-DET-DS-POL.4

References:

I-DET-DS-ELEC.066	Bias Voltage

System 1:	Electronic Systems (6.03.01.07)

System 2:	Proton-Carbon Polarimeter (6.03.01.11.02.02)

Type:	ELEC

Description:	Bias voltage DC power will be provided from the electronics racks to support electronics the silicon photomultipliers.

Provider:	Electronic Systems (6.03.01.07)

Details:

Requirements:	P-DET-DS-POL.4

References:

I-DET-DS-ELEC.067	High Voltage

System 1:	Electronic Systems (6.03.01.07)

System 2:	Proton-Carbon Polarimeter (6.03.01.11.02.02)

Type:	ELEC

Description:	High voltage DC power will be provided from the electronics racks to support silicon sensors and gas detectors.

Provider:	Electronic Systems (6.03.01.07)

Details:

Requirements:	P-DET-DS-POL.4

References:

I-DET-DS-ELEC.068	Low Voltage

System 1:	Electronic Systems (6.03.01.07)

System 2:	Proton-Carbon Polarimeter (6.03.01.11.02.02)

Type:	ELEC

Description:	Low voltage DC power will be provided from the electronics racks to support electronics in the detector.

Provider:	Electronic Systems (6.03.01.07)

Details:

Requirements:	P-DET-DS-POL.4

References:

# Electronic Systems (DET-DS-ELEC) to Tracking Systems (DET-DS-TRAK)

I-DET-DS-ELEC.069	Bias Voltage

System 1:	Electronic Systems (6.03.01.07)

System 2:	Barrel Tracking Systems (6.03.01.02)

Type:	ELEC

Description:	Bias voltage DC power will be provided from the electronics racks to support electronics in the silicon photomultipliers.

Provider:	Electronic Systems (6.03.01.07)

Details:

Requirements:	P-DET-DS-TRAK.6

References:

I-DET-DS-ELEC.070	High Voltage

System 1:	Electronic Systems (6.03.01.07)

System 2:	Barrel Tracking Systems (6.03.01.02)

Type:	ELEC

Description:	High voltage DC power will be provided from the electronics racks to support silicon sensors and gas detectors.

Provider:	Electronic Systems (6.03.01.07)

Details:

Requirements:	P-DET-DS-TRAK.6

References:

I-DET-DS-ELEC.071	Low Voltage

System 1:	Electronic Systems (6.03.01.07)

System 2:	Barrel Tracking Systems (6.03.01.02)

Type:	ELEC

Description:	Low voltage DC power will be provided from the electronics racks to support electronics in the detector.

Provider:	Electronic Systems (6.03.01.07)

Details:

Requirements:	P-DET-DS-TRAK.6

References:

# Applicable Documents

List applicable documents considered to be part of this document. Applicable documents shall be obtained from the EIC SharePoint Document Center.

# Detailed change Log

| Revision | Date | Change |
| --- | --- | --- |

# References

- doi:10.17226/25171., National Academies of Sciences, Engineering, and Medicine, "An Assessment of U.S.-Based Electron-Ion Collider Science", The National Academies Press, Washington, DC, 2018.

- EIC-ORG-PLN-010, Electron-Ion Collider L1 Requirements (Global Requirements Document)

- BNL-221006-2021-FORE, EIC Conceptual Design Report

- arXiv:2103.05419, Science Requirements and Detector Concepts for the Electron-Ion Collider: EIC Yellow Report

- EIC-SEG-PLN-016, Requirements Management Plan

- EIC-SEG-PLN-022, Systems Engineering Management Plan

- EIC-SEG-PLN-020, EIC Interface Management Plan