Electron-Ion Collider, Requirements, Specifications, and Interfaces

Interface Control Document: [###]

Detector Infrastructure Systems (6.03.01.09) to Detector System (6.03.01)

Prepared by:

{{Sig_es_:signer1:signature}}	Date: {{Dte_es_:signer1:date}}

Name, Role

Electron-Ion Collider at Institution

Reviewed by:

{{Sig_es_:signer2:signature}}	Date: {{Dte_es_:signer2:date}}

Name, Role

Electron-Ion Collider at Institution

{{Sig_es_:signer3:signature}}	Date: {{Dte_es_:signer3:date}}

Name, Role

Electron-Ion Collider at Institution

{{Sig_es_:signer4:signature}}	Date: {{Dte_es_:signer4:date}}

Name, Role

Electron-Ion Collider at Institution

Approved by:

{{Sig_es_:signer5:signature}}	Date: {{Dte_es_:signer5:date}}

Name, Role

Electron-Ion Collider at Institution

REVISION HISTORY

| Revision # | Effective Date | Additional Reviewers | Summary of Change |
| --- | --- | --- | --- |
| 00 |  |  | Initial release. |

TABLE OF CONTENTS

1.	Purpose and Scope	5

2.	Introduction	5

2.1	Detector Infrastructure Systems	5

2.2	Overview of Interface Relationships	6

2.3	Interface Responsibilities	6

2.4	Safety and Environmental Considerations	6

2.5	Design Considerations	6

2.6	Organization	6

3.	Interface summary	6

4.	Detector Infrastructure Systems (DET-DS-INF) to Detector System (DET-DS)	49

5.	Detector Infrastructure Systems (DET-DS-INF) to Ancillary Detectors (DET-DS-ANC)	50

6.	Detector Infrastructure Systems (DET-DS-INF) to Data Acquisition and Computing Systems (DET-DS-COMP)	55

7.	Detector Infrastructure Systems (DET-DS-INF) to Electromagnetic Calorimetry Systems (DET-DS-ECAL)	59

8.	Detector Infrastructure Systems (DET-DS-INF) to Electronic Systems (DET-DS-ELEC)	67

9.	Detector Infrastructure Systems (DET-DS-INF) to Hadronic Calorimetry Systems (DET-DS-HCAL)	68

10.	Detector Infrastructure Systems (DET-DS-INF) to Solenoid Magnet (DET-DS-MAG)	78

11.	Detector Infrastructure Systems (DET-DS-INF) to Particle Identification Systems (DET-DS-PID)	83

12.	Detector Infrastructure Systems (DET-DS-INF) to Tracking Systems (DET-DS-TRAK)	95

13.	Applicable Documents	98

14.	Detailed change Log	98

15.	References	98

LIST OF ACRONYMS

| B0 | B0 is the name of the forward magnet just past the Hadron Endcap |  |
| --- | --- | --- |
| CDR | Conceptual Design Report |  |
| COTS | Commercial Off the Shelf |  |
| DAQ | Data Acquisition |  |
| DET | EIC Detector |  |
| DIRC | Detection of Internally Reflected Cherenkov Light |  |
| dRICH | Dual radiator ring imaging Cherenkov |  |
| DVCS | Deeply Virtual Compton Scattering |  |
| EIC | Electron-Ion Collider |  |
| EMCAL | Electro-magnetic Calorimeter |  |
| ESR | Electron Storage Ring |  |
| FCC | Federal Communications Commission |  |
| FDR | Final Design Review |  |
| GEM | Gas Electron Multiplier |  |
| HCAL | Hadronic Calorimeter |  |
| HSR | Hadron Storage Ring |  |
| HVAC | Heating/Ventilation/Air Conditioning |  |
| IP | Interaction Point |  |
| IR | Interaction Region |  |
| MPGD | Micro Pattern Gaseous Detectors |  |
| NAS | National Academy of Sciences |  |
| NEC | National Electric Code |  |
| NEMA | National Electrical Manufacturers Association |  |
| NFPA | National Fire Protection Association |  |
| NRTL | Nationally Recognized Testing Laboratory |  |
| PID | Particle Identification |  |
| PRR | Production Readiness Review |  |
| PWO | PbWO4 |  |
| RCS | Rapid Cycling Synchrotron |  |
| TBD | To Be Determined |  |
| UL | Underwriter Laboratories |  |
| WBS | Work Break Down Structure |  |

# Interface Control Document:

Detector Infrastructure Systems (6.03.01.09) to Detector System (6.03.01)

# Purpose and Scope

This document records the agreed upon interfaces between the Detector Infrastructure Systems and other sub-systems within the Detector System. These interfaces are based on the system requirements that are necessary to integrate these sub-systems as part of the Electron Ion Collider Project.

# Introduction

These interfaces describe the relationships between the Detector Infrastructure Systems and the Detector System including (examples by inclusion) located in (areas by inclusion).

## Detector Infrastructure Systems

Provide an introductory description of your sub-system and an overview of the things that it receives, provides, supports, constrains, or otherwise requires as part of its function --- specifically if these things are manifested as interfaces.

Detector System (DET-DS): Description of the TYPES of interfaces that exist between the ICD reference system and this sub-system, and a BRIEF overview of the roles they play.

Ancillary Detectors (DET-DS-ANC): Description of the TYPES of interfaces that exist between the ICD reference system and this sub-system, and a BRIEF overview of the roles they play.

Data Acquisition and Computing Systems (DET-DS-COMP): Description of the TYPES of interfaces that exist between the ICD reference system and this sub-system, and a BRIEF overview of the roles they play.

Electromagnetic Calorimetry Systems (DET-DS-ECAL): Description of the TYPES of interfaces that exist between the ICD reference system and this sub-system, and a BRIEF overview of the roles they play.

Electronic Systems (DET-DS-ELEC): Description of the TYPES of interfaces that exist between the ICD reference system and this sub-system, and a BRIEF overview of the roles they play.

Hadronic Calorimetry Systems (DET-DS-HCAL): Description of the TYPES of interfaces that exist between the ICD reference system and this sub-system, and a BRIEF overview of the roles they play.

Solenoid Magnet (DET-DS-MAG): Description of the TYPES of interfaces that exist between the ICD reference system and this sub-system, and a BRIEF overview of the roles they play.

Particle Identification Systems (DET-DS-PID): Description of the TYPES of interfaces that exist between the ICD reference system and this sub-system, and a BRIEF overview of the roles they play.

Tracking Systems (DET-DS-TRAK): Description of the TYPES of interfaces that exist between the ICD reference system and this sub-system, and a BRIEF overview of the roles they play.

## Overview of Interface Relationships

The following are the sub-systems within the Detector System that have interfaces connected to the Detector Infrastructure Systems.

## Interface Responsibilities

The various ownerships of responsibilities may be listed here. However, this section might also refer to the Provider section of the interface detail if all pertinent responsibilities are listed there. If this is the case, simply enter: “See the Provider section within the interface details for information about the responsibilities associated with individual interfaces.”

## Safety and Environmental Considerations

General safety and environmental considerations are identified here. Any considerations that are specific to the individual sub-system, will be addressed in external safety documents and are referenced here.

### Considerations for all systems

Add general safety and environmental considerations here.

### Special considerations and references

Add references here as necessary.

## Design Considerations

Notes here about general design considerations.

## Organization

The interfaces in this document are grouped by sub-system as identified in the sub-system identifier. All interfaces between the Detector Infrastructure Systems and a related sub-system will be included in a single section. Entries within that section will be sorted by the interface identifier or WBS number, as appropriate.

# Interface summary

This tables in this section provide a list of all interfaces that are defined within this document. The interfaces are grouped by sub-system/WBS. Each table has the following fields:

Section: the section number in this document where the interface is addressed.

WBS (1): the WBS number within the ICD reference system, unless the reference system is the owner.

WBS (2): the WBS number for the connected system.

ID: the interface identifier as defined in the Interface Requirement Document (IRD).

Name: the name of the interface as defined in the IRD.

Description: the description of the interface as defined in the IRD.

Owner: identifies the WBS number of the entity that is responsible for ensuring the interface is provided and that it meets validation/verification requirements.

Detector Infrastructure Systems to Detector System

| Section | WBS1 | WBS2 | ID | Name | Description | Owner |
| --- | --- | --- | --- | --- | --- | --- |
| 4 |  | 6.03.01 |  |  | Detector System |  |
| 4.1 | 6.03.01.09 | 6.03.01 | I-DET-DS-INF-INT.001 | Magnetic Material | No detectors or components constructed with magnetic material may be added to the detector without prior consultation with the magnet group. | 6.03.01.09.04 |
| 4.2 | 6.03.01.09 | 6.03.01 | I-DET-DS-INF-INT.002 | Magnetic Field Requirement | All subordinate detectors are dependent on the continuous operation of the solenoid and on the delivery of a consistent, stable magnetic field. | 6.03.01.09.04 |
| 4.3 | 6.03.01.09 | 6.03.01 | I-DET-DS-INF-INT.003 | Cabling/Services Conduit | The DIRC structure will provide integrated pathways for cabling and services to be delivered from the interior detectors (pf RICH/mRICH and silicon trackers) to the exterior infrastructure. | 6.03.01.09.04 |

Detector Infrastructure Systems to Ancillary Detectors

| Section | WBS1 | WBS2 | ID | Name | Description | Owner |
| --- | --- | --- | --- | --- | --- | --- |
| 5 |  | 6.03.01.10 |  |  | Ancillary Detectors |  |
| 5.1 | 6.03.01.09 | 6.03.01.10 | I-DET-DS-INF-BCK.004 | LowQ2 Support | The LowQ2 detector is supported by a freestanding platform that is adjacent to the outgoing electron beam. | 6.03.01.09.02 |
| 5.2 | 6.03.01.09 | 6.03.01.10 | I-DET-DS-INF-BCK.005 | Lumi Support System | The luminosity detector will be supported by an independent support system. | 6.03.01.09.02 |
| 5.3 | 6.03.01.09 | 6.03.01.10 | I-DET-DS-INF-FWD.001 | Detector Support | The detector will be supported by an integrated support stand that is fully integrated with the vacuum system and allows the position of the detector to be adjusted. | 6.03.01.09.03 |
| 5.4 | 6.03.01.09 | 6.03.01.10 | I-DET-DS-INF-FWD.002 | Detector Support | The detector will be supported by an integrated support stand that is fully integrated with the vacuum system and allows the position of the detector to be adjusted. | 6.03.01.09.03 |
| 5.5 | 6.03.01.09 | 6.03.01.10 | I-DET-DS-INF-FWD.006 | B0 Support System | The B0 detector will be supported in the interior of the B0 magnet and will be dependent on the B0 magnet design, and must be adequate to support the weight of the crystals and the tracking detectors. | 6.03.01.09.03 |
| 5.6 | 6.03.01.09 | 6.03.01.10 | I-DET-DS-INF-FWD.007 | ZDC Support System | The detector will be supported on a free standing structure. | 6.03.01.09.03 |
| 5.7 | 6.03.01.09 | 6.03.01.10 | I-DET-DS-INF-INT.004 | Space Constraint | B0 detector must fit entirely within the bore of the B0 magnet. | 6.03.01.09.04 |
| 5.8 | 6.03.01.09 | 6.03.01.10 | I-DET-DS-INF-INT.005 | Space Constraint | The position of the forward HCAL is bounded by the B0 magnet in the forward direction. Changes to the size or position of either must be coordinated with the other. | 6.03.01.09.04 |
| 5.9 | 6.03.01.09 | 6.03.01.10 | I-DET-DS-INF-INT.006 | Space Constraint | The position of and accessibility of the B0 detector in the backward direction is limited by the cryostat containing the final focusing magnets. | 6.03.01.09.04 |
| 5.10 | 6.03.01.09 | 6.03.01.10 | I-DET-DS-INF-INT.007 | Space Constraint | Access to the B0 detector is constrained by the vacuum valve in front of the B0 magnet. A minimum of 15 centimeters of free space will be required on the IP-side of the B0-dipole for the installation of the B0 detectors. | 6.03.01.09.04 |
| 5.11 | 6.03.01.09 | 6.03.01.10 | I-DET-DS-INF-INT.008 | Space Constraint | The electron and hadron beam pipes must be contained within the B0 detector. | 6.03.01.09.04 |
| 5.12 | 6.03.01.09 | 6.03.01.10 | I-DET-DS-INF-INT.009 | Position Constraint | The LowQ2 detector must be upstream in the electron direction from the Q3ER magnet. | 6.03.01.09.04 |
| 5.13 | 6.03.01.09 | 6.03.01.10 | I-DET-DS-INF-INT.010 | Position Constraint | The LowQ2 detector must be downstream in the electron direction from the B2ER magnet. | 6.03.01.09.04 |
| 5.14 | 6.03.01.09 | 6.03.01.10 | I-DET-DS-INF-INT.011 | Position Constraint | The LOWQ2 detector must be at the same level as the electron beamline. | 6.03.01.09.04 |
| 5.15 | 6.03.01.09 | 6.03.01.10 | I-DET-DS-INF-INT.040 | Process Cooling | A cooling system will be required to remove heat from the calorimeter and electronics readouts, to maintain an acceptable temperature. A chiller should be installed near the sub-system and a space should be allocated for this chiller. | 6.03.01.09.04 |
| 5.16 | 6.03.01.09 | 6.03.01.10 | I-DET-DS-INF-INT.041 | Process Cooling | Either a liquid or gas cooling system will be required to remove heat from the calorimeter, tracking and readout electronics, to maintain them at room temperature. | 6.03.01.09.04 |
| 5.17 | 6.03.01.09 | 6.03.01.10 | I-DET-DS-INF-INT.042 | Dipole Power | The two dipoles in front of the detector will require an infrastructure power source. | 6.03.01.09.04 |
| 5.18 | 6.03.01.09 | 6.03.01.10 | I-DET-DS-INF-INT.043 | Dipole Vacuum | The beamline between the two luminosity detector dipoles must be under vacuum. | 6.03.01.09.04 |
| 5.19 | 6.03.01.09 | 6.03.01.10 | I-DET-DS-INF-INT.044 | Process Cooling | A cooling system will be required to remove heat from the tracking and readout electronics to prevent heat buildup. | 6.03.01.09.04 |
| 5.20 | 6.03.01.09 | 6.03.01.10 | I-DET-DS-INF-INT.045 | Process Cooling | A cooling system will be required to remove heat from the tracking and readout electronics to prevent heat buildup. | 6.03.01.09.04 |
| 5.21 | 6.03.01.09 | 6.03.01.10 | I-DET-DS-INF-INT.046 | Process Cooling | A cooling system will be required to remove heat from the calorimeter and readout electronics to prevent heat buildup. | 6.03.01.09.04 |

Detector Infrastructure Systems to Data Acquisition and Computing Systems

| Section | WBS1 | WBS2 | ID | Name | Description | Owner |
| --- | --- | --- | --- | --- | --- | --- |
| 6 |  | 6.03.01.08 |  |  | Data Acquisition and Computing Systems |  |
| 6.1 | 6.03.01.09 | 6.03.01.08 | I-DET-DS-INF-BAR.013 | Fiber Support System | A fiber support system must be provided that allows the communications cabling to remain connected while the detector is moved from the experimental hall to the assembly area. | 6.03.01.09.01 |
| 6.2 | 6.03.01.09 | 6.03.01.08 | I-DET-DS-INF-INT.047 | Computer Room Cooling | HVAC systems must be provided to maintain stable temperature and humidity in the computer room. | 6.03.01.09.04 |
| 6.3 | 6.03.01.09 | 6.03.01.08 | I-DET-DS-INF-INT.048 | Experimental and Assembly Area Cooling | General HVAC systems must be provided to maintain a normal operating temperature in locations where DAQ systems are in use. | 6.03.01.09.04 |
| 6.4 | 6.03.01.09 | 6.03.01.08 | I-DET-DS-INF-INT.049 | Backup Power | UPS systems must be provided to support critical systems in order to tolerate transient conditions and to allow for controlled shutdown. | 6.03.01.09.04 |
| 6.5 | 6.03.01.09 | 6.03.01.08 | I-DET-DS-INF-INT.050 | Computer Room Power | Adequate power must be provided to support the DAQ systems and computing infrastructure. | 6.03.01.09.04 |
| 6.6 | 6.03.01.09 | 6.03.01.08 | I-DET-DS-INF-INT.051 | Control Room Power | Standard electrical service to support consoles and systems in the control room. | 6.03.01.09.04 |
| 6.7 | 6.03.01.09 | 6.03.01.08 | I-DET-DS-INF-INT.052 | Hall Power | Clean power (either transformer or source isolated) should be provided for all DAQ, computing and network equipment. | 6.03.01.09.04 |
| 6.8 | 6.03.01.09 | 6.03.01.08 | I-DET-DS-INF-INT.053 | Carriage Rack Space | Adequate rack space for the DAQ system will be required on the detector carriages. | 6.03.01.09.04 |
| 6.9 | 6.03.01.09 | 6.03.01.08 | I-DET-DS-INF-INT.054 | Computer Room | Adequate raised-floor space with racks will be required to support DAQ computers, communications systems, and computing infrastructure. | 6.03.01.09.04 |
| 6.10 | 6.03.01.09 | 6.03.01.08 | I-DET-DS-INF-INT.055 | Control Systems Furniture | Space for computing consoles and furniture adequate to manage the system during operation will be required. Consoles and furniture will be provided by DET-INT. | 6.03.01.09.04 |
| 6.11 | 6.03.01.09 | 6.03.01.08 | I-DET-DS-INF-INT.056 | Detector Plenums | Adequate space to route fiberoptic cables from the racks to the detector readout boards will be required. | 6.03.01.09.04 |
| 6.12 | 6.03.01.09 | 6.03.01.08 | I-DET-DS-INF-INT.057 | Fixed Platform Conduits | Adequate conduits and cableways must be provided to deliver cabling to the racks on the fixed platforms. | 6.03.01.09.04 |
| 6.13 | 6.03.01.09 | 6.03.01.08 | I-DET-DS-INF-INT.058 | Platform/Tunnel Rack Space | Adequate rack space for the DAQ system must be provided on the fixed platforms, tunnels, and in the experimental hall. | 6.03.01.09.04 |
| 6.14 | 6.03.01.09 | 6.03.01.08 | I-DET-DS-INF-INT.059 | Sensitive Electronics | DAQ systems that are sensitive to magnetism or radiation must be placed at a sufficient distance from sources OR shielding must be provided to protect the equipment. | 6.03.01.09.04 |
| 6.15 | 6.03.01.09 | 6.03.01.08 | I-DET-DS-INF-INT.060 | Tunnel Conduits | Conduits must be provided in the west and east tunnels to connect the detectors to the DAQ system. | 6.03.01.09.04 |
| 6.16 | 6.03.01.09 | 6.03.01.08 | I-DET-DS-INF-INT.061 | Fiber Support System | A fiber support system must be provided that allows the communications cabling to remain connected while the detector is moved from the experimental hall to the assembly area. | 6.03.01.09.04 |

Detector Infrastructure Systems to Electromagnetic Calorimetry Systems

| Section | WBS1 | WBS2 | ID | Name | Description | Owner |
| --- | --- | --- | --- | --- | --- | --- |
| 7 |  | 6.03.01.04 |  |  | Electromagnetic Calorimetry Systems |  |
| 7.1 | 6.03.01.09 | 6.03.01.04 | I-DET-DS-INF-BAR.001 | Barrel ECAL Support | The barrel ECAL will be supported by a structural support system that extends through the bore of the solenoid magnet and is supported by the barrel Hadron Calorimeter. | 6.03.01.09.01 |
| 7.2 | 6.03.01.09 | 6.03.01.04 | I-DET-DS-INF-BAR.002 | Weight Distributed | The weight of the barrel ECAL will be transferred to the barrel Hadron Calorimeter and must be accommodated by all intermediate and subsequent support systems. | 6.03.01.09.01 |
| 7.3 | 6.03.01.09 | 6.03.01.04 | I-DET-DS-INF-BAR.003 | Support Structure | The DIRC bar boxes will be supported by a frame inside the barrel Electromagnetic Calorimeter, that allows the boxes to be extracted using a system of rollers. | 6.03.01.09.01 |
| 7.4 | 6.03.01.09 | 6.03.01.04 | I-DET-DS-INF-BAR.004 | Backward ECAL Support | The backward ECAL will be supported by an integrated support structure that is attached to the DIRC support frame. | 6.03.01.09.01 |
| 7.5 | 6.03.01.09 | 6.03.01.04 | I-DET-DS-INF-BAR.005 | Weight Transfer | The weight of the backward ECAL will be transferred to the DIRC detector support and must be accommodated by all subsequent support systems. | 6.03.01.09.01 |
| 7.6 | 6.03.01.09 | 6.03.01.04 | I-DET-DS-INF-BAR.006 | EMCal Support System | The DIRC support system will provide support for the backward electromagnetic calorimeter. | 6.03.01.09.01 |
| 7.7 | 6.03.01.09 | 6.03.01.04 | I-DET-DS-INF-BAR.007 | Forward ECAL Support | The forward ECAL will be supported by the forward Hadron Calorimeter endcap. Because the forward ECAL must split into two parts to provide access to the barrel, each half must be independently affixed to the Hadron Calorimeter halves. | 6.03.01.09.01 |
| 7.8 | 6.03.01.09 | 6.03.01.04 | I-DET-DS-INF-BAR.008 | Weight Transfer | The weight of the forward ECAL will be transferred to the forward Hadron Calorimeter endcap and must be accommodated by all subsequent support systems. | 6.03.01.09.01 |
| 7.9 | 6.03.01.09 | 6.03.01.04 | I-DET-DS-INF-BAR.014 | Forward ECAL Mobility Support | Each half of the forward ECAL must be continuously supported and stabilized while it is moved between the opened and closed positions. | 6.03.01.09.01 |
| 7.10 | 6.03.01.09 | 6.03.01.04 | I-DET-DS-INF-FWD.004 | Sub-Detector Support | The forward HCAL will support the weight of the forward electromagnetic calorimeter that is embedded within it. | 6.03.01.09.03 |
| 7.11 | 6.03.01.09 | 6.03.01.04 | I-DET-DS-INF-FWD.008 | Forward ECAL Mobility Support | Each half of the forward ECAL must be continuously supported and stabilized while it is moved between the opened and closed positions. | 6.03.01.09.03 |
| 7.12 | 6.03.01.09 | 6.03.01.04 | I-DET-DS-INF-INT.012 | Space Constraint | The exterior radius of the barrel ECAL (and its support system) is limited by the interior bore of the solenoid magnet. Modifications to either must be coordinated. | 6.03.01.09.04 |
| 7.13 | 6.03.01.09 | 6.03.01.04 | I-DET-DS-INF-INT.013 | Space Constraint | The backward position and shape of the barrel EMCAL is limited by the size and shape of the DIRC readout supports. Changes to the size or position of either must be coordinated with the other. | 6.03.01.09.04 |
| 7.14 | 6.03.01.09 | 6.03.01.04 | I-DET-DS-INF-INT.014 | Space Constraint | The maximum size of the DIRC is limited to the interior bore of the barrel ECAL (and its support structures). Modifications to either must be coordinated. | 6.03.01.09.04 |
| 7.15 | 6.03.01.09 | 6.03.01.04 | I-DET-DS-INF-INT.015 | Space Constraint | The forward position and shape of the barrel ECAL is limited by the backward face of the Dual RICH detector and the adjacent cabling pathway that provides services to the interior detectors. Modifications to either must be coordinated. | 6.03.01.09.04 |
| 7.16 | 6.03.01.09 | 6.03.01.04 | I-DET-DS-INF-INT.016 | Space Constraint | The backward position of the backward ECAL is limited by the backward Hadron Calorimeter and the adjacent cabling pathway that provides services to the interior detectors. Modifications to either must be coordinated. | 6.03.01.09.04 |
| 7.17 | 6.03.01.09 | 6.03.01.04 | I-DET-DS-INF-INT.017 | Space Constraint | The maximum backward location for the DIRC is limited by the position of the backward electromagnetic calorimeter. Modifications to either must be coordinated. | 6.03.01.09.04 |
| 7.18 | 6.03.01.09 | 6.03.01.04 | I-DET-DS-INF-INT.018 | Space Constraint | The exterior radius of the backward ECAL is limited by the interior bore of the DIRC support system. Modifications to either must be coordinated. | 6.03.01.09.04 |
| 7.19 | 6.03.01.09 | 6.03.01.04 | I-DET-DS-INF-INT.019 | Heat Mitigation | The backward ECAL must mitigated the heat generated by the backward RICH detector. | 6.03.01.09.04 |
| 7.20 | 6.03.01.09 | 6.03.01.04 | I-DET-DS-INF-INT.020 | Space Constraint | The position of the backward ECAL in the forward direction is limited by the backward face of the pfRICH/mRICH detector. Modifications to either must be coordinated. | 6.03.01.09.04 |
| 7.21 | 6.03.01.09 | 6.03.01.04 | I-DET-DS-INF-INT.021 | Backward ECAL Bore Clearance | The bore of the backward ECAL must be designed to allow it to be inserted/removed over the existing beamline flanges. | 6.03.01.09.04 |
| 7.22 | 6.03.01.09 | 6.03.01.04 | I-DET-DS-INF-INT.022 | Space Constraint | The interior radius of the backward ECAL is governed by the size of the beamline. | 6.03.01.09.04 |
| 7.23 | 6.03.01.09 | 6.03.01.04 | I-DET-DS-INF-INT.023 | Space Constraint | The exterior radius of the forward ECAL is limited by the interior bore of the barrel Hadron Calorimeter. Modifications to either must be coordinated. | 6.03.01.09.04 |
| 7.24 | 6.03.01.09 | 6.03.01.04 | I-DET-DS-INF-INT.024 | Space Constraint | The forward position of the forward ECAL is limited by the forward Hadron Calorimeter. Modifications to either must be coordinated. | 6.03.01.09.04 |
| 7.25 | 6.03.01.09 | 6.03.01.04 | I-DET-DS-INF-INT.025 | Space Constraint | The maximum forward location for the dRICH is limited by the forward ECAL and the adjacent cabling pathway that provides services to the interior detectors. Modifications to either must be coordinated. | 6.03.01.09.04 |
| 7.26 | 6.03.01.09 | 6.03.01.04 | I-DET-DS-INF-INT.026 | Space Constraint | The interior radius of the forward ECAL is governed by the size of the beamline. Modifications to either must be coordinated. | 6.03.01.09.04 |
| 7.27 | 6.03.01.09 | 6.03.01.04 | I-DET-DS-INF-INT.062 | Imaging Layers Heat Rejection | The imaging layers must be cooled with an independent system (TBD). | 6.03.01.09.04 |
| 7.28 | 6.03.01.09 | 6.03.01.04 | I-DET-DS-INF-INT.063 | Silicon PM Heat Rejection | The barrel ECAL will require chilled water or LCW cooling to maintain a safe operating temperature. | 6.03.01.09.04 |
| 7.29 | 6.03.01.09 | 6.03.01.04 | I-DET-DS-INF-INT.064 | Heat Rejection | The backward ECAL will require chilled water or LCW cooling to maintain the temperature of the silicon photomultipliers, the electronics, and the crystals. | 6.03.01.09.04 |
| 7.30 | 6.03.01.09 | 6.03.01.04 | I-DET-DS-INF-INT.065 | Heat Rejection | The forward ECAL will require chilled water or LCW cooling to maintain a safe operating temperature. | 6.03.01.09.04 |

Detector Infrastructure Systems to Electronic Systems

| Section | WBS1 | WBS2 | ID | Name | Description | Owner |
| --- | --- | --- | --- | --- | --- | --- |
| 8 |  | 6.03.01.07 |  |  | Electronic Systems |  |
| 8.1 | 6.03.01.09 | 6.03.01.07 | I-DET-DS-INF-INT.066 | Clean A/C Power | Electronics components on the carriage will require clean or transformer isolated power. | 6.03.01.09.04 |
| 8.2 | 6.03.01.09 | 6.03.01.07 | I-DET-DS-INF-INT.067 | UPS Power | Critical systems, such as controls, will require sufficient UPS power to allow them to be shutdown graceful, or to tolerate transients. | 6.03.01.09.04 |
| 8.3 | 6.03.01.09 | 6.03.01.07 | I-DET-DS-INF-INT.068 | Computing Environment | The electronics and computing systems will require a stable environment in terms of temperature and humidity. | 6.03.01.09.04 |
| 8.4 | 6.03.01.09 | 6.03.01.07 | I-DET-DS-INF-INT.069 | EMI Shielding | The electronics and computing systems on the carriage may require some EMI shielding from detectors and other components. | 6.03.01.09.04 |
| 8.5 | 6.03.01.09 | 6.03.01.07 | I-DET-DS-INF-INT.070 | Cabling Pathways | Adequate space for cables and cable trays will be required to deliver low voltage and high voltage, fiber and signal cables from the carriage/platform to the sub-detectors. | 6.03.01.09.04 |
| 8.6 | 6.03.01.09 | 6.03.01.07 | I-DET-DS-INF-INT.071 | Platform Rack Space | Adequate space must be provided for computing enclosures (racks) on the south platform to support the electronics systems. | 6.03.01.09.04 |

Detector Infrastructure Systems to Hadronic Calorimetry Systems

| Section | WBS1 | WBS2 | ID | Name | Description | Owner |
| --- | --- | --- | --- | --- | --- | --- |
| 9 |  | 6.03.01.05 |  |  | Hadronic Calorimetry Systems |  |
| 9.1 | 6.03.01.09 | 6.03.01.05 | I-DET-DS-INF-BAR.001 | Barrel ECAL Support | The barrel ECAL will be supported by a structural support system that extends through the bore of the solenoid magnet and is supported by the barrel Hadron Calorimeter. | 6.03.01.09.01 |
| 9.2 | 6.03.01.09 | 6.03.01.05 | I-DET-DS-INF-BAR.002 | Weight Distributed | The weight of the barrel ECAL will be transferred to the barrel Hadron Calorimeter and must be accommodated by all intermediate and subsequent support systems. | 6.03.01.09.01 |
| 9.3 | 6.03.01.09 | 6.03.01.05 | I-DET-DS-INF-BAR.004 | Backward ECAL Support | The backward ECAL will be supported by an integrated support structure that is attached to the DIRC support frame. | 6.03.01.09.01 |
| 9.4 | 6.03.01.09 | 6.03.01.05 | I-DET-DS-INF-BAR.007 | Forward ECAL Support | The forward ECAL will be supported by the forward Hadron Calorimeter endcap. Because the forward ECAL must split into two parts to provide access to the barrel, each half must be independently affixed to the Hadron Calorimeter halves. | 6.03.01.09.01 |
| 9.5 | 6.03.01.09 | 6.03.01.05 | I-DET-DS-INF-BAR.008 | Weight Transfer | The weight of the forward ECAL will be transferred to the forward Hadron Calorimeter endcap and must be accommodated by all subsequent support systems. | 6.03.01.09.01 |
| 9.6 | 6.03.01.09 | 6.03.01.05 | I-DET-DS-INF-BAR.009 | Barrel HCAL Support | The barrel HCAL will be supported by a rolling carriage that allows the central detector to be moved between the assembly area and the experimental hall. | 6.03.01.09.01 |
| 9.7 | 6.03.01.09 | 6.03.01.05 | I-DET-DS-INF-BAR.010 | Sub-Detector Support | The barrel HCAL will support the weight of all of the sub-detectors and components within the central detector. | 6.03.01.09.01 |
| 9.8 | 6.03.01.09 | 6.03.01.05 | I-DET-DS-INF-BAR.011 | Weight Transfer | The cumulative weight of the HCAL and all of the sub-detectors it supports will be transferred to the detector carriage. | 6.03.01.09.01 |
| 9.9 | 6.03.01.09 | 6.03.01.05 | I-DET-DS-INF-BAR.012 | Dual RICH Support System | The Dual RICH will be supported by a structural system within the barrel hadron calorimeter. | 6.03.01.09.01 |
| 9.10 | 6.03.01.09 | 6.03.01.05 | I-DET-DS-INF-BCK.001 | Backward HCAL Mobility | Adequate clearance must be provided for either of the detector carriages to be rolled aside to allow access to sub-detectors within the barrel. | 6.03.01.09.02 |
| 9.11 | 6.03.01.09 | 6.03.01.05 | I-DET-DS-INF-BCK.002 | Backward HCAL Support | The backward HCAL consists of two halves, each of which are support by an independent carriage. | 6.03.01.09.02 |
| 9.12 | 6.03.01.09 | 6.03.01.05 | I-DET-DS-INF-BCK.003 | Weight Transfer | The weight of each half of the backward HCAL will be transferred to the carriage that is supporting it. | 6.03.01.09.02 |
| 9.13 | 6.03.01.09 | 6.03.01.05 | I-DET-DS-INF-FWD.003 | Forward HCAL Support | The forward HCAL consists of two halves, each of which are support by an independent carriage. | 6.03.01.09.03 |
| 9.14 | 6.03.01.09 | 6.03.01.05 | I-DET-DS-INF-FWD.004 | Sub-Detector Support | The forward HCAL will support the weight of the forward electromagnetic calorimeter that is embedded within it. | 6.03.01.09.03 |
| 9.15 | 6.03.01.09 | 6.03.01.05 | I-DET-DS-INF-FWD.005 | Weight Transfer | The weight of each half of the forward HCAL, and its embedded subdetectors, will be transferred to the associated carriage. | 6.03.01.09.03 |
| 9.16 | 6.03.01.09 | 6.03.01.05 | I-DET-DS-INF-FWD.009 | Forward HCAL Mobility | Adequate clearance must be provided for either of the detector carriages to be rolled aside to allow access to sub-detectors within the barrel. | 6.03.01.09.03 |
| 9.17 | 6.03.01.09 | 6.03.01.05 | I-DET-DS-INF-FWD.010 | Forward HCAL Mobility | Adequate clearance must be provided for either of the detector carriages to be rolled aside to allow access to sub-detectors within the barrel. | 6.03.01.09.03 |
| 9.18 | 6.03.01.09 | 6.03.01.05 | I-DET-DS-INF-INT.005 | Space Constraint | The position of the forward HCAL is bounded by the B0 magnet in the forward direction. Changes to the size or position of either must be coordinated with the other. | 6.03.01.09.04 |
| 9.19 | 6.03.01.09 | 6.03.01.05 | I-DET-DS-INF-INT.016 | Space Constraint | The backward position of the backward ECAL is limited by the backward Hadron Calorimeter and the adjacent cabling pathway that provides services to the interior detectors. Modifications to either must be coordinated. | 6.03.01.09.04 |
| 9.20 | 6.03.01.09 | 6.03.01.05 | I-DET-DS-INF-INT.023 | Space Constraint | The exterior radius of the forward ECAL is limited by the interior bore of the barrel Hadron Calorimeter. Modifications to either must be coordinated. | 6.03.01.09.04 |
| 9.21 | 6.03.01.09 | 6.03.01.05 | I-DET-DS-INF-INT.024 | Space Constraint | The forward position of the forward ECAL is limited by the forward Hadron Calorimeter. Modifications to either must be coordinated. | 6.03.01.09.04 |
| 9.22 | 6.03.01.09 | 6.03.01.05 | I-DET-DS-INF-INT.027 | Space Constraint | The forward position of the backward HCAL is limited by the barrel HCAL and the adjacent cabling pathway that provides services to the interior detectors. Modifications to either must be coordinated. | 6.03.01.09.04 |
| 9.23 | 6.03.01.09 | 6.03.01.05 | I-DET-DS-INF-INT.028 | Space Constraint | The exterior radius of the backward HCAL should be consistent with the radius of the barrel HCAL. | 6.03.01.09.04 |
| 9.24 | 6.03.01.09 | 6.03.01.05 | I-DET-DS-INF-INT.029 | Space Constraint | The exterior radius of the forward HCAL should be consistent with the radius of the barrel HCAL. | 6.03.01.09.04 |
| 9.25 | 6.03.01.09 | 6.03.01.05 | I-DET-DS-INF-INT.030 | Space Constraint | The backward position of the forward HCAL is limited by the barrel HCAL and the adjacent cabling pathway that provides services to the interior detectors. Modifications to either must be coordinated. | 6.03.01.09.04 |
| 9.26 | 6.03.01.09 | 6.03.01.05 | I-DET-DS-INF-INT.031 | External Space Constraint | The exterior radius of the solenoid magnet is limited by the interior bore of the barrel HCAL and necessary support structures. Modifications to either must be coordinated. | 6.03.01.09.04 |
| 9.27 | 6.03.01.09 | 6.03.01.05 | I-DET-DS-INF-INT.032 | Space Constraint | The size of the dRICH is limited by the interior bore of the barrel HCAL, and must provide adequate space for cables and services to itself and the interior detectors. Modifications to either must be coordinated. | 6.03.01.09.04 |
| 9.28 | 6.03.01.09 | 6.03.01.05 | I-DET-DS-INF-INT.033 | Space Constraint | The forward position of the backward HCAL is limited by the barrel HCAL and the adjacent cabling pathway that provides services to the interior detectors. Modifications to either must be coordinated. | 6.03.01.09.04 |
| 9.29 | 6.03.01.09 | 6.03.01.05 | I-DET-DS-INF-INT.034 | Space Constraint | The maximum backward location for the solenoid magnet is limited by the backward HCAL and the adjacent cabling pathway that provides services to the interior detectors. Modifications to either must be coordinated. | 6.03.01.09.04 |
| 9.30 | 6.03.01.09 | 6.03.01.05 | I-DET-DS-INF-INT.035 | Space Constraint | The maximum backward location for the DIRC is limited by the backward HCAL and the adjacent cabling pathway that provides services to the interior detectors. Modifications to either must be coordinated. | 6.03.01.09.04 |
| 9.31 | 6.03.01.09 | 6.03.01.05 | I-DET-DS-INF-INT.036 | Space Constraint | The position of the backward HCAL is limited in the backward direction by the adjacent accelerator magnets. | 6.03.01.09.04 |
| 9.32 | 6.03.01.09 | 6.03.01.05 | I-DET-DS-INF-INT.037 | Space Constraint | The interior radius of the backward HCAL is governed by the size of the beamline. | 6.03.01.09.04 |
| 9.33 | 6.03.01.09 | 6.03.01.05 | I-DET-DS-INF-INT.038 | Space Constraint | The interior radius of the forward HCAL is governed by the size of the beamline. | 6.03.01.09.04 |
| 9.34 | 6.03.01.09 | 6.03.01.05 | I-DET-DS-INF-INT.072 | Heat Rejection | Heat from the silicon photomultipliers and front end electronics will be rejected into the outside room via forced air. | 6.03.01.09.04 |
| 9.35 | 6.03.01.09 | 6.03.01.05 | I-DET-DS-INF-INT.073 | Space Constraint | The exterior radius of the barrel HCAL is limited by the detector carriage, the electronics platforms, and other Hall infrastructure. | 6.03.01.09.04 |
| 9.36 | 6.03.01.09 | 6.03.01.05 | I-DET-DS-INF-INT.074 | Heat Rejection | Heat from the silicon photomultipliers and front end electronics will be rejected into the outside room via forced air. | 6.03.01.09.04 |
| 9.37 | 6.03.01.09 | 6.03.01.05 | I-DET-DS-INF-INT.075 | Heat Rejection | Heat from the silicon photomultipliers and front end electronics will be rejected into the outside room via forced air. DO WE NEED CW FOR COOLING HERE? | 6.03.01.09.04 |

Detector Infrastructure Systems to Solenoid Magnet

| Section | WBS1 | WBS2 | ID | Name | Description | Owner |
| --- | --- | --- | --- | --- | --- | --- |
| 10 |  | 6.03.01.06 |  |  | Solenoid Magnet |  |
| 10.1 | 6.03.01.09 | 6.03.01.06 | I-DET-DS-INF-BAR.015 | External Support Structure | Total weight of the solenoid and all embedded components must be supported. | 6.03.01.09.01 |
| 10.2 | 6.03.01.09 | 6.03.01.06 | I-DET-DS-INF-BAR.017 | DIRC Readout Supports | The DIRC prism box containing the photosensor readouts, will be supported by an external structure supporting the ECAL within the solenoid magnet. | 6.03.01.09.01 |
| 10.3 | 6.03.01.09 | 6.03.01.06 | I-DET-DS-INF-INT.001 | Magnetic Material | No detectors or components constructed with magnetic material may be added to the detector without prior consultation with the magnet group. | 6.03.01.09.04 |
| 10.4 | 6.03.01.09 | 6.03.01.06 | I-DET-DS-INF-INT.002 | Magnetic Field Requirement | All subordinate detectors are dependent on the continuous operation of the solenoid and on the delivery of a consistent, stable magnetic field. | 6.03.01.09.04 |
| 10.5 | 6.03.01.09 | 6.03.01.06 | I-DET-DS-INF-INT.012 | Space Constraint | The exterior radius of the barrel ECAL (and its support system) is limited by the interior bore of the solenoid magnet. Modifications to either must be coordinated. | 6.03.01.09.04 |
| 10.6 | 6.03.01.09 | 6.03.01.06 | I-DET-DS-INF-INT.031 | External Space Constraint | The exterior radius of the solenoid magnet is limited by the interior bore of the barrel HCAL and necessary support structures. Modifications to either must be coordinated. | 6.03.01.09.04 |
| 10.7 | 6.03.01.09 | 6.03.01.06 | I-DET-DS-INF-INT.034 | Space Constraint | The maximum backward location for the solenoid magnet is limited by the backward HCAL and the adjacent cabling pathway that provides services to the interior detectors. Modifications to either must be coordinated. | 6.03.01.09.04 |
| 10.8 | 6.03.01.09 | 6.03.01.06 | I-DET-DS-INF-INT.039 | Magnet Cooling | The solenoid will require a continuous flow of cryogens to maintain temperature in both the experimental hall and the maintenance area. Interface includes cryogenic source and warm return. | 6.03.01.09.04 |
| 10.9 | 6.03.01.09 | 6.03.01.06 | I-DET-DS-INF-INT.076 | Power Supply Cooling | The power supply, located in an adjacent electrical room, will need to be cooled with low conductivity water. | 6.03.01.09.04 |
| 10.10 | 6.03.01.09 | 6.03.01.06 | I-DET-DS-INF-INT.077 | Magnet Cooling | The solenoid will require a continuous flow of cryogens to maintain temperature in both the experimental hall and the maintenance area. Interface includes cryogenic source and warm return. | 6.03.01.09.04 |
| 10.11 | 6.03.01.09 | 6.03.01.06 | I-DET-DS-INF-INT.078 | Electrical Power | Solenoid will be provided power via the power supply located in an adjacent room. | 6.03.01.09.04 |
| 10.12 | 6.03.01.09 | 6.03.01.06 | I-DET-DS-INF-INT.079 | Instrumentation Backup Power | The instrumentation rack for the solenoid will require backup/UPS power adequate to safely shutdown the system in the event of a power failure. | 6.03.01.09.04 |
| 10.13 | 6.03.01.09 | 6.03.01.06 | I-DET-DS-INF-INT.080 | Fringe Field | A boundary must be established to identify the extent of the magnet's stray field during operations. | 6.03.01.09.04 |
| 10.14 | 6.03.01.09 | 6.03.01.06 | I-DET-DS-INF-INT.081 | Ice Management | An ice management system will be required to melt accumulated ice on the chimney. (Water should evaporate and will not require drainage) | 6.03.01.09.04 |
| 10.15 | 6.03.01.09 | 6.03.01.06 | I-DET-DS-INF-INT.082 | Cryogenic Connection Point | The cryogenic connection must be as close as possible to the source. | 6.03.01.09.04 |
| 10.16 | 6.03.01.09 | 6.03.01.06 | I-DET-DS-INF-INT.083 | LCW Connection | The LCW water cooled leads must be fully supported along their path between the electrical room and the magnet. | 6.03.01.09.04 |
| 10.17 | 6.03.01.09 | 6.03.01.06 | I-DET-DS-INF-INT.097 | Space Constraint | The maximum forward location for the solenoid is limited by the position of the Dual RICH detector and the cabling plenum (gap) between them. Modifications to either must be coordinated. | 6.03.01.09.04 |
| 10.18 | 6.03.01.09 | 6.03.01.06 | I-DET-DS-INF-INT.114 | LHE Flow (Hall) | A supply and return of liquid helium must be provided to the experimental hall for cooling the solenoid magnet during operation. | 6.03.01.09.04 |
| 10.19 | 6.03.01.09 | 6.03.01.06 | I-DET-DS-INF-INT.115 | LHE Flow (Assembly Area) | A supply and return of liquid helium must be provided to the assembly hall for cooling the solenoid magnet during maintenance. | 6.03.01.09.04 |

Detector Infrastructure Systems to Particle Identification Systems

| Section | WBS1 | WBS2 | ID | Name | Description | Owner |
| --- | --- | --- | --- | --- | --- | --- |
| 11 |  | 6.03.01.03 |  |  | Particle Identification Systems |  |
| 11.1 | 6.03.01.09 | 6.03.01.03 | I-DET-DS-INF-BAR.003 | Support Structure | The DIRC bar boxes will be supported by a frame inside the barrel Electromagnetic Calorimeter, that allows the boxes to be extracted using a system of rollers. | 6.03.01.09.01 |
| 11.2 | 6.03.01.09 | 6.03.01.03 | I-DET-DS-INF-BAR.005 | Weight Transfer | The weight of the backward ECAL will be transferred to the DIRC detector support and must be accommodated by all subsequent support systems. | 6.03.01.09.01 |
| 11.3 | 6.03.01.09 | 6.03.01.03 | I-DET-DS-INF-BAR.006 | EMCal Support System | The DIRC support system will provide support for the backward electromagnetic calorimeter. | 6.03.01.09.01 |
| 11.4 | 6.03.01.09 | 6.03.01.03 | I-DET-DS-INF-BAR.012 | Dual RICH Support System | The Dual RICH will be supported by a structural system within the barrel hadron calorimeter. | 6.03.01.09.01 |
| 11.5 | 6.03.01.09 | 6.03.01.03 | I-DET-DS-INF-BAR.017 | DIRC Readout Supports | The DIRC prism box containing the photosensor readouts, will be supported by an external structure supporting the ECAL within the solenoid magnet. | 6.03.01.09.01 |
| 11.6 | 6.03.01.09 | 6.03.01.03 | I-DET-DS-INF-BAR.018 | Structural Support | The pfRICH/mRICH will be supported by the DIRC bar box support system. | 6.03.01.09.01 |
| 11.7 | 6.03.01.09 | 6.03.01.03 | I-DET-DS-INF-BAR.019 | Barrel TOF Support | The barrel Time of Flight will be supported by the DIRC bar box assembly. | 6.03.01.09.01 |
| 11.8 | 6.03.01.09 | 6.03.01.03 | I-DET-DS-INF-BAR.020 | pfRICH/mRICH Support System | The DIRC support system will provide support for the upstream PID detector system. | 6.03.01.09.01 |
| 11.9 | 6.03.01.09 | 6.03.01.03 | I-DET-DS-INF-BAR.021 | Silicon Tracker Support System | The DIRC support system will provide support for the silicon trackers in the detector barrel. | 6.03.01.09.01 |
| 11.10 | 6.03.01.09 | 6.03.01.03 | I-DET-DS-INF-BAR.022 | Weight Distributed | The weight of the barrel tracking systems will be transferred to the DIRC support system. | 6.03.01.09.01 |
| 11.11 | 6.03.01.09 | 6.03.01.03 | I-DET-DS-INF-FWD.011 | Space Constraint | The interior radius of the dRICH is governed by the size of the beamline and its associated flanges and support system. | 6.03.01.09.03 |
| 11.12 | 6.03.01.09 | 6.03.01.03 | I-DET-DS-INF-FWD.012 | Space Constraint | The maximum forward location for the tracking system is limited by the position of the AC LGAD Time of Flight Detector. Modifications to either must be coordinated. | 6.03.01.09.03 |
| 11.13 | 6.03.01.09 | 6.03.01.03 | I-DET-DS-INF-INT.003 | Cabling/Services Conduit | The DIRC structure will provide integrated pathways for cabling and services to be delivered from the interior detectors (pf RICH/mRICH and silicon trackers) to the exterior infrastructure. | 6.03.01.09.04 |
| 11.14 | 6.03.01.09 | 6.03.01.03 | I-DET-DS-INF-INT.013 | Space Constraint | The backward position and shape of the barrel EMCAL is limited by the size and shape of the DIRC readout supports. Changes to the size or position of either must be coordinated with the other. | 6.03.01.09.04 |
| 11.15 | 6.03.01.09 | 6.03.01.03 | I-DET-DS-INF-INT.014 | Space Constraint | The maximum size of the DIRC is limited to the interior bore of the barrel ECAL (and its support structures). Modifications to either must be coordinated. | 6.03.01.09.04 |
| 11.16 | 6.03.01.09 | 6.03.01.03 | I-DET-DS-INF-INT.015 | Space Constraint | The forward position and shape of the barrel ECAL is limited by the backward face of the Dual RICH detector and the adjacent cabling pathway that provides services to the interior detectors. Modifications to either must be coordinated. | 6.03.01.09.04 |
| 11.17 | 6.03.01.09 | 6.03.01.03 | I-DET-DS-INF-INT.017 | Space Constraint | The maximum backward location for the DIRC is limited by the position of the backward electromagnetic calorimeter. Modifications to either must be coordinated. | 6.03.01.09.04 |
| 11.18 | 6.03.01.09 | 6.03.01.03 | I-DET-DS-INF-INT.018 | Space Constraint | The exterior radius of the backward ECAL is limited by the interior bore of the DIRC support system. Modifications to either must be coordinated. | 6.03.01.09.04 |
| 11.19 | 6.03.01.09 | 6.03.01.03 | I-DET-DS-INF-INT.019 | Heat Mitigation | The backward ECAL must mitigated the heat generated by the backward RICH detector. | 6.03.01.09.04 |
| 11.20 | 6.03.01.09 | 6.03.01.03 | I-DET-DS-INF-INT.020 | Space Constraint | The position of the backward ECAL in the forward direction is limited by the backward face of the pfRICH/mRICH detector. Modifications to either must be coordinated. | 6.03.01.09.04 |
| 11.21 | 6.03.01.09 | 6.03.01.03 | I-DET-DS-INF-INT.025 | Space Constraint | The maximum forward location for the dRICH is limited by the forward ECAL and the adjacent cabling pathway that provides services to the interior detectors. Modifications to either must be coordinated. | 6.03.01.09.04 |
| 11.22 | 6.03.01.09 | 6.03.01.03 | I-DET-DS-INF-INT.032 | Space Constraint | The size of the dRICH is limited by the interior bore of the barrel HCAL, and must provide adequate space for cables and services to itself and the interior detectors. Modifications to either must be coordinated. | 6.03.01.09.04 |
| 11.23 | 6.03.01.09 | 6.03.01.03 | I-DET-DS-INF-INT.035 | Space Constraint | The maximum backward location for the DIRC is limited by the backward HCAL and the adjacent cabling pathway that provides services to the interior detectors. Modifications to either must be coordinated. | 6.03.01.09.04 |
| 11.24 | 6.03.01.09 | 6.03.01.03 | I-DET-DS-INF-INT.084 | Process Cooling | The heat generated by the photo-multiplier tubes and associated electronics must be removed using a process cooling system. | 6.03.01.09.04 |
| 11.25 | 6.03.01.09 | 6.03.01.03 | I-DET-DS-INF-INT.085 | Process Cooling | The heat generated by the detector and its electronics must be removed using a process cooling system. | 6.03.01.09.04 |
| 11.26 | 6.03.01.09 | 6.03.01.03 | I-DET-DS-INF-INT.086 | Process Cooling | The heat generated by the photomultipliers and associated electronics must be removed using a process cooling system. | 6.03.01.09.04 |
| 11.27 | 6.03.01.09 | 6.03.01.03 | I-DET-DS-INF-INT.087 | Dry Nitrogen | A continuous flow of dry nitrogen will be required to protect the aerogel radiator. | 6.03.01.09.04 |
| 11.28 | 6.03.01.09 | 6.03.01.03 | I-DET-DS-INF-INT.088 | Photosensor Cooling System | A fluid cooling system will be required to keep the operating temperature of the photosensors at -30 degrees Celsius. | 6.03.01.09.04 |
| 11.29 | 6.03.01.09 | 6.03.01.03 | I-DET-DS-INF-INT.089 | Gas Recovery System | A recovery system will need to be included for the radiator gas used by the dRICH to prevent accidental release. | 6.03.01.09.04 |
| 11.30 | 6.03.01.09 | 6.03.01.03 | I-DET-DS-INF-INT.090 | Insulation | Sufficient insulation will need to be added around all coolant lines to prevent condensation and ensure that ice doesn't form. | 6.03.01.09.04 |
| 11.31 | 6.03.01.09 | 6.03.01.03 | I-DET-DS-INF-INT.091 | Dry Nitrogen for Aerogel | A continuous flow of dry nitrogen will be required to protect the aerogel radiator. | 6.03.01.09.04 |
| 11.32 | 6.03.01.09 | 6.03.01.03 | I-DET-DS-INF-INT.092 | Dry Nitrogen for Photosensors | This gas will be required to create a protective atmosphere for the cold photosensors and to prevent condensation and freezing. | 6.03.01.09.04 |
| 11.33 | 6.03.01.09 | 6.03.01.03 | I-DET-DS-INF-INT.093 | Radiator Gas | A continuous recirculating flow of radiator gas will be required. | 6.03.01.09.04 |
| 11.34 | 6.03.01.09 | 6.03.01.03 | I-DET-DS-INF-INT.097 | Space Constraint | The maximum forward location for the solenoid is limited by the position of the Dual RICH detector and the cabling plenum (gap) between them. Modifications to either must be coordinated. | 6.03.01.09.04 |
| 11.35 | 6.03.01.09 | 6.03.01.03 | I-DET-DS-INF-INT.098 | Space Constraint | The maximum diameter of the barrel Time of Flight detector is limited by the interior bore of the DIRC detector. Modifications to either must be coordinated. | 6.03.01.09.04 |
| 11.36 | 6.03.01.09 | 6.03.01.03 | I-DET-DS-INF-INT.099 | Space Constraint | The exterior radius of the pfRICH/mRICH is limited by the interior bore of the DIRC support system. Modifications to either must be coordinated. | 6.03.01.09.04 |
| 11.37 | 6.03.01.09 | 6.03.01.03 | I-DET-DS-INF-INT.100 | Space Constraint | The maximum forward location for the DIRC is limited by the position of the dRICH detector and the cabling plenum between them. Modifications to either must be coordinated. | 6.03.01.09.04 |
| 11.38 | 6.03.01.09 | 6.03.01.03 | I-DET-DS-INF-INT.101 | Service Pathway | Conduits must be provided within the DIRC support system that are adequate to deliver services (power, signal, cooling) to the barrel tracking detectors. | 6.03.01.09.04 |
| 11.39 | 6.03.01.09 | 6.03.01.03 | I-DET-DS-INF-INT.102 | Space Constraint | The exterior radius of the silicon trackers is limited by the interior bore of the DIRC support system. Modifications to either must be coordinated. | 6.03.01.09.04 |
| 11.40 | 6.03.01.09 | 6.03.01.03 | I-DET-DS-INF-INT.103 | Space Constraint | The maximum forward location for the DIRC is limited by the position of the silicon trackers. | 6.03.01.09.04 |
| 11.41 | 6.03.01.09 | 6.03.01.03 | I-DET-DS-INF-INT.104 | Space Constraint | The maximum backward location for the barrel Time of Flight detector is limited by the position of the pfRICH/mRICH detector and the adjacent cabling pathway. Modifications to either must be coordinated. | 6.03.01.09.04 |
| 11.42 | 6.03.01.09 | 6.03.01.03 | I-DET-DS-INF-INT.105 | Space Constraint | The maximum forward location for the barrel Time of Flight detector is limited by the position of the forward Time of Flight detector and the adjacent cabling pathway. Modifications to either must be coordinated. | 6.03.01.09.04 |
| 11.43 | 6.03.01.09 | 6.03.01.03 | I-DET-DS-INF-INT.106 | Space Constraint | The maximum size of the silicon trackers is limited by the interior radius of the barrel Time of Flight detector. Modifications to either must be coordinated. | 6.03.01.09.04 |
| 11.44 | 6.03.01.09 | 6.03.01.03 | I-DET-DS-INF-INT.107 | Space Constraint | The maximum backward location for the tracking system is limited by the position of the pfRICH/mRICH. Modifications to either must be coordinated. | 6.03.01.09.04 |
| 11.45 | 6.03.01.09 | 6.03.01.03 | I-DET-DS-INF-INT.108 | Space Constraint | The interior radius of the pfRICH is governed by the size of the beamline and its associated flanges and support system. | 6.03.01.09.04 |
| 11.46 | 6.03.01.09 | 6.03.01.03 | I-DET-DS-INF-INT.137 | Space Constraint | Rack space on the platform must be provided for a small laser system to monitor the response of the photsensors and allow adjustment of timing correlations within the detector. | 6.03.01.09.04 |
| 11.47 | 6.03.01.09 | 6.03.01.03 | I-DET-DS-INF-INT.138 | Space Constraint | Rack space on the platform must be provided for a small laser system to monitor the response of the photsensors and allow adjustment of timing correlations within the detector. | 6.03.01.09.04 |
| 11.48 | 6.03.01.09 | 6.03.01.03 | I-DET-DS-INF-INT.139 | Space Constraint | Rack space on the platform must be provided for a small laser system to monitor the response of the photsensors and allow adjustment of timing correlations within the detector. | 6.03.01.09.04 |

Detector Infrastructure Systems to Tracking Systems

| Section | WBS1 | WBS2 | ID | Name | Description | Owner |
| --- | --- | --- | --- | --- | --- | --- |
| 12 |  | 6.03.01.02 |  |  | Tracking Systems |  |
| 12.1 | 6.03.01.09 | 6.03.01.02 | I-DET-DS-INF-BAR.016 | Support Structure | A single structural support system will support the silicon detectors and the micro-pattern gaseous detectors within the DIRC detector. | 6.03.01.09.01 |
| 12.2 | 6.03.01.09 | 6.03.01.02 | I-DET-DS-INF-BAR.021 | Silicon Tracker Support System | The DIRC support system will provide support for the silicon trackers in the detector barrel. | 6.03.01.09.01 |
| 12.3 | 6.03.01.09 | 6.03.01.02 | I-DET-DS-INF-BAR.022 | Weight Distributed | The weight of the barrel tracking systems will be transferred to the DIRC support system. | 6.03.01.09.01 |
| 12.4 | 6.03.01.09 | 6.03.01.02 | I-DET-DS-INF-FWD.012 | Space Constraint | The maximum forward location for the tracking system is limited by the position of the AC LGAD Time of Flight Detector. Modifications to either must be coordinated. | 6.03.01.09.03 |
| 12.5 | 6.03.01.09 | 6.03.01.02 | I-DET-DS-INF-FWD.013 | Space Constraint | The interior radius of the tracking detectors is governed by the size of the beamline. | 6.03.01.09.03 |
| 12.6 | 6.03.01.09 | 6.03.01.02 | I-DET-DS-INF-INT.094 | Heat Rejection | Air, liquid or other cooling technology will be required for the tracking detectors. | 6.03.01.09.04 |
| 12.7 | 6.03.01.09 | 6.03.01.02 | I-DET-DS-INF-INT.095 | Ionization Gas | Gas will need to be provided to the trackers for detector operation. | 6.03.01.09.04 |
| 12.8 | 6.03.01.09 | 6.03.01.02 | I-DET-DS-INF-INT.096 | Support Structure | A single structural support system will support the silicon detectors and the micro-pattern gaseous detectors within the DIRC detector. | 6.03.01.09.04 |
| 12.9 | 6.03.01.09 | 6.03.01.02 | I-DET-DS-INF-INT.101 | Service Pathway | Conduits must be provided within the DIRC support system that are adequate to deliver services (power, signal, cooling) to the barrel tracking detectors. | 6.03.01.09.04 |
| 12.10 | 6.03.01.09 | 6.03.01.02 | I-DET-DS-INF-INT.102 | Space Constraint | The exterior radius of the silicon trackers is limited by the interior bore of the DIRC support system. Modifications to either must be coordinated. | 6.03.01.09.04 |
| 12.11 | 6.03.01.09 | 6.03.01.02 | I-DET-DS-INF-INT.103 | Space Constraint | The maximum forward location for the DIRC is limited by the position of the silicon trackers. | 6.03.01.09.04 |
| 12.12 | 6.03.01.09 | 6.03.01.02 | I-DET-DS-INF-INT.106 | Space Constraint | The maximum size of the silicon trackers is limited by the interior radius of the barrel Time of Flight detector. Modifications to either must be coordinated. | 6.03.01.09.04 |
| 12.13 | 6.03.01.09 | 6.03.01.02 | I-DET-DS-INF-INT.107 | Space Constraint | The maximum backward location for the tracking system is limited by the position of the pfRICH/mRICH. Modifications to either must be coordinated. | 6.03.01.09.04 |

# Detector Infrastructure Systems (DET-DS-INF) to Detector System (DET-DS)

I-DET-DS-INF-INT.001	Magnetic Material

System 1:	Detector System (6.03.01)

System 2:	Solenoid Magnet (6.03.01.06)

Type:	ENV

Description:	No detectors or components constructed with magnetic material may be added to the detector without prior consultation with the magnet group.

Provider:	Detector Infrastructure and Utilities Integration (6.03.01.09.04)

Details:	See details in the Detector Infrastructure Interface Control Document.

Requirements:	P-DET-DS-MAG.13, P-DET-DS-MAG.14

References:

I-DET-DS-INF-INT.002	Magnetic Field Requirement

System 1:	Detector System (6.03.01)

System 2:	Solenoid Magnet (6.03.01.06)

Type:	PARAM

Description:	All subordinate detectors are dependent on the continuous operation of the solenoid and on the delivery of a consistent, stable magnetic field.

Provider:	Detector Infrastructure and Utilities Integration (6.03.01.09.04)

Details:	See details in the Detector Infrastructure Interface Control Document.

Requirements:	P-DET-DS-MAG.13

References:

I-DET-DS-INF-INT.003	Cabling/Services Conduit

System 1:	Detector System (6.03.01)

System 2:	Barrel DIRC Particle ID Systems (6.03.01.03)

Type:	SPACE

Description:	The DIRC structure will provide integrated pathways for cabling and services to be delivered from the interior detectors (pf RICH/mRICH and silicon trackers) to the exterior infrastructure.

Provider:	Detector Infrastructure and Utilities Integration (6.03.01.09.04)

Details:	See details in the Detector Infrastructure Interface Control Document.

Requirements:	P-DET-DS-PID.4, P-DET-DS-PID.5

References:

# Detector Infrastructure Systems (DET-DS-INF) to Ancillary Detectors (DET-DS-ANC)

I-DET-DS-INF-BCK.004	LowQ2 Support

System 1:	Electron Endcap Structures (6.03.01.09.02)

System 2:	Low Q2 Detectors (6.03.01.10)

Type:	STRUCT

Description:	The LowQ2 detector is supported by a freestanding platform that is adjacent to the outgoing electron beam.

Provider:	Electron Endcap Structures (6.03.01.09.02)

Details:	See details in the Detector Infrastructure Interface Control Document.

Requirements:	P-DET-DS-ANC.1, P-DET-DS-ANC.2

References:

I-DET-DS-INF-BCK.005	Lumi Support System

System 1:	Electron Endcap Structures (6.03.01.09.02)

System 2:	Luminosity Detectors (6.03.01.10)

Type:	STRUCT

Description:	The luminosity detector will be supported by an independent support system.

Provider:	Electron Endcap Structures (6.03.01.09.02)

Details:	See details in the Detector Infrastructure Interface Control Document.

Requirements:	P-DET-DS-ANC.1, P-DET-DS-ANC.2

References:

I-DET-DS-INF-FWD.001	Detector Support

System 1:	Off-Momentum Detectors (6.03.01.10)

System 2:	IR Accelerator Systems (6.04.03)

Type:	STRUCT

Description:	The detector will be supported by an integrated support stand that is fully integrated with the vacuum system and allows the position of the detector to be adjusted.

Provider:	Hadron Endcap Structures (6.03.01.09.03)

Details:	See details in the Detector Infrastructure Interface Control Document.

Requirements:	P-DET-DS-ANC.1, P-DET-DS-ANC.2

References:

I-DET-DS-INF-FWD.002	Detector Support

System 1:	Roman Pots (6.03.01.10)

System 2:	IR Accelerator Systems (6.04.03)

Type:	STRUCT

Description:	The detector will be supported by an integrated support stand that is fully integrated with the vacuum system and allows the position of the detector to be adjusted.

Provider:	Hadron Endcap Structures (6.03.01.09.03)

Details:	See details in the Detector Infrastructure Interface Control Document.

Requirements:	P-DET-DS-ANC.1, P-DET-DS-ANC.2

References:

I-DET-DS-INF-FWD.006	B0 Support System

System 1:	B-Zero Detectors (6.03.01.10)

System 2:	IR Superconducting Magnets (6.04.02)

Type:	STRUCT

Description:	The B0 detector will be supported in the interior of the B0 magnet and will be dependent on the B0 magnet design, and must be adequate to support the weight of the crystals and the tracking detectors.

Provider:	Hadron Endcap Structures (6.03.01.09.03)

Details:	See details in the Detector Infrastructure Interface Control Document.

Requirements:	P-DET-DS-ANC.1, P-DET-DS-ANC.2

References:

I-DET-DS-INF-FWD.007	ZDC Support System

System 1:	Hadron Endcap Structures (6.03.01.09.03)

System 2:	Zero Degree Calorimeter (6.03.01.10)

Type:	STRUCT

Description:	The detector will be supported on a free standing structure.

Provider:	Hadron Endcap Structures (6.03.01.09.03)

Details:	See details in the Detector Infrastructure Interface Control Document.

Requirements:	P-DET-DS-ANC.1, P-DET-DS-ANC.2

References:

I-DET-DS-INF-INT.004	Space Constraint

System 1:	B-Zero Detectors (6.03.01.10)

System 2:	B-Zero Detectors (6.03.01.10)

Type:	SPACE

Description:	B0 detector must fit entirely within the bore of the B0 magnet.

Provider:	Detector Infrastructure and Utilities Integration (6.03.01.09.04)

Details:	See details in the Detector Infrastructure Interface Control Document.

Requirements:	P-DET-DS-ANC.1

References:

I-DET-DS-INF-INT.005	Space Constraint

System 1:	B-Zero Detectors (6.03.01.10)

System 2:	Forward HCal Systems (6.03.01.05)

Type:	SPACE

Description:	The position of the forward HCAL is bounded by the B0 magnet in the forward direction. Changes to the size or position of either must be coordinated with the other.

Provider:	Detector Infrastructure and Utilities Integration (6.03.01.09.04)

Details:	See details in the Detector Infrastructure Interface Control Document.

Requirements:	P-DET-DS-ANC.1, P-DET-DS-HCAL.1

References:

I-DET-DS-INF-INT.006	Space Constraint

System 1:	B-Zero Detectors (6.03.01.10)

System 2:	IR Superconducting Magnets (6.04.02)

Type:	SPACE

Description:	The position of and accessibility of the B0 detector in the backward direction is limited by the cryostat containing the final focusing magnets.

Provider:	Detector Infrastructure and Utilities Integration (6.03.01.09.04)

Details:	See details in the Detector Infrastructure Interface Control Document.

Requirements:	P-DET-DS-ANC.1

References:

I-DET-DS-INF-INT.007	Space Constraint

System 1:	B-Zero Detectors (6.03.01.10)

System 2:	IR Accelerator Systems (6.04.03)

Type:	SPACE

Description:	Access to the B0 detector is constrained by the vacuum valve in front of the B0 magnet. A minimum of 15 centimeters of free space will be required on the IP-side of the B0-dipole for the installation of the B0 detectors.

Provider:	Detector Infrastructure and Utilities Integration (6.03.01.09.04)

Details:	See details in the Detector Infrastructure Interface Control Document.

Requirements:	P-DET-DS-ANC.1

References:

I-DET-DS-INF-INT.008	Space Constraint

System 1:	B-Zero Detectors (6.03.01.10)

System 2:	IR Accelerator Systems (6.04.03)

Type:	SPACE

Description:	The electron and hadron beam pipes must be contained within the B0 detector.

Provider:	Detector Infrastructure and Utilities Integration (6.03.01.09.04)

Details:	See details in the Detector Infrastructure Interface Control Document.

Requirements:	P-DET-DS-ANC.1

References:

I-DET-DS-INF-INT.009	Position Constraint

System 1:	Low Q2 Detectors (6.03.01.10)

System 2:	IR Accelerator Systems (6.04.03)

Type:	SPACE

Description:	The LowQ2 detector must be upstream in the electron direction from the Q3ER magnet.

Provider:	Detector Infrastructure and Utilities Integration (6.03.01.09.04)

Details:	See details in the Detector Infrastructure Interface Control Document.

Requirements:	P-DET-DS-ANC.1

References:

I-DET-DS-INF-INT.010	Position Constraint

System 1:	Low Q2 Detectors (6.03.01.10)

System 2:	IR Accelerator Systems (6.04.03)

Type:	SPACE

Description:	The LowQ2 detector must be downstream in the electron direction from the B2ER magnet.

Provider:	Detector Infrastructure and Utilities Integration (6.03.01.09.04)

Details:	See details in the Detector Infrastructure Interface Control Document.

Requirements:	P-DET-DS-ANC.1

References:

I-DET-DS-INF-INT.011	Position Constraint

System 1:	Low Q2 Detectors (6.03.01.10)

System 2:	IR Systems Integration (6.04.04)

Type:	SPACE

Description:	The LOWQ2 detector must be at the same level as the electron beamline.

Provider:	Detector Infrastructure and Utilities Integration (6.03.01.09.04)

Details:	See details in the Detector Infrastructure Interface Control Document.

Requirements:	P-DET-DS-ANC.1

References:

I-DET-DS-INF-INT.040	Process Cooling

System 1:	B-Zero Detectors (6.03.01.10)

System 2:	IR Systems Integration (6.04.04)

Type:	SPACE

Description:	A cooling system will be required to remove heat from the calorimeter and electronics readouts, to maintain an acceptable temperature. A chiller should be installed near  the sub-system and  a space should be allocated  for this chiller.

Provider:	Detector Infrastructure and Utilities Integration (6.03.01.09.04)

Details:	See details in the Detector Infrastructure Interface Control Document.

Requirements:	P-DET-DS-ANC.3

References:

I-DET-DS-INF-INT.041	Process Cooling

System 1:	Detector Infrastructure and Utilities Integration (6.03.01.09.04)

System 2:	Low Q2 Detectors (6.03.01.10)

Type:	COOL

Description:	Either a liquid or gas cooling system will be required to remove heat from the calorimeter, tracking and readout electronics, to maintain them at room temperature.

Provider:	Detector Infrastructure and Utilities Integration (6.03.01.09.04)

Details:	See details in the Detector Infrastructure Interface Control Document.

Requirements:	P-DET-DS-ANC.3

References:

I-DET-DS-INF-INT.042	Dipole Power

System 1:	Detector Infrastructure and Utilities Integration (6.03.01.09.04)

System 2:	Luminosity Detectors (6.03.01.10)

Type:	ELEC

Description:	The two dipoles in front of the detector will require an infrastructure power source.

Provider:	Detector Infrastructure and Utilities Integration (6.03.01.09.04)

Details:	See details in the Detector Infrastructure Interface Control Document.

Requirements:	P-DET-DS-ANC.4

References:

I-DET-DS-INF-INT.043	Dipole Vacuum

System 1:	Detector Infrastructure and Utilities Integration (6.03.01.09.04)

System 2:	Luminosity Detectors (6.03.01.10)

Type:	VAC

Description:	The beamline between the two luminosity detector dipoles must be under vacuum.

Provider:	Detector Infrastructure and Utilities Integration (6.03.01.09.04)

Details:	See details in the Detector Infrastructure Interface Control Document.

Requirements:	P-DET-DS-ANC-LUMI.10

References:

I-DET-DS-INF-INT.044	Process Cooling

System 1:	Detector Infrastructure and Utilities Integration (6.03.01.09.04)

System 2:	Off-Momentum Detectors (6.03.01.10)

Type:	COOL

Description:	A cooling system will be required to remove heat from the tracking and readout electronics to prevent heat buildup.

Provider:	Detector Infrastructure and Utilities Integration (6.03.01.09.04)

Details:	See details in the Detector Infrastructure Interface Control Document.

Requirements:	P-DET-DS-ANC.3

References:

I-DET-DS-INF-INT.045	Process Cooling

System 1:	Detector Infrastructure and Utilities Integration (6.03.01.09.04)

System 2:	Roman Pots (6.03.01.10)

Type:	COOL

Description:	A cooling system will be required to remove heat from the tracking and readout electronics to prevent heat buildup.

Provider:	Detector Infrastructure and Utilities Integration (6.03.01.09.04)

Details:	See details in the Detector Infrastructure Interface Control Document.

Requirements:	P-DET-DS-ANC.3

References:

I-DET-DS-INF-INT.046	Process Cooling

System 1:	Detector Infrastructure and Utilities Integration (6.03.01.09.04)

System 2:	Zero Degree Calorimeter (6.03.01.10)

Type:	COOL

Description:	A cooling system will be required to remove heat from the calorimeter and readout electronics to prevent heat buildup.

Provider:	Detector Infrastructure and Utilities Integration (6.03.01.09.04)

Details:	See details in the Detector Infrastructure Interface Control Document.

Requirements:	P-DET-DS-ANC.3

References:

# Detector Infrastructure Systems (DET-DS-INF) to Data Acquisition and Computing Systems (DET-DS-COMP)

I-DET-DS-INF-BAR.013	Fiber Support System

System 1:	Barrel Detector Structures (6.03.01.09.01)

System 2:	Online DAQ and Computing Systems (6.03.01.08.02)

Type:	STRUCT

Description:	A fiber support system must be provided that allows the communications cabling to remain connected while the detector is moved from the experimental hall to the assembly area.

Provider:	Barrel Detector Structures (6.03.01.09.01)

Details:	See details in the Detector Infrastructure Interface Control Document.

Requirements:	P-DET-DS-COMP.2

References:

I-DET-DS-INF-INT.047	Computer Room Cooling

System 1:	Detector Infrastructure and Utilities Integration (6.03.01.09.04)

System 2:	Online DAQ and Computing Systems (6.03.01.08.02)

Type:	COOL

Description:	HVAC systems must be provided to maintain stable temperature and humidity in the computer room.

Provider:	Detector Infrastructure and Utilities Integration (6.03.01.09.04)

Details:	See details in the Detector Infrastructure Interface Control Document.

Requirements:	P-DET-DS-COMP.3

References:

I-DET-DS-INF-INT.048	Experimental and Assembly Area Cooling

System 1:	Detector Infrastructure and Utilities Integration (6.03.01.09.04)

System 2:	Online DAQ and Computing Systems (6.03.01.08.02)

Type:	COOL

Description:	General HVAC systems must be provided to maintain a normal operating temperature in locations where DAQ systems are in use.

Provider:	Detector Infrastructure and Utilities Integration (6.03.01.09.04)

Details:	See details in the Detector Infrastructure Interface Control Document.

Requirements:	P-DET-DS-COMP.3

References:

I-DET-DS-INF-INT.049	Backup Power

System 1:	Detector Infrastructure and Utilities Integration (6.03.01.09.04)

System 2:	Online DAQ and Computing Systems (6.03.01.08.02)

Type:	ELEC

Description:	UPS systems must be provided to support critical systems in order to tolerate transient conditions and to allow for controlled shutdown.

Provider:	Detector Infrastructure and Utilities Integration (6.03.01.09.04)

Details:	See details in the Detector Infrastructure Interface Control Document.

Requirements:	P-DET-DS-COMP.4

References:

I-DET-DS-INF-INT.050	Computer Room Power

System 1:	Detector Infrastructure and Utilities Integration (6.03.01.09.04)

System 2:	Online DAQ and Computing Systems (6.03.01.08.02)

Type:	ELEC

Description:	Adequate power must be provided to support the DAQ systems and computing infrastructure.

Provider:	Detector Infrastructure and Utilities Integration (6.03.01.09.04)

Details:	See details in the Detector Infrastructure Interface Control Document.

Requirements:	P-DET-DS-COMP.4

References:

I-DET-DS-INF-INT.051	Control Room Power

System 1:	Detector Infrastructure and Utilities Integration (6.03.01.09.04)

System 2:	Online DAQ and Computing Systems (6.03.01.08.02)

Type:	ELEC

Description:	Standard electrical service to support consoles and systems in the control room.

Provider:	Detector Infrastructure and Utilities Integration (6.03.01.09.04)

Details:	See details in the Detector Infrastructure Interface Control Document.

Requirements:	P-DET-DS-COMP.4

References:

I-DET-DS-INF-INT.052	Hall Power

System 1:	Detector Infrastructure and Utilities Integration (6.03.01.09.04)

System 2:	Online DAQ and Computing Systems (6.03.01.08.02)

Type:	ELEC

Description:	Clean power (either transformer or source isolated) should be provided for all DAQ, computing and network equipment.

Provider:	Detector Infrastructure and Utilities Integration (6.03.01.09.04)

Details:	See details in the Detector Infrastructure Interface Control Document.

Requirements:	P-DET-DS-COMP.6

References:

I-DET-DS-INF-INT.053	Carriage Rack Space

System 1:	Detector Infrastructure and Utilities Integration (6.03.01.09.04)

System 2:	Online DAQ and Computing Systems (6.03.01.08.02)

Type:	SPACE

Description:	Adequate rack space for the DAQ system will be required on the detector carriages.

Provider:	Detector Infrastructure and Utilities Integration (6.03.01.09.04)

Details:	See details in the Detector Infrastructure Interface Control Document.

Requirements:	P-DET-DS-COMP.2

References:

I-DET-DS-INF-INT.054	Computer Room

System 1:	Detector Infrastructure and Utilities Integration (6.03.01.09.04)

System 2:	Online DAQ and Computing Systems (6.03.01.08.02)

Type:	SPACE

Description:	Adequate raised-floor space with racks will be required  to support DAQ computers, communications systems, and computing infrastructure.

Provider:	Detector Infrastructure and Utilities Integration (6.03.01.09.04)

Details:	See details in the Detector Infrastructure Interface Control Document.

Requirements:	P-DET-DS-COMP.9

References:

I-DET-DS-INF-INT.055	Control Systems Furniture

System 1:	Detector Infrastructure and Utilities Integration (6.03.01.09.04)

System 2:	Online DAQ and Computing Systems (6.03.01.08.02)

Type:	SPACE

Description:	Space for computing consoles and furniture adequate to manage the system during operation will be required. Consoles and furniture will be provided by DET-INT.

Provider:	Detector Infrastructure and Utilities Integration (6.03.01.09.04)

Details:	See details in the Detector Infrastructure Interface Control Document.

Requirements:	P-DET-DS-COMP.2

References:

I-DET-DS-INF-INT.056	Detector Plenums

System 1:	Detector Infrastructure and Utilities Integration (6.03.01.09.04)

System 2:	Online DAQ and Computing Systems (6.03.01.08.02)

Type:	SPACE

Description:	Adequate space to route fiberoptic cables from the racks to the detector readout boards will be required.

Provider:	Detector Infrastructure and Utilities Integration (6.03.01.09.04)

Details:	See details in the Detector Infrastructure Interface Control Document.

Requirements:	P-DET-DS-COMP.2

References:

I-DET-DS-INF-INT.057	Fixed Platform Conduits

System 1:	Detector Infrastructure and Utilities Integration (6.03.01.09.04)

System 2:	Online DAQ and Computing Systems (6.03.01.08.02)

Type:	SPACE

Description:	Adequate conduits and cableways must be provided to deliver cabling to the racks on the fixed platforms.

Provider:	Detector Infrastructure and Utilities Integration (6.03.01.09.04)

Details:	See details in the Detector Infrastructure Interface Control Document.

Requirements:	P-DET-DS-COMP.2

References:

I-DET-DS-INF-INT.058	Platform/Tunnel Rack Space

System 1:	Detector Infrastructure and Utilities Integration (6.03.01.09.04)

System 2:	Online DAQ and Computing Systems (6.03.01.08.02)

Type:	SPACE

Description:	Adequate rack space for the DAQ system must be provided on the fixed platforms, tunnels, and in the experimental hall.

Provider:	Detector Infrastructure and Utilities Integration (6.03.01.09.04)

Details:	See details in the Detector Infrastructure Interface Control Document.

Requirements:	P-DET-DS-COMP.2

References:

I-DET-DS-INF-INT.059	Sensitive Electronics

System 1:	Detector Infrastructure and Utilities Integration (6.03.01.09.04)

System 2:	Online DAQ and Computing Systems (6.03.01.08.02)

Type:	SPACE

Description:	DAQ systems that are sensitive to magnetism or radiation must be placed at a sufficient distance from sources OR shielding must be provided to protect the equipment.

Provider:	Detector Infrastructure and Utilities Integration (6.03.01.09.04)

Details:	See details in the Detector Infrastructure Interface Control Document.

Requirements:	P-DET-DS-COMP.14

References:

I-DET-DS-INF-INT.060	Tunnel Conduits

System 1:	Detector Infrastructure and Utilities Integration (6.03.01.09.04)

System 2:	Online DAQ and Computing Systems (6.03.01.08.02)

Type:	SPACE

Description:	Conduits must be provided in the west and east tunnels to connect the detectors to the DAQ system.

Provider:	Detector Infrastructure and Utilities Integration (6.03.01.09.04)

Details:	See details in the Detector Infrastructure Interface Control Document.

Requirements:	P-DET-DS-COMP.8

References:

I-DET-DS-INF-INT.061	Fiber Support System

System 1:	Detector Infrastructure and Utilities Integration (6.03.01.09.04)

System 2:	Online DAQ and Computing Systems (6.03.01.08.02)

Type:	STRUCT

Description:	A fiber support system must be provided that allows the communications cabling to remain connected while the detector is moved from the experimental hall to the assembly area.

Provider:	Detector Infrastructure and Utilities Integration (6.03.01.09.04)

Details:	See details in the Detector Infrastructure Interface Control Document.

Requirements:	P-DET-DS-COMP.8

References:

# Detector Infrastructure Systems (DET-DS-INF) to Electromagnetic Calorimetry Systems (DET-DS-ECAL)

I-DET-DS-INF-BAR.001	Barrel ECAL Support

System 1:	Barrel EMCal Systems (6.03.01.04)

System 2:	Barrel HCal Systems (6.03.01.05)

Type:	STRUCT

Description:	The barrel ECAL will be supported by a structural support system that extends through the bore of the solenoid magnet and is supported by the barrel Hadron Calorimeter.

Provider:	Barrel Detector Structures (6.03.01.09.01)

Details:	See details in the Detector Infrastructure Interface Control Document.

Requirements:	P-DET-DS-ECAL.2, P-DET-DS-HCAL.2

References:

I-DET-DS-INF-BAR.002	Weight Distributed

System 1:	Barrel EMCal Systems (6.03.01.04)

System 2:	Forward HCal Systems (6.03.01.05)

Type:	STRUCT

Description:	The weight of the barrel ECAL will be transferred to the barrel Hadron Calorimeter and must be accommodated by all intermediate and subsequent support systems.

Provider:	Barrel Detector Structures (6.03.01.09.01)

Details:	See details in the Detector Infrastructure Interface Control Document.

Requirements:	P-DET-DS-ECAL.2, P-DET-DS-HCAL.2

References:

I-DET-DS-INF-BAR.003	Support Structure

System 1:	Barrel EMCal Systems (6.03.01.04)

System 2:	Barrel DIRC Particle ID Systems (6.03.01.03)

Type:	STRUCT

Description:	The DIRC bar boxes will be supported by a frame inside the barrel Electromagnetic Calorimeter, that allows the boxes to be extracted using a system of rollers.

Provider:	Barrel Detector Structures (6.03.01.09.01)

Details:	See details in the Detector Infrastructure Interface Control Document.

Requirements:	P-DET-DS-ECAL.1, P-DET-DS-ECAL.2, P-DET-DS-PID.4, P-DET-DS-PID.5

References:

I-DET-DS-INF-BAR.004	Backward ECAL Support

System 1:	Backward EMCal Systems (6.03.01.04)

System 2:	Forward HCal Systems (6.03.01.05)

Type:	STRUCT

Description:	The backward ECAL will be supported by an integrated support structure that is attached to the DIRC support frame.

Provider:	Barrel Detector Structures (6.03.01.09.01)

Details:	See details in the Detector Infrastructure Interface Control Document.

Requirements:	P-DET-DS-ECAL.2, P-DET-DS-HCAL.2

References:

I-DET-DS-INF-BAR.005	Weight Transfer

System 1:	Backward EMCal Systems (6.03.01.04)

System 2:	Barrel DIRC Particle ID Systems (6.03.01.03)

Type:	STRUCT

Description:	The weight of the backward ECAL will be transferred to the DIRC detector support and must be accommodated by all subsequent support systems.

Provider:	Barrel Detector Structures (6.03.01.09.01)

Details:	See details in the Detector Infrastructure Interface Control Document.

Requirements:	P-DET-DS-ECAL.2, P-DET-DS-HCAL.2

References:

I-DET-DS-INF-BAR.006	EMCal Support System

System 1:	Backward EMCal Systems (6.03.01.04)

System 2:	Barrel DIRC Particle ID Systems (6.03.01.03)

Type:	STRUCT

Description:	The DIRC support system will provide support for the backward electromagnetic calorimeter.

Provider:	Barrel Detector Structures (6.03.01.09.01)

Details:	See details in the Detector Infrastructure Interface Control Document.

Requirements:	P-DET-DS-ECAL.2, P-DET-DS-PID.4

References:

I-DET-DS-INF-BAR.007	Forward ECAL Support

System 1:	Forward EMCal Systems (6.03.01.04)

System 2:	Forward HCal Systems (6.03.01.05)

Type:	STRUCT

Description:	The forward ECAL will be supported by the forward Hadron Calorimeter endcap. Because the forward ECAL must split into two parts to provide access to the barrel, each half must be independently affixed to the Hadron Calorimeter halves.

Provider:	Barrel Detector Structures (6.03.01.09.01)

Details:	See details in the Detector Infrastructure Interface Control Document.

Requirements:	P-DET-DS-ECAL.1, P-DET-DS-ECAL.2, P-DET-DS-HCAL.2

References:

I-DET-DS-INF-BAR.008	Weight Transfer

System 1:	Forward EMCal Systems (6.03.01.04)

System 2:	Forward HCal Systems (6.03.01.05)

Type:	STRUCT

Description:	The weight of the forward ECAL will be transferred to the forward Hadron Calorimeter endcap and must be accommodated by all subsequent support systems.

Provider:	Barrel Detector Structures (6.03.01.09.01)

Details:	See details in the Detector Infrastructure Interface Control Document.

Requirements:	P-DET-DS-ECAL.1, P-DET-DS-ECAL.2, P-DET-DS-HCAL.2

References:

I-DET-DS-INF-BAR.014	Forward ECAL Mobility Support

System 1:	Barrel Detector Structures (6.03.01.09.01)

System 2:	Forward EMCal Systems (6.03.01.04)

Type:	MECH

Description:	Each half of the forward ECAL must be continuously supported and stabilized while it is moved between the opened and closed positions.

Provider:	Barrel Detector Structures (6.03.01.09.01)

Details:	See details in the Detector Infrastructure Interface Control Document.

Requirements:	P-DET-DS-ECAL.1, P-DET-DS-ECAL.2

References:

I-DET-DS-INF-FWD.004	Sub-Detector Support

System 1:	Forward HCal Systems (6.03.01.05)

System 2:	Forward EMCal Systems (6.03.01.04)

Type:	STRUCT

Description:	The forward HCAL will support the weight of the forward electromagnetic calorimeter that is embedded within it.

Provider:	Hadron Endcap Structures (6.03.01.09.03)

Details:	See details in the Detector Infrastructure Interface Control Document.

Requirements:	P-DET-DS-HCAL.2

References:

I-DET-DS-INF-FWD.008	Forward ECAL Mobility Support

System 1:	Hadron Endcap Structures (6.03.01.09.03)

System 2:	Forward EMCal Systems (6.03.01.04)

Type:	MECH

Description:	Each half of the forward ECAL must be continuously supported and stabilized while it is moved between the opened and closed positions.

Provider:	Hadron Endcap Structures (6.03.01.09.03)

Details:	See details in the Detector Infrastructure Interface Control Document.

Requirements:	P-DET-DS-ECAL.1, P-DET-DS-ECAL.2

References:

I-DET-DS-INF-INT.012	Space Constraint

System 1:	Barrel EMCal Systems (6.03.01.04)

System 2:	Solenoid Magnet (6.03.01.06)

Type:	SPACE

Description:	The exterior radius of the barrel ECAL (and its support system) is limited by the interior bore of the solenoid magnet. Modifications to either must be coordinated.

Provider:	Detector Infrastructure and Utilities Integration (6.03.01.09.04)

Details:	See details in the Detector Infrastructure Interface Control Document.

Requirements:	P-DET-DS-ECAL.1, P-DET-DS-MAG.1

References:

I-DET-DS-INF-INT.013	Space Constraint

System 1:	Barrel EMCal Systems (6.03.01.04)

System 2:	Barrel DIRC Particle ID Systems (6.03.01.03)

Type:	SPACE

Description:	The backward position and shape of the barrel EMCAL is limited by the size and shape of the DIRC readout supports.  Changes to the size or position of either must be coordinated with the other.

Provider:	Detector Infrastructure and Utilities Integration (6.03.01.09.04)

Details:	See details in the Detector Infrastructure Interface Control Document.

Requirements:	P-DET-DS-ECAL.1, P-DET-DS-PID.5

References:

I-DET-DS-INF-INT.014	Space Constraint

System 1:	Barrel EMCal Systems (6.03.01.04)

System 2:	Barrel DIRC Particle ID Systems (6.03.01.03)

Type:	SPACE

Description:	The maximum size of the DIRC is limited to the interior bore of the barrel ECAL (and its support structures).  Modifications to either must be coordinated.

Provider:	Detector Infrastructure and Utilities Integration (6.03.01.09.04)

Details:	See details in the Detector Infrastructure Interface Control Document.

Requirements:	P-DET-DS-ECAL.1, P-DET-DS-PID.5

References:

I-DET-DS-INF-INT.015	Space Constraint

System 1:	Barrel EMCal Systems (6.03.01.04)

System 2:	Forward RICH Detector Systems (6.03.01.03)

Type:	SPACE

Description:	The forward position and shape of the barrel ECAL is limited  by the backward face of the Dual RICH detector and the adjacent cabling pathway that provides services to the interior detectors. Modifications to either must be coordinated.

Provider:	Detector Infrastructure and Utilities Integration (6.03.01.09.04)

Details:	See details in the Detector Infrastructure Interface Control Document.

Requirements:	P-DET-DS-ECAL.1, P-DET-DS-PID.5

References:

I-DET-DS-INF-INT.016	Space Constraint

System 1:	Backward EMCal Systems (6.03.01.04)

System 2:	Backward HCal Systems (6.03.01.05)

Type:	SPACE

Description:	The backward position of the backward ECAL is limited by the backward Hadron Calorimeter and the adjacent cabling pathway that provides services to the interior detectors. Modifications to either must be coordinated.

Provider:	Detector Infrastructure and Utilities Integration (6.03.01.09.04)

Details:	See details in the Detector Infrastructure Interface Control Document.

Requirements:	P-DET-DS-ECAL.1, P-DET-DS-HCAL.1

References:

I-DET-DS-INF-INT.017	Space Constraint

System 1:	Backward EMCal Systems (6.03.01.04)

System 2:	Barrel DIRC Particle ID Systems (6.03.01.03)

Type:	SPACE

Description:	The maximum backward location for the DIRC  is limited by the position of the backward electromagnetic calorimeter. Modifications to either must be coordinated.

Provider:	Detector Infrastructure and Utilities Integration (6.03.01.09.04)

Details:	See details in the Detector Infrastructure Interface Control Document.

Requirements:	P-DET-DS-ECAL.1, P-DET-DS-PID.5

References:

I-DET-DS-INF-INT.018	Space Constraint

System 1:	Backward EMCal Systems (6.03.01.04)

System 2:	Barrel DIRC Particle ID Systems (6.03.01.03)

Type:	SPACE

Description:	The exterior radius of the backward ECAL is limited by the interior bore of the DIRC support system. Modifications to either must be coordinated.

Provider:	Detector Infrastructure and Utilities Integration (6.03.01.09.04)

Details:	See details in the Detector Infrastructure Interface Control Document.

Requirements:	P-DET-DS-ECAL.1, P-DET-DS-PID.5

References:

I-DET-DS-INF-INT.019	Heat Mitigation

System 1:	Backward EMCal Systems (6.03.01.04)

System 2:	Backward Particle ID Systems (6.03.01.03)

Type:	ENV

Description:	The backward ECAL must mitigated the heat generated by the backward RICH detector.

Provider:	Detector Infrastructure and Utilities Integration (6.03.01.09.04)

Details:	See details in the Detector Infrastructure Interface Control Document.

Requirements:	P-DET-DS-ECAL.3

References:

I-DET-DS-INF-INT.020	Space Constraint

System 1:	Backward EMCal Systems (6.03.01.04)

System 2:	Backward RICH Detector Systems (6.03.01.03)

Type:	SPACE

Description:	The position of the backward ECAL in the forward direction is limited by the backward face of the pfRICH/mRICH detector.  Modifications to either must be coordinated.

Provider:	Detector Infrastructure and Utilities Integration (6.03.01.09.04)

Details:	See details in the Detector Infrastructure Interface Control Document.

Requirements:	P-DET-DS-ECAL.1, P-DET-DS-PID.5

References:

I-DET-DS-INF-INT.021	Backward ECAL Bore Clearance

System 1:	Backward EMCal Systems (6.03.01.04)

System 2:	IR Accelerator Systems (6.04.03)

Type:	SPACE

Description:	The bore of the backward ECAL must be designed to allow it to be inserted/removed over the existing beamline flanges.

Provider:	Detector Infrastructure and Utilities Integration (6.03.01.09.04)

Details:	See details in the Detector Infrastructure Interface Control Document.

Requirements:	P-DET-DS-ECAL.1

References:

I-DET-DS-INF-INT.022	Space Constraint

System 1:	Backward EMCal Systems (6.03.01.04)

System 2:	IR Accelerator Systems (6.04.03)

Type:	SPACE

Description:	The interior radius of the backward ECAL is governed by the size of the beamline.

Provider:	Detector Infrastructure and Utilities Integration (6.03.01.09.04)

Details:	See details in the Detector Infrastructure Interface Control Document.

Requirements:	P-DET-DS-ECAL.1

References:

I-DET-DS-INF-INT.023	Space Constraint

System 1:	Forward EMCal Systems (6.03.01.04)

System 2:	Barrel HCal Systems (6.03.01.05)

Type:	SPACE

Description:	The exterior radius of the forward ECAL is limited by the interior bore of the barrel Hadron Calorimeter. Modifications to either must be coordinated.

Provider:	Detector Infrastructure and Utilities Integration (6.03.01.09.04)

Details:	See details in the Detector Infrastructure Interface Control Document.

Requirements:	P-DET-DS-ECAL.1, P-DET-DS-HCAL.1

References:

I-DET-DS-INF-INT.024	Space Constraint

System 1:	Forward EMCal Systems (6.03.01.04)

System 2:	Forward HCal Systems (6.03.01.05)

Type:	SPACE

Description:	The forward position of the forward ECAL is limited by the forward Hadron Calorimeter. Modifications to either must be coordinated.

Provider:	Detector Infrastructure and Utilities Integration (6.03.01.09.04)

Details:	See details in the Detector Infrastructure Interface Control Document.

Requirements:	P-DET-DS-ECAL.1, P-DET-DS-HCAL.1

References:

I-DET-DS-INF-INT.025	Space Constraint

System 1:	Forward EMCal Systems (6.03.01.04)

System 2:	Forward RICH Detector Systems (6.03.01.03)

Type:	SPACE

Description:	The maximum forward location for the dRICH is limited by the forward ECAL  and the adjacent cabling pathway that provides services to the interior detectors. Modifications to either must be coordinated.

Provider:	Detector Infrastructure and Utilities Integration (6.03.01.09.04)

Details:	See details in the Detector Infrastructure Interface Control Document.

Requirements:	P-DET-DS-ECAL.1, P-DET-DS-PID.5

References:

I-DET-DS-INF-INT.026	Space Constraint

System 1:	Forward EMCal Systems (6.03.01.04)

System 2:	To Be Determined (TBD)

Type:	SPACE

Description:	The interior radius of the forward ECAL is governed by the size of the beamline. Modifications to either must be coordinated.

Provider:	Detector Infrastructure and Utilities Integration (6.03.01.09.04)

Details:	See details in the Detector Infrastructure Interface Control Document.

Requirements:	P-DET-DS-ECAL.1

References:

I-DET-DS-INF-INT.062	Imaging Layers Heat Rejection

System 1:	Detector Infrastructure and Utilities Integration (6.03.01.09.04)

System 2:	Barrel EMCal Systems (6.03.01.04)

Type:	COOL

Description:	The imaging layers must be cooled with an independent system (TBD).

Provider:	Detector Infrastructure and Utilities Integration (6.03.01.09.04)

Details:	See details in the Detector Infrastructure Interface Control Document.

Requirements:	P-DET-DS-ECAL.3

References:

I-DET-DS-INF-INT.063	Silicon PM Heat Rejection

System 1:	Detector Infrastructure and Utilities Integration (6.03.01.09.04)

System 2:	Barrel EMCal Systems (6.03.01.04)

Type:	COOL

Description:	The barrel ECAL will require chilled water or LCW cooling to maintain a safe operating temperature.

Provider:	Detector Infrastructure and Utilities Integration (6.03.01.09.04)

Details:	See details in the Detector Infrastructure Interface Control Document.

Requirements:	P-DET-DS-ECAL.3

References:

I-DET-DS-INF-INT.064	Heat Rejection

System 1:	Detector Infrastructure and Utilities Integration (6.03.01.09.04)

System 2:	Backward EMCal Systems (6.03.01.04)

Type:	COOL

Description:	The backward ECAL will require chilled water or LCW cooling to maintain the temperature of the silicon photomultipliers, the electronics, and the crystals.

Provider:	Detector Infrastructure and Utilities Integration (6.03.01.09.04)

Details:	See details in the Detector Infrastructure Interface Control Document.

Requirements:	P-DET-DS-ECAL.3

References:

I-DET-DS-INF-INT.065	Heat Rejection

System 1:	Detector Infrastructure and Utilities Integration (6.03.01.09.04)

System 2:	Forward EMCal Systems (6.03.01.04)

Type:	COOL

Description:	The forward ECAL will require chilled water or LCW cooling to maintain a safe operating temperature.

Provider:	Detector Infrastructure and Utilities Integration (6.03.01.09.04)

Details:	See details in the Detector Infrastructure Interface Control Document.

Requirements:	P-DET-DS-ECAL.3

References:

# Detector Infrastructure Systems (DET-DS-INF) to Electronic Systems (DET-DS-ELEC)

I-DET-DS-INF-INT.066	Clean A/C Power

System 1:	Detector Infrastructure and Utilities Integration (6.03.01.09.04)

System 2:	Electronic Systems (6.03.01.07)

Type:	ELEC

Description:	Electronics components on the carriage will require clean or transformer isolated power.

Provider:	Detector Infrastructure and Utilities Integration (6.03.01.09.04)

Details:	See details in the Detector Infrastructure Interface Control Document.

Requirements:	P-DET-DS-ELEC.4

References:

I-DET-DS-INF-INT.067	UPS Power

System 1:	Detector Infrastructure and Utilities Integration (6.03.01.09.04)

System 2:	Electronic Systems (6.03.01.07)

Type:	ELEC

Description:	Critical systems, such as controls, will require sufficient UPS power to allow them to be shutdown graceful, or to tolerate transients.

Provider:	Detector Infrastructure and Utilities Integration (6.03.01.09.04)

Details:	See details in the Detector Infrastructure Interface Control Document.

Requirements:	P-DET-DS-ELEC.4

References:

I-DET-DS-INF-INT.068	Computing Environment

System 1:	Detector Infrastructure and Utilities Integration (6.03.01.09.04)

System 2:	Electronic Systems (6.03.01.07)

Type:	ENV

Description:	The electronics and computing systems will require a stable environment in terms of temperature and humidity.

Provider:	Detector Infrastructure and Utilities Integration (6.03.01.09.04)

Details:	See details in the Detector Infrastructure Interface Control Document.

Requirements:	P-DET-DS-ELEC.3

References:

I-DET-DS-INF-INT.069	EMI Shielding

System 1:	Detector Infrastructure and Utilities Integration (6.03.01.09.04)

System 2:	Electronic Systems (6.03.01.07)

Type:	ENV

Description:	The electronics and computing systems on the carriage may require some EMI shielding from detectors and other components.

Provider:	Detector Infrastructure and Utilities Integration (6.03.01.09.04)

Details:	See details in the Detector Infrastructure Interface Control Document.

Requirements:	P-DET-DS-ELEC.6

References:

I-DET-DS-INF-INT.070	Cabling Pathways

System 1:	Detector Infrastructure and Utilities Integration (6.03.01.09.04)

System 2:	Electronic Systems (6.03.01.07)

Type:	SPACE

Description:	Adequate space for cables and cable trays will be required to deliver low voltage and high voltage, fiber and signal cables from the carriage/platform to the sub-detectors.

Provider:	Detector Infrastructure and Utilities Integration (6.03.01.09.04)

Details:	See details in the Detector Infrastructure Interface Control Document.

Requirements:	P-DET-DS-ELEC.1

References:

I-DET-DS-INF-INT.071	Platform Rack Space

System 1:	Detector Infrastructure and Utilities Integration (6.03.01.09.04)

System 2:	Electronic Systems (6.03.01.07)

Type:	SPACE

Description:	Adequate space must be provided for computing enclosures (racks) on the south platform to support the electronics systems.

Provider:	Detector Infrastructure and Utilities Integration (6.03.01.09.04)

Details:	See details in the Detector Infrastructure Interface Control Document.

Requirements:	P-DET-DS-ELEC.1

References:

# Detector Infrastructure Systems (DET-DS-INF) to Hadronic Calorimetry Systems (DET-DS-HCAL)

I-DET-DS-INF-BAR.001	Barrel ECAL Support

System 1:	Barrel EMCal Systems (6.03.01.04)

System 2:	Barrel HCal Systems (6.03.01.05)

Type:	STRUCT

Description:	The barrel ECAL will be supported by a structural support system that extends through the bore of the solenoid magnet and is supported by the barrel Hadron Calorimeter.

Provider:	Barrel Detector Structures (6.03.01.09.01)

Details:	See details in the Detector Infrastructure Interface Control Document.

Requirements:	P-DET-DS-ECAL.2, P-DET-DS-HCAL.2

References:

I-DET-DS-INF-BAR.002	Weight Distributed

System 1:	Barrel EMCal Systems (6.03.01.04)

System 2:	Forward HCal Systems (6.03.01.05)

Type:	STRUCT

Description:	The weight of the barrel ECAL will be transferred to the barrel Hadron Calorimeter and must be accommodated by all intermediate and subsequent support systems.

Provider:	Barrel Detector Structures (6.03.01.09.01)

Details:	See details in the Detector Infrastructure Interface Control Document.

Requirements:	P-DET-DS-ECAL.2, P-DET-DS-HCAL.2

References:

I-DET-DS-INF-BAR.004	Backward ECAL Support

System 1:	Backward EMCal Systems (6.03.01.04)

System 2:	Forward HCal Systems (6.03.01.05)

Type:	STRUCT

Description:	The backward ECAL will be supported by an integrated support structure that is attached to the DIRC support frame.

Provider:	Barrel Detector Structures (6.03.01.09.01)

Details:	See details in the Detector Infrastructure Interface Control Document.

Requirements:	P-DET-DS-ECAL.2, P-DET-DS-HCAL.2

References:

I-DET-DS-INF-BAR.007	Forward ECAL Support

System 1:	Forward EMCal Systems (6.03.01.04)

System 2:	Forward HCal Systems (6.03.01.05)

Type:	STRUCT

Description:	The forward ECAL will be supported by the forward Hadron Calorimeter endcap. Because the forward ECAL must split into two parts to provide access to the barrel, each half must be independently affixed to the Hadron Calorimeter halves.

Provider:	Barrel Detector Structures (6.03.01.09.01)

Details:	See details in the Detector Infrastructure Interface Control Document.

Requirements:	P-DET-DS-ECAL.1, P-DET-DS-ECAL.2, P-DET-DS-HCAL.2

References:

I-DET-DS-INF-BAR.008	Weight Transfer

System 1:	Forward EMCal Systems (6.03.01.04)

System 2:	Forward HCal Systems (6.03.01.05)

Type:	STRUCT

Description:	The weight of the forward ECAL will be transferred to the forward Hadron Calorimeter endcap and must be accommodated by all subsequent support systems.

Provider:	Barrel Detector Structures (6.03.01.09.01)

Details:	See details in the Detector Infrastructure Interface Control Document.

Requirements:	P-DET-DS-ECAL.1, P-DET-DS-ECAL.2, P-DET-DS-HCAL.2

References:

I-DET-DS-INF-BAR.009	Barrel HCAL Support

System 1:	Barrel HCal Systems (6.03.01.05)

System 2:	Barrel HCal Systems (6.03.01.05)

Type:	STRUCT

Description:	The barrel HCAL will be supported by a rolling carriage that allows the central detector to be moved between the assembly area and the experimental hall.

Provider:	Barrel Detector Structures (6.03.01.09.01)

Details:	See details in the Detector Infrastructure Interface Control Document.

Requirements:	P-DET-DS-HCAL.1, P-DET-DS-HCAL.2

References:

I-DET-DS-INF-BAR.010	Sub-Detector Support

System 1:	Barrel HCal Systems (6.03.01.05)

System 2:	Forward HCal Systems (6.03.01.05)

Type:	STRUCT

Description:	The barrel HCAL will support the weight of all of the sub-detectors and components within the central detector.

Provider:	Barrel Detector Structures (6.03.01.09.01)

Details:	See details in the Detector Infrastructure Interface Control Document.

Requirements:	P-DET-DS-HCAL.2

References:

I-DET-DS-INF-BAR.011	Weight Transfer

System 1:	Barrel HCal Systems (6.03.01.05)

System 2:	Forward HCal Systems (6.03.01.05)

Type:	STRUCT

Description:	The cumulative weight of the HCAL and all of the sub-detectors it supports will be transferred to the detector carriage.

Provider:	Barrel Detector Structures (6.03.01.09.01)

Details:	See details in the Detector Infrastructure Interface Control Document.

Requirements:	P-DET-DS-HCAL.2

References:

I-DET-DS-INF-BAR.012	Dual RICH Support System

System 1:	Barrel HCal Systems (6.03.01.05)

System 2:	Forward Particle ID Systems (6.03.01.03)

Type:	STRUCT

Description:	The Dual RICH will be supported by a structural system within the barrel hadron calorimeter.

Provider:	Barrel Detector Structures (6.03.01.09.01)

Details:	See details in the Detector Infrastructure Interface Control Document.

Requirements:	P-DET-DS-HCAL.2, P-DET-DS-PID.4

References:

I-DET-DS-INF-BCK.001	Backward HCAL Mobility

System 1:	Backward HCal Systems (6.03.01.05)

System 2:	Forward HCal Systems (6.03.01.05)

Type:	MECH

Description:	Adequate clearance must be provided for either of the detector carriages to be rolled aside to allow access to sub-detectors within the barrel.

Provider:	Electron Endcap Structures (6.03.01.09.02)

Details:	See details in the Detector Infrastructure Interface Control Document.

Requirements:	P-DET-DS-HCAL.2

References:

I-DET-DS-INF-BCK.002	Backward HCAL Support

System 1:	Backward HCal Systems (6.03.01.05)

System 2:	Forward HCal Systems (6.03.01.05)

Type:	STRUCT

Description:	The backward HCAL consists of two halves, each of which are support by an independent carriage.

Provider:	Electron Endcap Structures (6.03.01.09.02)

Details:	See details in the Detector Infrastructure Interface Control Document.

Requirements:	P-DET-DS-HCAL.2

References:

I-DET-DS-INF-BCK.003	Weight Transfer

System 1:	Backward HCal Systems (6.03.01.05)

System 2:	Forward HCal Systems (6.03.01.05)

Type:	STRUCT

Description:	The weight of each half of the backward HCAL will be transferred to the carriage that is supporting it.

Provider:	Electron Endcap Structures (6.03.01.09.02)

Details:	See details in the Detector Infrastructure Interface Control Document.

Requirements:	P-DET-DS-HCAL.2

References:

I-DET-DS-INF-FWD.003	Forward HCAL Support

System 1:	Forward HCal Systems (6.03.01.05)

System 2:	Forward HCal Systems (6.03.01.05)

Type:	STRUCT

Description:	The forward HCAL consists of two halves, each of which are support by an independent carriage.

Provider:	Hadron Endcap Structures (6.03.01.09.03)

Details:	See details in the Detector Infrastructure Interface Control Document.

Requirements:	P-DET-DS-HCAL.2

References:

I-DET-DS-INF-FWD.004	Sub-Detector Support

System 1:	Forward HCal Systems (6.03.01.05)

System 2:	Forward EMCal Systems (6.03.01.04)

Type:	STRUCT

Description:	The forward HCAL will support the weight of the forward electromagnetic calorimeter that is embedded within it.

Provider:	Hadron Endcap Structures (6.03.01.09.03)

Details:	See details in the Detector Infrastructure Interface Control Document.

Requirements:	P-DET-DS-HCAL.2

References:

I-DET-DS-INF-FWD.005	Weight Transfer

System 1:	Forward HCal Systems (6.03.01.05)

System 2:	Forward HCal Systems (6.03.01.05)

Type:	STRUCT

Description:	The weight of each half of the forward HCAL, and its embedded subdetectors, will be transferred to the associated carriage.

Provider:	Hadron Endcap Structures (6.03.01.09.03)

Details:	See details in the Detector Infrastructure Interface Control Document.

Requirements:	P-DET-DS-HCAL.2

References:

I-DET-DS-INF-FWD.009	Forward HCAL Mobility

System 1:	Hadron Endcap Structures (6.03.01.09.03)

System 2:	Forward HCal Systems (6.03.01.05)

Type:	MECH

Description:	Adequate clearance must be provided for either of the detector carriages to be rolled aside to allow access to sub-detectors within the barrel.

Provider:	Hadron Endcap Structures (6.03.01.09.03)

Details:	See details in the Detector Infrastructure Interface Control Document.

Requirements:	P-DET-DS-HCAL.1, P-DET-DS-HCAL.2

References:

I-DET-DS-INF-FWD.010	Forward HCAL Mobility

System 1:	Hadron Endcap Structures (6.03.01.09.03)

System 2:	Forward HCal Systems (6.03.01.05)

Type:	DELETED

Description:	Adequate clearance must be provided for either of the detector carriages to be rolled aside to allow access to sub-detectors within the barrel.

Provider:	Hadron Endcap Structures (6.03.01.09.03)

Details:	See details in the Detector Infrastructure Interface Control Document.

Requirements:	P-DET-DS-HCAL.1, P-DET-DS-HCAL.2

References:

I-DET-DS-INF-INT.005	Space Constraint

System 1:	B-Zero Detectors (6.03.01.10)

System 2:	Forward HCal Systems (6.03.01.05)

Type:	SPACE

Description:	The position of the forward HCAL is bounded by the B0 magnet in the forward direction. Changes to the size or position of either must be coordinated with the other.

Provider:	Detector Infrastructure and Utilities Integration (6.03.01.09.04)

Details:	See details in the Detector Infrastructure Interface Control Document.

Requirements:	P-DET-DS-ANC.1, P-DET-DS-HCAL.1

References:

I-DET-DS-INF-INT.016	Space Constraint

System 1:	Backward EMCal Systems (6.03.01.04)

System 2:	Backward HCal Systems (6.03.01.05)

Type:	SPACE

Description:	The backward position of the backward ECAL is limited by the backward Hadron Calorimeter and the adjacent cabling pathway that provides services to the interior detectors. Modifications to either must be coordinated.

Provider:	Detector Infrastructure and Utilities Integration (6.03.01.09.04)

Details:	See details in the Detector Infrastructure Interface Control Document.

Requirements:	P-DET-DS-ECAL.1, P-DET-DS-HCAL.1

References:

I-DET-DS-INF-INT.023	Space Constraint

System 1:	Forward EMCal Systems (6.03.01.04)

System 2:	Barrel HCal Systems (6.03.01.05)

Type:	SPACE

Description:	The exterior radius of the forward ECAL is limited by the interior bore of the barrel Hadron Calorimeter. Modifications to either must be coordinated.

Provider:	Detector Infrastructure and Utilities Integration (6.03.01.09.04)

Details:	See details in the Detector Infrastructure Interface Control Document.

Requirements:	P-DET-DS-ECAL.1, P-DET-DS-HCAL.1

References:

I-DET-DS-INF-INT.024	Space Constraint

System 1:	Forward EMCal Systems (6.03.01.04)

System 2:	Forward HCal Systems (6.03.01.05)

Type:	SPACE

Description:	The forward position of the forward ECAL is limited by the forward Hadron Calorimeter. Modifications to either must be coordinated.

Provider:	Detector Infrastructure and Utilities Integration (6.03.01.09.04)

Details:	See details in the Detector Infrastructure Interface Control Document.

Requirements:	P-DET-DS-ECAL.1, P-DET-DS-HCAL.1

References:

I-DET-DS-INF-INT.027	Space Constraint

System 1:	Barrel HCal Systems (6.03.01.05)

System 2:	Backward HCal Systems (6.03.01.05)

Type:	SPACE

Description:	The forward position of the backward HCAL is limited by the barrel HCAL and the adjacent cabling pathway that provides services to the interior detectors. Modifications to either must be coordinated.

Provider:	Detector Infrastructure and Utilities Integration (6.03.01.09.04)

Details:	See details in the Detector Infrastructure Interface Control Document.

Requirements:	P-DET-DS-HCAL.1

References:

I-DET-DS-INF-INT.028	Space Constraint

System 1:	Barrel HCal Systems (6.03.01.05)

System 2:	Backward HCal Systems (6.03.01.05)

Type:	SPACE

Description:	The exterior radius of the backward HCAL should be consistent with the radius of the barrel HCAL.

Provider:	Detector Infrastructure and Utilities Integration (6.03.01.09.04)

Details:	See details in the Detector Infrastructure Interface Control Document.

Requirements:	P-DET-DS-HCAL.1

References:

I-DET-DS-INF-INT.029	Space Constraint

System 1:	Barrel HCal Systems (6.03.01.05)

System 2:	Forward HCal Systems (6.03.01.05)

Type:	SPACE

Description:	The exterior radius of the forward HCAL should be consistent with the radius of the barrel HCAL.

Provider:	Detector Infrastructure and Utilities Integration (6.03.01.09.04)

Details:	See details in the Detector Infrastructure Interface Control Document.

Requirements:	P-DET-DS-HCAL.1

References:

I-DET-DS-INF-INT.030	Space Constraint

System 1:	Barrel HCal Systems (6.03.01.05)

System 2:	Forward HCal Systems (6.03.01.05)

Type:	SPACE

Description:	The backward position of the forward HCAL is limited by the barrel HCAL and the adjacent cabling pathway that provides services to the interior detectors. Modifications to either must be coordinated.

Provider:	Detector Infrastructure and Utilities Integration (6.03.01.09.04)

Details:	See details in the Detector Infrastructure Interface Control Document.

Requirements:	P-DET-DS-HCAL.1

References:

I-DET-DS-INF-INT.031	External Space Constraint

System 1:	Barrel HCal Systems (6.03.01.05)

System 2:	Solenoid Magnet (6.03.01.06)

Type:	SPACE

Description:	The exterior radius of the solenoid magnet is limited by the interior bore of the barrel HCAL and necessary support structures. Modifications to either must be coordinated.

Provider:	Detector Infrastructure and Utilities Integration (6.03.01.09.04)

Details:	See details in the Detector Infrastructure Interface Control Document.

Requirements:	P-DET-DS-HCAL.1, P-DET-DS-MAG.1

References:

I-DET-DS-INF-INT.032	Space Constraint

System 1:	Barrel HCal Systems (6.03.01.05)

System 2:	Forward RICH Detector Systems (6.03.01.03)

Type:	SPACE

Description:	The size of the dRICH is limited by the interior bore of the barrel HCAL, and must provide adequate space for cables and services to itself and the interior detectors. Modifications to either must be coordinated.

Provider:	Detector Infrastructure and Utilities Integration (6.03.01.09.04)

Details:	See details in the Detector Infrastructure Interface Control Document.

Requirements:	P-DET-DS-HCAL.1, P-DET-DS-PID.5

References:

I-DET-DS-INF-INT.033	Space Constraint

System 1:	Backward HCal Systems (6.03.01.05)

System 2:	Barrel HCal Systems (6.03.01.05)

Type:	SPACE

Description:	The forward position of the backward HCAL is limited by the barrel HCAL and the adjacent cabling pathway that provides services to the interior detectors. Modifications to either must be coordinated.

Provider:	Detector Infrastructure and Utilities Integration (6.03.01.09.04)

Details:	See details in the Detector Infrastructure Interface Control Document.

Requirements:	P-DET-DS-HCAL.1

References:

I-DET-DS-INF-INT.034	Space Constraint

System 1:	Backward HCal Systems (6.03.01.05)

System 2:	Solenoid Magnet (6.03.01.06)

Type:	SPACE

Description:	The maximum backward location for the solenoid magnet is limited by the backward HCAL and the adjacent cabling pathway that provides services to the interior detectors. Modifications to either must be coordinated.

Provider:	Detector Infrastructure and Utilities Integration (6.03.01.09.04)

Details:	See details in the Detector Infrastructure Interface Control Document.

Requirements:	P-DET-DS-HCAL.1, P-DET-DS-MAG.1

References:

I-DET-DS-INF-INT.035	Space Constraint

System 1:	Backward HCal Systems (6.03.01.05)

System 2:	Barrel DIRC Particle ID Systems (6.03.01.03)

Type:	SPACE

Description:	The maximum backward location for the DIRC is limited by the backward HCAL and the adjacent cabling pathway that provides services to the interior detectors. Modifications to either must be coordinated.

Provider:	Detector Infrastructure and Utilities Integration (6.03.01.09.04)

Details:	See details in the Detector Infrastructure Interface Control Document.

Requirements:	P-DET-DS-HCAL.1, P-DET-DS-PID.5

References:

I-DET-DS-INF-INT.036	Space Constraint

System 1:	Backward HCal Systems (6.03.01.05)

System 2:	Interaction Region (6.04)

Type:	SPACE

Description:	The position of the backward HCAL is limited in the backward direction by the adjacent accelerator magnets.

Provider:	Detector Infrastructure and Utilities Integration (6.03.01.09.04)

Details:	See details in the Detector Infrastructure Interface Control Document.

Requirements:	P-DET-DS-HCAL.1

References:

I-DET-DS-INF-INT.037	Space Constraint

System 1:	Backward HCal Systems (6.03.01.05)

System 2:	IR Accelerator Systems (6.04.03)

Type:	SPACE

Description:	The interior radius of the backward HCAL is governed by the size of the beamline.

Provider:	Detector Infrastructure and Utilities Integration (6.03.01.09.04)

Details:	See details in the Detector Infrastructure Interface Control Document.

Requirements:	P-DET-DS-HCAL.1

References:

I-DET-DS-INF-INT.038	Space Constraint

System 1:	Forward HCal Systems (6.03.01.05)

System 2:	IR Accelerator Systems (6.04.03)

Type:	SPACE

Description:	The interior radius of the forward HCAL is governed by the size of the beamline.

Provider:	Detector Infrastructure and Utilities Integration (6.03.01.09.04)

Details:	See details in the Detector Infrastructure Interface Control Document.

Requirements:	P-DET-DS-HCAL.1

References:

I-DET-DS-INF-INT.072	Heat Rejection

System 1:	Detector Infrastructure and Utilities Integration (6.03.01.09.04)

System 2:	Barrel HCal Systems (6.03.01.05)

Type:	COOL

Description:	Heat from the silicon photomultipliers and front end electronics will be rejected into the outside room via forced air.

Provider:	Detector Infrastructure and Utilities Integration (6.03.01.09.04)

Details:	See details in the Detector Infrastructure Interface Control Document.

Requirements:	P-DET-DS-HCAL.3

References:

I-DET-DS-INF-INT.073	Space Constraint

System 1:	Detector Infrastructure and Utilities Integration (6.03.01.09.04)

System 2:	Barrel HCal Systems (6.03.01.05)

Type:	SPACE

Description:	The exterior radius of the barrel HCAL is limited by the detector carriage, the electronics platforms, and other Hall infrastructure.

Provider:	Detector Infrastructure and Utilities Integration (6.03.01.09.04)

Details:	See details in the Detector Infrastructure Interface Control Document.

Requirements:	P-DET-DS-HCAL.1

References:

I-DET-DS-INF-INT.074	Heat Rejection

System 1:	Detector Infrastructure and Utilities Integration (6.03.01.09.04)

System 2:	Backward HCal Systems (6.03.01.05)

Type:	COOL

Description:	Heat from the silicon photomultipliers and front end electronics will be rejected into the outside room via forced air.

Provider:	Detector Infrastructure and Utilities Integration (6.03.01.09.04)

Details:	See details in the Detector Infrastructure Interface Control Document.

Requirements:	P-DET-DS-HCAL.3

References:

I-DET-DS-INF-INT.075	Heat Rejection

System 1:	Detector Infrastructure and Utilities Integration (6.03.01.09.04)

System 2:	Forward HCal Systems (6.03.01.05)

Type:	COOL

Description:	Heat from the silicon photomultipliers and front end electronics will be rejected into the outside room via forced air. DO WE NEED CW FOR COOLING HERE?

Provider:	Detector Infrastructure and Utilities Integration (6.03.01.09.04)

Details:	See details in the Detector Infrastructure Interface Control Document.

Requirements:	P-DET-DS-HCAL.3

References:

# Detector Infrastructure Systems (DET-DS-INF) to Solenoid Magnet (DET-DS-MAG)

I-DET-DS-INF-BAR.015	External Support Structure

System 1:	Barrel Detector Structures (6.03.01.09.01)

System 2:	Solenoid Magnet (6.03.01.06)

Type:	STRUCT

Description:	Total weight of the solenoid and all embedded components must be supported.

Provider:	Barrel Detector Structures (6.03.01.09.01)

Details:	See details in the Detector Infrastructure Interface Control Document.

Requirements:	P-DET-DS-MAG.2

References:

I-DET-DS-INF-BAR.017	DIRC Readout Supports

System 1:	Solenoid Magnet (6.03.01.06)

System 2:	Barrel DIRC Particle ID Systems (6.03.01.03)

Type:	STRUCT

Description:	The DIRC prism box containing the photosensor readouts, will be supported by an external structure supporting the ECAL within the solenoid magnet.

Provider:	Barrel Detector Structures (6.03.01.09.01)

Details:	See details in the Detector Infrastructure Interface Control Document.

Requirements:	P-DET-DS-MAG.1, P-DET-DS-MAG.2, P-DET-DS-PID.4

References:

I-DET-DS-INF-INT.001	Magnetic Material

System 1:	Detector System (6.03.01)

System 2:	Solenoid Magnet (6.03.01.06)

Type:	ENV

Description:	No detectors or components constructed with magnetic material may be added to the detector without prior consultation with the magnet group.

Provider:	Detector Infrastructure and Utilities Integration (6.03.01.09.04)

Details:	See details in the Detector Infrastructure Interface Control Document.

Requirements:	P-DET-DS-MAG.13, P-DET-DS-MAG.14

References:

I-DET-DS-INF-INT.002	Magnetic Field Requirement

System 1:	Detector System (6.03.01)

System 2:	Solenoid Magnet (6.03.01.06)

Type:	PARAM

Description:	All subordinate detectors are dependent on the continuous operation of the solenoid and on the delivery of a consistent, stable magnetic field.

Provider:	Detector Infrastructure and Utilities Integration (6.03.01.09.04)

Details:	See details in the Detector Infrastructure Interface Control Document.

Requirements:	P-DET-DS-MAG.13

References:

I-DET-DS-INF-INT.012	Space Constraint

System 1:	Barrel EMCal Systems (6.03.01.04)

System 2:	Solenoid Magnet (6.03.01.06)

Type:	SPACE

Description:	The exterior radius of the barrel ECAL (and its support system) is limited by the interior bore of the solenoid magnet. Modifications to either must be coordinated.

Provider:	Detector Infrastructure and Utilities Integration (6.03.01.09.04)

Details:	See details in the Detector Infrastructure Interface Control Document.

Requirements:	P-DET-DS-ECAL.1, P-DET-DS-MAG.1

References:

I-DET-DS-INF-INT.031	External Space Constraint

System 1:	Barrel HCal Systems (6.03.01.05)

System 2:	Solenoid Magnet (6.03.01.06)

Type:	SPACE

Description:	The exterior radius of the solenoid magnet is limited by the interior bore of the barrel HCAL and necessary support structures. Modifications to either must be coordinated.

Provider:	Detector Infrastructure and Utilities Integration (6.03.01.09.04)

Details:	See details in the Detector Infrastructure Interface Control Document.

Requirements:	P-DET-DS-HCAL.1, P-DET-DS-MAG.1

References:

I-DET-DS-INF-INT.034	Space Constraint

System 1:	Backward HCal Systems (6.03.01.05)

System 2:	Solenoid Magnet (6.03.01.06)

Type:	SPACE

Description:	The maximum backward location for the solenoid magnet is limited by the backward HCAL and the adjacent cabling pathway that provides services to the interior detectors. Modifications to either must be coordinated.

Provider:	Detector Infrastructure and Utilities Integration (6.03.01.09.04)

Details:	See details in the Detector Infrastructure Interface Control Document.

Requirements:	P-DET-DS-HCAL.1, P-DET-DS-MAG.1

References:

I-DET-DS-INF-INT.039	Magnet Cooling

System 1:	Detector Infrastructure and Utilities Integration (6.03.01.09.04)

System 2:	Solenoid Magnet (6.03.01.06)

Type:	CRYO

Description:	The solenoid will require a continuous flow of cryogens to maintain temperature in both the experimental hall and the maintenance area.  Interface includes cryogenic source and warm return.

Provider:	Detector Infrastructure and Utilities Integration (6.03.01.09.04)

Details:	See details in the Detector Infrastructure Interface Control Document.

Requirements:	P-DET-DS-MAG.4

References:

I-DET-DS-INF-INT.076	Power Supply Cooling

System 1:	Detector Infrastructure and Utilities Integration (6.03.01.09.04)

System 2:	Solenoid Magnet (6.03.01.06)

Type:	COOL

Description:	The power supply, located in an adjacent electrical room, will need to be cooled with low conductivity water.

Provider:	Detector Infrastructure and Utilities Integration (6.03.01.09.04)

Details:	See details in the Detector Infrastructure Interface Control Document.

Requirements:	P-DET-DS-MAG.3

References:

I-DET-DS-INF-INT.077	Magnet Cooling

System 1:	Detector Infrastructure and Utilities Integration (6.03.01.09.04)

System 2:	Solenoid Magnet (6.03.01.06)

Type:	CRYO

Description:	The solenoid will require a continuous flow of cryogens to maintain temperature in both the experimental hall and the maintenance area.  Interface includes cryogenic source and warm return.

Provider:	Detector Infrastructure and Utilities Integration (6.03.01.09.04)

Details:	See details in the Detector Infrastructure Interface Control Document.

Requirements:	P-DET-DS-MAG.4, P-DET-DS-MAG-CCR.1, P-DET-DS-MAG-CCR.2, P-DET-DS-MAG-CCR.3

References:

I-DET-DS-INF-INT.078	Electrical Power

System 1:	Detector Infrastructure and Utilities Integration (6.03.01.09.04)

System 2:	Solenoid Magnet (6.03.01.06)

Type:	ELEC

Description:	Solenoid will be provided power via the power supply located in an adjacent room.

Provider:	Detector Infrastructure and Utilities Integration (6.03.01.09.04)

Details:	See details in the Detector Infrastructure Interface Control Document.

Requirements:	P-DET-DS-MAG.6

References:

I-DET-DS-INF-INT.079	Instrumentation Backup Power

System 1:	Detector Infrastructure and Utilities Integration (6.03.01.09.04)

System 2:	Solenoid Magnet (6.03.01.06)

Type:	ELEC

Description:	The instrumentation rack for the solenoid will require backup/UPS power adequate to safely shutdown the system in the event of a power failure.

Provider:	Detector Infrastructure and Utilities Integration (6.03.01.09.04)

Details:	See details in the Detector Infrastructure Interface Control Document.

Requirements:	P-DET-DS-MAG.6

References:

I-DET-DS-INF-INT.080	Fringe Field

System 1:	Detector Infrastructure and Utilities Integration (6.03.01.09.04)

System 2:	Solenoid Magnet (6.03.01.06)

Type:	ENV

Description:	A boundary must be established to identify the extent of the magnet's stray field during operations.

Provider:	Detector Infrastructure and Utilities Integration (6.03.01.09.04)

Details:	See details in the Detector Infrastructure Interface Control Document.

Requirements:	P-DET-DS-MAG.14

References:

I-DET-DS-INF-INT.081	Ice Management

System 1:	Detector Infrastructure and Utilities Integration (6.03.01.09.04)

System 2:	Solenoid Magnet (6.03.01.06)

Type:	ENV

Description:	An ice management system will be required to melt accumulated ice on the chimney. (Water should evaporate and will not require drainage)

Provider:	Detector Infrastructure and Utilities Integration (6.03.01.09.04)

Details:	See details in the Detector Infrastructure Interface Control Document.

Requirements:	P-DET-DS-MAG.5

References:

I-DET-DS-INF-INT.082	Cryogenic Connection Point

System 1:	Detector Infrastructure and Utilities Integration (6.03.01.09.04)

System 2:	Solenoid Magnet (6.03.01.06)

Type:	SPACE

Description:	The cryogenic connection must be as close as possible to the source.

Provider:	Detector Infrastructure and Utilities Integration (6.03.01.09.04)

Details:	See details in the Detector Infrastructure Interface Control Document.

Requirements:	P-DET-DS-MAG.4

References:

I-DET-DS-INF-INT.083	LCW Connection

System 1:	Detector Infrastructure and Utilities Integration (6.03.01.09.04)

System 2:	Solenoid Magnet (6.03.01.06)

Type:	SPACE

Description:	The LCW water cooled leads must be fully supported along their path between the electrical room and the magnet.

Provider:	Detector Infrastructure and Utilities Integration (6.03.01.09.04)

Details:	See details in the Detector Infrastructure Interface Control Document.

Requirements:	P-DET-DS-MAG.2

References:

I-DET-DS-INF-INT.097	Space Constraint

System 1:	Solenoid Magnet (6.03.01.06)

System 2:	Forward RICH Detector Systems (6.03.01.03)

Type:	SPACE

Description:	The maximum forward location for the solenoid is limited by the position of the Dual RICH detector and the cabling plenum (gap) between them. Modifications to either must be coordinated.

Provider:	Detector Infrastructure and Utilities Integration (6.03.01.09.04)

Details:	See details in the Detector Infrastructure Interface Control Document.

Requirements:	P-DET-DS-MAG.1, P-DET-DS-PID.5

References:

I-DET-DS-INF-INT.114	LHE Flow (Hall)

System 1:	Solenoid Magnet (6.03.01.06)

System 2:	IR Systems Integration (6.04.04)

Type:	CRYO

Description:	A supply and return of liquid helium must be provided to the experimental hall for cooling the solenoid magnet during operation.

Provider:	Detector Infrastructure and Utilities Integration (6.03.01.09.04)

Details:	See details in the Detector Infrastructure Interface Control Document.

Requirements:	P-DET-DS-MAG-CCR.1, P-DET-DS-MAG-CCR.2, P-DET-DS-MAG-CCR.3, P-DET-DS-MAG-CCR.4, P-DET-DS-INF-CRYO.1, P-DET-DS-INF-CRYO.2, P-DET-DS-INF-CRYO.3, P-DET-DS-INF-CRYO.4

References:

I-DET-DS-INF-INT.115	LHE Flow (Assembly Area)

System 1:	Solenoid Magnet (6.03.01.06)

System 2:	IR Systems Integration (6.04.04)

Type:	CRYO

Description:	A supply and return of liquid helium must be provided to the assembly hall for cooling the solenoid magnet during maintenance.

Provider:	Detector Infrastructure and Utilities Integration (6.03.01.09.04)

Details:	See details in the Detector Infrastructure Interface Control Document.

Requirements:	P-DET-DS-MAG-CCR.1, P-DET-DS-MAG-CCR.2, P-DET-DS-MAG-CCR.3, P-DET-DS-MAG-CCR.4, P-DET-DS-INF-CRYO.1, P-DET-DS-INF-CRYO.2, P-DET-DS-INF-CRYO.3, P-DET-DS-INF-CRYO.4

References:

# Detector Infrastructure Systems (DET-DS-INF) to Particle Identification Systems (DET-DS-PID)

I-DET-DS-INF-BAR.003	Support Structure

System 1:	Barrel EMCal Systems (6.03.01.04)

System 2:	Barrel DIRC Particle ID Systems (6.03.01.03)

Type:	STRUCT

Description:	The DIRC bar boxes will be supported by a frame inside the barrel Electromagnetic Calorimeter, that allows the boxes to be extracted using a system of rollers.

Provider:	Barrel Detector Structures (6.03.01.09.01)

Details:	See details in the Detector Infrastructure Interface Control Document.

Requirements:	P-DET-DS-ECAL.1, P-DET-DS-ECAL.2, P-DET-DS-PID.4, P-DET-DS-PID.5

References:

I-DET-DS-INF-BAR.005	Weight Transfer

System 1:	Backward EMCal Systems (6.03.01.04)

System 2:	Barrel DIRC Particle ID Systems (6.03.01.03)

Type:	STRUCT

Description:	The weight of the backward ECAL will be transferred to the DIRC detector support and must be accommodated by all subsequent support systems.

Provider:	Barrel Detector Structures (6.03.01.09.01)

Details:	See details in the Detector Infrastructure Interface Control Document.

Requirements:	P-DET-DS-ECAL.2, P-DET-DS-HCAL.2

References:

I-DET-DS-INF-BAR.006	EMCal Support System

System 1:	Backward EMCal Systems (6.03.01.04)

System 2:	Barrel DIRC Particle ID Systems (6.03.01.03)

Type:	STRUCT

Description:	The DIRC support system will provide support for the backward electromagnetic calorimeter.

Provider:	Barrel Detector Structures (6.03.01.09.01)

Details:	See details in the Detector Infrastructure Interface Control Document.

Requirements:	P-DET-DS-ECAL.2, P-DET-DS-PID.4

References:

I-DET-DS-INF-BAR.012	Dual RICH Support System

System 1:	Barrel HCal Systems (6.03.01.05)

System 2:	Forward Particle ID Systems (6.03.01.03)

Type:	STRUCT

Description:	The Dual RICH will be supported by a structural system within the barrel hadron calorimeter.

Provider:	Barrel Detector Structures (6.03.01.09.01)

Details:	See details in the Detector Infrastructure Interface Control Document.

Requirements:	P-DET-DS-HCAL.2, P-DET-DS-PID.4

References:

I-DET-DS-INF-BAR.017	DIRC Readout Supports

System 1:	Solenoid Magnet (6.03.01.06)

System 2:	Barrel DIRC Particle ID Systems (6.03.01.03)

Type:	STRUCT

Description:	The DIRC prism box containing the photosensor readouts, will be supported by an external structure supporting the ECAL within the solenoid magnet.

Provider:	Barrel Detector Structures (6.03.01.09.01)

Details:	See details in the Detector Infrastructure Interface Control Document.

Requirements:	P-DET-DS-MAG.1, P-DET-DS-MAG.2, P-DET-DS-PID.4

References:

I-DET-DS-INF-BAR.018	Structural Support

System 1:	Barrel Particle ID Systems (6.03.01.03)

System 2:	Backward Particle ID Systems (6.03.01.03)

Type:	STRUCT

Description:	The pfRICH/mRICH will be supported by the DIRC bar box support system.

Provider:	Barrel Detector Structures (6.03.01.09.01)

Details:	See details in the Detector Infrastructure Interface Control Document.

Requirements:	P-DET-DS-PID.4, P-DET-DS-PID.5

References:

I-DET-DS-INF-BAR.019	Barrel TOF Support

System 1:	Barrel DIRC Particle ID Systems (6.03.01.03)

System 2:	Barrel TOF Particle ID Systems (6.03.01.03)

Type:	STRUCT

Description:	The barrel Time of Flight will be supported by the DIRC bar box assembly.

Provider:	Barrel Detector Structures (6.03.01.09.01)

Details:	See details in the Detector Infrastructure Interface Control Document.

Requirements:	P-DET-DS-PID.4, P-DET-DS-PID.5

References:

I-DET-DS-INF-BAR.020	pfRICH/mRICH Support System

System 1:	Barrel DIRC Particle ID Systems (6.03.01.03)

System 2:	Backward RICH Detector Systems (6.03.01.03)

Type:	STRUCT

Description:	The DIRC support system will provide support for the upstream PID detector system.

Provider:	Barrel Detector Structures (6.03.01.09.01)

Details:	See details in the Detector Infrastructure Interface Control Document.

Requirements:	P-DET-DS-PID.4, P-DET-DS-PID.5

References:

I-DET-DS-INF-BAR.021	Silicon Tracker Support System

System 1:	Barrel DIRC Particle ID Systems (6.03.01.03)

System 2:	Tracking Systems (6.03.01.02)

Type:	STRUCT

Description:	The DIRC support system will provide support for the silicon trackers in the detector barrel.

Provider:	Barrel Detector Structures (6.03.01.09.01)

Details:	See details in the Detector Infrastructure Interface Control Document.

Requirements:	P-DET-DS-PID.4, P-DET-DS-PID.5, P-DET-DS-TRAK.8, P-DET-DS-TRAK.9

References:

I-DET-DS-INF-BAR.022	Weight Distributed

System 1:	Barrel DIRC Particle ID Systems (6.03.01.03)

System 2:	Barrel Tracking Systems (6.03.01.02)

Type:	STRUCT

Description:	The weight of the barrel tracking systems will be transferred to the DIRC support system.

Provider:	Barrel Detector Structures (6.03.01.09.01)

Details:	See details in the Detector Infrastructure Interface Control Document.

Requirements:	P-DET-DS-PID.4, P-DET-DS-TRAK.8

References:

I-DET-DS-INF-FWD.011	Space Constraint

System 1:	Forward RICH Detector Systems (6.03.01.03)

System 2:	IR Accelerator Systems (6.04.03)

Type:	SPACE

Description:	The interior radius of the dRICH is governed by the size of the beamline and its associated flanges and support system.

Provider:	Hadron Endcap Structures (6.03.01.09.03)

Details:	See details in the Detector Infrastructure Interface Control Document.

Requirements:	P-DET-DS-PID.5

References:

I-DET-DS-INF-FWD.012	Space Constraint

System 1:	Forward Time of Flight Systems (6.03.01.03)

System 2:	Barrel Tracking Systems (6.03.01.02)

Type:	SPACE

Description:	The maximum forward location for the tracking system is limited by the position of the AC LGAD Time of Flight Detector. Modifications to either must be coordinated.

Provider:	Hadron Endcap Structures (6.03.01.09.03)

Details:	See details in the Detector Infrastructure Interface Control Document.

Requirements:	P-DET-DS-PID.5, P-DET-DS-TRAK.9

References:

I-DET-DS-INF-INT.003	Cabling/Services Conduit

System 1:	Detector System (6.03.01)

System 2:	Barrel DIRC Particle ID Systems (6.03.01.03)

Type:	SPACE

Description:	The DIRC structure will provide integrated pathways for cabling and services to be delivered from the interior detectors (pf RICH/mRICH and silicon trackers) to the exterior infrastructure.

Provider:	Detector Infrastructure and Utilities Integration (6.03.01.09.04)

Details:	See details in the Detector Infrastructure Interface Control Document.

Requirements:	P-DET-DS-PID.4, P-DET-DS-PID.5

References:

I-DET-DS-INF-INT.013	Space Constraint

System 1:	Barrel EMCal Systems (6.03.01.04)

System 2:	Barrel DIRC Particle ID Systems (6.03.01.03)

Type:	SPACE

Description:	The backward position and shape of the barrel EMCAL is limited by the size and shape of the DIRC readout supports.  Changes to the size or position of either must be coordinated with the other.

Provider:	Detector Infrastructure and Utilities Integration (6.03.01.09.04)

Details:	See details in the Detector Infrastructure Interface Control Document.

Requirements:	P-DET-DS-ECAL.1, P-DET-DS-PID.5

References:

I-DET-DS-INF-INT.014	Space Constraint

System 1:	Barrel EMCal Systems (6.03.01.04)

System 2:	Barrel DIRC Particle ID Systems (6.03.01.03)

Type:	SPACE

Description:	The maximum size of the DIRC is limited to the interior bore of the barrel ECAL (and its support structures).  Modifications to either must be coordinated.

Provider:	Detector Infrastructure and Utilities Integration (6.03.01.09.04)

Details:	See details in the Detector Infrastructure Interface Control Document.

Requirements:	P-DET-DS-ECAL.1, P-DET-DS-PID.5

References:

I-DET-DS-INF-INT.015	Space Constraint

System 1:	Barrel EMCal Systems (6.03.01.04)

System 2:	Forward RICH Detector Systems (6.03.01.03)

Type:	SPACE

Description:	The forward position and shape of the barrel ECAL is limited  by the backward face of the Dual RICH detector and the adjacent cabling pathway that provides services to the interior detectors. Modifications to either must be coordinated.

Provider:	Detector Infrastructure and Utilities Integration (6.03.01.09.04)

Details:	See details in the Detector Infrastructure Interface Control Document.

Requirements:	P-DET-DS-ECAL.1, P-DET-DS-PID.5

References:

I-DET-DS-INF-INT.017	Space Constraint

System 1:	Backward EMCal Systems (6.03.01.04)

System 2:	Barrel DIRC Particle ID Systems (6.03.01.03)

Type:	SPACE

Description:	The maximum backward location for the DIRC  is limited by the position of the backward electromagnetic calorimeter. Modifications to either must be coordinated.

Provider:	Detector Infrastructure and Utilities Integration (6.03.01.09.04)

Details:	See details in the Detector Infrastructure Interface Control Document.

Requirements:	P-DET-DS-ECAL.1, P-DET-DS-PID.5

References:

I-DET-DS-INF-INT.018	Space Constraint

System 1:	Backward EMCal Systems (6.03.01.04)

System 2:	Barrel DIRC Particle ID Systems (6.03.01.03)

Type:	SPACE

Description:	The exterior radius of the backward ECAL is limited by the interior bore of the DIRC support system. Modifications to either must be coordinated.

Provider:	Detector Infrastructure and Utilities Integration (6.03.01.09.04)

Details:	See details in the Detector Infrastructure Interface Control Document.

Requirements:	P-DET-DS-ECAL.1, P-DET-DS-PID.5

References:

I-DET-DS-INF-INT.019	Heat Mitigation

System 1:	Backward EMCal Systems (6.03.01.04)

System 2:	Backward Particle ID Systems (6.03.01.03)

Type:	ENV

Description:	The backward ECAL must mitigated the heat generated by the backward RICH detector.

Provider:	Detector Infrastructure and Utilities Integration (6.03.01.09.04)

Details:	See details in the Detector Infrastructure Interface Control Document.

Requirements:	P-DET-DS-ECAL.3

References:

I-DET-DS-INF-INT.020	Space Constraint

System 1:	Backward EMCal Systems (6.03.01.04)

System 2:	Backward RICH Detector Systems (6.03.01.03)

Type:	SPACE

Description:	The position of the backward ECAL in the forward direction is limited by the backward face of the pfRICH/mRICH detector.  Modifications to either must be coordinated.

Provider:	Detector Infrastructure and Utilities Integration (6.03.01.09.04)

Details:	See details in the Detector Infrastructure Interface Control Document.

Requirements:	P-DET-DS-ECAL.1, P-DET-DS-PID.5

References:

I-DET-DS-INF-INT.025	Space Constraint

System 1:	Forward EMCal Systems (6.03.01.04)

System 2:	Forward RICH Detector Systems (6.03.01.03)

Type:	SPACE

Description:	The maximum forward location for the dRICH is limited by the forward ECAL  and the adjacent cabling pathway that provides services to the interior detectors. Modifications to either must be coordinated.

Provider:	Detector Infrastructure and Utilities Integration (6.03.01.09.04)

Details:	See details in the Detector Infrastructure Interface Control Document.

Requirements:	P-DET-DS-ECAL.1, P-DET-DS-PID.5

References:

I-DET-DS-INF-INT.032	Space Constraint

System 1:	Barrel HCal Systems (6.03.01.05)

System 2:	Forward RICH Detector Systems (6.03.01.03)

Type:	SPACE

Description:	The size of the dRICH is limited by the interior bore of the barrel HCAL, and must provide adequate space for cables and services to itself and the interior detectors. Modifications to either must be coordinated.

Provider:	Detector Infrastructure and Utilities Integration (6.03.01.09.04)

Details:	See details in the Detector Infrastructure Interface Control Document.

Requirements:	P-DET-DS-HCAL.1, P-DET-DS-PID.5

References:

I-DET-DS-INF-INT.035	Space Constraint

System 1:	Backward HCal Systems (6.03.01.05)

System 2:	Barrel DIRC Particle ID Systems (6.03.01.03)

Type:	SPACE

Description:	The maximum backward location for the DIRC is limited by the backward HCAL and the adjacent cabling pathway that provides services to the interior detectors. Modifications to either must be coordinated.

Provider:	Detector Infrastructure and Utilities Integration (6.03.01.09.04)

Details:	See details in the Detector Infrastructure Interface Control Document.

Requirements:	P-DET-DS-HCAL.1, P-DET-DS-PID.5

References:

I-DET-DS-INF-INT.084	Process Cooling

System 1:	Detector Infrastructure and Utilities Integration (6.03.01.09.04)

System 2:	Barrel DIRC Particle ID Systems (6.03.01.03)

Type:	COOL

Description:	The heat generated by the photo-multiplier tubes and associated electronics must be removed using a process cooling system.

Provider:	Detector Infrastructure and Utilities Integration (6.03.01.09.04)

Details:	See details in the Detector Infrastructure Interface Control Document.

Requirements:	P-DET-DS-PID.1

References:

I-DET-DS-INF-INT.085	Process Cooling

System 1:	Detector Infrastructure and Utilities Integration (6.03.01.09.04)

System 2:	Barrel TOF Particle ID Systems (6.03.01.03)

Type:	COOL

Description:	The heat generated by the detector and its electronics must be removed using a process cooling system.

Provider:	Detector Infrastructure and Utilities Integration (6.03.01.09.04)

Details:	See details in the Detector Infrastructure Interface Control Document.

Requirements:	P-DET-DS-PID.1

References:

I-DET-DS-INF-INT.086	Process Cooling

System 1:	Detector Infrastructure and Utilities Integration (6.03.01.09.04)

System 2:	Backward Particle ID Systems (6.03.01.03)

Type:	COOL

Description:	The heat generated by the photomultipliers and associated electronics must be removed using a process cooling system.

Provider:	Detector Infrastructure and Utilities Integration (6.03.01.09.04)

Details:	See details in the Detector Infrastructure Interface Control Document.

Requirements:	P-DET-DS-PID.1

References:

I-DET-DS-INF-INT.087	Dry Nitrogen

System 1:	Detector Infrastructure and Utilities Integration (6.03.01.09.04)

System 2:	Backward Particle ID Systems (6.03.01.03)

Type:	GAS/FLUID

Description:	A continuous flow of dry nitrogen will be required to protect the aerogel radiator.

Provider:	Detector Infrastructure and Utilities Integration (6.03.01.09.04)

Details:	See details in the Detector Infrastructure Interface Control Document.

Requirements:	P-DET-DS-PID-FWD-RICH.7

References:

I-DET-DS-INF-INT.088	Photosensor Cooling System

System 1:	Detector Infrastructure and Utilities Integration (6.03.01.09.04)

System 2:	Forward Particle ID Systems (6.03.01.03)

Type:	COOL

Description:	A fluid cooling system will be required to keep the operating temperature of the photosensors at -30 degrees Celsius.

Provider:	Detector Infrastructure and Utilities Integration (6.03.01.09.04)

Details:	See details in the Detector Infrastructure Interface Control Document.

Requirements:	P-DET-DS-PID.1

References:

I-DET-DS-INF-INT.089	Gas Recovery System

System 1:	Detector Infrastructure and Utilities Integration (6.03.01.09.04)

System 2:	Forward Particle ID Systems (6.03.01.03)

Type:	ENV

Description:	A recovery system will need to be included for the radiator gas used by the dRICH to prevent accidental release.

Provider:	Detector Infrastructure and Utilities Integration (6.03.01.09.04)

Details:	See details in the Detector Infrastructure Interface Control Document.

Requirements:	P-DET-DS-PID-FWD-RICH.6

References:

I-DET-DS-INF-INT.090	Insulation

System 1:	Detector Infrastructure and Utilities Integration (6.03.01.09.04)

System 2:	Forward Particle ID Systems (6.03.01.03)

Type:	ENV

Description:	Sufficient insulation will need to be added around all coolant lines to prevent condensation and ensure that ice doesn't form.

Provider:	Detector Infrastructure and Utilities Integration (6.03.01.09.04)

Details:	See details in the Detector Infrastructure Interface Control Document.

Requirements:	P-DET-DS-PID.1

References:

I-DET-DS-INF-INT.091	Dry Nitrogen for Aerogel

System 1:	Detector Infrastructure and Utilities Integration (6.03.01.09.04)

System 2:	Forward Particle ID Systems (6.03.01.03)

Type:	GAS/FLUID

Description:	A continuous flow of dry nitrogen will be required to protect the aerogel radiator.

Provider:	Detector Infrastructure and Utilities Integration (6.03.01.09.04)

Details:	See details in the Detector Infrastructure Interface Control Document.

Requirements:	P-DET-DS-PID.1

References:

I-DET-DS-INF-INT.092	Dry Nitrogen for Photosensors

System 1:	Detector Infrastructure and Utilities Integration (6.03.01.09.04)

System 2:	Forward Particle ID Systems (6.03.01.03)

Type:	GAS/FLUID

Description:	This gas will be required to create a protective atmosphere for the cold photosensors and to prevent condensation and freezing.

Provider:	Detector Infrastructure and Utilities Integration (6.03.01.09.04)

Details:	See details in the Detector Infrastructure Interface Control Document.

Requirements:	P-DET-DS-PID.1

References:

I-DET-DS-INF-INT.093	Radiator Gas

System 1:	Detector Infrastructure and Utilities Integration (6.03.01.09.04)

System 2:	Forward Particle ID Systems (6.03.01.03)

Type:	GAS/FLUID

Description:	A continuous recirculating flow of radiator gas will be required.

Provider:	Detector Infrastructure and Utilities Integration (6.03.01.09.04)

Details:	See details in the Detector Infrastructure Interface Control Document.

Requirements:	P-DET-DS-PID-FWD-RICH.6

References:

I-DET-DS-INF-INT.097	Space Constraint

System 1:	Solenoid Magnet (6.03.01.06)

System 2:	Forward RICH Detector Systems (6.03.01.03)

Type:	SPACE

Description:	The maximum forward location for the solenoid is limited by the position of the Dual RICH detector and the cabling plenum (gap) between them. Modifications to either must be coordinated.

Provider:	Detector Infrastructure and Utilities Integration (6.03.01.09.04)

Details:	See details in the Detector Infrastructure Interface Control Document.

Requirements:	P-DET-DS-MAG.1, P-DET-DS-PID.5

References:

I-DET-DS-INF-INT.098	Space Constraint

System 1:	Barrel DIRC Particle ID Systems (6.03.01.03)

System 2:	Barrel TOF Particle ID Systems (6.03.01.03)

Type:	SPACE

Description:	The maximum diameter of the barrel Time of Flight detector is limited by the interior bore of the DIRC detector. Modifications to either must be coordinated.

Provider:	Detector Infrastructure and Utilities Integration (6.03.01.09.04)

Details:	See details in the Detector Infrastructure Interface Control Document.

Requirements:	P-DET-DS-PID.5

References:

I-DET-DS-INF-INT.099	Space Constraint

System 1:	Barrel DIRC Particle ID Systems (6.03.01.03)

System 2:	Backward RICH Detector Systems (6.03.01.03)

Type:	SPACE

Description:	The exterior radius of the pfRICH/mRICH is limited by the interior bore of the DIRC support system. Modifications to either must be coordinated.

Provider:	Detector Infrastructure and Utilities Integration (6.03.01.09.04)

Details:	See details in the Detector Infrastructure Interface Control Document.

Requirements:	P-DET-DS-PID.5

References:

I-DET-DS-INF-INT.100	Space Constraint

System 1:	Barrel DIRC Particle ID Systems (6.03.01.03)

System 2:	Forward RICH Detector Systems (6.03.01.03)

Type:	SPACE

Description:	The maximum forward location for the DIRC is limited by the position of the dRICH detector and the cabling plenum between them. Modifications to either must be coordinated.

Provider:	Detector Infrastructure and Utilities Integration (6.03.01.09.04)

Details:	See details in the Detector Infrastructure Interface Control Document.

Requirements:	P-DET-DS-PID.5

References:

I-DET-DS-INF-INT.101	Service Pathway

System 1:	Barrel DIRC Particle ID Systems (6.03.01.03)

System 2:	Barrel Tracking Systems (6.03.01.02)

Type:	SPACE

Description:	Conduits must be provided within the DIRC support system that are adequate to deliver services (power, signal, cooling) to the barrel tracking detectors.

Provider:	Detector Infrastructure and Utilities Integration (6.03.01.09.04)

Details:	See details in the Detector Infrastructure Interface Control Document.

Requirements:	P-DET-DS-PID.5, P-DET-DS-TRAK.9

References:

I-DET-DS-INF-INT.102	Space Constraint

System 1:	Barrel DIRC Particle ID Systems (6.03.01.03)

System 2:	Barrel Tracking Systems (6.03.01.02)

Type:	SPACE

Description:	The exterior radius of the silicon trackers is limited by the interior bore of the DIRC support system. Modifications to either must be coordinated.

Provider:	Detector Infrastructure and Utilities Integration (6.03.01.09.04)

Details:	See details in the Detector Infrastructure Interface Control Document.

Requirements:	P-DET-DS-PID.5, P-DET-DS-TRAK.9

References:

I-DET-DS-INF-INT.103	Space Constraint

System 1:	Barrel DIRC Particle ID Systems (6.03.01.03)

System 2:	Barrel Tracking Systems (6.03.01.02)

Type:	SPACE

Description:	The maximum forward location for the DIRC is limited by the position of the silicon trackers.

Provider:	Detector Infrastructure and Utilities Integration (6.03.01.09.04)

Details:	See details in the Detector Infrastructure Interface Control Document.

Requirements:	P-DET-DS-PID.5, P-DET-DS-TRAK.9

References:

I-DET-DS-INF-INT.104	Space Constraint

System 1:	Barrel TOF Particle ID Systems (6.03.01.03)

System 2:	Backward RICH Detector Systems (6.03.01.03)

Type:	SPACE

Description:	The maximum backward location for the barrel Time of Flight  detector is limited by the position of the pfRICH/mRICH detector and the adjacent cabling pathway. Modifications to either must be coordinated.

Provider:	Detector Infrastructure and Utilities Integration (6.03.01.09.04)

Details:	See details in the Detector Infrastructure Interface Control Document.

Requirements:	P-DET-DS-PID.5

References:

I-DET-DS-INF-INT.105	Space Constraint

System 1:	Barrel TOF Particle ID Systems (6.03.01.03)

System 2:	Forward Time of Flight Systems (6.03.01.03)

Type:	SPACE

Description:	The maximum forward location for the barrel Time of Flight detector is limited by the position of the forward Time of Flight detector and the adjacent cabling pathway. Modifications to either must be coordinated.

Provider:	Detector Infrastructure and Utilities Integration (6.03.01.09.04)

Details:	See details in the Detector Infrastructure Interface Control Document.

Requirements:	P-DET-DS-PID.5

References:

I-DET-DS-INF-INT.106	Space Constraint

System 1:	Barrel TOF Particle ID Systems (6.03.01.03)

System 2:	Barrel Tracking Systems (6.03.01.02)

Type:	SPACE

Description:	The maximum size of the silicon trackers is limited by the interior radius of the barrel Time of Flight detector. Modifications to either must be coordinated.

Provider:	Detector Infrastructure and Utilities Integration (6.03.01.09.04)

Details:	See details in the Detector Infrastructure Interface Control Document.

Requirements:	P-DET-DS-PID.5, P-DET-DS-TRAK.9

References:

I-DET-DS-INF-INT.107	Space Constraint

System 1:	Backward RICH Detector Systems (6.03.01.03)

System 2:	Barrel Tracking Systems (6.03.01.02)

Type:	SPACE

Description:	The maximum backward location for the tracking system is limited by the position of the pfRICH/mRICH. Modifications to either must be coordinated.

Provider:	Detector Infrastructure and Utilities Integration (6.03.01.09.04)

Details:	See details in the Detector Infrastructure Interface Control Document.

Requirements:	P-DET-DS-PID.5, P-DET-DS-TRAK.9

References:

I-DET-DS-INF-INT.108	Space Constraint

System 1:	Backward RICH Detector Systems (6.03.01.03)

System 2:	IR Accelerator Systems (6.04.03)

Type:	SPACE

Description:	The interior radius of the pfRICH is governed by the size of the beamline and its associated flanges and support system.

Provider:	Detector Infrastructure and Utilities Integration (6.03.01.09.04)

Details:	See details in the Detector Infrastructure Interface Control Document.

Requirements:	P-DET-DS-PID.5

References:

I-DET-DS-INF-INT.137	Space Constraint

System 1:	Detector Infrastructure and Utilities Integration (6.03.01.09.04)

System 2:	Barrel DIRC Particle ID Systems (6.03.01.03)

Type:	SPACE

Description:	Rack space on the platform must be provided for a small laser system to monitor the response of the photsensors and allow adjustment of timing correlations within the detector.

Provider:	Detector Infrastructure and Utilities Integration (6.03.01.09.04)

Details:	See details in the Detector Infrastructure Interface Control Document.

Requirements:	P-DET-DS-PID-BAR-DIRC.7

References:

I-DET-DS-INF-INT.138	Space Constraint

System 1:	Detector Infrastructure and Utilities Integration (6.03.01.09.04)

System 2:	Forward RICH Detector Systems (6.03.01.03)

Type:	SPACE

Description:	Rack space on the platform must be provided for a small laser system to monitor the response of the photsensors and allow adjustment of timing correlations within the detector.

Provider:	Detector Infrastructure and Utilities Integration (6.03.01.09.04)

Details:	See details in the Detector Infrastructure Interface Control Document.

Requirements:	P-DET-DS-PID-FWD-RICH.10

References:

I-DET-DS-INF-INT.139	Space Constraint

System 1:	Detector Infrastructure and Utilities Integration (6.03.01.09.04)

System 2:	Backward RICH Detector Systems (6.03.01.03)

Type:	SPACE

Description:	Rack space on the platform must be provided for a small laser system to monitor the response of the photsensors and allow adjustment of timing correlations within the detector.

Provider:	Detector Infrastructure and Utilities Integration (6.03.01.09.04)

Details:	See details in the Detector Infrastructure Interface Control Document.

Requirements:	P-DET-DS-PID-BCK-RICH.7

References:

# Detector Infrastructure Systems (DET-DS-INF) to Tracking Systems (DET-DS-TRAK)

I-DET-DS-INF-BAR.016	Support Structure

System 1:	Barrel Detector Structures (6.03.01.09.01)

System 2:	Barrel Tracking Systems (6.03.01.02)

Type:	STRUCT

Description:	A single structural support system will support the silicon detectors and the micro-pattern gaseous detectors within the DIRC detector.

Provider:	Barrel Detector Structures (6.03.01.09.01)

Details:	See details in the Detector Infrastructure Interface Control Document.

Requirements:	P-DET-DS-TRAK.8

References:

I-DET-DS-INF-BAR.021	Silicon Tracker Support System

System 1:	Barrel DIRC Particle ID Systems (6.03.01.03)

System 2:	Tracking Systems (6.03.01.02)

Type:	STRUCT

Description:	The DIRC support system will provide support for the silicon trackers in the detector barrel.

Provider:	Barrel Detector Structures (6.03.01.09.01)

Details:	See details in the Detector Infrastructure Interface Control Document.

Requirements:	P-DET-DS-PID.4, P-DET-DS-PID.5, P-DET-DS-TRAK.8, P-DET-DS-TRAK.9

References:

I-DET-DS-INF-BAR.022	Weight Distributed

System 1:	Barrel DIRC Particle ID Systems (6.03.01.03)

System 2:	Barrel Tracking Systems (6.03.01.02)

Type:	STRUCT

Description:	The weight of the barrel tracking systems will be transferred to the DIRC support system.

Provider:	Barrel Detector Structures (6.03.01.09.01)

Details:	See details in the Detector Infrastructure Interface Control Document.

Requirements:	P-DET-DS-PID.4, P-DET-DS-TRAK.8

References:

I-DET-DS-INF-FWD.012	Space Constraint

System 1:	Forward Time of Flight Systems (6.03.01.03)

System 2:	Barrel Tracking Systems (6.03.01.02)

Type:	SPACE

Description:	The maximum forward location for the tracking system is limited by the position of the AC LGAD Time of Flight Detector. Modifications to either must be coordinated.

Provider:	Hadron Endcap Structures (6.03.01.09.03)

Details:	See details in the Detector Infrastructure Interface Control Document.

Requirements:	P-DET-DS-PID.5, P-DET-DS-TRAK.9

References:

I-DET-DS-INF-FWD.013	Space Constraint

System 1:	Barrel Tracking Systems (6.03.01.02)

System 2:	IR Accelerator Systems (6.04.03)

Type:	SPACE

Description:	The interior radius of the tracking detectors is governed by the size of the beamline.

Provider:	Hadron Endcap Structures (6.03.01.09.03)

Details:	See details in the Detector Infrastructure Interface Control Document.

Requirements:	P-DET-DS-TRAK.9

References:

I-DET-DS-INF-INT.094	Heat Rejection

System 1:	Detector Infrastructure and Utilities Integration (6.03.01.09.04)

System 2:	Barrel Tracking Systems (6.03.01.02)

Type:	COOL

Description:	Air, liquid or other cooling technology will be required for the tracking detectors.

Provider:	Detector Infrastructure and Utilities Integration (6.03.01.09.04)

Details:	See details in the Detector Infrastructure Interface Control Document.

Requirements:	P-DET-DS-TRAK.5

References:

I-DET-DS-INF-INT.095	Ionization Gas

System 1:	Detector Infrastructure and Utilities Integration (6.03.01.09.04)

System 2:	Barrel Tracking Systems (6.03.01.02)

Type:	GAS/FLUID

Description:	Gas will need to be provided to the trackers for detector operation.

Provider:	Detector Infrastructure and Utilities Integration (6.03.01.09.04)

Details:	See details in the Detector Infrastructure Interface Control Document.

Requirements:	P-DET-DS-TRAK.4

References:

I-DET-DS-INF-INT.096	Support Structure

System 1:	Detector Infrastructure and Utilities Integration (6.03.01.09.04)

System 2:	Barrel Tracking Systems (6.03.01.02)

Type:	STRUCT

Description:	A single structural support system will support the silicon detectors and the micro-pattern gaseous detectors within the DIRC detector.

Provider:	Detector Infrastructure and Utilities Integration (6.03.01.09.04)

Details:	See details in the Detector Infrastructure Interface Control Document.

Requirements:	P-DET-DS-TRAK.8

References:

I-DET-DS-INF-INT.101	Service Pathway

System 1:	Barrel DIRC Particle ID Systems (6.03.01.03)

System 2:	Barrel Tracking Systems (6.03.01.02)

Type:	SPACE

Description:	Conduits must be provided within the DIRC support system that are adequate to deliver services (power, signal, cooling) to the barrel tracking detectors.

Provider:	Detector Infrastructure and Utilities Integration (6.03.01.09.04)

Details:	See details in the Detector Infrastructure Interface Control Document.

Requirements:	P-DET-DS-PID.5, P-DET-DS-TRAK.9

References:

I-DET-DS-INF-INT.102	Space Constraint

System 1:	Barrel DIRC Particle ID Systems (6.03.01.03)

System 2:	Barrel Tracking Systems (6.03.01.02)

Type:	SPACE

Description:	The exterior radius of the silicon trackers is limited by the interior bore of the DIRC support system. Modifications to either must be coordinated.

Provider:	Detector Infrastructure and Utilities Integration (6.03.01.09.04)

Details:	See details in the Detector Infrastructure Interface Control Document.

Requirements:	P-DET-DS-PID.5, P-DET-DS-TRAK.9

References:

I-DET-DS-INF-INT.103	Space Constraint

System 1:	Barrel DIRC Particle ID Systems (6.03.01.03)

System 2:	Barrel Tracking Systems (6.03.01.02)

Type:	SPACE

Description:	The maximum forward location for the DIRC is limited by the position of the silicon trackers.

Provider:	Detector Infrastructure and Utilities Integration (6.03.01.09.04)

Details:	See details in the Detector Infrastructure Interface Control Document.

Requirements:	P-DET-DS-PID.5, P-DET-DS-TRAK.9

References:

I-DET-DS-INF-INT.106	Space Constraint

System 1:	Barrel TOF Particle ID Systems (6.03.01.03)

System 2:	Barrel Tracking Systems (6.03.01.02)

Type:	SPACE

Description:	The maximum size of the silicon trackers is limited by the interior radius of the barrel Time of Flight detector. Modifications to either must be coordinated.

Provider:	Detector Infrastructure and Utilities Integration (6.03.01.09.04)

Details:	See details in the Detector Infrastructure Interface Control Document.

Requirements:	P-DET-DS-PID.5, P-DET-DS-TRAK.9

References:

I-DET-DS-INF-INT.107	Space Constraint

System 1:	Backward RICH Detector Systems (6.03.01.03)

System 2:	Barrel Tracking Systems (6.03.01.02)

Type:	SPACE

Description:	The maximum backward location for the tracking system is limited by the position of the pfRICH/mRICH. Modifications to either must be coordinated.

Provider:	Detector Infrastructure and Utilities Integration (6.03.01.09.04)

Details:	See details in the Detector Infrastructure Interface Control Document.

Requirements:	P-DET-DS-PID.5, P-DET-DS-TRAK.9

References:

# Applicable Documents

List applicable documents considered to be part of this document. Applicable documents shall be obtained from the EIC SharePoint Document Center.

# Detailed change Log

| Revision | Date | Change |
| --- | --- | --- |

# References

- doi:10.17226/25171., National Academies of Sciences, Engineering, and Medicine, "An Assessment of U.S.-Based Electron-Ion Collider Science", The National Academies Press, Washington, DC, 2018.

- EIC-ORG-PLN-010, Electron-Ion Collider L1 Requirements (Global Requirements Document)

- BNL-221006-2021-FORE, EIC Conceptual Design Report

- arXiv:2103.05419, Science Requirements and Detector Concepts for the Electron-Ion Collider: EIC Yellow Report

- EIC-SEG-PLN-016, Requirements Management Plan

- EIC-SEG-PLN-022, Systems Engineering Management Plan

- EIC-SEG-PLN-020, EIC Interface Management Plan