Electron-Ion Collider, Requirements, Specifications, and Interfaces

Interface Control Document: [###]

Detector Infrastructure Systems (6.03.01.09) to IR Accelerator Systems (6.04.03)

Prepared by:

{{Sig_es_:signer1:signature}}	Date: {{Dte_es_:signer1:date}}

Name, Role

Electron-Ion Collider at Institution

Reviewed by:

{{Sig_es_:signer2:signature}}	Date: {{Dte_es_:signer2:date}}

Name, Role

Electron-Ion Collider at Institution

{{Sig_es_:signer3:signature}}	Date: {{Dte_es_:signer3:date}}

Name, Role

Electron-Ion Collider at Institution

{{Sig_es_:signer4:signature}}	Date: {{Dte_es_:signer4:date}}

Name, Role

Electron-Ion Collider at Institution

Approved by:

{{Sig_es_:signer5:signature}}	Date: {{Dte_es_:signer5:date}}

Name, Role

Electron-Ion Collider at Institution

REVISION HISTORY

| Revision # | Effective Date | Additional Reviewers | Summary of Change |
| --- | --- | --- | --- |
| 00 |  |  | Initial release. |

TABLE OF CONTENTS

1.	Purpose and Scope	5

2.	Introduction	5

2.1	Detector Infrastructure Systems	5

2.2	Overview of Interface Relationships	5

2.3	Interface Responsibilities	5

2.4	Safety and Environmental Considerations	5

2.5	Design Considerations	5

2.6	Organization	6

3.	Interface summary	6

4.	Detector Infrastructure Systems (DET-DS-INF) to IR Accelerator Systems (IR-AS)	9

5.	Applicable Documents	13

6.	Detailed change Log	13

7.	References	14

LIST OF ACRONYMS

| B0 | B0 is the name of the forward magnet just past the Hadron Endcap |  |
| --- | --- | --- |
| CDR | Conceptual Design Report |  |
| COTS | Commercial Off the Shelf |  |
| DAQ | Data Acquisition |  |
| DET | EIC Detector |  |
| DIRC | Detection of Internally Reflected Cherenkov Light |  |
| dRICH | Dual radiator ring imaging Cherenkov |  |
| DVCS | Deeply Virtual Compton Scattering |  |
| EIC | Electron-Ion Collider |  |
| EMCAL | Electro-magnetic Calorimeter |  |
| ESR | Electron Storage Ring |  |
| FCC | Federal Communications Commission |  |
| FDR | Final Design Review |  |
| GEM | Gas Electron Multiplier |  |
| HCAL | Hadronic Calorimeter |  |
| HSR | Hadron Storage Ring |  |
| HVAC | Heating/Ventilation/Air Conditioning |  |
| IP | Interaction Point |  |
| IR | Interaction Region |  |
| MPGD | Micro Pattern Gaseous Detectors |  |
| NAS | National Academy of Sciences |  |
| NEC | National Electric Code |  |
| NEMA | National Electrical Manufacturers Association |  |
| NFPA | National Fire Protection Association |  |
| NRTL | Nationally Recognized Testing Laboratory |  |
| PID | Particle Identification |  |
| PRR | Production Readiness Review |  |
| PWO | PbWO4 |  |
| RCS | Rapid Cycling Synchrotron |  |
| TBD | To Be Determined |  |
| UL | Underwriter Laboratories |  |
| WBS | Work Break Down Structure |  |

# Interface Control Document:

Detector Infrastructure Systems (6.03.01.09) to IR Accelerator Systems (6.04.03)

# Purpose and Scope

This document records the agreed upon interfaces between the Detector Infrastructure Systems and other sub-systems within the IR Accelerator Systems. These interfaces are based on the system requirements that are necessary to integrate these sub-systems as part of the Electron Ion Collider Project.

# Introduction

These interfaces describe the relationships between the Detector Infrastructure Systems and the IR Accelerator Systems including (examples by inclusion) located in (areas by inclusion).

## Detector Infrastructure Systems

Provide an introductory description of your sub-system and an overview of the things that it receives, provides, supports, constrains, or otherwise requires as part of its function --- specifically if these things are manifested as interfaces.

IR Accelerator Systems (IR-AS): Description of the TYPES of interfaces that exist between the ICD reference system and this sub-system, and a BRIEF overview of the roles they play.

## Overview of Interface Relationships

The following are the sub-systems within the IR Accelerator Systems that have interfaces connected to the Detector Infrastructure Systems.

## Interface Responsibilities

The various ownerships of responsibilities may be listed here. However, this section might also refer to the Provider section of the interface detail if all pertinent responsibilities are listed there. If this is the case, simply enter: “See the Provider section within the interface details for information about the responsibilities associated with individual interfaces.”

## Safety and Environmental Considerations

General safety and environmental considerations are identified here. Any considerations that are specific to the individual sub-system, will be addressed in external safety documents and are referenced here.

### Considerations for all systems

Add general safety and environmental considerations here.

### Special considerations and references

Add references here as necessary.

## Design Considerations

Notes here about general design considerations.

## Organization

The interfaces in this document are grouped by sub-system as identified in the sub-system identifier. All interfaces between the Detector Infrastructure Systems and a related sub-system will be included in a single section. Entries within that section will be sorted by the interface identifier or WBS number, as appropriate.

# Interface summary

This tables in this section provide a list of all interfaces that are defined within this document. The interfaces are grouped by sub-system/WBS. Each table has the following fields:

Section: the section number in this document where the interface is addressed.

WBS (1): the WBS number within the ICD reference system, unless the reference system is the owner.

WBS (2): the WBS number for the connected system.

ID: the interface identifier as defined in the Interface Requirement Document (IRD).

Name: the name of the interface as defined in the IRD.

Description: the description of the interface as defined in the IRD.

Owner: identifies the WBS number of the entity that is responsible for ensuring the interface is provided and that it meets validation/verification requirements.

Detector Infrastructure Systems to IR Accelerator Systems

| Section | WBS1 | WBS2 | ID | Name | Description | Owner |
| --- | --- | --- | --- | --- | --- | --- |
| 4 |  | 6.04.03 |  |  | IR Accelerator Systems |  |
| 4.1 | 6.03.01.09 | 6.04.03 | I-DET-DS-INF-FWD.001 | Detector Support | The detector will be supported by an integrated support stand that is fully integrated with the vacuum system and allows the position of the detector to be adjusted. | 6.03.01.09.03 |
| 4.2 | 6.03.01.09 | 6.04.03 | I-DET-DS-INF-FWD.002 | Detector Support | The detector will be supported by an integrated support stand that is fully integrated with the vacuum system and allows the position of the detector to be adjusted. | 6.03.01.09.03 |
| 4.3 | 6.03.01.09 | 6.04.03 | I-DET-DS-INF-FWD.011 | Space Constraint | The interior radius of the dRICH is governed by the size of the beamline and its associated flanges and support system. | 6.03.01.09.03 |
| 4.4 | 6.03.01.09 | 6.04.03 | I-DET-DS-INF-FWD.013 | Space Constraint | The interior radius of the tracking detectors is governed by the size of the beamline. | 6.03.01.09.03 |
| 4.5 | 6.03.01.09 | 6.04.03 | I-DET-DS-INF-INT.007 | Space Constraint | Access to the B0 detector is constrained by the vacuum valve in front of the B0 magnet. A minimum of 15 centimeters of free space will be required on the IP-side of the B0-dipole for the installation of the B0 detectors. | 6.03.01.09.04 |
| 4.6 | 6.03.01.09 | 6.04.03 | I-DET-DS-INF-INT.008 | Space Constraint | The electron and hadron beam pipes must be contained within the B0 detector. | 6.03.01.09.04 |
| 4.7 | 6.03.01.09 | 6.04.03 | I-DET-DS-INF-INT.009 | Position Constraint | The LowQ2 detector must be upstream in the electron direction from the Q3ER magnet. | 6.03.01.09.04 |
| 4.8 | 6.03.01.09 | 6.04.03 | I-DET-DS-INF-INT.010 | Position Constraint | The LowQ2 detector must be downstream in the electron direction from the B2ER magnet. | 6.03.01.09.04 |
| 4.9 | 6.03.01.09 | 6.04.03 | I-DET-DS-INF-INT.021 | Backward ECAL Bore Clearance | The bore of the backward ECAL must be designed to allow it to be inserted/removed over the existing beamline flanges. | 6.03.01.09.04 |
| 4.10 | 6.03.01.09 | 6.04.03 | I-DET-DS-INF-INT.022 | Space Constraint | The interior radius of the backward ECAL is governed by the size of the beamline. | 6.03.01.09.04 |
| 4.11 | 6.03.01.09 | 6.04.03 | I-DET-DS-INF-INT.037 | Space Constraint | The interior radius of the backward HCAL is governed by the size of the beamline. | 6.03.01.09.04 |
| 4.12 | 6.03.01.09 | 6.04.03 | I-DET-DS-INF-INT.038 | Space Constraint | The interior radius of the forward HCAL is governed by the size of the beamline. | 6.03.01.09.04 |
| 4.13 | 6.03.01.09 | 6.04.03 | I-DET-DS-INF-INT.108 | Space Constraint | The interior radius of the pfRICH is governed by the size of the beamline and its associated flanges and support system. | 6.03.01.09.04 |
| 4.14 | 6.03.01.09 | 6.04.03 | I-DET-DS-INF-INT.134 | Beampipe Constraints for Central Detector | The size, configuration, and clearances of the beampipe, valves and connectors in the ePIC barrel must be coordinated with the design of the interior detector systems. | 6.03.01.09.04 |
| 4.15 | 6.03.01.09 | 6.04.03 | I-DET-DS-INF-INT.135 | Beampipe Constraints for Forward Endcap | The size, configuration, and clearances of the beampipe, valves and connectors through the forward endcap must be coordinated with the design of the detector systems. | 6.03.01.09.04 |
| 4.16 | 6.03.01.09 | 6.04.03 | I-DET-DS-INF-INT.136 | Beampipe Constraints for Backward Endcap | The size, configuration, and clearances of the beampipe, valves and connectors through the backward endcap must be coordinated with the design of the detector systems. | 6.03.01.09.04 |

# Detector Infrastructure Systems (DET-DS-INF) to IR Accelerator Systems (IR-AS)

I-DET-DS-INF-FWD.001	Detector Support

System 1:	Off-Momentum Detectors (6.03.01.10)

System 2:	IR Accelerator Systems (6.04.03)

Type:	STRUCT

Description:	The detector will be supported by an integrated support stand that is fully integrated with the vacuum system and allows the position of the detector to be adjusted.

Provider:	Hadron Endcap Structures (6.03.01.09.03)

Details:	See details in the Detector System Interface Control Document.

Requirements:	P-DET-DS-ANC.1, P-DET-DS-ANC.2

References:

I-DET-DS-INF-FWD.002	Detector Support

System 1:	Roman Pots (6.03.01.10)

System 2:	IR Accelerator Systems (6.04.03)

Type:	STRUCT

Description:	The detector will be supported by an integrated support stand that is fully integrated with the vacuum system and allows the position of the detector to be adjusted.

Provider:	Hadron Endcap Structures (6.03.01.09.03)

Details:	See details in the Detector System Interface Control Document.

Requirements:	P-DET-DS-ANC.1, P-DET-DS-ANC.2

References:

I-DET-DS-INF-FWD.011	Space Constraint

System 1:	Forward RICH Detector Systems (6.03.01.03)

System 2:	IR Accelerator Systems (6.04.03)

Type:	SPACE

Description:	The interior radius of the dRICH is governed by the size of the beamline and its associated flanges and support system.

Provider:	Hadron Endcap Structures (6.03.01.09.03)

Details:	See details in the Detector System Interface Control Document.

Requirements:	P-DET-DS-PID.5

References:

I-DET-DS-INF-FWD.013	Space Constraint

System 1:	Barrel Tracking Systems (6.03.01.02)

System 2:	IR Accelerator Systems (6.04.03)

Type:	SPACE

Description:	The interior radius of the tracking detectors is governed by the size of the beamline.

Provider:	Hadron Endcap Structures (6.03.01.09.03)

Details:	See details in the Detector System Interface Control Document.

Requirements:	P-DET-DS-TRAK.9

References:

I-DET-DS-INF-INT.007	Space Constraint

System 1:	B-Zero Detectors (6.03.01.10)

System 2:	IR Accelerator Systems (6.04.03)

Type:	SPACE

Description:	Access to the B0 detector is constrained by the vacuum valve in front of the B0 magnet. A minimum of 15 centimeters of free space will be required on the IP-side of the B0-dipole for the installation of the B0 detectors.

Provider:	Detector Infrastructure and Utilities Integration (6.03.01.09.04)

Details:	See details in the Detector System Interface Control Document.

Requirements:	P-DET-DS-ANC.1

References:

I-DET-DS-INF-INT.008	Space Constraint

System 1:	B-Zero Detectors (6.03.01.10)

System 2:	IR Accelerator Systems (6.04.03)

Type:	SPACE

Description:	The electron and hadron beam pipes must be contained within the B0 detector.

Provider:	Detector Infrastructure and Utilities Integration (6.03.01.09.04)

Details:	See details in the Detector System Interface Control Document.

Requirements:	P-DET-DS-ANC.1

References:

I-DET-DS-INF-INT.009	Position Constraint

System 1:	Low Q2 Detectors (6.03.01.10)

System 2:	IR Accelerator Systems (6.04.03)

Type:	SPACE

Description:	The LowQ2 detector must be upstream in the electron direction from the Q3ER magnet.

Provider:	Detector Infrastructure and Utilities Integration (6.03.01.09.04)

Details:	See details in the Detector System Interface Control Document.

Requirements:	P-DET-DS-ANC.1

References:

I-DET-DS-INF-INT.010	Position Constraint

System 1:	Low Q2 Detectors (6.03.01.10)

System 2:	IR Accelerator Systems (6.04.03)

Type:	SPACE

Description:	The LowQ2 detector must be downstream in the electron direction from the B2ER magnet.

Provider:	Detector Infrastructure and Utilities Integration (6.03.01.09.04)

Details:	See details in the Detector System Interface Control Document.

Requirements:	P-DET-DS-ANC.1

References:

I-DET-DS-INF-INT.021	Backward ECAL Bore Clearance

System 1:	Backward EMCal Systems (6.03.01.04)

System 2:	IR Accelerator Systems (6.04.03)

Type:	SPACE

Description:	The bore of the backward ECAL must be designed to allow it to be inserted/removed over the existing beamline flanges.

Provider:	Detector Infrastructure and Utilities Integration (6.03.01.09.04)

Details:	See details in the Detector System Interface Control Document.

Requirements:	P-DET-DS-ECAL.1

References:

I-DET-DS-INF-INT.022	Space Constraint

System 1:	Backward EMCal Systems (6.03.01.04)

System 2:	IR Accelerator Systems (6.04.03)

Type:	SPACE

Description:	The interior radius of the backward ECAL is governed by the size of the beamline.

Provider:	Detector Infrastructure and Utilities Integration (6.03.01.09.04)

Details:	See details in the Detector System Interface Control Document.

Requirements:	P-DET-DS-ECAL.1

References:

I-DET-DS-INF-INT.037	Space Constraint

System 1:	Backward HCal Systems (6.03.01.05)

System 2:	IR Accelerator Systems (6.04.03)

Type:	SPACE

Description:	The interior radius of the backward HCAL is governed by the size of the beamline.

Provider:	Detector Infrastructure and Utilities Integration (6.03.01.09.04)

Details:	See details in the Detector System Interface Control Document.

Requirements:	P-DET-DS-HCAL.1

References:

I-DET-DS-INF-INT.038	Space Constraint

System 1:	Forward HCal Systems (6.03.01.05)

System 2:	IR Accelerator Systems (6.04.03)

Type:	SPACE

Description:	The interior radius of the forward HCAL is governed by the size of the beamline.

Provider:	Detector Infrastructure and Utilities Integration (6.03.01.09.04)

Details:	See details in the Detector System Interface Control Document.

Requirements:	P-DET-DS-HCAL.1

References:

I-DET-DS-INF-INT.108	Space Constraint

System 1:	Backward RICH Detector Systems (6.03.01.03)

System 2:	IR Accelerator Systems (6.04.03)

Type:	SPACE

Description:	The interior radius of the pfRICH is governed by the size of the beamline and its associated flanges and support system.

Provider:	Detector Infrastructure and Utilities Integration (6.03.01.09.04)

Details:	See details in the Detector System Interface Control Document.

Requirements:	P-DET-DS-PID.5

References:

I-DET-DS-INF-INT.134	Beampipe Constraints for Central Detector

System 1:	Detector Infrastructure and Utilities Integration (6.03.01.09.04)

System 2:	IR Accelerator Systems (6.04.03)

Type:	SPACE

Description:	The size, configuration, and clearances of the beampipe, valves and connectors in the ePIC barrel must be coordinated with the design of the interior detector systems.

Provider:	Detector Infrastructure and Utilities Integration (6.03.01.09.04)

Details:	See details in the Detector System Interface Control Document.

Requirements:	F-DET-DS-INF.4

References:

I-DET-DS-INF-INT.135	Beampipe Constraints for Forward Endcap

System 1:	Detector Infrastructure and Utilities Integration (6.03.01.09.04)

System 2:	IR Accelerator Systems (6.04.03)

Type:	SPACE

Description:	The size, configuration, and clearances of the beampipe, valves and connectors through the forward endcap must be coordinated with the design of the detector systems.

Provider:	Detector Infrastructure and Utilities Integration (6.03.01.09.04)

Details:	See details in the Detector System Interface Control Document.

Requirements:	F-DET-DS-INF.4

References:

I-DET-DS-INF-INT.136	Beampipe Constraints for Backward Endcap

System 1:	Detector Infrastructure and Utilities Integration (6.03.01.09.04)

System 2:	IR Accelerator Systems (6.04.03)

Type:	SPACE

Description:	The size, configuration, and clearances of the beampipe, valves and connectors through the backward endcap must be coordinated with the design of the detector systems.

Provider:	Detector Infrastructure and Utilities Integration (6.03.01.09.04)

Details:	See details in the Detector System Interface Control Document.

Requirements:	F-DET-DS-INF.4

References:

# Applicable Documents

List applicable documents considered to be part of this document. Applicable documents shall be obtained from the EIC SharePoint Document Center.

# Detailed change Log

| Revision | Date | Change |
| --- | --- | --- |

# References

- doi:10.17226/25171., National Academies of Sciences, Engineering, and Medicine, "An Assessment of U.S.-Based Electron-Ion Collider Science", The National Academies Press, Washington, DC, 2018.

- EIC-ORG-PLN-010, Electron-Ion Collider L1 Requirements (Global Requirements Document)

- BNL-221006-2021-FORE, EIC Conceptual Design Report

- arXiv:2103.05419, Science Requirements and Detector Concepts for the Electron-Ion Collider: EIC Yellow Report

- EIC-SEG-PLN-016, Requirements Management Plan

- EIC-SEG-PLN-022, Systems Engineering Management Plan

- EIC-SEG-PLN-020, EIC Interface Management Plan