Electron-Ion Collider, Requirements, Specifications, and Interfaces

Interface Control Document: [###]

Solenoid Magnet (6.03.01.06) to Detector System (6.03.01)

Prepared by:

{{Sig_es_:signer1:signature}}	Date: {{Dte_es_:signer1:date}}

Name, Role

Electron-Ion Collider at Institution

Reviewed by:

{{Sig_es_:signer2:signature}}	Date: {{Dte_es_:signer2:date}}

Name, Role

Electron-Ion Collider at Institution

{{Sig_es_:signer3:signature}}	Date: {{Dte_es_:signer3:date}}

Name, Role

Electron-Ion Collider at Institution

{{Sig_es_:signer4:signature}}	Date: {{Dte_es_:signer4:date}}

Name, Role

Electron-Ion Collider at Institution

Approved by:

{{Sig_es_:signer5:signature}}	Date: {{Dte_es_:signer5:date}}

Name, Role

Electron-Ion Collider at Institution

REVISION HISTORY

| Revision # | Effective Date | Additional Reviewers | Summary of Change |
| --- | --- | --- | --- |
| 00 |  |  | Initial release. |

TABLE OF CONTENTS

1.	Purpose and Scope	5

2.	Introduction	5

2.1	Solenoid Magnet	5

2.2	Overview of Interface Relationships	5

2.3	Interface Responsibilities	5

2.4	Safety and Environmental Considerations	6

2.5	Design Considerations	6

2.6	Organization	6

3.	Interface summary	6

4.	Solenoid Magnet (DET-DS-MAG) to Detector System (DET-DS)	16

5.	Solenoid Magnet (DET-DS-MAG) to Data Acquisition and Computing Systems (DET-DS-COMP)	16

6.	Solenoid Magnet (DET-DS-MAG) to Electromagnetic Calorimetry Systems (DET-DS-ECAL)	18

7.	Solenoid Magnet (DET-DS-MAG) to Electronic Systems (DET-DS-ELEC)	18

8.	Solenoid Magnet (DET-DS-MAG) to Hadronic Calorimetry Systems (DET-DS-HCAL)	20

9.	Solenoid Magnet (DET-DS-MAG) to Detector Infrastructure (DET-DS-INF)	21

10.	Solenoid Magnet (DET-DS-MAG) to Particle Identification Systems (DET-DS-PID)	26

11.	Applicable Documents	26

12.	Detailed change Log	26

13.	References	27

LIST OF ACRONYMS

| B0 | B0 is the name of the forward magnet just past the Hadron Endcap |  |
| --- | --- | --- |
| CDR | Conceptual Design Report |  |
| COTS | Commercial Off the Shelf |  |
| DAQ | Data Acquisition |  |
| DET | EIC Detector |  |
| DIRC | Detection of Internally Reflected Cherenkov Light |  |
| dRICH | Dual radiator ring imaging Cherenkov |  |
| DVCS | Deeply Virtual Compton Scattering |  |
| EIC | Electron-Ion Collider |  |
| EMCAL | Electro-magnetic Calorimeter |  |
| ESR | Electron Storage Ring |  |
| FCC | Federal Communications Commission |  |
| FDR | Final Design Review |  |
| GEM | Gas Electron Multiplier |  |
| HCAL | Hadronic Calorimeter |  |
| HSR | Hadron Storage Ring |  |
| HVAC | Heating/Ventilation/Air Conditioning |  |
| IP | Interaction Point |  |
| IR | Interaction Region |  |
| MPGD | Micro Pattern Gaseous Detectors |  |
| NAS | National Academy of Sciences |  |
| NEC | National Electric Code |  |
| NEMA | National Electrical Manufacturers Association |  |
| NFPA | National Fire Protection Association |  |
| NRTL | Nationally Recognized Testing Laboratory |  |
| PID | Particle Identification |  |
| PRR | Production Readiness Review |  |
| PWO | PbWO4 |  |
| RCS | Rapid Cycling Synchrotron |  |
| TBD | To Be Determined |  |
| UL | Underwriter Laboratories |  |
| WBS | Work Break Down Structure |  |

# Interface Control Document:

Solenoid Magnet (6.03.01.06) to Detector System (6.03.01)

# Purpose and Scope

This document records the agreed upon interfaces between the Solenoid Magnet and other sub-systems within the Detector System. These interfaces are based on the system requirements that are necessary to integrate these sub-systems as part of the Electron Ion Collider Project.

# Introduction

These interfaces describe the relationships between the Solenoid Magnet and the Detector System including (examples by inclusion) located in (areas by inclusion).

## Solenoid Magnet

Provide an introductory description of your sub-system and an overview of the things that it receives, provides, supports, constrains, or otherwise requires as part of its function --- specifically if these things are manifested as interfaces.

Detector System (DET-DS): Description of the TYPES of interfaces that exist between the ICD reference system and this sub-system, and a BRIEF overview of the roles they play.

Data Acquisition and Computing Systems (DET-DS-COMP): Description of the TYPES of interfaces that exist between the ICD reference system and this sub-system, and a BRIEF overview of the roles they play.

Electromagnetic Calorimetry Systems (DET-DS-ECAL): Description of the TYPES of interfaces that exist between the ICD reference system and this sub-system, and a BRIEF overview of the roles they play.

Electronic Systems (DET-DS-ELEC): Description of the TYPES of interfaces that exist between the ICD reference system and this sub-system, and a BRIEF overview of the roles they play.

Hadronic Calorimetry Systems (DET-DS-HCAL): Description of the TYPES of interfaces that exist between the ICD reference system and this sub-system, and a BRIEF overview of the roles they play.

Detector Infrastructure (DET-DS-INF): Description of the TYPES of interfaces that exist between the ICD reference system and this sub-system, and a BRIEF overview of the roles they play.

Particle Identification Systems (DET-DS-PID): Description of the TYPES of interfaces that exist between the ICD reference system and this sub-system, and a BRIEF overview of the roles they play.

## Overview of Interface Relationships

The following are the sub-systems within the Detector System that have interfaces connected to the Solenoid Magnet.

## Interface Responsibilities

The various ownerships of responsibilities may be listed here. However, this section might also refer to the Provider section of the interface detail if all pertinent responsibilities are listed there. If this is the case, simply enter: “See the Provider section within the interface details for information about the responsibilities associated with individual interfaces.”

## Safety and Environmental Considerations

General safety and environmental considerations are identified here. Any considerations that are specific to the individual sub-system, will be addressed in external safety documents and are referenced here.

### Considerations for all systems

Add general safety and environmental considerations here.

### Special considerations and references

Add references here as necessary.

## Design Considerations

Notes here about general design considerations.

## Organization

The interfaces in this document are grouped by sub-system as identified in the sub-system identifier. All interfaces between the Solenoid Magnet and a related sub-system will be included in a single section. Entries within that section will be sorted by the interface identifier or WBS number, as appropriate.

# Interface summary

This tables in this section provide a list of all interfaces that are defined within this document. The interfaces are grouped by sub-system/WBS. Each table has the following fields:

Section: the section number in this document where the interface is addressed.

WBS (1): the WBS number within the ICD reference system, unless the reference system is the owner.

WBS (2): the WBS number for the connected system.

ID: the interface identifier as defined in the Interface Requirement Document (IRD).

Name: the name of the interface as defined in the IRD.

Description: the description of the interface as defined in the IRD.

Owner: identifies the WBS number of the entity that is responsible for ensuring the interface is provided and that it meets validation/verification requirements.

Solenoid Magnet to Detector System

| Section | WBS1 | WBS2 | ID | Name | Description | Owner |
| --- | --- | --- | --- | --- | --- | --- |
| 4 |  | 6.03.01 |  |  | Detector System |  |
| 4.1 | 6.03.01.06 | 6.03.01 | I-DET-DS-INF-INT.001 | Magnetic Material | No detectors or components constructed with magnetic material may be added to the detector without prior consultation with the magnet group. | 6.03.01.09.04 |
| 4.2 | 6.03.01.06 | 6.03.01 | I-DET-DS-INF-INT.002 | Magnetic Field Requirement | All subordinate detectors are dependent on the continuous operation of the solenoid and on the delivery of a consistent, stable magnetic field. | 6.03.01.09.04 |

Solenoid Magnet to Data Acquisition and Computing Systems

| Section | WBS1 | WBS2 | ID | Name | Description | Owner |
| --- | --- | --- | --- | --- | --- | --- |
| 5 |  | 6.03.01.08 |  |  | Data Acquisition and Computing Systems |  |
| 5.1 | 6.03.01.06 | 6.03.01.08 | I-DET-DS-COMP-ONLINE.040 | Controls and Instrumentation | Control cabling for the magnet instrumentation will be run from the solenoid to the instrumentation rack on the carriage. | 6.03.01.08.02 |
| 5.2 | 6.03.01.06 | 6.03.01.08 | I-DET-DS-COMP-ONLINE.041 | Monitoring Data | Live monitoring data from the solenoid's operation must be recorded and maintained for diagnostic purposes. | 6.03.01.08.02 |
| 5.3 | 6.03.01.06 | 6.03.01.08 | I-DET-DS-COMP-ONLINE.042 | Slow Controls | A network connection will be provided from the DAQ system to the solenoid cryogenic's slow controls interface. | 6.03.01.08.02 |
| 5.4 | 6.03.01.06 | 6.03.01.08 | I-DET-DS-COMP-ONLINE.043 | Slow Controls | A network connection will be provided from the DAQ system to the solenoid's slow controls interface. | 6.03.01.08.02 |
| 5.5 | 6.03.01.06 | 6.03.01.08 | I-DET-DS-COMP-ONLINE.044 | Slow Controls | A network connection will be provided from the DAQ system to the slow controls interface of the solenoid's power supply. | 6.03.01.08.02 |

Solenoid Magnet to Electromagnetic Calorimetry Systems

| Section | WBS1 | WBS2 | ID | Name | Description | Owner |
| --- | --- | --- | --- | --- | --- | --- |
| 6 |  | 6.03.01.04 |  |  | Electromagnetic Calorimetry Systems |  |
| 6.1 | 6.03.01.06 | 6.03.01.04 | I-DET-DS-INF-INT.012 | Space Constraint | The exterior radius of the barrel ECAL (and its support system) is limited by the interior bore of the solenoid magnet. Modifications to either must be coordinated. | 6.03.01.09.04 |

Solenoid Magnet to Electronic Systems

| Section | WBS1 | WBS2 | ID | Name | Description | Owner |
| --- | --- | --- | --- | --- | --- | --- |
| 7 |  | 6.03.01.07 |  |  | Electronic Systems |  |
| 7.1 | 6.03.01.06 | 6.03.01.07 | I-DET-DS-ELEC.037 | Bias Voltage | Bias voltage DC power will be provided from the electronics racks to support the magnet cryogenic systems. | 6.03.01.07 |
| 7.2 | 6.03.01.06 | 6.03.01.07 | I-DET-DS-ELEC.038 | High Voltage | High voltage DC power will be provided from the electronics racks to support the magnet instrumentation and control systems. | 6.03.01.07 |
| 7.3 | 6.03.01.06 | 6.03.01.07 | I-DET-DS-ELEC.039 | Low Voltage | Lower voltage DC power will be provided from the electronics racks to support the magnet cryogenic systems. | 6.03.01.07 |
| 7.4 | 6.03.01.06 | 6.03.01.07 | I-DET-DS-ELEC.040 | Bias Voltage | Bias voltage DC power will be provided from the electronics racks to support the magnet instrumentation and control systems. | 6.03.01.07 |
| 7.5 | 6.03.01.06 | 6.03.01.07 | I-DET-DS-ELEC.041 | High Voltage | High voltage DC power will be provided from the electronics racks to support the magnet power supply. | 6.03.01.07 |
| 7.6 | 6.03.01.06 | 6.03.01.07 | I-DET-DS-ELEC.042 | Low Voltage | Low voltage DC power will be provided from the electronics racks to support the magnet instrumentation and control systems. | 6.03.01.07 |
| 7.7 | 6.03.01.06 | 6.03.01.07 | I-DET-DS-ELEC.043 | Bias Voltage | Bias voltage DC power will be provided from the electronics racks to support the magnet power supply. | 6.03.01.07 |
| 7.8 | 6.03.01.06 | 6.03.01.07 | I-DET-DS-ELEC.044 | High Voltage | High voltage DC power will be provided from the electronics racks to support electronics in the detector. | 6.03.01.07 |
| 7.9 | 6.03.01.06 | 6.03.01.07 | I-DET-DS-ELEC.045 | Low Voltage | Low voltage DC power will be provided from the electronics racks to support the magnet power supply. | 6.03.01.07 |

Solenoid Magnet to Hadronic Calorimetry Systems

| Section | WBS1 | WBS2 | ID | Name | Description | Owner |
| --- | --- | --- | --- | --- | --- | --- |
| 8 |  | 6.03.01.05 |  |  | Hadronic Calorimetry Systems |  |
| 8.1 | 6.03.01.06 | 6.03.01.05 | I-DET-DS-INF-INT.031 | External Space Constraint | The exterior radius of the solenoid magnet is limited by the interior bore of the barrel HCAL and necessary support structures. Modifications to either must be coordinated. | 6.03.01.09.04 |
| 8.2 | 6.03.01.06 | 6.03.01.05 | I-DET-DS-INF-INT.034 | Space Constraint | The maximum backward location for the solenoid magnet is limited by the backward HCAL and the adjacent cabling pathway that provides services to the interior detectors. Modifications to either must be coordinated. | 6.03.01.09.04 |

Solenoid Magnet to Detector Infrastructure

| Section | WBS1 | WBS2 | ID | Name | Description | Owner |
| --- | --- | --- | --- | --- | --- | --- |
| 9 |  | 6.03.01.09 |  |  | Detector Infrastructure |  |
| 9.1 | 6.03.01.06 | 6.03.01.09 | I-DET-DS-INF-BAR.015 | External Support Structure | Total weight of the solenoid and all embedded components must be supported. | 6.03.01.09.01 |
| 9.2 | 6.03.01.06 | 6.03.01.09 | I-DET-DS-INF-BAR.017 | DIRC Readout Supports | The DIRC prism box containing the photosensor readouts, will be supported by an external structure supporting the ECAL within the solenoid magnet. | 6.03.01.09.01 |
| 9.3 | 6.03.01.06 | 6.03.01.09 | I-DET-DS-INF-INT.001 | Magnetic Material | No detectors or components constructed with magnetic material may be added to the detector without prior consultation with the magnet group. | 6.03.01.09.04 |
| 9.4 | 6.03.01.06 | 6.03.01.09 | I-DET-DS-INF-INT.002 | Magnetic Field Requirement | All subordinate detectors are dependent on the continuous operation of the solenoid and on the delivery of a consistent, stable magnetic field. | 6.03.01.09.04 |
| 9.5 | 6.03.01.06 | 6.03.01.09 | I-DET-DS-INF-INT.012 | Space Constraint | The exterior radius of the barrel ECAL (and its support system) is limited by the interior bore of the solenoid magnet. Modifications to either must be coordinated. | 6.03.01.09.04 |
| 9.6 | 6.03.01.06 | 6.03.01.09 | I-DET-DS-INF-INT.031 | External Space Constraint | The exterior radius of the solenoid magnet is limited by the interior bore of the barrel HCAL and necessary support structures. Modifications to either must be coordinated. | 6.03.01.09.04 |
| 9.7 | 6.03.01.06 | 6.03.01.09 | I-DET-DS-INF-INT.034 | Space Constraint | The maximum backward location for the solenoid magnet is limited by the backward HCAL and the adjacent cabling pathway that provides services to the interior detectors. Modifications to either must be coordinated. | 6.03.01.09.04 |
| 9.8 | 6.03.01.06 | 6.03.01.09 | I-DET-DS-INF-INT.039 | Magnet Cooling | The solenoid will require a continuous flow of cryogens to maintain temperature in both the experimental hall and the maintenance area. Interface includes cryogenic source and warm return. | 6.03.01.09.04 |
| 9.9 | 6.03.01.06 | 6.03.01.09 | I-DET-DS-INF-INT.076 | Power Supply Cooling | The power supply, located in an adjacent electrical room, will need to be cooled with low conductivity water. | 6.03.01.09.04 |
| 9.10 | 6.03.01.06 | 6.03.01.09 | I-DET-DS-INF-INT.077 | Magnet Cooling | The solenoid will require a continuous flow of cryogens to maintain temperature in both the experimental hall and the maintenance area. Interface includes cryogenic source and warm return. | 6.03.01.09.04 |
| 9.11 | 6.03.01.06 | 6.03.01.09 | I-DET-DS-INF-INT.078 | Electrical Power | Solenoid will be provided power via the power supply located in an adjacent room. | 6.03.01.09.04 |
| 9.12 | 6.03.01.06 | 6.03.01.09 | I-DET-DS-INF-INT.079 | Instrumentation Backup Power | The instrumentation rack for the solenoid will require backup/UPS power adequate to safely shutdown the system in the event of a power failure. | 6.03.01.09.04 |
| 9.13 | 6.03.01.06 | 6.03.01.09 | I-DET-DS-INF-INT.080 | Fringe Field | A boundary must be established to identify the extent of the magnet's stray field during operations. | 6.03.01.09.04 |
| 9.14 | 6.03.01.06 | 6.03.01.09 | I-DET-DS-INF-INT.081 | Ice Management | An ice management system will be required to melt accumulated ice on the chimney. (Water should evaporate and will not require drainage) | 6.03.01.09.04 |
| 9.15 | 6.03.01.06 | 6.03.01.09 | I-DET-DS-INF-INT.082 | Cryogenic Connection Point | The cryogenic connection must be as close as possible to the source. | 6.03.01.09.04 |
| 9.16 | 6.03.01.06 | 6.03.01.09 | I-DET-DS-INF-INT.083 | LCW Connection | The LCW water cooled leads must be fully supported along their path between the electrical room and the magnet. | 6.03.01.09.04 |
| 9.17 | 6.03.01.06 | 6.03.01.09 | I-DET-DS-INF-INT.097 | Space Constraint | The maximum forward location for the solenoid is limited by the position of the Dual RICH detector and the cabling plenum (gap) between them. Modifications to either must be coordinated. | 6.03.01.09.04 |
| 9.18 | 6.03.01.06 | 6.03.01.09 | I-DET-DS-INF-INT.114 | LHE Flow (Hall) | A supply and return of liquid helium must be provided to the experimental hall for cooling the solenoid magnet during operation. | 6.03.01.09.04 |
| 9.19 | 6.03.01.06 | 6.03.01.09 | I-DET-DS-INF-INT.115 | LHE Flow (Assembly Area) | A supply and return of liquid helium must be provided to the assembly hall for cooling the solenoid magnet during maintenance. | 6.03.01.09.04 |

Solenoid Magnet to Particle Identification Systems

| Section | WBS1 | WBS2 | ID | Name | Description | Owner |
| --- | --- | --- | --- | --- | --- | --- |
| 10 |  | 6.03.01.03 |  |  | Particle Identification Systems |  |
| 10.1 | 6.03.01.06 | 6.03.01.03 | I-DET-DS-INF-BAR.017 | DIRC Readout Supports | The DIRC prism box containing the photosensor readouts, will be supported by an external structure supporting the ECAL within the solenoid magnet. | 6.03.01.09.01 |
| 10.2 | 6.03.01.06 | 6.03.01.03 | I-DET-DS-INF-INT.097 | Space Constraint | The maximum forward location for the solenoid is limited by the position of the Dual RICH detector and the cabling plenum (gap) between them. Modifications to either must be coordinated. | 6.03.01.09.04 |

# Solenoid Magnet (DET-DS-MAG) to Detector System (DET-DS)

I-DET-DS-INF-INT.001	Magnetic Material

System 1:	Detector System (6.03.01)

System 2:	Solenoid Magnet (6.03.01.06)

Type:	ENV

Description:	No detectors or components constructed with magnetic material may be added to the detector without prior consultation with the magnet group.

Provider:	Detector Infrastructure and Utilities Integration (6.03.01.09.04)

Details:	See details in the Detector Infrastructure Interface Control Document.

Requirements:	P-DET-DS-MAG.13, P-DET-DS-MAG.14

References:

I-DET-DS-INF-INT.002	Magnetic Field Requirement

System 1:	Detector System (6.03.01)

System 2:	Solenoid Magnet (6.03.01.06)

Type:	PARAM

Description:	All subordinate detectors are dependent on the continuous operation of the solenoid and on the delivery of a consistent, stable magnetic field.

Provider:	Detector Infrastructure and Utilities Integration (6.03.01.09.04)

Details:	See details in the Detector Infrastructure Interface Control Document.

Requirements:	P-DET-DS-MAG.13

References:

# Solenoid Magnet (DET-DS-MAG) to Data Acquisition and Computing Systems (DET-DS-COMP)

I-DET-DS-COMP-ONLINE.040	Controls and Instrumentation

System 1:	Online DAQ and Computing Systems (6.03.01.08.02)

System 2:	Solenoid Magnet (6.03.01.06)

Type:	CONTROL

Description:	Control cabling for the magnet instrumentation will be run from the solenoid to the instrumentation rack on the carriage.

Provider:	Online DAQ and Computing Systems (6.03.01.08.02)

Details:	See details in the Data Acquisition and Computing Systems Interface Control Document.

Requirements:	F-DET-DS-COMP.6

References:

I-DET-DS-COMP-ONLINE.041	Monitoring Data

System 1:	Online DAQ and Computing Systems (6.03.01.08.02)

System 2:	Solenoid Magnet (6.03.01.06)

Type:	DATA

Description:	Live monitoring data from the solenoid's operation must be recorded and maintained for diagnostic purposes.

Provider:	Online DAQ and Computing Systems (6.03.01.08.02)

Details:	See details in the Data Acquisition and Computing Systems Interface Control Document.

Requirements:	F-DET-DS-COMP.6

References:

I-DET-DS-COMP-ONLINE.042	Slow Controls

System 1:	Online DAQ and Computing Systems (6.03.01.08.02)

System 2:	Magnet Cryogenics (6.03.01.06)

Type:	CONTROL

Description:	A network connection will be provided from the DAQ system to the solenoid cryogenic's slow controls interface.

Provider:	Online DAQ and Computing Systems (6.03.01.08.02)

Details:	See details in the Data Acquisition and Computing Systems Interface Control Document.

Requirements:	F-DET-DS-COMP-SC.1

References:

I-DET-DS-COMP-ONLINE.043	Slow Controls

System 1:	Online DAQ and Computing Systems (6.03.01.08.02)

System 2:	Magnet Instrumentation and Control (6.03.01.06)

Type:	CONTROL

Description:	A network connection will be provided from the DAQ system to the solenoid's slow controls interface.

Provider:	Online DAQ and Computing Systems (6.03.01.08.02)

Details:	See details in the Data Acquisition and Computing Systems Interface Control Document.

Requirements:	F-DET-DS-COMP.6

References:

I-DET-DS-COMP-ONLINE.044	Slow Controls

System 1:	Online DAQ and Computing Systems (6.03.01.08.02)

System 2:	Magnet Power Supply (6.03.01.06)

Type:	CONTROL

Description:	A network connection will be provided from the DAQ system to the slow controls interface of the solenoid's  power supply.

Provider:	Online DAQ and Computing Systems (6.03.01.08.02)

Details:	See details in the Data Acquisition and Computing Systems Interface Control Document.

Requirements:	F-DET-DS-COMP.6

References:

# Solenoid Magnet (DET-DS-MAG) to Electromagnetic Calorimetry Systems (DET-DS-ECAL)

I-DET-DS-INF-INT.012	Space Constraint

System 1:	Barrel EMCal Systems (6.03.01.04)

System 2:	Solenoid Magnet (6.03.01.06)

Type:	SPACE

Description:	The exterior radius of the barrel ECAL (and its support system) is limited by the interior bore of the solenoid magnet. Modifications to either must be coordinated.

Provider:	Detector Infrastructure and Utilities Integration (6.03.01.09.04)

Details:	See details in the Detector Infrastructure Interface Control Document.

Requirements:	P-DET-DS-ECAL.1, P-DET-DS-MAG.1

References:

# Solenoid Magnet (DET-DS-MAG) to Electronic Systems (DET-DS-ELEC)

I-DET-DS-ELEC.037	Bias Voltage

System 1:	Electronic Systems (6.03.01.07)

System 2:	Magnet Cryogenics (6.03.01.06)

Type:	ELEC

Description:	Bias voltage DC power will be provided from the electronics racks to support the magnet cryogenic systems.

Provider:	Electronic Systems (6.03.01.07)

Details:	See details in the Electronic Systems Interface Control Document.

Requirements:	P-DET-DS-MAG.6

References:

I-DET-DS-ELEC.038	High Voltage

System 1:	Electronic Systems (6.03.01.07)

System 2:	Magnet Cryogenics (6.03.01.06)

Type:	ELEC

Description:	High voltage DC power will be provided from the electronics racks to support the magnet instrumentation and control systems.

Provider:	Electronic Systems (6.03.01.07)

Details:	See details in the Electronic Systems Interface Control Document.

Requirements:	P-DET-DS-MAG.6

References:

I-DET-DS-ELEC.039	Low Voltage

System 1:	Electronic Systems (6.03.01.07)

System 2:	Magnet Cryogenics (6.03.01.06)

Type:	ELEC

Description:	Lower voltage DC power will be provided from the electronics racks to support the magnet cryogenic systems.

Provider:	Electronic Systems (6.03.01.07)

Details:	See details in the Electronic Systems Interface Control Document.

Requirements:	P-DET-DS-MAG.6

References:

I-DET-DS-ELEC.040	Bias Voltage

System 1:	Electronic Systems (6.03.01.07)

System 2:	Magnet Instrumentation and Control (6.03.01.06)

Type:	ELEC

Description:	Bias voltage DC power will be provided from the electronics racks to support the magnet instrumentation and control systems.

Provider:	Electronic Systems (6.03.01.07)

Details:	See details in the Electronic Systems Interface Control Document.

Requirements:	P-DET-DS-MAG.6

References:

I-DET-DS-ELEC.041	High Voltage

System 1:	Electronic Systems (6.03.01.07)

System 2:	Magnet Instrumentation and Control (6.03.01.06)

Type:	ELEC

Description:	High voltage DC power will be provided from the electronics racks to support the magnet power supply.

Provider:	Electronic Systems (6.03.01.07)

Details:	See details in the Electronic Systems Interface Control Document.

Requirements:	P-DET-DS-MAG.6

References:

I-DET-DS-ELEC.042	Low Voltage

System 1:	Electronic Systems (6.03.01.07)

System 2:	Magnet Instrumentation and Control (6.03.01.06)

Type:	ELEC

Description:	Low voltage DC power will be provided from the electronics racks to support the magnet instrumentation and control systems.

Provider:	Electronic Systems (6.03.01.07)

Details:	See details in the Electronic Systems Interface Control Document.

Requirements:	P-DET-DS-MAG.6

References:

I-DET-DS-ELEC.043	Bias Voltage

System 1:	Electronic Systems (6.03.01.07)

System 2:	Magnet Power Supply (6.03.01.06)

Type:	ELEC

Description:	Bias voltage DC power will be provided from the electronics racks to support the magnet power supply.

Provider:	Electronic Systems (6.03.01.07)

Details:	See details in the Electronic Systems Interface Control Document.

Requirements:	P-DET-DS-MAG.6

References:

I-DET-DS-ELEC.044	High Voltage

System 1:	Electronic Systems (6.03.01.07)

System 2:	Magnet Power Supply (6.03.01.06)

Type:	ELEC

Description:	High voltage DC power will be provided from the electronics racks to support electronics in the detector.

Provider:	Electronic Systems (6.03.01.07)

Details:	See details in the Electronic Systems Interface Control Document.

Requirements:	P-DET-DS-MAG.6

References:

I-DET-DS-ELEC.045	Low Voltage

System 1:	Electronic Systems (6.03.01.07)

System 2:	Magnet Power Supply (6.03.01.06)

Type:	ELEC

Description:	Low voltage DC power will be provided from the electronics racks to support the magnet power supply.

Provider:	Electronic Systems (6.03.01.07)

Details:	See details in the Electronic Systems Interface Control Document.

Requirements:	P-DET-DS-MAG.6

References:

# Solenoid Magnet (DET-DS-MAG) to Hadronic Calorimetry Systems (DET-DS-HCAL)

I-DET-DS-INF-INT.031	External Space Constraint

System 1:	Barrel HCal Systems (6.03.01.05)

System 2:	Solenoid Magnet (6.03.01.06)

Type:	SPACE

Description:	The exterior radius of the solenoid magnet is limited by the interior bore of the barrel HCAL and necessary support structures. Modifications to either must be coordinated.

Provider:	Detector Infrastructure and Utilities Integration (6.03.01.09.04)

Details:	See details in the Detector Infrastructure Interface Control Document.

Requirements:	P-DET-DS-HCAL.1, P-DET-DS-MAG.1

References:

I-DET-DS-INF-INT.034	Space Constraint

System 1:	Backward HCal Systems (6.03.01.05)

System 2:	Solenoid Magnet (6.03.01.06)

Type:	SPACE

Description:	The maximum backward location for the solenoid magnet is limited by the backward HCAL and the adjacent cabling pathway that provides services to the interior detectors. Modifications to either must be coordinated.

Provider:	Detector Infrastructure and Utilities Integration (6.03.01.09.04)

Details:	See details in the Detector Infrastructure Interface Control Document.

Requirements:	P-DET-DS-HCAL.1, P-DET-DS-MAG.1

References:

# Solenoid Magnet (DET-DS-MAG) to Detector Infrastructure (DET-DS-INF)

I-DET-DS-INF-BAR.015	External Support Structure

System 1:	Barrel Detector Structures (6.03.01.09.01)

System 2:	Solenoid Magnet (6.03.01.06)

Type:	STRUCT

Description:	Total weight of the solenoid and all embedded components must be supported.

Provider:	Barrel Detector Structures (6.03.01.09.01)

Details:	See details in the Detector Infrastructure Interface Control Document.

Requirements:	P-DET-DS-MAG.2

References:

I-DET-DS-INF-BAR.017	DIRC Readout Supports

System 1:	Solenoid Magnet (6.03.01.06)

System 2:	Barrel DIRC Particle ID Systems (6.03.01.03)

Type:	STRUCT

Description:	The DIRC prism box containing the photosensor readouts, will be supported by an external structure supporting the ECAL within the solenoid magnet.

Provider:	Barrel Detector Structures (6.03.01.09.01)

Details:	See details in the Detector Infrastructure Interface Control Document.

Requirements:	P-DET-DS-MAG.1, P-DET-DS-MAG.2, P-DET-DS-PID.4

References:

I-DET-DS-INF-INT.001	Magnetic Material

System 1:	Detector System (6.03.01)

System 2:	Solenoid Magnet (6.03.01.06)

Type:	ENV

Description:	No detectors or components constructed with magnetic material may be added to the detector without prior consultation with the magnet group.

Provider:	Detector Infrastructure and Utilities Integration (6.03.01.09.04)

Details:	See details in the Detector Infrastructure Interface Control Document.

Requirements:	P-DET-DS-MAG.13, P-DET-DS-MAG.14

References:

I-DET-DS-INF-INT.002	Magnetic Field Requirement

System 1:	Detector System (6.03.01)

System 2:	Solenoid Magnet (6.03.01.06)

Type:	PARAM

Description:	All subordinate detectors are dependent on the continuous operation of the solenoid and on the delivery of a consistent, stable magnetic field.

Provider:	Detector Infrastructure and Utilities Integration (6.03.01.09.04)

Details:	See details in the Detector Infrastructure Interface Control Document.

Requirements:	P-DET-DS-MAG.13

References:

I-DET-DS-INF-INT.012	Space Constraint

System 1:	Barrel EMCal Systems (6.03.01.04)

System 2:	Solenoid Magnet (6.03.01.06)

Type:	SPACE

Description:	The exterior radius of the barrel ECAL (and its support system) is limited by the interior bore of the solenoid magnet. Modifications to either must be coordinated.

Provider:	Detector Infrastructure and Utilities Integration (6.03.01.09.04)

Details:	See details in the Detector Infrastructure Interface Control Document.

Requirements:	P-DET-DS-ECAL.1, P-DET-DS-MAG.1

References:

I-DET-DS-INF-INT.031	External Space Constraint

System 1:	Barrel HCal Systems (6.03.01.05)

System 2:	Solenoid Magnet (6.03.01.06)

Type:	SPACE

Description:	The exterior radius of the solenoid magnet is limited by the interior bore of the barrel HCAL and necessary support structures. Modifications to either must be coordinated.

Provider:	Detector Infrastructure and Utilities Integration (6.03.01.09.04)

Details:	See details in the Detector Infrastructure Interface Control Document.

Requirements:	P-DET-DS-HCAL.1, P-DET-DS-MAG.1

References:

I-DET-DS-INF-INT.034	Space Constraint

System 1:	Backward HCal Systems (6.03.01.05)

System 2:	Solenoid Magnet (6.03.01.06)

Type:	SPACE

Description:	The maximum backward location for the solenoid magnet is limited by the backward HCAL and the adjacent cabling pathway that provides services to the interior detectors. Modifications to either must be coordinated.

Provider:	Detector Infrastructure and Utilities Integration (6.03.01.09.04)

Details:	See details in the Detector Infrastructure Interface Control Document.

Requirements:	P-DET-DS-HCAL.1, P-DET-DS-MAG.1

References:

I-DET-DS-INF-INT.039	Magnet Cooling

System 1:	Detector Infrastructure and Utilities Integration (6.03.01.09.04)

System 2:	Solenoid Magnet (6.03.01.06)

Type:	CRYO

Description:	The solenoid will require a continuous flow of cryogens to maintain temperature in both the experimental hall and the maintenance area.  Interface includes cryogenic source and warm return.

Provider:	Detector Infrastructure and Utilities Integration (6.03.01.09.04)

Details:	See details in the Detector Infrastructure Interface Control Document.

Requirements:	P-DET-DS-MAG.4

References:

I-DET-DS-INF-INT.076	Power Supply Cooling

System 1:	Detector Infrastructure and Utilities Integration (6.03.01.09.04)

System 2:	Solenoid Magnet (6.03.01.06)

Type:	COOL

Description:	The power supply, located in an adjacent electrical room, will need to be cooled with low conductivity water.

Provider:	Detector Infrastructure and Utilities Integration (6.03.01.09.04)

Details:	See details in the Detector Infrastructure Interface Control Document.

Requirements:	P-DET-DS-MAG.3

References:

I-DET-DS-INF-INT.077	Magnet Cooling

System 1:	Detector Infrastructure and Utilities Integration (6.03.01.09.04)

System 2:	Solenoid Magnet (6.03.01.06)

Type:	CRYO

Description:	The solenoid will require a continuous flow of cryogens to maintain temperature in both the experimental hall and the maintenance area.  Interface includes cryogenic source and warm return.

Provider:	Detector Infrastructure and Utilities Integration (6.03.01.09.04)

Details:	See details in the Detector Infrastructure Interface Control Document.

Requirements:	P-DET-DS-MAG.4, P-DET-DS-MAG-CCR.1, P-DET-DS-MAG-CCR.2, P-DET-DS-MAG-CCR.3

References:

I-DET-DS-INF-INT.078	Electrical Power

System 1:	Detector Infrastructure and Utilities Integration (6.03.01.09.04)

System 2:	Solenoid Magnet (6.03.01.06)

Type:	ELEC

Description:	Solenoid will be provided power via the power supply located in an adjacent room.

Provider:	Detector Infrastructure and Utilities Integration (6.03.01.09.04)

Details:	See details in the Detector Infrastructure Interface Control Document.

Requirements:	P-DET-DS-MAG.6

References:

I-DET-DS-INF-INT.079	Instrumentation Backup Power

System 1:	Detector Infrastructure and Utilities Integration (6.03.01.09.04)

System 2:	Solenoid Magnet (6.03.01.06)

Type:	ELEC

Description:	The instrumentation rack for the solenoid will require backup/UPS power adequate to safely shutdown the system in the event of a power failure.

Provider:	Detector Infrastructure and Utilities Integration (6.03.01.09.04)

Details:	See details in the Detector Infrastructure Interface Control Document.

Requirements:	P-DET-DS-MAG.6

References:

I-DET-DS-INF-INT.080	Fringe Field

System 1:	Detector Infrastructure and Utilities Integration (6.03.01.09.04)

System 2:	Solenoid Magnet (6.03.01.06)

Type:	ENV

Description:	A boundary must be established to identify the extent of the magnet's stray field during operations.

Provider:	Detector Infrastructure and Utilities Integration (6.03.01.09.04)

Details:	See details in the Detector Infrastructure Interface Control Document.

Requirements:	P-DET-DS-MAG.14

References:

I-DET-DS-INF-INT.081	Ice Management

System 1:	Detector Infrastructure and Utilities Integration (6.03.01.09.04)

System 2:	Solenoid Magnet (6.03.01.06)

Type:	ENV

Description:	An ice management system will be required to melt accumulated ice on the chimney. (Water should evaporate and will not require drainage)

Provider:	Detector Infrastructure and Utilities Integration (6.03.01.09.04)

Details:	See details in the Detector Infrastructure Interface Control Document.

Requirements:	P-DET-DS-MAG.5

References:

I-DET-DS-INF-INT.082	Cryogenic Connection Point

System 1:	Detector Infrastructure and Utilities Integration (6.03.01.09.04)

System 2:	Solenoid Magnet (6.03.01.06)

Type:	SPACE

Description:	The cryogenic connection must be as close as possible to the source.

Provider:	Detector Infrastructure and Utilities Integration (6.03.01.09.04)

Details:	See details in the Detector Infrastructure Interface Control Document.

Requirements:	P-DET-DS-MAG.4

References:

I-DET-DS-INF-INT.083	LCW Connection

System 1:	Detector Infrastructure and Utilities Integration (6.03.01.09.04)

System 2:	Solenoid Magnet (6.03.01.06)

Type:	SPACE

Description:	The LCW water cooled leads must be fully supported along their path between the electrical room and the magnet.

Provider:	Detector Infrastructure and Utilities Integration (6.03.01.09.04)

Details:	See details in the Detector Infrastructure Interface Control Document.

Requirements:	P-DET-DS-MAG.2

References:

I-DET-DS-INF-INT.097	Space Constraint

System 1:	Solenoid Magnet (6.03.01.06)

System 2:	Forward RICH Detector Systems (6.03.01.03)

Type:	SPACE

Description:	The maximum forward location for the solenoid is limited by the position of the Dual RICH detector and the cabling plenum (gap) between them. Modifications to either must be coordinated.

Provider:	Detector Infrastructure and Utilities Integration (6.03.01.09.04)

Details:	See details in the Detector Infrastructure Interface Control Document.

Requirements:	P-DET-DS-MAG.1, P-DET-DS-PID.5

References:

I-DET-DS-INF-INT.114	LHE Flow (Hall)

System 1:	Solenoid Magnet (6.03.01.06)

System 2:	IR Systems Integration (6.04.04)

Type:	CRYO

Description:	A supply and return of liquid helium must be provided to the experimental hall for cooling the solenoid magnet during operation.

Provider:	Detector Infrastructure and Utilities Integration (6.03.01.09.04)

Details:	See details in the Detector Infrastructure Interface Control Document.

Requirements:	P-DET-DS-MAG-CCR.1, P-DET-DS-MAG-CCR.2, P-DET-DS-MAG-CCR.3, P-DET-DS-MAG-CCR.4, P-DET-DS-INF-CRYO.1, P-DET-DS-INF-CRYO.2, P-DET-DS-INF-CRYO.3, P-DET-DS-INF-CRYO.4

References:

I-DET-DS-INF-INT.115	LHE Flow (Assembly Area)

System 1:	Solenoid Magnet (6.03.01.06)

System 2:	IR Systems Integration (6.04.04)

Type:	CRYO

Description:	A supply and return of liquid helium must be provided to the assembly hall for cooling the solenoid magnet during maintenance.

Provider:	Detector Infrastructure and Utilities Integration (6.03.01.09.04)

Details:	See details in the Detector Infrastructure Interface Control Document.

Requirements:	P-DET-DS-MAG-CCR.1, P-DET-DS-MAG-CCR.2, P-DET-DS-MAG-CCR.3, P-DET-DS-MAG-CCR.4, P-DET-DS-INF-CRYO.1, P-DET-DS-INF-CRYO.2, P-DET-DS-INF-CRYO.3, P-DET-DS-INF-CRYO.4

References:

# Solenoid Magnet (DET-DS-MAG) to Particle Identification Systems (DET-DS-PID)

I-DET-DS-INF-BAR.017	DIRC Readout Supports

System 1:	Solenoid Magnet (6.03.01.06)

System 2:	Barrel DIRC Particle ID Systems (6.03.01.03)

Type:	STRUCT

Description:	The DIRC prism box containing the photosensor readouts, will be supported by an external structure supporting the ECAL within the solenoid magnet.

Provider:	Barrel Detector Structures (6.03.01.09.01)

Details:	See details in the Detector Infrastructure Interface Control Document.

Requirements:	P-DET-DS-MAG.1, P-DET-DS-MAG.2, P-DET-DS-PID.4

References:

I-DET-DS-INF-INT.097	Space Constraint

System 1:	Solenoid Magnet (6.03.01.06)

System 2:	Forward RICH Detector Systems (6.03.01.03)

Type:	SPACE

Description:	The maximum forward location for the solenoid is limited by the position of the Dual RICH detector and the cabling plenum (gap) between them. Modifications to either must be coordinated.

Provider:	Detector Infrastructure and Utilities Integration (6.03.01.09.04)

Details:	See details in the Detector Infrastructure Interface Control Document.

Requirements:	P-DET-DS-MAG.1, P-DET-DS-PID.5

References:

# Applicable Documents

List applicable documents considered to be part of this document. Applicable documents shall be obtained from the EIC SharePoint Document Center.

# Detailed change Log

| Revision | Date | Change |
| --- | --- | --- |

# References

- doi:10.17226/25171., National Academies of Sciences, Engineering, and Medicine, "An Assessment of U.S.-Based Electron-Ion Collider Science", The National Academies Press, Washington, DC, 2018.

- EIC-ORG-PLN-010, Electron-Ion Collider L1 Requirements (Global Requirements Document)

- BNL-221006-2021-FORE, EIC Conceptual Design Report

- arXiv:2103.05419, Science Requirements and Detector Concepts for the Electron-Ion Collider: EIC Yellow Report

- EIC-SEG-PLN-016, Requirements Management Plan

- EIC-SEG-PLN-022, Systems Engineering Management Plan

- EIC-SEG-PLN-020, EIC Interface Management Plan