Electron-Ion Collider, Requirements, Specifications, and Interfaces

Interface Control Document: [###]

Particle Identification Systems (6.03.01.03) to Detector System (6.03.01)

Prepared by:

{{Sig_es_:signer1:signature}}	Date: {{Dte_es_:signer1:date}}

Name, Role

Electron-Ion Collider at Institution

Reviewed by:

{{Sig_es_:signer2:signature}}	Date: {{Dte_es_:signer2:date}}

Name, Role

Electron-Ion Collider at Institution

{{Sig_es_:signer3:signature}}	Date: {{Dte_es_:signer3:date}}

Name, Role

Electron-Ion Collider at Institution

{{Sig_es_:signer4:signature}}	Date: {{Dte_es_:signer4:date}}

Name, Role

Electron-Ion Collider at Institution

Approved by:

{{Sig_es_:signer5:signature}}	Date: {{Dte_es_:signer5:date}}

Name, Role

Electron-Ion Collider at Institution

REVISION HISTORY

| Revision # | Effective Date | Additional Reviewers | Summary of Change |
| --- | --- | --- | --- |
| 00 |  |  | Initial release. |

TABLE OF CONTENTS

1.	Purpose and Scope	5

2.	Introduction	5

2.1	Particle Identification Systems	5

2.2	Overview of Interface Relationships	5

2.3	Interface Responsibilities	6

2.4	Safety and Environmental Considerations	6

2.5	Design Considerations	6

2.6	Organization	6

3.	Interface summary	6

4.	Particle Identification Systems (DET-DS-PID) to Detector System (DET-DS)	28

5.	Particle Identification Systems (DET-DS-PID) to Data Acquisition and Computing Systems (DET-DS-COMP)	29

6.	Particle Identification Systems (DET-DS-PID) to Electromagnetic Calorimetry Systems (DET-DS-ECAL)	32

7.	Particle Identification Systems (DET-DS-PID) to Electronic Systems (DET-DS-ELEC)	35

8.	Particle Identification Systems (DET-DS-PID) to Hadronic Calorimetry Systems (DET-DS-HCAL)	38

9.	Particle Identification Systems (DET-DS-PID) to Detector Infrastructure (DET-DS-INF)	39

10.	Particle Identification Systems (DET-DS-PID) to Solenoid Magnet (DET-DS-MAG)	51

11.	Particle Identification Systems (DET-DS-PID) to Tracking Systems (DET-DS-TRAK)	52

12.	Applicable Documents	54

13.	Detailed change Log	54

14.	References	54

LIST OF ACRONYMS

| B0 | B0 is the name of the forward magnet just past the Hadron Endcap |  |
| --- | --- | --- |
| CDR | Conceptual Design Report |  |
| COTS | Commercial Off the Shelf |  |
| DAQ | Data Acquisition |  |
| DET | EIC Detector |  |
| DIRC | Detection of Internally Reflected Cherenkov Light |  |
| dRICH | Dual radiator ring imaging Cherenkov |  |
| DVCS | Deeply Virtual Compton Scattering |  |
| EIC | Electron-Ion Collider |  |
| EMCAL | Electro-magnetic Calorimeter |  |
| ESR | Electron Storage Ring |  |
| FCC | Federal Communications Commission |  |
| FDR | Final Design Review |  |
| GEM | Gas Electron Multiplier |  |
| HCAL | Hadronic Calorimeter |  |
| HSR | Hadron Storage Ring |  |
| HVAC | Heating/Ventilation/Air Conditioning |  |
| IP | Interaction Point |  |
| IR | Interaction Region |  |
| MPGD | Micro Pattern Gaseous Detectors |  |
| NAS | National Academy of Sciences |  |
| NEC | National Electric Code |  |
| NEMA | National Electrical Manufacturers Association |  |
| NFPA | National Fire Protection Association |  |
| NRTL | Nationally Recognized Testing Laboratory |  |
| PID | Particle Identification |  |
| PRR | Production Readiness Review |  |
| PWO | PbWO4 |  |
| RCS | Rapid Cycling Synchrotron |  |
| TBD | To Be Determined |  |
| UL | Underwriter Laboratories |  |
| WBS | Work Break Down Structure |  |

# Interface Control Document:

Particle Identification Systems (6.03.01.03) to Detector System (6.03.01)

# Purpose and Scope

This document records the agreed upon interfaces between the Particle Identification Systems and other sub-systems within the Detector System. These interfaces are based on the system requirements that are necessary to integrate these sub-systems as part of the Electron Ion Collider Project.

# Introduction

These interfaces describe the relationships between the Particle Identification Systems and the Detector System including (examples by inclusion) located in (areas by inclusion).

## Particle Identification Systems

Provide an introductory description of your sub-system and an overview of the things that it receives, provides, supports, constrains, or otherwise requires as part of its function --- specifically if these things are manifested as interfaces.

Detector System (DET-DS): Description of the TYPES of interfaces that exist between the ICD reference system and this sub-system, and a BRIEF overview of the roles they play.

Data Acquisition and Computing Systems (DET-DS-COMP): Description of the TYPES of interfaces that exist between the ICD reference system and this sub-system, and a BRIEF overview of the roles they play.

Electromagnetic Calorimetry Systems (DET-DS-ECAL): Description of the TYPES of interfaces that exist between the ICD reference system and this sub-system, and a BRIEF overview of the roles they play.

Electronic Systems (DET-DS-ELEC): Description of the TYPES of interfaces that exist between the ICD reference system and this sub-system, and a BRIEF overview of the roles they play.

Hadronic Calorimetry Systems (DET-DS-HCAL): Description of the TYPES of interfaces that exist between the ICD reference system and this sub-system, and a BRIEF overview of the roles they play.

Detector Infrastructure (DET-DS-INF): Description of the TYPES of interfaces that exist between the ICD reference system and this sub-system, and a BRIEF overview of the roles they play.

Solenoid Magnet (DET-DS-MAG): Description of the TYPES of interfaces that exist between the ICD reference system and this sub-system, and a BRIEF overview of the roles they play.

Tracking Systems (DET-DS-TRAK): Description of the TYPES of interfaces that exist between the ICD reference system and this sub-system, and a BRIEF overview of the roles they play.

## Overview of Interface Relationships

The following are the sub-systems within the Detector System that have interfaces connected to the Particle Identification Systems.

## Interface Responsibilities

The various ownerships of responsibilities may be listed here. However, this section might also refer to the Provider section of the interface detail if all pertinent responsibilities are listed there. If this is the case, simply enter: “See the Provider section within the interface details for information about the responsibilities associated with individual interfaces.”

## Safety and Environmental Considerations

General safety and environmental considerations are identified here. Any considerations that are specific to the individual sub-system, will be addressed in external safety documents and are referenced here.

### Considerations for all systems

Add general safety and environmental considerations here.

### Special considerations and references

Add references here as necessary.

## Design Considerations

Notes here about general design considerations.

## Organization

The interfaces in this document are grouped by sub-system as identified in the sub-system identifier. All interfaces between the Particle Identification Systems and a related sub-system will be included in a single section. Entries within that section will be sorted by the interface identifier or WBS number, as appropriate.

# Interface summary

This tables in this section provide a list of all interfaces that are defined within this document. The interfaces are grouped by sub-system/WBS. Each table has the following fields:

Section: the section number in this document where the interface is addressed.

WBS (1): the WBS number within the ICD reference system, unless the reference system is the owner.

WBS (2): the WBS number for the connected system.

ID: the interface identifier as defined in the Interface Requirement Document (IRD).

Name: the name of the interface as defined in the IRD.

Description: the description of the interface as defined in the IRD.

Owner: identifies the WBS number of the entity that is responsible for ensuring the interface is provided and that it meets validation/verification requirements.

Particle Identification Systems to Detector System

| Section | WBS1 | WBS2 | ID | Name | Description | Owner |
| --- | --- | --- | --- | --- | --- | --- |
| 4 |  | 6.03.01 |  |  | Detector System |  |
| 4.1 | 6.03.01.03 | 6.03.01 | I-DET-DS-INF-INT.003 | Cabling/Services Conduit | The DIRC structure will provide integrated pathways for cabling and services to be delivered from the interior detectors (pf RICH/mRICH and silicon trackers) to the exterior infrastructure. | 6.03.01.09.04 |

Particle Identification Systems to Data Acquisition and Computing Systems

| Section | WBS1 | WBS2 | ID | Name | Description | Owner |
| --- | --- | --- | --- | --- | --- | --- |
| 5 |  | 6.03.01.08 |  |  | Data Acquisition and Computing Systems |  |
| 5.1 | 6.03.01.03 | 6.03.01.08 | I-DET-DS-COMP-ONLINE.045 | Data Transfer and Control Interface | A fiber connection will be provided from the DAQ system to the barrel PID detector's readout board to perform configuration, control, and data acquisition. | 6.03.01.08.02 |
| 5.2 | 6.03.01.03 | 6.03.01.08 | I-DET-DS-COMP-ONLINE.046 | Slow Controls | A network connection will be provided from the DAQ system to the barrel PID detector's slow controls interface. | 6.03.01.08.02 |
| 5.3 | 6.03.01.03 | 6.03.01.08 | I-DET-DS-COMP-ONLINE.047 | Timing Interface | A fiber connection will be provided from the DAQ system to the barrel PID detector's readout board for timing synchronization. | 6.03.01.08.02 |
| 5.4 | 6.03.01.03 | 6.03.01.08 | I-DET-DS-COMP-ONLINE.048 | Data Acquisition | Signal cables will run from the DIRC electronics to the DAQ system. | 6.03.01.08.02 |
| 5.5 | 6.03.01.03 | 6.03.01.08 | I-DET-DS-COMP-ONLINE.049 | Slow Control Interface | Signal cables from environmental sensors will run from the DIRC electronics to the DAQ system to provide detector shutdown/protection system. | 6.03.01.08.02 |
| 5.6 | 6.03.01.03 | 6.03.01.08 | I-DET-DS-COMP-ONLINE.050 | Data Acquisition | Signal cables will run from the DIRC electronics to the DAQ system. | 6.03.01.08.02 |
| 5.7 | 6.03.01.03 | 6.03.01.08 | I-DET-DS-COMP-ONLINE.051 | Slow Control Interface | Signal cables from environmental sensors will run from the DIRC electronics to the DAQ system to provide detector shutdown/protection system. | 6.03.01.08.02 |
| 5.8 | 6.03.01.03 | 6.03.01.08 | I-DET-DS-COMP-ONLINE.052 | Data Acquisition | Signal cables will run from the pfRICH/mRICH electronics to the DAQ system. | 6.03.01.08.02 |
| 5.9 | 6.03.01.03 | 6.03.01.08 | I-DET-DS-COMP-ONLINE.053 | Slow Control Interface | Signal cables from environmental sensors will run from the pfRICH/mRICH electronics to the DAQ system to provide a detector shutdown/protection system. | 6.03.01.08.02 |
| 5.10 | 6.03.01.03 | 6.03.01.08 | I-DET-DS-COMP-ONLINE.054 | Data Acquisition | Signal cables will run from the dRICH electronics to the DAQ system. | 6.03.01.08.02 |
| 5.11 | 6.03.01.03 | 6.03.01.08 | I-DET-DS-COMP-ONLINE.055 | Slow Control Interface | Signal cables from environmental sensors will run from the cold photosensors to the DAQ system to provide a detector shutdown/protection system. | 6.03.01.08.02 |
| 5.12 | 6.03.01.03 | 6.03.01.08 | I-DET-DS-PID-BAR-DIRC.001 | Light Pulser System | The light pulser system will require synchronization using a clock that will be provided by data acquisition. | 6.03.01.03 |
| 5.13 | 6.03.01.03 | 6.03.01.08 | I-DET-DS-PID-BCK-RICH.001 | Light Pulser System | The light pulser system will require synchronization using a clock that will be provided by data acquisition. | 6.03.01.03 |
| 5.14 | 6.03.01.03 | 6.03.01.08 | I-DET-DS-PID-FWD-RICH.001 | Light Pulser System | The light pulser system will require synchronization using a clock that will be provided by data acquisition. | 6.03.01.03 |

Particle Identification Systems to Electromagnetic Calorimetry Systems

| Section | WBS1 | WBS2 | ID | Name | Description | Owner |
| --- | --- | --- | --- | --- | --- | --- |
| 6 |  | 6.03.01.04 |  |  | Electromagnetic Calorimetry Systems |  |
| 6.1 | 6.03.01.03 | 6.03.01.04 | I-DET-DS-INF-BAR.003 | Support Structure | The DIRC bar boxes will be supported by a frame inside the barrel Electromagnetic Calorimeter, that allows the boxes to be extracted using a system of rollers. | 6.03.01.09.01 |
| 6.2 | 6.03.01.03 | 6.03.01.04 | I-DET-DS-INF-BAR.005 | Weight Transfer | The weight of the backward ECAL will be transferred to the DIRC detector support and must be accommodated by all subsequent support systems. | 6.03.01.09.01 |
| 6.3 | 6.03.01.03 | 6.03.01.04 | I-DET-DS-INF-BAR.006 | EMCal Support System | The DIRC support system will provide support for the backward electromagnetic calorimeter. | 6.03.01.09.01 |
| 6.4 | 6.03.01.03 | 6.03.01.04 | I-DET-DS-INF-INT.013 | Space Constraint | The backward position and shape of the barrel EMCAL is limited by the size and shape of the DIRC readout supports. Changes to the size or position of either must be coordinated with the other. | 6.03.01.09.04 |
| 6.5 | 6.03.01.03 | 6.03.01.04 | I-DET-DS-INF-INT.014 | Space Constraint | The maximum size of the DIRC is limited to the interior bore of the barrel ECAL (and its support structures). Modifications to either must be coordinated. | 6.03.01.09.04 |
| 6.6 | 6.03.01.03 | 6.03.01.04 | I-DET-DS-INF-INT.015 | Space Constraint | The forward position and shape of the barrel ECAL is limited by the backward face of the Dual RICH detector and the adjacent cabling pathway that provides services to the interior detectors. Modifications to either must be coordinated. | 6.03.01.09.04 |
| 6.7 | 6.03.01.03 | 6.03.01.04 | I-DET-DS-INF-INT.017 | Space Constraint | The maximum backward location for the DIRC is limited by the position of the backward electromagnetic calorimeter. Modifications to either must be coordinated. | 6.03.01.09.04 |
| 6.8 | 6.03.01.03 | 6.03.01.04 | I-DET-DS-INF-INT.018 | Space Constraint | The exterior radius of the backward ECAL is limited by the interior bore of the DIRC support system. Modifications to either must be coordinated. | 6.03.01.09.04 |
| 6.9 | 6.03.01.03 | 6.03.01.04 | I-DET-DS-INF-INT.019 | Heat Mitigation | The backward ECAL must mitigated the heat generated by the backward RICH detector. | 6.03.01.09.04 |
| 6.10 | 6.03.01.03 | 6.03.01.04 | I-DET-DS-INF-INT.020 | Space Constraint | The position of the backward ECAL in the forward direction is limited by the backward face of the pfRICH/mRICH detector. Modifications to either must be coordinated. | 6.03.01.09.04 |
| 6.11 | 6.03.01.03 | 6.03.01.04 | I-DET-DS-INF-INT.025 | Space Constraint | The maximum forward location for the dRICH is limited by the forward ECAL and the adjacent cabling pathway that provides services to the interior detectors. Modifications to either must be coordinated. | 6.03.01.09.04 |

Particle Identification Systems to Electronic Systems

| Section | WBS1 | WBS2 | ID | Name | Description | Owner |
| --- | --- | --- | --- | --- | --- | --- |
| 7 |  | 6.03.01.07 |  |  | Electronic Systems |  |
| 7.1 | 6.03.01.03 | 6.03.01.07 | I-DET-DS-ELEC.046 | Bias Voltage | Bias voltage DC power will be provided from the electronics racks to support electronics the silicon photomultipliers. | 6.03.01.07 |
| 7.2 | 6.03.01.03 | 6.03.01.07 | I-DET-DS-ELEC.047 | High Voltage | High voltage DC power will be provided from the electronics racks to support silicon sensors and gas detectors. | 6.03.01.07 |
| 7.3 | 6.03.01.03 | 6.03.01.07 | I-DET-DS-ELEC.048 | Low Voltage | Low voltage DC power will be provided from the electronics racks to support electronics in the detector. | 6.03.01.07 |
| 7.4 | 6.03.01.03 | 6.03.01.07 | I-DET-DS-ELEC.049 | High Voltage | High voltage power will be delivered from the power supplies on the carriage to the photosensors in the DIRC. | 6.03.01.07 |
| 7.5 | 6.03.01.03 | 6.03.01.07 | I-DET-DS-ELEC.050 | Low Voltage | Low voltage power will be delivered from the power supplies on the carriage to the DIRC electronics. | 6.03.01.07 |
| 7.6 | 6.03.01.03 | 6.03.01.07 | I-DET-DS-ELEC.051 | High Voltage | High voltage power will be delivered from the power supplies on the carriage to the photosensors in the DIRC. | 6.03.01.07 |
| 7.7 | 6.03.01.03 | 6.03.01.07 | I-DET-DS-ELEC.052 | Low Voltage | Low voltage power will be delivered from the power supplies on the carriage to the DIRC electronics. | 6.03.01.07 |
| 7.8 | 6.03.01.03 | 6.03.01.07 | I-DET-DS-ELEC.053 | High Voltage | High voltage power will be delivered from the power supplies on the carriage to the photosensors in the pfRICH/mRICH. | 6.03.01.07 |
| 7.9 | 6.03.01.03 | 6.03.01.07 | I-DET-DS-ELEC.054 | Low Voltage | Low voltage power will be delivered from the power supplies on the carriage to the pfRICH/mRICH electronics. | 6.03.01.07 |
| 7.10 | 6.03.01.03 | 6.03.01.07 | I-DET-DS-ELEC.055 | Bias Voltage | Bias voltage DC power will be delivered from the power supplies on the carriage to the photosensors. | 6.03.01.07 |
| 7.11 | 6.03.01.03 | 6.03.01.07 | I-DET-DS-ELEC.056 | Low Voltage | Low voltage power will be delivered from the power supplies on the carriage to the dRICH electronics. | 6.03.01.07 |

Particle Identification Systems to Hadronic Calorimetry Systems

| Section | WBS1 | WBS2 | ID | Name | Description | Owner |
| --- | --- | --- | --- | --- | --- | --- |
| 8 |  | 6.03.01.05 |  |  | Hadronic Calorimetry Systems |  |
| 8.1 | 6.03.01.03 | 6.03.01.05 | I-DET-DS-INF-BAR.012 | Dual RICH Support System | The Dual RICH will be supported by a structural system within the barrel hadron calorimeter. | 6.03.01.09.01 |
| 8.2 | 6.03.01.03 | 6.03.01.05 | I-DET-DS-INF-INT.032 | Space Constraint | The size of the dRICH is limited by the interior bore of the barrel HCAL, and must provide adequate space for cables and services to itself and the interior detectors. Modifications to either must be coordinated. | 6.03.01.09.04 |
| 8.3 | 6.03.01.03 | 6.03.01.05 | I-DET-DS-INF-INT.035 | Space Constraint | The maximum backward location for the DIRC is limited by the backward HCAL and the adjacent cabling pathway that provides services to the interior detectors. Modifications to either must be coordinated. | 6.03.01.09.04 |

Particle Identification Systems to Detector Infrastructure

| Section | WBS1 | WBS2 | ID | Name | Description | Owner |
| --- | --- | --- | --- | --- | --- | --- |
| 9 |  | 6.03.01.09 |  |  | Detector Infrastructure |  |
| 9.1 | 6.03.01.03 | 6.03.01.09 | I-DET-DS-INF-BAR.003 | Support Structure | The DIRC bar boxes will be supported by a frame inside the barrel Electromagnetic Calorimeter, that allows the boxes to be extracted using a system of rollers. | 6.03.01.09.01 |
| 9.2 | 6.03.01.03 | 6.03.01.09 | I-DET-DS-INF-BAR.005 | Weight Transfer | The weight of the backward ECAL will be transferred to the DIRC detector support and must be accommodated by all subsequent support systems. | 6.03.01.09.01 |
| 9.3 | 6.03.01.03 | 6.03.01.09 | I-DET-DS-INF-BAR.006 | EMCal Support System | The DIRC support system will provide support for the backward electromagnetic calorimeter. | 6.03.01.09.01 |
| 9.4 | 6.03.01.03 | 6.03.01.09 | I-DET-DS-INF-BAR.012 | Dual RICH Support System | The Dual RICH will be supported by a structural system within the barrel hadron calorimeter. | 6.03.01.09.01 |
| 9.5 | 6.03.01.03 | 6.03.01.09 | I-DET-DS-INF-BAR.017 | DIRC Readout Supports | The DIRC prism box containing the photosensor readouts, will be supported by an external structure supporting the ECAL within the solenoid magnet. | 6.03.01.09.01 |
| 9.6 | 6.03.01.03 | 6.03.01.09 | I-DET-DS-INF-BAR.018 | Structural Support | The pfRICH/mRICH will be supported by the DIRC bar box support system. | 6.03.01.09.01 |
| 9.7 | 6.03.01.03 | 6.03.01.09 | I-DET-DS-INF-BAR.019 | Barrel TOF Support | The barrel Time of Flight will be supported by the DIRC bar box assembly. | 6.03.01.09.01 |
| 9.8 | 6.03.01.03 | 6.03.01.09 | I-DET-DS-INF-BAR.020 | pfRICH/mRICH Support System | The DIRC support system will provide support for the upstream PID detector system. | 6.03.01.09.01 |
| 9.9 | 6.03.01.03 | 6.03.01.09 | I-DET-DS-INF-BAR.021 | Silicon Tracker Support System | The DIRC support system will provide support for the silicon trackers in the detector barrel. | 6.03.01.09.01 |
| 9.10 | 6.03.01.03 | 6.03.01.09 | I-DET-DS-INF-BAR.022 | Weight Distributed | The weight of the barrel tracking systems will be transferred to the DIRC support system. | 6.03.01.09.01 |
| 9.11 | 6.03.01.03 | 6.03.01.09 | I-DET-DS-INF-FWD.011 | Space Constraint | The interior radius of the dRICH is governed by the size of the beamline and its associated flanges and support system. | 6.03.01.09.03 |
| 9.12 | 6.03.01.03 | 6.03.01.09 | I-DET-DS-INF-FWD.012 | Space Constraint | The maximum forward location for the tracking system is limited by the position of the AC LGAD Time of Flight Detector. Modifications to either must be coordinated. | 6.03.01.09.03 |
| 9.13 | 6.03.01.03 | 6.03.01.09 | I-DET-DS-INF-INT.003 | Cabling/Services Conduit | The DIRC structure will provide integrated pathways for cabling and services to be delivered from the interior detectors (pf RICH/mRICH and silicon trackers) to the exterior infrastructure. | 6.03.01.09.04 |
| 9.14 | 6.03.01.03 | 6.03.01.09 | I-DET-DS-INF-INT.013 | Space Constraint | The backward position and shape of the barrel EMCAL is limited by the size and shape of the DIRC readout supports. Changes to the size or position of either must be coordinated with the other. | 6.03.01.09.04 |
| 9.15 | 6.03.01.03 | 6.03.01.09 | I-DET-DS-INF-INT.014 | Space Constraint | The maximum size of the DIRC is limited to the interior bore of the barrel ECAL (and its support structures). Modifications to either must be coordinated. | 6.03.01.09.04 |
| 9.16 | 6.03.01.03 | 6.03.01.09 | I-DET-DS-INF-INT.015 | Space Constraint | The forward position and shape of the barrel ECAL is limited by the backward face of the Dual RICH detector and the adjacent cabling pathway that provides services to the interior detectors. Modifications to either must be coordinated. | 6.03.01.09.04 |
| 9.17 | 6.03.01.03 | 6.03.01.09 | I-DET-DS-INF-INT.017 | Space Constraint | The maximum backward location for the DIRC is limited by the position of the backward electromagnetic calorimeter. Modifications to either must be coordinated. | 6.03.01.09.04 |
| 9.18 | 6.03.01.03 | 6.03.01.09 | I-DET-DS-INF-INT.018 | Space Constraint | The exterior radius of the backward ECAL is limited by the interior bore of the DIRC support system. Modifications to either must be coordinated. | 6.03.01.09.04 |
| 9.19 | 6.03.01.03 | 6.03.01.09 | I-DET-DS-INF-INT.019 | Heat Mitigation | The backward ECAL must mitigated the heat generated by the backward RICH detector. | 6.03.01.09.04 |
| 9.20 | 6.03.01.03 | 6.03.01.09 | I-DET-DS-INF-INT.020 | Space Constraint | The position of the backward ECAL in the forward direction is limited by the backward face of the pfRICH/mRICH detector. Modifications to either must be coordinated. | 6.03.01.09.04 |
| 9.21 | 6.03.01.03 | 6.03.01.09 | I-DET-DS-INF-INT.025 | Space Constraint | The maximum forward location for the dRICH is limited by the forward ECAL and the adjacent cabling pathway that provides services to the interior detectors. Modifications to either must be coordinated. | 6.03.01.09.04 |
| 9.22 | 6.03.01.03 | 6.03.01.09 | I-DET-DS-INF-INT.032 | Space Constraint | The size of the dRICH is limited by the interior bore of the barrel HCAL, and must provide adequate space for cables and services to itself and the interior detectors. Modifications to either must be coordinated. | 6.03.01.09.04 |
| 9.23 | 6.03.01.03 | 6.03.01.09 | I-DET-DS-INF-INT.035 | Space Constraint | The maximum backward location for the DIRC is limited by the backward HCAL and the adjacent cabling pathway that provides services to the interior detectors. Modifications to either must be coordinated. | 6.03.01.09.04 |
| 9.24 | 6.03.01.03 | 6.03.01.09 | I-DET-DS-INF-INT.084 | Process Cooling | The heat generated by the photo-multiplier tubes and associated electronics must be removed using a process cooling system. | 6.03.01.09.04 |
| 9.25 | 6.03.01.03 | 6.03.01.09 | I-DET-DS-INF-INT.085 | Process Cooling | The heat generated by the detector and its electronics must be removed using a process cooling system. | 6.03.01.09.04 |
| 9.26 | 6.03.01.03 | 6.03.01.09 | I-DET-DS-INF-INT.086 | Process Cooling | The heat generated by the photomultipliers and associated electronics must be removed using a process cooling system. | 6.03.01.09.04 |
| 9.27 | 6.03.01.03 | 6.03.01.09 | I-DET-DS-INF-INT.087 | Dry Nitrogen | A continuous flow of dry nitrogen will be required to protect the aerogel radiator. | 6.03.01.09.04 |
| 9.28 | 6.03.01.03 | 6.03.01.09 | I-DET-DS-INF-INT.088 | Photosensor Cooling System | A fluid cooling system will be required to keep the operating temperature of the photosensors at -30 degrees Celsius. | 6.03.01.09.04 |
| 9.29 | 6.03.01.03 | 6.03.01.09 | I-DET-DS-INF-INT.089 | Gas Recovery System | A recovery system will need to be included for the radiator gas used by the dRICH to prevent accidental release. | 6.03.01.09.04 |
| 9.30 | 6.03.01.03 | 6.03.01.09 | I-DET-DS-INF-INT.090 | Insulation | Sufficient insulation will need to be added around all coolant lines to prevent condensation and ensure that ice doesn't form. | 6.03.01.09.04 |
| 9.31 | 6.03.01.03 | 6.03.01.09 | I-DET-DS-INF-INT.091 | Dry Nitrogen for Aerogel | A continuous flow of dry nitrogen will be required to protect the aerogel radiator. | 6.03.01.09.04 |
| 9.32 | 6.03.01.03 | 6.03.01.09 | I-DET-DS-INF-INT.092 | Dry Nitrogen for Photosensors | This gas will be required to create a protective atmosphere for the cold photosensors and to prevent condensation and freezing. | 6.03.01.09.04 |
| 9.33 | 6.03.01.03 | 6.03.01.09 | I-DET-DS-INF-INT.093 | Radiator Gas | A continuous recirculating flow of radiator gas will be required. | 6.03.01.09.04 |
| 9.34 | 6.03.01.03 | 6.03.01.09 | I-DET-DS-INF-INT.097 | Space Constraint | The maximum forward location for the solenoid is limited by the position of the Dual RICH detector and the cabling plenum (gap) between them. Modifications to either must be coordinated. | 6.03.01.09.04 |
| 9.35 | 6.03.01.03 | 6.03.01.09 | I-DET-DS-INF-INT.098 | Space Constraint | The maximum diameter of the barrel Time of Flight detector is limited by the interior bore of the DIRC detector. Modifications to either must be coordinated. | 6.03.01.09.04 |
| 9.36 | 6.03.01.03 | 6.03.01.09 | I-DET-DS-INF-INT.099 | Space Constraint | The exterior radius of the pfRICH/mRICH is limited by the interior bore of the DIRC support system. Modifications to either must be coordinated. | 6.03.01.09.04 |
| 9.37 | 6.03.01.03 | 6.03.01.09 | I-DET-DS-INF-INT.100 | Space Constraint | The maximum forward location for the DIRC is limited by the position of the dRICH detector and the cabling plenum between them. Modifications to either must be coordinated. | 6.03.01.09.04 |
| 9.38 | 6.03.01.03 | 6.03.01.09 | I-DET-DS-INF-INT.101 | Service Pathway | Conduits must be provided within the DIRC support system that are adequate to deliver services (power, signal, cooling) to the barrel tracking detectors. | 6.03.01.09.04 |
| 9.39 | 6.03.01.03 | 6.03.01.09 | I-DET-DS-INF-INT.102 | Space Constraint | The exterior radius of the silicon trackers is limited by the interior bore of the DIRC support system. Modifications to either must be coordinated. | 6.03.01.09.04 |
| 9.40 | 6.03.01.03 | 6.03.01.09 | I-DET-DS-INF-INT.103 | Space Constraint | The maximum forward location for the DIRC is limited by the position of the silicon trackers. | 6.03.01.09.04 |
| 9.41 | 6.03.01.03 | 6.03.01.09 | I-DET-DS-INF-INT.104 | Space Constraint | The maximum backward location for the barrel Time of Flight detector is limited by the position of the pfRICH/mRICH detector and the adjacent cabling pathway. Modifications to either must be coordinated. | 6.03.01.09.04 |
| 9.42 | 6.03.01.03 | 6.03.01.09 | I-DET-DS-INF-INT.105 | Space Constraint | The maximum forward location for the barrel Time of Flight detector is limited by the position of the forward Time of Flight detector and the adjacent cabling pathway. Modifications to either must be coordinated. | 6.03.01.09.04 |
| 9.43 | 6.03.01.03 | 6.03.01.09 | I-DET-DS-INF-INT.106 | Space Constraint | The maximum size of the silicon trackers is limited by the interior radius of the barrel Time of Flight detector. Modifications to either must be coordinated. | 6.03.01.09.04 |
| 9.44 | 6.03.01.03 | 6.03.01.09 | I-DET-DS-INF-INT.107 | Space Constraint | The maximum backward location for the tracking system is limited by the position of the pfRICH/mRICH. Modifications to either must be coordinated. | 6.03.01.09.04 |
| 9.45 | 6.03.01.03 | 6.03.01.09 | I-DET-DS-INF-INT.108 | Space Constraint | The interior radius of the pfRICH is governed by the size of the beamline and its associated flanges and support system. | 6.03.01.09.04 |
| 9.46 | 6.03.01.03 | 6.03.01.09 | I-DET-DS-INF-INT.137 | Space Constraint | Rack space on the platform must be provided for a small laser system to monitor the response of the photsensors and allow adjustment of timing correlations within the detector. | 6.03.01.09.04 |
| 9.47 | 6.03.01.03 | 6.03.01.09 | I-DET-DS-INF-INT.138 | Space Constraint | Rack space on the platform must be provided for a small laser system to monitor the response of the photsensors and allow adjustment of timing correlations within the detector. | 6.03.01.09.04 |
| 9.48 | 6.03.01.03 | 6.03.01.09 | I-DET-DS-INF-INT.139 | Space Constraint | Rack space on the platform must be provided for a small laser system to monitor the response of the photsensors and allow adjustment of timing correlations within the detector. | 6.03.01.09.04 |

Particle Identification Systems to Solenoid Magnet

| Section | WBS1 | WBS2 | ID | Name | Description | Owner |
| --- | --- | --- | --- | --- | --- | --- |
| 10 |  | 6.03.01.06 |  |  | Solenoid Magnet |  |
| 10.1 | 6.03.01.03 | 6.03.01.06 | I-DET-DS-INF-BAR.017 | DIRC Readout Supports | The DIRC prism box containing the photosensor readouts, will be supported by an external structure supporting the ECAL within the solenoid magnet. | 6.03.01.09.01 |
| 10.2 | 6.03.01.03 | 6.03.01.06 | I-DET-DS-INF-INT.097 | Space Constraint | The maximum forward location for the solenoid is limited by the position of the Dual RICH detector and the cabling plenum (gap) between them. Modifications to either must be coordinated. | 6.03.01.09.04 |

Particle Identification Systems to Tracking Systems

| Section | WBS1 | WBS2 | ID | Name | Description | Owner |
| --- | --- | --- | --- | --- | --- | --- |
| 11 |  | 6.03.01.02 |  |  | Tracking Systems |  |
| 11.1 | 6.03.01.03 | 6.03.01.02 | I-DET-DS-INF-BAR.021 | Silicon Tracker Support System | The DIRC support system will provide support for the silicon trackers in the detector barrel. | 6.03.01.09.01 |
| 11.2 | 6.03.01.03 | 6.03.01.02 | I-DET-DS-INF-BAR.022 | Weight Distributed | The weight of the barrel tracking systems will be transferred to the DIRC support system. | 6.03.01.09.01 |
| 11.3 | 6.03.01.03 | 6.03.01.02 | I-DET-DS-INF-FWD.012 | Space Constraint | The maximum forward location for the tracking system is limited by the position of the AC LGAD Time of Flight Detector. Modifications to either must be coordinated. | 6.03.01.09.03 |
| 11.4 | 6.03.01.03 | 6.03.01.02 | I-DET-DS-INF-INT.101 | Service Pathway | Conduits must be provided within the DIRC support system that are adequate to deliver services (power, signal, cooling) to the barrel tracking detectors. | 6.03.01.09.04 |
| 11.5 | 6.03.01.03 | 6.03.01.02 | I-DET-DS-INF-INT.102 | Space Constraint | The exterior radius of the silicon trackers is limited by the interior bore of the DIRC support system. Modifications to either must be coordinated. | 6.03.01.09.04 |
| 11.6 | 6.03.01.03 | 6.03.01.02 | I-DET-DS-INF-INT.103 | Space Constraint | The maximum forward location for the DIRC is limited by the position of the silicon trackers. | 6.03.01.09.04 |
| 11.7 | 6.03.01.03 | 6.03.01.02 | I-DET-DS-INF-INT.106 | Space Constraint | The maximum size of the silicon trackers is limited by the interior radius of the barrel Time of Flight detector. Modifications to either must be coordinated. | 6.03.01.09.04 |
| 11.8 | 6.03.01.03 | 6.03.01.02 | I-DET-DS-INF-INT.107 | Space Constraint | The maximum backward location for the tracking system is limited by the position of the pfRICH/mRICH. Modifications to either must be coordinated. | 6.03.01.09.04 |

# Particle Identification Systems (DET-DS-PID) to Detector System (DET-DS)

I-DET-DS-INF-INT.003	Cabling/Services Conduit

System 1:	Detector System (6.03.01)

System 2:	Barrel DIRC Particle ID Systems (6.03.01.03)

Type:	SPACE

Description:	The DIRC structure will provide integrated pathways for cabling and services to be delivered from the interior detectors (pf RICH/mRICH and silicon trackers) to the exterior infrastructure.

Provider:	Detector Infrastructure and Utilities Integration (6.03.01.09.04)

Details:	See details in the Detector Infrastructure Interface Control Document.

Requirements:	P-DET-DS-PID.4, P-DET-DS-PID.5

References:

# Particle Identification Systems (DET-DS-PID) to Data Acquisition and Computing Systems (DET-DS-COMP)

I-DET-DS-COMP-ONLINE.045	Data Transfer and Control Interface

System 1:	Online DAQ and Computing Systems (6.03.01.08.02)

System 2:	Barrel Particle ID Systems (6.03.01.03)

Type:	DATA

Description:	A fiber connection will be provided from the DAQ system to the barrel PID detector's readout board to perform configuration, control, and data acquisition.

Provider:	Online DAQ and Computing Systems (6.03.01.08.02)

Details:	See details in the Data Acquisition and Computing Systems Interface Control Document.

Requirements:	F-DET-DS-COMP.6

References:

I-DET-DS-COMP-ONLINE.046	Slow Controls

System 1:	Online DAQ and Computing Systems (6.03.01.08.02)

System 2:	Barrel Particle ID Systems (6.03.01.03)

Type:	CONTROL

Description:	A network connection will be provided from the DAQ system to the barrel PID detector's slow controls interface.

Provider:	Online DAQ and Computing Systems (6.03.01.08.02)

Details:	See details in the Data Acquisition and Computing Systems Interface Control Document.

Requirements:	F-DET-DS-COMP-SC.1

References:

I-DET-DS-COMP-ONLINE.047	Timing Interface

System 1:	Online DAQ and Computing Systems (6.03.01.08.02)

System 2:	Barrel Particle ID Systems (6.03.01.03)

Type:	DATA

Description:	A fiber connection will be provided from the DAQ system to the barrel PID detector's readout board for timing synchronization.

Provider:	Online DAQ and Computing Systems (6.03.01.08.02)

Details:	See details in the Data Acquisition and Computing Systems Interface Control Document.

Requirements:	F-DET-DS-COMP.11

References:

I-DET-DS-COMP-ONLINE.048	Data Acquisition

System 1:	Online DAQ and Computing Systems (6.03.01.08.02)

System 2:	Barrel DIRC Particle ID Systems (6.03.01.03)

Type:	DATA

Description:	Signal cables will run from the DIRC electronics to the DAQ system.

Provider:	Online DAQ and Computing Systems (6.03.01.08.02)

Details:	See details in the Data Acquisition and Computing Systems Interface Control Document.

Requirements:	F-DET-DS-COMP.6

References:

I-DET-DS-COMP-ONLINE.049	Slow Control Interface

System 1:	Online DAQ and Computing Systems (6.03.01.08.02)

System 2:	Barrel DIRC Particle ID Systems (6.03.01.03)

Type:	CONTROL

Description:	Signal cables from environmental sensors will run from the DIRC electronics to the DAQ system to provide detector  shutdown/protection system.

Provider:	Online DAQ and Computing Systems (6.03.01.08.02)

Details:	See details in the Data Acquisition and Computing Systems Interface Control Document.

Requirements:	F-DET-DS-COMP-SC.1

References:

I-DET-DS-COMP-ONLINE.050	Data Acquisition

System 1:	Online DAQ and Computing Systems (6.03.01.08.02)

System 2:	Barrel TOF Particle ID Systems (6.03.01.03)

Type:	DATA

Description:	Signal cables will run from the DIRC electronics to the DAQ system.

Provider:	Online DAQ and Computing Systems (6.03.01.08.02)

Details:	See details in the Data Acquisition and Computing Systems Interface Control Document.

Requirements:	F-DET-DS-COMP.6

References:

I-DET-DS-COMP-ONLINE.051	Slow Control Interface

System 1:	Online DAQ and Computing Systems (6.03.01.08.02)

System 2:	Barrel TOF Particle ID Systems (6.03.01.03)

Type:	CONTROL

Description:	Signal cables from environmental sensors will run from the DIRC electronics to the DAQ system to provide detector  shutdown/protection system.

Provider:	Online DAQ and Computing Systems (6.03.01.08.02)

Details:	See details in the Data Acquisition and Computing Systems Interface Control Document.

Requirements:	F-DET-DS-COMP.6

References:

I-DET-DS-COMP-ONLINE.052	Data Acquisition

System 1:	Online DAQ and Computing Systems (6.03.01.08.02)

System 2:	Backward Particle ID Systems (6.03.01.03)

Type:	DATA

Description:	Signal cables will run from the pfRICH/mRICH electronics to the DAQ system.

Provider:	Online DAQ and Computing Systems (6.03.01.08.02)

Details:	See details in the Data Acquisition and Computing Systems Interface Control Document.

Requirements:	F-DET-DS-COMP.6

References:

I-DET-DS-COMP-ONLINE.053	Slow Control Interface

System 1:	Online DAQ and Computing Systems (6.03.01.08.02)

System 2:	Backward Particle ID Systems (6.03.01.03)

Type:	CONTROL

Description:	Signal cables from environmental sensors will run from the pfRICH/mRICH electronics to the DAQ system to provide a detector  shutdown/protection system.

Provider:	Online DAQ and Computing Systems (6.03.01.08.02)

Details:	See details in the Data Acquisition and Computing Systems Interface Control Document.

Requirements:	F-DET-DS-COMP.6

References:

I-DET-DS-COMP-ONLINE.054	Data Acquisition

System 1:	Online DAQ and Computing Systems (6.03.01.08.02)

System 2:	Forward Particle ID Systems (6.03.01.03)

Type:	DATA

Description:	Signal cables will run from the dRICH electronics to the DAQ system.

Provider:	Online DAQ and Computing Systems (6.03.01.08.02)

Details:	See details in the Data Acquisition and Computing Systems Interface Control Document.

Requirements:	F-DET-DS-COMP.6

References:

I-DET-DS-COMP-ONLINE.055	Slow Control Interface

System 1:	Online DAQ and Computing Systems (6.03.01.08.02)

System 2:	Forward Particle ID Systems (6.03.01.03)

Type:	CONTROL

Description:	Signal cables from environmental sensors will run from the cold photosensors to the DAQ system to provide a detector  shutdown/protection system.

Provider:	Online DAQ and Computing Systems (6.03.01.08.02)

Details:	See details in the Data Acquisition and Computing Systems Interface Control Document.

Requirements:	F-DET-DS-COMP-SC.1

References:

I-DET-DS-PID-BAR-DIRC.001	Light Pulser System

System 1:	Barrel DIRC Particle ID Systems (6.03.01.03)

System 2:	Online DAQ and Computing Systems (6.03.01.08.02)

Type:	CONTROL

Description:	The light pulser system will require synchronization using a clock that will be provided by data acquisition.

Provider:	Barrel DIRC Particle ID Systems (6.03.01.03)

Details:	See details in the Particle Identification Systems Interface Control Document.

Requirements:	P-DET-DS-PID-BAR-DIRC.4

References:

I-DET-DS-PID-BCK-RICH.001	Light Pulser System

System 1:	Backward RICH Detector Systems (6.03.01.03)

System 2:	Online DAQ and Computing Systems (6.03.01.08.02)

Type:	CONTROL

Description:	The light pulser system will require synchronization using a clock that will be provided by data acquisition.

Provider:	Backward RICH Detector Systems (6.03.01.03)

Details:	See details in the Particle Identification Systems Interface Control Document.

Requirements:	P-DET-DS-PID-BCK-RICH.4

References:

I-DET-DS-PID-FWD-RICH.001	Light Pulser System

System 1:	Forward RICH Detector Systems (6.03.01.03)

System 2:	Online DAQ and Computing Systems (6.03.01.08.02)

Type:	CONTROL

Description:	The light pulser system will require synchronization using a clock that will be provided by data acquisition.

Provider:	Forward RICH Detector Systems (6.03.01.03)

Details:	See details in the Particle Identification Systems Interface Control Document.

Requirements:	P-DET-DS-PID-FWD-RICH.4

References:

# Particle Identification Systems (DET-DS-PID) to Electromagnetic Calorimetry Systems (DET-DS-ECAL)

I-DET-DS-INF-BAR.003	Support Structure

System 1:	Barrel EMCal Systems (6.03.01.04)

System 2:	Barrel DIRC Particle ID Systems (6.03.01.03)

Type:	STRUCT

Description:	The DIRC bar boxes will be supported by a frame inside the barrel Electromagnetic Calorimeter, that allows the boxes to be extracted using a system of rollers.

Provider:	Barrel Detector Structures (6.03.01.09.01)

Details:	See details in the Detector Infrastructure Interface Control Document.

Requirements:	P-DET-DS-ECAL.1, P-DET-DS-ECAL.2, P-DET-DS-PID.4, P-DET-DS-PID.5

References:

I-DET-DS-INF-BAR.005	Weight Transfer

System 1:	Backward EMCal Systems (6.03.01.04)

System 2:	Barrel DIRC Particle ID Systems (6.03.01.03)

Type:	STRUCT

Description:	The weight of the backward ECAL will be transferred to the DIRC detector support and must be accommodated by all subsequent support systems.

Provider:	Barrel Detector Structures (6.03.01.09.01)

Details:	See details in the Detector Infrastructure Interface Control Document.

Requirements:	P-DET-DS-ECAL.2, P-DET-DS-HCAL.2

References:

I-DET-DS-INF-BAR.006	EMCal Support System

System 1:	Backward EMCal Systems (6.03.01.04)

System 2:	Barrel DIRC Particle ID Systems (6.03.01.03)

Type:	STRUCT

Description:	The DIRC support system will provide support for the backward electromagnetic calorimeter.

Provider:	Barrel Detector Structures (6.03.01.09.01)

Details:	See details in the Detector Infrastructure Interface Control Document.

Requirements:	P-DET-DS-ECAL.2, P-DET-DS-PID.4

References:

I-DET-DS-INF-INT.013	Space Constraint

System 1:	Barrel EMCal Systems (6.03.01.04)

System 2:	Barrel DIRC Particle ID Systems (6.03.01.03)

Type:	SPACE

Description:	The backward position and shape of the barrel EMCAL is limited by the size and shape of the DIRC readout supports.  Changes to the size or position of either must be coordinated with the other.

Provider:	Detector Infrastructure and Utilities Integration (6.03.01.09.04)

Details:	See details in the Detector Infrastructure Interface Control Document.

Requirements:	P-DET-DS-ECAL.1, P-DET-DS-PID.5

References:

I-DET-DS-INF-INT.014	Space Constraint

System 1:	Barrel EMCal Systems (6.03.01.04)

System 2:	Barrel DIRC Particle ID Systems (6.03.01.03)

Type:	SPACE

Description:	The maximum size of the DIRC is limited to the interior bore of the barrel ECAL (and its support structures).  Modifications to either must be coordinated.

Provider:	Detector Infrastructure and Utilities Integration (6.03.01.09.04)

Details:	See details in the Detector Infrastructure Interface Control Document.

Requirements:	P-DET-DS-ECAL.1, P-DET-DS-PID.5

References:

I-DET-DS-INF-INT.015	Space Constraint

System 1:	Barrel EMCal Systems (6.03.01.04)

System 2:	Forward RICH Detector Systems (6.03.01.03)

Type:	SPACE

Description:	The forward position and shape of the barrel ECAL is limited  by the backward face of the Dual RICH detector and the adjacent cabling pathway that provides services to the interior detectors. Modifications to either must be coordinated.

Provider:	Detector Infrastructure and Utilities Integration (6.03.01.09.04)

Details:	See details in the Detector Infrastructure Interface Control Document.

Requirements:	P-DET-DS-ECAL.1, P-DET-DS-PID.5

References:

I-DET-DS-INF-INT.017	Space Constraint

System 1:	Backward EMCal Systems (6.03.01.04)

System 2:	Barrel DIRC Particle ID Systems (6.03.01.03)

Type:	SPACE

Description:	The maximum backward location for the DIRC  is limited by the position of the backward electromagnetic calorimeter. Modifications to either must be coordinated.

Provider:	Detector Infrastructure and Utilities Integration (6.03.01.09.04)

Details:	See details in the Detector Infrastructure Interface Control Document.

Requirements:	P-DET-DS-ECAL.1, P-DET-DS-PID.5

References:

I-DET-DS-INF-INT.018	Space Constraint

System 1:	Backward EMCal Systems (6.03.01.04)

System 2:	Barrel DIRC Particle ID Systems (6.03.01.03)

Type:	SPACE

Description:	The exterior radius of the backward ECAL is limited by the interior bore of the DIRC support system. Modifications to either must be coordinated.

Provider:	Detector Infrastructure and Utilities Integration (6.03.01.09.04)

Details:	See details in the Detector Infrastructure Interface Control Document.

Requirements:	P-DET-DS-ECAL.1, P-DET-DS-PID.5

References:

I-DET-DS-INF-INT.019	Heat Mitigation

System 1:	Backward EMCal Systems (6.03.01.04)

System 2:	Backward Particle ID Systems (6.03.01.03)

Type:	ENV

Description:	The backward ECAL must mitigated the heat generated by the backward RICH detector.

Provider:	Detector Infrastructure and Utilities Integration (6.03.01.09.04)

Details:	See details in the Detector Infrastructure Interface Control Document.

Requirements:	P-DET-DS-ECAL.3

References:

I-DET-DS-INF-INT.020	Space Constraint

System 1:	Backward EMCal Systems (6.03.01.04)

System 2:	Backward RICH Detector Systems (6.03.01.03)

Type:	SPACE

Description:	The position of the backward ECAL in the forward direction is limited by the backward face of the pfRICH/mRICH detector.  Modifications to either must be coordinated.

Provider:	Detector Infrastructure and Utilities Integration (6.03.01.09.04)

Details:	See details in the Detector Infrastructure Interface Control Document.

Requirements:	P-DET-DS-ECAL.1, P-DET-DS-PID.5

References:

I-DET-DS-INF-INT.025	Space Constraint

System 1:	Forward EMCal Systems (6.03.01.04)

System 2:	Forward RICH Detector Systems (6.03.01.03)

Type:	SPACE

Description:	The maximum forward location for the dRICH is limited by the forward ECAL  and the adjacent cabling pathway that provides services to the interior detectors. Modifications to either must be coordinated.

Provider:	Detector Infrastructure and Utilities Integration (6.03.01.09.04)

Details:	See details in the Detector Infrastructure Interface Control Document.

Requirements:	P-DET-DS-ECAL.1, P-DET-DS-PID.5

References:

# Particle Identification Systems (DET-DS-PID) to Electronic Systems (DET-DS-ELEC)

I-DET-DS-ELEC.046	Bias Voltage

System 1:	Electronic Systems (6.03.01.07)

System 2:	Barrel Particle ID Systems (6.03.01.03)

Type:	ELEC

Description:	Bias voltage DC power will be provided from the electronics racks to support electronics the silicon photomultipliers.

Provider:	Electronic Systems (6.03.01.07)

Details:	See details in the Electronic Systems Interface Control Document.

Requirements:	P-DET-DS-PID.2

References:

I-DET-DS-ELEC.047	High Voltage

System 1:	Electronic Systems (6.03.01.07)

System 2:	Barrel Particle ID Systems (6.03.01.03)

Type:	ELEC

Description:	High voltage DC power will be provided from the electronics racks to support silicon sensors and gas detectors.

Provider:	Electronic Systems (6.03.01.07)

Details:	See details in the Electronic Systems Interface Control Document.

Requirements:	P-DET-DS-PID.2

References:

I-DET-DS-ELEC.048	Low Voltage

System 1:	Electronic Systems (6.03.01.07)

System 2:	Barrel Particle ID Systems (6.03.01.03)

Type:	ELEC

Description:	Low voltage DC power will be provided from the electronics racks to support electronics in the detector.

Provider:	Electronic Systems (6.03.01.07)

Details:	See details in the Electronic Systems Interface Control Document.

Requirements:	P-DET-DS-PID.2

References:

I-DET-DS-ELEC.049	High Voltage

System 1:	Electronic Systems (6.03.01.07)

System 2:	Barrel DIRC Particle ID Systems (6.03.01.03)

Type:	ELEC

Description:	High voltage power will be delivered from the power supplies on the carriage to the photosensors in the DIRC.

Provider:	Electronic Systems (6.03.01.07)

Details:	See details in the Electronic Systems Interface Control Document.

Requirements:	P-DET-DS-PID.2

References:

I-DET-DS-ELEC.050	Low Voltage

System 1:	Electronic Systems (6.03.01.07)

System 2:	Barrel DIRC Particle ID Systems (6.03.01.03)

Type:	ELEC

Description:	Low voltage power will be delivered from the power supplies on the carriage to the DIRC electronics.

Provider:	Electronic Systems (6.03.01.07)

Details:	See details in the Electronic Systems Interface Control Document.

Requirements:	P-DET-DS-PID.2

References:

I-DET-DS-ELEC.051	High Voltage

System 1:	Electronic Systems (6.03.01.07)

System 2:	Barrel TOF Particle ID Systems (6.03.01.03)

Type:	ELEC

Description:	High voltage power will be delivered from the power supplies on the carriage to the photosensors in the DIRC.

Provider:	Electronic Systems (6.03.01.07)

Details:	See details in the Electronic Systems Interface Control Document.

Requirements:	P-DET-DS-PID.2

References:

I-DET-DS-ELEC.052	Low Voltage

System 1:	Electronic Systems (6.03.01.07)

System 2:	Barrel TOF Particle ID Systems (6.03.01.03)

Type:	ELEC

Description:	Low voltage power will be delivered from the power supplies on the carriage to the DIRC electronics.

Provider:	Electronic Systems (6.03.01.07)

Details:	See details in the Electronic Systems Interface Control Document.

Requirements:	P-DET-DS-PID.2

References:

I-DET-DS-ELEC.053	High Voltage

System 1:	Electronic Systems (6.03.01.07)

System 2:	Backward Particle ID Systems (6.03.01.03)

Type:	ELEC

Description:	High voltage power will be delivered from the power supplies on the carriage to the photosensors in the pfRICH/mRICH.

Provider:	Electronic Systems (6.03.01.07)

Details:	See details in the Electronic Systems Interface Control Document.

Requirements:	P-DET-DS-PID.2

References:

I-DET-DS-ELEC.054	Low Voltage

System 1:	Electronic Systems (6.03.01.07)

System 2:	Backward Particle ID Systems (6.03.01.03)

Type:	ELEC

Description:	Low voltage power will be delivered from the power supplies on the carriage to the pfRICH/mRICH electronics.

Provider:	Electronic Systems (6.03.01.07)

Details:	See details in the Electronic Systems Interface Control Document.

Requirements:	P-DET-DS-PID.2

References:

I-DET-DS-ELEC.055	Bias Voltage

System 1:	Electronic Systems (6.03.01.07)

System 2:	Forward Particle ID Systems (6.03.01.03)

Type:	ELEC

Description:	Bias voltage DC power will be delivered from the power supplies on the carriage to the photosensors.

Provider:	Electronic Systems (6.03.01.07)

Details:	See details in the Electronic Systems Interface Control Document.

Requirements:	P-DET-DS-PID.2

References:

I-DET-DS-ELEC.056	Low Voltage

System 1:	Electronic Systems (6.03.01.07)

System 2:	Forward Particle ID Systems (6.03.01.03)

Type:	ELEC

Description:	Low voltage power will be delivered from the power supplies on the carriage to the dRICH electronics.

Provider:	Electronic Systems (6.03.01.07)

Details:	See details in the Electronic Systems Interface Control Document.

Requirements:	P-DET-DS-PID.2

References:

# Particle Identification Systems (DET-DS-PID) to Hadronic Calorimetry Systems (DET-DS-HCAL)

I-DET-DS-INF-BAR.012	Dual RICH Support System

System 1:	Barrel HCal Systems (6.03.01.05)

System 2:	Forward Particle ID Systems (6.03.01.03)

Type:	STRUCT

Description:	The Dual RICH will be supported by a structural system within the barrel hadron calorimeter.

Provider:	Barrel Detector Structures (6.03.01.09.01)

Details:	See details in the Detector Infrastructure Interface Control Document.

Requirements:	P-DET-DS-HCAL.2, P-DET-DS-PID.4

References:

I-DET-DS-INF-INT.032	Space Constraint

System 1:	Barrel HCal Systems (6.03.01.05)

System 2:	Forward RICH Detector Systems (6.03.01.03)

Type:	SPACE

Description:	The size of the dRICH is limited by the interior bore of the barrel HCAL, and must provide adequate space for cables and services to itself and the interior detectors. Modifications to either must be coordinated.

Provider:	Detector Infrastructure and Utilities Integration (6.03.01.09.04)

Details:	See details in the Detector Infrastructure Interface Control Document.

Requirements:	P-DET-DS-HCAL.1, P-DET-DS-PID.5

References:

I-DET-DS-INF-INT.035	Space Constraint

System 1:	Backward HCal Systems (6.03.01.05)

System 2:	Barrel DIRC Particle ID Systems (6.03.01.03)

Type:	SPACE

Description:	The maximum backward location for the DIRC is limited by the backward HCAL and the adjacent cabling pathway that provides services to the interior detectors. Modifications to either must be coordinated.

Provider:	Detector Infrastructure and Utilities Integration (6.03.01.09.04)

Details:	See details in the Detector Infrastructure Interface Control Document.

Requirements:	P-DET-DS-HCAL.1, P-DET-DS-PID.5

References:

# Particle Identification Systems (DET-DS-PID) to Detector Infrastructure (DET-DS-INF)

I-DET-DS-INF-BAR.003	Support Structure

System 1:	Barrel EMCal Systems (6.03.01.04)

System 2:	Barrel DIRC Particle ID Systems (6.03.01.03)

Type:	STRUCT

Description:	The DIRC bar boxes will be supported by a frame inside the barrel Electromagnetic Calorimeter, that allows the boxes to be extracted using a system of rollers.

Provider:	Barrel Detector Structures (6.03.01.09.01)

Details:	See details in the Detector Infrastructure Interface Control Document.

Requirements:	P-DET-DS-ECAL.1, P-DET-DS-ECAL.2, P-DET-DS-PID.4, P-DET-DS-PID.5

References:

I-DET-DS-INF-BAR.005	Weight Transfer

System 1:	Backward EMCal Systems (6.03.01.04)

System 2:	Barrel DIRC Particle ID Systems (6.03.01.03)

Type:	STRUCT

Description:	The weight of the backward ECAL will be transferred to the DIRC detector support and must be accommodated by all subsequent support systems.

Provider:	Barrel Detector Structures (6.03.01.09.01)

Details:	See details in the Detector Infrastructure Interface Control Document.

Requirements:	P-DET-DS-ECAL.2, P-DET-DS-HCAL.2

References:

I-DET-DS-INF-BAR.006	EMCal Support System

System 1:	Backward EMCal Systems (6.03.01.04)

System 2:	Barrel DIRC Particle ID Systems (6.03.01.03)

Type:	STRUCT

Description:	The DIRC support system will provide support for the backward electromagnetic calorimeter.

Provider:	Barrel Detector Structures (6.03.01.09.01)

Details:	See details in the Detector Infrastructure Interface Control Document.

Requirements:	P-DET-DS-ECAL.2, P-DET-DS-PID.4

References:

I-DET-DS-INF-BAR.012	Dual RICH Support System

System 1:	Barrel HCal Systems (6.03.01.05)

System 2:	Forward Particle ID Systems (6.03.01.03)

Type:	STRUCT

Description:	The Dual RICH will be supported by a structural system within the barrel hadron calorimeter.

Provider:	Barrel Detector Structures (6.03.01.09.01)

Details:	See details in the Detector Infrastructure Interface Control Document.

Requirements:	P-DET-DS-HCAL.2, P-DET-DS-PID.4

References:

I-DET-DS-INF-BAR.017	DIRC Readout Supports

System 1:	Solenoid Magnet (6.03.01.06)

System 2:	Barrel DIRC Particle ID Systems (6.03.01.03)

Type:	STRUCT

Description:	The DIRC prism box containing the photosensor readouts, will be supported by an external structure supporting the ECAL within the solenoid magnet.

Provider:	Barrel Detector Structures (6.03.01.09.01)

Details:	See details in the Detector Infrastructure Interface Control Document.

Requirements:	P-DET-DS-MAG.1, P-DET-DS-MAG.2, P-DET-DS-PID.4

References:

I-DET-DS-INF-BAR.018	Structural Support

System 1:	Barrel Particle ID Systems (6.03.01.03)

System 2:	Backward Particle ID Systems (6.03.01.03)

Type:	STRUCT

Description:	The pfRICH/mRICH will be supported by the DIRC bar box support system.

Provider:	Barrel Detector Structures (6.03.01.09.01)

Details:	See details in the Detector Infrastructure Interface Control Document.

Requirements:	P-DET-DS-PID.4, P-DET-DS-PID.5

References:

I-DET-DS-INF-BAR.019	Barrel TOF Support

System 1:	Barrel DIRC Particle ID Systems (6.03.01.03)

System 2:	Barrel TOF Particle ID Systems (6.03.01.03)

Type:	STRUCT

Description:	The barrel Time of Flight will be supported by the DIRC bar box assembly.

Provider:	Barrel Detector Structures (6.03.01.09.01)

Details:	See details in the Detector Infrastructure Interface Control Document.

Requirements:	P-DET-DS-PID.4, P-DET-DS-PID.5

References:

I-DET-DS-INF-BAR.020	pfRICH/mRICH Support System

System 1:	Barrel DIRC Particle ID Systems (6.03.01.03)

System 2:	Backward RICH Detector Systems (6.03.01.03)

Type:	STRUCT

Description:	The DIRC support system will provide support for the upstream PID detector system.

Provider:	Barrel Detector Structures (6.03.01.09.01)

Details:	See details in the Detector Infrastructure Interface Control Document.

Requirements:	P-DET-DS-PID.4, P-DET-DS-PID.5

References:

I-DET-DS-INF-BAR.021	Silicon Tracker Support System

System 1:	Barrel DIRC Particle ID Systems (6.03.01.03)

System 2:	Tracking Systems (6.03.01.02)

Type:	STRUCT

Description:	The DIRC support system will provide support for the silicon trackers in the detector barrel.

Provider:	Barrel Detector Structures (6.03.01.09.01)

Details:	See details in the Detector Infrastructure Interface Control Document.

Requirements:	P-DET-DS-PID.4, P-DET-DS-PID.5, P-DET-DS-TRAK.8, P-DET-DS-TRAK.9

References:

I-DET-DS-INF-BAR.022	Weight Distributed

System 1:	Barrel DIRC Particle ID Systems (6.03.01.03)

System 2:	Barrel Tracking Systems (6.03.01.02)

Type:	STRUCT

Description:	The weight of the barrel tracking systems will be transferred to the DIRC support system.

Provider:	Barrel Detector Structures (6.03.01.09.01)

Details:	See details in the Detector Infrastructure Interface Control Document.

Requirements:	P-DET-DS-PID.4, P-DET-DS-TRAK.8

References:

I-DET-DS-INF-FWD.011	Space Constraint

System 1:	Forward RICH Detector Systems (6.03.01.03)

System 2:	IR Accelerator Systems (6.04.03)

Type:	SPACE

Description:	The interior radius of the dRICH is governed by the size of the beamline and its associated flanges and support system.

Provider:	Hadron Endcap Structures (6.03.01.09.03)

Details:	See details in the Detector Infrastructure Interface Control Document.

Requirements:	P-DET-DS-PID.5

References:

I-DET-DS-INF-FWD.012	Space Constraint

System 1:	Forward Time of Flight Systems (6.03.01.03)

System 2:	Barrel Tracking Systems (6.03.01.02)

Type:	SPACE

Description:	The maximum forward location for the tracking system is limited by the position of the AC LGAD Time of Flight Detector. Modifications to either must be coordinated.

Provider:	Hadron Endcap Structures (6.03.01.09.03)

Details:	See details in the Detector Infrastructure Interface Control Document.

Requirements:	P-DET-DS-PID.5, P-DET-DS-TRAK.9

References:

I-DET-DS-INF-INT.003	Cabling/Services Conduit

System 1:	Detector System (6.03.01)

System 2:	Barrel DIRC Particle ID Systems (6.03.01.03)

Type:	SPACE

Description:	The DIRC structure will provide integrated pathways for cabling and services to be delivered from the interior detectors (pf RICH/mRICH and silicon trackers) to the exterior infrastructure.

Provider:	Detector Infrastructure and Utilities Integration (6.03.01.09.04)

Details:	See details in the Detector Infrastructure Interface Control Document.

Requirements:	P-DET-DS-PID.4, P-DET-DS-PID.5

References:

I-DET-DS-INF-INT.013	Space Constraint

System 1:	Barrel EMCal Systems (6.03.01.04)

System 2:	Barrel DIRC Particle ID Systems (6.03.01.03)

Type:	SPACE

Description:	The backward position and shape of the barrel EMCAL is limited by the size and shape of the DIRC readout supports.  Changes to the size or position of either must be coordinated with the other.

Provider:	Detector Infrastructure and Utilities Integration (6.03.01.09.04)

Details:	See details in the Detector Infrastructure Interface Control Document.

Requirements:	P-DET-DS-ECAL.1, P-DET-DS-PID.5

References:

I-DET-DS-INF-INT.014	Space Constraint

System 1:	Barrel EMCal Systems (6.03.01.04)

System 2:	Barrel DIRC Particle ID Systems (6.03.01.03)

Type:	SPACE

Description:	The maximum size of the DIRC is limited to the interior bore of the barrel ECAL (and its support structures).  Modifications to either must be coordinated.

Provider:	Detector Infrastructure and Utilities Integration (6.03.01.09.04)

Details:	See details in the Detector Infrastructure Interface Control Document.

Requirements:	P-DET-DS-ECAL.1, P-DET-DS-PID.5

References:

I-DET-DS-INF-INT.015	Space Constraint

System 1:	Barrel EMCal Systems (6.03.01.04)

System 2:	Forward RICH Detector Systems (6.03.01.03)

Type:	SPACE

Description:	The forward position and shape of the barrel ECAL is limited  by the backward face of the Dual RICH detector and the adjacent cabling pathway that provides services to the interior detectors. Modifications to either must be coordinated.

Provider:	Detector Infrastructure and Utilities Integration (6.03.01.09.04)

Details:	See details in the Detector Infrastructure Interface Control Document.

Requirements:	P-DET-DS-ECAL.1, P-DET-DS-PID.5

References:

I-DET-DS-INF-INT.017	Space Constraint

System 1:	Backward EMCal Systems (6.03.01.04)

System 2:	Barrel DIRC Particle ID Systems (6.03.01.03)

Type:	SPACE

Description:	The maximum backward location for the DIRC  is limited by the position of the backward electromagnetic calorimeter. Modifications to either must be coordinated.

Provider:	Detector Infrastructure and Utilities Integration (6.03.01.09.04)

Details:	See details in the Detector Infrastructure Interface Control Document.

Requirements:	P-DET-DS-ECAL.1, P-DET-DS-PID.5

References:

I-DET-DS-INF-INT.018	Space Constraint

System 1:	Backward EMCal Systems (6.03.01.04)

System 2:	Barrel DIRC Particle ID Systems (6.03.01.03)

Type:	SPACE

Description:	The exterior radius of the backward ECAL is limited by the interior bore of the DIRC support system. Modifications to either must be coordinated.

Provider:	Detector Infrastructure and Utilities Integration (6.03.01.09.04)

Details:	See details in the Detector Infrastructure Interface Control Document.

Requirements:	P-DET-DS-ECAL.1, P-DET-DS-PID.5

References:

I-DET-DS-INF-INT.019	Heat Mitigation

System 1:	Backward EMCal Systems (6.03.01.04)

System 2:	Backward Particle ID Systems (6.03.01.03)

Type:	ENV

Description:	The backward ECAL must mitigated the heat generated by the backward RICH detector.

Provider:	Detector Infrastructure and Utilities Integration (6.03.01.09.04)

Details:	See details in the Detector Infrastructure Interface Control Document.

Requirements:	P-DET-DS-ECAL.3

References:

I-DET-DS-INF-INT.020	Space Constraint

System 1:	Backward EMCal Systems (6.03.01.04)

System 2:	Backward RICH Detector Systems (6.03.01.03)

Type:	SPACE

Description:	The position of the backward ECAL in the forward direction is limited by the backward face of the pfRICH/mRICH detector.  Modifications to either must be coordinated.

Provider:	Detector Infrastructure and Utilities Integration (6.03.01.09.04)

Details:	See details in the Detector Infrastructure Interface Control Document.

Requirements:	P-DET-DS-ECAL.1, P-DET-DS-PID.5

References:

I-DET-DS-INF-INT.025	Space Constraint

System 1:	Forward EMCal Systems (6.03.01.04)

System 2:	Forward RICH Detector Systems (6.03.01.03)

Type:	SPACE

Description:	The maximum forward location for the dRICH is limited by the forward ECAL  and the adjacent cabling pathway that provides services to the interior detectors. Modifications to either must be coordinated.

Provider:	Detector Infrastructure and Utilities Integration (6.03.01.09.04)

Details:	See details in the Detector Infrastructure Interface Control Document.

Requirements:	P-DET-DS-ECAL.1, P-DET-DS-PID.5

References:

I-DET-DS-INF-INT.032	Space Constraint

System 1:	Barrel HCal Systems (6.03.01.05)

System 2:	Forward RICH Detector Systems (6.03.01.03)

Type:	SPACE

Description:	The size of the dRICH is limited by the interior bore of the barrel HCAL, and must provide adequate space for cables and services to itself and the interior detectors. Modifications to either must be coordinated.

Provider:	Detector Infrastructure and Utilities Integration (6.03.01.09.04)

Details:	See details in the Detector Infrastructure Interface Control Document.

Requirements:	P-DET-DS-HCAL.1, P-DET-DS-PID.5

References:

I-DET-DS-INF-INT.035	Space Constraint

System 1:	Backward HCal Systems (6.03.01.05)

System 2:	Barrel DIRC Particle ID Systems (6.03.01.03)

Type:	SPACE

Description:	The maximum backward location for the DIRC is limited by the backward HCAL and the adjacent cabling pathway that provides services to the interior detectors. Modifications to either must be coordinated.

Provider:	Detector Infrastructure and Utilities Integration (6.03.01.09.04)

Details:	See details in the Detector Infrastructure Interface Control Document.

Requirements:	P-DET-DS-HCAL.1, P-DET-DS-PID.5

References:

I-DET-DS-INF-INT.084	Process Cooling

System 1:	Detector Infrastructure and Utilities Integration (6.03.01.09.04)

System 2:	Barrel DIRC Particle ID Systems (6.03.01.03)

Type:	COOL

Description:	The heat generated by the photo-multiplier tubes and associated electronics must be removed using a process cooling system.

Provider:	Detector Infrastructure and Utilities Integration (6.03.01.09.04)

Details:	See details in the Detector Infrastructure Interface Control Document.

Requirements:	P-DET-DS-PID.1

References:

I-DET-DS-INF-INT.085	Process Cooling

System 1:	Detector Infrastructure and Utilities Integration (6.03.01.09.04)

System 2:	Barrel TOF Particle ID Systems (6.03.01.03)

Type:	COOL

Description:	The heat generated by the detector and its electronics must be removed using a process cooling system.

Provider:	Detector Infrastructure and Utilities Integration (6.03.01.09.04)

Details:	See details in the Detector Infrastructure Interface Control Document.

Requirements:	P-DET-DS-PID.1

References:

I-DET-DS-INF-INT.086	Process Cooling

System 1:	Detector Infrastructure and Utilities Integration (6.03.01.09.04)

System 2:	Backward Particle ID Systems (6.03.01.03)

Type:	COOL

Description:	The heat generated by the photomultipliers and associated electronics must be removed using a process cooling system.

Provider:	Detector Infrastructure and Utilities Integration (6.03.01.09.04)

Details:	See details in the Detector Infrastructure Interface Control Document.

Requirements:	P-DET-DS-PID.1

References:

I-DET-DS-INF-INT.087	Dry Nitrogen

System 1:	Detector Infrastructure and Utilities Integration (6.03.01.09.04)

System 2:	Backward Particle ID Systems (6.03.01.03)

Type:	GAS/FLUID

Description:	A continuous flow of dry nitrogen will be required to protect the aerogel radiator.

Provider:	Detector Infrastructure and Utilities Integration (6.03.01.09.04)

Details:	See details in the Detector Infrastructure Interface Control Document.

Requirements:	P-DET-DS-PID-FWD-RICH.7

References:

I-DET-DS-INF-INT.088	Photosensor Cooling System

System 1:	Detector Infrastructure and Utilities Integration (6.03.01.09.04)

System 2:	Forward Particle ID Systems (6.03.01.03)

Type:	COOL

Description:	A fluid cooling system will be required to keep the operating temperature of the photosensors at -30 degrees Celsius.

Provider:	Detector Infrastructure and Utilities Integration (6.03.01.09.04)

Details:	See details in the Detector Infrastructure Interface Control Document.

Requirements:	P-DET-DS-PID.1

References:

I-DET-DS-INF-INT.089	Gas Recovery System

System 1:	Detector Infrastructure and Utilities Integration (6.03.01.09.04)

System 2:	Forward Particle ID Systems (6.03.01.03)

Type:	ENV

Description:	A recovery system will need to be included for the radiator gas used by the dRICH to prevent accidental release.

Provider:	Detector Infrastructure and Utilities Integration (6.03.01.09.04)

Details:	See details in the Detector Infrastructure Interface Control Document.

Requirements:	P-DET-DS-PID-FWD-RICH.6

References:

I-DET-DS-INF-INT.090	Insulation

System 1:	Detector Infrastructure and Utilities Integration (6.03.01.09.04)

System 2:	Forward Particle ID Systems (6.03.01.03)

Type:	ENV

Description:	Sufficient insulation will need to be added around all coolant lines to prevent condensation and ensure that ice doesn't form.

Provider:	Detector Infrastructure and Utilities Integration (6.03.01.09.04)

Details:	See details in the Detector Infrastructure Interface Control Document.

Requirements:	P-DET-DS-PID.1

References:

I-DET-DS-INF-INT.091	Dry Nitrogen for Aerogel

System 1:	Detector Infrastructure and Utilities Integration (6.03.01.09.04)

System 2:	Forward Particle ID Systems (6.03.01.03)

Type:	GAS/FLUID

Description:	A continuous flow of dry nitrogen will be required to protect the aerogel radiator.

Provider:	Detector Infrastructure and Utilities Integration (6.03.01.09.04)

Details:	See details in the Detector Infrastructure Interface Control Document.

Requirements:	P-DET-DS-PID.1

References:

I-DET-DS-INF-INT.092	Dry Nitrogen for Photosensors

System 1:	Detector Infrastructure and Utilities Integration (6.03.01.09.04)

System 2:	Forward Particle ID Systems (6.03.01.03)

Type:	GAS/FLUID

Description:	This gas will be required to create a protective atmosphere for the cold photosensors and to prevent condensation and freezing.

Provider:	Detector Infrastructure and Utilities Integration (6.03.01.09.04)

Details:	See details in the Detector Infrastructure Interface Control Document.

Requirements:	P-DET-DS-PID.1

References:

I-DET-DS-INF-INT.093	Radiator Gas

System 1:	Detector Infrastructure and Utilities Integration (6.03.01.09.04)

System 2:	Forward Particle ID Systems (6.03.01.03)

Type:	GAS/FLUID

Description:	A continuous recirculating flow of radiator gas will be required.

Provider:	Detector Infrastructure and Utilities Integration (6.03.01.09.04)

Details:	See details in the Detector Infrastructure Interface Control Document.

Requirements:	P-DET-DS-PID-FWD-RICH.6

References:

I-DET-DS-INF-INT.097	Space Constraint

System 1:	Solenoid Magnet (6.03.01.06)

System 2:	Forward RICH Detector Systems (6.03.01.03)

Type:	SPACE

Description:	The maximum forward location for the solenoid is limited by the position of the Dual RICH detector and the cabling plenum (gap) between them. Modifications to either must be coordinated.

Provider:	Detector Infrastructure and Utilities Integration (6.03.01.09.04)

Details:	See details in the Detector Infrastructure Interface Control Document.

Requirements:	P-DET-DS-MAG.1, P-DET-DS-PID.5

References:

I-DET-DS-INF-INT.098	Space Constraint

System 1:	Barrel DIRC Particle ID Systems (6.03.01.03)

System 2:	Barrel TOF Particle ID Systems (6.03.01.03)

Type:	SPACE

Description:	The maximum diameter of the barrel Time of Flight detector is limited by the interior bore of the DIRC detector. Modifications to either must be coordinated.

Provider:	Detector Infrastructure and Utilities Integration (6.03.01.09.04)

Details:	See details in the Detector Infrastructure Interface Control Document.

Requirements:	P-DET-DS-PID.5

References:

I-DET-DS-INF-INT.099	Space Constraint

System 1:	Barrel DIRC Particle ID Systems (6.03.01.03)

System 2:	Backward RICH Detector Systems (6.03.01.03)

Type:	SPACE

Description:	The exterior radius of the pfRICH/mRICH is limited by the interior bore of the DIRC support system. Modifications to either must be coordinated.

Provider:	Detector Infrastructure and Utilities Integration (6.03.01.09.04)

Details:	See details in the Detector Infrastructure Interface Control Document.

Requirements:	P-DET-DS-PID.5

References:

I-DET-DS-INF-INT.100	Space Constraint

System 1:	Barrel DIRC Particle ID Systems (6.03.01.03)

System 2:	Forward RICH Detector Systems (6.03.01.03)

Type:	SPACE

Description:	The maximum forward location for the DIRC is limited by the position of the dRICH detector and the cabling plenum between them. Modifications to either must be coordinated.

Provider:	Detector Infrastructure and Utilities Integration (6.03.01.09.04)

Details:	See details in the Detector Infrastructure Interface Control Document.

Requirements:	P-DET-DS-PID.5

References:

I-DET-DS-INF-INT.101	Service Pathway

System 1:	Barrel DIRC Particle ID Systems (6.03.01.03)

System 2:	Barrel Tracking Systems (6.03.01.02)

Type:	SPACE

Description:	Conduits must be provided within the DIRC support system that are adequate to deliver services (power, signal, cooling) to the barrel tracking detectors.

Provider:	Detector Infrastructure and Utilities Integration (6.03.01.09.04)

Details:	See details in the Detector Infrastructure Interface Control Document.

Requirements:	P-DET-DS-PID.5, P-DET-DS-TRAK.9

References:

I-DET-DS-INF-INT.102	Space Constraint

System 1:	Barrel DIRC Particle ID Systems (6.03.01.03)

System 2:	Barrel Tracking Systems (6.03.01.02)

Type:	SPACE

Description:	The exterior radius of the silicon trackers is limited by the interior bore of the DIRC support system. Modifications to either must be coordinated.

Provider:	Detector Infrastructure and Utilities Integration (6.03.01.09.04)

Details:	See details in the Detector Infrastructure Interface Control Document.

Requirements:	P-DET-DS-PID.5, P-DET-DS-TRAK.9

References:

I-DET-DS-INF-INT.103	Space Constraint

System 1:	Barrel DIRC Particle ID Systems (6.03.01.03)

System 2:	Barrel Tracking Systems (6.03.01.02)

Type:	SPACE

Description:	The maximum forward location for the DIRC is limited by the position of the silicon trackers.

Provider:	Detector Infrastructure and Utilities Integration (6.03.01.09.04)

Details:	See details in the Detector Infrastructure Interface Control Document.

Requirements:	P-DET-DS-PID.5, P-DET-DS-TRAK.9

References:

I-DET-DS-INF-INT.104	Space Constraint

System 1:	Barrel TOF Particle ID Systems (6.03.01.03)

System 2:	Backward RICH Detector Systems (6.03.01.03)

Type:	SPACE

Description:	The maximum backward location for the barrel Time of Flight  detector is limited by the position of the pfRICH/mRICH detector and the adjacent cabling pathway. Modifications to either must be coordinated.

Provider:	Detector Infrastructure and Utilities Integration (6.03.01.09.04)

Details:	See details in the Detector Infrastructure Interface Control Document.

Requirements:	P-DET-DS-PID.5

References:

I-DET-DS-INF-INT.105	Space Constraint

System 1:	Barrel TOF Particle ID Systems (6.03.01.03)

System 2:	Forward Time of Flight Systems (6.03.01.03)

Type:	SPACE

Description:	The maximum forward location for the barrel Time of Flight detector is limited by the position of the forward Time of Flight detector and the adjacent cabling pathway. Modifications to either must be coordinated.

Provider:	Detector Infrastructure and Utilities Integration (6.03.01.09.04)

Details:	See details in the Detector Infrastructure Interface Control Document.

Requirements:	P-DET-DS-PID.5

References:

I-DET-DS-INF-INT.106	Space Constraint

System 1:	Barrel TOF Particle ID Systems (6.03.01.03)

System 2:	Barrel Tracking Systems (6.03.01.02)

Type:	SPACE

Description:	The maximum size of the silicon trackers is limited by the interior radius of the barrel Time of Flight detector. Modifications to either must be coordinated.

Provider:	Detector Infrastructure and Utilities Integration (6.03.01.09.04)

Details:	See details in the Detector Infrastructure Interface Control Document.

Requirements:	P-DET-DS-PID.5, P-DET-DS-TRAK.9

References:

I-DET-DS-INF-INT.107	Space Constraint

System 1:	Backward RICH Detector Systems (6.03.01.03)

System 2:	Barrel Tracking Systems (6.03.01.02)

Type:	SPACE

Description:	The maximum backward location for the tracking system is limited by the position of the pfRICH/mRICH. Modifications to either must be coordinated.

Provider:	Detector Infrastructure and Utilities Integration (6.03.01.09.04)

Details:	See details in the Detector Infrastructure Interface Control Document.

Requirements:	P-DET-DS-PID.5, P-DET-DS-TRAK.9

References:

I-DET-DS-INF-INT.108	Space Constraint

System 1:	Backward RICH Detector Systems (6.03.01.03)

System 2:	IR Accelerator Systems (6.04.03)

Type:	SPACE

Description:	The interior radius of the pfRICH is governed by the size of the beamline and its associated flanges and support system.

Provider:	Detector Infrastructure and Utilities Integration (6.03.01.09.04)

Details:	See details in the Detector Infrastructure Interface Control Document.

Requirements:	P-DET-DS-PID.5

References:

I-DET-DS-INF-INT.137	Space Constraint

System 1:	Detector Infrastructure and Utilities Integration (6.03.01.09.04)

System 2:	Barrel DIRC Particle ID Systems (6.03.01.03)

Type:	SPACE

Description:	Rack space on the platform must be provided for a small laser system to monitor the response of the photsensors and allow adjustment of timing correlations within the detector.

Provider:	Detector Infrastructure and Utilities Integration (6.03.01.09.04)

Details:	See details in the Detector Infrastructure Interface Control Document.

Requirements:	P-DET-DS-PID-BAR-DIRC.7

References:

I-DET-DS-INF-INT.138	Space Constraint

System 1:	Detector Infrastructure and Utilities Integration (6.03.01.09.04)

System 2:	Forward RICH Detector Systems (6.03.01.03)

Type:	SPACE

Description:	Rack space on the platform must be provided for a small laser system to monitor the response of the photsensors and allow adjustment of timing correlations within the detector.

Provider:	Detector Infrastructure and Utilities Integration (6.03.01.09.04)

Details:	See details in the Detector Infrastructure Interface Control Document.

Requirements:	P-DET-DS-PID-FWD-RICH.10

References:

I-DET-DS-INF-INT.139	Space Constraint

System 1:	Detector Infrastructure and Utilities Integration (6.03.01.09.04)

System 2:	Backward RICH Detector Systems (6.03.01.03)

Type:	SPACE

Description:	Rack space on the platform must be provided for a small laser system to monitor the response of the photsensors and allow adjustment of timing correlations within the detector.

Provider:	Detector Infrastructure and Utilities Integration (6.03.01.09.04)

Details:	See details in the Detector Infrastructure Interface Control Document.

Requirements:	P-DET-DS-PID-BCK-RICH.7

References:

# Particle Identification Systems (DET-DS-PID) to Solenoid Magnet (DET-DS-MAG)

I-DET-DS-INF-BAR.017	DIRC Readout Supports

System 1:	Solenoid Magnet (6.03.01.06)

System 2:	Barrel DIRC Particle ID Systems (6.03.01.03)

Type:	STRUCT

Description:	The DIRC prism box containing the photosensor readouts, will be supported by an external structure supporting the ECAL within the solenoid magnet.

Provider:	Barrel Detector Structures (6.03.01.09.01)

Details:	See details in the Detector Infrastructure Interface Control Document.

Requirements:	P-DET-DS-MAG.1, P-DET-DS-MAG.2, P-DET-DS-PID.4

References:

I-DET-DS-INF-INT.097	Space Constraint

System 1:	Solenoid Magnet (6.03.01.06)

System 2:	Forward RICH Detector Systems (6.03.01.03)

Type:	SPACE

Description:	The maximum forward location for the solenoid is limited by the position of the Dual RICH detector and the cabling plenum (gap) between them. Modifications to either must be coordinated.

Provider:	Detector Infrastructure and Utilities Integration (6.03.01.09.04)

Details:	See details in the Detector Infrastructure Interface Control Document.

Requirements:	P-DET-DS-MAG.1, P-DET-DS-PID.5

References:

# Particle Identification Systems (DET-DS-PID) to Tracking Systems (DET-DS-TRAK)

I-DET-DS-INF-BAR.021	Silicon Tracker Support System

System 1:	Barrel DIRC Particle ID Systems (6.03.01.03)

System 2:	Tracking Systems (6.03.01.02)

Type:	STRUCT

Description:	The DIRC support system will provide support for the silicon trackers in the detector barrel.

Provider:	Barrel Detector Structures (6.03.01.09.01)

Details:	See details in the Detector Infrastructure Interface Control Document.

Requirements:	P-DET-DS-PID.4, P-DET-DS-PID.5, P-DET-DS-TRAK.8, P-DET-DS-TRAK.9

References:

I-DET-DS-INF-BAR.022	Weight Distributed

System 1:	Barrel DIRC Particle ID Systems (6.03.01.03)

System 2:	Barrel Tracking Systems (6.03.01.02)

Type:	STRUCT

Description:	The weight of the barrel tracking systems will be transferred to the DIRC support system.

Provider:	Barrel Detector Structures (6.03.01.09.01)

Details:	See details in the Detector Infrastructure Interface Control Document.

Requirements:	P-DET-DS-PID.4, P-DET-DS-TRAK.8

References:

I-DET-DS-INF-FWD.012	Space Constraint

System 1:	Forward Time of Flight Systems (6.03.01.03)

System 2:	Barrel Tracking Systems (6.03.01.02)

Type:	SPACE

Description:	The maximum forward location for the tracking system is limited by the position of the AC LGAD Time of Flight Detector. Modifications to either must be coordinated.

Provider:	Hadron Endcap Structures (6.03.01.09.03)

Details:	See details in the Detector Infrastructure Interface Control Document.

Requirements:	P-DET-DS-PID.5, P-DET-DS-TRAK.9

References:

I-DET-DS-INF-INT.101	Service Pathway

System 1:	Barrel DIRC Particle ID Systems (6.03.01.03)

System 2:	Barrel Tracking Systems (6.03.01.02)

Type:	SPACE

Description:	Conduits must be provided within the DIRC support system that are adequate to deliver services (power, signal, cooling) to the barrel tracking detectors.

Provider:	Detector Infrastructure and Utilities Integration (6.03.01.09.04)

Details:	See details in the Detector Infrastructure Interface Control Document.

Requirements:	P-DET-DS-PID.5, P-DET-DS-TRAK.9

References:

I-DET-DS-INF-INT.102	Space Constraint

System 1:	Barrel DIRC Particle ID Systems (6.03.01.03)

System 2:	Barrel Tracking Systems (6.03.01.02)

Type:	SPACE

Description:	The exterior radius of the silicon trackers is limited by the interior bore of the DIRC support system. Modifications to either must be coordinated.

Provider:	Detector Infrastructure and Utilities Integration (6.03.01.09.04)

Details:	See details in the Detector Infrastructure Interface Control Document.

Requirements:	P-DET-DS-PID.5, P-DET-DS-TRAK.9

References:

I-DET-DS-INF-INT.103	Space Constraint

System 1:	Barrel DIRC Particle ID Systems (6.03.01.03)

System 2:	Barrel Tracking Systems (6.03.01.02)

Type:	SPACE

Description:	The maximum forward location for the DIRC is limited by the position of the silicon trackers.

Provider:	Detector Infrastructure and Utilities Integration (6.03.01.09.04)

Details:	See details in the Detector Infrastructure Interface Control Document.

Requirements:	P-DET-DS-PID.5, P-DET-DS-TRAK.9

References:

I-DET-DS-INF-INT.106	Space Constraint

System 1:	Barrel TOF Particle ID Systems (6.03.01.03)

System 2:	Barrel Tracking Systems (6.03.01.02)

Type:	SPACE

Description:	The maximum size of the silicon trackers is limited by the interior radius of the barrel Time of Flight detector. Modifications to either must be coordinated.

Provider:	Detector Infrastructure and Utilities Integration (6.03.01.09.04)

Details:	See details in the Detector Infrastructure Interface Control Document.

Requirements:	P-DET-DS-PID.5, P-DET-DS-TRAK.9

References:

I-DET-DS-INF-INT.107	Space Constraint

System 1:	Backward RICH Detector Systems (6.03.01.03)

System 2:	Barrel Tracking Systems (6.03.01.02)

Type:	SPACE

Description:	The maximum backward location for the tracking system is limited by the position of the pfRICH/mRICH. Modifications to either must be coordinated.

Provider:	Detector Infrastructure and Utilities Integration (6.03.01.09.04)

Details:	See details in the Detector Infrastructure Interface Control Document.

Requirements:	P-DET-DS-PID.5, P-DET-DS-TRAK.9

References:

# Applicable Documents

List applicable documents considered to be part of this document. Applicable documents shall be obtained from the EIC SharePoint Document Center.

# Detailed change Log

| Revision | Date | Change |
| --- | --- | --- |

# References

- doi:10.17226/25171., National Academies of Sciences, Engineering, and Medicine, "An Assessment of U.S.-Based Electron-Ion Collider Science", The National Academies Press, Washington, DC, 2018.

- EIC-ORG-PLN-010, Electron-Ion Collider L1 Requirements (Global Requirements Document)

- BNL-221006-2021-FORE, EIC Conceptual Design Report

- arXiv:2103.05419, Science Requirements and Detector Concepts for the Electron-Ion Collider: EIC Yellow Report

- EIC-SEG-PLN-016, Requirements Management Plan

- EIC-SEG-PLN-022, Systems Engineering Management Plan

- EIC-SEG-PLN-020, EIC Interface Management Plan