Electron-Ion Collider, Requirements, Specifications, and Interfaces

Interface Control Document: [###]

Hadron Polarimetry Systems (6.03.01.11.02) to Detector System (6.03.01)

Prepared by:

{{Sig_es_:signer1:signature}}	Date: {{Dte_es_:signer1:date}}

Name, Role

Electron-Ion Collider at Institution

Reviewed by:

{{Sig_es_:signer2:signature}}	Date: {{Dte_es_:signer2:date}}

Name, Role

Electron-Ion Collider at Institution

{{Sig_es_:signer3:signature}}	Date: {{Dte_es_:signer3:date}}

Name, Role

Electron-Ion Collider at Institution

{{Sig_es_:signer4:signature}}	Date: {{Dte_es_:signer4:date}}

Name, Role

Electron-Ion Collider at Institution

Approved by:

{{Sig_es_:signer5:signature}}	Date: {{Dte_es_:signer5:date}}

Name, Role

Electron-Ion Collider at Institution

REVISION HISTORY

| Revision # | Effective Date | Additional Reviewers | Summary of Change |
| --- | --- | --- | --- |
| 00 |  |  | Initial release. |

TABLE OF CONTENTS

1.	Purpose and Scope	5

2.	Introduction	5

2.1	Hadron Polarimetry Systems	5

2.2	Overview of Interface Relationships	5

2.3	Interface Responsibilities	5

2.4	Safety and Environmental Considerations	5

2.5	Design Considerations	6

2.6	Organization	6

3.	Interface summary	6

4.	Hadron Polarimetry Systems (DET-DS-POL-HPOL) to Data Acquisition and Computing Systems (DET-DS-COMP)	10

5.	Hadron Polarimetry Systems (DET-DS-POL-HPOL) to Electronic Systems (DET-DS-ELEC)	14

6.	Applicable Documents	16

7.	Detailed change Log	16

8.	References	16

LIST OF ACRONYMS

| B0 | B0 is the name of the forward magnet just past the Hadron Endcap |  |
| --- | --- | --- |
| CDR | Conceptual Design Report |  |
| COTS | Commercial Off the Shelf |  |
| DAQ | Data Acquisition |  |
| DET | EIC Detector |  |
| DIRC | Detection of Internally Reflected Cherenkov Light |  |
| dRICH | Dual radiator ring imaging Cherenkov |  |
| DVCS | Deeply Virtual Compton Scattering |  |
| EIC | Electron-Ion Collider |  |
| EMCAL | Electro-magnetic Calorimeter |  |
| ESR | Electron Storage Ring |  |
| FCC | Federal Communications Commission |  |
| FDR | Final Design Review |  |
| GEM | Gas Electron Multiplier |  |
| HCAL | Hadronic Calorimeter |  |
| HSR | Hadron Storage Ring |  |
| HVAC | Heating/Ventilation/Air Conditioning |  |
| IP | Interaction Point |  |
| IR | Interaction Region |  |
| MPGD | Micro Pattern Gaseous Detectors |  |
| NAS | National Academy of Sciences |  |
| NEC | National Electric Code |  |
| NEMA | National Electrical Manufacturers Association |  |
| NFPA | National Fire Protection Association |  |
| NRTL | Nationally Recognized Testing Laboratory |  |
| PID | Particle Identification |  |
| PRR | Production Readiness Review |  |
| PWO | PbWO4 |  |
| RCS | Rapid Cycling Synchrotron |  |
| TBD | To Be Determined |  |
| UL | Underwriter Laboratories |  |
| WBS | Work Break Down Structure |  |

# Interface Control Document:

Hadron Polarimetry Systems (6.03.01.11.02) to Detector System (6.03.01)

# Purpose and Scope

This document records the agreed upon interfaces between the Hadron Polarimetry Systems and other sub-systems within the Detector System. These interfaces are based on the system requirements that are necessary to integrate these sub-systems as part of the Electron Ion Collider Project.

# Introduction

These interfaces describe the relationships between the Hadron Polarimetry Systems and the Detector System including (examples by inclusion) located in (areas by inclusion).

## Hadron Polarimetry Systems

Provide an introductory description of your sub-system and an overview of the things that it receives, provides, supports, constrains, or otherwise requires as part of its function --- specifically if these things are manifested as interfaces.

Data Acquisition and Computing Systems (DET-DS-COMP): Description of the TYPES of interfaces that exist between the ICD reference system and this sub-system, and a BRIEF overview of the roles they play.

Electronic Systems (DET-DS-ELEC): Description of the TYPES of interfaces that exist between the ICD reference system and this sub-system, and a BRIEF overview of the roles they play.

## Overview of Interface Relationships

The following are the sub-systems within the Detector System that have interfaces connected to the Hadron Polarimetry Systems.

## Interface Responsibilities

The various ownerships of responsibilities may be listed here. However, this section might also refer to the Provider section of the interface detail if all pertinent responsibilities are listed there. If this is the case, simply enter: “See the Provider section within the interface details for information about the responsibilities associated with individual interfaces.”

## Safety and Environmental Considerations

General safety and environmental considerations are identified here. Any considerations that are specific to the individual sub-system, will be addressed in external safety documents and are referenced here.

### Considerations for all systems

Add general safety and environmental considerations here.

### Special considerations and references

Add references here as necessary.

## Design Considerations

Notes here about general design considerations.

## Organization

The interfaces in this document are grouped by sub-system as identified in the sub-system identifier. All interfaces between the Hadron Polarimetry Systems and a related sub-system will be included in a single section. Entries within that section will be sorted by the interface identifier or WBS number, as appropriate.

# Interface summary

This tables in this section provide a list of all interfaces that are defined within this document. The interfaces are grouped by sub-system/WBS. Each table has the following fields:

Section: the section number in this document where the interface is addressed.

WBS (1): the WBS number within the ICD reference system, unless the reference system is the owner.

WBS (2): the WBS number for the connected system.

ID: the interface identifier as defined in the Interface Requirement Document (IRD).

Name: the name of the interface as defined in the IRD.

Description: the description of the interface as defined in the IRD.

Owner: identifies the WBS number of the entity that is responsible for ensuring the interface is provided and that it meets validation/verification requirements.

Hadron Polarimetry Systems to Data Acquisition and Computing Systems

| Section | WBS1 | WBS2 | ID | Name | Description | Owner |
| --- | --- | --- | --- | --- | --- | --- |
| 4 |  | 6.03.01.08 |  |  | Data Acquisition and Computing Systems |  |
| 4.1 | 6.03.01.11.02 | 6.03.01.08 | I-DET-DS-COMP-ONLINE.067 | Readout cables IR-4 | Readout cables need to run from the polarimeter detectors to the data acquisition system at IR-4. | 6.03.01.08.02 |
| 4.2 | 6.03.01.11.02 | 6.03.01.08 | I-DET-DS-COMP-ONLINE.068 | Slow Control Database IR-4 | Slow control information should be written to the HSR database (5 second intervals) | 6.03.01.08.02 |
| 4.3 | 6.03.01.11.02 | 6.03.01.08 | I-DET-DS-COMP-ONLINE.069 | Data Transfer and Control Interface | A fiber connection will be provided from the DAQ system to the HJET polarimeter's readout board to perform configuration, control, and data acquisition. | 6.03.01.08.02 |
| 4.4 | 6.03.01.11.02 | 6.03.01.08 | I-DET-DS-COMP-ONLINE.070 | Slow Control Cables IR-4 | Slow control cables for the HJET target system need to run from the hall at IR-4 to the polarimeter in the tunnel. | 6.03.01.08.02 |
| 4.5 | 6.03.01.11.02 | 6.03.01.08 | I-DET-DS-COMP-ONLINE.071 | Slow Controls | A network connection will be provided from the DAQ system to the HJET polarimeter's slow controls interface. | 6.03.01.08.02 |
| 4.6 | 6.03.01.11.02 | 6.03.01.08 | I-DET-DS-COMP-ONLINE.072 | Timing Interface | A fiber connection will be provided from the DAQ system to the HJET polarimeter's readout board for timing synchronization. | 6.03.01.08.02 |
| 4.7 | 6.03.01.11.02 | 6.03.01.08 | I-DET-DS-COMP-ONLINE.073 | Data Transfer and Control Interface | A fiber connection will be provided from the DAQ system to the proton-carbon polarimeter's readout board to perform configuration, control, and data acquisition. | 6.03.01.08.02 |
| 4.8 | 6.03.01.11.02 | 6.03.01.08 | I-DET-DS-COMP-ONLINE.074 | Slow Control Database IR-6 | Results from the target scan polarimeter runs should be written to the HSR database | 6.03.01.08.02 |
| 4.9 | 6.03.01.11.02 | 6.03.01.08 | I-DET-DS-COMP-ONLINE.075 | Slow Controls | A network connection will be provided from the DAQ system to the proton-carbon polarimeter's slow controls interface. | 6.03.01.08.02 |
| 4.10 | 6.03.01.11.02 | 6.03.01.08 | I-DET-DS-COMP-ONLINE.076 | Timing Interface | A fiber connection will be provided from the DAQ system to the proton-carbon polarimeter's readout board for timing synchronization. | 6.03.01.08.02 |
| 4.11 | 6.03.01.11.02 | 6.03.01.08 | I-DET-DS-COMP-ONLINE.077 | Slow Control IR-4 | Slow control cables for the target motion need to run from the hall at IR-4 to the polarimeter. | 6.03.01.08.02 |
| 4.12 | 6.03.01.11.02 | 6.03.01.08 | I-DET-DS-COMP-ONLINE.078 | Target Operation IR-4 | Fiber target operation should be controlled from the HSR main control room | 6.03.01.08.02 |
| 4.13 | 6.03.01.11.02 | 6.03.01.08 | I-DET-DS-COMP-ONLINE.079 | Slow Control IR-6 | Slow control cables for the target motion need to run from the DAQ room at IR-6 to the polarimeter. | 6.03.01.08.02 |
| 4.14 | 6.03.01.11.02 | 6.03.01.08 | I-DET-DS-COMP-ONLINE.080 | Target Operation IR-6 | Fiber target operation should be controlled from the EPIC control room | 6.03.01.08.02 |
| 4.15 | 6.03.01.11.02 | 6.03.01.08 | I-DET-DS-POL-HPOL-PC-IR6.002 | Data Acquisition IR-6 | The data acquisition for the local polarimeter shall be integrated into the experimental readout. | 6.03.01.11.02.02 |

Hadron Polarimetry Systems to Electronic Systems

| Section | WBS1 | WBS2 | ID | Name | Description | Owner |
| --- | --- | --- | --- | --- | --- | --- |
| 5 |  | 6.03.01.07 |  |  | Electronic Systems |  |
| 5.1 | 6.03.01.11.02 | 6.03.01.07 | I-DET-DS-ELEC.063 | Bias Voltage | Bias voltage DC power will be provided from the electronics racks to support electronics the silicon photomultipliers. | 6.03.01.07 |
| 5.2 | 6.03.01.11.02 | 6.03.01.07 | I-DET-DS-ELEC.064 | High Voltage | High voltage DC power will be provided from the electronics racks to support silicon sensors and gas detectors. | 6.03.01.07 |
| 5.3 | 6.03.01.11.02 | 6.03.01.07 | I-DET-DS-ELEC.065 | Low Voltage | Low voltage DC power will be provided from the electronics racks to support electronics in the detector. | 6.03.01.07 |
| 5.4 | 6.03.01.11.02 | 6.03.01.07 | I-DET-DS-ELEC.066 | Bias Voltage | Bias voltage DC power will be provided from the electronics racks to support electronics the silicon photomultipliers. | 6.03.01.07 |
| 5.5 | 6.03.01.11.02 | 6.03.01.07 | I-DET-DS-ELEC.067 | High Voltage | High voltage DC power will be provided from the electronics racks to support silicon sensors and gas detectors. | 6.03.01.07 |
| 5.6 | 6.03.01.11.02 | 6.03.01.07 | I-DET-DS-ELEC.068 | Low Voltage | Low voltage DC power will be provided from the electronics racks to support electronics in the detector. | 6.03.01.07 |

# Hadron Polarimetry Systems (DET-DS-POL-HPOL) to Data Acquisition and Computing Systems (DET-DS-COMP)

I-DET-DS-COMP-ONLINE.067	Readout cables IR-4

System 1:	Online DAQ and Computing Systems (6.03.01.08.02)

System 2:	Hadron Polarimetry (6.03.01.11.02)

Type:	DATA

Description:	Readout cables need to run from the polarimeter detectors to the data acquisition system at IR-4.

Provider:	Online DAQ and Computing Systems (6.03.01.08.02)

Details:	See details in the Data Acquisition and Computing Systems Interface Control Document.

Requirements:	F-DET-DS-COMP.6

References:

I-DET-DS-COMP-ONLINE.068	Slow Control Database IR-4

System 1:	Online DAQ and Computing Systems (6.03.01.08.02)

System 2:	Hadron Polarimetry (6.03.01.11.02)

Type:	DATA

Description:	Slow control information should be written to the HSR database (5 second intervals)

Provider:	Online DAQ and Computing Systems (6.03.01.08.02)

Details:	See details in the Data Acquisition and Computing Systems Interface Control Document.

Requirements:	F-DET-DS-COMP-SC.1

References:

I-DET-DS-COMP-ONLINE.069	Data Transfer and Control Interface

System 1:	Online DAQ and Computing Systems (6.03.01.08.02)

System 2:	HJET Polarimetery (6.03.01.11.02.01)

Type:	DATA

Description:	A fiber connection will be provided from the DAQ system to the HJET polarimeter's readout board to perform configuration, control, and data acquisition.

Provider:	Online DAQ and Computing Systems (6.03.01.08.02)

Details:	See details in the Data Acquisition and Computing Systems Interface Control Document.

Requirements:	F-DET-DS-COMP.6

References:

I-DET-DS-COMP-ONLINE.070	Slow Control Cables IR-4

System 1:	Online DAQ and Computing Systems (6.03.01.08.02)

System 2:	HJET Polarimetery (6.03.01.11.02.01)

Type:	DATA

Description:	Slow control cables for the HJET target system need to run from the hall at IR-4 to the polarimeter in the tunnel.

Provider:	Online DAQ and Computing Systems (6.03.01.08.02)

Details:	See details in the Data Acquisition and Computing Systems Interface Control Document.

Requirements:	F-DET-DS-COMP-SC.1

References:

I-DET-DS-COMP-ONLINE.071	Slow Controls

System 1:	Online DAQ and Computing Systems (6.03.01.08.02)

System 2:	HJET Polarimetery (6.03.01.11.02.01)

Type:	CONTROL

Description:	A network connection will be provided from the DAQ system to the HJET polarimeter's slow controls interface.

Provider:	Online DAQ and Computing Systems (6.03.01.08.02)

Details:	See details in the Data Acquisition and Computing Systems Interface Control Document.

Requirements:	F-DET-DS-COMP-SC.1

References:

I-DET-DS-COMP-ONLINE.072	Timing Interface

System 1:	Online DAQ and Computing Systems (6.03.01.08.02)

System 2:	HJET Polarimetery (6.03.01.11.02.01)

Type:	DATA

Description:	A fiber connection will be provided from the DAQ system to the HJET polarimeter's readout board for timing synchronization.

Provider:	Online DAQ and Computing Systems (6.03.01.08.02)

Details:	See details in the Data Acquisition and Computing Systems Interface Control Document.

Requirements:	F-DET-DS-COMP.11

References:

I-DET-DS-COMP-ONLINE.073	Data Transfer and Control Interface

System 1:	Online DAQ and Computing Systems (6.03.01.08.02)

System 2:	Proton-Carbon Polarimeter (6.03.01.11.02.02)

Type:	DATA

Description:	A fiber connection will be provided from the DAQ system to the proton-carbon polarimeter's readout board to perform configuration, control, and data acquisition.

Provider:	Online DAQ and Computing Systems (6.03.01.08.02)

Details:	See details in the Data Acquisition and Computing Systems Interface Control Document.

Requirements:	F-DET-DS-COMP.6

References:

I-DET-DS-COMP-ONLINE.074	Slow Control Database IR-6

System 1:	Online DAQ and Computing Systems (6.03.01.08.02)

System 2:	Proton-Carbon Polarimeter (6.03.01.11.02.02)

Type:	DATA

Description:	Results from the target scan polarimeter runs should be written to the HSR database

Provider:	Online DAQ and Computing Systems (6.03.01.08.02)

Details:	See details in the Data Acquisition and Computing Systems Interface Control Document.

Requirements:	F-DET-DS-COMP-SC.1

References:

I-DET-DS-COMP-ONLINE.075	Slow Controls

System 1:	Online DAQ and Computing Systems (6.03.01.08.02)

System 2:	Proton-Carbon Polarimeter (6.03.01.11.02.02)

Type:	CONTROL

Description:	A network connection will be provided from the DAQ system to the proton-carbon polarimeter's slow controls interface.

Provider:	Online DAQ and Computing Systems (6.03.01.08.02)

Details:	See details in the Data Acquisition and Computing Systems Interface Control Document.

Requirements:	F-DET-DS-COMP-SC.1

References:

I-DET-DS-COMP-ONLINE.076	Timing Interface

System 1:	Online DAQ and Computing Systems (6.03.01.08.02)

System 2:	Proton-Carbon Polarimeter (6.03.01.11.02.02)

Type:	DATA

Description:	A fiber connection will be provided from the DAQ system to the proton-carbon polarimeter's readout board for timing synchronization.

Provider:	Online DAQ and Computing Systems (6.03.01.08.02)

Details:	See details in the Data Acquisition and Computing Systems Interface Control Document.

Requirements:	F-DET-DS-COMP.11

References:

I-DET-DS-COMP-ONLINE.077	Slow Control IR-4

System 1:	Online DAQ and Computing Systems (6.03.01.08.02)

System 2:	Proton-Carbon Polarimeter in IR 4 (6.03.01.11.02.02)

Type:	CONTROL

Description:	Slow control cables for the target motion need to run from the hall at IR-4 to the polarimeter.

Provider:	Online DAQ and Computing Systems (6.03.01.08.02)

Details:	See details in the Data Acquisition and Computing Systems Interface Control Document.

Requirements:	F-DET-DS-COMP-SC.1

References:

I-DET-DS-COMP-ONLINE.078	Target Operation IR-4

System 1:	Online DAQ and Computing Systems (6.03.01.08.02)

System 2:	Proton-Carbon Polarimeter in IR 4 (6.03.01.11.02.02)

Type:	CONTROL

Description:	Fiber target operation should be controlled from the HSR main control room

Provider:	Online DAQ and Computing Systems (6.03.01.08.02)

Details:	See details in the Data Acquisition and Computing Systems Interface Control Document.

Requirements:	F-DET-DS-COMP.6

References:

I-DET-DS-COMP-ONLINE.079	Slow Control IR-6

System 1:	Online DAQ and Computing Systems (6.03.01.08.02)

System 2:	Proton-Carbon Polarimeter in IR 6 (6.03.01.11.02.02)

Type:	CONTROL

Description:	Slow control cables for the target motion need to run from the DAQ room at IR-6 to the polarimeter.

Provider:	Online DAQ and Computing Systems (6.03.01.08.02)

Details:	See details in the Data Acquisition and Computing Systems Interface Control Document.

Requirements:	F-DET-DS-COMP-SC.1

References:

I-DET-DS-COMP-ONLINE.080	Target Operation IR-6

System 1:	Online DAQ and Computing Systems (6.03.01.08.02)

System 2:	Proton-Carbon Polarimeter in IR 6 (6.03.01.11.02.02)

Type:	CONTROL

Description:	Fiber target operation should be controlled from the EPIC control room

Provider:	Online DAQ and Computing Systems (6.03.01.08.02)

Details:	See details in the Data Acquisition and Computing Systems Interface Control Document.

Requirements:	F-DET-DS-COMP.6

References:

I-DET-DS-POL-HPOL-PC-IR6.002	Data Acquisition IR-6

System 1:	Proton-Carbon Polarimeter in IR 6 (6.03.01.11.02.02)

System 2:	Online DAQ and Computing Systems (6.03.01.08.02)

Type:	SPACE

Description:	The data acquisition for the local polarimeter shall be integrated into the experimental readout.

Provider:	Proton-Carbon Polarimeter in IR 6 (6.03.01.11.02.02)

Details:	See details in the Polarimetry and Luminosity Interface Control Document.

Requirements:	P-DET-DS-POL.1

References:

# Hadron Polarimetry Systems (DET-DS-POL-HPOL) to Electronic Systems (DET-DS-ELEC)

I-DET-DS-ELEC.063	Bias Voltage

System 1:	Electronic Systems (6.03.01.07)

System 2:	HJET Polarimetery (6.03.01.11.02.01)

Type:	ELEC

Description:	Bias voltage DC power will be provided from the electronics racks to support electronics the silicon photomultipliers.

Provider:	Electronic Systems (6.03.01.07)

Details:	See details in the Electronic Systems Interface Control Document.

Requirements:	P-DET-DS-POL.4

References:

I-DET-DS-ELEC.064	High Voltage

System 1:	Electronic Systems (6.03.01.07)

System 2:	HJET Polarimetery (6.03.01.11.02.01)

Type:	ELEC

Description:	High voltage DC power will be provided from the electronics racks to support silicon sensors and gas detectors.

Provider:	Electronic Systems (6.03.01.07)

Details:	See details in the Electronic Systems Interface Control Document.

Requirements:	P-DET-DS-POL.4

References:

I-DET-DS-ELEC.065	Low Voltage

System 1:	Electronic Systems (6.03.01.07)

System 2:	HJET Polarimetery (6.03.01.11.02.01)

Type:	ELEC

Description:	Low voltage DC power will be provided from the electronics racks to support electronics in the detector.

Provider:	Electronic Systems (6.03.01.07)

Details:	See details in the Electronic Systems Interface Control Document.

Requirements:	P-DET-DS-POL.4

References:

I-DET-DS-ELEC.066	Bias Voltage

System 1:	Electronic Systems (6.03.01.07)

System 2:	Proton-Carbon Polarimeter (6.03.01.11.02.02)

Type:	ELEC

Description:	Bias voltage DC power will be provided from the electronics racks to support electronics the silicon photomultipliers.

Provider:	Electronic Systems (6.03.01.07)

Details:	See details in the Electronic Systems Interface Control Document.

Requirements:	P-DET-DS-POL.4

References:

I-DET-DS-ELEC.067	High Voltage

System 1:	Electronic Systems (6.03.01.07)

System 2:	Proton-Carbon Polarimeter (6.03.01.11.02.02)

Type:	ELEC

Description:	High voltage DC power will be provided from the electronics racks to support silicon sensors and gas detectors.

Provider:	Electronic Systems (6.03.01.07)

Details:	See details in the Electronic Systems Interface Control Document.

Requirements:	P-DET-DS-POL.4

References:

I-DET-DS-ELEC.068	Low Voltage

System 1:	Electronic Systems (6.03.01.07)

System 2:	Proton-Carbon Polarimeter (6.03.01.11.02.02)

Type:	ELEC

Description:	Low voltage DC power will be provided from the electronics racks to support electronics in the detector.

Provider:	Electronic Systems (6.03.01.07)

Details:	See details in the Electronic Systems Interface Control Document.

Requirements:	P-DET-DS-POL.4

References:

# Applicable Documents

List applicable documents considered to be part of this document. Applicable documents shall be obtained from the EIC SharePoint Document Center.

# Detailed change Log

| Revision | Date | Change |
| --- | --- | --- |

# References

- doi:10.17226/25171., National Academies of Sciences, Engineering, and Medicine, "An Assessment of U.S.-Based Electron-Ion Collider Science", The National Academies Press, Washington, DC, 2018.

- EIC-ORG-PLN-010, Electron-Ion Collider L1 Requirements (Global Requirements Document)

- BNL-221006-2021-FORE, EIC Conceptual Design Report

- arXiv:2103.05419, Science Requirements and Detector Concepts for the Electron-Ion Collider: EIC Yellow Report

- EIC-SEG-PLN-016, Requirements Management Plan

- EIC-SEG-PLN-022, Systems Engineering Management Plan

- EIC-SEG-PLN-020, EIC Interface Management Plan