Electron-Ion Collider, Requirements, Specifications, and Interfaces

Interface Control Document: [###]

Tracking Systems (6.03.01.02) to Detector System (6.03.01)

Prepared by:

{{Sig_es_:signer1:signature}}	Date: {{Dte_es_:signer1:date}}

Name, Role

Electron-Ion Collider at Institution

Reviewed by:

{{Sig_es_:signer2:signature}}	Date: {{Dte_es_:signer2:date}}

Name, Role

Electron-Ion Collider at Institution

{{Sig_es_:signer3:signature}}	Date: {{Dte_es_:signer3:date}}

Name, Role

Electron-Ion Collider at Institution

{{Sig_es_:signer4:signature}}	Date: {{Dte_es_:signer4:date}}

Name, Role

Electron-Ion Collider at Institution

Approved by:

{{Sig_es_:signer5:signature}}	Date: {{Dte_es_:signer5:date}}

Name, Role

Electron-Ion Collider at Institution

REVISION HISTORY

| Revision # | Effective Date | Additional Reviewers | Summary of Change |
| --- | --- | --- | --- |
| 00 |  |  | Initial release. |

TABLE OF CONTENTS

1.	Purpose and Scope	5

2.	Introduction	5

2.1	Tracking Systems	5

2.2	Overview of Interface Relationships	5

2.3	Interface Responsibilities	5

2.4	Safety and Environmental Considerations	5

2.5	Design Considerations	6

2.6	Organization	6

3.	Interface summary	6

4.	Tracking Systems (DET-DS-TRAK) to Data Acquisition and Computing Systems (DET-DS-COMP)	12

5.	Tracking Systems (DET-DS-TRAK) to Electronic Systems (DET-DS-ELEC)	13

6.	Tracking Systems (DET-DS-TRAK) to Detector Infrastructure (DET-DS-INF)	14

7.	Tracking Systems (DET-DS-TRAK) to Particle Identification Systems (DET-DS-PID)	17

8.	Applicable Documents	19

9.	Detailed change Log	19

10.	References	20

LIST OF ACRONYMS

| B0 | B0 is the name of the forward magnet just past the Hadron Endcap |  |
| --- | --- | --- |
| CDR | Conceptual Design Report |  |
| COTS | Commercial Off the Shelf |  |
| DAQ | Data Acquisition |  |
| DET | EIC Detector |  |
| DIRC | Detection of Internally Reflected Cherenkov Light |  |
| dRICH | Dual radiator ring imaging Cherenkov |  |
| DVCS | Deeply Virtual Compton Scattering |  |
| EIC | Electron-Ion Collider |  |
| EMCAL | Electro-magnetic Calorimeter |  |
| ESR | Electron Storage Ring |  |
| FCC | Federal Communications Commission |  |
| FDR | Final Design Review |  |
| GEM | Gas Electron Multiplier |  |
| HCAL | Hadronic Calorimeter |  |
| HSR | Hadron Storage Ring |  |
| HVAC | Heating/Ventilation/Air Conditioning |  |
| IP | Interaction Point |  |
| IR | Interaction Region |  |
| MPGD | Micro Pattern Gaseous Detectors |  |
| NAS | National Academy of Sciences |  |
| NEC | National Electric Code |  |
| NEMA | National Electrical Manufacturers Association |  |
| NFPA | National Fire Protection Association |  |
| NRTL | Nationally Recognized Testing Laboratory |  |
| PID | Particle Identification |  |
| PRR | Production Readiness Review |  |
| PWO | PbWO4 |  |
| RCS | Rapid Cycling Synchrotron |  |
| TBD | To Be Determined |  |
| UL | Underwriter Laboratories |  |
| WBS | Work Break Down Structure |  |

# Interface Control Document:

Tracking Systems (6.03.01.02) to Detector System (6.03.01)

# Purpose and Scope

This document records the agreed upon interfaces between the Tracking Systems and other sub-systems within the Detector System. These interfaces are based on the system requirements that are necessary to integrate these sub-systems as part of the Electron Ion Collider Project.

# Introduction

These interfaces describe the relationships between the Tracking Systems and the Detector System including (examples by inclusion) located in (areas by inclusion).

## Tracking Systems

Provide an introductory description of your sub-system and an overview of the things that it receives, provides, supports, constrains, or otherwise requires as part of its function --- specifically if these things are manifested as interfaces.

Data Acquisition and Computing Systems (DET-DS-COMP): Description of the TYPES of interfaces that exist between the ICD reference system and this sub-system, and a BRIEF overview of the roles they play.

Electronic Systems (DET-DS-ELEC): Description of the TYPES of interfaces that exist between the ICD reference system and this sub-system, and a BRIEF overview of the roles they play.

Detector Infrastructure (DET-DS-INF): Description of the TYPES of interfaces that exist between the ICD reference system and this sub-system, and a BRIEF overview of the roles they play.

Particle Identification Systems (DET-DS-PID): Description of the TYPES of interfaces that exist between the ICD reference system and this sub-system, and a BRIEF overview of the roles they play.

## Overview of Interface Relationships

The following are the sub-systems within the Detector System that have interfaces connected to the Tracking Systems.

## Interface Responsibilities

The various ownerships of responsibilities may be listed here. However, this section might also refer to the Provider section of the interface detail if all pertinent responsibilities are listed there. If this is the case, simply enter: “See the Provider section within the interface details for information about the responsibilities associated with individual interfaces.”

## Safety and Environmental Considerations

General safety and environmental considerations are identified here. Any considerations that are specific to the individual sub-system, will be addressed in external safety documents and are referenced here.

### Considerations for all systems

Add general safety and environmental considerations here.

### Special considerations and references

Add references here as necessary.

## Design Considerations

Notes here about general design considerations.

## Organization

The interfaces in this document are grouped by sub-system as identified in the sub-system identifier. All interfaces between the Tracking Systems and a related sub-system will be included in a single section. Entries within that section will be sorted by the interface identifier or WBS number, as appropriate.

# Interface summary

This tables in this section provide a list of all interfaces that are defined within this document. The interfaces are grouped by sub-system/WBS. Each table has the following fields:

Section: the section number in this document where the interface is addressed.

WBS (1): the WBS number within the ICD reference system, unless the reference system is the owner.

WBS (2): the WBS number for the connected system.

ID: the interface identifier as defined in the Interface Requirement Document (IRD).

Name: the name of the interface as defined in the IRD.

Description: the description of the interface as defined in the IRD.

Owner: identifies the WBS number of the entity that is responsible for ensuring the interface is provided and that it meets validation/verification requirements.

Tracking Systems to Data Acquisition and Computing Systems

| Section | WBS1 | WBS2 | ID | Name | Description | Owner |
| --- | --- | --- | --- | --- | --- | --- |
| 4 |  | 6.03.01.08 |  |  | Data Acquisition and Computing Systems |  |
| 4.1 | 6.03.01.02 | 6.03.01.08 | I-DET-DS-COMP-ONLINE.081 | Data Acquisition | Cables required to transfer data from the detector to the online data acquisition system. | 6.03.01.08.02 |
| 4.2 | 6.03.01.02 | 6.03.01.08 | I-DET-DS-COMP-ONLINE.082 | Data Transfer and Control Interface | A fiber connection will be provided from the DAQ system to the barrel tracking system's readout board to perform configuration, control, and data acquisition. | 6.03.01.08.02 |
| 4.3 | 6.03.01.02 | 6.03.01.08 | I-DET-DS-COMP-ONLINE.083 | Slow Controls | A network connection will be provided from the DAQ system to the barrel tracking system's slow controls interface. | 6.03.01.08.02 |
| 4.4 | 6.03.01.02 | 6.03.01.08 | I-DET-DS-COMP-ONLINE.084 | Timing Interface | A fiber connection will be provided from the DAQ system to the barrel tracking system's readout board for timing synchronization. | 6.03.01.08.02 |

Tracking Systems to Electronic Systems

| Section | WBS1 | WBS2 | ID | Name | Description | Owner |
| --- | --- | --- | --- | --- | --- | --- |
| 5 |  | 6.03.01.07 |  |  | Electronic Systems |  |
| 5.1 | 6.03.01.02 | 6.03.01.07 | I-DET-DS-ELEC.069 | Bias Voltage | Bias voltage DC power will be provided from the electronics racks to support electronics in the silicon photomultipliers. | 6.03.01.07 |
| 5.2 | 6.03.01.02 | 6.03.01.07 | I-DET-DS-ELEC.070 | High Voltage | High voltage DC power will be provided from the electronics racks to support silicon sensors and gas detectors. | 6.03.01.07 |
| 5.3 | 6.03.01.02 | 6.03.01.07 | I-DET-DS-ELEC.071 | Low Voltage | Low voltage DC power will be provided from the electronics racks to support electronics in the detector. | 6.03.01.07 |

Tracking Systems to Detector Infrastructure

| Section | WBS1 | WBS2 | ID | Name | Description | Owner |
| --- | --- | --- | --- | --- | --- | --- |
| 6 |  | 6.03.01.09 |  |  | Detector Infrastructure |  |
| 6.1 | 6.03.01.02 | 6.03.01.09 | I-DET-DS-INF-BAR.016 | Support Structure | A single structural support system will support the silicon detectors and the micro-pattern gaseous detectors within the DIRC detector. | 6.03.01.09.01 |
| 6.2 | 6.03.01.02 | 6.03.01.09 | I-DET-DS-INF-BAR.021 | Silicon Tracker Support System | The DIRC support system will provide support for the silicon trackers in the detector barrel. | 6.03.01.09.01 |
| 6.3 | 6.03.01.02 | 6.03.01.09 | I-DET-DS-INF-BAR.022 | Weight Distributed | The weight of the barrel tracking systems will be transferred to the DIRC support system. | 6.03.01.09.01 |
| 6.4 | 6.03.01.02 | 6.03.01.09 | I-DET-DS-INF-FWD.012 | Space Constraint | The maximum forward location for the tracking system is limited by the position of the AC LGAD Time of Flight Detector. Modifications to either must be coordinated. | 6.03.01.09.03 |
| 6.5 | 6.03.01.02 | 6.03.01.09 | I-DET-DS-INF-FWD.013 | Space Constraint | The interior radius of the tracking detectors is governed by the size of the beamline. | 6.03.01.09.03 |
| 6.6 | 6.03.01.02 | 6.03.01.09 | I-DET-DS-INF-INT.094 | Heat Rejection | Air, liquid or other cooling technology will be required for the tracking detectors. | 6.03.01.09.04 |
| 6.7 | 6.03.01.02 | 6.03.01.09 | I-DET-DS-INF-INT.095 | Ionization Gas | Gas will need to be provided to the trackers for detector operation. | 6.03.01.09.04 |
| 6.8 | 6.03.01.02 | 6.03.01.09 | I-DET-DS-INF-INT.096 | Support Structure | A single structural support system will support the silicon detectors and the micro-pattern gaseous detectors within the DIRC detector. | 6.03.01.09.04 |
| 6.9 | 6.03.01.02 | 6.03.01.09 | I-DET-DS-INF-INT.101 | Service Pathway | Conduits must be provided within the DIRC support system that are adequate to deliver services (power, signal, cooling) to the barrel tracking detectors. | 6.03.01.09.04 |
| 6.10 | 6.03.01.02 | 6.03.01.09 | I-DET-DS-INF-INT.102 | Space Constraint | The exterior radius of the silicon trackers is limited by the interior bore of the DIRC support system. Modifications to either must be coordinated. | 6.03.01.09.04 |
| 6.11 | 6.03.01.02 | 6.03.01.09 | I-DET-DS-INF-INT.103 | Space Constraint | The maximum forward location for the DIRC is limited by the position of the silicon trackers. | 6.03.01.09.04 |
| 6.12 | 6.03.01.02 | 6.03.01.09 | I-DET-DS-INF-INT.106 | Space Constraint | The maximum size of the silicon trackers is limited by the interior radius of the barrel Time of Flight detector. Modifications to either must be coordinated. | 6.03.01.09.04 |
| 6.13 | 6.03.01.02 | 6.03.01.09 | I-DET-DS-INF-INT.107 | Space Constraint | The maximum backward location for the tracking system is limited by the position of the pfRICH/mRICH. Modifications to either must be coordinated. | 6.03.01.09.04 |

Tracking Systems to Particle Identification Systems

| Section | WBS1 | WBS2 | ID | Name | Description | Owner |
| --- | --- | --- | --- | --- | --- | --- |
| 7 |  | 6.03.01.03 |  |  | Particle Identification Systems |  |
| 7.1 | 6.03.01.02 | 6.03.01.03 | I-DET-DS-INF-BAR.021 | Silicon Tracker Support System | The DIRC support system will provide support for the silicon trackers in the detector barrel. | 6.03.01.09.01 |
| 7.2 | 6.03.01.02 | 6.03.01.03 | I-DET-DS-INF-BAR.022 | Weight Distributed | The weight of the barrel tracking systems will be transferred to the DIRC support system. | 6.03.01.09.01 |
| 7.3 | 6.03.01.02 | 6.03.01.03 | I-DET-DS-INF-FWD.012 | Space Constraint | The maximum forward location for the tracking system is limited by the position of the AC LGAD Time of Flight Detector. Modifications to either must be coordinated. | 6.03.01.09.03 |
| 7.4 | 6.03.01.02 | 6.03.01.03 | I-DET-DS-INF-INT.101 | Service Pathway | Conduits must be provided within the DIRC support system that are adequate to deliver services (power, signal, cooling) to the barrel tracking detectors. | 6.03.01.09.04 |
| 7.5 | 6.03.01.02 | 6.03.01.03 | I-DET-DS-INF-INT.102 | Space Constraint | The exterior radius of the silicon trackers is limited by the interior bore of the DIRC support system. Modifications to either must be coordinated. | 6.03.01.09.04 |
| 7.6 | 6.03.01.02 | 6.03.01.03 | I-DET-DS-INF-INT.103 | Space Constraint | The maximum forward location for the DIRC is limited by the position of the silicon trackers. | 6.03.01.09.04 |
| 7.7 | 6.03.01.02 | 6.03.01.03 | I-DET-DS-INF-INT.106 | Space Constraint | The maximum size of the silicon trackers is limited by the interior radius of the barrel Time of Flight detector. Modifications to either must be coordinated. | 6.03.01.09.04 |
| 7.8 | 6.03.01.02 | 6.03.01.03 | I-DET-DS-INF-INT.107 | Space Constraint | The maximum backward location for the tracking system is limited by the position of the pfRICH/mRICH. Modifications to either must be coordinated. | 6.03.01.09.04 |

# Tracking Systems (DET-DS-TRAK) to Data Acquisition and Computing Systems (DET-DS-COMP)

I-DET-DS-COMP-ONLINE.081	Data Acquisition

System 1:	Online DAQ and Computing Systems (6.03.01.08.02)

System 2:	Barrel Tracking Systems (6.03.01.02)

Type:	DATA

Description:	Cables required to transfer data from the detector to the online data acquisition system.

Provider:	Online DAQ and Computing Systems (6.03.01.08.02)

Details:	See details in the Data Acquisition and Computing Systems Interface Control Document.

Requirements:	F-DET-DS-COMP.6

References:

I-DET-DS-COMP-ONLINE.082	Data Transfer and Control Interface

System 1:	Online DAQ and Computing Systems (6.03.01.08.02)

System 2:	Barrel Tracking Systems (6.03.01.02)

Type:	DATA

Description:	A fiber connection will be provided from the DAQ system to the barrel tracking system's readout board to perform configuration, control, and data acquisition.

Provider:	Online DAQ and Computing Systems (6.03.01.08.02)

Details:	See details in the Data Acquisition and Computing Systems Interface Control Document.

Requirements:	F-DET-DS-COMP.6

References:

I-DET-DS-COMP-ONLINE.083	Slow Controls

System 1:	Online DAQ and Computing Systems (6.03.01.08.02)

System 2:	Barrel Tracking Systems (6.03.01.02)

Type:	CONTROL

Description:	A network connection will be provided from the DAQ system to the barrel tracking system's slow controls interface.

Provider:	Online DAQ and Computing Systems (6.03.01.08.02)

Details:	See details in the Data Acquisition and Computing Systems Interface Control Document.

Requirements:	F-DET-DS-COMP-SC.1

References:

I-DET-DS-COMP-ONLINE.084	Timing Interface

System 1:	Online DAQ and Computing Systems (6.03.01.08.02)

System 2:	Barrel Tracking Systems (6.03.01.02)

Type:	DATA

Description:	A fiber connection will be provided from the DAQ system to the barrel tracking system's readout board for timing synchronization.

Provider:	Online DAQ and Computing Systems (6.03.01.08.02)

Details:	See details in the Data Acquisition and Computing Systems Interface Control Document.

Requirements:	F-DET-DS-COMP.11

References:

# Tracking Systems (DET-DS-TRAK) to Electronic Systems (DET-DS-ELEC)

I-DET-DS-ELEC.069	Bias Voltage

System 1:	Electronic Systems (6.03.01.07)

System 2:	Barrel Tracking Systems (6.03.01.02)

Type:	ELEC

Description:	Bias voltage DC power will be provided from the electronics racks to support electronics in the silicon photomultipliers.

Provider:	Electronic Systems (6.03.01.07)

Details:	See details in the Electronic Systems Interface Control Document.

Requirements:	P-DET-DS-TRAK.6

References:

I-DET-DS-ELEC.070	High Voltage

System 1:	Electronic Systems (6.03.01.07)

System 2:	Barrel Tracking Systems (6.03.01.02)

Type:	ELEC

Description:	High voltage DC power will be provided from the electronics racks to support silicon sensors and gas detectors.

Provider:	Electronic Systems (6.03.01.07)

Details:	See details in the Electronic Systems Interface Control Document.

Requirements:	P-DET-DS-TRAK.6

References:

I-DET-DS-ELEC.071	Low Voltage

System 1:	Electronic Systems (6.03.01.07)

System 2:	Barrel Tracking Systems (6.03.01.02)

Type:	ELEC

Description:	Low voltage DC power will be provided from the electronics racks to support electronics in the detector.

Provider:	Electronic Systems (6.03.01.07)

Details:	See details in the Electronic Systems Interface Control Document.

Requirements:	P-DET-DS-TRAK.6

References:

# Tracking Systems (DET-DS-TRAK) to Detector Infrastructure (DET-DS-INF)

I-DET-DS-INF-BAR.016	Support Structure

System 1:	Barrel Detector Structures (6.03.01.09.01)

System 2:	Barrel Tracking Systems (6.03.01.02)

Type:	STRUCT

Description:	A single structural support system will support the silicon detectors and the micro-pattern gaseous detectors within the DIRC detector.

Provider:	Barrel Detector Structures (6.03.01.09.01)

Details:	See details in the Detector Infrastructure Interface Control Document.

Requirements:	P-DET-DS-TRAK.8

References:

I-DET-DS-INF-BAR.021	Silicon Tracker Support System

System 1:	Barrel DIRC Particle ID Systems (6.03.01.03)

System 2:	Tracking Systems (6.03.01.02)

Type:	STRUCT

Description:	The DIRC support system will provide support for the silicon trackers in the detector barrel.

Provider:	Barrel Detector Structures (6.03.01.09.01)

Details:	See details in the Detector Infrastructure Interface Control Document.

Requirements:	P-DET-DS-PID.4, P-DET-DS-PID.5, P-DET-DS-TRAK.8, P-DET-DS-TRAK.9

References:

I-DET-DS-INF-BAR.022	Weight Distributed

System 1:	Barrel DIRC Particle ID Systems (6.03.01.03)

System 2:	Barrel Tracking Systems (6.03.01.02)

Type:	STRUCT

Description:	The weight of the barrel tracking systems will be transferred to the DIRC support system.

Provider:	Barrel Detector Structures (6.03.01.09.01)

Details:	See details in the Detector Infrastructure Interface Control Document.

Requirements:	P-DET-DS-PID.4, P-DET-DS-TRAK.8

References:

I-DET-DS-INF-FWD.012	Space Constraint

System 1:	Forward Time of Flight Systems (6.03.01.03)

System 2:	Barrel Tracking Systems (6.03.01.02)

Type:	SPACE

Description:	The maximum forward location for the tracking system is limited by the position of the AC LGAD Time of Flight Detector. Modifications to either must be coordinated.

Provider:	Hadron Endcap Structures (6.03.01.09.03)

Details:	See details in the Detector Infrastructure Interface Control Document.

Requirements:	P-DET-DS-PID.5, P-DET-DS-TRAK.9

References:

I-DET-DS-INF-FWD.013	Space Constraint

System 1:	Barrel Tracking Systems (6.03.01.02)

System 2:	IR Accelerator Systems (6.04.03)

Type:	SPACE

Description:	The interior radius of the tracking detectors is governed by the size of the beamline.

Provider:	Hadron Endcap Structures (6.03.01.09.03)

Details:	See details in the Detector Infrastructure Interface Control Document.

Requirements:	P-DET-DS-TRAK.9

References:

I-DET-DS-INF-INT.094	Heat Rejection

System 1:	Detector Infrastructure and Utilities Integration (6.03.01.09.04)

System 2:	Barrel Tracking Systems (6.03.01.02)

Type:	COOL

Description:	Air, liquid or other cooling technology will be required for the tracking detectors.

Provider:	Detector Infrastructure and Utilities Integration (6.03.01.09.04)

Details:	See details in the Detector Infrastructure Interface Control Document.

Requirements:	P-DET-DS-TRAK.5

References:

I-DET-DS-INF-INT.095	Ionization Gas

System 1:	Detector Infrastructure and Utilities Integration (6.03.01.09.04)

System 2:	Barrel Tracking Systems (6.03.01.02)

Type:	GAS/FLUID

Description:	Gas will need to be provided to the trackers for detector operation.

Provider:	Detector Infrastructure and Utilities Integration (6.03.01.09.04)

Details:	See details in the Detector Infrastructure Interface Control Document.

Requirements:	P-DET-DS-TRAK.4

References:

I-DET-DS-INF-INT.096	Support Structure

System 1:	Detector Infrastructure and Utilities Integration (6.03.01.09.04)

System 2:	Barrel Tracking Systems (6.03.01.02)

Type:	STRUCT

Description:	A single structural support system will support the silicon detectors and the micro-pattern gaseous detectors within the DIRC detector.

Provider:	Detector Infrastructure and Utilities Integration (6.03.01.09.04)

Details:	See details in the Detector Infrastructure Interface Control Document.

Requirements:	P-DET-DS-TRAK.8

References:

I-DET-DS-INF-INT.101	Service Pathway

System 1:	Barrel DIRC Particle ID Systems (6.03.01.03)

System 2:	Barrel Tracking Systems (6.03.01.02)

Type:	SPACE

Description:	Conduits must be provided within the DIRC support system that are adequate to deliver services (power, signal, cooling) to the barrel tracking detectors.

Provider:	Detector Infrastructure and Utilities Integration (6.03.01.09.04)

Details:	See details in the Detector Infrastructure Interface Control Document.

Requirements:	P-DET-DS-PID.5, P-DET-DS-TRAK.9

References:

I-DET-DS-INF-INT.102	Space Constraint

System 1:	Barrel DIRC Particle ID Systems (6.03.01.03)

System 2:	Barrel Tracking Systems (6.03.01.02)

Type:	SPACE

Description:	The exterior radius of the silicon trackers is limited by the interior bore of the DIRC support system. Modifications to either must be coordinated.

Provider:	Detector Infrastructure and Utilities Integration (6.03.01.09.04)

Details:	See details in the Detector Infrastructure Interface Control Document.

Requirements:	P-DET-DS-PID.5, P-DET-DS-TRAK.9

References:

I-DET-DS-INF-INT.103	Space Constraint

System 1:	Barrel DIRC Particle ID Systems (6.03.01.03)

System 2:	Barrel Tracking Systems (6.03.01.02)

Type:	SPACE

Description:	The maximum forward location for the DIRC is limited by the position of the silicon trackers.

Provider:	Detector Infrastructure and Utilities Integration (6.03.01.09.04)

Details:	See details in the Detector Infrastructure Interface Control Document.

Requirements:	P-DET-DS-PID.5, P-DET-DS-TRAK.9

References:

I-DET-DS-INF-INT.106	Space Constraint

System 1:	Barrel TOF Particle ID Systems (6.03.01.03)

System 2:	Barrel Tracking Systems (6.03.01.02)

Type:	SPACE

Description:	The maximum size of the silicon trackers is limited by the interior radius of the barrel Time of Flight detector. Modifications to either must be coordinated.

Provider:	Detector Infrastructure and Utilities Integration (6.03.01.09.04)

Details:	See details in the Detector Infrastructure Interface Control Document.

Requirements:	P-DET-DS-PID.5, P-DET-DS-TRAK.9

References:

I-DET-DS-INF-INT.107	Space Constraint

System 1:	Backward RICH Detector Systems (6.03.01.03)

System 2:	Barrel Tracking Systems (6.03.01.02)

Type:	SPACE

Description:	The maximum backward location for the tracking system is limited by the position of the pfRICH/mRICH. Modifications to either must be coordinated.

Provider:	Detector Infrastructure and Utilities Integration (6.03.01.09.04)

Details:	See details in the Detector Infrastructure Interface Control Document.

Requirements:	P-DET-DS-PID.5, P-DET-DS-TRAK.9

References:

# Tracking Systems (DET-DS-TRAK) to Particle Identification Systems (DET-DS-PID)

I-DET-DS-INF-BAR.021	Silicon Tracker Support System

System 1:	Barrel DIRC Particle ID Systems (6.03.01.03)

System 2:	Tracking Systems (6.03.01.02)

Type:	STRUCT

Description:	The DIRC support system will provide support for the silicon trackers in the detector barrel.

Provider:	Barrel Detector Structures (6.03.01.09.01)

Details:	See details in the Detector Infrastructure Interface Control Document.

Requirements:	P-DET-DS-PID.4, P-DET-DS-PID.5, P-DET-DS-TRAK.8, P-DET-DS-TRAK.9

References:

I-DET-DS-INF-BAR.022	Weight Distributed

System 1:	Barrel DIRC Particle ID Systems (6.03.01.03)

System 2:	Barrel Tracking Systems (6.03.01.02)

Type:	STRUCT

Description:	The weight of the barrel tracking systems will be transferred to the DIRC support system.

Provider:	Barrel Detector Structures (6.03.01.09.01)

Details:	See details in the Detector Infrastructure Interface Control Document.

Requirements:	P-DET-DS-PID.4, P-DET-DS-TRAK.8

References:

I-DET-DS-INF-FWD.012	Space Constraint

System 1:	Forward Time of Flight Systems (6.03.01.03)

System 2:	Barrel Tracking Systems (6.03.01.02)

Type:	SPACE

Description:	The maximum forward location for the tracking system is limited by the position of the AC LGAD Time of Flight Detector. Modifications to either must be coordinated.

Provider:	Hadron Endcap Structures (6.03.01.09.03)

Details:	See details in the Detector Infrastructure Interface Control Document.

Requirements:	P-DET-DS-PID.5, P-DET-DS-TRAK.9

References:

I-DET-DS-INF-INT.101	Service Pathway

System 1:	Barrel DIRC Particle ID Systems (6.03.01.03)

System 2:	Barrel Tracking Systems (6.03.01.02)

Type:	SPACE

Description:	Conduits must be provided within the DIRC support system that are adequate to deliver services (power, signal, cooling) to the barrel tracking detectors.

Provider:	Detector Infrastructure and Utilities Integration (6.03.01.09.04)

Details:	See details in the Detector Infrastructure Interface Control Document.

Requirements:	P-DET-DS-PID.5, P-DET-DS-TRAK.9

References:

I-DET-DS-INF-INT.102	Space Constraint

System 1:	Barrel DIRC Particle ID Systems (6.03.01.03)

System 2:	Barrel Tracking Systems (6.03.01.02)

Type:	SPACE

Description:	The exterior radius of the silicon trackers is limited by the interior bore of the DIRC support system. Modifications to either must be coordinated.

Provider:	Detector Infrastructure and Utilities Integration (6.03.01.09.04)

Details:	See details in the Detector Infrastructure Interface Control Document.

Requirements:	P-DET-DS-PID.5, P-DET-DS-TRAK.9

References:

I-DET-DS-INF-INT.103	Space Constraint

System 1:	Barrel DIRC Particle ID Systems (6.03.01.03)

System 2:	Barrel Tracking Systems (6.03.01.02)

Type:	SPACE

Description:	The maximum forward location for the DIRC is limited by the position of the silicon trackers.

Provider:	Detector Infrastructure and Utilities Integration (6.03.01.09.04)

Details:	See details in the Detector Infrastructure Interface Control Document.

Requirements:	P-DET-DS-PID.5, P-DET-DS-TRAK.9

References:

I-DET-DS-INF-INT.106	Space Constraint

System 1:	Barrel TOF Particle ID Systems (6.03.01.03)

System 2:	Barrel Tracking Systems (6.03.01.02)

Type:	SPACE

Description:	The maximum size of the silicon trackers is limited by the interior radius of the barrel Time of Flight detector. Modifications to either must be coordinated.

Provider:	Detector Infrastructure and Utilities Integration (6.03.01.09.04)

Details:	See details in the Detector Infrastructure Interface Control Document.

Requirements:	P-DET-DS-PID.5, P-DET-DS-TRAK.9

References:

I-DET-DS-INF-INT.107	Space Constraint

System 1:	Backward RICH Detector Systems (6.03.01.03)

System 2:	Barrel Tracking Systems (6.03.01.02)

Type:	SPACE

Description:	The maximum backward location for the tracking system is limited by the position of the pfRICH/mRICH. Modifications to either must be coordinated.

Provider:	Detector Infrastructure and Utilities Integration (6.03.01.09.04)

Details:	See details in the Detector Infrastructure Interface Control Document.

Requirements:	P-DET-DS-PID.5, P-DET-DS-TRAK.9

References:

# Applicable Documents

List applicable documents considered to be part of this document. Applicable documents shall be obtained from the EIC SharePoint Document Center.

# Detailed change Log

| Revision | Date | Change |
| --- | --- | --- |

# References

- doi:10.17226/25171., National Academies of Sciences, Engineering, and Medicine, "An Assessment of U.S.-Based Electron-Ion Collider Science", The National Academies Press, Washington, DC, 2018.

- EIC-ORG-PLN-010, Electron-Ion Collider L1 Requirements (Global Requirements Document)

- BNL-221006-2021-FORE, EIC Conceptual Design Report

- arXiv:2103.05419, Science Requirements and Detector Concepts for the Electron-Ion Collider: EIC Yellow Report

- EIC-SEG-PLN-016, Requirements Management Plan

- EIC-SEG-PLN-022, Systems Engineering Management Plan

- EIC-SEG-PLN-020, EIC Interface Management Plan