Electron-Ion Collider, Requirements, Specifications, and Interfaces

Interface Control Document: [###]

Electronic Systems (6.03.01.07) to Ancillary Detectors (6.03.01.10)

Prepared by:

{{Sig_es_:signer1:signature}}	Date: {{Dte_es_:signer1:date}}

Name, Role

Electron-Ion Collider at Institution

Reviewed by:

{{Sig_es_:signer2:signature}}	Date: {{Dte_es_:signer2:date}}

Name, Role

Electron-Ion Collider at Institution

{{Sig_es_:signer3:signature}}	Date: {{Dte_es_:signer3:date}}

Name, Role

Electron-Ion Collider at Institution

{{Sig_es_:signer4:signature}}	Date: {{Dte_es_:signer4:date}}

Name, Role

Electron-Ion Collider at Institution

Approved by:

{{Sig_es_:signer5:signature}}	Date: {{Dte_es_:signer5:date}}

Name, Role

Electron-Ion Collider at Institution

REVISION HISTORY

| Revision # | Effective Date | Additional Reviewers | Summary of Change |
| --- | --- | --- | --- |
| 00 |  |  | Initial release. |

TABLE OF CONTENTS

1.	Purpose and Scope	6

2.	Introduction	6

2.1	Electronic Systems	6

2.2	Overview of Interface Relationships	6

2.3	Safety and Environmental Considerations	7

2.4	Design Considerations	7

2.5	Organization	7

3.	Interface summary	7

4.	Electronic Systems (DET-ELEC) to B-Zero Detectors (DET-ANC-B0)	16

5.	Electronic Systems (DET-ELEC) to Low Q2 Detectors (DET-ANC-LOWQ2)	16

6.	Electronic Systems (DET-ELEC) to Luminosity Detectors (DET-ANC-LUMI)	16

7.	Electronic Systems (DET-ELEC) to Off-Momentum Detectors (DET-ANC-OFFMO)	17

8.	Electronic Systems (DET-ELEC) to Roman Pots (DET-ANC-ROMAN)	17

9.	Electronic Systems (DET-ELEC) to Zero Degree Calorimeter (DET-ANC-ZDC)	17

10.	Electronic Systems (DET-ELEC) to Online DAQ and Computing Systems (DET-COMP-ONLINE)	18

11.	Electronic Systems (DET-ELEC) to Barrel EMCal Systems (DET-ECAL-BAR)	18

12.	Electronic Systems (DET-ELEC) to Backward EMCal Systems (DET-ECAL-BCK)	18

13.	Electronic Systems (DET-ELEC) to Forward EMCal Systems (DET-ECAL-FWD)	19

14.	Electronic Systems (DET-ELEC) to Barrel HCal Systems (DET-HCAL-BAR)	19

15.	Electronic Systems (DET-ELEC) to Backward HCal Systems (DET-HCAL-BCK)	19

16.	Electronic Systems (DET-ELEC) to Forward HCal Systems (DET-HCAL-FWD)	20

17.	Electronic Systems (DET-ELEC) to Detector Infrastructure and Utilities Integration (DET-INF-INT)	20

18.	Electronic Systems (DET-ELEC) to Barrel Particle ID Systems (DET-PID-BAR)	22

19.	Electronic Systems (DET-ELEC) to Backward Particle ID Systems (DET-PID-BCK)	22

20.	Electronic Systems (DET-ELEC) to Forward Particle ID Systems (DET-PID-FWD)	23

21.	Electronic Systems (DET-ELEC) to Electron Polarimetry (DET-POL-EPOL)	23

22.	Electronic Systems (DET-ELEC) to Hadron Polarimetry (DET-POL-HPOL)	24

23.	Electronic Systems (DET-ELEC) to Barrel Tracking Systems (DET-TRAK-BAR)	24

24.	Applicable Documents	25

25.	Detailed change Log	25

26.	References	25

LIST OF ACRONYMS

| B0 | B0 is the name of the forward magnet just past the Hadron Endcap |  |
| --- | --- | --- |
| CDR | Conceptual Design Report |  |
| COTS | Commercial Off the Shelf |  |
| DAQ | Data Acquisition |  |
| DET | EIC Detector |  |
| DIRC | Detection of Internally Reflected Cherenkov Light |  |
| dRICH | Dual radiator ring imaging Cherenkov |  |
| DVCS | Deeply Virtual Compton Scattering |  |
| EIC | Electron-Ion Collider |  |
| EMCAL | Electro-magnetic Calorimeter |  |
| ESR | Electron Storage Ring |  |
| FCC | Federal Communications Commission |  |
| FDR | Final Design Review |  |
| GEM | Gas Electron Multiplier |  |
| HCAL | Hadronic Calorimeter |  |
| HSR | Hadron Storage Ring |  |
| HVAC | Heating/Ventilation/Air Conditioning |  |
| IP | Interaction Point |  |
| IR | Interaction Region |  |
| MPGD | Micro Pattern Gaseous Detectors |  |
| NAS | National Academy of Sciences |  |
| NEC | National Electric Code |  |
| NEMA | National Electrical Manufacturers Association |  |
| NFPA | National Fire Protection Association |  |
| NRTL | Nationally Recognized Testing Laboratory |  |
| PID | Particle Identification |  |
| PRR | Production Readiness Review |  |
| PWO | PbWO4 |  |
| RCS | Rapid Cycling Synchrotron |  |
| TBD | To Be Determined |  |
| UL | Underwriter Laboratories |  |
| WBS | Work Break Down Structure |  |

# Interface Control Document:

Electronic Systems (6.03.01.07) to Ancillary Detectors (6.03.01.10)

# Purpose and Scope

This document records the agreed upon interfaces between the Electronic Systems and other sub-systems within the Ancillary Detectors. These interfaces are based on the system requirements that are necessary to integrate these sub-systems as part of the Electron Ion Collider Project.

# Introduction

These interfaces describe the relationships between the Electronic Systems and the Ancillary Detectors including (examples by inclusion) located in (areas by inclusion).

## Electronic Systems

Provide an introductory description of your sub-system and an overview of the things that it receives, provides, supports, constrains, or otherwise requires as part of its function --- specifically if these things are manifested as interfaces.

## Overview of Interface Relationships

The following are the sub-systems within the Ancillary Detectors that have interfaces connected to the Electronic Systems.

B-Zero Detectors (DET-ANC-B0): Identify the general types of interface relationships between the ICD reference system and this sub-system.

Low Q2 Detectors (DET-ANC-LOWQ2): Identify the general types of interface relationships between the ICD reference system and this sub-system.

Luminosity Detectors (DET-ANC-LUMI): Identify the general types of interface relationships between the ICD reference system and this sub-system.

Off-Momentum Detectors (DET-ANC-OFFMO): Identify the general types of interface relationships between the ICD reference system and this sub-system.

Roman Pots (DET-ANC-ROMAN): Identify the general types of interface relationships between the ICD reference system and this sub-system.

Zero Degree Calorimeter (DET-ANC-ZDC): Identify the general types of interface relationships between the ICD reference system and this sub-system.

Online DAQ and Computing Systems (DET-COMP-ONLINE): Identify the general types of interface relationships between the ICD reference system and this sub-system.

Barrel EMCal Systems (DET-ECAL-BAR): Identify the general types of interface relationships between the ICD reference system and this sub-system.

Backward EMCal Systems (DET-ECAL-BCK): Identify the general types of interface relationships between the ICD reference system and this sub-system.

Forward EMCal Systems (DET-ECAL-FWD): Identify the general types of interface relationships between the ICD reference system and this sub-system.

Barrel HCal Systems (DET-HCAL-BAR): Identify the general types of interface relationships between the ICD reference system and this sub-system.

Backward HCal Systems (DET-HCAL-BCK): Identify the general types of interface relationships between the ICD reference system and this sub-system.

Forward HCal Systems (DET-HCAL-FWD): Identify the general types of interface relationships between the ICD reference system and this sub-system.

Detector Infrastructure and Utilities Integration (DET-INF-INT): Identify the general types of interface relationships between the ICD reference system and this sub-system.

Barrel Particle ID Systems (DET-PID-BAR): Identify the general types of interface relationships between the ICD reference system and this sub-system.

Backward Particle ID Systems (DET-PID-BCK): Identify the general types of interface relationships between the ICD reference system and this sub-system.

Forward Particle ID Systems (DET-PID-FWD): Identify the general types of interface relationships between the ICD reference system and this sub-system.

Electron Polarimetry (DET-POL-EPOL): Identify the general types of interface relationships between the ICD reference system and this sub-system.

Hadron Polarimetry (DET-POL-HPOL): Identify the general types of interface relationships between the ICD reference system and this sub-system.

Barrel Tracking Systems (DET-TRAK-BAR): Identify the general types of interface relationships between the ICD reference system and this sub-system.

## Safety and Environmental Considerations

General safety and environmental considerations are identified here. Any considerations that are specific to the individual sub-system, will be addressed in external safety documents and are referenced here.

### Considerations for all systems

Add general safety and environmental considerations here.

### Special considerations and references

Add references here as necessary.

## Design Considerations

Notes here about general design considerations.

## Organization

The interfaces in this document are grouped by sub-system as identified in the sub-system identifier. All interfaces between the Electronic Systems and a related sub-system will be included in a single section. Entries within that section will be sorted by the interface identifier or WBS number, as appropriate.

# Interface summary

This tables in this section provide a list of all interfaces that are defined within this document. The interfaces are grouped by sub-system/WBS. Each table has the following fields:

Section: the section number in this document where the interface is addressed.

WBS (1): the WBS number within the ICD reference system, unless the reference system is the owner.

WBS (2): the WBS number for the connected system.

ID: the interface identifier as defined in the Interface Requirement Document (IRD).

Name: the name of the interface as defined in the IRD.

Description: the description of the interface as defined in the IRD.

Owner: identifies the WBS number of the entity that is responsible for ensuring the interface is provided and that it meets validation/verification requirements.

Electronic Systems to B-Zero Detectors

| Section | WBS1 | WBS2 | ID | Name | Description | Owner |
| --- | --- | --- | --- | --- | --- | --- |
| 4 |  | 6.03.01.10 |  |  | B-Zero Detectors |  |
| 4.1 | 6.03.01.07 | 6.03.01.10 | I-DET-ELEC.001 | Power | High voltage, low voltage, and bias voltage DC power will be provided from the electronics racks on the south platform | 6.03.01.07 |

Electronic Systems to Low Q2 Detectors

| Section | WBS1 | WBS2 | ID | Name | Description | Owner |
| --- | --- | --- | --- | --- | --- | --- |
| 5 |  | 6.03.01.10 |  |  | Low Q2 Detectors |  |
| 5.1 | 6.03.01.07 | 6.03.01.10 | I-DET-ELEC.002 | Power | High voltage, low voltage, and bias voltage DC power will be provided from the electronics racks on the south platform | 6.03.01.07 |

Electronic Systems to Luminosity Detectors

| Section | WBS1 | WBS2 | ID | Name | Description | Owner |
| --- | --- | --- | --- | --- | --- | --- |
| 6 |  | 6.03.01.10 |  |  | Luminosity Detectors |  |
| 6.1 | 6.03.01.07 | 6.03.01.10 | I-DET-ELEC.003 | Power | High voltage, low voltage, and bias voltage DC power will be provided from the electronics racks on the south platform | 6.03.01.07 |

Electronic Systems to Off-Momentum Detectors

| Section | WBS1 | WBS2 | ID | Name | Description | Owner |
| --- | --- | --- | --- | --- | --- | --- |
| 7 |  | 6.03.01.10 |  |  | Off-Momentum Detectors |  |
| 7.1 | 6.03.01.07 | 6.03.01.10 | I-DET-ELEC.004 | Power | High voltage, low voltage, and bias voltage DC power will be provided from the electronics racks on the south platform | 6.03.01.07 |

Electronic Systems to Roman Pots

| Section | WBS1 | WBS2 | ID | Name | Description | Owner |
| --- | --- | --- | --- | --- | --- | --- |
| 8 |  | 6.03.01.10 |  |  | Roman Pots |  |
| 8.1 | 6.03.01.07 | 6.03.01.10 | I-DET-ELEC.005 | Power | High voltage, low voltage, and bias voltage DC power will be provided from the electronics racks on the south platform | 6.03.01.07 |

Electronic Systems to Zero Degree Calorimeter

| Section | WBS1 | WBS2 | ID | Name | Description | Owner |
| --- | --- | --- | --- | --- | --- | --- |
| 9 |  | 6.03.01.10 |  |  | Zero Degree Calorimeter |  |
| 9.1 | 6.03.01.07 | 6.03.01.10 | I-DET-ELEC.006 | Power | High voltage, low voltage, and bias voltage DC power will be provided from the electronics racks on the south platform | 6.03.01.07 |

Electronic Systems to Online DAQ and Computing Systems

| Section | WBS1 | WBS2 | ID | Name | Description | Owner |
| --- | --- | --- | --- | --- | --- | --- |
| 10 |  | 6.03.01.08.02 |  |  | Online DAQ and Computing Systems |  |
| 10.1 | 6.03.01.07 | 6.03.01.08.02 | I-DET-COMP-ONLINE.020 | Monitoring Systems | Monitoring system cables will run from the south platform to the DAQ control room to allow monitoring of the power supplies and the electronics. | 6.03.01.08.02 |
| 10.2 | 6.03.01.07 | 6.03.01.08.02 | I-DET-COMP-ONLINE.021 | Slow Controls | Controls cables allowing fast shutdown and machine interlocks will run from the south platform to the DAQ control room. | 6.03.01.08.02 |

Electronic Systems to Barrel EMCal Systems

| Section | WBS1 | WBS2 | ID | Name | Description | Owner |
| --- | --- | --- | --- | --- | --- | --- |
| 11 |  | 6.03.01.04 |  |  | Barrel EMCal Systems |  |
| 11.1 | 6.03.01.07 | 6.03.01.04 | I-DET-ELEC.007 | Power | High voltage, low voltage, and bias voltage DC power will be provided from the electronics racks on the south platform | 6.03.01.07 |

Electronic Systems to Backward EMCal Systems

| Section | WBS1 | WBS2 | ID | Name | Description | Owner |
| --- | --- | --- | --- | --- | --- | --- |
| 12 |  | 6.03.01.04 |  |  | Backward EMCal Systems |  |
| 12.1 | 6.03.01.07 | 6.03.01.04 | I-DET-ELEC.008 | Power | High voltage, low voltage, and bias voltage DC power will be provided from the electronics racks on the south platform | 6.03.01.07 |

Electronic Systems to Forward EMCal Systems

| Section | WBS1 | WBS2 | ID | Name | Description | Owner |
| --- | --- | --- | --- | --- | --- | --- |
| 13 |  | 6.03.01.04 |  |  | Forward EMCal Systems |  |
| 13.1 | 6.03.01.07 | 6.03.01.04 | I-DET-ELEC.009 | Power | High voltage, low voltage, and bias voltage DC power will be provided from the electronics racks on the south platform | 6.03.01.07 |

Electronic Systems to Barrel HCal Systems

| Section | WBS1 | WBS2 | ID | Name | Description | Owner |
| --- | --- | --- | --- | --- | --- | --- |
| 14 |  | 6.03.01.05 |  |  | Barrel HCal Systems |  |
| 14.1 | 6.03.01.07 | 6.03.01.05 | I-DET-ELEC.010 | Power | High voltage, low voltage, and bias voltage DC power will be provided from the electronics racks on the south platform | 6.03.01.07 |

Electronic Systems to Backward HCal Systems

| Section | WBS1 | WBS2 | ID | Name | Description | Owner |
| --- | --- | --- | --- | --- | --- | --- |
| 15 |  | 6.03.01.05 |  |  | Backward HCal Systems |  |
| 15.1 | 6.03.01.07 | 6.03.01.05 | I-DET-ELEC.011 | Power | High voltage, low voltage, and bias voltage DC power will be provided from the electronics racks on the south platform | 6.03.01.07 |

Electronic Systems to Forward HCal Systems

| Section | WBS1 | WBS2 | ID | Name | Description | Owner |
| --- | --- | --- | --- | --- | --- | --- |
| 16 |  | 6.03.01.05 |  |  | Forward HCal Systems |  |
| 16.1 | 6.03.01.07 | 6.03.01.05 | I-DET-ELEC.012 | Power | High voltage, low voltage, and bias voltage DC power will be provided from the electronics racks on the south platform | 6.03.01.07 |

Electronic Systems to Detector Infrastructure and Utilities Integration

| Section | WBS1 | WBS2 | ID | Name | Description | Owner |
| --- | --- | --- | --- | --- | --- | --- |
| 17 |  | 6.03.01.09.04 |  |  | Detector Infrastructure and Utilities Integration |  |
| 17.1 | 6.03.01.07 | 6.03.01.09.04 | I-DET-INF-INT.067 | Clean A/C Power | Electronics components on the carriage will require clean or transformer isolated power. | 6.03.01.09.04 |
| 17.2 | 6.03.01.07 | 6.03.01.09.04 | I-DET-INF-INT.068 | UPS Power | Critical systems, such as controls, will require sufficient UPS power to allow them to be shutdown graceful, or to tolerate transients. | 6.03.01.09.04 |
| 17.3 | 6.03.01.07 | 6.03.01.09.04 | I-DET-INF-INT.069 | Computing Environment | The electronics and computing systems will require a stable environment in terms of temperature and humidity. | 6.03.01.09.04 |
| 17.4 | 6.03.01.07 | 6.03.01.09.04 | I-DET-INF-INT.070 | EMI Shielding | The electronics and computing systems on the carriage may require some EMI shielding from detectors and other components. | 6.03.01.09.04 |
| 17.5 | 6.03.01.07 | 6.03.01.09.04 | I-DET-INF-INT.071 | Cabling Pathways | Adequate space for cables and cable trays will be required to deliver low voltage and high voltage, fiber and signal cables from the carriage/platform to the sub-detectors. | 6.03.01.09.04 |
| 17.6 | 6.03.01.07 | 6.03.01.09.04 | I-DET-INF-INT.072 | Platform Rack Space | Adequate space must be provided for computing enclosures (racks) on the south platform to support the electronics systems. | 6.03.01.09.04 |

Electronic Systems to Barrel Particle ID Systems

| Section | WBS1 | WBS2 | ID | Name | Description | Owner |
| --- | --- | --- | --- | --- | --- | --- |
| 18 |  | 6.03.01.03 |  |  | Barrel Particle ID Systems |  |
| 18.1 | 6.03.01.07 | 6.03.01.03 | I-DET-ELEC.013 | Power | High voltage, low voltage, and bias voltage DC power will be provided from the electronics racks on the south platform | 6.03.01.07 |
| 18.2 | 6.03.01.07 | 6.03.01.03 | I-DET-ELEC.014 | Power | High voltage and low voltage DC power will be provided from the electronics racks on the south platform. | 6.03.01.07 |
| 18.3 | 6.03.01.07 | 6.03.01.03 | I-DET-ELEC.015 | Power | High voltage and low voltage DC power will be provided from the electronics racks on the south platform. | 6.03.01.07 |

Electronic Systems to Backward Particle ID Systems

| Section | WBS1 | WBS2 | ID | Name | Description | Owner |
| --- | --- | --- | --- | --- | --- | --- |
| 19 |  | 6.03.01.03 |  |  | Backward Particle ID Systems |  |
| 19.1 | 6.03.01.07 | 6.03.01.03 | I-DET-ELEC.016 | Power | High voltage and low voltage DC power will be provided from the electronics racks on the south platform. | 6.03.01.07 |

Electronic Systems to Forward Particle ID Systems

| Section | WBS1 | WBS2 | ID | Name | Description | Owner |
| --- | --- | --- | --- | --- | --- | --- |
| 20 |  | 6.03.01.03 |  |  | Forward Particle ID Systems |  |
| 20.1 | 6.03.01.07 | 6.03.01.03 | I-DET-ELEC.017 | Power | Low voltage and bias voltage DC power will be provided from the electronics racks on the south platform. | 6.03.01.07 |

Electronic Systems to Electron Polarimetry

| Section | WBS1 | WBS2 | ID | Name | Description | Owner |
| --- | --- | --- | --- | --- | --- | --- |
| 21 |  | 6.03.01.11.01 |  |  | Electron Polarimetry |  |
| 21.1 | 6.03.01.07 | 6.03.01.11.01 | I-DET-ELEC.018 | Power | High voltage, low voltage, and bias voltage DC power will be provided from the electronics racks on the south platform | 6.03.01.07 |
| 21.2 | 6.03.01.07 | 6.03.01.11.01 | I-DET-ELEC.019 | Power | High voltage, low voltage, and bias voltage DC power will be provided from the electronics racks on the south platform | 6.03.01.07 |

Electronic Systems to Hadron Polarimetry

| Section | WBS1 | WBS2 | ID | Name | Description | Owner |
| --- | --- | --- | --- | --- | --- | --- |
| 22 |  | 6.03.01.11.02 |  |  | Hadron Polarimetry |  |
| 22.1 | 6.03.01.07 | 6.03.01.11.02 | I-DET-ELEC.020 | Power | High voltage, low voltage, and bias voltage DC power will be provided from the electronics racks on the south platform | 6.03.01.07 |
| 22.2 | 6.03.01.07 | 6.03.01.11.02 | I-DET-ELEC.021 | Power | High voltage, low voltage, and bias voltage DC power will be provided from the electronics racks on the south platform | 6.03.01.07 |

Electronic Systems to Barrel Tracking Systems

| Section | WBS1 | WBS2 | ID | Name | Description | Owner |
| --- | --- | --- | --- | --- | --- | --- |
| 23 |  | 6.03.01.02 |  |  | Barrel Tracking Systems |  |
| 23.1 | 6.03.01.07 | 6.03.01.02 | I-DET-ELEC.022 | Power | High voltage, low voltage, and bias voltage DC power will be provided from the electronics racks on the south platform | 6.03.01.07 |

# Electronic Systems (DET-ELEC) to B-Zero Detectors (DET-ANC-B0)

I-DET-ELEC.001	Power

System 1:	Electronic Systems (6.03.01.07)

System 2:	B-Zero Detectors (6.03.01.10)

Type:	ELEC

Description:	High voltage, low voltage, and bias voltage DC power will be provided from the electronics racks on the south platform

Provider:	Electronic Systems (6.03.01.07)

Details:

Requirements:	P-DET-ANC.4

References:

# Electronic Systems (DET-ELEC) to Low Q2 Detectors (DET-ANC-LOWQ2)

I-DET-ELEC.002	Power

System 1:	Electronic Systems (6.03.01.07)

System 2:	Low Q2 Detectors (6.03.01.10)

Type:	ELEC

Description:	High voltage, low voltage, and bias voltage DC power will be provided from the electronics racks on the south platform

Provider:	Electronic Systems (6.03.01.07)

Details:

Requirements:	P-DET-ANC.4

References:

# Electronic Systems (DET-ELEC) to Luminosity Detectors (DET-ANC-LUMI)

I-DET-ELEC.003	Power

System 1:	Electronic Systems (6.03.01.07)

System 2:	Luminosity Detectors (6.03.01.10)

Type:	ELEC

Description:	High voltage, low voltage, and bias voltage DC power will be provided from the electronics racks on the south platform

Provider:	Electronic Systems (6.03.01.07)

Details:

Requirements:	P-DET-ANC.4

References:

# Electronic Systems (DET-ELEC) to Off-Momentum Detectors (DET-ANC-OFFMO)

I-DET-ELEC.004	Power

System 1:	Electronic Systems (6.03.01.07)

System 2:	Off-Momentum Detectors (6.03.01.10)

Type:	ELEC

Description:	High voltage, low voltage, and bias voltage DC power will be provided from the electronics racks on the south platform

Provider:	Electronic Systems (6.03.01.07)

Details:

Requirements:	P-DET-ANC.4

References:

# Electronic Systems (DET-ELEC) to Roman Pots (DET-ANC-ROMAN)

I-DET-ELEC.005	Power

System 1:	Electronic Systems (6.03.01.07)

System 2:	Roman Pots (6.03.01.10)

Type:	ELEC

Description:	High voltage, low voltage, and bias voltage DC power will be provided from the electronics racks on the south platform

Provider:	Electronic Systems (6.03.01.07)

Details:

Requirements:	P-DET-ANC.4

References:

# Electronic Systems (DET-ELEC) to Zero Degree Calorimeter (DET-ANC-ZDC)

I-DET-ELEC.006	Power

System 1:	Electronic Systems (6.03.01.07)

System 2:	Zero Degree Calorimeter (6.03.01.10)

Type:	ELEC

Description:	High voltage, low voltage, and bias voltage DC power will be provided from the electronics racks on the south platform

Provider:	Electronic Systems (6.03.01.07)

Details:

Requirements:	P-DET-ANC.4

References:

# Electronic Systems (DET-ELEC) to Online DAQ and Computing Systems (DET-COMP-ONLINE)

I-DET-COMP-ONLINE.020	Monitoring Systems

System 1:	Online DAQ and Computing Systems (6.03.01.08.02)

System 2:	Electronic Systems (6.03.01.07)

Type:	CONTROL

Description:	Monitoring system cables will run from the south platform to the DAQ control room to allow monitoring of the power supplies and the electronics.

Provider:	Online DAQ and Computing Systems (6.03.01.08.02)

Details:	See details in the Online DAQ and Computing Systems Interface Control Document.

Requirements:	F-DET-COMP.6

References:

I-DET-COMP-ONLINE.021	Slow Controls

System 1:	Online DAQ and Computing Systems (6.03.01.08.02)

System 2:	Electronic Systems (6.03.01.07)

Type:	CONTROL

Description:	Controls cables allowing fast shutdown and machine interlocks will run from the south platform to the DAQ control room.

Provider:	Online DAQ and Computing Systems (6.03.01.08.02)

Details:	See details in the Online DAQ and Computing Systems Interface Control Document.

Requirements:	F-DET-COMP-SC.1

References:

# Electronic Systems (DET-ELEC) to Barrel EMCal Systems (DET-ECAL-BAR)

I-DET-ELEC.007	Power

System 1:	Electronic Systems (6.03.01.07)

System 2:	Barrel EMCal Systems (6.03.01.04)

Type:	ELEC

Description:	High voltage, low voltage, and bias voltage DC power will be provided from the electronics racks on the south platform

Provider:	Electronic Systems (6.03.01.07)

Details:

Requirements:	P-DET-ECAL.4

References:

# Electronic Systems (DET-ELEC) to Backward EMCal Systems (DET-ECAL-BCK)

I-DET-ELEC.008	Power

System 1:	Electronic Systems (6.03.01.07)

System 2:	Backward EMCal Systems (6.03.01.04)

Type:	ELEC

Description:	High voltage, low voltage, and bias voltage DC power will be provided from the electronics racks on the south platform

Provider:	Electronic Systems (6.03.01.07)

Details:

Requirements:	P-DET-ECAL.4

References:

# Electronic Systems (DET-ELEC) to Forward EMCal Systems (DET-ECAL-FWD)

I-DET-ELEC.009	Power

System 1:	Electronic Systems (6.03.01.07)

System 2:	Forward EMCal Systems (6.03.01.04)

Type:	ELEC

Description:	High voltage, low voltage, and bias voltage DC power will be provided from the electronics racks on the south platform

Provider:	Electronic Systems (6.03.01.07)

Details:

Requirements:	P-DET-ECAL.4

References:

# Electronic Systems (DET-ELEC) to Barrel HCal Systems (DET-HCAL-BAR)

I-DET-ELEC.010	Power

System 1:	Electronic Systems (6.03.01.07)

System 2:	Barrel HCal Systems (6.03.01.05)

Type:	ELEC

Description:	High voltage, low voltage, and bias voltage DC power will be provided from the electronics racks on the south platform

Provider:	Electronic Systems (6.03.01.07)

Details:

Requirements:	P-DET-HCAL.4

References:

# Electronic Systems (DET-ELEC) to Backward HCal Systems (DET-HCAL-BCK)

I-DET-ELEC.011	Power

System 1:	Electronic Systems (6.03.01.07)

System 2:	Backward HCal Systems (6.03.01.05)

Type:	ELEC

Description:	High voltage, low voltage, and bias voltage DC power will be provided from the electronics racks on the south platform

Provider:	Electronic Systems (6.03.01.07)

Details:

Requirements:	P-DET-HCAL.4

References:

# Electronic Systems (DET-ELEC) to Forward HCal Systems (DET-HCAL-FWD)

I-DET-ELEC.012	Power

System 1:	Electronic Systems (6.03.01.07)

System 2:	Forward HCal Systems (6.03.01.05)

Type:	ELEC

Description:	High voltage, low voltage, and bias voltage DC power will be provided from the electronics racks on the south platform

Provider:	Electronic Systems (6.03.01.07)

Details:

Requirements:	P-DET-HCAL.4

References:

# Electronic Systems (DET-ELEC) to Detector Infrastructure and Utilities Integration (DET-INF-INT)

I-DET-INF-INT.067	Clean A/C Power

System 1:	Detector Infrastructure and Utilities Integration (6.03.01.09.04)

System 2:	Electronic Systems (6.03.01.07)

Type:	ELEC

Description:	Electronics components on the carriage will require clean or transformer isolated power.

Provider:	Detector Infrastructure and Utilities Integration (6.03.01.09.04)

Details:	See details in the Detector Infrastructure and Utilities Integration Interface Control Document.

Requirements:	P-DET-ELEC.4

References:

I-DET-INF-INT.068	UPS Power

System 1:	Detector Infrastructure and Utilities Integration (6.03.01.09.04)

System 2:	Electronic Systems (6.03.01.07)

Type:	ELEC

Description:	Critical systems, such as controls, will require sufficient UPS power to allow them to be shutdown graceful, or to tolerate transients.

Provider:	Detector Infrastructure and Utilities Integration (6.03.01.09.04)

Details:	See details in the Detector Infrastructure and Utilities Integration Interface Control Document.

Requirements:	P-DET-ELEC.4

References:

I-DET-INF-INT.069	Computing Environment

System 1:	Detector Infrastructure and Utilities Integration (6.03.01.09.04)

System 2:	Electronic Systems (6.03.01.07)

Type:	ENV

Description:	The electronics and computing systems will require a stable environment in terms of temperature and humidity.

Provider:	Detector Infrastructure and Utilities Integration (6.03.01.09.04)

Details:	See details in the Detector Infrastructure and Utilities Integration Interface Control Document.

Requirements:	P-DET-ELEC.3

References:

I-DET-INF-INT.070	EMI Shielding

System 1:	Detector Infrastructure and Utilities Integration (6.03.01.09.04)

System 2:	Electronic Systems (6.03.01.07)

Type:	ENV

Description:	The electronics and computing systems on the carriage may require some EMI shielding from detectors and other components.

Provider:	Detector Infrastructure and Utilities Integration (6.03.01.09.04)

Details:	See details in the Detector Infrastructure and Utilities Integration Interface Control Document.

Requirements:	P-DET-ELEC.6

References:

I-DET-INF-INT.071	Cabling Pathways

System 1:	Detector Infrastructure and Utilities Integration (6.03.01.09.04)

System 2:	Electronic Systems (6.03.01.07)

Type:	SPACE

Description:	Adequate space for cables and cable trays will be required to deliver low voltage and high voltage, fiber and signal cables from the carriage/platform to the sub-detectors.

Provider:	Detector Infrastructure and Utilities Integration (6.03.01.09.04)

Details:	See details in the Detector Infrastructure and Utilities Integration Interface Control Document.

Requirements:	P-DET-ELEC.1

References:

I-DET-INF-INT.072	Platform Rack Space

System 1:	Detector Infrastructure and Utilities Integration (6.03.01.09.04)

System 2:	Electronic Systems (6.03.01.07)

Type:	SPACE

Description:	Adequate space must be provided for computing enclosures (racks) on the south platform to support the electronics systems.

Provider:	Detector Infrastructure and Utilities Integration (6.03.01.09.04)

Details:	See details in the Detector Infrastructure and Utilities Integration Interface Control Document.

Requirements:	P-DET-ELEC.1

References:

# Electronic Systems (DET-ELEC) to Barrel Particle ID Systems (DET-PID-BAR)

I-DET-ELEC.013	Power

System 1:	Electronic Systems (6.03.01.07)

System 2:	Barrel Particle ID Systems (6.03.01.03)

Type:	ELEC

Description:	High voltage, low voltage, and bias voltage DC power will be provided from the electronics racks on the south platform

Provider:	Electronic Systems (6.03.01.07)

Details:

Requirements:	P-DET-PID.2

References:

I-DET-ELEC.014	Power

System 1:	Electronic Systems (6.03.01.07)

System 2:	Barrel DIRC Particle ID Systems (6.03.01.03)

Type:	ELEC

Description:	High voltage and low voltage DC power will be provided from the electronics racks on the south platform.

Provider:	Electronic Systems (6.03.01.07)

Details:

Requirements:	P-DET-PID.2

References:

I-DET-ELEC.015	Power

System 1:	Electronic Systems (6.03.01.07)

System 2:	Barrel TOF Particle ID Systems (6.03.01.03)

Type:	ELEC

Description:	High voltage and low voltage DC power will be provided from the electronics racks on the south platform.

Provider:	Electronic Systems (6.03.01.07)

Details:

Requirements:	P-DET-PID.2

References:

# Electronic Systems (DET-ELEC) to Backward Particle ID Systems (DET-PID-BCK)

I-DET-ELEC.016	Power

System 1:	Electronic Systems (6.03.01.07)

System 2:	Backward Particle ID Systems (6.03.01.03)

Type:	ELEC

Description:	High voltage and low voltage DC power will be provided from the electronics racks on the south platform.

Provider:	Electronic Systems (6.03.01.07)

Details:

Requirements:	P-DET-PID.2

References:

# Electronic Systems (DET-ELEC) to Forward Particle ID Systems (DET-PID-FWD)

I-DET-ELEC.017	Power

System 1:	Electronic Systems (6.03.01.07)

System 2:	Forward Particle ID Systems (6.03.01.03)

Type:	ELEC

Description:	Low voltage and bias voltage DC power will be provided from the electronics racks on the south platform.

Provider:	Electronic Systems (6.03.01.07)

Details:

Requirements:	P-DET-PID.2

References:

# Electronic Systems (DET-ELEC) to Electron Polarimetry (DET-POL-EPOL)

I-DET-ELEC.018	Power

System 1:	Electronic Systems (6.03.01.07)

System 2:	Electron Storage Ring Polarimetry (6.03.01.11.01.01)

Type:	ELEC

Description:	High voltage, low voltage, and bias voltage DC power will be provided from the electronics racks on the south platform

Provider:	Electronic Systems (6.03.01.07)

Details:

Requirements:	P-DET-POL.4

References:

I-DET-ELEC.019	Power

System 1:	Electronic Systems (6.03.01.07)

System 2:	Rapid Cycling Synchrotron Polarimetry (6.03.01.11.01.02)

Type:	ELEC

Description:	High voltage, low voltage, and bias voltage DC power will be provided from the electronics racks on the south platform

Provider:	Electronic Systems (6.03.01.07)

Details:

Requirements:	P-DET-POL.4

References:

# Electronic Systems (DET-ELEC) to Hadron Polarimetry (DET-POL-HPOL)

I-DET-ELEC.020	Power

System 1:	Electronic Systems (6.03.01.07)

System 2:	HJET Polarimetery (6.03.01.11.02.01)

Type:	ELEC

Description:	High voltage, low voltage, and bias voltage DC power will be provided from the electronics racks on the south platform

Provider:	Electronic Systems (6.03.01.07)

Details:

Requirements:	P-DET-POL.4

References:

I-DET-ELEC.021	Power

System 1:	Electronic Systems (6.03.01.07)

System 2:	Proton-Carbon Polarimeter (6.03.01.11.02.02)

Type:	ELEC

Description:	High voltage, low voltage, and bias voltage DC power will be provided from the electronics racks on the south platform

Provider:	Electronic Systems (6.03.01.07)

Details:

Requirements:	P-DET-POL.4

References:

# Electronic Systems (DET-ELEC) to Barrel Tracking Systems (DET-TRAK-BAR)

I-DET-ELEC.022	Power

System 1:	Electronic Systems (6.03.01.07)

System 2:	Barrel Tracking Systems (6.03.01.02)

Type:	ELEC

Description:	High voltage, low voltage, and bias voltage DC power will be provided from the electronics racks on the south platform

Provider:	Electronic Systems (6.03.01.07)

Details:

Requirements:	P-DET-TRAK.6

References:

# Applicable Documents

List applicable documents considered to be part of this document. Applicable documents shall be obtained from the EIC SharePoint Document Center.

# Detailed change Log

| Revision | Date | Change |
| --- | --- | --- |

# References

- doi:10.17226/25171., National Academies of Sciences, Engineering, and Medicine, "An Assessment of U.S.-Based Electron-Ion Collider Science", The National Academies Press, Washington, DC, 2018.

- EIC-ORG-PLN-010, Electron-Ion Collider L1 Requirements (Global Requirements Document)

- BNL-221006-2021-FORE, EIC Conceptual Design Report

- arXiv:2103.05419, Science Requirements and Detector Concepts for the Electron-Ion Collider: EIC Yellow Report

- EIC-SEG-PLN-016, Requirements Management Plan

- EIC-SEG-PLN-022, Systems Engineering Management Plan

- EIC-SEG-PLN-020, EIC Interface Management Plan