Electron-Ion Collider, Requirements, Specifications, and Interfaces

Interface Control Document: [###]

Hadronic Calorimetry Systems (6.03.01.05) to Ancillary Detectors (6.03.01.10)

Prepared by:

{{Sig_es_:signer1:signature}}	Date: {{Dte_es_:signer1:date}}

Name, Role

Electron-Ion Collider at Institution

Reviewed by:

{{Sig_es_:signer2:signature}}	Date: {{Dte_es_:signer2:date}}

Name, Role

Electron-Ion Collider at Institution

{{Sig_es_:signer3:signature}}	Date: {{Dte_es_:signer3:date}}

Name, Role

Electron-Ion Collider at Institution

{{Sig_es_:signer4:signature}}	Date: {{Dte_es_:signer4:date}}

Name, Role

Electron-Ion Collider at Institution

Approved by:

{{Sig_es_:signer5:signature}}	Date: {{Dte_es_:signer5:date}}

Name, Role

Electron-Ion Collider at Institution

REVISION HISTORY

| Revision # | Effective Date | Additional Reviewers | Summary of Change |
| --- | --- | --- | --- |
| 00 |  |  | Initial release. |

TABLE OF CONTENTS

1.	Purpose and Scope	5

2.	Introduction	5

2.1	Hadronic Calorimetry Systems	5

2.2	Overview of Interface Relationships	5

2.3	Safety and Environmental Considerations	6

2.4	Design Considerations	6

2.5	Organization	6

3.	Interface summary	6

4.	Hadronic Calorimetry Systems (DET-HCAL) to B-Zero Detectors (DET-ANC-B0)	23

5.	Hadronic Calorimetry Systems (DET-HCAL) to Online DAQ and Computing Systems (DET-COMP-ONLINE)	23

6.	Hadronic Calorimetry Systems (DET-HCAL) to Barrel EMCal Systems (DET-ECAL-BAR)	25

7.	Hadronic Calorimetry Systems (DET-HCAL) to Backward EMCal Systems (DET-ECAL-BCK)	25

8.	Hadronic Calorimetry Systems (DET-HCAL) to Forward EMCal Systems (DET-ECAL-FWD)	26

9.	Hadronic Calorimetry Systems (DET-HCAL) to Electronic Systems (DET-ELEC)	28

10.	Hadronic Calorimetry Systems (DET-HCAL) to Barrel Detector Structures (DET-INF-BAR)	29

11.	Hadronic Calorimetry Systems (DET-HCAL) to Electron Endcap Structures (DET-INF-BCK)	31

12.	Hadronic Calorimetry Systems (DET-HCAL) to Hadron Endcap Structures (DET-INF-FWD)	31

13.	Hadronic Calorimetry Systems (DET-HCAL) to Detector Infrastructure and Utilities Integration (DET-INF-INT)	33

14.	Hadronic Calorimetry Systems (DET-HCAL) to Solenoid Magnet (DET-MAG)	39

15.	Hadronic Calorimetry Systems (DET-HCAL) to Barrel Particle ID Systems (DET-PID-BAR)	39

16.	Hadronic Calorimetry Systems (DET-HCAL) to Forward Particle ID Systems (DET-PID-FWD)	40

17.	Applicable Documents	40

18.	Detailed change Log	40

19.	References	41

LIST OF ACRONYMS

| B0 | B0 is the name of the forward magnet just past the Hadron Endcap |  |
| --- | --- | --- |
| CDR | Conceptual Design Report |  |
| COTS | Commercial Off the Shelf |  |
| DAQ | Data Acquisition |  |
| DET | EIC Detector |  |
| DIRC | Detection of Internally Reflected Cherenkov Light |  |
| dRICH | Dual radiator ring imaging Cherenkov |  |
| DVCS | Deeply Virtual Compton Scattering |  |
| EIC | Electron-Ion Collider |  |
| EMCAL | Electro-magnetic Calorimeter |  |
| ESR | Electron Storage Ring |  |
| FCC | Federal Communications Commission |  |
| FDR | Final Design Review |  |
| GEM | Gas Electron Multiplier |  |
| HCAL | Hadronic Calorimeter |  |
| HSR | Hadron Storage Ring |  |
| HVAC | Heating/Ventilation/Air Conditioning |  |
| IP | Interaction Point |  |
| IR | Interaction Region |  |
| MPGD | Micro Pattern Gaseous Detectors |  |
| NAS | National Academy of Sciences |  |
| NEC | National Electric Code |  |
| NEMA | National Electrical Manufacturers Association |  |
| NFPA | National Fire Protection Association |  |
| NRTL | Nationally Recognized Testing Laboratory |  |
| PID | Particle Identification |  |
| PRR | Production Readiness Review |  |
| PWO | PbWO4 |  |
| RCS | Rapid Cycling Synchrotron |  |
| TBD | To Be Determined |  |
| UL | Underwriter Laboratories |  |
| WBS | Work Break Down Structure |  |

# Interface Control Document:

Hadronic Calorimetry Systems (6.03.01.05) to Ancillary Detectors (6.03.01.10)

# Purpose and Scope

This document records the agreed upon interfaces between the Hadronic Calorimetry Systems and other sub-systems within the Ancillary Detectors. These interfaces are based on the system requirements that are necessary to integrate these sub-systems as part of the Electron Ion Collider Project.

# Introduction

These interfaces describe the relationships between the Hadronic Calorimetry Systems and the Ancillary Detectors including (examples by inclusion) located in (areas by inclusion).

## Hadronic Calorimetry Systems

Provide an introductory description of your sub-system and an overview of the things that it receives, provides, supports, constrains, or otherwise requires as part of its function --- specifically if these things are manifested as interfaces.

## Overview of Interface Relationships

The following are the sub-systems within the Ancillary Detectors that have interfaces connected to the Hadronic Calorimetry Systems.

B-Zero Detectors (DET-ANC-B0): Identify the general types of interface relationships between the ICD reference system and this sub-system.

Online DAQ and Computing Systems (DET-COMP-ONLINE): Identify the general types of interface relationships between the ICD reference system and this sub-system.

Barrel EMCal Systems (DET-ECAL-BAR): Identify the general types of interface relationships between the ICD reference system and this sub-system.

Backward EMCal Systems (DET-ECAL-BCK): Identify the general types of interface relationships between the ICD reference system and this sub-system.

Forward EMCal Systems (DET-ECAL-FWD): Identify the general types of interface relationships between the ICD reference system and this sub-system.

Electronic Systems (DET-ELEC): Identify the general types of interface relationships between the ICD reference system and this sub-system.

Barrel Detector Structures (DET-INF-BAR): Identify the general types of interface relationships between the ICD reference system and this sub-system.

Electron Endcap Structures (DET-INF-BCK): Identify the general types of interface relationships between the ICD reference system and this sub-system.

Hadron Endcap Structures (DET-INF-FWD): Identify the general types of interface relationships between the ICD reference system and this sub-system.

Detector Infrastructure and Utilities Integration (DET-INF-INT): Identify the general types of interface relationships between the ICD reference system and this sub-system.

Solenoid Magnet (DET-MAG): The size and position of the solenoid magnet is constrained by the barrel and backward hadron calorimeter systems.

Barrel Particle ID Systems (DET-PID-BAR): Identify the general types of interface relationships between the ICD reference system and this sub-system.

Forward Particle ID Systems (DET-PID-FWD): Identify the general types of interface relationships between the ICD reference system and this sub-system.

## Safety and Environmental Considerations

General safety and environmental considerations are identified here. Any considerations that are specific to the individual sub-system, will be addressed in external safety documents and are referenced here.

### Considerations for all systems

Add general safety and environmental considerations here.

### Special considerations and references

Add references here as necessary.

## Design Considerations

Notes here about general design considerations.

## Organization

The interfaces in this document are grouped by sub-system as identified in the sub-system identifier. All interfaces between the Hadronic Calorimetry Systems and a related sub-system will be included in a single section. Entries within that section will be sorted by the interface identifier or WBS number, as appropriate.

# Interface summary

This tables in this section provide a list of all interfaces that are defined within this document. The interfaces are grouped by sub-system/WBS. Each table has the following fields:

Section: the section number in this document where the interface is addressed.

WBS (1): the WBS number within the ICD reference system, unless the reference system is the owner.

WBS (2): the WBS number for the connected system.

ID: the interface identifier as defined in the Interface Requirement Document (IRD).

Name: the name of the interface as defined in the IRD.

Description: the description of the interface as defined in the IRD.

Owner: identifies the WBS number of the entity that is responsible for ensuring the interface is provided and that it meets validation/verification requirements.

Hadronic Calorimetry Systems to B-Zero Detectors

| Section | WBS1 | WBS2 | ID | Name | Description | Owner |
| --- | --- | --- | --- | --- | --- | --- |
| 4 |  | 6.03.01.10 |  |  | B-Zero Detectors |  |
| 4.1 | 6.03.01.05 | 6.03.01.10 | I-DET-INF-INT.005 | Space Constraint | The position of the forward HCAL is bounded by the required 50cm clearance between the backward HCAL surface and the B0 magnet in the forward direction. Changes to the size or position of either must be coordinated with the other. | 6.03.01.09.04 |

Hadronic Calorimetry Systems to Online DAQ and Computing Systems

| Section | WBS1 | WBS2 | ID | Name | Description | Owner |
| --- | --- | --- | --- | --- | --- | --- |
| 5 |  | 6.03.01.08.02 |  |  | Online DAQ and Computing Systems |  |
| 5.1 | 6.03.01.05 | 6.03.01.08.02 | I-DET-COMP-ONLINE.022 | Data Transfer and Control Interface | A fiber connection will be provided from the DAQ system to the barrel HCAL's readout board to perform configuration, control, data acquisition, and timing. | 6.03.01.08.02 |
| 5.2 | 6.03.01.05 | 6.03.01.08.02 | I-DET-COMP-ONLINE.023 | Slow Controls | A connection from the DAQ system to the barrel HCAL's slow controls interface. | 6.03.01.08.02 |
| 5.3 | 6.03.01.05 | 6.03.01.08.02 | I-DET-COMP-ONLINE.024 | Data Transfer and Control Interface | A fiber connection will be provided from the DAQ system to the backward HCAL's readout board to perform configuration, control, data acquisition, and timing. | 6.03.01.08.02 |
| 5.4 | 6.03.01.05 | 6.03.01.08.02 | I-DET-COMP-ONLINE.025 | Slow Controls | A connection from the DAQ system to the barrel HCAL's slow controls interface. CLARIFY with Data acquisition whether a slow control interface will be independent of the regular control interface. | 6.03.01.08.02 |
| 5.5 | 6.03.01.05 | 6.03.01.08.02 | I-DET-COMP-ONLINE.026 | Data Transfer and Control Interface | A fiber connection will be provided from the DAQ system to the forward HCAL's readout board to perform configuration, control, data acquisition, and timing. | 6.03.01.08.02 |
| 5.6 | 6.03.01.05 | 6.03.01.08.02 | I-DET-COMP-ONLINE.027 | Slow Controls | A connection from the DAQ system to the forward HCAL's slow controls interface. CLARIFY with Data acquisition whether a slow control interface will be independent of the regular control interface. | 6.03.01.08.02 |

Hadronic Calorimetry Systems to Barrel EMCal Systems

| Section | WBS1 | WBS2 | ID | Name | Description | Owner |
| --- | --- | --- | --- | --- | --- | --- |
| 6 |  | 6.03.01.04 |  |  | Barrel EMCal Systems |  |
| 6.1 | 6.03.01.05 | 6.03.01.04 | I-DET-INF-BAR.001 | Barrel ECAL Support | The barrel ECAL will be supported by a structural support system that runs across the end of the barrel along the surface at 3 and 9 o'clock to the barrel Hadron Calorimeter. | 6.03.01.09.01 |
| 6.2 | 6.03.01.05 | 6.03.01.04 | I-DET-INF-BAR.002 | Weight Distributed | The weight of the barrel ECAL will be transferred to the barrel Hadron Calorimeter and must be accommodated by the dedicated support systems. | 6.03.01.09.01 |

Hadronic Calorimetry Systems to Backward EMCal Systems

| Section | WBS1 | WBS2 | ID | Name | Description | Owner |
| --- | --- | --- | --- | --- | --- | --- |
| 7 |  | 6.03.01.04 |  |  | Backward EMCal Systems |  |
| 7.1 | 6.03.01.05 | 6.03.01.04 | I-DET-INF-BAR.004 | Backward ECAL Support | The backward ECAL will be supported by rails integrated into the global support tube | 6.03.01.09.01 |
| 7.2 | 6.03.01.05 | 6.03.01.04 | I-DET-INF-INT.016 | Space Constraint | The backward position of the backward ECAL is limited by the backward Hadron Calorimeter and the adjacent cabling pathway that provides services to the interior detectors. Modifications to either must be coordinated. | 6.03.01.09.04 |

Hadronic Calorimetry Systems to Forward EMCal Systems

| Section | WBS1 | WBS2 | ID | Name | Description | Owner |
| --- | --- | --- | --- | --- | --- | --- |
| 8 |  | 6.03.01.04 |  |  | Forward EMCal Systems |  |
| 8.1 | 6.03.01.05 | 6.03.01.04 | I-DET-INF-BAR.007 | Forward ECAL Support | The forward ECAL will be supported by the forward Hadron Calorimeter endcap. Because the forward ECAL must split into two parts to provide access to the barrel, each half must be independently affixed to the Hadron Calorimeter halves. | 6.03.01.09.01 |
| 8.2 | 6.03.01.05 | 6.03.01.04 | I-DET-INF-BAR.008 | Weight Transfer | The weight of the forward ECAL will be transferred to the forward Hadron Calorimeter endcap and must be accommodated by all subsequent support systems. | 6.03.01.09.01 |
| 8.3 | 6.03.01.05 | 6.03.01.04 | I-DET-INF-FWD.004 | Sub-Detector Support | The forward HCAL will support the weight of the forward electromagnetic calorimeter that is embedded within it. | 6.03.01.09.03 |
| 8.4 | 6.03.01.05 | 6.03.01.04 | I-DET-INF-INT.023 | Space Constraint | The exterior radius of the forward ECAL is limited by the interior bore of the barrel Hadron Calorimeter. Modifications to either must be coordinated. | 6.03.01.09.04 |
| 8.5 | 6.03.01.05 | 6.03.01.04 | I-DET-INF-INT.024 | Kinematic Overlap | The exterior radius of the forward ECAL is constrained to provide continuous coverage with the barrel electromagnetic calorimeter. Modifications to either must be coordinated. | 6.03.01.09.04 |
| 8.6 | 6.03.01.05 | 6.03.01.04 | I-DET-INF-INT.025 | Space Constraint | The forward position of the forward ECAL is limited by the forward Hadron Calorimeter. Modifications to either must be coordinated. | 6.03.01.09.04 |

Hadronic Calorimetry Systems to Electronic Systems

| Section | WBS1 | WBS2 | ID | Name | Description | Owner |
| --- | --- | --- | --- | --- | --- | --- |
| 9 |  | 6.03.01.07 |  |  | Electronic Systems |  |
| 9.1 | 6.03.01.05 | 6.03.01.07 | I-DET-ELEC.010 | Power | High voltage, low voltage, and bias voltage DC power will be provided from the electronics racks on the south platform | 6.03.01.07 |
| 9.2 | 6.03.01.05 | 6.03.01.07 | I-DET-ELEC.011 | Power | High voltage, low voltage, and bias voltage DC power will be provided from the electronics racks on the south platform | 6.03.01.07 |
| 9.3 | 6.03.01.05 | 6.03.01.07 | I-DET-ELEC.012 | Power | High voltage, low voltage, and bias voltage DC power will be provided from the electronics racks on the south platform | 6.03.01.07 |

Hadronic Calorimetry Systems to Barrel Detector Structures

| Section | WBS1 | WBS2 | ID | Name | Description | Owner |
| --- | --- | --- | --- | --- | --- | --- |
| 10 |  | 6.03.01.09.01 |  |  | Barrel Detector Structures |  |
| 10.1 | 6.03.01.05 | 6.03.01.09.01 | I-DET-INF-BAR.001 | Barrel ECAL Support | The barrel ECAL will be supported by a structural support system that runs across the end of the barrel along the surface at 3 and 9 o'clock to the barrel Hadron Calorimeter. | 6.03.01.09.01 |
| 10.2 | 6.03.01.05 | 6.03.01.09.01 | I-DET-INF-BAR.002 | Weight Distributed | The weight of the barrel ECAL will be transferred to the barrel Hadron Calorimeter and must be accommodated by the dedicated support systems. | 6.03.01.09.01 |
| 10.3 | 6.03.01.05 | 6.03.01.09.01 | I-DET-INF-BAR.004 | Backward ECAL Support | The backward ECAL will be supported by rails integrated into the global support tube | 6.03.01.09.01 |
| 10.4 | 6.03.01.05 | 6.03.01.09.01 | I-DET-INF-BAR.007 | Forward ECAL Support | The forward ECAL will be supported by the forward Hadron Calorimeter endcap. Because the forward ECAL must split into two parts to provide access to the barrel, each half must be independently affixed to the Hadron Calorimeter halves. | 6.03.01.09.01 |
| 10.5 | 6.03.01.05 | 6.03.01.09.01 | I-DET-INF-BAR.008 | Weight Transfer | The weight of the forward ECAL will be transferred to the forward Hadron Calorimeter endcap and must be accommodated by all subsequent support systems. | 6.03.01.09.01 |
| 10.6 | 6.03.01.05 | 6.03.01.09.01 | I-DET-INF-BAR.009 | Barrel HCAL Support | The barrel HCAL will be supported by a rolling carriage that allows the central detector to be moved between the assembly area and the experimental hall. | 6.03.01.09.01 |
| 10.7 | 6.03.01.05 | 6.03.01.09.01 | I-DET-INF-BAR.010 | Sub-Detector Support | The barrel HCAL will support the weight of all of the sub-detectors and components within the central detector. | 6.03.01.09.01 |
| 10.8 | 6.03.01.05 | 6.03.01.09.01 | I-DET-INF-BAR.011 | Weight Transfer | The cumulative weight of the HCAL and all of the sub-detectors it supports will be transferred to the detector carriage. | 6.03.01.09.01 |
| 10.9 | 6.03.01.05 | 6.03.01.09.01 | I-DET-INF-BAR.012 | Dual RICH Support System | The Dual RICH will be supported by a structural system within the barrel hadron calorimeter. | 6.03.01.09.01 |

Hadronic Calorimetry Systems to Electron Endcap Structures

| Section | WBS1 | WBS2 | ID | Name | Description | Owner |
| --- | --- | --- | --- | --- | --- | --- |
| 11 |  | 6.03.01.09.02 |  |  | Electron Endcap Structures |  |
| 11.1 | 6.03.01.05 | 6.03.01.09.02 | I-DET-INF-BCK.001 | Backward HCAL Mobility | Adequate clearance must be provided for either of the endcap detector carriages to be rolled aside to allow access to sub-detectors within the barrel. | 6.03.01.09.02 |
| 11.2 | 6.03.01.05 | 6.03.01.09.02 | I-DET-INF-BCK.002 | Backward HCAL Support | The backward HCAL consists of two halves, each of which are supported by an independent carriage. | 6.03.01.09.02 |

Hadronic Calorimetry Systems to Hadron Endcap Structures

| Section | WBS1 | WBS2 | ID | Name | Description | Owner |
| --- | --- | --- | --- | --- | --- | --- |
| 12 |  | 6.03.01.09.03 |  |  | Hadron Endcap Structures |  |
| 12.1 | 6.03.01.05 | 6.03.01.09.03 | I-DET-INF-FWD.003 | Forward HCAL Support | The forward HCAL consists of two halves, each of which are support by an independent carriage. | 6.03.01.09.03 |
| 12.2 | 6.03.01.05 | 6.03.01.09.03 | I-DET-INF-FWD.004 | Sub-Detector Support | The forward HCAL will support the weight of the forward electromagnetic calorimeter that is embedded within it. | 6.03.01.09.03 |
| 12.3 | 6.03.01.05 | 6.03.01.09.03 | I-DET-INF-FWD.008 | Forward HCAL Mobility | Adequate clearance must be provided for either of the detector carriages to be rolled aside to allow access to sub-detectors within the barrel. | 6.03.01.09.03 |
| 12.4 | 6.03.01.05 | 6.03.01.09.03 | I-DET-INF-FWD.009 | Forward HCAL Mobility | Adequate clearance must be provided for either of the detector carriages to be rolled aside to allow access to sub-detectors within the barrel. | 6.03.01.09.03 |

Hadronic Calorimetry Systems to Detector Infrastructure and Utilities Integration

| Section | WBS1 | WBS2 | ID | Name | Description | Owner |
| --- | --- | --- | --- | --- | --- | --- |
| 13 |  | 6.03.01.09.04 |  |  | Detector Infrastructure and Utilities Integration |  |
| 13.1 | 6.03.01.05 | 6.03.01.09.04 | I-DET-INF-INT.005 | Space Constraint | The position of the forward HCAL is bounded by the required 50cm clearance between the backward HCAL surface and the B0 magnet in the forward direction. Changes to the size or position of either must be coordinated with the other. | 6.03.01.09.04 |
| 13.2 | 6.03.01.05 | 6.03.01.09.04 | I-DET-INF-INT.016 | Space Constraint | The backward position of the backward ECAL is limited by the backward Hadron Calorimeter and the adjacent cabling pathway that provides services to the interior detectors. Modifications to either must be coordinated. | 6.03.01.09.04 |
| 13.3 | 6.03.01.05 | 6.03.01.09.04 | I-DET-INF-INT.023 | Space Constraint | The exterior radius of the forward ECAL is limited by the interior bore of the barrel Hadron Calorimeter. Modifications to either must be coordinated. | 6.03.01.09.04 |
| 13.4 | 6.03.01.05 | 6.03.01.09.04 | I-DET-INF-INT.024 | Kinematic Overlap | The exterior radius of the forward ECAL is constrained to provide continuous coverage with the barrel electromagnetic calorimeter. Modifications to either must be coordinated. | 6.03.01.09.04 |
| 13.5 | 6.03.01.05 | 6.03.01.09.04 | I-DET-INF-INT.025 | Space Constraint | The forward position of the forward ECAL is limited by the forward Hadron Calorimeter. Modifications to either must be coordinated. | 6.03.01.09.04 |
| 13.6 | 6.03.01.05 | 6.03.01.09.04 | I-DET-INF-INT.028 | Space Constraint | The forward position of the backward HCAL is limited by the barrel HCAL and the adjacent cabling pathway that provides services to the interior detectors. Modifications to either must be coordinated. | 6.03.01.09.04 |
| 13.7 | 6.03.01.05 | 6.03.01.09.04 | I-DET-INF-INT.029 | Space Constraint | The exterior radius of the backward HCAL should be consistent with the radius of the barrel HCAL. | 6.03.01.09.04 |
| 13.8 | 6.03.01.05 | 6.03.01.09.04 | I-DET-INF-INT.030 | Space Constraint | The exterior radius of the forward HCAL should be consistent with the radius of the barrel HCAL. | 6.03.01.09.04 |
| 13.9 | 6.03.01.05 | 6.03.01.09.04 | I-DET-INF-INT.031 | Space Constraint | The backward position of the forward HCAL is limited by the barrel HCAL and the adjacent cabling pathway that provides services to the interior detectors. Modifications to either must be coordinated. | 6.03.01.09.04 |
| 13.10 | 6.03.01.05 | 6.03.01.09.04 | I-DET-INF-INT.032 | External Space Constraint | The exterior radius of the solenoid magnet is limited by the interior bore of the barrel HCAL and necessary support structures. Modifications to either must be coordinated. | 6.03.01.09.04 |
| 13.11 | 6.03.01.05 | 6.03.01.09.04 | I-DET-INF-INT.033 | Space Constraint | The size of the dRICH is limited by the interior bore of the barrel HCAL, and must provide adequate space for cables and services to itself and the interior detectors. Modifications to either must be coordinated. | 6.03.01.09.04 |
| 13.12 | 6.03.01.05 | 6.03.01.09.04 | I-DET-INF-INT.034 | Space Constraint | The forward position of the backward HCAL is limited by the barrel HCAL and the adjacent cabling pathway that provides services to the interior detectors. Modifications to either must be coordinated. | 6.03.01.09.04 |
| 13.13 | 6.03.01.05 | 6.03.01.09.04 | I-DET-INF-INT.035 | Space Constraint | The maximum backward location for the solenoid magnet is limited by the backward HCAL and the adjacent cabling pathway that provides services to the interior detectors. Modifications to either must be coordinated. | 6.03.01.09.04 |
| 13.14 | 6.03.01.05 | 6.03.01.09.04 | I-DET-INF-INT.036 | Space Constraint | The maximum backward location for the DIRC is limited by the backward endcap support structure and the adjacent cabling pathway that provides services to the interior detectors. Modifications to either must be coordinated. | 6.03.01.09.04 |
| 13.15 | 6.03.01.05 | 6.03.01.09.04 | I-DET-INF-INT.037 | Space Constraint | The position of the backward HCAL is limited in the backward direction by the adjacent accelerator magnets and the 50 cm clearance between the backward HCal surface and the backward IR magnets | 6.03.01.09.04 |
| 13.16 | 6.03.01.05 | 6.03.01.09.04 | I-DET-INF-INT.038 | Space Constraint | The interior radius of the backward HCAL is governed by the size of the beamline. | 6.03.01.09.04 |
| 13.17 | 6.03.01.05 | 6.03.01.09.04 | I-DET-INF-INT.039 | Space Constraint | The interior radius of the forward HCAL is governed by the size of the beamline. | 6.03.01.09.04 |
| 13.18 | 6.03.01.05 | 6.03.01.09.04 | I-DET-INF-INT.073 | Heat Rejection | Heat from the forward HCAL silicon photomultipliers and front-end electronics will be released into the outside room via forced air. | 6.03.01.09.04 |
| 13.19 | 6.03.01.05 | 6.03.01.09.04 | I-DET-INF-INT.074 | Space Constraint | The exterior radius of the barrel HCAL is limited by the detector carriage, the electronics platforms, and other Hall infrastructure. | 6.03.01.09.04 |
| 13.20 | 6.03.01.05 | 6.03.01.09.04 | I-DET-INF-INT.075 | Heat Rejection | Heat from the silicon photomultipliers and front-end electronics will be released into the outside room via forced air. | 6.03.01.09.04 |
| 13.21 | 6.03.01.05 | 6.03.01.09.04 | I-DET-INF-INT.076 | Heat Rejection | Heat from the silicon photomultipliers and front-end electronics will be released into the outside room via forced air. DO WE NEED CW FOR COOLING HERE? | 6.03.01.09.04 |

Hadronic Calorimetry Systems to Solenoid Magnet

| Section | WBS1 | WBS2 | ID | Name | Description | Owner |
| --- | --- | --- | --- | --- | --- | --- |
| 14 |  | 6.03.01.06 |  |  | Solenoid Magnet |  |
| 14.1 | 6.03.01.05 | 6.03.01.06 | I-DET-INF-INT.032 | External Space Constraint | The exterior radius of the solenoid magnet is limited by the interior bore of the barrel HCAL and necessary support structures. Modifications to either must be coordinated. | 6.03.01.09.04 |
| 14.2 | 6.03.01.05 | 6.03.01.06 | I-DET-INF-INT.035 | Space Constraint | The maximum backward location for the solenoid magnet is limited by the backward HCAL and the adjacent cabling pathway that provides services to the interior detectors. Modifications to either must be coordinated. | 6.03.01.09.04 |

Hadronic Calorimetry Systems to Barrel Particle ID Systems

| Section | WBS1 | WBS2 | ID | Name | Description | Owner |
| --- | --- | --- | --- | --- | --- | --- |
| 15 |  | 6.03.01.03 |  |  | Barrel Particle ID Systems |  |
| 15.1 | 6.03.01.05 | 6.03.01.03 | I-DET-INF-INT.036 | Space Constraint | The maximum backward location for the DIRC is limited by the backward endcap support structure and the adjacent cabling pathway that provides services to the interior detectors. Modifications to either must be coordinated. | 6.03.01.09.04 |

Hadronic Calorimetry Systems to Forward Particle ID Systems

| Section | WBS1 | WBS2 | ID | Name | Description | Owner |
| --- | --- | --- | --- | --- | --- | --- |
| 16 |  | 6.03.01.03 |  |  | Forward Particle ID Systems |  |
| 16.1 | 6.03.01.05 | 6.03.01.03 | I-DET-INF-BAR.012 | Dual RICH Support System | The Dual RICH will be supported by a structural system within the barrel hadron calorimeter. | 6.03.01.09.01 |
| 16.2 | 6.03.01.05 | 6.03.01.03 | I-DET-INF-INT.033 | Space Constraint | The size of the dRICH is limited by the interior bore of the barrel HCAL, and must provide adequate space for cables and services to itself and the interior detectors. Modifications to either must be coordinated. | 6.03.01.09.04 |

# Hadronic Calorimetry Systems (DET-HCAL) to B-Zero Detectors (DET-ANC-B0)

I-DET-INF-INT.005	Space Constraint

System 1:	B-Zero Detectors (6.03.01.10)

System 2:	Forward HCal Systems (6.03.01.05)

Type:	SPACE

Description:	The position of the forward HCAL is bounded by the required 50cm clearance between the backward HCAL surface and the B0 magnet in the forward direction. Changes to the size or position of either must be coordinated with the other.

Provider:	Detector Infrastructure and Utilities Integration (6.03.01.09.04)

Details:	See details in the Detector Infrastructure and Utilities Integration Interface Control Document.

Requirements:	P-DET-ANC.1, P-DET-HCAL.1

References:

# Hadronic Calorimetry Systems (DET-HCAL) to Online DAQ and Computing Systems (DET-COMP-ONLINE)

I-DET-COMP-ONLINE.022	Data Transfer and Control Interface

System 1:	Online DAQ and Computing Systems (6.03.01.08.02)

System 2:	Barrel HCal Systems (6.03.01.05)

Type:	DATA

Description:	A fiber connection will be provided from the DAQ system to the barrel HCAL's readout board to perform configuration, control, data acquisition, and timing.

Provider:	Online DAQ and Computing Systems (6.03.01.08.02)

Details:	See details in the Online DAQ and Computing Systems Interface Control Document.

Requirements:	F-DET-COMP.6, F-DET-COMP.11

References:

I-DET-COMP-ONLINE.023	Slow Controls

System 1:	Online DAQ and Computing Systems (6.03.01.08.02)

System 2:	Barrel HCal Systems (6.03.01.05)

Type:	CONTROL

Description:	A connection from the DAQ system to the barrel HCAL's slow controls interface.

Provider:	Online DAQ and Computing Systems (6.03.01.08.02)

Details:	See details in the Online DAQ and Computing Systems Interface Control Document.

Requirements:	F-DET-COMP-SC.1

References:

I-DET-COMP-ONLINE.024	Data Transfer and Control Interface

System 1:	Online DAQ and Computing Systems (6.03.01.08.02)

System 2:	Backward HCal Systems (6.03.01.05)

Type:	DATA

Description:	A fiber connection will be provided from the DAQ system to the backward HCAL's readout board to perform configuration, control, data acquisition, and timing.

Provider:	Online DAQ and Computing Systems (6.03.01.08.02)

Details:	See details in the Online DAQ and Computing Systems Interface Control Document.

Requirements:	F-DET-COMP.6, F-DET-COMP.11

References:

I-DET-COMP-ONLINE.025	Slow Controls

System 1:	Online DAQ and Computing Systems (6.03.01.08.02)

System 2:	Backward HCal Systems (6.03.01.05)

Type:	CONTROL

Description:	A connection from the DAQ system to the barrel HCAL's slow controls interface. CLARIFY with Data acquisition whether a slow control interface will be independent of the regular control interface.

Provider:	Online DAQ and Computing Systems (6.03.01.08.02)

Details:	See details in the Online DAQ and Computing Systems Interface Control Document.

Requirements:	F-DET-COMP-SC.1

References:

I-DET-COMP-ONLINE.026	Data Transfer and Control Interface

System 1:	Online DAQ and Computing Systems (6.03.01.08.02)

System 2:	Forward HCal Systems (6.03.01.05)

Type:	DATA

Description:	A fiber connection will be provided from the DAQ system to the forward HCAL's readout board to perform configuration, control, data acquisition, and timing.

Provider:	Online DAQ and Computing Systems (6.03.01.08.02)

Details:	See details in the Online DAQ and Computing Systems Interface Control Document.

Requirements:	F-DET-COMP.6, F-DET-COMP.11

References:

I-DET-COMP-ONLINE.027	Slow Controls

System 1:	Online DAQ and Computing Systems (6.03.01.08.02)

System 2:	Forward HCal Systems (6.03.01.05)

Type:	CONTROL

Description:	A connection from the DAQ system to the forward HCAL's slow controls interface. CLARIFY with Data acquisition whether a slow control interface will be independent of the regular control interface.

Provider:	Online DAQ and Computing Systems (6.03.01.08.02)

Details:	See details in the Online DAQ and Computing Systems Interface Control Document.

Requirements:	F-DET-COMP-SC.1

References:

# Hadronic Calorimetry Systems (DET-HCAL) to Barrel EMCal Systems (DET-ECAL-BAR)

I-DET-INF-BAR.001	Barrel ECAL Support

System 1:	Barrel EMCal Systems (6.03.01.04)

System 2:	Barrel HCal Systems (6.03.01.05)

Type:	STRUCT

Description:	The barrel ECAL will be supported by a structural support system that runs across the end of the barrel along the surface at 3 and 9 o'clock to the barrel Hadron Calorimeter.

Provider:	Barrel Detector Structures (6.03.01.09.01)

Details:	See details in the Barrel Detector Structures Interface Control Document.

Requirements:	P-DET-ECAL.2, P-DET-HCAL.2

References:

I-DET-INF-BAR.002	Weight Distributed

System 1:	Barrel EMCal Systems (6.03.01.04)

System 2:	Forward HCal Systems (6.03.01.05)

Type:	STRUCT

Description:	The weight of the barrel ECAL will be transferred to the barrel Hadron Calorimeter and must be accommodated by the dedicated support systems.

Provider:	Barrel Detector Structures (6.03.01.09.01)

Details:	See details in the Barrel Detector Structures Interface Control Document.

Requirements:	P-DET-ECAL.2, P-DET-HCAL.2

References:

# Hadronic Calorimetry Systems (DET-HCAL) to Backward EMCal Systems (DET-ECAL-BCK)

I-DET-INF-BAR.004	Backward ECAL Support

System 1:	Backward EMCal Systems (6.03.01.04)

System 2:	Forward HCal Systems (6.03.01.05)

Type:	STRUCT

Description:	The backward ECAL will be supported by rails integrated into the global support tube

Provider:	Barrel Detector Structures (6.03.01.09.01)

Details:	See details in the Barrel Detector Structures Interface Control Document.

Requirements:	P-DET-ECAL.2, P-DET-HCAL.2

References:

I-DET-INF-INT.016	Space Constraint

System 1:	Backward EMCal Systems (6.03.01.04)

System 2:	Backward HCal Systems (6.03.01.05)

Type:	SPACE

Description:	The backward position of the backward ECAL is limited by the backward Hadron Calorimeter and the adjacent cabling pathway that provides services to the interior detectors. Modifications to either must be coordinated.

Provider:	Detector Infrastructure and Utilities Integration (6.03.01.09.04)

Details:	See details in the Detector Infrastructure and Utilities Integration Interface Control Document.

Requirements:	P-DET-ECAL.1, P-DET-HCAL.1

References:

# Hadronic Calorimetry Systems (DET-HCAL) to Forward EMCal Systems (DET-ECAL-FWD)

I-DET-INF-BAR.007	Forward ECAL Support

System 1:	Forward EMCal Systems (6.03.01.04)

System 2:	Forward HCal Systems (6.03.01.05)

Type:	STRUCT

Description:	The forward ECAL will be supported by the forward Hadron Calorimeter endcap. Because the forward ECAL must split into two parts to provide access to the barrel, each half must be independently affixed to the Hadron Calorimeter halves.

Provider:	Barrel Detector Structures (6.03.01.09.01)

Details:	See details in the Barrel Detector Structures Interface Control Document.

Requirements:	P-DET-ECAL.1, P-DET-ECAL.2, P-DET-HCAL.2

References:

I-DET-INF-BAR.008	Weight Transfer

System 1:	Forward EMCal Systems (6.03.01.04)

System 2:	Forward HCal Systems (6.03.01.05)

Type:	STRUCT

Description:	The weight of the forward ECAL will be transferred to the forward Hadron Calorimeter endcap and must be accommodated by all subsequent support systems.

Provider:	Barrel Detector Structures (6.03.01.09.01)

Details:	See details in the Barrel Detector Structures Interface Control Document.

Requirements:	P-DET-ECAL.1, P-DET-ECAL.2, P-DET-HCAL.2

References:

I-DET-INF-FWD.004	Sub-Detector Support

System 1:	Forward HCal Systems (6.03.01.05)

System 2:	Forward EMCal Systems (6.03.01.04)

Type:	STRUCT

Description:	The forward HCAL will support the weight of the forward electromagnetic calorimeter that is embedded within it.

Provider:	Hadron Endcap Structures (6.03.01.09.03)

Details:	See details in the Hadron Endcap Structures Interface Control Document.

Requirements:	P-DET-HCAL.2

References:

I-DET-INF-INT.023	Space Constraint

System 1:	Forward EMCal Systems (6.03.01.04)

System 2:	Barrel HCal Systems (6.03.01.05)

Type:	SPACE

Description:	The exterior radius of the forward ECAL is limited by the interior bore of the barrel Hadron Calorimeter. Modifications to either must be coordinated.

Provider:	Detector Infrastructure and Utilities Integration (6.03.01.09.04)

Details:	See details in the Detector Infrastructure and Utilities Integration Interface Control Document.

Requirements:	P-DET-ECAL.1, P-DET-HCAL.1

References:

I-DET-INF-INT.024	Kinematic Overlap

System 1:	Forward EMCal Systems (6.03.01.04)

System 2:	Barrel HCal Systems (6.03.01.05)

Type:	SPACE

Description:	The exterior radius of the forward ECAL is constrained to provide continuous coverage with the barrel electromagnetic calorimeter. Modifications to either must be coordinated.

Provider:	Detector Infrastructure and Utilities Integration (6.03.01.09.04)

Details:	See details in the Detector Infrastructure and Utilities Integration Interface Control Document.

Requirements:	P-DET-ECAL.1, P-DET-HCAL.1

References:

I-DET-INF-INT.025	Space Constraint

System 1:	Forward EMCal Systems (6.03.01.04)

System 2:	Forward HCal Systems (6.03.01.05)

Type:	SPACE

Description:	The forward position of the forward ECAL is limited by the forward Hadron Calorimeter. Modifications to either must be coordinated.

Provider:	Detector Infrastructure and Utilities Integration (6.03.01.09.04)

Details:	See details in the Detector Infrastructure and Utilities Integration Interface Control Document.

Requirements:	P-DET-ECAL.1, P-DET-HCAL.1

References:

# Hadronic Calorimetry Systems (DET-HCAL) to Electronic Systems (DET-ELEC)

I-DET-ELEC.010	Power

System 1:	Electronic Systems (6.03.01.07)

System 2:	Barrel HCal Systems (6.03.01.05)

Type:	ELEC

Description:	High voltage, low voltage, and bias voltage DC power will be provided from the electronics racks on the south platform

Provider:	Electronic Systems (6.03.01.07)

Details:	See details in the Electronic Systems Interface Control Document.

Requirements:	P-DET-HCAL.4

References:

I-DET-ELEC.011	Power

System 1:	Electronic Systems (6.03.01.07)

System 2:	Backward HCal Systems (6.03.01.05)

Type:	ELEC

Description:	High voltage, low voltage, and bias voltage DC power will be provided from the electronics racks on the south platform

Provider:	Electronic Systems (6.03.01.07)

Details:	See details in the Electronic Systems Interface Control Document.

Requirements:	P-DET-HCAL.4

References:

I-DET-ELEC.012	Power

System 1:	Electronic Systems (6.03.01.07)

System 2:	Forward HCal Systems (6.03.01.05)

Type:	ELEC

Description:	High voltage, low voltage, and bias voltage DC power will be provided from the electronics racks on the south platform

Provider:	Electronic Systems (6.03.01.07)

Details:	See details in the Electronic Systems Interface Control Document.

Requirements:	P-DET-HCAL.4

References:

# Hadronic Calorimetry Systems (DET-HCAL) to Barrel Detector Structures (DET-INF-BAR)

I-DET-INF-BAR.001	Barrel ECAL Support

System 1:	Barrel EMCal Systems (6.03.01.04)

System 2:	Barrel HCal Systems (6.03.01.05)

Type:	STRUCT

Description:	The barrel ECAL will be supported by a structural support system that runs across the end of the barrel along the surface at 3 and 9 o'clock to the barrel Hadron Calorimeter.

Provider:	Barrel Detector Structures (6.03.01.09.01)

Details:	See details in the Barrel Detector Structures Interface Control Document.

Requirements:	P-DET-ECAL.2, P-DET-HCAL.2

References:

I-DET-INF-BAR.002	Weight Distributed

System 1:	Barrel EMCal Systems (6.03.01.04)

System 2:	Forward HCal Systems (6.03.01.05)

Type:	STRUCT

Description:	The weight of the barrel ECAL will be transferred to the barrel Hadron Calorimeter and must be accommodated by the dedicated support systems.

Provider:	Barrel Detector Structures (6.03.01.09.01)

Details:	See details in the Barrel Detector Structures Interface Control Document.

Requirements:	P-DET-ECAL.2, P-DET-HCAL.2

References:

I-DET-INF-BAR.004	Backward ECAL Support

System 1:	Backward EMCal Systems (6.03.01.04)

System 2:	Forward HCal Systems (6.03.01.05)

Type:	STRUCT

Description:	The backward ECAL will be supported by rails integrated into the global support tube

Provider:	Barrel Detector Structures (6.03.01.09.01)

Details:	See details in the Barrel Detector Structures Interface Control Document.

Requirements:	P-DET-ECAL.2, P-DET-HCAL.2

References:

I-DET-INF-BAR.007	Forward ECAL Support

System 1:	Forward EMCal Systems (6.03.01.04)

System 2:	Forward HCal Systems (6.03.01.05)

Type:	STRUCT

Description:	The forward ECAL will be supported by the forward Hadron Calorimeter endcap. Because the forward ECAL must split into two parts to provide access to the barrel, each half must be independently affixed to the Hadron Calorimeter halves.

Provider:	Barrel Detector Structures (6.03.01.09.01)

Details:	See details in the Barrel Detector Structures Interface Control Document.

Requirements:	P-DET-ECAL.1, P-DET-ECAL.2, P-DET-HCAL.2

References:

I-DET-INF-BAR.008	Weight Transfer

System 1:	Forward EMCal Systems (6.03.01.04)

System 2:	Forward HCal Systems (6.03.01.05)

Type:	STRUCT

Description:	The weight of the forward ECAL will be transferred to the forward Hadron Calorimeter endcap and must be accommodated by all subsequent support systems.

Provider:	Barrel Detector Structures (6.03.01.09.01)

Details:	See details in the Barrel Detector Structures Interface Control Document.

Requirements:	P-DET-ECAL.1, P-DET-ECAL.2, P-DET-HCAL.2

References:

I-DET-INF-BAR.009	Barrel HCAL Support

System 1:	Barrel HCal Systems (6.03.01.05)

System 2:	Barrel HCal Systems (6.03.01.05)

Type:	STRUCT

Description:	The barrel HCAL will be supported by a rolling carriage that allows the central detector to be moved between the assembly area and the experimental hall.

Provider:	Barrel Detector Structures (6.03.01.09.01)

Details:	See details in the Barrel Detector Structures Interface Control Document.

Requirements:	P-DET-HCAL.1, P-DET-HCAL.2

References:

I-DET-INF-BAR.010	Sub-Detector Support

System 1:	Barrel HCal Systems (6.03.01.05)

System 2:	Forward HCal Systems (6.03.01.05)

Type:	STRUCT

Description:	The barrel HCAL will support the weight of all of the sub-detectors and components within the central detector.

Provider:	Barrel Detector Structures (6.03.01.09.01)

Details:	See details in the Barrel Detector Structures Interface Control Document.

Requirements:	P-DET-HCAL.2

References:

I-DET-INF-BAR.011	Weight Transfer

System 1:	Barrel HCal Systems (6.03.01.05)

System 2:	Forward HCal Systems (6.03.01.05)

Type:	STRUCT

Description:	The cumulative weight of the HCAL and all of the sub-detectors it supports will be transferred to the detector carriage.

Provider:	Barrel Detector Structures (6.03.01.09.01)

Details:	See details in the Barrel Detector Structures Interface Control Document.

Requirements:	P-DET-HCAL.2

References:

I-DET-INF-BAR.012	Dual RICH Support System

System 1:	Barrel HCal Systems (6.03.01.05)

System 2:	Forward Particle ID Systems (6.03.01.03)

Type:	STRUCT

Description:	The Dual RICH will be supported by a structural system within the barrel hadron calorimeter.

Provider:	Barrel Detector Structures (6.03.01.09.01)

Details:	See details in the Barrel Detector Structures Interface Control Document.

Requirements:	P-DET-HCAL.2, P-DET-PID.4

References:

# Hadronic Calorimetry Systems (DET-HCAL) to Electron Endcap Structures (DET-INF-BCK)

I-DET-INF-BCK.001	Backward HCAL Mobility

System 1:	Backward HCal Systems (6.03.01.05)

System 2:	Forward HCal Systems (6.03.01.05)

Type:	MECH

Description:	Adequate clearance must be provided for either of the endcap detector carriages to be rolled aside to allow access to sub-detectors within the barrel.

Provider:	Electron Endcap Structures (6.03.01.09.02)

Details:	See details in the Electron Endcap Structures Interface Control Document.

Requirements:	P-DET-HCAL.2

References:

I-DET-INF-BCK.002	Backward HCAL Support

System 1:	Backward HCal Systems (6.03.01.05)

System 2:	Forward HCal Systems (6.03.01.05)

Type:	STRUCT

Description:	The backward HCAL consists of two halves, each of which are supported by an independent carriage.

Provider:	Electron Endcap Structures (6.03.01.09.02)

Details:	See details in the Electron Endcap Structures Interface Control Document.

Requirements:	P-DET-HCAL.2

References:

# Hadronic Calorimetry Systems (DET-HCAL) to Hadron Endcap Structures (DET-INF-FWD)

I-DET-INF-FWD.003	Forward HCAL Support

System 1:	Forward HCal Systems (6.03.01.05)

System 2:	Forward HCal Systems (6.03.01.05)

Type:	STRUCT

Description:	The forward HCAL consists of two halves, each of which are support by an independent carriage.

Provider:	Hadron Endcap Structures (6.03.01.09.03)

Details:	See details in the Hadron Endcap Structures Interface Control Document.

Requirements:	P-DET-HCAL.2

References:

I-DET-INF-FWD.004	Sub-Detector Support

System 1:	Forward HCal Systems (6.03.01.05)

System 2:	Forward EMCal Systems (6.03.01.04)

Type:	STRUCT

Description:	The forward HCAL will support the weight of the forward electromagnetic calorimeter that is embedded within it.

Provider:	Hadron Endcap Structures (6.03.01.09.03)

Details:	See details in the Hadron Endcap Structures Interface Control Document.

Requirements:	P-DET-HCAL.2

References:

I-DET-INF-FWD.008	Forward HCAL Mobility

System 1:	Hadron Endcap Structures (6.03.01.09.03)

System 2:	Forward HCal Systems (6.03.01.05)

Type:	MECH

Description:	Adequate clearance must be provided for either of the detector carriages to be rolled aside to allow access to sub-detectors within the barrel.

Provider:	Hadron Endcap Structures (6.03.01.09.03)

Details:	See details in the Hadron Endcap Structures Interface Control Document.

Requirements:	P-DET-HCAL.1, P-DET-HCAL.2

References:

I-DET-INF-FWD.009	Forward HCAL Mobility

System 1:	Hadron Endcap Structures (6.03.01.09.03)

System 2:	Forward HCal Systems (6.03.01.05)

Type:	DELETED

Description:	Adequate clearance must be provided for either of the detector carriages to be rolled aside to allow access to sub-detectors within the barrel.

Provider:	Hadron Endcap Structures (6.03.01.09.03)

Details:	See details in the Hadron Endcap Structures Interface Control Document.

Requirements:	P-DET-HCAL.1, P-DET-HCAL.2

References:

# Hadronic Calorimetry Systems (DET-HCAL) to Detector Infrastructure and Utilities Integration (DET-INF-INT)

I-DET-INF-INT.005	Space Constraint

System 1:	B-Zero Detectors (6.03.01.10)

System 2:	Forward HCal Systems (6.03.01.05)

Type:	SPACE

Description:	The position of the forward HCAL is bounded by the required 50cm clearance between the backward HCAL surface and the B0 magnet in the forward direction. Changes to the size or position of either must be coordinated with the other.

Provider:	Detector Infrastructure and Utilities Integration (6.03.01.09.04)

Details:	See details in the Detector Infrastructure and Utilities Integration Interface Control Document.

Requirements:	P-DET-ANC.1, P-DET-HCAL.1

References:

I-DET-INF-INT.016	Space Constraint

System 1:	Backward EMCal Systems (6.03.01.04)

System 2:	Backward HCal Systems (6.03.01.05)

Type:	SPACE

Description:	The backward position of the backward ECAL is limited by the backward Hadron Calorimeter and the adjacent cabling pathway that provides services to the interior detectors. Modifications to either must be coordinated.

Provider:	Detector Infrastructure and Utilities Integration (6.03.01.09.04)

Details:	See details in the Detector Infrastructure and Utilities Integration Interface Control Document.

Requirements:	P-DET-ECAL.1, P-DET-HCAL.1

References:

I-DET-INF-INT.023	Space Constraint

System 1:	Forward EMCal Systems (6.03.01.04)

System 2:	Barrel HCal Systems (6.03.01.05)

Type:	SPACE

Description:	The exterior radius of the forward ECAL is limited by the interior bore of the barrel Hadron Calorimeter. Modifications to either must be coordinated.

Provider:	Detector Infrastructure and Utilities Integration (6.03.01.09.04)

Details:	See details in the Detector Infrastructure and Utilities Integration Interface Control Document.

Requirements:	P-DET-ECAL.1, P-DET-HCAL.1

References:

I-DET-INF-INT.024	Kinematic Overlap

System 1:	Forward EMCal Systems (6.03.01.04)

System 2:	Barrel HCal Systems (6.03.01.05)

Type:	SPACE

Description:	The exterior radius of the forward ECAL is constrained to provide continuous coverage with the barrel electromagnetic calorimeter. Modifications to either must be coordinated.

Provider:	Detector Infrastructure and Utilities Integration (6.03.01.09.04)

Details:	See details in the Detector Infrastructure and Utilities Integration Interface Control Document.

Requirements:	P-DET-ECAL.1, P-DET-HCAL.1

References:

I-DET-INF-INT.025	Space Constraint

System 1:	Forward EMCal Systems (6.03.01.04)

System 2:	Forward HCal Systems (6.03.01.05)

Type:	SPACE

Description:	The forward position of the forward ECAL is limited by the forward Hadron Calorimeter. Modifications to either must be coordinated.

Provider:	Detector Infrastructure and Utilities Integration (6.03.01.09.04)

Details:	See details in the Detector Infrastructure and Utilities Integration Interface Control Document.

Requirements:	P-DET-ECAL.1, P-DET-HCAL.1

References:

I-DET-INF-INT.028	Space Constraint

System 1:	Barrel HCal Systems (6.03.01.05)

System 2:	Backward HCal Systems (6.03.01.05)

Type:	SPACE

Description:	The forward position of the backward HCAL is limited by the barrel HCAL and the adjacent cabling pathway that provides services to the interior detectors. Modifications to either must be coordinated.

Provider:	Detector Infrastructure and Utilities Integration (6.03.01.09.04)

Details:	See details in the Detector Infrastructure and Utilities Integration Interface Control Document.

Requirements:	P-DET-HCAL.1

References:

I-DET-INF-INT.029	Space Constraint

System 1:	Barrel HCal Systems (6.03.01.05)

System 2:	Backward HCal Systems (6.03.01.05)

Type:	SPACE

Description:	The exterior radius of the backward HCAL should be consistent with the radius of the barrel HCAL.

Provider:	Detector Infrastructure and Utilities Integration (6.03.01.09.04)

Details:	See details in the Detector Infrastructure and Utilities Integration Interface Control Document.

Requirements:	P-DET-HCAL.1

References:

I-DET-INF-INT.030	Space Constraint

System 1:	Barrel HCal Systems (6.03.01.05)

System 2:	Forward HCal Systems (6.03.01.05)

Type:	SPACE

Description:	The exterior radius of the forward HCAL should be consistent with the radius of the barrel HCAL.

Provider:	Detector Infrastructure and Utilities Integration (6.03.01.09.04)

Details:	See details in the Detector Infrastructure and Utilities Integration Interface Control Document.

Requirements:	P-DET-HCAL.1

References:

I-DET-INF-INT.031	Space Constraint

System 1:	Barrel HCal Systems (6.03.01.05)

System 2:	Forward HCal Systems (6.03.01.05)

Type:	SPACE

Description:	The backward position of the forward HCAL is limited by the barrel HCAL and the adjacent cabling pathway that provides services to the interior detectors. Modifications to either must be coordinated.

Provider:	Detector Infrastructure and Utilities Integration (6.03.01.09.04)

Details:	See details in the Detector Infrastructure and Utilities Integration Interface Control Document.

Requirements:	P-DET-HCAL.1

References:

I-DET-INF-INT.032	External Space Constraint

System 1:	Barrel HCal Systems (6.03.01.05)

System 2:	Solenoid Magnet (6.03.01.06)

Type:	SPACE

Description:	The exterior radius of the solenoid magnet is limited by the interior bore of the barrel HCAL and necessary support structures. Modifications to either must be coordinated.

Provider:	Detector Infrastructure and Utilities Integration (6.03.01.09.04)

Details:	See details in the Detector Infrastructure and Utilities Integration Interface Control Document.

Requirements:	P-DET-HCAL.1, P-DET-MAG.1

References:

I-DET-INF-INT.033	Space Constraint

System 1:	Barrel HCal Systems (6.03.01.05)

System 2:	Forward RICH Detector Systems (6.03.01.03)

Type:	SPACE

Description:	The size of the dRICH is limited by the interior bore of the barrel HCAL, and must provide adequate space for cables and services to itself and the interior detectors. Modifications to either must be coordinated.

Provider:	Detector Infrastructure and Utilities Integration (6.03.01.09.04)

Details:	See details in the Detector Infrastructure and Utilities Integration Interface Control Document.

Requirements:	P-DET-HCAL.1, P-DET-PID.5

References:

I-DET-INF-INT.034	Space Constraint

System 1:	Backward HCal Systems (6.03.01.05)

System 2:	Barrel HCal Systems (6.03.01.05)

Type:	SPACE

Description:	The forward position of the backward HCAL is limited by the barrel HCAL and the adjacent cabling pathway that provides services to the interior detectors. Modifications to either must be coordinated.

Provider:	Detector Infrastructure and Utilities Integration (6.03.01.09.04)

Details:	See details in the Detector Infrastructure and Utilities Integration Interface Control Document.

Requirements:	P-DET-HCAL.1

References:

I-DET-INF-INT.035	Space Constraint

System 1:	Backward HCal Systems (6.03.01.05)

System 2:	Solenoid Magnet (6.03.01.06)

Type:	SPACE

Description:	The maximum backward location for the solenoid magnet is limited by the backward HCAL and the adjacent cabling pathway that provides services to the interior detectors. Modifications to either must be coordinated.

Provider:	Detector Infrastructure and Utilities Integration (6.03.01.09.04)

Details:	See details in the Detector Infrastructure and Utilities Integration Interface Control Document.

Requirements:	P-DET-HCAL.1, P-DET-MAG.1

References:

I-DET-INF-INT.036	Space Constraint

System 1:	Backward HCal Systems (6.03.01.05)

System 2:	Barrel DIRC Particle ID Systems (6.03.01.03)

Type:	SPACE

Description:	The maximum backward location for the DIRC is limited by the backward endcap support structure and the adjacent cabling pathway that provides services to the interior detectors. Modifications to either must be coordinated.

Provider:	Detector Infrastructure and Utilities Integration (6.03.01.09.04)

Details:	See details in the Detector Infrastructure and Utilities Integration Interface Control Document.

Requirements:	P-DET-HCAL.1, P-DET-PID.5

References:

I-DET-INF-INT.037	Space Constraint

System 1:	Backward HCal Systems (6.03.01.05)

System 2:	Interaction Region (6.04)

Type:	SPACE

Description:	The position of the backward HCAL is limited in the backward direction by the adjacent accelerator magnets and the 50 cm clearance between the backward HCal surface and the backward IR magnets

Provider:	Detector Infrastructure and Utilities Integration (6.03.01.09.04)

Details:	See details in the Detector Infrastructure and Utilities Integration Interface Control Document.

Requirements:	P-DET-HCAL.1

References:

I-DET-INF-INT.038	Space Constraint

System 1:	Backward HCal Systems (6.03.01.05)

System 2:	IR Accelerator Systems (6.04.03)

Type:	SPACE

Description:	The interior radius of the backward HCAL is governed by the size of the beamline.

Provider:	Detector Infrastructure and Utilities Integration (6.03.01.09.04)

Details:	See details in the Detector Infrastructure and Utilities Integration Interface Control Document.

Requirements:	P-DET-HCAL.1

References:

I-DET-INF-INT.039	Space Constraint

System 1:	Forward HCal Systems (6.03.01.05)

System 2:	IR Accelerator Systems (6.04.03)

Type:	SPACE

Description:	The interior radius of the forward HCAL is governed by the size of the beamline.

Provider:	Detector Infrastructure and Utilities Integration (6.03.01.09.04)

Details:	See details in the Detector Infrastructure and Utilities Integration Interface Control Document.

Requirements:	P-DET-HCAL.1

References:

I-DET-INF-INT.073	Heat Rejection

System 1:	Detector Infrastructure and Utilities Integration (6.03.01.09.04)

System 2:	Barrel HCal Systems (6.03.01.05)

Type:	COOL

Description:	Heat from the forward HCAL silicon photomultipliers and front-end electronics will be released into the outside room via forced air.

Provider:	Detector Infrastructure and Utilities Integration (6.03.01.09.04)

Details:	See details in the Detector Infrastructure and Utilities Integration Interface Control Document.

Requirements:	P-DET-HCAL.3

References:

I-DET-INF-INT.074	Space Constraint

System 1:	Detector Infrastructure and Utilities Integration (6.03.01.09.04)

System 2:	Barrel HCal Systems (6.03.01.05)

Type:	SPACE

Description:	The exterior radius of the barrel HCAL is limited by the detector carriage, the electronics platforms, and other Hall infrastructure.

Provider:	Detector Infrastructure and Utilities Integration (6.03.01.09.04)

Details:	See details in the Detector Infrastructure and Utilities Integration Interface Control Document.

Requirements:	P-DET-HCAL.1

References:

I-DET-INF-INT.075	Heat Rejection

System 1:	Detector Infrastructure and Utilities Integration (6.03.01.09.04)

System 2:	Backward HCal Systems (6.03.01.05)

Type:	COOL

Description:	Heat from the silicon photomultipliers and front-end electronics will be released into the outside room via forced air.

Provider:	Detector Infrastructure and Utilities Integration (6.03.01.09.04)

Details:	See details in the Detector Infrastructure and Utilities Integration Interface Control Document.

Requirements:	P-DET-HCAL.3

References:

I-DET-INF-INT.076	Heat Rejection

System 1:	Detector Infrastructure and Utilities Integration (6.03.01.09.04)

System 2:	Forward HCal Systems (6.03.01.05)

Type:	COOL

Description:	Heat from the silicon photomultipliers and front-end electronics will be released into the outside room via forced air. DO WE NEED CW FOR COOLING HERE?

Provider:	Detector Infrastructure and Utilities Integration (6.03.01.09.04)

Details:	See details in the Detector Infrastructure and Utilities Integration Interface Control Document.

Requirements:	P-DET-HCAL.3

References:

# Hadronic Calorimetry Systems (DET-HCAL) to Solenoid Magnet (DET-MAG)

I-DET-INF-INT.032	External Space Constraint

System 1:	Barrel HCal Systems (6.03.01.05)

System 2:	Solenoid Magnet (6.03.01.06)

Type:	SPACE

Description:	The exterior radius of the solenoid magnet is limited by the interior bore of the barrel HCAL and necessary support structures. Modifications to either must be coordinated.

Provider:	Detector Infrastructure and Utilities Integration (6.03.01.09.04)

Details:	See details in the Detector Infrastructure and Utilities Integration Interface Control Document.

Requirements:	P-DET-HCAL.1, P-DET-MAG.1

References:

I-DET-INF-INT.035	Space Constraint

System 1:	Backward HCal Systems (6.03.01.05)

System 2:	Solenoid Magnet (6.03.01.06)

Type:	SPACE

Description:	The maximum backward location for the solenoid magnet is limited by the backward HCAL and the adjacent cabling pathway that provides services to the interior detectors. Modifications to either must be coordinated.

Provider:	Detector Infrastructure and Utilities Integration (6.03.01.09.04)

Details:	See details in the Detector Infrastructure and Utilities Integration Interface Control Document.

Requirements:	P-DET-HCAL.1, P-DET-MAG.1

References:

# Hadronic Calorimetry Systems (DET-HCAL) to Barrel Particle ID Systems (DET-PID-BAR)

I-DET-INF-INT.036	Space Constraint

System 1:	Backward HCal Systems (6.03.01.05)

System 2:	Barrel DIRC Particle ID Systems (6.03.01.03)

Type:	SPACE

Description:	The maximum backward location for the DIRC is limited by the backward endcap support structure and the adjacent cabling pathway that provides services to the interior detectors. Modifications to either must be coordinated.

Provider:	Detector Infrastructure and Utilities Integration (6.03.01.09.04)

Details:	See details in the Detector Infrastructure and Utilities Integration Interface Control Document.

Requirements:	P-DET-HCAL.1, P-DET-PID.5

References:

# Hadronic Calorimetry Systems (DET-HCAL) to Forward Particle ID Systems (DET-PID-FWD)

I-DET-INF-BAR.012	Dual RICH Support System

System 1:	Barrel HCal Systems (6.03.01.05)

System 2:	Forward Particle ID Systems (6.03.01.03)

Type:	STRUCT

Description:	The Dual RICH will be supported by a structural system within the barrel hadron calorimeter.

Provider:	Barrel Detector Structures (6.03.01.09.01)

Details:	See details in the Barrel Detector Structures Interface Control Document.

Requirements:	P-DET-HCAL.2, P-DET-PID.4

References:

I-DET-INF-INT.033	Space Constraint

System 1:	Barrel HCal Systems (6.03.01.05)

System 2:	Forward RICH Detector Systems (6.03.01.03)

Type:	SPACE

Description:	The size of the dRICH is limited by the interior bore of the barrel HCAL, and must provide adequate space for cables and services to itself and the interior detectors. Modifications to either must be coordinated.

Provider:	Detector Infrastructure and Utilities Integration (6.03.01.09.04)

Details:	See details in the Detector Infrastructure and Utilities Integration Interface Control Document.

Requirements:	P-DET-HCAL.1, P-DET-PID.5

References:

# Applicable Documents

List applicable documents considered to be part of this document. Applicable documents shall be obtained from the EIC SharePoint Document Center.

# Detailed change Log

| Revision | Date | Change |
| --- | --- | --- |

# References

- doi:10.17226/25171., National Academies of Sciences, Engineering, and Medicine, "An Assessment of U.S.-Based Electron-Ion Collider Science", The National Academies Press, Washington, DC, 2018.

- EIC-ORG-PLN-010, Electron-Ion Collider L1 Requirements (Global Requirements Document)

- BNL-221006-2021-FORE, EIC Conceptual Design Report

- arXiv:2103.05419, Science Requirements and Detector Concepts for the Electron-Ion Collider: EIC Yellow Report

- EIC-SEG-PLN-016, Requirements Management Plan

- EIC-SEG-PLN-022, Systems Engineering Management Plan

- EIC-SEG-PLN-020, EIC Interface Management Plan