Electron-Ion Collider, Requirements, Specifications, and Interfaces

Interface Control Document: [###]

Detector Infrastructure Systems (6.03.01.09) to Ancillary Detectors (6.03.01.10)

Prepared by:

{{Sig_es_:signer1:signature}}	Date: {{Dte_es_:signer1:date}}

Name, Role

Electron-Ion Collider at Institution

Reviewed by:

{{Sig_es_:signer2:signature}}	Date: {{Dte_es_:signer2:date}}

Name, Role

Electron-Ion Collider at Institution

{{Sig_es_:signer3:signature}}	Date: {{Dte_es_:signer3:date}}

Name, Role

Electron-Ion Collider at Institution

{{Sig_es_:signer4:signature}}	Date: {{Dte_es_:signer4:date}}

Name, Role

Electron-Ion Collider at Institution

Approved by:

{{Sig_es_:signer5:signature}}	Date: {{Dte_es_:signer5:date}}

Name, Role

Electron-Ion Collider at Institution

REVISION HISTORY

| Revision # | Effective Date | Additional Reviewers | Summary of Change |
| --- | --- | --- | --- |
| 00 |  |  | Initial release. |

TABLE OF CONTENTS

1.	Purpose and Scope	6

2.	Introduction	6

2.1	Detector Infrastructure Systems	6

2.2	Overview of Interface Relationships	7

2.3	Safety and Environmental Considerations	9

2.4	Design Considerations	9

2.5	Organization	13

3.	Interface summary	13

4.	Detector Infrastructure Systems (DET-INF) to B-Zero Detectors (DET-ANC-B0)	61

5.	Detector Infrastructure Systems (DET-INF) to Low Q2 Detectors (DET-ANC-LOWQ2)	63

6.	Detector Infrastructure Systems (DET-INF) to Luminosity Detectors (DET-ANC-LUMI)	65

7.	Detector Infrastructure Systems (DET-INF) to Off-Momentum Detectors (DET-ANC-OFFMO)	65

8.	Detector Infrastructure Systems (DET-INF) to Roman Pots (DET-ANC-ROMAN)	66

9.	Detector Infrastructure Systems (DET-INF) to Zero Degree Calorimeter (DET-ANC-ZDC)	67

10.	Detector Infrastructure Systems (DET-INF) to Online DAQ and Computing Systems (DET-COMP-ONLINE)	67

11.	Detector Infrastructure Systems (DET-INF) to  (DET-DS)	72

12.	Detector Infrastructure Systems (DET-INF) to Barrel EMCal Systems (DET-ECAL-BAR)	72

13.	Detector Infrastructure Systems (DET-INF) to Backward EMCal Systems (DET-ECAL-BCK)	75

14.	Detector Infrastructure Systems (DET-INF) to Forward EMCal Systems (DET-ECAL-FWD)	78

15.	Detector Infrastructure Systems (DET-INF) to Electronic Systems (DET-ELEC)	81

16.	Detector Infrastructure Systems (DET-INF) to Barrel HCal Systems (DET-HCAL-BAR)	82

17.	Detector Infrastructure Systems (DET-INF) to Backward HCal Systems (DET-HCAL-BCK)	87

18.	Detector Infrastructure Systems (DET-INF) to Forward HCal Systems (DET-HCAL-FWD)	90

19.	Detector Infrastructure Systems (DET-INF) to Solenoid Magnet (DET-MAG)	95

20.	Detector Infrastructure Systems (DET-INF) to Magnet Instrumentation and Control (DET-MAG-I&C)	100

21.	Detector Infrastructure Systems (DET-INF) to Magnet Power Supply (DET-MAG-PSU)	100

22.	Detector Infrastructure Systems (DET-INF) to Barrel Particle ID Systems (DET-PID-BAR)	101

23.	Detector Infrastructure Systems (DET-INF) to Backward Particle ID Systems (DET-PID-BCK)	107

24.	Detector Infrastructure Systems (DET-INF) to Forward Particle ID Systems (DET-PID-FWD)	110

25.	Detector Infrastructure Systems (DET-INF) to Tracking Systems (DET-TRAK)	115

26.	Detector Infrastructure Systems (DET-INF) to Barrel Tracking Systems (DET-TRAK-BAR)	115

27.	Applicable Documents	118

28.	Detailed change Log	118

29.	References	118

LIST OF ACRONYMS

| B0 | B0 is the name of the forward magnet just past the Hadron Endcap |  |
| --- | --- | --- |
| CDR | Conceptual Design Report |  |
| COTS | Commercial Off the Shelf |  |
| DAQ | Data Acquisition |  |
| DET | EIC Detector |  |
| DIRC | Detection of Internally Reflected Cherenkov Light |  |
| dRICH | Dual radiator ring imaging Cherenkov |  |
| DVCS | Deeply Virtual Compton Scattering |  |
| EIC | Electron-Ion Collider |  |
| EMCAL | Electro-magnetic Calorimeter |  |
| ESR | Electron Storage Ring |  |
| FCC | Federal Communications Commission |  |
| FDR | Final Design Review |  |
| GEM | Gas Electron Multiplier |  |
| HCAL | Hadronic Calorimeter |  |
| HSR | Hadron Storage Ring |  |
| HVAC | Heating/Ventilation/Air Conditioning |  |
| IP | Interaction Point |  |
| IR | Interaction Region |  |
| MPGD | Micro Pattern Gaseous Detectors |  |
| NAS | National Academy of Sciences |  |
| NEC | National Electric Code |  |
| NEMA | National Electrical Manufacturers Association |  |
| NFPA | National Fire Protection Association |  |
| NRTL | Nationally Recognized Testing Laboratory |  |
| PID | Particle Identification |  |
| PRR | Production Readiness Review |  |
| PWO | PbWO4 |  |
| RCS | Rapid Cycling Synchrotron |  |
| TBD | To Be Determined |  |
| UL | Underwriter Laboratories |  |
| WBS | Work Break Down Structure |  |

# Interface Control Document:

Detector Infrastructure Systems (6.03.01.09) to Ancillary Detectors (6.03.01.10)

# Purpose and Scope

This document records the agreed upon interfaces between the Detector Infrastructure Systems and other sub-systems within the Ancillary Detectors. These interfaces are based on the system requirements that are necessary to integrate these sub-systems as part of the Electron Ion Collider Project.

# Introduction

These interfaces describe the relationships between the Detector Infrastructure Systems and the Ancillary Detectors and are divided into two key areas:

Detector Envelope Definition and Control: this includes issues such as physical dimensions of components, interface locations, clearances, keep-out zones, dynamic movements, material deflections, weight distribution, integration, accessibility, and environmental constraints.

Resource Provisioning and Distribution: this includes the provisioning and distribution of resources such as power, cooling, low-conductivity water, cryogens, fluids, gases, etc., as well as the removal of waste energy (heat) and material.

## Detector Infrastructure Systems

Detector infrastructure represents a broad collection of interfaces that tie together all of the subsystems within the detector.  As stated in the introduction, the interfaces within the detector are largely divided into the two categories below.

### Detector Envelope Definition and Control

The subsystem envelopes define the allowable three-dimensional space within which each subsystem must remain throughout all phases of integration, operation, and maintenance. This includes its physical geometry, mounting features, keep-out zones, and any expected motion, growth, or displacement under environmental loads. Together, these controls ensure that all subsystems can be integrated without conflict, maintain required clearances under all operating conditions, and support reliable system-level performance and maintainability.

The envelope controls:

Physical Dimensions and Shape:

Establishes maximum external dimensions and geometric boundaries to ensure all subsystems fit within the allocated volume without interference.

Mounting and Interface Locations:

Specifies the locations and tolerances of mechanical interfaces, fasteners, alignment features, and support structures to ensure compatibility with adjoining subsystems.

Clearances and Keep-Out Zones:

Defines required separation distances from adjacent subsystems, cables, access paths, and structural elements. These zones prevent physical interference during assembly, operation, or service.

Dynamic Movements and Deflections:

Accounts for expected deflections, vibration-induced motion, thermal expansion, and any mechanism actuation, ensuring that no part of the subsystem encroaches beyond the approved boundary.

Mass Properties Placement:

Constrains the placement of high-mass components within the envelope to maintain the system’s mass distribution, center of gravity, and structural loading requirements.

Integration and Accessibility Requirements:

Ensures adequate access for installation, alignment, cabling, inspection, and removal without violating adjacent subsystem envelopes.

Environmental Constraints:

Identifies any volume needed for thermal insulation, shielding, or other environment-driven additions that must be accounted for in the overall system layout.

### Resource Provisioning and Distribution

These interfaces describe how material, energy, and waste are exchanged between the larger, external infrastructure and the detector, and between each of the subsystems within the detector. While the detector envelope identifies the various openings and pathways that will exist, these interfaces describe the physical mechanisms that must be accommodated, what they will carry, and how they will potentially interact or interfere with one another.

        - External Infrastructure

These interfaces include the coordination of incoming cryogenic supply and return connections, low conductivity water services, bulk power to the switchboards, and any other resource that is between the detector and an external system.

Internal Infrastructure

These interfaces describe how energy, material and waste are exchanged between the detector’s subsystems. This includes the internal distribution of power, cooling water, data cabling, fluids, gases, and other materials, as well as the removal of waste heat, fluids, and gases that may accumulate within the detector.

## Overview of Interface Relationships

The following are the sub-systems within the Ancillary Detectors that have interfaces connected to the Detector Infrastructure Systems.

B-Zero Detectors (DET-ANC-B0): Identify the general types of interface relationships between the ICD reference system and this sub-system.

Low Q2 Detectors (DET-ANC-LOWQ2): Identify the general types of interface relationships between the ICD reference system and this sub-system.

Luminosity Detectors (DET-ANC-LUMI): Identify the general types of interface relationships between the ICD reference system and this sub-system.

Off-Momentum Detectors (DET-ANC-OFFMO): Identify the general types of interface relationships between the ICD reference system and this sub-system.

Roman Pots (DET-ANC-ROMAN): Identify the general types of interface relationships between the ICD reference system and this sub-system.

Zero Degree Calorimeter (DET-ANC-ZDC): Identify the general types of interface relationships between the ICD reference system and this sub-system.

Online DAQ and Computing Systems (DET-COMP-ONLINE): Identify the general types of interface relationships between the ICD reference system and this sub-system.

(DET-DS): Identify the general types of interface relationships between the ICD reference system and this sub-system.

Barrel EMCal Systems (DET-ECAL-BAR): Identify the general types of interface relationships between the ICD reference system and this sub-system.

Backward EMCal Systems (DET-ECAL-BCK): Identify the general types of interface relationships between the ICD reference system and this sub-system.

Forward EMCal Systems (DET-ECAL-FWD): Identify the general types of interface relationships between the ICD reference system and this sub-system.

Electronic Systems (DET-ELEC): Identify the general types of interface relationships between the ICD reference system and this sub-system.

Barrel HCal Systems (DET-HCAL-BAR): Identify the general types of interface relationships between the ICD reference system and this sub-system.

Backward HCal Systems (DET-HCAL-BCK): Identify the general types of interface relationships between the ICD reference system and this sub-system.

Forward HCal Systems (DET-HCAL-FWD): Identify the general types of interface relationships between the ICD reference system and this sub-system.

Solenoid Magnet (DET-MAG): These connections provide physical support and stabilization for the magnet, as well as resources such as cooling water, cryogenics, power and communications

Magnet Instrumentation and Control (DET-MAG-I&C): Identify the general types of interface relationships between the ICD reference system and this sub-system.

Magnet Power Supply (DET-MAG-PSU): Identify the general types of interface relationships between the ICD reference system and this sub-system.

Barrel Particle ID Systems (DET-PID-BAR): Identify the general types of interface relationships between the ICD reference system and this sub-system.

Backward Particle ID Systems (DET-PID-BCK): Identify the general types of interface relationships between the ICD reference system and this sub-system.

Forward Particle ID Systems (DET-PID-FWD): Identify the general types of interface relationships between the ICD reference system and this sub-system.

Tracking Systems (DET-TRAK): Identify the general types of interface relationships between the ICD reference system and this sub-system.

Barrel Tracking Systems (DET-TRAK-BAR): Identify the general types of interface relationships between the ICD reference system and this sub-system.

## Safety and Environmental Considerations

For all materials and components, safety factors will be calculated based on yield strength. As detailed in the infrastructure requirements, parts (excluding bolts) with a safety factor or 3 or greater will generally be accepted without further review, while parts with a safety factor between 1.5 and 3 will require an internal review to determine acceptability.  Any part with a safety factor below 1.5 will be considered unacceptable.

For bolts, a safety factor of 2 or higher may be accepted without further review, while bolts with a safety factor between 1 and 2 must undergo an internal review to determine acceptability. Bolts with a safety factor of 1 or below will be consider unacceptable.

Note, that engineering judgement may be used in unique situations where a needed component goes below the prescribed safety factor.

In all cases, wherever applicable, governing industry standards will guide the design and construction of all infrastructure components. In the absence of a governing standard, the applicable policies of the Department of Energy or Brookhaven National Laboratory will be used.

## Design Considerations

This section contains the current design considerations for the detector infrastructure.

### Detector Envelopes

The following figures illustrate the physical space constraints within the detector.  These diagrams include significantly more detail than is included in the interface definitions and should provide clarity as to how the defined space interfaces were established.

Color Code Legend for Detector Components

Cross-sectional View of Detector Envelopes

End View of Detector Envelopes

Cross-sectional View of Internal Detector Subsystems

## Organization

The interfaces in this document are grouped by sub-system as identified in the sub-system identifier. All interfaces between the Detector Infrastructure Systems and a related sub-system will be included in a single section. Entries within that section will be sorted by the interface identifier or WBS number, as appropriate.

# Interface summary

This tables in this section provide a list of all interfaces that are defined within this document. The interfaces are grouped by sub-system/WBS. Each table has the following fields:

Section: the section number in this document where the interface is addressed.

WBS (1): the WBS number within the ICD reference system, unless the reference system is the owner.

WBS (2): the WBS number for the connected system.

ID: the interface identifier as defined in the Interface Requirement Document (IRD).

Name: the name of the interface as defined in the IRD.

Description: the description of the interface as defined in the IRD.

Owner: identifies the WBS number of the entity that is responsible for ensuring the interface is provided and that it meets validation/verification requirements.

Detector Infrastructure Systems to B-Zero Detectors

| Section | WBS1 | WBS2 | ID | Name | Description | Owner |
| --- | --- | --- | --- | --- | --- | --- |
| 4 |  | 6.03.01.10 |  |  | B-Zero Detectors |  |
| 4.1 | 6.03.01.09 | 6.03.01.10 | I-DET-INF-FWD.005 | B0 Support System | The B0 detector will be supported in the interior of the B0 magnet and will be dependent on the B0 magnet design, and must be adequate to support the weight of the crystals and the tracking detectors. | 6.03.01.09.03 |
| 4.2 | 6.03.01.09 | 6.03.01.10 | I-DET-INF-INT.004 | Space Constraint | B0 detector must fit entirely within the bore of the B0 magnet. | 6.03.01.09.04 |
| 4.3 | 6.03.01.09 | 6.03.01.10 | I-DET-INF-INT.005 | Space Constraint | The position of the forward HCAL is bounded by the required 50cm clearance between the backward HCAL surface and the B0 magnet in the forward direction. Changes to the size or position of either must be coordinated with the other. | 6.03.01.09.04 |
| 4.4 | 6.03.01.09 | 6.03.01.10 | I-DET-INF-INT.006 | Space Constraint | The position of and accessibility of the B0 detector in the backward direction is limited by the cryostat containing the final focusing magnets. | 6.03.01.09.04 |
| 4.5 | 6.03.01.09 | 6.03.01.10 | I-DET-INF-INT.007 | Space Constraint | Access to the B0 detector is constrained by the vacuum valve in front of the B0 magnet. A minimum of 15 centimeters of free space will be required on the IP-side of the B0-dipole for the installation of the B0 detectors. | 6.03.01.09.04 |
| 4.6 | 6.03.01.09 | 6.03.01.10 | I-DET-INF-INT.008 | Space Constraint | The electron and hadron beampipes must be contained within the B0 detector. | 6.03.01.09.04 |
| 4.7 | 6.03.01.09 | 6.03.01.10 | I-DET-INF-INT.041 | Process Cooling | A cooling system will be required to remove heat from the B0-calorimeter and electronics readouts, to maintain an acceptable temperature. A chiller should be installed near the sub-system and a space should be allocated for this chiller. | 6.03.01.09.04 |

Detector Infrastructure Systems to Low Q2 Detectors

| Section | WBS1 | WBS2 | ID | Name | Description | Owner |
| --- | --- | --- | --- | --- | --- | --- |
| 5 |  | 6.03.01.10 |  |  | Low Q2 Detectors |  |
| 5.1 | 6.03.01.09 | 6.03.01.10 | I-DET-INF-BCK.003 | LowQ2 Support | The LowQ2 detector is supported by a freestanding platform that is adjacent to the outgoing electron beam. | 6.03.01.09.02 |
| 5.2 | 6.03.01.09 | 6.03.01.10 | I-DET-INF-INT.009 | Position Constraint | The Low-Q2 detector must be upstream in the electron direction from the Q3ER magnet. | 6.03.01.09.04 |
| 5.3 | 6.03.01.09 | 6.03.01.10 | I-DET-INF-INT.010 | Position Constraint | The Low-Q2 detector must be downstream in the electron direction from the B2ER magnet. | 6.03.01.09.04 |
| 5.4 | 6.03.01.09 | 6.03.01.10 | I-DET-INF-INT.011 | Position Constraint | The Low-Q2 detector must be at the same level as the electron beamline. | 6.03.01.09.04 |
| 5.5 | 6.03.01.09 | 6.03.01.10 | I-DET-INF-INT.042 | Process Cooling | Either a liquid or gas cooling system will be required to remove heat from the Low-Q2 calorimeter, tracking and readout electronics, to maintain them at room temperature. | 6.03.01.09.04 |

Detector Infrastructure Systems to Luminosity Detectors

| Section | WBS1 | WBS2 | ID | Name | Description | Owner |
| --- | --- | --- | --- | --- | --- | --- |
| 6 |  | 6.03.01.10 |  |  | Luminosity Detectors |  |
| 6.1 | 6.03.01.09 | 6.03.01.10 | I-DET-INF-BCK.004 | Lumi Support System | The luminosity detector will be supported by an independent support structure / table. | 6.03.01.09.02 |
| 6.2 | 6.03.01.09 | 6.03.01.10 | I-DET-INF-INT.043 | Dipole Power | The two dipoles in front of the luminosity detectors will require an infrastructure power source. | 6.03.01.09.04 |
| 6.3 | 6.03.01.09 | 6.03.01.10 | I-DET-INF-INT.044 | Dipole Vacuum | The beamline between the two luminosity detector dipoles must be under vacuum. | 6.03.01.09.04 |

Detector Infrastructure Systems to Off-Momentum Detectors

| Section | WBS1 | WBS2 | ID | Name | Description | Owner |
| --- | --- | --- | --- | --- | --- | --- |
| 7 |  | 6.03.01.10 |  |  | Off-Momentum Detectors |  |
| 7.1 | 6.03.01.09 | 6.03.01.10 | I-DET-INF-FWD.001 | Detector Support | The off-momentum detectors will be supported by an integrated support stand that is fully integrated with the vacuum system and allows the position of the detector to be adjusted. | 6.03.01.09.03 |
| 7.2 | 6.03.01.09 | 6.03.01.10 | I-DET-INF-INT.045 | Process Cooling | A cooling system will be required to remove heat from the tracking and readout electronics to prevent heat buildup. | 6.03.01.09.04 |

Detector Infrastructure Systems to Roman Pots

| Section | WBS1 | WBS2 | ID | Name | Description | Owner |
| --- | --- | --- | --- | --- | --- | --- |
| 8 |  | 6.03.01.10 |  |  | Roman Pots |  |
| 8.1 | 6.03.01.09 | 6.03.01.10 | I-DET-INF-FWD.002 | Detector Support | The roman-pot detectors will be supported by an integrated support stand that is fully integrated with the vacuum system and allows the position of the detector to be adjusted. | 6.03.01.09.03 |
| 8.2 | 6.03.01.09 | 6.03.01.10 | I-DET-INF-INT.046 | Process Cooling | A cooling system will be required to remove heat from the tracking and readout electronics to prevent heat buildup. | 6.03.01.09.04 |

Detector Infrastructure Systems to Zero Degree Calorimeter

| Section | WBS1 | WBS2 | ID | Name | Description | Owner |
| --- | --- | --- | --- | --- | --- | --- |
| 9 |  | 6.03.01.10 |  |  | Zero Degree Calorimeter |  |
| 9.1 | 6.03.01.09 | 6.03.01.10 | I-DET-INF-FWD.006 | ZDC Support System | The detector will be supported on a free-standing structure / table | 6.03.01.09.03 |
| 9.2 | 6.03.01.09 | 6.03.01.10 | I-DET-INF-INT.047 | Process Cooling | A cooling system will be required to remove heat from the calorimeter and readout electronics to prevent heat buildup. | 6.03.01.09.04 |

Detector Infrastructure Systems to Online DAQ and Computing Systems

| Section | WBS1 | WBS2 | ID | Name | Description | Owner |
| --- | --- | --- | --- | --- | --- | --- |
| 10 |  | 6.03.01.08.02 |  |  | Online DAQ and Computing Systems |  |
| 10.1 | 6.03.01.09 | 6.03.01.08.02 | I-DET-INF-BAR.013 | Fiber Support System | A fiber support system must be provided that allows the communications cabling to remain connected while the detector is moved from the experimental hall to the assembly area. | 6.03.01.09.01 |
| 10.2 | 6.03.01.09 | 6.03.01.08.02 | I-DET-INF-INT.048 | Computer Room Cooling | HVAC systems must be provided to maintain stable temperature and humidity in the DAQ room. | 6.03.01.09.04 |
| 10.3 | 6.03.01.09 | 6.03.01.08.02 | I-DET-INF-INT.049 | Experimental and Assembly Area Cooling | General HVAC systems must be provided to maintain a normal operating temperature in locations where DAQ systems are in use. | 6.03.01.09.04 |
| 10.4 | 6.03.01.09 | 6.03.01.08.02 | I-DET-INF-INT.050 | Backup Power | UPS systems must be provided to support critical systems in order to tolerate transient conditions and to allow for controlled shutdown. | 6.03.01.09.04 |
| 10.5 | 6.03.01.09 | 6.03.01.08.02 | I-DET-INF-INT.051 | Computer Room Power | Adequate power must be provided to support the DAQ systems and computing infrastructure. | 6.03.01.09.04 |
| 10.6 | 6.03.01.09 | 6.03.01.08.02 | I-DET-INF-INT.052 | Control Room Power | Standard electrical service to support consoles and systems in the control room. | 6.03.01.09.04 |
| 10.7 | 6.03.01.09 | 6.03.01.08.02 | I-DET-INF-INT.053 | Hall Power | Clean power (either transformer or source isolated) should be provided for all DAQ, computing and network equipment. | 6.03.01.09.04 |
| 10.8 | 6.03.01.09 | 6.03.01.08.02 | I-DET-INF-INT.054 | Carriage Rack Space | Adequate rack space for the DAQ system will be required on the detector carriages. | 6.03.01.09.04 |
| 10.9 | 6.03.01.09 | 6.03.01.08.02 | I-DET-INF-INT.055 | Computer Room | Adequate raised-floor space with racks will be required to support DAQ computers, communications systems, and computing infrastructure. | 6.03.01.09.04 |
| 10.10 | 6.03.01.09 | 6.03.01.08.02 | I-DET-INF-INT.056 | Control Systems Furniture | Space for computing consoles and furniture adequate to manage the system during operation will be required. Consoles and furniture will be provided by DET-INT. | 6.03.01.09.04 |
| 10.11 | 6.03.01.09 | 6.03.01.08.02 | I-DET-INF-INT.057 | Detector Plenums | Adequate space to route fiberoptic cables from the racks to the detector readout boards will be required. | 6.03.01.09.04 |
| 10.12 | 6.03.01.09 | 6.03.01.08.02 | I-DET-INF-INT.058 | Fixed Platform Conduits | Adequate conduits and cableways must be provided to deliver cabling to the racks on the fixed platforms. | 6.03.01.09.04 |
| 10.13 | 6.03.01.09 | 6.03.01.08.02 | I-DET-INF-INT.059 | Platform/Tunnel Rack Space | Adequate rack space for the DAQ system must be provided on the fixed platforms, tunnels, and in the experimental hall. | 6.03.01.09.04 |
| 10.14 | 6.03.01.09 | 6.03.01.08.02 | I-DET-INF-INT.060 | Sensitive Electronics | DAQ systems that are sensitive to magnetism or radiation must be placed at a sufficient distance from sources OR shielding must be provided to protect the equipment. | 6.03.01.09.04 |
| 10.15 | 6.03.01.09 | 6.03.01.08.02 | I-DET-INF-INT.061 | Tunnel Conduits | Conduits must be provided in the west and east tunnels to connect the detectors to the DAQ system. | 6.03.01.09.04 |
| 10.16 | 6.03.01.09 | 6.03.01.08.02 | I-DET-INF-INT.062 | Fiber Support System | A fiber support system must be provided that allows the communications cabling to remain connected while the detector is moved from the experimental hall to the assembly area. | 6.03.01.09.04 |

Detector Infrastructure Systems to

| Section | WBS1 | WBS2 | ID | Name | Description | Owner |
| --- | --- | --- | --- | --- | --- | --- |
| 11 |  |  |  |  |  |  |
| 11.1 | 6.03.01.09 |  | I-DET-INF-INT.001 | Magnetic Material | No detectors or components constructed with magnetic material may be added to the detector without prior consultation with the magnet group. | 6.03.01.09.04 |
| 11.2 | 6.03.01.09 |  | I-DET-INF-INT.002 | Magnetic Field Requirement | Detector performance is dependent on the continuous operation of the solenoid and on the delivery of a consistent, stable magnetic field. | 6.03.01.09.04 |
| 11.3 | 6.03.01.09 |  | I-DET-INF-INT.003 | Cabling/Services Conduit | The global support tube will provide integrated pathways for cabling and services to be delivered from the interior detectors (pf RICH, inner MPGD, barrel ToF and silicon trackers) to the exterior infrastructure. | 6.03.01.09.04 |

Detector Infrastructure Systems to Barrel EMCal Systems

| Section | WBS1 | WBS2 | ID | Name | Description | Owner |
| --- | --- | --- | --- | --- | --- | --- |
| 12 |  | 6.03.01.04 |  |  | Barrel EMCal Systems |  |
| 12.1 | 6.03.01.09 | 6.03.01.04 | I-DET-INF-BAR.001 | Barrel ECAL Support | The barrel ECAL will be supported by a structural support system that runs across the end of the barrel along the surface at 3 and 9 o'clock to the barrel Hadron Calorimeter. | 6.03.01.09.01 |
| 12.2 | 6.03.01.09 | 6.03.01.04 | I-DET-INF-BAR.002 | Weight Distributed | The weight of the barrel ECAL will be transferred to the barrel Hadron Calorimeter and must be accommodated by the dedicated support systems. | 6.03.01.09.01 |
| 12.3 | 6.03.01.09 | 6.03.01.04 | I-DET-INF-BAR.003 | Support Structure | The DIRC bar boxes will be supported by the global support tube inside the barrel Electromagnetic Calorimeter, that allows the boxes to be extracted using a system of rollers. | 6.03.01.09.01 |
| 12.4 | 6.03.01.09 | 6.03.01.04 | I-DET-INF-INT.012 | Space Constraint | The exterior radius of the barrel ECAL (and its support system) is limited by the interior bore of the solenoid magnet. Modifications to either must be coordinated. | 6.03.01.09.04 |
| 12.5 | 6.03.01.09 | 6.03.01.04 | I-DET-INF-INT.013 | Space Constraint | The backward position and shape of the barrel EMCAL is limited by the size and shape of the DIRC readout supports. Changes to the size or position of either must be coordinated with the other. | 6.03.01.09.04 |
| 12.6 | 6.03.01.09 | 6.03.01.04 | I-DET-INF-INT.014 | Space Constraint | The maximum size of the DIRC is limited to the interior bore of the barrel ECAL (and its support structures). Modifications to either must be coordinated. | 6.03.01.09.04 |
| 12.7 | 6.03.01.09 | 6.03.01.04 | I-DET-INF-INT.015 | Space Constraint | The forward position and shape of the barrel ECAL is limited by the backward face of the Dual RICH detector and the adjacent cabling pathway that provides services to the interior detectors. Modifications to either must be coordinated. | 6.03.01.09.04 |
| 12.8 | 6.03.01.09 | 6.03.01.04 | I-DET-INF-INT.063 | Imaging Layers Heat Rejection | The AstroPix imaging layers must be cooled with an independent system (TBD). | 6.03.01.09.04 |
| 12.9 | 6.03.01.09 | 6.03.01.04 | I-DET-INF-INT.064 | Silicon PM Heat Rejection | The barrel ECAL readout electronics will require chilled water or LCW cooling to maintain a safe operating temperature. | 6.03.01.09.04 |

Detector Infrastructure Systems to Backward EMCal Systems

| Section | WBS1 | WBS2 | ID | Name | Description | Owner |
| --- | --- | --- | --- | --- | --- | --- |
| 13 |  | 6.03.01.04 |  |  | Backward EMCal Systems |  |
| 13.1 | 6.03.01.09 | 6.03.01.04 | I-DET-INF-BAR.004 | Backward ECAL Support | The backward ECAL will be supported by rails integrated into the global support tube | 6.03.01.09.01 |
| 13.2 | 6.03.01.09 | 6.03.01.04 | I-DET-INF-BAR.005 | Weight Transfer | The weight of the backward ECAL will be transferred to the global support tube and must be accommodated by all subsequent support systems. | 6.03.01.09.01 |
| 13.3 | 6.03.01.09 | 6.03.01.04 | I-DET-INF-BAR.006 | EMCal Support System | The global support tube will provide support for the backward electromagnetic calorimeter. | 6.03.01.09.01 |
| 13.4 | 6.03.01.09 | 6.03.01.04 | I-DET-INF-INT.016 | Space Constraint | The backward position of the backward ECAL is limited by the backward Hadron Calorimeter and the adjacent cabling pathway that provides services to the interior detectors. Modifications to either must be coordinated. | 6.03.01.09.04 |
| 13.5 | 6.03.01.09 | 6.03.01.04 | I-DET-INF-INT.017 | Space Constraint | The maximum backward location for the DIRC is limited by the position of the backward endcap support structure and the adjacent cabling pathway that provides services to the interior detectors. Modifications to either must be coordinated. | 6.03.01.09.04 |
| 13.6 | 6.03.01.09 | 6.03.01.04 | I-DET-INF-INT.018 | Space Constraint | The exterior radius of the backward ECAL is limited by the interior bore of the global support tube and the space needed for all the services needed by the interior detectors. Modifications to either must be coordinated. | 6.03.01.09.04 |
| 13.7 | 6.03.01.09 | 6.03.01.04 | I-DET-INF-INT.019 | Heat Mitigation | The backward ECAL must mitigated the heat generated by the backward RICH detector (pfRICH) | 6.03.01.09.04 |
| 13.8 | 6.03.01.09 | 6.03.01.04 | I-DET-INF-INT.020 | Space Constraint | The position of the backward ECAL in the forward direction is limited by the backward face of the pfRICH detector. Modifications to either must be coordinated. | 6.03.01.09.04 |
| 13.9 | 6.03.01.09 | 6.03.01.04 | I-DET-INF-INT.021 | Backward ECAL Bore Clearance | The bore of the backward ECAL must be designed to allow it to be inserted/removed over the existing beamline flanges. | 6.03.01.09.04 |
| 13.10 | 6.03.01.09 | 6.03.01.04 | I-DET-INF-INT.022 | Space Constraint | The interior radius of the backward ECAL is governed by the size of the beamline. | 6.03.01.09.04 |
| 13.11 | 6.03.01.09 | 6.03.01.04 | I-DET-INF-INT.065 | Heat Rejection | The backward ECAL will require chilled water or LCW cooling to maintain the temperature of the silicon photomultipliers, the electronics, and the crystals. | 6.03.01.09.04 |

Detector Infrastructure Systems to Forward EMCal Systems

| Section | WBS1 | WBS2 | ID | Name | Description | Owner |
| --- | --- | --- | --- | --- | --- | --- |
| 14 |  | 6.03.01.04 |  |  | Forward EMCal Systems |  |
| 14.1 | 6.03.01.09 | 6.03.01.04 | I-DET-INF-BAR.007 | Forward ECAL Support | The forward ECAL will be supported by the forward Hadron Calorimeter endcap. Because the forward ECAL must split into two parts to provide access to the barrel, each half must be independently affixed to the Hadron Calorimeter halves. | 6.03.01.09.01 |
| 14.2 | 6.03.01.09 | 6.03.01.04 | I-DET-INF-BAR.008 | Weight Transfer | The weight of the forward ECAL will be transferred to the forward Hadron Calorimeter endcap and must be accommodated by all subsequent support systems. | 6.03.01.09.01 |
| 14.3 | 6.03.01.09 | 6.03.01.04 | I-DET-INF-FWD.004 | Sub-Detector Support | The forward HCAL will support the weight of the forward electromagnetic calorimeter that is embedded within it. | 6.03.01.09.03 |
| 14.4 | 6.03.01.09 | 6.03.01.04 | I-DET-INF-FWD.007 | Forward ECAL Mobility Support | Each half of the forward ECAL must be continuously supported and stabilized while it is moved between the opened and closed positions. | 6.03.01.09.03 |
| 14.5 | 6.03.01.09 | 6.03.01.04 | I-DET-INF-INT.023 | Space Constraint | The exterior radius of the forward ECAL is limited by the interior bore of the barrel Hadron Calorimeter. Modifications to either must be coordinated. | 6.03.01.09.04 |
| 14.6 | 6.03.01.09 | 6.03.01.04 | I-DET-INF-INT.024 | Kinematic Overlap | The exterior radius of the forward ECAL is constrained to provide continuous coverage with the barrel electromagnetic calorimeter. Modifications to either must be coordinated. | 6.03.01.09.04 |
| 14.7 | 6.03.01.09 | 6.03.01.04 | I-DET-INF-INT.025 | Space Constraint | The forward position of the forward ECAL is limited by the forward Hadron Calorimeter. Modifications to either must be coordinated. | 6.03.01.09.04 |
| 14.8 | 6.03.01.09 | 6.03.01.04 | I-DET-INF-INT.026 | Space Constraint | The maximum forward location for the dRICH is limited by the forward ECAL and the adjacent cabling pathway that provides services to the interior detectors. Modifications to either must be coordinated. | 6.03.01.09.04 |
| 14.9 | 6.03.01.09 | 6.03.01.04 | I-DET-INF-INT.066 | Heat Rejection | The forward ECAL readout electronics will require chilled water or LCW cooling to maintain a safe operating temperature. | 6.03.01.09.04 |

Detector Infrastructure Systems to Electronic Systems

| Section | WBS1 | WBS2 | ID | Name | Description | Owner |
| --- | --- | --- | --- | --- | --- | --- |
| 15 |  | 6.03.01.07 |  |  | Electronic Systems |  |
| 15.1 | 6.03.01.09 | 6.03.01.07 | I-DET-INF-INT.067 | Clean A/C Power | Electronics components on the carriage will require clean or transformer isolated power. | 6.03.01.09.04 |
| 15.2 | 6.03.01.09 | 6.03.01.07 | I-DET-INF-INT.068 | UPS Power | Critical systems, such as controls, will require sufficient UPS power to allow them to be shutdown graceful, or to tolerate transients. | 6.03.01.09.04 |
| 15.3 | 6.03.01.09 | 6.03.01.07 | I-DET-INF-INT.069 | Computing Environment | The electronics and computing systems will require a stable environment in terms of temperature and humidity. | 6.03.01.09.04 |
| 15.4 | 6.03.01.09 | 6.03.01.07 | I-DET-INF-INT.070 | EMI Shielding | The electronics and computing systems on the carriage may require some EMI shielding from detectors and other components. | 6.03.01.09.04 |
| 15.5 | 6.03.01.09 | 6.03.01.07 | I-DET-INF-INT.071 | Cabling Pathways | Adequate space for cables and cable trays will be required to deliver low voltage and high voltage, fiber and signal cables from the carriage/platform to the sub-detectors. | 6.03.01.09.04 |
| 15.6 | 6.03.01.09 | 6.03.01.07 | I-DET-INF-INT.072 | Platform Rack Space | Adequate space must be provided for computing enclosures (racks) on the south platform to support the electronics systems. | 6.03.01.09.04 |

Detector Infrastructure Systems to Barrel HCal Systems

| Section | WBS1 | WBS2 | ID | Name | Description | Owner |
| --- | --- | --- | --- | --- | --- | --- |
| 16 |  | 6.03.01.05 |  |  | Barrel HCal Systems |  |
| 16.1 | 6.03.01.09 | 6.03.01.05 | I-DET-INF-BAR.001 | Barrel ECAL Support | The barrel ECAL will be supported by a structural support system that runs across the end of the barrel along the surface at 3 and 9 o'clock to the barrel Hadron Calorimeter. | 6.03.01.09.01 |
| 16.2 | 6.03.01.09 | 6.03.01.05 | I-DET-INF-BAR.009 | Barrel HCAL Support | The barrel HCAL will be supported by a rolling carriage that allows the central detector to be moved between the assembly area and the experimental hall. | 6.03.01.09.01 |
| 16.3 | 6.03.01.09 | 6.03.01.05 | I-DET-INF-BAR.010 | Sub-Detector Support | The barrel HCAL will support the weight of all of the sub-detectors and components within the central detector. | 6.03.01.09.01 |
| 16.4 | 6.03.01.09 | 6.03.01.05 | I-DET-INF-BAR.011 | Weight Transfer | The cumulative weight of the HCAL and all of the sub-detectors it supports will be transferred to the detector carriage. | 6.03.01.09.01 |
| 16.5 | 6.03.01.09 | 6.03.01.05 | I-DET-INF-BAR.012 | Dual RICH Support System | The Dual RICH will be supported by a structural system within the barrel hadron calorimeter. | 6.03.01.09.01 |
| 16.6 | 6.03.01.09 | 6.03.01.05 | I-DET-INF-INT.023 | Space Constraint | The exterior radius of the forward ECAL is limited by the interior bore of the barrel Hadron Calorimeter. Modifications to either must be coordinated. | 6.03.01.09.04 |
| 16.7 | 6.03.01.09 | 6.03.01.05 | I-DET-INF-INT.024 | Kinematic Overlap | The exterior radius of the forward ECAL is constrained to provide continuous coverage with the barrel electromagnetic calorimeter. Modifications to either must be coordinated. | 6.03.01.09.04 |
| 16.8 | 6.03.01.09 | 6.03.01.05 | I-DET-INF-INT.028 | Space Constraint | The forward position of the backward HCAL is limited by the barrel HCAL and the adjacent cabling pathway that provides services to the interior detectors. Modifications to either must be coordinated. | 6.03.01.09.04 |
| 16.9 | 6.03.01.09 | 6.03.01.05 | I-DET-INF-INT.029 | Space Constraint | The exterior radius of the backward HCAL should be consistent with the radius of the barrel HCAL. | 6.03.01.09.04 |
| 16.10 | 6.03.01.09 | 6.03.01.05 | I-DET-INF-INT.030 | Space Constraint | The exterior radius of the forward HCAL should be consistent with the radius of the barrel HCAL. | 6.03.01.09.04 |
| 16.11 | 6.03.01.09 | 6.03.01.05 | I-DET-INF-INT.031 | Space Constraint | The backward position of the forward HCAL is limited by the barrel HCAL and the adjacent cabling pathway that provides services to the interior detectors. Modifications to either must be coordinated. | 6.03.01.09.04 |
| 16.12 | 6.03.01.09 | 6.03.01.05 | I-DET-INF-INT.032 | External Space Constraint | The exterior radius of the solenoid magnet is limited by the interior bore of the barrel HCAL and necessary support structures. Modifications to either must be coordinated. | 6.03.01.09.04 |
| 16.13 | 6.03.01.09 | 6.03.01.05 | I-DET-INF-INT.033 | Space Constraint | The size of the dRICH is limited by the interior bore of the barrel HCAL, and must provide adequate space for cables and services to itself and the interior detectors. Modifications to either must be coordinated. | 6.03.01.09.04 |
| 16.14 | 6.03.01.09 | 6.03.01.05 | I-DET-INF-INT.034 | Space Constraint | The forward position of the backward HCAL is limited by the barrel HCAL and the adjacent cabling pathway that provides services to the interior detectors. Modifications to either must be coordinated. | 6.03.01.09.04 |
| 16.15 | 6.03.01.09 | 6.03.01.05 | I-DET-INF-INT.073 | Heat Rejection | Heat from the forward HCAL silicon photomultipliers and front-end electronics will be released into the outside room via forced air. | 6.03.01.09.04 |
| 16.16 | 6.03.01.09 | 6.03.01.05 | I-DET-INF-INT.074 | Space Constraint | The exterior radius of the barrel HCAL is limited by the detector carriage, the electronics platforms, and other Hall infrastructure. | 6.03.01.09.04 |

Detector Infrastructure Systems to Backward HCal Systems

| Section | WBS1 | WBS2 | ID | Name | Description | Owner |
| --- | --- | --- | --- | --- | --- | --- |
| 17 |  | 6.03.01.05 |  |  | Backward HCal Systems |  |
| 17.1 | 6.03.01.09 | 6.03.01.05 | I-DET-INF-BCK.001 | Backward HCAL Mobility | Adequate clearance must be provided for either of the endcap detector carriages to be rolled aside to allow access to sub-detectors within the barrel. | 6.03.01.09.02 |
| 17.2 | 6.03.01.09 | 6.03.01.05 | I-DET-INF-BCK.002 | Backward HCAL Support | The backward HCAL consists of two halves, each of which are supported by an independent carriage. | 6.03.01.09.02 |
| 17.3 | 6.03.01.09 | 6.03.01.05 | I-DET-INF-INT.016 | Space Constraint | The backward position of the backward ECAL is limited by the backward Hadron Calorimeter and the adjacent cabling pathway that provides services to the interior detectors. Modifications to either must be coordinated. | 6.03.01.09.04 |
| 17.4 | 6.03.01.09 | 6.03.01.05 | I-DET-INF-INT.028 | Space Constraint | The forward position of the backward HCAL is limited by the barrel HCAL and the adjacent cabling pathway that provides services to the interior detectors. Modifications to either must be coordinated. | 6.03.01.09.04 |
| 17.5 | 6.03.01.09 | 6.03.01.05 | I-DET-INF-INT.029 | Space Constraint | The exterior radius of the backward HCAL should be consistent with the radius of the barrel HCAL. | 6.03.01.09.04 |
| 17.6 | 6.03.01.09 | 6.03.01.05 | I-DET-INF-INT.034 | Space Constraint | The forward position of the backward HCAL is limited by the barrel HCAL and the adjacent cabling pathway that provides services to the interior detectors. Modifications to either must be coordinated. | 6.03.01.09.04 |
| 17.7 | 6.03.01.09 | 6.03.01.05 | I-DET-INF-INT.035 | Space Constraint | The maximum backward location for the solenoid magnet is limited by the backward HCAL and the adjacent cabling pathway that provides services to the interior detectors. Modifications to either must be coordinated. | 6.03.01.09.04 |
| 17.8 | 6.03.01.09 | 6.03.01.05 | I-DET-INF-INT.036 | Space Constraint | The maximum backward location for the DIRC is limited by the backward endcap support structure and the adjacent cabling pathway that provides services to the interior detectors. Modifications to either must be coordinated. | 6.03.01.09.04 |
| 17.9 | 6.03.01.09 | 6.03.01.05 | I-DET-INF-INT.037 | Space Constraint | The position of the backward HCAL is limited in the backward direction by the adjacent accelerator magnets and the 50 cm clearance between the backward HCal surface and the backward IR magnets | 6.03.01.09.04 |
| 17.10 | 6.03.01.09 | 6.03.01.05 | I-DET-INF-INT.038 | Space Constraint | The interior radius of the backward HCAL is governed by the size of the beamline. | 6.03.01.09.04 |
| 17.11 | 6.03.01.09 | 6.03.01.05 | I-DET-INF-INT.075 | Heat Rejection | Heat from the silicon photomultipliers and front-end electronics will be released into the outside room via forced air. | 6.03.01.09.04 |

Detector Infrastructure Systems to Forward HCal Systems

| Section | WBS1 | WBS2 | ID | Name | Description | Owner |
| --- | --- | --- | --- | --- | --- | --- |
| 18 |  | 6.03.01.05 |  |  | Forward HCal Systems |  |
| 18.1 | 6.03.01.09 | 6.03.01.05 | I-DET-INF-BAR.002 | Weight Distributed | The weight of the barrel ECAL will be transferred to the barrel Hadron Calorimeter and must be accommodated by the dedicated support systems. | 6.03.01.09.01 |
| 18.2 | 6.03.01.09 | 6.03.01.05 | I-DET-INF-BAR.004 | Backward ECAL Support | The backward ECAL will be supported by rails integrated into the global support tube | 6.03.01.09.01 |
| 18.3 | 6.03.01.09 | 6.03.01.05 | I-DET-INF-BAR.007 | Forward ECAL Support | The forward ECAL will be supported by the forward Hadron Calorimeter endcap. Because the forward ECAL must split into two parts to provide access to the barrel, each half must be independently affixed to the Hadron Calorimeter halves. | 6.03.01.09.01 |
| 18.4 | 6.03.01.09 | 6.03.01.05 | I-DET-INF-BAR.008 | Weight Transfer | The weight of the forward ECAL will be transferred to the forward Hadron Calorimeter endcap and must be accommodated by all subsequent support systems. | 6.03.01.09.01 |
| 18.5 | 6.03.01.09 | 6.03.01.05 | I-DET-INF-BAR.010 | Sub-Detector Support | The barrel HCAL will support the weight of all of the sub-detectors and components within the central detector. | 6.03.01.09.01 |
| 18.6 | 6.03.01.09 | 6.03.01.05 | I-DET-INF-BAR.011 | Weight Transfer | The cumulative weight of the HCAL and all of the sub-detectors it supports will be transferred to the detector carriage. | 6.03.01.09.01 |
| 18.7 | 6.03.01.09 | 6.03.01.05 | I-DET-INF-BCK.001 | Backward HCAL Mobility | Adequate clearance must be provided for either of the endcap detector carriages to be rolled aside to allow access to sub-detectors within the barrel. | 6.03.01.09.02 |
| 18.8 | 6.03.01.09 | 6.03.01.05 | I-DET-INF-BCK.002 | Backward HCAL Support | The backward HCAL consists of two halves, each of which are supported by an independent carriage. | 6.03.01.09.02 |
| 18.9 | 6.03.01.09 | 6.03.01.05 | I-DET-INF-FWD.003 | Forward HCAL Support | The forward HCAL consists of two halves, each of which are support by an independent carriage. | 6.03.01.09.03 |
| 18.10 | 6.03.01.09 | 6.03.01.05 | I-DET-INF-FWD.004 | Sub-Detector Support | The forward HCAL will support the weight of the forward electromagnetic calorimeter that is embedded within it. | 6.03.01.09.03 |
| 18.11 | 6.03.01.09 | 6.03.01.05 | I-DET-INF-FWD.008 | Forward HCAL Mobility | Adequate clearance must be provided for either of the detector carriages to be rolled aside to allow access to sub-detectors within the barrel. | 6.03.01.09.03 |
| 18.12 | 6.03.01.09 | 6.03.01.05 | I-DET-INF-FWD.009 | Forward HCAL Mobility | Adequate clearance must be provided for either of the detector carriages to be rolled aside to allow access to sub-detectors within the barrel. | 6.03.01.09.03 |
| 18.13 | 6.03.01.09 | 6.03.01.05 | I-DET-INF-INT.005 | Space Constraint | The position of the forward HCAL is bounded by the required 50cm clearance between the backward HCAL surface and the B0 magnet in the forward direction. Changes to the size or position of either must be coordinated with the other. | 6.03.01.09.04 |
| 18.14 | 6.03.01.09 | 6.03.01.05 | I-DET-INF-INT.025 | Space Constraint | The forward position of the forward ECAL is limited by the forward Hadron Calorimeter. Modifications to either must be coordinated. | 6.03.01.09.04 |
| 18.15 | 6.03.01.09 | 6.03.01.05 | I-DET-INF-INT.030 | Space Constraint | The exterior radius of the forward HCAL should be consistent with the radius of the barrel HCAL. | 6.03.01.09.04 |
| 18.16 | 6.03.01.09 | 6.03.01.05 | I-DET-INF-INT.031 | Space Constraint | The backward position of the forward HCAL is limited by the barrel HCAL and the adjacent cabling pathway that provides services to the interior detectors. Modifications to either must be coordinated. | 6.03.01.09.04 |
| 18.17 | 6.03.01.09 | 6.03.01.05 | I-DET-INF-INT.039 | Space Constraint | The interior radius of the forward HCAL is governed by the size of the beamline. | 6.03.01.09.04 |
| 18.18 | 6.03.01.09 | 6.03.01.05 | I-DET-INF-INT.076 | Heat Rejection | Heat from the silicon photomultipliers and front-end electronics will be released into the outside room via forced air. DO WE NEED CW FOR COOLING HERE? | 6.03.01.09.04 |

Detector Infrastructure Systems to Solenoid Magnet

| Section | WBS1 | WBS2 | ID | Name | Description | Owner |
| --- | --- | --- | --- | --- | --- | --- |
| 19 |  | 6.03.01.06 |  |  | Solenoid Magnet |  |
| 19.1 | 6.03.01.09 | 6.03.01.06 | I-DET-INF-BAR.014 | External Support Structure | Total weight of the solenoid and all embedded components must be supported. | 6.03.01.09.01 |
| 19.2 | 6.03.01.09 | 6.03.01.06 | I-DET-INF-BAR.016 | DIRC Readout Supports | The DIRC prism box containing the photosensor readouts, will be supported by an external structure supporting through the global support tube | 6.03.01.09.01 |
| 19.3 | 6.03.01.09 | 6.03.01.06 | I-DET-INF-INT.001 | Magnetic Material | No detectors or components constructed with magnetic material may be added to the detector without prior consultation with the magnet group. | 6.03.01.09.04 |
| 19.4 | 6.03.01.09 | 6.03.01.06 | I-DET-INF-INT.002 | Magnetic Field Requirement | Detector performance is dependent on the continuous operation of the solenoid and on the delivery of a consistent, stable magnetic field. | 6.03.01.09.04 |
| 19.5 | 6.03.01.09 | 6.03.01.06 | I-DET-INF-INT.012 | Space Constraint | The exterior radius of the barrel ECAL (and its support system) is limited by the interior bore of the solenoid magnet. Modifications to either must be coordinated. | 6.03.01.09.04 |
| 19.6 | 6.03.01.09 | 6.03.01.06 | I-DET-INF-INT.032 | External Space Constraint | The exterior radius of the solenoid magnet is limited by the interior bore of the barrel HCAL and necessary support structures. Modifications to either must be coordinated. | 6.03.01.09.04 |
| 19.7 | 6.03.01.09 | 6.03.01.06 | I-DET-INF-INT.035 | Space Constraint | The maximum backward location for the solenoid magnet is limited by the backward HCAL and the adjacent cabling pathway that provides services to the interior detectors. Modifications to either must be coordinated. | 6.03.01.09.04 |
| 19.8 | 6.03.01.09 | 6.03.01.06 | I-DET-INF-INT.040 | Magnet Cooling | The solenoid will require a continuous flow of cryogens to maintain temperature in both the experimental hall and the maintenance area. Interface includes cryogenic source and warm return. | 6.03.01.09.04 |
| 19.9 | 6.03.01.09 | 6.03.01.06 | I-DET-INF-INT.077 | Power Supply Cooling | The power supply, located in an adjacent electrical room, will need to be cooled with low conductivity water. | 6.03.01.09.04 |
| 19.10 | 6.03.01.09 | 6.03.01.06 | I-DET-INF-INT.078 | Magnet Cooling | The solenoid will require a continuous flow of cryogens to maintain temperature in both the experimental hall and the maintenance area. Interface includes cryogenic source and warm return. | 6.03.01.09.04 |
| 19.11 | 6.03.01.09 | 6.03.01.06 | I-DET-INF-INT.079 | Electrical Power | Solenoid will be provided power via the power supply located in an adjacent room. | 6.03.01.09.04 |
| 19.12 | 6.03.01.09 | 6.03.01.06 | I-DET-INF-INT.080 | Instrumentation Backup Power | The instrumentation rack for the solenoid will require backup/UPS power adequate to safely shutdown the system in the event of a power failure. | 6.03.01.09.04 |
| 19.13 | 6.03.01.09 | 6.03.01.06 | I-DET-INF-INT.081 | Fringe Field | A boundary must be established to identify the extent of the magnet's stray field during operations. | 6.03.01.09.04 |
| 19.14 | 6.03.01.09 | 6.03.01.06 | I-DET-INF-INT.082 | Ice Management | An ice management system will be required to melt accumulated ice on the chimney. (Water should evaporate and will not require drainage) | 6.03.01.09.04 |
| 19.15 | 6.03.01.09 | 6.03.01.06 | I-DET-INF-INT.083 | Cryogenic Connection Point | The cryogenic connection must be as close as possible to the source. | 6.03.01.09.04 |
| 19.16 | 6.03.01.09 | 6.03.01.06 | I-DET-INF-INT.084 | LCW Connection | The LCW water cooled leads must be fully supported along their path between the electrical room and the magnet. | 6.03.01.09.04 |
| 19.17 | 6.03.01.09 | 6.03.01.06 | I-DET-INF-INT.097 | Space Constraint | The maximum forward location for the solenoid is limited by the position of the Dual RICH detector and the cabling plenum (gap) between them. Modifications to either must be coordinated. | 6.03.01.09.04 |
| 19.18 | 6.03.01.09 | 6.03.01.06 | I-DET-INF-INT.115 | LHE Flow (Hall) | A supply and return of liquid helium must be provided to the experimental hall for cooling the solenoid magnet during operation. | 6.03.01.09.04 |
| 19.19 | 6.03.01.09 | 6.03.01.06 | I-DET-INF-INT.116 | LHE Flow (Assembly Area) | A supply and return of liquid helium must be provided to the assembly hall for cooling the solenoid magnet during maintenance. | 6.03.01.09.04 |

Detector Infrastructure Systems to Magnet Instrumentation and Control

| Section | WBS1 | WBS2 | ID | Name | Description | Owner |
| --- | --- | --- | --- | --- | --- | --- |
| 20 |  | 6.03.01.06 |  |  | Magnet Instrumentation and Control |  |
| 20.1 | 6.03.01.09 | 6.03.01.06 | I-DET-INF-INT.141 | Power | Low voltage DC power will be provided from the electronics racks on the south platform | 6.03.01.09.04 |

Detector Infrastructure Systems to Magnet Power Supply

| Section | WBS1 | WBS2 | ID | Name | Description | Owner |
| --- | --- | --- | --- | --- | --- | --- |
| 21 |  | 6.03.01.06 |  |  | Magnet Power Supply |  |
| 21.1 | 6.03.01.09 | 6.03.01.06 | I-DET-INF-INT.142 | Power | High voltage and low voltage DC power will be provided from the power supply room. | 6.03.01.09.04 |

Detector Infrastructure Systems to Barrel Particle ID Systems

| Section | WBS1 | WBS2 | ID | Name | Description | Owner |
| --- | --- | --- | --- | --- | --- | --- |
| 22 |  | 6.03.01.03 |  |  | Barrel Particle ID Systems |  |
| 22.1 | 6.03.01.09 | 6.03.01.03 | I-DET-INF-BAR.003 | Support Structure | The DIRC bar boxes will be supported by the global support tube inside the barrel Electromagnetic Calorimeter, that allows the boxes to be extracted using a system of rollers. | 6.03.01.09.01 |
| 22.2 | 6.03.01.09 | 6.03.01.03 | I-DET-INF-BAR.005 | Weight Transfer | The weight of the backward ECAL will be transferred to the global support tube and must be accommodated by all subsequent support systems. | 6.03.01.09.01 |
| 22.3 | 6.03.01.09 | 6.03.01.03 | I-DET-INF-BAR.006 | EMCal Support System | The global support tube will provide support for the backward electromagnetic calorimeter. | 6.03.01.09.01 |
| 22.4 | 6.03.01.09 | 6.03.01.03 | I-DET-INF-BAR.016 | DIRC Readout Supports | The DIRC prism box containing the photosensor readouts, will be supported by an external structure supporting through the global support tube | 6.03.01.09.01 |
| 22.5 | 6.03.01.09 | 6.03.01.03 | I-DET-INF-BAR.018 | Barrel TOF Support | The global support tube will provide support for the barrel time-of-flight system. | 6.03.01.09.01 |
| 22.6 | 6.03.01.09 | 6.03.01.03 | I-DET-INF-BAR.019 | Barrel DIRC Support | The global support tube will provide support for the DIRC system. | 6.03.01.09.01 |
| 22.7 | 6.03.01.09 | 6.03.01.03 | I-DET-INF-INT.003 | Cabling/Services Conduit | The global support tube will provide integrated pathways for cabling and services to be delivered from the interior detectors (pf RICH, inner MPGD, barrel ToF and silicon trackers) to the exterior infrastructure. | 6.03.01.09.04 |
| 22.8 | 6.03.01.09 | 6.03.01.03 | I-DET-INF-INT.013 | Space Constraint | The backward position and shape of the barrel EMCAL is limited by the size and shape of the DIRC readout supports. Changes to the size or position of either must be coordinated with the other. | 6.03.01.09.04 |
| 22.9 | 6.03.01.09 | 6.03.01.03 | I-DET-INF-INT.014 | Space Constraint | The maximum size of the DIRC is limited to the interior bore of the barrel ECAL (and its support structures). Modifications to either must be coordinated. | 6.03.01.09.04 |
| 22.10 | 6.03.01.09 | 6.03.01.03 | I-DET-INF-INT.017 | Space Constraint | The maximum backward location for the DIRC is limited by the position of the backward endcap support structure and the adjacent cabling pathway that provides services to the interior detectors. Modifications to either must be coordinated. | 6.03.01.09.04 |
| 22.11 | 6.03.01.09 | 6.03.01.03 | I-DET-INF-INT.018 | Space Constraint | The exterior radius of the backward ECAL is limited by the interior bore of the global support tube and the space needed for all the services needed by the interior detectors. Modifications to either must be coordinated. | 6.03.01.09.04 |
| 22.12 | 6.03.01.09 | 6.03.01.03 | I-DET-INF-INT.036 | Space Constraint | The maximum backward location for the DIRC is limited by the backward endcap support structure and the adjacent cabling pathway that provides services to the interior detectors. Modifications to either must be coordinated. | 6.03.01.09.04 |
| 22.13 | 6.03.01.09 | 6.03.01.03 | I-DET-INF-INT.085 | Process Cooling | The heat generated by the photo-multiplier tubes and associated electronics must be removed using a process cooling system. | 6.03.01.09.04 |
| 22.14 | 6.03.01.09 | 6.03.01.03 | I-DET-INF-INT.086 | Process Cooling | The heat generated by the detector and its electronics must be removed using a process cooling system. | 6.03.01.09.04 |
| 22.15 | 6.03.01.09 | 6.03.01.03 | I-DET-INF-INT.098 | Space Constraint | The exterior radius of the pfRICH is limited by the interior bore of the global support tube and the space needed for services for the interior detectors. Modifications to either must be coordinated. | 6.03.01.09.04 |
| 22.16 | 6.03.01.09 | 6.03.01.03 | I-DET-INF-INT.099 | Space Constraint | The exterior radius of the pfRICH is limited by the interior bore of the global support tube and the space needed for services for the interior detectors. Modifications to either must be coordinated. | 6.03.01.09.04 |
| 22.17 | 6.03.01.09 | 6.03.01.03 | I-DET-INF-INT.100 | Space Constraint | The maximum forward location for the DIRC is limited by the position of the dRICH detector and the cabling plenum between them. Modifications to either must be coordinated. | 6.03.01.09.04 |
| 22.18 | 6.03.01.09 | 6.03.01.03 | I-DET-INF-INT.101 | Service Pathway | Conduits must be provided within the global support tube that are adequate to deliver services (power, signal, cooling) to the barrel tracking detectors. | 6.03.01.09.04 |
| 22.19 | 6.03.01.09 | 6.03.01.03 | I-DET-INF-INT.104 | Space Constraint | The maximum forward location for the DIRC is limited by the service gap in front of the forward RICH detector. | 6.03.01.09.04 |
| 22.20 | 6.03.01.09 | 6.03.01.03 | I-DET-INF-INT.105 | Space Constraint | The maximum backward location for the barrel Time of Flight detector is limited by the position of the pfRICH detector and the adjacent cabling pathway. Modifications to either must be coordinated. | 6.03.01.09.04 |
| 22.21 | 6.03.01.09 | 6.03.01.03 | I-DET-INF-INT.106 | Space Constraint | The maximum forward location for the barrel Time of Flight detector is limited by the position of the forward Time of Flight detector and the adjacent cabling pathway. Modifications to either must be coordinated. | 6.03.01.09.04 |
| 22.22 | 6.03.01.09 | 6.03.01.03 | I-DET-INF-INT.107 | Space Constraint | The maximum size of the silicon trackers is limited by the interior radius of the pixel support tube. Modifications to either must be coordinated. | 6.03.01.09.04 |
| 22.23 | 6.03.01.09 | 6.03.01.03 | I-DET-INF-INT.138 | Space Constraint | Rack space on the platform must be provided for a small laser system to monitor the response of the photosensors and allow adjustment of timing correlations within the detector. | 6.03.01.09.04 |

Detector Infrastructure Systems to Backward Particle ID Systems

| Section | WBS1 | WBS2 | ID | Name | Description | Owner |
| --- | --- | --- | --- | --- | --- | --- |
| 23 |  | 6.03.01.03 |  |  | Backward Particle ID Systems |  |
| 23.1 | 6.03.01.09 | 6.03.01.03 | I-DET-INF-BAR.017 | Structural Support | The global support tube will provide support for the backward PID detector system. | 6.03.01.09.01 |
| 23.2 | 6.03.01.09 | 6.03.01.03 | I-DET-INF-INT.019 | Heat Mitigation | The backward ECAL must mitigated the heat generated by the backward RICH detector (pfRICH) | 6.03.01.09.04 |
| 23.3 | 6.03.01.09 | 6.03.01.03 | I-DET-INF-INT.020 | Space Constraint | The position of the backward ECAL in the forward direction is limited by the backward face of the pfRICH detector. Modifications to either must be coordinated. | 6.03.01.09.04 |
| 23.4 | 6.03.01.09 | 6.03.01.03 | I-DET-INF-INT.087 | Process Cooling | The heat generated by the photon-sensors and associated electronics must be removed using a process cooling system. | 6.03.01.09.04 |
| 23.5 | 6.03.01.09 | 6.03.01.03 | I-DET-INF-INT.088 | Dry Nitrogen | A continuous flow of dry nitrogen will be required to protect the aerogel radiator. | 6.03.01.09.04 |
| 23.6 | 6.03.01.09 | 6.03.01.03 | I-DET-INF-INT.099 | Space Constraint | The exterior radius of the pfRICH is limited by the interior bore of the global support tube and the space needed for services for the interior detectors. Modifications to either must be coordinated. | 6.03.01.09.04 |
| 23.7 | 6.03.01.09 | 6.03.01.03 | I-DET-INF-INT.105 | Space Constraint | The maximum backward location for the barrel Time of Flight detector is limited by the position of the pfRICH detector and the adjacent cabling pathway. Modifications to either must be coordinated. | 6.03.01.09.04 |
| 23.8 | 6.03.01.09 | 6.03.01.03 | I-DET-INF-INT.108 | Space Constraint | The maximum backward location for the tracking system is limited by the position of the pfRICH. Modifications to either must be coordinated. | 6.03.01.09.04 |
| 23.9 | 6.03.01.09 | 6.03.01.03 | I-DET-INF-INT.109 | Space Constraint | The interior radius of the pfRICH is governed by the size of the beamline and its associated flanges and support system. | 6.03.01.09.04 |
| 23.10 | 6.03.01.09 | 6.03.01.03 | I-DET-INF-INT.140 | Space Constraint | Rack space on the platform must be provided for a small laser system to monitor the response of the photosensors and allow adjustment of timing correlations within the detector. | 6.03.01.09.04 |

Detector Infrastructure Systems to Forward Particle ID Systems

| Section | WBS1 | WBS2 | ID | Name | Description | Owner |
| --- | --- | --- | --- | --- | --- | --- |
| 24 |  | 6.03.01.03 |  |  | Forward Particle ID Systems |  |
| 24.1 | 6.03.01.09 | 6.03.01.03 | I-DET-INF-BAR.012 | Dual RICH Support System | The Dual RICH will be supported by a structural system within the barrel hadron calorimeter. | 6.03.01.09.01 |
| 24.2 | 6.03.01.09 | 6.03.01.03 | I-DET-INF-FWD.010 | Space Constraint | The interior radius of the dRICH is governed by the size of the beamline and its associated flanges and support system. | 6.03.01.09.03 |
| 24.3 | 6.03.01.09 | 6.03.01.03 | I-DET-INF-FWD.011 | Space Constraint | The maximum forward location for the tracking system is limited by the position of the forward AC LGAD Time of Flight Detector. Modifications to either must be coordinated. | 6.03.01.09.03 |
| 24.4 | 6.03.01.09 | 6.03.01.03 | I-DET-INF-INT.015 | Space Constraint | The forward position and shape of the barrel ECAL is limited by the backward face of the Dual RICH detector and the adjacent cabling pathway that provides services to the interior detectors. Modifications to either must be coordinated. | 6.03.01.09.04 |
| 24.5 | 6.03.01.09 | 6.03.01.03 | I-DET-INF-INT.026 | Space Constraint | The maximum forward location for the dRICH is limited by the forward ECAL and the adjacent cabling pathway that provides services to the interior detectors. Modifications to either must be coordinated. | 6.03.01.09.04 |
| 24.6 | 6.03.01.09 | 6.03.01.03 | I-DET-INF-INT.033 | Space Constraint | The size of the dRICH is limited by the interior bore of the barrel HCAL, and must provide adequate space for cables and services to itself and the interior detectors. Modifications to either must be coordinated. | 6.03.01.09.04 |
| 24.7 | 6.03.01.09 | 6.03.01.03 | I-DET-INF-INT.089 | Photosensor Cooling System | A fluid cooling system will be required to keep the operating temperature of the photosensors at -30 degrees Celsius. | 6.03.01.09.04 |
| 24.8 | 6.03.01.09 | 6.03.01.03 | I-DET-INF-INT.090 | Gas Recovery System | A recovery system will need to be included for the radiator gas used by the dRICH to prevent accidental release. | 6.03.01.09.04 |
| 24.9 | 6.03.01.09 | 6.03.01.03 | I-DET-INF-INT.091 | Insulation | Sufficient insulation will need to be added around all coolant lines to prevent condensation and ensure that ice doesn't form. | 6.03.01.09.04 |
| 24.10 | 6.03.01.09 | 6.03.01.03 | I-DET-INF-INT.092 | Dry Nitrogen for Aerogel | A continuous flow of dry nitrogen will be required to protect the aerogel radiator. | 6.03.01.09.04 |
| 24.11 | 6.03.01.09 | 6.03.01.03 | I-DET-INF-INT.093 | Dry Nitrogen for Photosensors | This gas will be required to create a protective atmosphere for the cold photosensors and to prevent condensation and freezing. | 6.03.01.09.04 |
| 24.12 | 6.03.01.09 | 6.03.01.03 | I-DET-INF-INT.094 | Radiator Gas | A continuous recirculating flow of radiator gas will be required. | 6.03.01.09.04 |
| 24.13 | 6.03.01.09 | 6.03.01.03 | I-DET-INF-INT.097 | Space Constraint | The maximum forward location for the solenoid is limited by the position of the Dual RICH detector and the cabling plenum (gap) between them. Modifications to either must be coordinated. | 6.03.01.09.04 |
| 24.14 | 6.03.01.09 | 6.03.01.03 | I-DET-INF-INT.100 | Space Constraint | The maximum forward location for the DIRC is limited by the position of the dRICH detector and the cabling plenum between them. Modifications to either must be coordinated. | 6.03.01.09.04 |
| 24.15 | 6.03.01.09 | 6.03.01.03 | I-DET-INF-INT.104 | Space Constraint | The maximum forward location for the DIRC is limited by the service gap in front of the forward RICH detector. | 6.03.01.09.04 |
| 24.16 | 6.03.01.09 | 6.03.01.03 | I-DET-INF-INT.106 | Space Constraint | The maximum forward location for the barrel Time of Flight detector is limited by the position of the forward Time of Flight detector and the adjacent cabling pathway. Modifications to either must be coordinated. | 6.03.01.09.04 |
| 24.17 | 6.03.01.09 | 6.03.01.03 | I-DET-INF-INT.139 | Space Constraint | Rack space on the platform must be provided for a small laser system to monitor the response of the photosensors and allow adjustment of timing correlations within the detector. | 6.03.01.09.04 |

Detector Infrastructure Systems to Tracking Systems

| Section | WBS1 | WBS2 | ID | Name | Description | Owner |
| --- | --- | --- | --- | --- | --- | --- |
| 25 |  | 6.03.01.02 |  |  | Tracking Systems |  |
| 25.1 | 6.03.01.09 | 6.03.01.02 | I-DET-INF-BAR.020 | Silicon Tracker Support System | The pixel support tube will provide support for the silicon trackers in the detector barrel. | 6.03.01.09.01 |

Detector Infrastructure Systems to Barrel Tracking Systems

| Section | WBS1 | WBS2 | ID | Name | Description | Owner |
| --- | --- | --- | --- | --- | --- | --- |
| 26 |  | 6.03.01.02 |  |  | Barrel Tracking Systems |  |
| 26.1 | 6.03.01.09 | 6.03.01.02 | I-DET-INF-BAR.015 | Support Structure | Composite structural support systems will support the silicon detectors, the micro-pattern gaseous detectors, and the time-of-flight detectors via a global support tube. | 6.03.01.09.01 |
| 26.2 | 6.03.01.09 | 6.03.01.02 | I-DET-INF-BAR.021 | Barrel MPGD Support | The global support tube will provide support for the barrel MPGD tracking detectors. | 6.03.01.09.01 |
| 26.3 | 6.03.01.09 | 6.03.01.02 | I-DET-INF-FWD.011 | Space Constraint | The maximum forward location for the tracking system is limited by the position of the forward AC LGAD Time of Flight Detector. Modifications to either must be coordinated. | 6.03.01.09.03 |
| 26.4 | 6.03.01.09 | 6.03.01.02 | I-DET-INF-FWD.012 | Space Constraint | The interior radius of the tracking detectors is governed by the size of the beampipe. | 6.03.01.09.03 |
| 26.5 | 6.03.01.09 | 6.03.01.02 | I-DET-INF-INT.095 | Heat Rejection | Air, liquid or other cooling technology will be required for the tracking detectors. | 6.03.01.09.04 |
| 26.6 | 6.03.01.09 | 6.03.01.02 | I-DET-INF-INT.096 | Ionization Gas | Gas will need to be provided to the trackers for detector operation. | 6.03.01.09.04 |
| 26.7 | 6.03.01.09 | 6.03.01.02 | I-DET-INF-INT.101 | Service Pathway | Conduits must be provided within the global support tube that are adequate to deliver services (power, signal, cooling) to the barrel tracking detectors. | 6.03.01.09.04 |
| 26.8 | 6.03.01.09 | 6.03.01.02 | I-DET-INF-INT.102 | Space Constraint | The exterior radius of the silicon trackers is limited by the interior bore of pixel support tube. Modifications to either must be coordinated. | 6.03.01.09.04 |
| 26.9 | 6.03.01.09 | 6.03.01.02 | I-DET-INF-INT.103 | Space Constraint | The interior radius of the inner MPGD barrel trackers is limited by the exterior radius of the pixel support tube. Modifications to either must be coordinated. | 6.03.01.09.04 |
| 26.10 | 6.03.01.09 | 6.03.01.02 | I-DET-INF-INT.107 | Space Constraint | The maximum size of the silicon trackers is limited by the interior radius of the pixel support tube. Modifications to either must be coordinated. | 6.03.01.09.04 |
| 26.11 | 6.03.01.09 | 6.03.01.02 | I-DET-INF-INT.108 | Space Constraint | The maximum backward location for the tracking system is limited by the position of the pfRICH. Modifications to either must be coordinated. | 6.03.01.09.04 |

# Detector Infrastructure Systems (DET-INF) to B-Zero Detectors (DET-ANC-B0)

I-DET-INF-FWD.005	B0 Support System

System 1:	B-Zero Detectors (6.03.01.10)

System 2:	IR Superconducting Magnets (6.04.02)

Type:	STRUCT

Description:	The B0 detector will be supported in the interior of the B0 magnet and will be dependent on the B0 magnet design, and must be adequate to support the weight of the crystals and the tracking detectors.

Provider:	Hadron Endcap Structures (6.03.01.09.03)

Details:	See details in the Hadron Endcap Structures Interface Control Document.

Requirements:	P-DET-ANC.1, P-DET-ANC.2

References:

I-DET-INF-INT.004	Space Constraint

System 1:	B-Zero Detectors (6.03.01.10)

System 2:	B-Zero Detectors (6.03.01.10)

Type:	SPACE

Description:	B0 detector must fit entirely within the bore of the B0 magnet.

Provider:	Detector Infrastructure and Utilities Integration (6.03.01.09.04)

Details:	See details in the Detector Infrastructure and Utilities Integration Interface Control Document.

Requirements:	P-DET-ANC.1

References:

I-DET-INF-INT.005	Space Constraint

System 1:	B-Zero Detectors (6.03.01.10)

System 2:	Forward HCal Systems (6.03.01.05)

Type:	SPACE

Description:	The position of the forward HCAL is bounded by the required 50cm clearance between the backward HCAL surface and the B0 magnet in the forward direction. Changes to the size or position of either must be coordinated with the other.

Provider:	Detector Infrastructure and Utilities Integration (6.03.01.09.04)

Details:	See details in the Detector Infrastructure and Utilities Integration Interface Control Document.

Requirements:	P-DET-ANC.1, P-DET-HCAL.1

References:

I-DET-INF-INT.006	Space Constraint

System 1:	B-Zero Detectors (6.03.01.10)

System 2:	IR Superconducting Magnets (6.04.02)

Type:	SPACE

Description:	The position of and accessibility of the B0 detector in the backward direction is limited by the cryostat containing the final focusing magnets.

Provider:	Detector Infrastructure and Utilities Integration (6.03.01.09.04)

Details:	See details in the Detector Infrastructure and Utilities Integration Interface Control Document.

Requirements:	P-DET-ANC.1

References:

I-DET-INF-INT.007	Space Constraint

System 1:	B-Zero Detectors (6.03.01.10)

System 2:	IR Accelerator Systems (6.04.03)

Type:	SPACE

Description:	Access to the B0 detector is constrained by the vacuum valve in front of the B0 magnet. A minimum of 15 centimeters of free space will be required on the IP-side of the B0-dipole for the installation of the B0 detectors.

Provider:	Detector Infrastructure and Utilities Integration (6.03.01.09.04)

Details:	See details in the Detector Infrastructure and Utilities Integration Interface Control Document.

Requirements:	P-DET-ANC.1

References:

I-DET-INF-INT.008	Space Constraint

System 1:	B-Zero Detectors (6.03.01.10)

System 2:	IR Accelerator Systems (6.04.03)

Type:	SPACE

Description:	The electron and hadron beampipes must be contained within the B0 detector.

Provider:	Detector Infrastructure and Utilities Integration (6.03.01.09.04)

Details:	See details in the Detector Infrastructure and Utilities Integration Interface Control Document.

Requirements:	P-DET-ANC.1

References:

I-DET-INF-INT.041	Process Cooling

System 1:	B-Zero Detectors (6.03.01.10)

System 2:	IR System Installation and Final Integeration (6.04.04)

Type:	SPACE

Description:	A cooling system will be required to remove heat from the B0-calorimeter and electronics readouts, to maintain an acceptable temperature. A chiller should be installed near the sub-system and a space should be allocated for this chiller.

Provider:	Detector Infrastructure and Utilities Integration (6.03.01.09.04)

Details:	See details in the Detector Infrastructure and Utilities Integration Interface Control Document.

Requirements:	P-DET-ANC.3

References:

# Detector Infrastructure Systems (DET-INF) to Low Q2 Detectors (DET-ANC-LOWQ2)

I-DET-INF-BCK.003	LowQ2 Support

System 1:	Electron Endcap Structures (6.03.01.09.02)

System 2:	Low Q2 Detectors (6.03.01.10)

Type:	STRUCT

Description:	The LowQ2 detector is supported by a freestanding platform that is adjacent to the outgoing electron beam.

Provider:	Electron Endcap Structures (6.03.01.09.02)

Details:	See details in the Electron Endcap Structures Interface Control Document.

Requirements:	P-DET-ANC.1, P-DET-ANC.2

References:

I-DET-INF-INT.009	Position Constraint

System 1:	Low Q2 Detectors (6.03.01.10)

System 2:	IR Accelerator Systems (6.04.03)

Type:	SPACE

Description:	The Low-Q2 detector must be upstream in the electron direction from the Q3ER magnet.

Provider:	Detector Infrastructure and Utilities Integration (6.03.01.09.04)

Details:	See details in the Detector Infrastructure and Utilities Integration Interface Control Document.

Requirements:	P-DET-ANC.1

References:

I-DET-INF-INT.010	Position Constraint

System 1:	Low Q2 Detectors (6.03.01.10)

System 2:	IR Accelerator Systems (6.04.03)

Type:	SPACE

Description:	The Low-Q2 detector must be downstream in the electron direction from the B2ER magnet.

Provider:	Detector Infrastructure and Utilities Integration (6.03.01.09.04)

Details:	See details in the Detector Infrastructure and Utilities Integration Interface Control Document.

Requirements:	P-DET-ANC.1

References:

I-DET-INF-INT.011	Position Constraint

System 1:	Low Q2 Detectors (6.03.01.10)

System 2:	IR System Installation and Final Integeration (6.04.04)

Type:	SPACE

Description:	The Low-Q2 detector must be at the same level as the electron beamline.

Provider:	Detector Infrastructure and Utilities Integration (6.03.01.09.04)

Details:	See details in the Detector Infrastructure and Utilities Integration Interface Control Document.

Requirements:	P-DET-ANC.1

References:

I-DET-INF-INT.042	Process Cooling

System 1:	Detector Infrastructure and Utilities Integration (6.03.01.09.04)

System 2:	Low Q2 Detectors (6.03.01.10)

Type:	COOL

Description:	Either a liquid or gas cooling system will be required to remove heat from the Low-Q2 calorimeter, tracking and readout electronics, to maintain them at room temperature.

Provider:	Detector Infrastructure and Utilities Integration (6.03.01.09.04)

Details:	See details in the Detector Infrastructure and Utilities Integration Interface Control Document.

Requirements:	P-DET-ANC.3

References:

# Detector Infrastructure Systems (DET-INF) to Luminosity Detectors (DET-ANC-LUMI)

I-DET-INF-BCK.004	Lumi Support System

System 1:	Electron Endcap Structures (6.03.01.09.02)

System 2:	Luminosity Detectors (6.03.01.10)

Type:	STRUCT

Description:	The luminosity detector will be supported by an independent support structure / table.

Provider:	Electron Endcap Structures (6.03.01.09.02)

Details:	See details in the Electron Endcap Structures Interface Control Document.

Requirements:	P-DET-ANC.1, P-DET-ANC.2

References:

I-DET-INF-INT.043	Dipole Power

System 1:	Detector Infrastructure and Utilities Integration (6.03.01.09.04)

System 2:	Luminosity Detectors (6.03.01.10)

Type:	ELEC

Description:	The two dipoles in front of the luminosity detectors will require an infrastructure power source.

Provider:	Detector Infrastructure and Utilities Integration (6.03.01.09.04)

Details:	See details in the Detector Infrastructure and Utilities Integration Interface Control Document.

Requirements:	P-DET-ANC.4

References:

I-DET-INF-INT.044	Dipole Vacuum

System 1:	Detector Infrastructure and Utilities Integration (6.03.01.09.04)

System 2:	Luminosity Detectors (6.03.01.10)

Type:	VAC

Description:	The beamline between the two luminosity detector dipoles must be under vacuum.

Provider:	Detector Infrastructure and Utilities Integration (6.03.01.09.04)

Details:	See details in the Detector Infrastructure and Utilities Integration Interface Control Document.

Requirements:	P-DET-ANC-LUMI.10

References:

# Detector Infrastructure Systems (DET-INF) to Off-Momentum Detectors (DET-ANC-OFFMO)

I-DET-INF-FWD.001	Detector Support

System 1:	Off-Momentum Detectors (6.03.01.10)

System 2:	IR Accelerator Systems (6.04.03)

Type:	STRUCT

Description:	The off-momentum detectors will be supported by an integrated support stand that is fully integrated with the vacuum system and allows the position of the detector to be adjusted.

Provider:	Hadron Endcap Structures (6.03.01.09.03)

Details:	See details in the Hadron Endcap Structures Interface Control Document.

Requirements:	P-DET-ANC.1, P-DET-ANC.2

References:

I-DET-INF-INT.045	Process Cooling

System 1:	Detector Infrastructure and Utilities Integration (6.03.01.09.04)

System 2:	Off-Momentum Detectors (6.03.01.10)

Type:	COOL

Description:	A cooling system will be required to remove heat from the tracking and readout electronics to prevent heat buildup.

Provider:	Detector Infrastructure and Utilities Integration (6.03.01.09.04)

Details:	See details in the Detector Infrastructure and Utilities Integration Interface Control Document.

Requirements:	P-DET-ANC.3

References:

# Detector Infrastructure Systems (DET-INF) to Roman Pots (DET-ANC-ROMAN)

I-DET-INF-FWD.002	Detector Support

System 1:	Roman Pots (6.03.01.10)

System 2:	IR Accelerator Systems (6.04.03)

Type:	STRUCT

Description:	The roman-pot detectors will be supported by an integrated support stand that is fully integrated with the vacuum system and allows the position of the detector to be adjusted.

Provider:	Hadron Endcap Structures (6.03.01.09.03)

Details:	See details in the Hadron Endcap Structures Interface Control Document.

Requirements:	P-DET-ANC.1, P-DET-ANC.2

References:

I-DET-INF-INT.046	Process Cooling

System 1:	Detector Infrastructure and Utilities Integration (6.03.01.09.04)

System 2:	Roman Pots (6.03.01.10)

Type:	COOL

Description:	A cooling system will be required to remove heat from the tracking and readout electronics to prevent heat buildup.

Provider:	Detector Infrastructure and Utilities Integration (6.03.01.09.04)

Details:	See details in the Detector Infrastructure and Utilities Integration Interface Control Document.

Requirements:	P-DET-ANC.3

References:

# Detector Infrastructure Systems (DET-INF) to Zero Degree Calorimeter (DET-ANC-ZDC)

I-DET-INF-FWD.006	ZDC Support System

System 1:	Hadron Endcap Structures (6.03.01.09.03)

System 2:	Zero Degree Calorimeter (6.03.01.10)

Type:	STRUCT

Description:	The detector will be supported on a free-standing structure / table

Provider:	Hadron Endcap Structures (6.03.01.09.03)

Details:	See details in the Hadron Endcap Structures Interface Control Document.

Requirements:	P-DET-ANC.1, P-DET-ANC.2

References:

I-DET-INF-INT.047	Process Cooling

System 1:	Detector Infrastructure and Utilities Integration (6.03.01.09.04)

System 2:	Zero Degree Calorimeter (6.03.01.10)

Type:	COOL

Description:	A cooling system will be required to remove heat from the calorimeter and readout electronics to prevent heat buildup.

Provider:	Detector Infrastructure and Utilities Integration (6.03.01.09.04)

Details:	See details in the Detector Infrastructure and Utilities Integration Interface Control Document.

Requirements:	P-DET-ANC.3

References:

# Detector Infrastructure Systems (DET-INF) to Online DAQ and Computing Systems (DET-COMP-ONLINE)

I-DET-INF-BAR.013	Fiber Support System

System 1:	Barrel Detector Structures (6.03.01.09.01)

System 2:	Online DAQ and Computing Systems (6.03.01.08.02)

Type:	STRUCT

Description:	A fiber support system must be provided that allows the communications cabling to remain connected while the detector is moved from the experimental hall to the assembly area.

Provider:	Barrel Detector Structures (6.03.01.09.01)

Details:	See details in the Barrel Detector Structures Interface Control Document.

Requirements:	P-DET-COMP.2

References:

I-DET-INF-INT.048	Computer Room Cooling

System 1:	Detector Infrastructure and Utilities Integration (6.03.01.09.04)

System 2:	Online DAQ and Computing Systems (6.03.01.08.02)

Type:	COOL

Description:	HVAC systems must be provided to maintain stable temperature and humidity in the DAQ room.

Provider:	Detector Infrastructure and Utilities Integration (6.03.01.09.04)

Details:	See details in the Detector Infrastructure and Utilities Integration Interface Control Document.

Requirements:	P-DET-COMP.3

References:

I-DET-INF-INT.049	Experimental and Assembly Area Cooling

System 1:	Detector Infrastructure and Utilities Integration (6.03.01.09.04)

System 2:	Online DAQ and Computing Systems (6.03.01.08.02)

Type:	COOL

Description:	General HVAC systems must be provided to maintain a normal operating temperature in locations where DAQ systems are in use.

Provider:	Detector Infrastructure and Utilities Integration (6.03.01.09.04)

Details:	See details in the Detector Infrastructure and Utilities Integration Interface Control Document.

Requirements:	P-DET-COMP.3

References:

I-DET-INF-INT.050	Backup Power

System 1:	Detector Infrastructure and Utilities Integration (6.03.01.09.04)

System 2:	Online DAQ and Computing Systems (6.03.01.08.02)

Type:	ELEC

Description:	UPS systems must be provided to support critical systems in order to tolerate transient conditions and to allow for controlled shutdown.

Provider:	Detector Infrastructure and Utilities Integration (6.03.01.09.04)

Details:	See details in the Detector Infrastructure and Utilities Integration Interface Control Document.

Requirements:	P-DET-COMP.4

References:

I-DET-INF-INT.051	Computer Room Power

System 1:	Detector Infrastructure and Utilities Integration (6.03.01.09.04)

System 2:	Online DAQ and Computing Systems (6.03.01.08.02)

Type:	ELEC

Description:	Adequate power must be provided to support the DAQ systems and computing infrastructure.

Provider:	Detector Infrastructure and Utilities Integration (6.03.01.09.04)

Details:	See details in the Detector Infrastructure and Utilities Integration Interface Control Document.

Requirements:	P-DET-COMP.4

References:

I-DET-INF-INT.052	Control Room Power

System 1:	Detector Infrastructure and Utilities Integration (6.03.01.09.04)

System 2:	Online DAQ and Computing Systems (6.03.01.08.02)

Type:	ELEC

Description:	Standard electrical service to support consoles and systems in the control room.

Provider:	Detector Infrastructure and Utilities Integration (6.03.01.09.04)

Details:	See details in the Detector Infrastructure and Utilities Integration Interface Control Document.

Requirements:	P-DET-COMP.4

References:

I-DET-INF-INT.053	Hall Power

System 1:	Detector Infrastructure and Utilities Integration (6.03.01.09.04)

System 2:	Online DAQ and Computing Systems (6.03.01.08.02)

Type:	ELEC

Description:	Clean power (either transformer or source isolated) should be provided for all DAQ, computing and network equipment.

Provider:	Detector Infrastructure and Utilities Integration (6.03.01.09.04)

Details:	See details in the Detector Infrastructure and Utilities Integration Interface Control Document.

Requirements:	P-DET-COMP.6

References:

I-DET-INF-INT.054	Carriage Rack Space

System 1:	Detector Infrastructure and Utilities Integration (6.03.01.09.04)

System 2:	Online DAQ and Computing Systems (6.03.01.08.02)

Type:	SPACE

Description:	Adequate rack space for the DAQ system will be required on the detector carriages.

Provider:	Detector Infrastructure and Utilities Integration (6.03.01.09.04)

Details:	See details in the Detector Infrastructure and Utilities Integration Interface Control Document.

Requirements:	P-DET-COMP.2

References:

I-DET-INF-INT.055	Computer Room

System 1:	Detector Infrastructure and Utilities Integration (6.03.01.09.04)

System 2:	Online DAQ and Computing Systems (6.03.01.08.02)

Type:	SPACE

Description:	Adequate raised-floor space with racks will be required to support DAQ computers, communications systems, and computing infrastructure.

Provider:	Detector Infrastructure and Utilities Integration (6.03.01.09.04)

Details:	See details in the Detector Infrastructure and Utilities Integration Interface Control Document.

Requirements:	P-DET-COMP.9

References:

I-DET-INF-INT.056	Control Systems Furniture

System 1:	Detector Infrastructure and Utilities Integration (6.03.01.09.04)

System 2:	Online DAQ and Computing Systems (6.03.01.08.02)

Type:	SPACE

Description:	Space for computing consoles and furniture adequate to manage the system during operation will be required. Consoles and furniture will be provided by DET-INT.

Provider:	Detector Infrastructure and Utilities Integration (6.03.01.09.04)

Details:	See details in the Detector Infrastructure and Utilities Integration Interface Control Document.

Requirements:	P-DET-COMP.2

References:

I-DET-INF-INT.057	Detector Plenums

System 1:	Detector Infrastructure and Utilities Integration (6.03.01.09.04)

System 2:	Online DAQ and Computing Systems (6.03.01.08.02)

Type:	SPACE

Description:	Adequate space to route fiberoptic cables from the racks to the detector readout boards will be required.

Provider:	Detector Infrastructure and Utilities Integration (6.03.01.09.04)

Details:	See details in the Detector Infrastructure and Utilities Integration Interface Control Document.

Requirements:	P-DET-COMP.2

References:

I-DET-INF-INT.058	Fixed Platform Conduits

System 1:	Detector Infrastructure and Utilities Integration (6.03.01.09.04)

System 2:	Online DAQ and Computing Systems (6.03.01.08.02)

Type:	SPACE

Description:	Adequate conduits and cableways must be provided to deliver cabling to the racks on the fixed platforms.

Provider:	Detector Infrastructure and Utilities Integration (6.03.01.09.04)

Details:	See details in the Detector Infrastructure and Utilities Integration Interface Control Document.

Requirements:	P-DET-COMP.2

References:

I-DET-INF-INT.059	Platform/Tunnel Rack Space

System 1:	Detector Infrastructure and Utilities Integration (6.03.01.09.04)

System 2:	Online DAQ and Computing Systems (6.03.01.08.02)

Type:	SPACE

Description:	Adequate rack space for the DAQ system must be provided on the fixed platforms, tunnels, and in the experimental hall.

Provider:	Detector Infrastructure and Utilities Integration (6.03.01.09.04)

Details:	See details in the Detector Infrastructure and Utilities Integration Interface Control Document.

Requirements:	P-DET-COMP.2

References:

I-DET-INF-INT.060	Sensitive Electronics

System 1:	Detector Infrastructure and Utilities Integration (6.03.01.09.04)

System 2:	Online DAQ and Computing Systems (6.03.01.08.02)

Type:	SPACE

Description:	DAQ systems that are sensitive to magnetism or radiation must be placed at a sufficient distance from sources OR shielding must be provided to protect the equipment.

Provider:	Detector Infrastructure and Utilities Integration (6.03.01.09.04)

Details:	See details in the Detector Infrastructure and Utilities Integration Interface Control Document.

Requirements:	P-DET-COMP.14

References:

I-DET-INF-INT.061	Tunnel Conduits

System 1:	Detector Infrastructure and Utilities Integration (6.03.01.09.04)

System 2:	Online DAQ and Computing Systems (6.03.01.08.02)

Type:	SPACE

Description:	Conduits must be provided in the west and east tunnels to connect the detectors to the DAQ system.

Provider:	Detector Infrastructure and Utilities Integration (6.03.01.09.04)

Details:	See details in the Detector Infrastructure and Utilities Integration Interface Control Document.

Requirements:	P-DET-COMP.8

References:

I-DET-INF-INT.062	Fiber Support System

System 1:	Detector Infrastructure and Utilities Integration (6.03.01.09.04)

System 2:	Online DAQ and Computing Systems (6.03.01.08.02)

Type:	STRUCT

Description:	A fiber support system must be provided that allows the communications cabling to remain connected while the detector is moved from the experimental hall to the assembly area.

Provider:	Detector Infrastructure and Utilities Integration (6.03.01.09.04)

Details:	See details in the Detector Infrastructure and Utilities Integration Interface Control Document.

Requirements:	P-DET-COMP.8

References:

# Detector Infrastructure Systems (DET-INF) to  (DET-DS)

I-DET-INF-INT.001	Magnetic Material

System 1:	DET-DS (DET-DS)

System 2:	Solenoid Magnet (6.03.01.06)

Type:	ENV

Description:	No detectors or components constructed with magnetic material may be added to the detector without prior consultation with the magnet group.

Provider:	Detector Infrastructure and Utilities Integration (6.03.01.09.04)

Details:	See details in the Detector Infrastructure and Utilities Integration Interface Control Document.

Requirements:	P-DET-MAG.13, P-DET-MAG.14

References:

I-DET-INF-INT.002	Magnetic Field Requirement

System 1:	DET-DS (DET-DS)

System 2:	Solenoid Magnet (6.03.01.06)

Type:	PARAM

Description:	Detector performance is dependent on the continuous operation of the solenoid and on the delivery of a consistent, stable magnetic field.

Provider:	Detector Infrastructure and Utilities Integration (6.03.01.09.04)

Details:	See details in the Detector Infrastructure and Utilities Integration Interface Control Document.

Requirements:	P-DET-MAG.13

References:

I-DET-INF-INT.003	Cabling/Services Conduit

System 1:	DET-DS (DET-DS)

System 2:	Barrel DIRC Particle ID Systems (6.03.01.03)

Type:	SPACE

Description:	The global support tube will provide integrated pathways for cabling and services to be delivered from the interior detectors (pf RICH, inner MPGD, barrel ToF and silicon trackers) to the exterior infrastructure.

Provider:	Detector Infrastructure and Utilities Integration (6.03.01.09.04)

Details:	See details in the Detector Infrastructure and Utilities Integration Interface Control Document.

Requirements:	P-DET-PID.4, P-DET-PID.5

References:

# Detector Infrastructure Systems (DET-INF) to Barrel EMCal Systems (DET-ECAL-BAR)

I-DET-INF-BAR.001	Barrel ECAL Support

System 1:	Barrel EMCal Systems (6.03.01.04)

System 2:	Barrel HCal Systems (6.03.01.05)

Type:	STRUCT

Description:	The barrel ECAL will be supported by a structural support system that runs across the end of the barrel along the surface at 3 and 9 o'clock to the barrel Hadron Calorimeter.

Provider:	Barrel Detector Structures (6.03.01.09.01)

Details:	See details in the Barrel Detector Structures Interface Control Document.

Requirements:	P-DET-ECAL.2, P-DET-HCAL.2

References:

I-DET-INF-BAR.002	Weight Distributed

System 1:	Barrel EMCal Systems (6.03.01.04)

System 2:	Forward HCal Systems (6.03.01.05)

Type:	STRUCT

Description:	The weight of the barrel ECAL will be transferred to the barrel Hadron Calorimeter and must be accommodated by the dedicated support systems.

Provider:	Barrel Detector Structures (6.03.01.09.01)

Details:	See details in the Barrel Detector Structures Interface Control Document.

Requirements:	P-DET-ECAL.2, P-DET-HCAL.2

References:

I-DET-INF-BAR.003	Support Structure

System 1:	Barrel EMCal Systems (6.03.01.04)

System 2:	Barrel DIRC Particle ID Systems (6.03.01.03)

Type:	STRUCT

Description:	The DIRC bar boxes will be supported by the global support tube inside the barrel Electromagnetic Calorimeter, that allows the boxes to be extracted using a system of rollers.

Provider:	Barrel Detector Structures (6.03.01.09.01)

Details:	See details in the Barrel Detector Structures Interface Control Document.

Requirements:	P-DET-ECAL.1, P-DET-ECAL.2, P-DET-PID.4, P-DET-PID.5

References:

I-DET-INF-INT.012	Space Constraint

System 1:	Barrel EMCal Systems (6.03.01.04)

System 2:	Solenoid Magnet (6.03.01.06)

Type:	SPACE

Description:	The exterior radius of the barrel ECAL (and its support system) is limited by the interior bore of the solenoid magnet. Modifications to either must be coordinated.

Provider:	Detector Infrastructure and Utilities Integration (6.03.01.09.04)

Details:	See details in the Detector Infrastructure and Utilities Integration Interface Control Document.

Requirements:	P-DET-ECAL.1, P-DET-MAG.1

References:

I-DET-INF-INT.013	Space Constraint

System 1:	Barrel EMCal Systems (6.03.01.04)

System 2:	Barrel DIRC Particle ID Systems (6.03.01.03)

Type:	SPACE

Description:	The backward position and shape of the barrel EMCAL is limited by the size and shape of the DIRC readout supports. Changes to the size or position of either must be coordinated with the other.

Provider:	Detector Infrastructure and Utilities Integration (6.03.01.09.04)

Details:	See details in the Detector Infrastructure and Utilities Integration Interface Control Document.

Requirements:	P-DET-ECAL.1, P-DET-PID.5

References:

I-DET-INF-INT.014	Space Constraint

System 1:	Barrel EMCal Systems (6.03.01.04)

System 2:	Barrel DIRC Particle ID Systems (6.03.01.03)

Type:	SPACE

Description:	The maximum size of the DIRC is limited to the interior bore of the barrel ECAL (and its support structures). Modifications to either must be coordinated.

Provider:	Detector Infrastructure and Utilities Integration (6.03.01.09.04)

Details:	See details in the Detector Infrastructure and Utilities Integration Interface Control Document.

Requirements:	P-DET-ECAL.1, P-DET-PID.5

References:

I-DET-INF-INT.015	Space Constraint

System 1:	Barrel EMCal Systems (6.03.01.04)

System 2:	Forward RICH Detector Systems (6.03.01.03)

Type:	SPACE

Description:	The forward position and shape of the barrel ECAL is limited by the backward face of the Dual RICH detector and the adjacent cabling pathway that provides services to the interior detectors. Modifications to either must be coordinated.

Provider:	Detector Infrastructure and Utilities Integration (6.03.01.09.04)

Details:	See details in the Detector Infrastructure and Utilities Integration Interface Control Document.

Requirements:	P-DET-ECAL.1, P-DET-PID.5

References:

I-DET-INF-INT.063	Imaging Layers Heat Rejection

System 1:	Detector Infrastructure and Utilities Integration (6.03.01.09.04)

System 2:	Barrel EMCal Systems (6.03.01.04)

Type:	COOL

Description:	The AstroPix imaging layers must be cooled with an independent system (TBD).

Provider:	Detector Infrastructure and Utilities Integration (6.03.01.09.04)

Details:	See details in the Detector Infrastructure and Utilities Integration Interface Control Document.

Requirements:	P-DET-ECAL.3

References:

I-DET-INF-INT.064	Silicon PM Heat Rejection

System 1:	Detector Infrastructure and Utilities Integration (6.03.01.09.04)

System 2:	Barrel EMCal Systems (6.03.01.04)

Type:	COOL

Description:	The barrel ECAL readout electronics will require chilled water or LCW cooling to maintain a safe operating temperature.

Provider:	Detector Infrastructure and Utilities Integration (6.03.01.09.04)

Details:	See details in the Detector Infrastructure and Utilities Integration Interface Control Document.

Requirements:	P-DET-ECAL.3

References:

# Detector Infrastructure Systems (DET-INF) to Backward EMCal Systems (DET-ECAL-BCK)

I-DET-INF-BAR.004	Backward ECAL Support

System 1:	Backward EMCal Systems (6.03.01.04)

System 2:	Forward HCal Systems (6.03.01.05)

Type:	STRUCT

Description:	The backward ECAL will be supported by rails integrated into the global support tube

Provider:	Barrel Detector Structures (6.03.01.09.01)

Details:	See details in the Barrel Detector Structures Interface Control Document.

Requirements:	P-DET-ECAL.2, P-DET-HCAL.2

References:

I-DET-INF-BAR.005	Weight Transfer

System 1:	Backward EMCal Systems (6.03.01.04)

System 2:	Barrel DIRC Particle ID Systems (6.03.01.03)

Type:	STRUCT

Description:	The weight of the backward ECAL will be transferred to the global support tube and must be accommodated by all subsequent support systems.

Provider:	Barrel Detector Structures (6.03.01.09.01)

Details:	See details in the Barrel Detector Structures Interface Control Document.

Requirements:	P-DET-ECAL.2, P-DET-HCAL.2

References:

I-DET-INF-BAR.006	EMCal Support System

System 1:	Backward EMCal Systems (6.03.01.04)

System 2:	Barrel DIRC Particle ID Systems (6.03.01.03)

Type:	STRUCT

Description:	The global support tube will provide support for the backward electromagnetic calorimeter.

Provider:	Barrel Detector Structures (6.03.01.09.01)

Details:	See details in the Barrel Detector Structures Interface Control Document.

Requirements:	P-DET-ECAL.2, P-DET-PID.4

References:

I-DET-INF-INT.016	Space Constraint

System 1:	Backward EMCal Systems (6.03.01.04)

System 2:	Backward HCal Systems (6.03.01.05)

Type:	SPACE

Description:	The backward position of the backward ECAL is limited by the backward Hadron Calorimeter and the adjacent cabling pathway that provides services to the interior detectors. Modifications to either must be coordinated.

Provider:	Detector Infrastructure and Utilities Integration (6.03.01.09.04)

Details:	See details in the Detector Infrastructure and Utilities Integration Interface Control Document.

Requirements:	P-DET-ECAL.1, P-DET-HCAL.1

References:

I-DET-INF-INT.017	Space Constraint

System 1:	Backward EMCal Systems (6.03.01.04)

System 2:	Barrel DIRC Particle ID Systems (6.03.01.03)

Type:	SPACE

Description:	The maximum backward location for the DIRC is limited by the position of the backward endcap support structure and the adjacent cabling pathway that provides services to the interior detectors. Modifications to either must be coordinated.

Provider:	Detector Infrastructure and Utilities Integration (6.03.01.09.04)

Details:	See details in the Detector Infrastructure and Utilities Integration Interface Control Document.

Requirements:	P-DET-ECAL.1, P-DET-PID.5

References:

I-DET-INF-INT.018	Space Constraint

System 1:	Backward EMCal Systems (6.03.01.04)

System 2:	Barrel DIRC Particle ID Systems (6.03.01.03)

Type:	SPACE

Description:	The exterior radius of the backward ECAL is limited by the interior bore of the global support tube and the space needed for all the services needed by the interior detectors. Modifications to either must be coordinated.

Provider:	Detector Infrastructure and Utilities Integration (6.03.01.09.04)

Details:	See details in the Detector Infrastructure and Utilities Integration Interface Control Document.

Requirements:	P-DET-ECAL.1, P-DET-PID.5

References:

I-DET-INF-INT.019	Heat Mitigation

System 1:	Backward EMCal Systems (6.03.01.04)

System 2:	Backward Particle ID Systems (6.03.01.03)

Type:	ENV

Description:	The backward ECAL must mitigated the heat generated by the backward RICH detector (pfRICH)

Provider:	Detector Infrastructure and Utilities Integration (6.03.01.09.04)

Details:	See details in the Detector Infrastructure and Utilities Integration Interface Control Document.

Requirements:	P-DET-ECAL.3

References:

I-DET-INF-INT.020	Space Constraint

System 1:	Backward EMCal Systems (6.03.01.04)

System 2:	Backward RICH Detector Systems (6.03.01.03)

Type:	SPACE

Description:	The position of the backward ECAL in the forward direction is limited by the backward face of the pfRICH detector. Modifications to either must be coordinated.

Provider:	Detector Infrastructure and Utilities Integration (6.03.01.09.04)

Details:	See details in the Detector Infrastructure and Utilities Integration Interface Control Document.

Requirements:	P-DET-ECAL.1, P-DET-PID.5

References:

I-DET-INF-INT.021	Backward ECAL Bore Clearance

System 1:	Backward EMCal Systems (6.03.01.04)

System 2:	IR Accelerator Systems (6.04.03)

Type:	SPACE

Description:	The bore of the backward ECAL must be designed to allow it to be inserted/removed over the existing beamline flanges.

Provider:	Detector Infrastructure and Utilities Integration (6.03.01.09.04)

Details:	See details in the Detector Infrastructure and Utilities Integration Interface Control Document.

Requirements:	P-DET-ECAL.1

References:

I-DET-INF-INT.022	Space Constraint

System 1:	Backward EMCal Systems (6.03.01.04)

System 2:	IR Accelerator Systems (6.04.03)

Type:	SPACE

Description:	The interior radius of the backward ECAL is governed by the size of the beamline.

Provider:	Detector Infrastructure and Utilities Integration (6.03.01.09.04)

Details:	See details in the Detector Infrastructure and Utilities Integration Interface Control Document.

Requirements:	P-DET-ECAL.1

References:

I-DET-INF-INT.065	Heat Rejection

System 1:	Detector Infrastructure and Utilities Integration (6.03.01.09.04)

System 2:	Backward EMCal Systems (6.03.01.04)

Type:	COOL

Description:	The backward ECAL will require chilled water or LCW cooling to maintain the temperature of the silicon photomultipliers, the electronics, and the crystals.

Provider:	Detector Infrastructure and Utilities Integration (6.03.01.09.04)

Details:	See details in the Detector Infrastructure and Utilities Integration Interface Control Document.

Requirements:	P-DET-ECAL.3

References:

# Detector Infrastructure Systems (DET-INF) to Forward EMCal Systems (DET-ECAL-FWD)

I-DET-INF-BAR.007	Forward ECAL Support

System 1:	Forward EMCal Systems (6.03.01.04)

System 2:	Forward HCal Systems (6.03.01.05)

Type:	STRUCT

Description:	The forward ECAL will be supported by the forward Hadron Calorimeter endcap. Because the forward ECAL must split into two parts to provide access to the barrel, each half must be independently affixed to the Hadron Calorimeter halves.

Provider:	Barrel Detector Structures (6.03.01.09.01)

Details:	See details in the Barrel Detector Structures Interface Control Document.

Requirements:	P-DET-ECAL.1, P-DET-ECAL.2, P-DET-HCAL.2

References:

I-DET-INF-BAR.008	Weight Transfer

System 1:	Forward EMCal Systems (6.03.01.04)

System 2:	Forward HCal Systems (6.03.01.05)

Type:	STRUCT

Description:	The weight of the forward ECAL will be transferred to the forward Hadron Calorimeter endcap and must be accommodated by all subsequent support systems.

Provider:	Barrel Detector Structures (6.03.01.09.01)

Details:	See details in the Barrel Detector Structures Interface Control Document.

Requirements:	P-DET-ECAL.1, P-DET-ECAL.2, P-DET-HCAL.2

References:

I-DET-INF-FWD.004	Sub-Detector Support

System 1:	Forward HCal Systems (6.03.01.05)

System 2:	Forward EMCal Systems (6.03.01.04)

Type:	STRUCT

Description:	The forward HCAL will support the weight of the forward electromagnetic calorimeter that is embedded within it.

Provider:	Hadron Endcap Structures (6.03.01.09.03)

Details:	See details in the Hadron Endcap Structures Interface Control Document.

Requirements:	P-DET-HCAL.2

References:

I-DET-INF-FWD.007	Forward ECAL Mobility Support

System 1:	Hadron Endcap Structures (6.03.01.09.03)

System 2:	Forward EMCal Systems (6.03.01.04)

Type:	MECH

Description:	Each half of the forward ECAL must be continuously supported and stabilized while it is moved between the opened and closed positions.

Provider:	Hadron Endcap Structures (6.03.01.09.03)

Details:	See details in the Hadron Endcap Structures Interface Control Document.

Requirements:	P-DET-ECAL.1, P-DET-ECAL.2

References:

I-DET-INF-INT.023	Space Constraint

System 1:	Forward EMCal Systems (6.03.01.04)

System 2:	Barrel HCal Systems (6.03.01.05)

Type:	SPACE

Description:	The exterior radius of the forward ECAL is limited by the interior bore of the barrel Hadron Calorimeter. Modifications to either must be coordinated.

Provider:	Detector Infrastructure and Utilities Integration (6.03.01.09.04)

Details:	See details in the Detector Infrastructure and Utilities Integration Interface Control Document.

Requirements:	P-DET-ECAL.1, P-DET-HCAL.1

References:

I-DET-INF-INT.024	Kinematic Overlap

System 1:	Forward EMCal Systems (6.03.01.04)

System 2:	Barrel HCal Systems (6.03.01.05)

Type:	SPACE

Description:	The exterior radius of the forward ECAL is constrained to provide continuous coverage with the barrel electromagnetic calorimeter. Modifications to either must be coordinated.

Provider:	Detector Infrastructure and Utilities Integration (6.03.01.09.04)

Details:	See details in the Detector Infrastructure and Utilities Integration Interface Control Document.

Requirements:	P-DET-ECAL.1, P-DET-HCAL.1

References:

I-DET-INF-INT.025	Space Constraint

System 1:	Forward EMCal Systems (6.03.01.04)

System 2:	Forward HCal Systems (6.03.01.05)

Type:	SPACE

Description:	The forward position of the forward ECAL is limited by the forward Hadron Calorimeter. Modifications to either must be coordinated.

Provider:	Detector Infrastructure and Utilities Integration (6.03.01.09.04)

Details:	See details in the Detector Infrastructure and Utilities Integration Interface Control Document.

Requirements:	P-DET-ECAL.1, P-DET-HCAL.1

References:

I-DET-INF-INT.026	Space Constraint

System 1:	Forward EMCal Systems (6.03.01.04)

System 2:	Forward RICH Detector Systems (6.03.01.03)

Type:	SPACE

Description:	The maximum forward location for the dRICH is limited by the forward ECAL and the adjacent cabling pathway that provides services to the interior detectors. Modifications to either must be coordinated.

Provider:	Detector Infrastructure and Utilities Integration (6.03.01.09.04)

Details:	See details in the Detector Infrastructure and Utilities Integration Interface Control Document.

Requirements:	P-DET-ECAL.1, P-DET-PID.5

References:

I-DET-INF-INT.066	Heat Rejection

System 1:	Detector Infrastructure and Utilities Integration (6.03.01.09.04)

System 2:	Forward EMCal Systems (6.03.01.04)

Type:	COOL

Description:	The forward ECAL readout electronics will require chilled water or LCW cooling to maintain a safe operating temperature.

Provider:	Detector Infrastructure and Utilities Integration (6.03.01.09.04)

Details:	See details in the Detector Infrastructure and Utilities Integration Interface Control Document.

Requirements:	P-DET-ECAL.3

References:

# Detector Infrastructure Systems (DET-INF) to Electronic Systems (DET-ELEC)

I-DET-INF-INT.067	Clean A/C Power

System 1:	Detector Infrastructure and Utilities Integration (6.03.01.09.04)

System 2:	Electronic Systems (6.03.01.07)

Type:	ELEC

Description:	Electronics components on the carriage will require clean or transformer isolated power.

Provider:	Detector Infrastructure and Utilities Integration (6.03.01.09.04)

Details:	See details in the Detector Infrastructure and Utilities Integration Interface Control Document.

Requirements:	P-DET-ELEC.4

References:

I-DET-INF-INT.068	UPS Power

System 1:	Detector Infrastructure and Utilities Integration (6.03.01.09.04)

System 2:	Electronic Systems (6.03.01.07)

Type:	ELEC

Description:	Critical systems, such as controls, will require sufficient UPS power to allow them to be shutdown graceful, or to tolerate transients.

Provider:	Detector Infrastructure and Utilities Integration (6.03.01.09.04)

Details:	See details in the Detector Infrastructure and Utilities Integration Interface Control Document.

Requirements:	P-DET-ELEC.4

References:

I-DET-INF-INT.069	Computing Environment

System 1:	Detector Infrastructure and Utilities Integration (6.03.01.09.04)

System 2:	Electronic Systems (6.03.01.07)

Type:	ENV

Description:	The electronics and computing systems will require a stable environment in terms of temperature and humidity.

Provider:	Detector Infrastructure and Utilities Integration (6.03.01.09.04)

Details:	See details in the Detector Infrastructure and Utilities Integration Interface Control Document.

Requirements:	P-DET-ELEC.3

References:

I-DET-INF-INT.070	EMI Shielding

System 1:	Detector Infrastructure and Utilities Integration (6.03.01.09.04)

System 2:	Electronic Systems (6.03.01.07)

Type:	ENV

Description:	The electronics and computing systems on the carriage may require some EMI shielding from detectors and other components.

Provider:	Detector Infrastructure and Utilities Integration (6.03.01.09.04)

Details:	See details in the Detector Infrastructure and Utilities Integration Interface Control Document.

Requirements:	P-DET-ELEC.6

References:

I-DET-INF-INT.071	Cabling Pathways

System 1:	Detector Infrastructure and Utilities Integration (6.03.01.09.04)

System 2:	Electronic Systems (6.03.01.07)

Type:	SPACE

Description:	Adequate space for cables and cable trays will be required to deliver low voltage and high voltage, fiber and signal cables from the carriage/platform to the sub-detectors.

Provider:	Detector Infrastructure and Utilities Integration (6.03.01.09.04)

Details:	See details in the Detector Infrastructure and Utilities Integration Interface Control Document.

Requirements:	P-DET-ELEC.1

References:

I-DET-INF-INT.072	Platform Rack Space

System 1:	Detector Infrastructure and Utilities Integration (6.03.01.09.04)

System 2:	Electronic Systems (6.03.01.07)

Type:	SPACE

Description:	Adequate space must be provided for computing enclosures (racks) on the south platform to support the electronics systems.

Provider:	Detector Infrastructure and Utilities Integration (6.03.01.09.04)

Details:	See details in the Detector Infrastructure and Utilities Integration Interface Control Document.

Requirements:	P-DET-ELEC.1

References:

# Detector Infrastructure Systems (DET-INF) to Barrel HCal Systems (DET-HCAL-BAR)

I-DET-INF-BAR.001	Barrel ECAL Support

System 1:	Barrel EMCal Systems (6.03.01.04)

System 2:	Barrel HCal Systems (6.03.01.05)

Type:	STRUCT

Description:	The barrel ECAL will be supported by a structural support system that runs across the end of the barrel along the surface at 3 and 9 o'clock to the barrel Hadron Calorimeter.

Provider:	Barrel Detector Structures (6.03.01.09.01)

Details:	See details in the Barrel Detector Structures Interface Control Document.

Requirements:	P-DET-ECAL.2, P-DET-HCAL.2

References:

I-DET-INF-BAR.009	Barrel HCAL Support

System 1:	Barrel HCal Systems (6.03.01.05)

System 2:	Barrel HCal Systems (6.03.01.05)

Type:	STRUCT

Description:	The barrel HCAL will be supported by a rolling carriage that allows the central detector to be moved between the assembly area and the experimental hall.

Provider:	Barrel Detector Structures (6.03.01.09.01)

Details:	See details in the Barrel Detector Structures Interface Control Document.

Requirements:	P-DET-HCAL.1, P-DET-HCAL.2

References:

I-DET-INF-BAR.010	Sub-Detector Support

System 1:	Barrel HCal Systems (6.03.01.05)

System 2:	Forward HCal Systems (6.03.01.05)

Type:	STRUCT

Description:	The barrel HCAL will support the weight of all of the sub-detectors and components within the central detector.

Provider:	Barrel Detector Structures (6.03.01.09.01)

Details:	See details in the Barrel Detector Structures Interface Control Document.

Requirements:	P-DET-HCAL.2

References:

I-DET-INF-BAR.011	Weight Transfer

System 1:	Barrel HCal Systems (6.03.01.05)

System 2:	Forward HCal Systems (6.03.01.05)

Type:	STRUCT

Description:	The cumulative weight of the HCAL and all of the sub-detectors it supports will be transferred to the detector carriage.

Provider:	Barrel Detector Structures (6.03.01.09.01)

Details:	See details in the Barrel Detector Structures Interface Control Document.

Requirements:	P-DET-HCAL.2

References:

I-DET-INF-BAR.012	Dual RICH Support System

System 1:	Barrel HCal Systems (6.03.01.05)

System 2:	Forward Particle ID Systems (6.03.01.03)

Type:	STRUCT

Description:	The Dual RICH will be supported by a structural system within the barrel hadron calorimeter.

Provider:	Barrel Detector Structures (6.03.01.09.01)

Details:	See details in the Barrel Detector Structures Interface Control Document.

Requirements:	P-DET-HCAL.2, P-DET-PID.4

References:

I-DET-INF-INT.023	Space Constraint

System 1:	Forward EMCal Systems (6.03.01.04)

System 2:	Barrel HCal Systems (6.03.01.05)

Type:	SPACE

Description:	The exterior radius of the forward ECAL is limited by the interior bore of the barrel Hadron Calorimeter. Modifications to either must be coordinated.

Provider:	Detector Infrastructure and Utilities Integration (6.03.01.09.04)

Details:	See details in the Detector Infrastructure and Utilities Integration Interface Control Document.

Requirements:	P-DET-ECAL.1, P-DET-HCAL.1

References:

I-DET-INF-INT.024	Kinematic Overlap

System 1:	Forward EMCal Systems (6.03.01.04)

System 2:	Barrel HCal Systems (6.03.01.05)

Type:	SPACE

Description:	The exterior radius of the forward ECAL is constrained to provide continuous coverage with the barrel electromagnetic calorimeter. Modifications to either must be coordinated.

Provider:	Detector Infrastructure and Utilities Integration (6.03.01.09.04)

Details:	See details in the Detector Infrastructure and Utilities Integration Interface Control Document.

Requirements:	P-DET-ECAL.1, P-DET-HCAL.1

References:

I-DET-INF-INT.028	Space Constraint

System 1:	Barrel HCal Systems (6.03.01.05)

System 2:	Backward HCal Systems (6.03.01.05)

Type:	SPACE

Description:	The forward position of the backward HCAL is limited by the barrel HCAL and the adjacent cabling pathway that provides services to the interior detectors. Modifications to either must be coordinated.

Provider:	Detector Infrastructure and Utilities Integration (6.03.01.09.04)

Details:	See details in the Detector Infrastructure and Utilities Integration Interface Control Document.

Requirements:	P-DET-HCAL.1

References:

I-DET-INF-INT.029	Space Constraint

System 1:	Barrel HCal Systems (6.03.01.05)

System 2:	Backward HCal Systems (6.03.01.05)

Type:	SPACE

Description:	The exterior radius of the backward HCAL should be consistent with the radius of the barrel HCAL.

Provider:	Detector Infrastructure and Utilities Integration (6.03.01.09.04)

Details:	See details in the Detector Infrastructure and Utilities Integration Interface Control Document.

Requirements:	P-DET-HCAL.1

References:

I-DET-INF-INT.030	Space Constraint

System 1:	Barrel HCal Systems (6.03.01.05)

System 2:	Forward HCal Systems (6.03.01.05)

Type:	SPACE

Description:	The exterior radius of the forward HCAL should be consistent with the radius of the barrel HCAL.

Provider:	Detector Infrastructure and Utilities Integration (6.03.01.09.04)

Details:	See details in the Detector Infrastructure and Utilities Integration Interface Control Document.

Requirements:	P-DET-HCAL.1

References:

I-DET-INF-INT.031	Space Constraint

System 1:	Barrel HCal Systems (6.03.01.05)

System 2:	Forward HCal Systems (6.03.01.05)

Type:	SPACE

Description:	The backward position of the forward HCAL is limited by the barrel HCAL and the adjacent cabling pathway that provides services to the interior detectors. Modifications to either must be coordinated.

Provider:	Detector Infrastructure and Utilities Integration (6.03.01.09.04)

Details:	See details in the Detector Infrastructure and Utilities Integration Interface Control Document.

Requirements:	P-DET-HCAL.1

References:

I-DET-INF-INT.032	External Space Constraint

System 1:	Barrel HCal Systems (6.03.01.05)

System 2:	Solenoid Magnet (6.03.01.06)

Type:	SPACE

Description:	The exterior radius of the solenoid magnet is limited by the interior bore of the barrel HCAL and necessary support structures. Modifications to either must be coordinated.

Provider:	Detector Infrastructure and Utilities Integration (6.03.01.09.04)

Details:	See details in the Detector Infrastructure and Utilities Integration Interface Control Document.

Requirements:	P-DET-HCAL.1, P-DET-MAG.1

References:

I-DET-INF-INT.033	Space Constraint

System 1:	Barrel HCal Systems (6.03.01.05)

System 2:	Forward RICH Detector Systems (6.03.01.03)

Type:	SPACE

Description:	The size of the dRICH is limited by the interior bore of the barrel HCAL, and must provide adequate space for cables and services to itself and the interior detectors. Modifications to either must be coordinated.

Provider:	Detector Infrastructure and Utilities Integration (6.03.01.09.04)

Details:	See details in the Detector Infrastructure and Utilities Integration Interface Control Document.

Requirements:	P-DET-HCAL.1, P-DET-PID.5

References:

I-DET-INF-INT.034	Space Constraint

System 1:	Backward HCal Systems (6.03.01.05)

System 2:	Barrel HCal Systems (6.03.01.05)

Type:	SPACE

Description:	The forward position of the backward HCAL is limited by the barrel HCAL and the adjacent cabling pathway that provides services to the interior detectors. Modifications to either must be coordinated.

Provider:	Detector Infrastructure and Utilities Integration (6.03.01.09.04)

Details:	See details in the Detector Infrastructure and Utilities Integration Interface Control Document.

Requirements:	P-DET-HCAL.1

References:

I-DET-INF-INT.073	Heat Rejection

System 1:	Detector Infrastructure and Utilities Integration (6.03.01.09.04)

System 2:	Barrel HCal Systems (6.03.01.05)

Type:	COOL

Description:	Heat from the forward HCAL silicon photomultipliers and front-end electronics will be released into the outside room via forced air.

Provider:	Detector Infrastructure and Utilities Integration (6.03.01.09.04)

Details:	See details in the Detector Infrastructure and Utilities Integration Interface Control Document.

Requirements:	P-DET-HCAL.3

References:

I-DET-INF-INT.074	Space Constraint

System 1:	Detector Infrastructure and Utilities Integration (6.03.01.09.04)

System 2:	Barrel HCal Systems (6.03.01.05)

Type:	SPACE

Description:	The exterior radius of the barrel HCAL is limited by the detector carriage, the electronics platforms, and other Hall infrastructure.

Provider:	Detector Infrastructure and Utilities Integration (6.03.01.09.04)

Details:	See details in the Detector Infrastructure and Utilities Integration Interface Control Document.

Requirements:	P-DET-HCAL.1

References:

# Detector Infrastructure Systems (DET-INF) to Backward HCal Systems (DET-HCAL-BCK)

I-DET-INF-BCK.001	Backward HCAL Mobility

System 1:	Backward HCal Systems (6.03.01.05)

System 2:	Forward HCal Systems (6.03.01.05)

Type:	MECH

Description:	Adequate clearance must be provided for either of the endcap detector carriages to be rolled aside to allow access to sub-detectors within the barrel.

Provider:	Electron Endcap Structures (6.03.01.09.02)

Details:	See details in the Electron Endcap Structures Interface Control Document.

Requirements:	P-DET-HCAL.2

References:

I-DET-INF-BCK.002	Backward HCAL Support

System 1:	Backward HCal Systems (6.03.01.05)

System 2:	Forward HCal Systems (6.03.01.05)

Type:	STRUCT

Description:	The backward HCAL consists of two halves, each of which are supported by an independent carriage.

Provider:	Electron Endcap Structures (6.03.01.09.02)

Details:	See details in the Electron Endcap Structures Interface Control Document.

Requirements:	P-DET-HCAL.2

References:

I-DET-INF-INT.016	Space Constraint

System 1:	Backward EMCal Systems (6.03.01.04)

System 2:	Backward HCal Systems (6.03.01.05)

Type:	SPACE

Description:	The backward position of the backward ECAL is limited by the backward Hadron Calorimeter and the adjacent cabling pathway that provides services to the interior detectors. Modifications to either must be coordinated.

Provider:	Detector Infrastructure and Utilities Integration (6.03.01.09.04)

Details:	See details in the Detector Infrastructure and Utilities Integration Interface Control Document.

Requirements:	P-DET-ECAL.1, P-DET-HCAL.1

References:

I-DET-INF-INT.028	Space Constraint

System 1:	Barrel HCal Systems (6.03.01.05)

System 2:	Backward HCal Systems (6.03.01.05)

Type:	SPACE

Description:	The forward position of the backward HCAL is limited by the barrel HCAL and the adjacent cabling pathway that provides services to the interior detectors. Modifications to either must be coordinated.

Provider:	Detector Infrastructure and Utilities Integration (6.03.01.09.04)

Details:	See details in the Detector Infrastructure and Utilities Integration Interface Control Document.

Requirements:	P-DET-HCAL.1

References:

I-DET-INF-INT.029	Space Constraint

System 1:	Barrel HCal Systems (6.03.01.05)

System 2:	Backward HCal Systems (6.03.01.05)

Type:	SPACE

Description:	The exterior radius of the backward HCAL should be consistent with the radius of the barrel HCAL.

Provider:	Detector Infrastructure and Utilities Integration (6.03.01.09.04)

Details:	See details in the Detector Infrastructure and Utilities Integration Interface Control Document.

Requirements:	P-DET-HCAL.1

References:

I-DET-INF-INT.034	Space Constraint

System 1:	Backward HCal Systems (6.03.01.05)

System 2:	Barrel HCal Systems (6.03.01.05)

Type:	SPACE

Description:	The forward position of the backward HCAL is limited by the barrel HCAL and the adjacent cabling pathway that provides services to the interior detectors. Modifications to either must be coordinated.

Provider:	Detector Infrastructure and Utilities Integration (6.03.01.09.04)

Details:	See details in the Detector Infrastructure and Utilities Integration Interface Control Document.

Requirements:	P-DET-HCAL.1

References:

I-DET-INF-INT.035	Space Constraint

System 1:	Backward HCal Systems (6.03.01.05)

System 2:	Solenoid Magnet (6.03.01.06)

Type:	SPACE

Description:	The maximum backward location for the solenoid magnet is limited by the backward HCAL and the adjacent cabling pathway that provides services to the interior detectors. Modifications to either must be coordinated.

Provider:	Detector Infrastructure and Utilities Integration (6.03.01.09.04)

Details:	See details in the Detector Infrastructure and Utilities Integration Interface Control Document.

Requirements:	P-DET-HCAL.1, P-DET-MAG.1

References:

I-DET-INF-INT.036	Space Constraint

System 1:	Backward HCal Systems (6.03.01.05)

System 2:	Barrel DIRC Particle ID Systems (6.03.01.03)

Type:	SPACE

Description:	The maximum backward location for the DIRC is limited by the backward endcap support structure and the adjacent cabling pathway that provides services to the interior detectors. Modifications to either must be coordinated.

Provider:	Detector Infrastructure and Utilities Integration (6.03.01.09.04)

Details:	See details in the Detector Infrastructure and Utilities Integration Interface Control Document.

Requirements:	P-DET-HCAL.1, P-DET-PID.5

References:

I-DET-INF-INT.037	Space Constraint

System 1:	Backward HCal Systems (6.03.01.05)

System 2:	Interaction Region (6.04)

Type:	SPACE

Description:	The position of the backward HCAL is limited in the backward direction by the adjacent accelerator magnets and the 50 cm clearance between the backward HCal surface and the backward IR magnets

Provider:	Detector Infrastructure and Utilities Integration (6.03.01.09.04)

Details:	See details in the Detector Infrastructure and Utilities Integration Interface Control Document.

Requirements:	P-DET-HCAL.1

References:

I-DET-INF-INT.038	Space Constraint

System 1:	Backward HCal Systems (6.03.01.05)

System 2:	IR Accelerator Systems (6.04.03)

Type:	SPACE

Description:	The interior radius of the backward HCAL is governed by the size of the beamline.

Provider:	Detector Infrastructure and Utilities Integration (6.03.01.09.04)

Details:	See details in the Detector Infrastructure and Utilities Integration Interface Control Document.

Requirements:	P-DET-HCAL.1

References:

I-DET-INF-INT.075	Heat Rejection

System 1:	Detector Infrastructure and Utilities Integration (6.03.01.09.04)

System 2:	Backward HCal Systems (6.03.01.05)

Type:	COOL

Description:	Heat from the silicon photomultipliers and front-end electronics will be released into the outside room via forced air.

Provider:	Detector Infrastructure and Utilities Integration (6.03.01.09.04)

Details:	See details in the Detector Infrastructure and Utilities Integration Interface Control Document.

Requirements:	P-DET-HCAL.3

References:

# Detector Infrastructure Systems (DET-INF) to Forward HCal Systems (DET-HCAL-FWD)

I-DET-INF-BAR.002	Weight Distributed

System 1:	Barrel EMCal Systems (6.03.01.04)

System 2:	Forward HCal Systems (6.03.01.05)

Type:	STRUCT

Description:	The weight of the barrel ECAL will be transferred to the barrel Hadron Calorimeter and must be accommodated by the dedicated support systems.

Provider:	Barrel Detector Structures (6.03.01.09.01)

Details:	See details in the Barrel Detector Structures Interface Control Document.

Requirements:	P-DET-ECAL.2, P-DET-HCAL.2

References:

I-DET-INF-BAR.004	Backward ECAL Support

System 1:	Backward EMCal Systems (6.03.01.04)

System 2:	Forward HCal Systems (6.03.01.05)

Type:	STRUCT

Description:	The backward ECAL will be supported by rails integrated into the global support tube

Provider:	Barrel Detector Structures (6.03.01.09.01)

Details:	See details in the Barrel Detector Structures Interface Control Document.

Requirements:	P-DET-ECAL.2, P-DET-HCAL.2

References:

I-DET-INF-BAR.007	Forward ECAL Support

System 1:	Forward EMCal Systems (6.03.01.04)

System 2:	Forward HCal Systems (6.03.01.05)

Type:	STRUCT

Description:	The forward ECAL will be supported by the forward Hadron Calorimeter endcap. Because the forward ECAL must split into two parts to provide access to the barrel, each half must be independently affixed to the Hadron Calorimeter halves.

Provider:	Barrel Detector Structures (6.03.01.09.01)

Details:	See details in the Barrel Detector Structures Interface Control Document.

Requirements:	P-DET-ECAL.1, P-DET-ECAL.2, P-DET-HCAL.2

References:

I-DET-INF-BAR.008	Weight Transfer

System 1:	Forward EMCal Systems (6.03.01.04)

System 2:	Forward HCal Systems (6.03.01.05)

Type:	STRUCT

Description:	The weight of the forward ECAL will be transferred to the forward Hadron Calorimeter endcap and must be accommodated by all subsequent support systems.

Provider:	Barrel Detector Structures (6.03.01.09.01)

Details:	See details in the Barrel Detector Structures Interface Control Document.

Requirements:	P-DET-ECAL.1, P-DET-ECAL.2, P-DET-HCAL.2

References:

I-DET-INF-BAR.010	Sub-Detector Support

System 1:	Barrel HCal Systems (6.03.01.05)

System 2:	Forward HCal Systems (6.03.01.05)

Type:	STRUCT

Description:	The barrel HCAL will support the weight of all of the sub-detectors and components within the central detector.

Provider:	Barrel Detector Structures (6.03.01.09.01)

Details:	See details in the Barrel Detector Structures Interface Control Document.

Requirements:	P-DET-HCAL.2

References:

I-DET-INF-BAR.011	Weight Transfer

System 1:	Barrel HCal Systems (6.03.01.05)

System 2:	Forward HCal Systems (6.03.01.05)

Type:	STRUCT

Description:	The cumulative weight of the HCAL and all of the sub-detectors it supports will be transferred to the detector carriage.

Provider:	Barrel Detector Structures (6.03.01.09.01)

Details:	See details in the Barrel Detector Structures Interface Control Document.

Requirements:	P-DET-HCAL.2

References:

I-DET-INF-BCK.001	Backward HCAL Mobility

System 1:	Backward HCal Systems (6.03.01.05)

System 2:	Forward HCal Systems (6.03.01.05)

Type:	MECH

Description:	Adequate clearance must be provided for either of the endcap detector carriages to be rolled aside to allow access to sub-detectors within the barrel.

Provider:	Electron Endcap Structures (6.03.01.09.02)

Details:	See details in the Electron Endcap Structures Interface Control Document.

Requirements:	P-DET-HCAL.2

References:

I-DET-INF-BCK.002	Backward HCAL Support

System 1:	Backward HCal Systems (6.03.01.05)

System 2:	Forward HCal Systems (6.03.01.05)

Type:	STRUCT

Description:	The backward HCAL consists of two halves, each of which are supported by an independent carriage.

Provider:	Electron Endcap Structures (6.03.01.09.02)

Details:	See details in the Electron Endcap Structures Interface Control Document.

Requirements:	P-DET-HCAL.2

References:

I-DET-INF-FWD.003	Forward HCAL Support

System 1:	Forward HCal Systems (6.03.01.05)

System 2:	Forward HCal Systems (6.03.01.05)

Type:	STRUCT

Description:	The forward HCAL consists of two halves, each of which are support by an independent carriage.

Provider:	Hadron Endcap Structures (6.03.01.09.03)

Details:	See details in the Hadron Endcap Structures Interface Control Document.

Requirements:	P-DET-HCAL.2

References:

I-DET-INF-FWD.004	Sub-Detector Support

System 1:	Forward HCal Systems (6.03.01.05)

System 2:	Forward EMCal Systems (6.03.01.04)

Type:	STRUCT

Description:	The forward HCAL will support the weight of the forward electromagnetic calorimeter that is embedded within it.

Provider:	Hadron Endcap Structures (6.03.01.09.03)

Details:	See details in the Hadron Endcap Structures Interface Control Document.

Requirements:	P-DET-HCAL.2

References:

I-DET-INF-FWD.008	Forward HCAL Mobility

System 1:	Hadron Endcap Structures (6.03.01.09.03)

System 2:	Forward HCal Systems (6.03.01.05)

Type:	MECH

Description:	Adequate clearance must be provided for either of the detector carriages to be rolled aside to allow access to sub-detectors within the barrel.

Provider:	Hadron Endcap Structures (6.03.01.09.03)

Details:	See details in the Hadron Endcap Structures Interface Control Document.

Requirements:	P-DET-HCAL.1, P-DET-HCAL.2

References:

I-DET-INF-FWD.009	Forward HCAL Mobility

System 1:	Hadron Endcap Structures (6.03.01.09.03)

System 2:	Forward HCal Systems (6.03.01.05)

Type:	DELETED

Description:	Adequate clearance must be provided for either of the detector carriages to be rolled aside to allow access to sub-detectors within the barrel.

Provider:	Hadron Endcap Structures (6.03.01.09.03)

Details:	See details in the Hadron Endcap Structures Interface Control Document.

Requirements:	P-DET-HCAL.1, P-DET-HCAL.2

References:

I-DET-INF-INT.005	Space Constraint

System 1:	B-Zero Detectors (6.03.01.10)

System 2:	Forward HCal Systems (6.03.01.05)

Type:	SPACE

Description:	The position of the forward HCAL is bounded by the required 50cm clearance between the backward HCAL surface and the B0 magnet in the forward direction. Changes to the size or position of either must be coordinated with the other.

Provider:	Detector Infrastructure and Utilities Integration (6.03.01.09.04)

Details:	See details in the Detector Infrastructure and Utilities Integration Interface Control Document.

Requirements:	P-DET-ANC.1, P-DET-HCAL.1

References:

I-DET-INF-INT.025	Space Constraint

System 1:	Forward EMCal Systems (6.03.01.04)

System 2:	Forward HCal Systems (6.03.01.05)

Type:	SPACE

Description:	The forward position of the forward ECAL is limited by the forward Hadron Calorimeter. Modifications to either must be coordinated.

Provider:	Detector Infrastructure and Utilities Integration (6.03.01.09.04)

Details:	See details in the Detector Infrastructure and Utilities Integration Interface Control Document.

Requirements:	P-DET-ECAL.1, P-DET-HCAL.1

References:

I-DET-INF-INT.030	Space Constraint

System 1:	Barrel HCal Systems (6.03.01.05)

System 2:	Forward HCal Systems (6.03.01.05)

Type:	SPACE

Description:	The exterior radius of the forward HCAL should be consistent with the radius of the barrel HCAL.

Provider:	Detector Infrastructure and Utilities Integration (6.03.01.09.04)

Details:	See details in the Detector Infrastructure and Utilities Integration Interface Control Document.

Requirements:	P-DET-HCAL.1

References:

I-DET-INF-INT.031	Space Constraint

System 1:	Barrel HCal Systems (6.03.01.05)

System 2:	Forward HCal Systems (6.03.01.05)

Type:	SPACE

Description:	The backward position of the forward HCAL is limited by the barrel HCAL and the adjacent cabling pathway that provides services to the interior detectors. Modifications to either must be coordinated.

Provider:	Detector Infrastructure and Utilities Integration (6.03.01.09.04)

Details:	See details in the Detector Infrastructure and Utilities Integration Interface Control Document.

Requirements:	P-DET-HCAL.1

References:

I-DET-INF-INT.039	Space Constraint

System 1:	Forward HCal Systems (6.03.01.05)

System 2:	IR Accelerator Systems (6.04.03)

Type:	SPACE

Description:	The interior radius of the forward HCAL is governed by the size of the beamline.

Provider:	Detector Infrastructure and Utilities Integration (6.03.01.09.04)

Details:	See details in the Detector Infrastructure and Utilities Integration Interface Control Document.

Requirements:	P-DET-HCAL.1

References:

I-DET-INF-INT.076	Heat Rejection

System 1:	Detector Infrastructure and Utilities Integration (6.03.01.09.04)

System 2:	Forward HCal Systems (6.03.01.05)

Type:	COOL

Description:	Heat from the silicon photomultipliers and front-end electronics will be released into the outside room via forced air. DO WE NEED CW FOR COOLING HERE?

Provider:	Detector Infrastructure and Utilities Integration (6.03.01.09.04)

Details:	See details in the Detector Infrastructure and Utilities Integration Interface Control Document.

Requirements:	P-DET-HCAL.3

References:

# Detector Infrastructure Systems (DET-INF) to Solenoid Magnet (DET-MAG)

I-DET-INF-BAR.014	External Support Structure

System 1:	Barrel Detector Structures (6.03.01.09.01)

System 2:	Solenoid Magnet (6.03.01.06)

Type:	STRUCT

Description:	Total weight of the solenoid and all embedded components must be supported.

Provider:	Barrel Detector Structures (6.03.01.09.01)

Details:	See details in the Barrel Detector Structures Interface Control Document.

Requirements:	P-DET-MAG.2

References:

I-DET-INF-BAR.016	DIRC Readout Supports

System 1:	Solenoid Magnet (6.03.01.06)

System 2:	Barrel DIRC Particle ID Systems (6.03.01.03)

Type:	STRUCT

Description:	The DIRC prism box containing the photosensor readouts, will be supported by an external structure supporting through the global support tube

Provider:	Barrel Detector Structures (6.03.01.09.01)

Details:	See details in the Barrel Detector Structures Interface Control Document.

Requirements:	P-DET-MAG.1, P-DET-MAG.2, P-DET-PID.4

References:

I-DET-INF-INT.001	Magnetic Material

System 1:	DET-DS (DET-DS)

System 2:	Solenoid Magnet (6.03.01.06)

Type:	ENV

Description:	No detectors or components constructed with magnetic material may be added to the detector without prior consultation with the magnet group.

Provider:	Detector Infrastructure and Utilities Integration (6.03.01.09.04)

Details:	See details in the Detector Infrastructure and Utilities Integration Interface Control Document.

Requirements:	P-DET-MAG.13, P-DET-MAG.14

References:

I-DET-INF-INT.002	Magnetic Field Requirement

System 1:	DET-DS (DET-DS)

System 2:	Solenoid Magnet (6.03.01.06)

Type:	PARAM

Description:	Detector performance is dependent on the continuous operation of the solenoid and on the delivery of a consistent, stable magnetic field.

Provider:	Detector Infrastructure and Utilities Integration (6.03.01.09.04)

Details:	See details in the Detector Infrastructure and Utilities Integration Interface Control Document.

Requirements:	P-DET-MAG.13

References:

I-DET-INF-INT.012	Space Constraint

System 1:	Barrel EMCal Systems (6.03.01.04)

System 2:	Solenoid Magnet (6.03.01.06)

Type:	SPACE

Description:	The exterior radius of the barrel ECAL (and its support system) is limited by the interior bore of the solenoid magnet. Modifications to either must be coordinated.

Provider:	Detector Infrastructure and Utilities Integration (6.03.01.09.04)

Details:	See details in the Detector Infrastructure and Utilities Integration Interface Control Document.

Requirements:	P-DET-ECAL.1, P-DET-MAG.1

References:

I-DET-INF-INT.032	External Space Constraint

System 1:	Barrel HCal Systems (6.03.01.05)

System 2:	Solenoid Magnet (6.03.01.06)

Type:	SPACE

Description:	The exterior radius of the solenoid magnet is limited by the interior bore of the barrel HCAL and necessary support structures. Modifications to either must be coordinated.

Provider:	Detector Infrastructure and Utilities Integration (6.03.01.09.04)

Details:	See details in the Detector Infrastructure and Utilities Integration Interface Control Document.

Requirements:	P-DET-HCAL.1, P-DET-MAG.1

References:

I-DET-INF-INT.035	Space Constraint

System 1:	Backward HCal Systems (6.03.01.05)

System 2:	Solenoid Magnet (6.03.01.06)

Type:	SPACE

Description:	The maximum backward location for the solenoid magnet is limited by the backward HCAL and the adjacent cabling pathway that provides services to the interior detectors. Modifications to either must be coordinated.

Provider:	Detector Infrastructure and Utilities Integration (6.03.01.09.04)

Details:	See details in the Detector Infrastructure and Utilities Integration Interface Control Document.

Requirements:	P-DET-HCAL.1, P-DET-MAG.1

References:

I-DET-INF-INT.040	Magnet Cooling

System 1:	Detector Infrastructure and Utilities Integration (6.03.01.09.04)

System 2:	Solenoid Magnet (6.03.01.06)

Type:	CRYO

Description:	The solenoid will require a continuous flow of cryogens to maintain temperature in both the experimental hall and the maintenance area. Interface includes cryogenic source and warm return.

Provider:	Detector Infrastructure and Utilities Integration (6.03.01.09.04)

Details:	See details in the Detector Infrastructure and Utilities Integration Interface Control Document.

Requirements:	P-DET-MAG.4

References:

I-DET-INF-INT.077	Power Supply Cooling

System 1:	Detector Infrastructure and Utilities Integration (6.03.01.09.04)

System 2:	Solenoid Magnet (6.03.01.06)

Type:	COOL

Description:	The power supply, located in an adjacent electrical room, will need to be cooled with low conductivity water.

Provider:	Detector Infrastructure and Utilities Integration (6.03.01.09.04)

Details:	See details in the Detector Infrastructure and Utilities Integration Interface Control Document.

Requirements:	P-DET-MAG.3

References:

I-DET-INF-INT.078	Magnet Cooling

System 1:	Detector Infrastructure and Utilities Integration (6.03.01.09.04)

System 2:	Solenoid Magnet (6.03.01.06)

Type:	CRYO

Description:	The solenoid will require a continuous flow of cryogens to maintain temperature in both the experimental hall and the maintenance area. Interface includes cryogenic source and warm return.

Provider:	Detector Infrastructure and Utilities Integration (6.03.01.09.04)

Details:	See details in the Detector Infrastructure and Utilities Integration Interface Control Document.

Requirements:	P-DET-MAG.4, P-DET-MAG-CCR.1, P-DET-MAG-CCR.2, P-DET-MAG-CCR.3

References:

I-DET-INF-INT.079	Electrical Power

System 1:	Detector Infrastructure and Utilities Integration (6.03.01.09.04)

System 2:	Solenoid Magnet (6.03.01.06)

Type:	ELEC

Description:	Solenoid will be provided power via the power supply located in an adjacent room.

Provider:	Detector Infrastructure and Utilities Integration (6.03.01.09.04)

Details:	See details in the Detector Infrastructure and Utilities Integration Interface Control Document.

Requirements:	P-DET-MAG.6

References:

I-DET-INF-INT.080	Instrumentation Backup Power

System 1:	Detector Infrastructure and Utilities Integration (6.03.01.09.04)

System 2:	Solenoid Magnet (6.03.01.06)

Type:	ELEC

Description:	The instrumentation rack for the solenoid will require backup/UPS power adequate to safely shutdown the system in the event of a power failure.

Provider:	Detector Infrastructure and Utilities Integration (6.03.01.09.04)

Details:	See details in the Detector Infrastructure and Utilities Integration Interface Control Document.

Requirements:	P-DET-MAG.6

References:

I-DET-INF-INT.081	Fringe Field

System 1:	Detector Infrastructure and Utilities Integration (6.03.01.09.04)

System 2:	Solenoid Magnet (6.03.01.06)

Type:	ENV

Description:	A boundary must be established to identify the extent of the magnet's stray field during operations.

Provider:	Detector Infrastructure and Utilities Integration (6.03.01.09.04)

Details:	See details in the Detector Infrastructure and Utilities Integration Interface Control Document.

Requirements:	P-DET-MAG.14

References:

I-DET-INF-INT.082	Ice Management

System 1:	Detector Infrastructure and Utilities Integration (6.03.01.09.04)

System 2:	Solenoid Magnet (6.03.01.06)

Type:	ENV

Description:	An ice management system will be required to melt accumulated ice on the chimney. (Water should evaporate and will not require drainage)

Provider:	Detector Infrastructure and Utilities Integration (6.03.01.09.04)

Details:	See details in the Detector Infrastructure and Utilities Integration Interface Control Document.

Requirements:	P-DET-MAG.5

References:

I-DET-INF-INT.083	Cryogenic Connection Point

System 1:	Detector Infrastructure and Utilities Integration (6.03.01.09.04)

System 2:	Solenoid Magnet (6.03.01.06)

Type:	SPACE

Description:	The cryogenic connection must be as close as possible to the source.

Provider:	Detector Infrastructure and Utilities Integration (6.03.01.09.04)

Details:	See details in the Detector Infrastructure and Utilities Integration Interface Control Document.

Requirements:	P-DET-MAG.4

References:

I-DET-INF-INT.084	LCW Connection

System 1:	Detector Infrastructure and Utilities Integration (6.03.01.09.04)

System 2:	Solenoid Magnet (6.03.01.06)

Type:	SPACE

Description:	The LCW water cooled leads must be fully supported along their path between the electrical room and the magnet.

Provider:	Detector Infrastructure and Utilities Integration (6.03.01.09.04)

Details:	See details in the Detector Infrastructure and Utilities Integration Interface Control Document.

Requirements:	P-DET-MAG.2

References:

I-DET-INF-INT.097	Space Constraint

System 1:	Solenoid Magnet (6.03.01.06)

System 2:	Forward RICH Detector Systems (6.03.01.03)

Type:	SPACE

Description:	The maximum forward location for the solenoid is limited by the position of the Dual RICH detector and the cabling plenum (gap) between them. Modifications to either must be coordinated.

Provider:	Detector Infrastructure and Utilities Integration (6.03.01.09.04)

Details:	See details in the Detector Infrastructure and Utilities Integration Interface Control Document.

Requirements:	P-DET-MAG.1, P-DET-PID.5

References:

I-DET-INF-INT.115	LHE Flow (Hall)

System 1:	Solenoid Magnet (6.03.01.06)

System 2:	IR System Installation and Final Integeration (6.04.04)

Type:	CRYO

Description:	A supply and return of liquid helium must be provided to the experimental hall for cooling the solenoid magnet during operation.

Provider:	Detector Infrastructure and Utilities Integration (6.03.01.09.04)

Details:	See details in the Detector Infrastructure and Utilities Integration Interface Control Document.

Requirements:	P-DET-MAG-CCR.1, P-DET-MAG-CCR.2, P-DET-MAG-CCR.3, P-DET-MAG-CCR.4, P-DET-INF-CRYO.1, P-DET-INF-CRYO.2, P-DET-INF-CRYO.3, P-DET-INF-CRYO.4

References:

I-DET-INF-INT.116	LHE Flow (Assembly Area)

System 1:	Solenoid Magnet (6.03.01.06)

System 2:	IR System Installation and Final Integeration (6.04.04)

Type:	CRYO

Description:	A supply and return of liquid helium must be provided to the assembly hall for cooling the solenoid magnet during maintenance.

Provider:	Detector Infrastructure and Utilities Integration (6.03.01.09.04)

Details:	See details in the Detector Infrastructure and Utilities Integration Interface Control Document.

Requirements:	P-DET-MAG-CCR.1, P-DET-MAG-CCR.2, P-DET-MAG-CCR.3, P-DET-MAG-CCR.4, P-DET-INF-CRYO.1, P-DET-INF-CRYO.2, P-DET-INF-CRYO.3, P-DET-INF-CRYO.4

References:

# Detector Infrastructure Systems (DET-INF) to Magnet Instrumentation and Control (DET-MAG-I&C)

I-DET-INF-INT.141	Power

System 1:	Detector Infrastructure and Utilities Integration (6.03.01.09.04)

System 2:	Magnet Instrumentation and Control (6.03.01.06)

Type:	ELEC

Description:	Low voltage DC power will be provided from the electronics racks on the south platform

Provider:	Detector Infrastructure and Utilities Integration (6.03.01.09.04)

Details:	See details in the Detector Infrastructure and Utilities Integration Interface Control Document.

Requirements:	P-DET-MAG.6

References:

# Detector Infrastructure Systems (DET-INF) to Magnet Power Supply (DET-MAG-PSU)

I-DET-INF-INT.142	Power

System 1:	Detector Infrastructure and Utilities Integration (6.03.01.09.04)

System 2:	Magnet Power Supply (6.03.01.06)

Type:	ELEC

Description:	High voltage and low voltage DC power will be provided from the power supply room.

Provider:	Detector Infrastructure and Utilities Integration (6.03.01.09.04)

Details:	See details in the Detector Infrastructure and Utilities Integration Interface Control Document.

Requirements:	P-DET-MAG.6

References:

# Detector Infrastructure Systems (DET-INF) to Barrel Particle ID Systems (DET-PID-BAR)

I-DET-INF-BAR.003	Support Structure

System 1:	Barrel EMCal Systems (6.03.01.04)

System 2:	Barrel DIRC Particle ID Systems (6.03.01.03)

Type:	STRUCT

Description:	The DIRC bar boxes will be supported by the global support tube inside the barrel Electromagnetic Calorimeter, that allows the boxes to be extracted using a system of rollers.

Provider:	Barrel Detector Structures (6.03.01.09.01)

Details:	See details in the Barrel Detector Structures Interface Control Document.

Requirements:	P-DET-ECAL.1, P-DET-ECAL.2, P-DET-PID.4, P-DET-PID.5

References:

I-DET-INF-BAR.005	Weight Transfer

System 1:	Backward EMCal Systems (6.03.01.04)

System 2:	Barrel DIRC Particle ID Systems (6.03.01.03)

Type:	STRUCT

Description:	The weight of the backward ECAL will be transferred to the global support tube and must be accommodated by all subsequent support systems.

Provider:	Barrel Detector Structures (6.03.01.09.01)

Details:	See details in the Barrel Detector Structures Interface Control Document.

Requirements:	P-DET-ECAL.2, P-DET-HCAL.2

References:

I-DET-INF-BAR.006	EMCal Support System

System 1:	Backward EMCal Systems (6.03.01.04)

System 2:	Barrel DIRC Particle ID Systems (6.03.01.03)

Type:	STRUCT

Description:	The global support tube will provide support for the backward electromagnetic calorimeter.

Provider:	Barrel Detector Structures (6.03.01.09.01)

Details:	See details in the Barrel Detector Structures Interface Control Document.

Requirements:	P-DET-ECAL.2, P-DET-PID.4

References:

I-DET-INF-BAR.016	DIRC Readout Supports

System 1:	Solenoid Magnet (6.03.01.06)

System 2:	Barrel DIRC Particle ID Systems (6.03.01.03)

Type:	STRUCT

Description:	The DIRC prism box containing the photosensor readouts, will be supported by an external structure supporting through the global support tube

Provider:	Barrel Detector Structures (6.03.01.09.01)

Details:	See details in the Barrel Detector Structures Interface Control Document.

Requirements:	P-DET-MAG.1, P-DET-MAG.2, P-DET-PID.4

References:

I-DET-INF-BAR.018	Barrel TOF Support

System 1:	Barrel Detector Structures (6.03.01.09.01)

System 2:	Barrel TOF Particle ID Systems (6.03.01.03)

Type:	STRUCT

Description:	The global support tube will provide support for the barrel time-of-flight system.

Provider:	Barrel Detector Structures (6.03.01.09.01)

Details:	See details in the Barrel Detector Structures Interface Control Document.

Requirements:	P-DET-PID.4, P-DET-PID.5

References:

I-DET-INF-BAR.019	Barrel DIRC Support

System 1:	Barrel Detector Structures (6.03.01.09.01)

System 2:	Barrel DIRC Particle ID Systems (6.03.01.03)

Type:	STRUCT

Description:	The global support tube will provide support for the DIRC system.

Provider:	Barrel Detector Structures (6.03.01.09.01)

Details:	See details in the Barrel Detector Structures Interface Control Document.

Requirements:	P-DET-PID.4, P-DET-PID.5

References:

I-DET-INF-INT.003	Cabling/Services Conduit

System 1:	DET-DS (DET-DS)

System 2:	Barrel DIRC Particle ID Systems (6.03.01.03)

Type:	SPACE

Description:	The global support tube will provide integrated pathways for cabling and services to be delivered from the interior detectors (pf RICH, inner MPGD, barrel ToF and silicon trackers) to the exterior infrastructure.

Provider:	Detector Infrastructure and Utilities Integration (6.03.01.09.04)

Details:	See details in the Detector Infrastructure and Utilities Integration Interface Control Document.

Requirements:	P-DET-PID.4, P-DET-PID.5

References:

I-DET-INF-INT.013	Space Constraint

System 1:	Barrel EMCal Systems (6.03.01.04)

System 2:	Barrel DIRC Particle ID Systems (6.03.01.03)

Type:	SPACE

Description:	The backward position and shape of the barrel EMCAL is limited by the size and shape of the DIRC readout supports. Changes to the size or position of either must be coordinated with the other.

Provider:	Detector Infrastructure and Utilities Integration (6.03.01.09.04)

Details:	See details in the Detector Infrastructure and Utilities Integration Interface Control Document.

Requirements:	P-DET-ECAL.1, P-DET-PID.5

References:

I-DET-INF-INT.014	Space Constraint

System 1:	Barrel EMCal Systems (6.03.01.04)

System 2:	Barrel DIRC Particle ID Systems (6.03.01.03)

Type:	SPACE

Description:	The maximum size of the DIRC is limited to the interior bore of the barrel ECAL (and its support structures). Modifications to either must be coordinated.

Provider:	Detector Infrastructure and Utilities Integration (6.03.01.09.04)

Details:	See details in the Detector Infrastructure and Utilities Integration Interface Control Document.

Requirements:	P-DET-ECAL.1, P-DET-PID.5

References:

I-DET-INF-INT.017	Space Constraint

System 1:	Backward EMCal Systems (6.03.01.04)

System 2:	Barrel DIRC Particle ID Systems (6.03.01.03)

Type:	SPACE

Description:	The maximum backward location for the DIRC is limited by the position of the backward endcap support structure and the adjacent cabling pathway that provides services to the interior detectors. Modifications to either must be coordinated.

Provider:	Detector Infrastructure and Utilities Integration (6.03.01.09.04)

Details:	See details in the Detector Infrastructure and Utilities Integration Interface Control Document.

Requirements:	P-DET-ECAL.1, P-DET-PID.5

References:

I-DET-INF-INT.018	Space Constraint

System 1:	Backward EMCal Systems (6.03.01.04)

System 2:	Barrel DIRC Particle ID Systems (6.03.01.03)

Type:	SPACE

Description:	The exterior radius of the backward ECAL is limited by the interior bore of the global support tube and the space needed for all the services needed by the interior detectors. Modifications to either must be coordinated.

Provider:	Detector Infrastructure and Utilities Integration (6.03.01.09.04)

Details:	See details in the Detector Infrastructure and Utilities Integration Interface Control Document.

Requirements:	P-DET-ECAL.1, P-DET-PID.5

References:

I-DET-INF-INT.036	Space Constraint

System 1:	Backward HCal Systems (6.03.01.05)

System 2:	Barrel DIRC Particle ID Systems (6.03.01.03)

Type:	SPACE

Description:	The maximum backward location for the DIRC is limited by the backward endcap support structure and the adjacent cabling pathway that provides services to the interior detectors. Modifications to either must be coordinated.

Provider:	Detector Infrastructure and Utilities Integration (6.03.01.09.04)

Details:	See details in the Detector Infrastructure and Utilities Integration Interface Control Document.

Requirements:	P-DET-HCAL.1, P-DET-PID.5

References:

I-DET-INF-INT.085	Process Cooling

System 1:	Detector Infrastructure and Utilities Integration (6.03.01.09.04)

System 2:	Barrel DIRC Particle ID Systems (6.03.01.03)

Type:	COOL

Description:	The heat generated by the photo-multiplier tubes and associated electronics must be removed using a process cooling system.

Provider:	Detector Infrastructure and Utilities Integration (6.03.01.09.04)

Details:	See details in the Detector Infrastructure and Utilities Integration Interface Control Document.

Requirements:	P-DET-PID.1

References:

I-DET-INF-INT.086	Process Cooling

System 1:	Detector Infrastructure and Utilities Integration (6.03.01.09.04)

System 2:	Barrel TOF Particle ID Systems (6.03.01.03)

Type:	COOL

Description:	The heat generated by the detector and its electronics must be removed using a process cooling system.

Provider:	Detector Infrastructure and Utilities Integration (6.03.01.09.04)

Details:	See details in the Detector Infrastructure and Utilities Integration Interface Control Document.

Requirements:	P-DET-PID.1

References:

I-DET-INF-INT.098	Space Constraint

System 1:	Barrel DIRC Particle ID Systems (6.03.01.03)

System 2:	Barrel TOF Particle ID Systems (6.03.01.03)

Type:	SPACE

Description:	The exterior radius of the pfRICH is limited by the interior bore of the global support tube and the space needed for services for the interior detectors. Modifications to either must be coordinated.

Provider:	Detector Infrastructure and Utilities Integration (6.03.01.09.04)

Details:	See details in the Detector Infrastructure and Utilities Integration Interface Control Document.

Requirements:	P-DET-PID.5

References:

I-DET-INF-INT.099	Space Constraint

System 1:	Barrel DIRC Particle ID Systems (6.03.01.03)

System 2:	Backward RICH Detector Systems (6.03.01.03)

Type:	SPACE

Description:	The exterior radius of the pfRICH is limited by the interior bore of the global support tube and the space needed for services for the interior detectors. Modifications to either must be coordinated.

Provider:	Detector Infrastructure and Utilities Integration (6.03.01.09.04)

Details:	See details in the Detector Infrastructure and Utilities Integration Interface Control Document.

Requirements:	P-DET-PID.5

References:

I-DET-INF-INT.100	Space Constraint

System 1:	Barrel DIRC Particle ID Systems (6.03.01.03)

System 2:	Forward RICH Detector Systems (6.03.01.03)

Type:	SPACE

Description:	The maximum forward location for the DIRC is limited by the position of the dRICH detector and the cabling plenum between them. Modifications to either must be coordinated.

Provider:	Detector Infrastructure and Utilities Integration (6.03.01.09.04)

Details:	See details in the Detector Infrastructure and Utilities Integration Interface Control Document.

Requirements:	P-DET-PID.5

References:

I-DET-INF-INT.101	Service Pathway

System 1:	Barrel DIRC Particle ID Systems (6.03.01.03)

System 2:	Barrel Tracking Systems (6.03.01.02)

Type:	SPACE

Description:	Conduits must be provided within the global support tube that are adequate to deliver services (power, signal, cooling) to the barrel tracking detectors.

Provider:	Detector Infrastructure and Utilities Integration (6.03.01.09.04)

Details:	See details in the Detector Infrastructure and Utilities Integration Interface Control Document.

Requirements:	P-DET-PID.5, P-DET-TRAK.9

References:

I-DET-INF-INT.104	Space Constraint

System 1:	Barrel DIRC Particle ID Systems (6.03.01.03)

System 2:	Forward RICH Detector Systems (6.03.01.03)

Type:	SPACE

Description:	The maximum forward location for the DIRC is limited by the service gap in front of the forward RICH detector.

Provider:	Detector Infrastructure and Utilities Integration (6.03.01.09.04)

Details:	See details in the Detector Infrastructure and Utilities Integration Interface Control Document.

Requirements:	P-DET-PID.5, P-DET-TRAK.9

References:

I-DET-INF-INT.105	Space Constraint

System 1:	Barrel TOF Particle ID Systems (6.03.01.03)

System 2:	Backward RICH Detector Systems (6.03.01.03)

Type:	SPACE

Description:	The maximum backward location for the barrel Time of Flight detector is limited by the position of the pfRICH detector and the adjacent cabling pathway. Modifications to either must be coordinated.

Provider:	Detector Infrastructure and Utilities Integration (6.03.01.09.04)

Details:	See details in the Detector Infrastructure and Utilities Integration Interface Control Document.

Requirements:	P-DET-PID.5

References:

I-DET-INF-INT.106	Space Constraint

System 1:	Barrel TOF Particle ID Systems (6.03.01.03)

System 2:	Forward Time of Flight Systems (6.03.01.03)

Type:	SPACE

Description:	The maximum forward location for the barrel Time of Flight detector is limited by the position of the forward Time of Flight detector and the adjacent cabling pathway. Modifications to either must be coordinated.

Provider:	Detector Infrastructure and Utilities Integration (6.03.01.09.04)

Details:	See details in the Detector Infrastructure and Utilities Integration Interface Control Document.

Requirements:	P-DET-PID.5

References:

I-DET-INF-INT.107	Space Constraint

System 1:	Barrel TOF Particle ID Systems (6.03.01.03)

System 2:	Barrel Tracking Systems (6.03.01.02)

Type:	SPACE

Description:	The maximum size of the silicon trackers is limited by the interior radius of the pixel support tube. Modifications to either must be coordinated.

Provider:	Detector Infrastructure and Utilities Integration (6.03.01.09.04)

Details:	See details in the Detector Infrastructure and Utilities Integration Interface Control Document.

Requirements:	P-DET-PID.5, P-DET-TRAK.9

References:

I-DET-INF-INT.138	Space Constraint

System 1:	Detector Infrastructure and Utilities Integration (6.03.01.09.04)

System 2:	Barrel DIRC Particle ID Systems (6.03.01.03)

Type:	SPACE

Description:	Rack space on the platform must be provided for a small laser system to monitor the response of the photosensors and allow adjustment of timing correlations within the detector.

Provider:	Detector Infrastructure and Utilities Integration (6.03.01.09.04)

Details:	See details in the Detector Infrastructure and Utilities Integration Interface Control Document.

Requirements:	P-DET-PID-BAR-DIRC.7

References:

# Detector Infrastructure Systems (DET-INF) to Backward Particle ID Systems (DET-PID-BCK)

I-DET-INF-BAR.017	Structural Support

System 1:	Barrel Detector Structures (6.03.01.09.01)

System 2:	Backward Particle ID Systems (6.03.01.03)

Type:	STRUCT

Description:	The global support tube will provide support for the backward PID detector system.

Provider:	Barrel Detector Structures (6.03.01.09.01)

Details:	See details in the Barrel Detector Structures Interface Control Document.

Requirements:	P-DET-PID.4, P-DET-PID.5

References:

I-DET-INF-INT.019	Heat Mitigation

System 1:	Backward EMCal Systems (6.03.01.04)

System 2:	Backward Particle ID Systems (6.03.01.03)

Type:	ENV

Description:	The backward ECAL must mitigated the heat generated by the backward RICH detector (pfRICH)

Provider:	Detector Infrastructure and Utilities Integration (6.03.01.09.04)

Details:	See details in the Detector Infrastructure and Utilities Integration Interface Control Document.

Requirements:	P-DET-ECAL.3

References:

I-DET-INF-INT.020	Space Constraint

System 1:	Backward EMCal Systems (6.03.01.04)

System 2:	Backward RICH Detector Systems (6.03.01.03)

Type:	SPACE

Description:	The position of the backward ECAL in the forward direction is limited by the backward face of the pfRICH detector. Modifications to either must be coordinated.

Provider:	Detector Infrastructure and Utilities Integration (6.03.01.09.04)

Details:	See details in the Detector Infrastructure and Utilities Integration Interface Control Document.

Requirements:	P-DET-ECAL.1, P-DET-PID.5

References:

I-DET-INF-INT.087	Process Cooling

System 1:	Detector Infrastructure and Utilities Integration (6.03.01.09.04)

System 2:	Backward Particle ID Systems (6.03.01.03)

Type:	COOL

Description:	The heat generated by the photon-sensors and associated electronics must be removed using a process cooling system.

Provider:	Detector Infrastructure and Utilities Integration (6.03.01.09.04)

Details:	See details in the Detector Infrastructure and Utilities Integration Interface Control Document.

Requirements:	P-DET-PID.1

References:

I-DET-INF-INT.088	Dry Nitrogen

System 1:	Detector Infrastructure and Utilities Integration (6.03.01.09.04)

System 2:	Backward Particle ID Systems (6.03.01.03)

Type:	GAS/FLUID

Description:	A continuous flow of dry nitrogen will be required to protect the aerogel radiator.

Provider:	Detector Infrastructure and Utilities Integration (6.03.01.09.04)

Details:	See details in the Detector Infrastructure and Utilities Integration Interface Control Document.

Requirements:	P-DET-PID-FWD-RICH.7

References:

I-DET-INF-INT.099	Space Constraint

System 1:	Barrel DIRC Particle ID Systems (6.03.01.03)

System 2:	Backward RICH Detector Systems (6.03.01.03)

Type:	SPACE

Description:	The exterior radius of the pfRICH is limited by the interior bore of the global support tube and the space needed for services for the interior detectors. Modifications to either must be coordinated.

Provider:	Detector Infrastructure and Utilities Integration (6.03.01.09.04)

Details:	See details in the Detector Infrastructure and Utilities Integration Interface Control Document.

Requirements:	P-DET-PID.5

References:

I-DET-INF-INT.105	Space Constraint

System 1:	Barrel TOF Particle ID Systems (6.03.01.03)

System 2:	Backward RICH Detector Systems (6.03.01.03)

Type:	SPACE

Description:	The maximum backward location for the barrel Time of Flight detector is limited by the position of the pfRICH detector and the adjacent cabling pathway. Modifications to either must be coordinated.

Provider:	Detector Infrastructure and Utilities Integration (6.03.01.09.04)

Details:	See details in the Detector Infrastructure and Utilities Integration Interface Control Document.

Requirements:	P-DET-PID.5

References:

I-DET-INF-INT.108	Space Constraint

System 1:	Backward RICH Detector Systems (6.03.01.03)

System 2:	Barrel Tracking Systems (6.03.01.02)

Type:	SPACE

Description:	The maximum backward location for the tracking system is limited by the position of the pfRICH. Modifications to either must be coordinated.

Provider:	Detector Infrastructure and Utilities Integration (6.03.01.09.04)

Details:	See details in the Detector Infrastructure and Utilities Integration Interface Control Document.

Requirements:	P-DET-PID.5, P-DET-TRAK.9

References:

I-DET-INF-INT.109	Space Constraint

System 1:	Backward RICH Detector Systems (6.03.01.03)

System 2:	IR Accelerator Systems (6.04.03)

Type:	SPACE

Description:	The interior radius of the pfRICH is governed by the size of the beamline and its associated flanges and support system.

Provider:	Detector Infrastructure and Utilities Integration (6.03.01.09.04)

Details:	See details in the Detector Infrastructure and Utilities Integration Interface Control Document.

Requirements:	P-DET-PID.5

References:

I-DET-INF-INT.140	Space Constraint

System 1:	Detector Infrastructure and Utilities Integration (6.03.01.09.04)

System 2:	Backward RICH Detector Systems (6.03.01.03)

Type:	SPACE

Description:	Rack space on the platform must be provided for a small laser system to monitor the response of the photosensors and allow adjustment of timing correlations within the detector.

Provider:	Detector Infrastructure and Utilities Integration (6.03.01.09.04)

Details:	See details in the Detector Infrastructure and Utilities Integration Interface Control Document.

Requirements:	P-DET-PID-BCK-RICH.7

References:

# Detector Infrastructure Systems (DET-INF) to Forward Particle ID Systems (DET-PID-FWD)

I-DET-INF-BAR.012	Dual RICH Support System

System 1:	Barrel HCal Systems (6.03.01.05)

System 2:	Forward Particle ID Systems (6.03.01.03)

Type:	STRUCT

Description:	The Dual RICH will be supported by a structural system within the barrel hadron calorimeter.

Provider:	Barrel Detector Structures (6.03.01.09.01)

Details:	See details in the Barrel Detector Structures Interface Control Document.

Requirements:	P-DET-HCAL.2, P-DET-PID.4

References:

I-DET-INF-FWD.010	Space Constraint

System 1:	Forward RICH Detector Systems (6.03.01.03)

System 2:	IR Accelerator Systems (6.04.03)

Type:	SPACE

Description:	The interior radius of the dRICH is governed by the size of the beamline and its associated flanges and support system.

Provider:	Hadron Endcap Structures (6.03.01.09.03)

Details:	See details in the Hadron Endcap Structures Interface Control Document.

Requirements:	P-DET-PID.5

References:

I-DET-INF-FWD.011	Space Constraint

System 1:	Forward Time of Flight Systems (6.03.01.03)

System 2:	Barrel Tracking Systems (6.03.01.02)

Type:	SPACE

Description:	The maximum forward location for the tracking system is limited by the position of the forward AC LGAD Time of Flight Detector. Modifications to either must be coordinated.

Provider:	Hadron Endcap Structures (6.03.01.09.03)

Details:	See details in the Hadron Endcap Structures Interface Control Document.

Requirements:	P-DET-PID.5, P-DET-TRAK.9

References:

I-DET-INF-INT.015	Space Constraint

System 1:	Barrel EMCal Systems (6.03.01.04)

System 2:	Forward RICH Detector Systems (6.03.01.03)

Type:	SPACE

Description:	The forward position and shape of the barrel ECAL is limited by the backward face of the Dual RICH detector and the adjacent cabling pathway that provides services to the interior detectors. Modifications to either must be coordinated.

Provider:	Detector Infrastructure and Utilities Integration (6.03.01.09.04)

Details:	See details in the Detector Infrastructure and Utilities Integration Interface Control Document.

Requirements:	P-DET-ECAL.1, P-DET-PID.5

References:

I-DET-INF-INT.026	Space Constraint

System 1:	Forward EMCal Systems (6.03.01.04)

System 2:	Forward RICH Detector Systems (6.03.01.03)

Type:	SPACE

Description:	The maximum forward location for the dRICH is limited by the forward ECAL and the adjacent cabling pathway that provides services to the interior detectors. Modifications to either must be coordinated.

Provider:	Detector Infrastructure and Utilities Integration (6.03.01.09.04)

Details:	See details in the Detector Infrastructure and Utilities Integration Interface Control Document.

Requirements:	P-DET-ECAL.1, P-DET-PID.5

References:

I-DET-INF-INT.033	Space Constraint

System 1:	Barrel HCal Systems (6.03.01.05)

System 2:	Forward RICH Detector Systems (6.03.01.03)

Type:	SPACE

Description:	The size of the dRICH is limited by the interior bore of the barrel HCAL, and must provide adequate space for cables and services to itself and the interior detectors. Modifications to either must be coordinated.

Provider:	Detector Infrastructure and Utilities Integration (6.03.01.09.04)

Details:	See details in the Detector Infrastructure and Utilities Integration Interface Control Document.

Requirements:	P-DET-HCAL.1, P-DET-PID.5

References:

I-DET-INF-INT.089	Photosensor Cooling System

System 1:	Detector Infrastructure and Utilities Integration (6.03.01.09.04)

System 2:	Forward Particle ID Systems (6.03.01.03)

Type:	COOL

Description:	A fluid cooling system will be required to keep the operating temperature of the photosensors at -30 degrees Celsius.

Provider:	Detector Infrastructure and Utilities Integration (6.03.01.09.04)

Details:	See details in the Detector Infrastructure and Utilities Integration Interface Control Document.

Requirements:	P-DET-PID.1

References:

I-DET-INF-INT.090	Gas Recovery System

System 1:	Detector Infrastructure and Utilities Integration (6.03.01.09.04)

System 2:	Forward Particle ID Systems (6.03.01.03)

Type:	ENV

Description:	A recovery system will need to be included for the radiator gas used by the dRICH to prevent accidental release.

Provider:	Detector Infrastructure and Utilities Integration (6.03.01.09.04)

Details:	See details in the Detector Infrastructure and Utilities Integration Interface Control Document.

Requirements:	P-DET-PID-FWD-RICH.6

References:

I-DET-INF-INT.091	Insulation

System 1:	Detector Infrastructure and Utilities Integration (6.03.01.09.04)

System 2:	Forward Particle ID Systems (6.03.01.03)

Type:	ENV

Description:	Sufficient insulation will need to be added around all coolant lines to prevent condensation and ensure that ice doesn't form.

Provider:	Detector Infrastructure and Utilities Integration (6.03.01.09.04)

Details:	See details in the Detector Infrastructure and Utilities Integration Interface Control Document.

Requirements:	P-DET-PID.1

References:

I-DET-INF-INT.092	Dry Nitrogen for Aerogel

System 1:	Detector Infrastructure and Utilities Integration (6.03.01.09.04)

System 2:	Forward Particle ID Systems (6.03.01.03)

Type:	GAS/FLUID

Description:	A continuous flow of dry nitrogen will be required to protect the aerogel radiator.

Provider:	Detector Infrastructure and Utilities Integration (6.03.01.09.04)

Details:	See details in the Detector Infrastructure and Utilities Integration Interface Control Document.

Requirements:	P-DET-PID.1

References:

I-DET-INF-INT.093	Dry Nitrogen for Photosensors

System 1:	Detector Infrastructure and Utilities Integration (6.03.01.09.04)

System 2:	Forward Particle ID Systems (6.03.01.03)

Type:	GAS/FLUID

Description:	This gas will be required to create a protective atmosphere for the cold photosensors and to prevent condensation and freezing.

Provider:	Detector Infrastructure and Utilities Integration (6.03.01.09.04)

Details:	See details in the Detector Infrastructure and Utilities Integration Interface Control Document.

Requirements:	P-DET-PID.1

References:

I-DET-INF-INT.094	Radiator Gas

System 1:	Detector Infrastructure and Utilities Integration (6.03.01.09.04)

System 2:	Forward Particle ID Systems (6.03.01.03)

Type:	GAS/FLUID

Description:	A continuous recirculating flow of radiator gas will be required.

Provider:	Detector Infrastructure and Utilities Integration (6.03.01.09.04)

Details:	See details in the Detector Infrastructure and Utilities Integration Interface Control Document.

Requirements:	P-DET-PID-FWD-RICH.6

References:

I-DET-INF-INT.097	Space Constraint

System 1:	Solenoid Magnet (6.03.01.06)

System 2:	Forward RICH Detector Systems (6.03.01.03)

Type:	SPACE

Description:	The maximum forward location for the solenoid is limited by the position of the Dual RICH detector and the cabling plenum (gap) between them. Modifications to either must be coordinated.

Provider:	Detector Infrastructure and Utilities Integration (6.03.01.09.04)

Details:	See details in the Detector Infrastructure and Utilities Integration Interface Control Document.

Requirements:	P-DET-MAG.1, P-DET-PID.5

References:

I-DET-INF-INT.100	Space Constraint

System 1:	Barrel DIRC Particle ID Systems (6.03.01.03)

System 2:	Forward RICH Detector Systems (6.03.01.03)

Type:	SPACE

Description:	The maximum forward location for the DIRC is limited by the position of the dRICH detector and the cabling plenum between them. Modifications to either must be coordinated.

Provider:	Detector Infrastructure and Utilities Integration (6.03.01.09.04)

Details:	See details in the Detector Infrastructure and Utilities Integration Interface Control Document.

Requirements:	P-DET-PID.5

References:

I-DET-INF-INT.104	Space Constraint

System 1:	Barrel DIRC Particle ID Systems (6.03.01.03)

System 2:	Forward RICH Detector Systems (6.03.01.03)

Type:	SPACE

Description:	The maximum forward location for the DIRC is limited by the service gap in front of the forward RICH detector.

Provider:	Detector Infrastructure and Utilities Integration (6.03.01.09.04)

Details:	See details in the Detector Infrastructure and Utilities Integration Interface Control Document.

Requirements:	P-DET-PID.5, P-DET-TRAK.9

References:

I-DET-INF-INT.106	Space Constraint

System 1:	Barrel TOF Particle ID Systems (6.03.01.03)

System 2:	Forward Time of Flight Systems (6.03.01.03)

Type:	SPACE

Description:	The maximum forward location for the barrel Time of Flight detector is limited by the position of the forward Time of Flight detector and the adjacent cabling pathway. Modifications to either must be coordinated.

Provider:	Detector Infrastructure and Utilities Integration (6.03.01.09.04)

Details:	See details in the Detector Infrastructure and Utilities Integration Interface Control Document.

Requirements:	P-DET-PID.5

References:

I-DET-INF-INT.139	Space Constraint

System 1:	Detector Infrastructure and Utilities Integration (6.03.01.09.04)

System 2:	Forward RICH Detector Systems (6.03.01.03)

Type:	SPACE

Description:	Rack space on the platform must be provided for a small laser system to monitor the response of the photosensors and allow adjustment of timing correlations within the detector.

Provider:	Detector Infrastructure and Utilities Integration (6.03.01.09.04)

Details:	See details in the Detector Infrastructure and Utilities Integration Interface Control Document.

Requirements:	P-DET-PID-FWD-RICH.10

References:

# Detector Infrastructure Systems (DET-INF) to Tracking Systems (DET-TRAK)

I-DET-INF-BAR.020	Silicon Tracker Support System

System 1:	Barrel Detector Structures (6.03.01.09.01)

System 2:	Tracking Systems (6.03.01.02)

Type:	STRUCT

Description:	The pixel support tube will provide support for the silicon trackers in the detector barrel.

Provider:	Barrel Detector Structures (6.03.01.09.01)

Details:	See details in the Barrel Detector Structures Interface Control Document.

Requirements:	P-DET-PID.4, P-DET-PID.5, P-DET-TRAK.8, P-DET-TRAK.9

References:

# Detector Infrastructure Systems (DET-INF) to Barrel Tracking Systems (DET-TRAK-BAR)

I-DET-INF-BAR.015	Support Structure

System 1:	Barrel Detector Structures (6.03.01.09.01)

System 2:	Barrel Tracking Systems (6.03.01.02)

Type:	STRUCT

Description:	Composite structural support systems will support the silicon detectors, the micro-pattern gaseous detectors, and the time-of-flight detectors via a global support tube.

Provider:	Barrel Detector Structures (6.03.01.09.01)

Details:	See details in the Barrel Detector Structures Interface Control Document.

Requirements:	P-DET-TRAK.8

References:

I-DET-INF-BAR.021	Barrel MPGD Support

System 1:	Barrel Detector Structures (6.03.01.09.01)

System 2:	Barrel Tracking Systems (6.03.01.02)

Type:	STRUCT

Description:	The global support tube will provide support for the barrel MPGD tracking detectors.

Provider:	Barrel Detector Structures (6.03.01.09.01)

Details:	See details in the Barrel Detector Structures Interface Control Document.

Requirements:	P-DET-PID.4, P-DET-TRAK.8

References:

I-DET-INF-FWD.011	Space Constraint

System 1:	Forward Time of Flight Systems (6.03.01.03)

System 2:	Barrel Tracking Systems (6.03.01.02)

Type:	SPACE

Description:	The maximum forward location for the tracking system is limited by the position of the forward AC LGAD Time of Flight Detector. Modifications to either must be coordinated.

Provider:	Hadron Endcap Structures (6.03.01.09.03)

Details:	See details in the Hadron Endcap Structures Interface Control Document.

Requirements:	P-DET-PID.5, P-DET-TRAK.9

References:

I-DET-INF-FWD.012	Space Constraint

System 1:	Barrel Tracking Systems (6.03.01.02)

System 2:	IR Accelerator Systems (6.04.03)

Type:	SPACE

Description:	The interior radius of the tracking detectors is governed by the size of the beampipe.

Provider:	Hadron Endcap Structures (6.03.01.09.03)

Details:	See details in the Hadron Endcap Structures Interface Control Document.

Requirements:	P-DET-TRAK.9

References:

I-DET-INF-INT.095	Heat Rejection

System 1:	Detector Infrastructure and Utilities Integration (6.03.01.09.04)

System 2:	Barrel Tracking Systems (6.03.01.02)

Type:	COOL

Description:	Air, liquid or other cooling technology will be required for the tracking detectors.

Provider:	Detector Infrastructure and Utilities Integration (6.03.01.09.04)

Details:	See details in the Detector Infrastructure and Utilities Integration Interface Control Document.

Requirements:	P-DET-TRAK.5

References:

I-DET-INF-INT.096	Ionization Gas

System 1:	Detector Infrastructure and Utilities Integration (6.03.01.09.04)

System 2:	Barrel Tracking Systems (6.03.01.02)

Type:	GAS/FLUID

Description:	Gas will need to be provided to the trackers for detector operation.

Provider:	Detector Infrastructure and Utilities Integration (6.03.01.09.04)

Details:	See details in the Detector Infrastructure and Utilities Integration Interface Control Document.

Requirements:	P-DET-TRAK.4

References:

I-DET-INF-INT.101	Service Pathway

System 1:	Barrel DIRC Particle ID Systems (6.03.01.03)

System 2:	Barrel Tracking Systems (6.03.01.02)

Type:	SPACE

Description:	Conduits must be provided within the global support tube that are adequate to deliver services (power, signal, cooling) to the barrel tracking detectors.

Provider:	Detector Infrastructure and Utilities Integration (6.03.01.09.04)

Details:	See details in the Detector Infrastructure and Utilities Integration Interface Control Document.

Requirements:	P-DET-PID.5, P-DET-TRAK.9

References:

I-DET-INF-INT.102	Space Constraint

System 1:	Detector Infrastructure and Utilities Integration (6.03.01.09.04)

System 2:	Barrel Tracking Systems (6.03.01.02)

Type:	SPACE

Description:	The exterior radius of the silicon trackers is limited by the interior bore of pixel support tube. Modifications to either must be coordinated.

Provider:	Detector Infrastructure and Utilities Integration (6.03.01.09.04)

Details:	See details in the Detector Infrastructure and Utilities Integration Interface Control Document.

Requirements:	P-DET-PID.5, P-DET-TRAK.9

References:

I-DET-INF-INT.103	Space Constraint

System 1:	Detector Infrastructure and Utilities Integration (6.03.01.09.04)

System 2:	Barrel Tracking Systems (6.03.01.02)

Type:	SPACE

Description:	The interior radius of the inner MPGD barrel trackers is limited by the exterior radius of the pixel support tube. Modifications to either must be coordinated.

Provider:	Detector Infrastructure and Utilities Integration (6.03.01.09.04)

Details:	See details in the Detector Infrastructure and Utilities Integration Interface Control Document.

Requirements:	P-DET-PID.5, P-DET-TRAK.9

References:

I-DET-INF-INT.107	Space Constraint

System 1:	Barrel TOF Particle ID Systems (6.03.01.03)

System 2:	Barrel Tracking Systems (6.03.01.02)

Type:	SPACE

Description:	The maximum size of the silicon trackers is limited by the interior radius of the pixel support tube. Modifications to either must be coordinated.

Provider:	Detector Infrastructure and Utilities Integration (6.03.01.09.04)

Details:	See details in the Detector Infrastructure and Utilities Integration Interface Control Document.

Requirements:	P-DET-PID.5, P-DET-TRAK.9

References:

I-DET-INF-INT.108	Space Constraint

System 1:	Backward RICH Detector Systems (6.03.01.03)

System 2:	Barrel Tracking Systems (6.03.01.02)

Type:	SPACE

Description:	The maximum backward location for the tracking system is limited by the position of the pfRICH. Modifications to either must be coordinated.

Provider:	Detector Infrastructure and Utilities Integration (6.03.01.09.04)

Details:	See details in the Detector Infrastructure and Utilities Integration Interface Control Document.

Requirements:	P-DET-PID.5, P-DET-TRAK.9

References:

# Applicable Documents

List applicable documents considered to be part of this document. Applicable documents shall be obtained from the EIC SharePoint Document Center.

# Detailed change Log

| Revision | Date | Change |
| --- | --- | --- |

# References

- doi:10.17226/25171., National Academies of Sciences, Engineering, and Medicine, "An Assessment of U.S.-Based Electron-Ion Collider Science", The National Academies Press, Washington, DC, 2018.

- EIC-ORG-PLN-010, Electron-Ion Collider L1 Requirements (Global Requirements Document)

- BNL-221006-2021-FORE, EIC Conceptual Design Report

- arXiv:2103.05419, Science Requirements and Detector Concepts for the Electron-Ion Collider: EIC Yellow Report

- EIC-SEG-PLN-016, Requirements Management Plan

- EIC-SEG-PLN-022, Systems Engineering Management Plan

- EIC-SEG-PLN-020, EIC Interface Management Plan