Electron-Ion Collider, Requirements, Specifications, and Interfaces

Interface Control Document: [###]

Electron Polarimetry Systems (6.03.01.11.01) to EIN System Installation and Final Integeration (6.05.04)

Prepared by:

{{Sig_es_:signer1:signature}}	Date: {{Dte_es_:signer1:date}}

Name, Role

Electron-Ion Collider at Institution

Reviewed by:

{{Sig_es_:signer2:signature}}	Date: {{Dte_es_:signer2:date}}

Name, Role

Electron-Ion Collider at Institution

{{Sig_es_:signer3:signature}}	Date: {{Dte_es_:signer3:date}}

Name, Role

Electron-Ion Collider at Institution

{{Sig_es_:signer4:signature}}	Date: {{Dte_es_:signer4:date}}

Name, Role

Electron-Ion Collider at Institution

Approved by:

{{Sig_es_:signer5:signature}}	Date: {{Dte_es_:signer5:date}}

Name, Role

Electron-Ion Collider at Institution

REVISION HISTORY

| Revision # | Effective Date | Additional Reviewers | Summary of Change |
| --- | --- | --- | --- |
| 00 |  |  | Initial release. |

TABLE OF CONTENTS

1.	Purpose and Scope	5

2.	Introduction	5

2.1	Electron Polarimetry Systems	5

2.2	Overview of Interface Relationships	5

2.3	Safety and Environmental Considerations	5

2.4	Design Considerations	5

2.5	Organization	5

3.	Interface summary	6

4.	Electron Polarimetry Systems (DET-POL-EPOL) to EIN System Installation and Final Integeration (EIN-SI)	7

5.	Applicable Documents	8

6.	Detailed change Log	8

7.	References	8

LIST OF ACRONYMS

| B0 | B0 is the name of the forward magnet just past the Hadron Endcap |  |
| --- | --- | --- |
| CDR | Conceptual Design Report |  |
| COTS | Commercial Off the Shelf |  |
| DAQ | Data Acquisition |  |
| DET | EIC Detector |  |
| DIRC | Detection of Internally Reflected Cherenkov Light |  |
| dRICH | Dual radiator ring imaging Cherenkov |  |
| DVCS | Deeply Virtual Compton Scattering |  |
| EIC | Electron-Ion Collider |  |
| EMCAL | Electro-magnetic Calorimeter |  |
| ESR | Electron Storage Ring |  |
| FCC | Federal Communications Commission |  |
| FDR | Final Design Review |  |
| GEM | Gas Electron Multiplier |  |
| HCAL | Hadronic Calorimeter |  |
| HSR | Hadron Storage Ring |  |
| HVAC | Heating/Ventilation/Air Conditioning |  |
| IP | Interaction Point |  |
| IR | Interaction Region |  |
| MPGD | Micro Pattern Gaseous Detectors |  |
| NAS | National Academy of Sciences |  |
| NEC | National Electric Code |  |
| NEMA | National Electrical Manufacturers Association |  |
| NFPA | National Fire Protection Association |  |
| NRTL | Nationally Recognized Testing Laboratory |  |
| PID | Particle Identification |  |
| PRR | Production Readiness Review |  |
| PWO | PbWO4 |  |
| RCS | Rapid Cycling Synchrotron |  |
| TBD | To Be Determined |  |
| UL | Underwriter Laboratories |  |
| WBS | Work Break Down Structure |  |

# Interface Control Document:

Electron Polarimetry Systems (6.03.01.11.01) to EIN System Installation and Final Integeration (6.05.04)

# Purpose and Scope

This document records the agreed upon interfaces between the Electron Polarimetry Systems and other sub-systems within the EIN System Installation and Final Integeration. These interfaces are based on the system requirements that are necessary to integrate these sub-systems as part of the Electron Ion Collider Project.

# Introduction

These interfaces describe the relationships between the Electron Polarimetry Systems and the EIN System Installation and Final Integeration including (examples by inclusion) located in (areas by inclusion).

## Electron Polarimetry Systems

Provide an introductory description of your sub-system and an overview of the things that it receives, provides, supports, constrains, or otherwise requires as part of its function --- specifically if these things are manifested as interfaces.

## Overview of Interface Relationships

The following are the sub-systems within the EIN System Installation and Final Integeration that have interfaces connected to the Electron Polarimetry Systems.

EIN System Installation and Final Integeration (EIN-SI): Identify the general types of interface relationships between the ICD reference system and this sub-system.

## Safety and Environmental Considerations

General safety and environmental considerations are identified here. Any considerations that are specific to the individual sub-system, will be addressed in external safety documents and are referenced here.

### Considerations for all systems

Add general safety and environmental considerations here.

### Special considerations and references

Add references here as necessary.

## Design Considerations

Notes here about general design considerations.

## Organization

The interfaces in this document are grouped by sub-system as identified in the sub-system identifier. All interfaces between the Electron Polarimetry Systems and a related sub-system will be included in a single section. Entries within that section will be sorted by the interface identifier or WBS number, as appropriate.

# Interface summary

This tables in this section provide a list of all interfaces that are defined within this document. The interfaces are grouped by sub-system/WBS. Each table has the following fields:

Section: the section number in this document where the interface is addressed.

WBS (1): the WBS number within the ICD reference system, unless the reference system is the owner.

WBS (2): the WBS number for the connected system.

ID: the interface identifier as defined in the Interface Requirement Document (IRD).

Name: the name of the interface as defined in the IRD.

Description: the description of the interface as defined in the IRD.

Owner: identifies the WBS number of the entity that is responsible for ensuring the interface is provided and that it meets validation/verification requirements.

Electron Polarimetry Systems to EIN System Installation and Final Integeration

| Section | WBS1 | WBS2 | ID | Name | Description | Owner |
| --- | --- | --- | --- | --- | --- | --- |
| 4 |  | 6.05.04 |  |  | EIN System Installation and Final Integeration |  |
| 4.1 | 6.03.01.11.01 | 6.05.04 | I-DET-POL-EPOL-RCS.011 | High Voltage Power | High voltage cabling will deliver power to the photon detectors from a high-power supply located in a shielded area. | 6.03.01.11.01.02 |
| 4.2 | 6.03.01.11.01 | 6.05.04 | I-DET-POL-EPOL-RCS.012 | Laser Enclosure Controls | Controls lines for the slow control system will be run from the laser lab to the laser enclosure on the beamline. | 6.03.01.11.01.02 |
| 4.3 | 6.03.01.11.01 | 6.05.04 | I-DET-POL-EPOL-RCS.016 | Low Voltage Power | Low voltage cabling will deliver power to the ASIC, which will require a pathway for a cable from a low noise power supply located in a shielded area. | 6.03.01.11.01.02 |
| 4.4 | 6.03.01.11.01 | 6.05.04 | I-DET-POL-EPOL-RCS.017 | Slow Control Power | The slow controls located in the laser enclosure on the RCS beamline will be powered using standard single-phase power. | 6.03.01.11.01.02 |

# Electron Polarimetry Systems (DET-POL-EPOL) to EIN System Installation and Final Integeration (EIN-SI)

I-DET-POL-EPOL-RCS.011	High Voltage Power

System 1:	Rapid Cycling Synchrotron Polarimetry (6.03.01.11.01.02)

System 2:	EIN System Installation and Final Integeration (6.05.04)

Type:	ELEC

Description:	High voltage cabling will deliver power to the photon detectors from a high-power supply located in a shielded area.

Provider:	Rapid Cycling Synchrotron Polarimetry (6.03.01.11.01.02)

Details:	See details in the Polarimetry and Luminosity Interface Control Document.

Requirements:	P-DET-POL.4

References:

I-DET-POL-EPOL-RCS.012	Laser Enclosure Controls

System 1:	Rapid Cycling Synchrotron Polarimetry (6.03.01.11.01.02)

System 2:	EIN System Installation and Final Integeration (6.05.04)

Type:	CONTROL

Description:	Controls lines for the slow control system will be run from the laser lab to the laser enclosure on the beamline.

Provider:	Rapid Cycling Synchrotron Polarimetry (6.03.01.11.01.02)

Details:	See details in the Polarimetry and Luminosity Interface Control Document.

Requirements:	P-DET-POL.5

References:

I-DET-POL-EPOL-RCS.016	Low Voltage Power

System 1:	Rapid Cycling Synchrotron Polarimetry (6.03.01.11.01.02)

System 2:	EIN System Installation and Final Integeration (6.05.04)

Type:	ELEC

Description:	Low voltage cabling will deliver power to the ASIC, which will require a pathway for a cable from a low noise power supply located in a shielded area.

Provider:	Rapid Cycling Synchrotron Polarimetry (6.03.01.11.01.02)

Details:	See details in the Polarimetry and Luminosity Interface Control Document.

Requirements:	P-DET-POL.4

References:

I-DET-POL-EPOL-RCS.017	Slow Control Power

System 1:	Rapid Cycling Synchrotron Polarimetry (6.03.01.11.01.02)

System 2:	EIN System Installation and Final Integeration (6.05.04)

Type:	ELEC

Description:	The slow controls located in the laser enclosure on the RCS beamline will be powered using standard single-phase power.

Provider:	Rapid Cycling Synchrotron Polarimetry (6.03.01.11.01.02)

Details:	See details in the Polarimetry and Luminosity Interface Control Document.

Requirements:	P-DET-POL.4

References:

# Applicable Documents

List applicable documents considered to be part of this document. Applicable documents shall be obtained from the EIC SharePoint Document Center.

# Detailed change Log

| Revision | Date | Change |
| --- | --- | --- |

# References

- doi:10.17226/25171., National Academies of Sciences, Engineering, and Medicine, "An Assessment of U.S.-Based Electron-Ion Collider Science", The National Academies Press, Washington, DC, 2018.

- EIC-ORG-PLN-010, Electron-Ion Collider L1 Requirements (Global Requirements Document)

- BNL-221006-2021-FORE, EIC Conceptual Design Report

- arXiv:2103.05419, Science Requirements and Detector Concepts for the Electron-Ion Collider: EIC Yellow Report

- EIC-SEG-PLN-016, Requirements Management Plan

- EIC-SEG-PLN-022, Systems Engineering Management Plan

- EIC-SEG-PLN-020, EIC Interface Management Plan