Electron-Ion Collider, Requirements, Specifications, and Interfaces

Interface Control Document: [###]

Tracking Systems (6.03.01.02) to Data Acquisition and Computing Systems (6.03.01.08)

Prepared by:

{{Sig_es_:signer1:signature}}	Date: {{Dte_es_:signer1:date}}

Name, Role

Electron-Ion Collider at Institution

Reviewed by:

{{Sig_es_:signer2:signature}}	Date: {{Dte_es_:signer2:date}}

Name, Role

Electron-Ion Collider at Institution

{{Sig_es_:signer3:signature}}	Date: {{Dte_es_:signer3:date}}

Name, Role

Electron-Ion Collider at Institution

{{Sig_es_:signer4:signature}}	Date: {{Dte_es_:signer4:date}}

Name, Role

Electron-Ion Collider at Institution

Approved by:

{{Sig_es_:signer5:signature}}	Date: {{Dte_es_:signer5:date}}

Name, Role

Electron-Ion Collider at Institution

REVISION HISTORY

| Revision # | Effective Date | Additional Reviewers | Summary of Change |
| --- | --- | --- | --- |
| 00 |  |  | Initial release. |

TABLE OF CONTENTS

1.	Purpose and Scope	5

2.	Introduction	5

2.1	Tracking Systems	5

2.2	Overview of Interface Relationships	5

2.3	Safety and Environmental Considerations	6

2.4	Design Considerations	6

2.5	Organization	6

3.	Interface summary	6

4.	Tracking Systems (DET-TRAK) to Online DAQ and Computing Systems (DET-COMP-ONLINE)	11

5.	Tracking Systems (DET-TRAK) to Electronic Systems (DET-ELEC)	12

6.	Tracking Systems (DET-TRAK) to Barrel Detector Structures (DET-INF-BAR)	12

7.	Tracking Systems (DET-TRAK) to Hadron Endcap Structures (DET-INF-FWD)	13

8.	Tracking Systems (DET-TRAK) to Detector Infrastructure and Utilities Integration (DET-INF-INT)	14

9.	Tracking Systems (DET-TRAK) to Barrel Particle ID Systems (DET-PID-BAR)	16

10.	Tracking Systems (DET-TRAK) to Backward Particle ID Systems (DET-PID-BCK)	16

11.	Tracking Systems (DET-TRAK) to Forward Particle ID Systems (DET-PID-FWD)	17

12.	Applicable Documents	17

13.	Detailed change Log	17

14.	References	17

LIST OF ACRONYMS

| B0 | B0 is the name of the forward magnet just past the Hadron Endcap |  |
| --- | --- | --- |
| CDR | Conceptual Design Report |  |
| COTS | Commercial Off the Shelf |  |
| DAQ | Data Acquisition |  |
| DET | EIC Detector |  |
| DIRC | Detection of Internally Reflected Cherenkov Light |  |
| dRICH | Dual radiator ring imaging Cherenkov |  |
| DVCS | Deeply Virtual Compton Scattering |  |
| EIC | Electron-Ion Collider |  |
| EMCAL | Electro-magnetic Calorimeter |  |
| ESR | Electron Storage Ring |  |
| FCC | Federal Communications Commission |  |
| FDR | Final Design Review |  |
| GEM | Gas Electron Multiplier |  |
| HCAL | Hadronic Calorimeter |  |
| HSR | Hadron Storage Ring |  |
| HVAC | Heating/Ventilation/Air Conditioning |  |
| IP | Interaction Point |  |
| IR | Interaction Region |  |
| MPGD | Micro Pattern Gaseous Detectors |  |
| NAS | National Academy of Sciences |  |
| NEC | National Electric Code |  |
| NEMA | National Electrical Manufacturers Association |  |
| NFPA | National Fire Protection Association |  |
| NRTL | Nationally Recognized Testing Laboratory |  |
| PID | Particle Identification |  |
| PRR | Production Readiness Review |  |
| PWO | PbWO4 |  |
| RCS | Rapid Cycling Synchrotron |  |
| TBD | To Be Determined |  |
| UL | Underwriter Laboratories |  |
| WBS | Work Break Down Structure |  |

# Interface Control Document:

Tracking Systems (6.03.01.02) to Data Acquisition and Computing Systems (6.03.01.08)

# Purpose and Scope

This document records the agreed upon interfaces between the Tracking Systems and other sub-systems within the Data Acquisition and Computing Systems. These interfaces are based on the system requirements that are necessary to integrate these sub-systems as part of the Electron Ion Collider Project.

# Introduction

These interfaces describe the relationships between the Tracking Systems and the Data Acquisition and Computing Systems including (examples by inclusion) located in (areas by inclusion).

## Tracking Systems

Provide an introductory description of your sub-system and an overview of the things that it receives, provides, supports, constrains, or otherwise requires as part of its function --- specifically if these things are manifested as interfaces.

## Overview of Interface Relationships

The following are the sub-systems within the Data Acquisition and Computing Systems that have interfaces connected to the Tracking Systems.

Online DAQ and Computing Systems (DET-COMP-ONLINE): Identify the general types of interface relationships between the ICD reference system and this sub-system.

Electronic Systems (DET-ELEC): Identify the general types of interface relationships between the ICD reference system and this sub-system.

Barrel Detector Structures (DET-INF-BAR): Identify the general types of interface relationships between the ICD reference system and this sub-system.

Hadron Endcap Structures (DET-INF-FWD): Identify the general types of interface relationships between the ICD reference system and this sub-system.

Detector Infrastructure and Utilities Integration (DET-INF-INT): Identify the general types of interface relationships between the ICD reference system and this sub-system.

Barrel Particle ID Systems (DET-PID-BAR): Identify the general types of interface relationships between the ICD reference system and this sub-system.

Backward Particle ID Systems (DET-PID-BCK): Identify the general types of interface relationships between the ICD reference system and this sub-system.

Forward Particle ID Systems (DET-PID-FWD): Identify the general types of interface relationships between the ICD reference system and this sub-system.

## Safety and Environmental Considerations

General safety and environmental considerations are identified here. Any considerations that are specific to the individual sub-system, will be addressed in external safety documents and are referenced here.

### Considerations for all systems

Add general safety and environmental considerations here.

### Special considerations and references

Add references here as necessary.

## Design Considerations

Notes here about general design considerations.

## Organization

The interfaces in this document are grouped by sub-system as identified in the sub-system identifier. All interfaces between the Tracking Systems and a related sub-system will be included in a single section. Entries within that section will be sorted by the interface identifier or WBS number, as appropriate.

# Interface summary

This tables in this section provide a list of all interfaces that are defined within this document. The interfaces are grouped by sub-system/WBS. Each table has the following fields:

Section: the section number in this document where the interface is addressed.

WBS (1): the WBS number within the ICD reference system, unless the reference system is the owner.

WBS (2): the WBS number for the connected system.

ID: the interface identifier as defined in the Interface Requirement Document (IRD).

Name: the name of the interface as defined in the IRD.

Description: the description of the interface as defined in the IRD.

Owner: identifies the WBS number of the entity that is responsible for ensuring the interface is provided and that it meets validation/verification requirements.

Tracking Systems to Online DAQ and Computing Systems

| Section | WBS1 | WBS2 | ID | Name | Description | Owner |
| --- | --- | --- | --- | --- | --- | --- |
| 4 |  | 6.03.01.08.02 |  |  | Online DAQ and Computing Systems |  |
| 4.1 | 6.03.01.02 | 6.03.01.08.02 | I-DET-COMP-ONLINE.064 | Data Acquisition | Cables required to transfer data from the detector to the online data acquisition system. | 6.03.01.08.02 |
| 4.2 | 6.03.01.02 | 6.03.01.08.02 | I-DET-COMP-ONLINE.065 | Data Transfer and Control Interface | A fiber connection will be provided from the DAQ system to the barrel tracking system's readout board to perform configuration, control, data acquisition, and timing. | 6.03.01.08.02 |
| 4.3 | 6.03.01.02 | 6.03.01.08.02 | I-DET-COMP-ONLINE.066 | Slow Controls | A network connection will be provided from the DAQ system to the barrel tracking system's slow controls interface. | 6.03.01.08.02 |

Tracking Systems to Electronic Systems

| Section | WBS1 | WBS2 | ID | Name | Description | Owner |
| --- | --- | --- | --- | --- | --- | --- |
| 5 |  | 6.03.01.07 |  |  | Electronic Systems |  |
| 5.1 | 6.03.01.02 | 6.03.01.07 | I-DET-ELEC.022 | Power | High voltage, low voltage, and bias voltage DC power will be provided from the electronics racks on the south platform | 6.03.01.07 |

Tracking Systems to Barrel Detector Structures

| Section | WBS1 | WBS2 | ID | Name | Description | Owner |
| --- | --- | --- | --- | --- | --- | --- |
| 6 |  | 6.03.01.09.01 |  |  | Barrel Detector Structures |  |
| 6.1 | 6.03.01.02 | 6.03.01.09.01 | I-DET-INF-BAR.015 | Support Structure | Composite structural support systems will support the silicon detectors, the micro-pattern gaseous detectors, and the time-of-flight detectors via a global support tube. | 6.03.01.09.01 |
| 6.2 | 6.03.01.02 | 6.03.01.09.01 | I-DET-INF-BAR.020 | Silicon Tracker Support System | The pixel support tube will provide support for the silicon trackers in the detector barrel. | 6.03.01.09.01 |
| 6.3 | 6.03.01.02 | 6.03.01.09.01 | I-DET-INF-BAR.021 | Barrel MPGD Support | The global support tube will provide support for the barrel MPGD tracking detectors. | 6.03.01.09.01 |

Tracking Systems to Hadron Endcap Structures

| Section | WBS1 | WBS2 | ID | Name | Description | Owner |
| --- | --- | --- | --- | --- | --- | --- |
| 7 |  | 6.03.01.09.03 |  |  | Hadron Endcap Structures |  |
| 7.1 | 6.03.01.02 | 6.03.01.09.03 | I-DET-INF-FWD.011 | Space Constraint | The maximum forward location for the tracking system is limited by the position of the forward AC LGAD Time of Flight Detector. Modifications to either must be coordinated. | 6.03.01.09.03 |
| 7.2 | 6.03.01.02 | 6.03.01.09.03 | I-DET-INF-FWD.012 | Space Constraint | The interior radius of the tracking detectors is governed by the size of the beampipe. | 6.03.01.09.03 |

Tracking Systems to Detector Infrastructure and Utilities Integration

| Section | WBS1 | WBS2 | ID | Name | Description | Owner |
| --- | --- | --- | --- | --- | --- | --- |
| 8 |  | 6.03.01.09.04 |  |  | Detector Infrastructure and Utilities Integration |  |
| 8.1 | 6.03.01.02 | 6.03.01.09.04 | I-DET-INF-INT.095 | Heat Rejection | Air, liquid or other cooling technology will be required for the tracking detectors. | 6.03.01.09.04 |
| 8.2 | 6.03.01.02 | 6.03.01.09.04 | I-DET-INF-INT.096 | Ionization Gas | Gas will need to be provided to the trackers for detector operation. | 6.03.01.09.04 |
| 8.3 | 6.03.01.02 | 6.03.01.09.04 | I-DET-INF-INT.101 | Service Pathway | Conduits must be provided within the global support tube that are adequate to deliver services (power, signal, cooling) to the barrel tracking detectors. | 6.03.01.09.04 |
| 8.4 | 6.03.01.02 | 6.03.01.09.04 | I-DET-INF-INT.102 | Space Constraint | The exterior radius of the silicon trackers is limited by the interior bore of pixel support tube. Modifications to either must be coordinated. | 6.03.01.09.04 |
| 8.5 | 6.03.01.02 | 6.03.01.09.04 | I-DET-INF-INT.103 | Space Constraint | The interior radius of the inner MPGD barrel trackers is limited by the exterior radius of the pixel support tube. Modifications to either must be coordinated. | 6.03.01.09.04 |
| 8.6 | 6.03.01.02 | 6.03.01.09.04 | I-DET-INF-INT.107 | Space Constraint | The maximum size of the silicon trackers is limited by the interior radius of the pixel support tube. Modifications to either must be coordinated. | 6.03.01.09.04 |
| 8.7 | 6.03.01.02 | 6.03.01.09.04 | I-DET-INF-INT.108 | Space Constraint | The maximum backward location for the tracking system is limited by the position of the pfRICH. Modifications to either must be coordinated. | 6.03.01.09.04 |

Tracking Systems to Barrel Particle ID Systems

| Section | WBS1 | WBS2 | ID | Name | Description | Owner |
| --- | --- | --- | --- | --- | --- | --- |
| 9 |  | 6.03.01.03 |  |  | Barrel Particle ID Systems |  |
| 9.1 | 6.03.01.02 | 6.03.01.03 | I-DET-INF-INT.101 | Service Pathway | Conduits must be provided within the global support tube that are adequate to deliver services (power, signal, cooling) to the barrel tracking detectors. | 6.03.01.09.04 |
| 9.2 | 6.03.01.02 | 6.03.01.03 | I-DET-INF-INT.107 | Space Constraint | The maximum size of the silicon trackers is limited by the interior radius of the pixel support tube. Modifications to either must be coordinated. | 6.03.01.09.04 |

Tracking Systems to Backward Particle ID Systems

| Section | WBS1 | WBS2 | ID | Name | Description | Owner |
| --- | --- | --- | --- | --- | --- | --- |
| 10 |  | 6.03.01.03 |  |  | Backward Particle ID Systems |  |
| 10.1 | 6.03.01.02 | 6.03.01.03 | I-DET-INF-INT.108 | Space Constraint | The maximum backward location for the tracking system is limited by the position of the pfRICH. Modifications to either must be coordinated. | 6.03.01.09.04 |

Tracking Systems to Forward Particle ID Systems

| Section | WBS1 | WBS2 | ID | Name | Description | Owner |
| --- | --- | --- | --- | --- | --- | --- |
| 11 |  | 6.03.01.03 |  |  | Forward Particle ID Systems |  |
| 11.1 | 6.03.01.02 | 6.03.01.03 | I-DET-INF-FWD.011 | Space Constraint | The maximum forward location for the tracking system is limited by the position of the forward AC LGAD Time of Flight Detector. Modifications to either must be coordinated. | 6.03.01.09.03 |

# Tracking Systems (DET-TRAK) to Online DAQ and Computing Systems (DET-COMP-ONLINE)

I-DET-COMP-ONLINE.064	Data Acquisition

System 1:	Online DAQ and Computing Systems (6.03.01.08.02)

System 2:	Barrel Tracking Systems (6.03.01.02)

Type:	DATA

Description:	Cables required to transfer data from the detector to the online data acquisition system.

Provider:	Online DAQ and Computing Systems (6.03.01.08.02)

Details:	See details in the Online DAQ and Computing Systems Interface Control Document.

Requirements:	F-DET-COMP.6

References:

I-DET-COMP-ONLINE.065	Data Transfer and Control Interface

System 1:	Online DAQ and Computing Systems (6.03.01.08.02)

System 2:	Barrel Tracking Systems (6.03.01.02)

Type:	DATA

Description:	A fiber connection will be provided from the DAQ system to the barrel tracking system's readout board to perform configuration, control, data acquisition, and timing.

Provider:	Online DAQ and Computing Systems (6.03.01.08.02)

Details:	See details in the Online DAQ and Computing Systems Interface Control Document.

Requirements:	F-DET-COMP.6, F-DET-COMP.11

References:

I-DET-COMP-ONLINE.066	Slow Controls

System 1:	Online DAQ and Computing Systems (6.03.01.08.02)

System 2:	Barrel Tracking Systems (6.03.01.02)

Type:	CONTROL

Description:	A network connection will be provided from the DAQ system to the barrel tracking system's slow controls interface.

Provider:	Online DAQ and Computing Systems (6.03.01.08.02)

Details:	See details in the Online DAQ and Computing Systems Interface Control Document.

Requirements:	F-DET-COMP-SC.1

References:

# Tracking Systems (DET-TRAK) to Electronic Systems (DET-ELEC)

I-DET-ELEC.022	Power

System 1:	Electronic Systems (6.03.01.07)

System 2:	Barrel Tracking Systems (6.03.01.02)

Type:	ELEC

Description:	High voltage, low voltage, and bias voltage DC power will be provided from the electronics racks on the south platform

Provider:	Electronic Systems (6.03.01.07)

Details:	See details in the Electronic Systems Interface Control Document.

Requirements:	P-DET-TRAK.6

References:

# Tracking Systems (DET-TRAK) to Barrel Detector Structures (DET-INF-BAR)

I-DET-INF-BAR.015	Support Structure

System 1:	Barrel Detector Structures (6.03.01.09.01)

System 2:	Barrel Tracking Systems (6.03.01.02)

Type:	STRUCT

Description:	Composite structural support systems will support the silicon detectors, the micro-pattern gaseous detectors, and the time-of-flight detectors via a global support tube.

Provider:	Barrel Detector Structures (6.03.01.09.01)

Details:	See details in the Barrel Detector Structures Interface Control Document.

Requirements:	P-DET-TRAK.8

References:

I-DET-INF-BAR.020	Silicon Tracker Support System

System 1:	Barrel Detector Structures (6.03.01.09.01)

System 2:	Tracking Systems (6.03.01.02)

Type:	STRUCT

Description:	The pixel support tube will provide support for the silicon trackers in the detector barrel.

Provider:	Barrel Detector Structures (6.03.01.09.01)

Details:	See details in the Barrel Detector Structures Interface Control Document.

Requirements:	P-DET-PID.4, P-DET-PID.5, P-DET-TRAK.8, P-DET-TRAK.9

References:

I-DET-INF-BAR.021	Barrel MPGD Support

System 1:	Barrel Detector Structures (6.03.01.09.01)

System 2:	Barrel Tracking Systems (6.03.01.02)

Type:	STRUCT

Description:	The global support tube will provide support for the barrel MPGD tracking detectors.

Provider:	Barrel Detector Structures (6.03.01.09.01)

Details:	See details in the Barrel Detector Structures Interface Control Document.

Requirements:	P-DET-PID.4, P-DET-TRAK.8

References:

# Tracking Systems (DET-TRAK) to Hadron Endcap Structures (DET-INF-FWD)

I-DET-INF-FWD.011	Space Constraint

System 1:	Forward Time of Flight Systems (6.03.01.03)

System 2:	Barrel Tracking Systems (6.03.01.02)

Type:	SPACE

Description:	The maximum forward location for the tracking system is limited by the position of the forward AC LGAD Time of Flight Detector. Modifications to either must be coordinated.

Provider:	Hadron Endcap Structures (6.03.01.09.03)

Details:	See details in the Hadron Endcap Structures Interface Control Document.

Requirements:	P-DET-PID.5, P-DET-TRAK.9

References:

I-DET-INF-FWD.012	Space Constraint

System 1:	Barrel Tracking Systems (6.03.01.02)

System 2:	IR Accelerator Systems (6.04.03)

Type:	SPACE

Description:	The interior radius of the tracking detectors is governed by the size of the beampipe.

Provider:	Hadron Endcap Structures (6.03.01.09.03)

Details:	See details in the Hadron Endcap Structures Interface Control Document.

Requirements:	P-DET-TRAK.9

References:

# Tracking Systems (DET-TRAK) to Detector Infrastructure and Utilities Integration (DET-INF-INT)

I-DET-INF-INT.095	Heat Rejection

System 1:	Detector Infrastructure and Utilities Integration (6.03.01.09.04)

System 2:	Barrel Tracking Systems (6.03.01.02)

Type:	COOL

Description:	Air, liquid or other cooling technology will be required for the tracking detectors.

Provider:	Detector Infrastructure and Utilities Integration (6.03.01.09.04)

Details:	See details in the Detector Infrastructure and Utilities Integration Interface Control Document.

Requirements:	P-DET-TRAK.5

References:

I-DET-INF-INT.096	Ionization Gas

System 1:	Detector Infrastructure and Utilities Integration (6.03.01.09.04)

System 2:	Barrel Tracking Systems (6.03.01.02)

Type:	GAS/FLUID

Description:	Gas will need to be provided to the trackers for detector operation.

Provider:	Detector Infrastructure and Utilities Integration (6.03.01.09.04)

Details:	See details in the Detector Infrastructure and Utilities Integration Interface Control Document.

Requirements:	P-DET-TRAK.4

References:

I-DET-INF-INT.101	Service Pathway

System 1:	Barrel DIRC Particle ID Systems (6.03.01.03)

System 2:	Barrel Tracking Systems (6.03.01.02)

Type:	SPACE

Description:	Conduits must be provided within the global support tube that are adequate to deliver services (power, signal, cooling) to the barrel tracking detectors.

Provider:	Detector Infrastructure and Utilities Integration (6.03.01.09.04)

Details:	See details in the Detector Infrastructure and Utilities Integration Interface Control Document.

Requirements:	P-DET-PID.5, P-DET-TRAK.9

References:

I-DET-INF-INT.102	Space Constraint

System 1:	Detector Infrastructure and Utilities Integration (6.03.01.09.04)

System 2:	Barrel Tracking Systems (6.03.01.02)

Type:	SPACE

Description:	The exterior radius of the silicon trackers is limited by the interior bore of pixel support tube. Modifications to either must be coordinated.

Provider:	Detector Infrastructure and Utilities Integration (6.03.01.09.04)

Details:	See details in the Detector Infrastructure and Utilities Integration Interface Control Document.

Requirements:	P-DET-PID.5, P-DET-TRAK.9

References:

I-DET-INF-INT.103	Space Constraint

System 1:	Detector Infrastructure and Utilities Integration (6.03.01.09.04)

System 2:	Barrel Tracking Systems (6.03.01.02)

Type:	SPACE

Description:	The interior radius of the inner MPGD barrel trackers is limited by the exterior radius of the pixel support tube. Modifications to either must be coordinated.

Provider:	Detector Infrastructure and Utilities Integration (6.03.01.09.04)

Details:	See details in the Detector Infrastructure and Utilities Integration Interface Control Document.

Requirements:	P-DET-PID.5, P-DET-TRAK.9

References:

I-DET-INF-INT.107	Space Constraint

System 1:	Barrel TOF Particle ID Systems (6.03.01.03)

System 2:	Barrel Tracking Systems (6.03.01.02)

Type:	SPACE

Description:	The maximum size of the silicon trackers is limited by the interior radius of the pixel support tube. Modifications to either must be coordinated.

Provider:	Detector Infrastructure and Utilities Integration (6.03.01.09.04)

Details:	See details in the Detector Infrastructure and Utilities Integration Interface Control Document.

Requirements:	P-DET-PID.5, P-DET-TRAK.9

References:

I-DET-INF-INT.108	Space Constraint

System 1:	Backward RICH Detector Systems (6.03.01.03)

System 2:	Barrel Tracking Systems (6.03.01.02)

Type:	SPACE

Description:	The maximum backward location for the tracking system is limited by the position of the pfRICH. Modifications to either must be coordinated.

Provider:	Detector Infrastructure and Utilities Integration (6.03.01.09.04)

Details:	See details in the Detector Infrastructure and Utilities Integration Interface Control Document.

Requirements:	P-DET-PID.5, P-DET-TRAK.9

References:

# Tracking Systems (DET-TRAK) to Barrel Particle ID Systems (DET-PID-BAR)

I-DET-INF-INT.101	Service Pathway

System 1:	Barrel DIRC Particle ID Systems (6.03.01.03)

System 2:	Barrel Tracking Systems (6.03.01.02)

Type:	SPACE

Description:	Conduits must be provided within the global support tube that are adequate to deliver services (power, signal, cooling) to the barrel tracking detectors.

Provider:	Detector Infrastructure and Utilities Integration (6.03.01.09.04)

Details:	See details in the Detector Infrastructure and Utilities Integration Interface Control Document.

Requirements:	P-DET-PID.5, P-DET-TRAK.9

References:

I-DET-INF-INT.107	Space Constraint

System 1:	Barrel TOF Particle ID Systems (6.03.01.03)

System 2:	Barrel Tracking Systems (6.03.01.02)

Type:	SPACE

Description:	The maximum size of the silicon trackers is limited by the interior radius of the pixel support tube. Modifications to either must be coordinated.

Provider:	Detector Infrastructure and Utilities Integration (6.03.01.09.04)

Details:	See details in the Detector Infrastructure and Utilities Integration Interface Control Document.

Requirements:	P-DET-PID.5, P-DET-TRAK.9

References:

# Tracking Systems (DET-TRAK) to Backward Particle ID Systems (DET-PID-BCK)

I-DET-INF-INT.108	Space Constraint

System 1:	Backward RICH Detector Systems (6.03.01.03)

System 2:	Barrel Tracking Systems (6.03.01.02)

Type:	SPACE

Description:	The maximum backward location for the tracking system is limited by the position of the pfRICH. Modifications to either must be coordinated.

Provider:	Detector Infrastructure and Utilities Integration (6.03.01.09.04)

Details:	See details in the Detector Infrastructure and Utilities Integration Interface Control Document.

Requirements:	P-DET-PID.5, P-DET-TRAK.9

References:

# Tracking Systems (DET-TRAK) to Forward Particle ID Systems (DET-PID-FWD)

I-DET-INF-FWD.011	Space Constraint

System 1:	Forward Time of Flight Systems (6.03.01.03)

System 2:	Barrel Tracking Systems (6.03.01.02)

Type:	SPACE

Description:	The maximum forward location for the tracking system is limited by the position of the forward AC LGAD Time of Flight Detector. Modifications to either must be coordinated.

Provider:	Hadron Endcap Structures (6.03.01.09.03)

Details:	See details in the Hadron Endcap Structures Interface Control Document.

Requirements:	P-DET-PID.5, P-DET-TRAK.9

References:

# Applicable Documents

List applicable documents considered to be part of this document. Applicable documents shall be obtained from the EIC SharePoint Document Center.

# Detailed change Log

| Revision | Date | Change |
| --- | --- | --- |

# References

- doi:10.17226/25171., National Academies of Sciences, Engineering, and Medicine, "An Assessment of U.S.-Based Electron-Ion Collider Science", The National Academies Press, Washington, DC, 2018.

- EIC-ORG-PLN-010, Electron-Ion Collider L1 Requirements (Global Requirements Document)

- BNL-221006-2021-FORE, EIC Conceptual Design Report

- arXiv:2103.05419, Science Requirements and Detector Concepts for the Electron-Ion Collider: EIC Yellow Report

- EIC-SEG-PLN-016, Requirements Management Plan

- EIC-SEG-PLN-022, Systems Engineering Management Plan

- EIC-SEG-PLN-020, EIC Interface Management Plan